# Carbon dioxide and methane emissions and pore fluid concentration data from peat mesocosm experiments under variable water table treatments

# Summary

This dataset includes greenhouse gas emissions measurements from peat mesocosm experiments using peat collected from sites in the East Anglian Fens. CO2 and CH4 emission data is provided for “short-term” experiments in which columns of drained peat were subjected to a high water table and then gradually drained over ~50 days with CO2 and CH4 emissions monitored as the water table is lowered. CO2 and CH4 emission and pore fluid concentration data is provided for “long-term” experiments in which mesocosms of drained peat were subjected to high water table (5cm above the soil surface) for a period of around 5 months. All data comes from mesocosm experiments carried out in the Department of Earth Sciences, University of Cambridge. Soils were collected from three sites in the East Anglian Fens. This data is associated with the paper ‘Methane emissions respond slowly to changes in soil water table depth in contrast to carbon dioxide emissions’ which can be read at https://doi.org/10.1088/2515-7620/ae50d7 (Marquand et al., 2026).

This work was carried out as part of the Centre for Landscape Regeneration at the University of Cambridge (Landscape Regeneration Solutions to the Interlinked Extinction and Climate Crises that support Sustainable Development, NE/W00495X/1) with analytical facilities funded by LASER-ENVI – A LASER spectrometer based ENVIronmental Gas and Gas-Isotope Facility (NE/V015435/1).

# File format

All data is provided in CSV format.

# File names

Three files are provided: “GHG_data_short-term_exps.csv” includes the CO2 and CH4 emission and water table depth data for the short-term mesocosm experiments; “GHG_data_long-term_exps.csv” includes the CO2 and CH4 emission data for the long-term mesocosm experiments; “GHG_data_pore_fluids.csv” includes the pore fluid dissolved CO2 and CH4 data from the long-term mesocosm experiments. The list of column headings and descriptions/unit can be found in Tables 1 – 3.

# Nature and units of recorded values

Cells are left blank where measurements of parameters were not made or where measurement or instrument errors prevented measurement.

Table 1. Description of variables in “GHG_data_short-term_exps.csv”

| Variable/Column header | Description | Units | Method |
| --- | --- | --- | --- |
| Experiment_number | Numerical code for each mesocosm experiment (numbered 1-3) |  |  |
| Site_name | Name of site from which soil was collected |  |  |
| Site_lat | Latitude of sampling location from which soil was collected (WGS 84) | Decimal degrees |  |
| Site_long | Longitude of sampling location from which soil was collected (WGS 84) | Decimal degrees |  |
| Soil_collection_date | Date when soils were collected from site | dd/mm/yyyy |  |
| Measurement_datetime | Date and time of measurement | dd/mm/yyyy HH:MM |  |
| Lab_temperature | Air temperature in lab | °C | 1 |
| WT_depth | Water table depth, negative values indicate depth of standing water above soil surface | cm | 2 |
| Flux_CO2 | Rate of emission of CO2 from mesocosm | mmol m-2 day-1 | 3 |
| SD_flux_CO2 | Standard deviation on repeat measurements of CO2 emission | mmol m-2 day-1 | 3 |
| Flux_CH4 | Rate of emission of CH4 from mesocosm | mmol m-2 day-1 | 3 |
| SD_flux_CH4 | Standard deviation on repeat measurements of CH4 emission | mmol m-2 day-1 | 3 |

Table 2. Description of variables in “GHG_data_long-term_exps.csv”

| Variable/Column header | Description | Units | Method |
| --- | --- | --- | --- |
| Experiment_number | Numerical code for each mesocosm experiment (numbered 4-7) |  |  |
| Site_name | Name of site from which soil was collected |  |  |
| Site_lat | Latitude of sampling location from which soil was collected (WGS 84) | Decimal degrees |  |
| Site_long | Longitude of sampling location from which soil was collected (WGS 84) | Decimal degrees |  |
| Soil_collection_date | Date when soils were collected from site | dd/mm/yyyy |  |
| Measurement_datetime | Date and time of measurement | dd/mm/yyyy HH:MM |  |
| Days_from_raising_WT | Days after water table raised in mesocosm, negative values for data before raising water table | days |  |
| WT_low_high | “low” for data collected when soil mesocosm was fully drained, “high” for data collected when soil mesocosm was waterlogged (~5cm of standing water) |  |  |
| Flux_CO2 | Rate of emission of CO2 from mesocosm | mmol m-2 day-1 | 3 |
| Observed_ebb | “y” when transient peaks in CH4 emissions were observed which are associated with ebullitive (bubble) emissions, else blank |  |  |
| D_Flux_CH4 | Rate of diffusive emission of CH4 from mesocosm | mmol m-2 day-1 | 3 |
| E_flux_CH4 | Rate of ebullitive emission of CH4 from mesocosm | mmol m-2 day-1 | 4 |
| Flux_CO2e | CO2-equivalent greenhouse gas flux including both CO2 and CH4 and using GWP100 = 28 for CH4 | g CO2e m-2 day-1 |  |

Table 3. Description of variables in “GHG_data_pore_fluids.csv”

| Variable/Column header | Description | Units | Method |
| --- | --- | --- | --- |
| Experiment_number | Numerical code for each mesocosm experiment (numbered 4, 5 and 7) |  |  |
| Site_name | Name of site from which soil was collected |  |  |
| Site_lat | Latitude of sampling location from which soil was collected (WGS 84) | Decimal degrees |  |
| Site_long | Longitude of sampling location from which soil was collected (WGS 84) | Decimal degrees |  |
| Soil_collection_date | Date when soils were collected from site | dd/mm/yyyy |  |
| Measurement_date | Date of measurement | dd/mm/yyyy |  |
| Days_from_raising_WT | Days since water table raised in mesocosm, negative values for data before raising water table | days |  |
| Sample_depth | Depth of pore fluid sample measured from top of soil in mesocosm tube, negative (or 0) values indicate measurements of standing water above soil surface | cm |  |
| Conc_CO2 | Dissolved CO2 concentration | mM | 5 |
| Conc_CH4 | Dissolved CH4 concentration | μM | 5 |

# Temporal coverage

Data comes from laboratory-based mesocosm experiments which were carried out between February 2023 and December 2025.

Experimental design/sampling regime
Six mesocosm experiments were carried out to investigate the response of CO2 and CH4 emissions to changes in water table depth in peat mesocosms. Soils were collected by Dutch augur from sites in the East Anglian Fens (site coordinates are in data files). Around 40cm of soil was added to each of six PVC mesocosm tubes with 7.5cm internal diameter and 10cm of headspace. In the short-term experiments (Experiments 1 to 3), the water table was initially raised to flood the soil and then gradually lowered over the course of 50 days. In the long-term experiments (Experiments 4 to 6), the water table was raised (around 5cm above the surface) and maintained raised for around 5 months. In an additional long-term experiment (Experiment 7), an intact core was collected by hammering the mesocosm tube (8.1cm internal diameter) into the ground before digging it up using a spade. Otherwise, this intact core was treated the same as the other long-term experiments. The all experiments, CO2 and CH4 fluxes were measured approximately twice a week. In Experiments 4, 5 and 7, pore fluid CO2 and CH4 fluxes were also measured various depths using RhizonTM samplers. Additional details can be found at https://doi.org/10.1088/2515-7620/ae50d7.

# Collection/Generation methods

Methods used are numbered in Tables 1 – 3 above. Note that all data was typed into a Microsoft Excel spreadsheet at the time of collection; sheets from the Excel spreadsheet were converted to CSV format for deposit in the data repository. Additional details can be found at https://doi.org/10.1088/2515-7620/ae50d7.

- Lab air temperature

Lab air temperature measured by Elitech RC-5+ Temperature Data Logger. Data logger was left in lab with mesocosm experiments.

- Water table depth

Mesocosms were carried out in transparent PVC tubes. Water table depth was measured using a ruler (measuring down from the soil surface) by visual inspection.

- Diffusive gas emission (CO2 and CH4)

Diffusive gas emission was measured using an open dynamic chamber approach. The mesocosm tube headspace was fitted with an inflow and outflow pipe. The outflow pipe was connected to a Picarro G2201-i Cavity Ringdown Spectrometer (CRDS) which measured the CH4 and CO2 concentration. The rate of emission of CO2 and CH4 was calculated from the different between the background air CO2 and CH4 concentration (measured using the CRDS before and after each flux measurement) and the CO2 and CH4 concentration in the mesocosm outflow gas by multiplying by the flow-through rate (20cm3 min-1). This is the standard method for dynamic gas flux chamber measurements (see for example Gao and Yates, 1998).

- Ebullitive gas flux

Methane ebullition events were identified as transient peaks in the methane concentration-time graph from the CRDS. The rate of ebullitive emission was calculated from the area under the transient concentration peak using a method similar to that of Yu et al. (2014). The stable baseline gas concentration associated with diffusive methane efflux was identified visually on the concentration time plot and the area under the transient ebullitive peaks was calculated numerically and divided by the measurement period (periods >6 hours were used for ebullitive flux measurements to mitigate temporal sampling bias associated with the stochastic timing of methane ebullition). All fluxes were divided by the cross-sectional area of the soil mesocosm surface and reported per square metre.

- Pore fluid CO2 and CH4 concentrations

Pore fluids were extracted from mesocosm tubes by drawing into a syringe from RhizonTM samplers (Rhizosphere Research Products) which were left in place throughout the experiments in predrilled holes in the mesocosm tube. Around 5mL of pore fluid was collected from each depth (~2cm resolution). Pore fluids were immediately injected into N2-flushed 120mL serum vials and shaken thoroughly to encourage headspace equilibration. The headspace CO2 and CH4 was measured using the CRDS. The amount of CO2 and CH4 in the headspace was calculated using the ideal gas law. CO2 and CH4 which remained dissolved in the pore fluid was accounted for by estimating the dissolved CO2 and CH4 using Henry’s Law. The total amount of CO2 and CH4 in the vial was divided by the volume of the pore fluid to estimate the original concentration of CO2 and CH4 in the pore fluid sample. These should be thought of as minimum concentrations as some gas may be lost due to degassing during sampling.

Fieldwork and/or laboratory instrumentation

- Picarro G2201-i Cavity Ringdown Spectrometer, serial no.: 3983-CFIDS2314

- Elitech RC-5+ PDF Temperature Datalogger, serial no.: EFI214103456

Calibration steps and values

Picarro G2201-i Cavity Ringdown Spectrometer

Calibrated by manufacturer and calibration checked upon local installation by manufacturer certified installation technician. Calibration regularly confirmed by measurements of ambient outdoor air (typically in range 400-450ppm CO2 and 1.8 to 2.2ppm CH4). Calibration additionally checked and confirmed regularly by repeat measurements of Calgaz certified (±2%) 10ppm, 100ppm and 1000ppm CH4 in tanks balanced with zero air or N2.

Elitech RC-5+ PDF Temperature Datalogger:

Calibrated by distributer (Elitech (UK) Limited), 29/04/2021, accurate ±0.5°C within range -25°C to 60°C.

Table 5 – Calibration data for Elitech RC-5+ PDF Temperature Datalogger

| Real temperature, °C | Measured temperature, °C | Error, °C |
| --- | --- | --- |
| 60 | 60.2 | +0.2 |
| 40 | 40.3 | +0.3 |
| 25 | 25.0 | 0.0 |
| 0 | 0.0 | 0.0 |
| -10 | -9.8 | +0.2 |
| -25 | -24.4 | +0.6 |

# Quality control

The measurement method for gas fluxes requires a stable gas concentration associated with diffusive emissions (see Methods 3 and 4). Where no stable concentration was measured, the emission flux was not recorded. Unstable measurements were most often associated with changes in the laboratory background air due to people entering/leaving the laboratory or breathing close to the inlet tube.

Missing pore fluid data is either due to measurement failure associated with inadequately sealed serum vials (see Methods 5) or low pore fluid volume caused by blockage or damage to RhizonTM sampler.

# Miscellaneous

Further information about experimental methods and data analysis can be found at https://doi.org/10.1088/2515-7620/ae50d7.

# References

Gao, F. and Yates, S.R. (1998) ‘Laboratory study of closed and dynamic flux chambers: Experimental results and implications for field application’, Journal of Geophysical Research: Atmospheres, 103(D20), pp. 26115–26125. Available at: https://doi.org/10.1029/98JD01346.

Marquand, T.I. et al. (2026) ‘Methane emissions respond slowly to changes in soil water table depth in contrast to carbon dioxide emissions’, Environmental Research Communications [Preprint]. Available at: https://doi.org/10.1088/2515-7620/ae50d7.

Yu, Z. et al. (2014) ‘Dynamics of methane ebullition from a peat monolith revealed from a dynamic flux chamber system’, Journal of Geophysical Research: Biogeosciences, 119(9), pp. 1789–1806. Available at: https://doi.org/10.1002/2014JG002654.