# Loch Leven monitoring silica data, 2007-2023

# Supporting Information

## Abstract

This is a publication of a dataset of silica concentrations in water from Loch Leven, Scotland, between years 2007 and 2023. The data were collected as part of the long-term monitoring programme of this lowland loch, with monitoring being generally performed every 2 weeks.

This programme began in 1968 and is still underway at the time of writing. Sampling and analyses were conducted by UK Centre for Ecology & Hydrology (UKCEH). This document describes the nature of the data, sampling and analysis methods used to obtain them, the data processing techniques used and the format of the output file. The data collection and processing were supported by the Natural Environment Research Council (NERC) award number NE/R016429/1 as part of the UK-SCAPE programme delivering National Capability.

## Sampling regime

In general, sampling was conducted on a fortnightly basis. On each sampling occasion, two main sites were visited. These are the Reed Bower and Sluices sites (see Table 1, below). The Reed Bower site is only accessible by boat, whereas the Sluices site is either accessed by boat, or when conditions do not allow, a representative sample is taken on foot from the nearby lake outlet. Although data is recorded as ‘Outlet’ rather than ‘Sluices’ when the boat is not used, these values have all been referenced as ‘Sluices’ (L / Sl8) in this dataset for ease of use.

Several circumstances can prevent sampling on occasion. These include resource constraints, bad weather or ice cover. When this has prevented safe use of a boat, only the Sluices site could be sampled and in extreme cases, neither site can be visited.

Silica analyses were conducted in a laboratory using samples collected in the field. The details of the sampling, sample processing and chemical analysis methods are detailed in this document.

## List of Sites

Table 1 and Figure 1 show a list of the sampling sites represented in the Loch Leven datasets by their corresponding Site IDs. Sites are recorded in the data using the last value in the column name, after the last underscore. These codes are used throughout the long-term monitoring programme at Loch Leven and refer to the Harbour (Site_H), Reed Bower (Site_RB) and the Sluices (Site_L), with some alternative IDs used in some data shown in Table 1. In this dataset, only the Reed Bower and the Sluices sites are represented.

Table 1 List of sampling sites represented in the Loch Leven datasets, showing their ID, name and location (as UK National Grid Easting and Northing, epsg:27700).

| Site ID |  | Site Name | Easting | Northing |
| --- | --- | --- | --- | --- |
| H |  | Harbour | 312251 | 701670 |
| RB / RB5 |  | Reed Bower | 313568 | 701141 |
| L / Sl8 |  | Sluices / Outflow | 317004 | 699380 |

Figure 1 Map of Loch Leven showing the location of the three monitoring sites, with names / IDs.

## Sampling, sample processing and analysis methods

In general, on each sampling occasion, duplicate 2 L water samples were collected from each site. At the Reed Bower site, an integrated sample was collected using a PVC tube with an internal diameter of 25 mm, which was weighted at one end. This tube was lowered into the water until the weighted end was within 0.25 m of the lake bottom, filling as it was lowered. The weighted end was then lifted out of the lake while ensuring that the other end of the tube remained stationary, ensuring that the sample was collected from the entire water column. At the Sluices site, water was collected using a bottle that was held 10 cm below the surface.

On return to the laboratory, samples were shaken and sub-samples of filtered (using Whatman® GF/C filters, within 6 hours of collection) / unfiltered water were taken from each duplicate site sample for soluble reactive silica (filtered and stored in fridge) and total diatom silica (unfiltered and stored in fridge) analyses.

Soluble reactive silica and total diatom silica concentrations in water samples were determined according to the colorimetric method outlined in Golterman et al. (1978), Paasche (1980) and Kirka (2000). The method uses acidic ammonium molybdate to form a silicomolybdate complex that is reduced with ascorbic acid to produce a blue-coloured solution (oxalic acid is added to eliminate the interference from any phospho-molybdate complex formation). Concentrations were determined by measuring the absorbance at 810 nm and comparing this against known standards using a Perkin Elmer Lambda 365 spectrophotometer. Prior to analysis, 1 mL of each unfiltered sample was digested by adding sodium carbonate solution (Na2CO3) and heated at 120oC for 30 minutes. The ‘gentle’ digestion of unfiltered samples using Na2CO3 converts the opaline silica of diatom frustules to a soluble form that can then be measured using the method for soluble reactive silica. The digestion is not considered strong enough to include any suspended sand particles from the sediments. This silica dataset aligns to previous data publications of the Loch Leven monitoring data on EIDC (https://catalogue.ceh.ac.uk/documents/ac973b0d-2c99-4e00-8931-22aa0881006d).

NB Due to resource constrains, analysis was focused mainly on samples collected at the Reed Bower site prior to November 2012. On some dates, samples were only collected at the Sluices site due to weather conditions, which did not allow sampling from a boat at Reed Bower. On a few occasions, results were omitted due to analytical issues.

## Sampling, measurement and analysis – References

Golterman, H.L., Clymo, R.S., Ohnstad, M.A.M. (1978). Methods for Physical and Chemical Analysis of Fresh Waters. IBP Handbook No. 8. Blackwell, Oxford.

Kirka A. 2000. Loch Leven Field Notes. CEH Edinburgh (unpublished).

Paasche E. (1980). Silicon content of five marine plankton diatom species measured with a rapid filter method. Limnol. Oceanogr. Vol. 25, No.3, pp 474-480.

## Sampling, measurement and analysis - further reading

Additional information can be gained from a special issue of the journal Hydrobiologia, and a report written for Scottish Natural Heritage.

Dudley, B.J., Spears, B.M., Carvalho, L., Gunn, I.D.M. & May, L., 2012. Water quality monitoring at Loch Leven 2008-2010 – Report of results. Scottish Natural Heritage Commissioned Report No. 511

Spears, B. M. & May, L. (editors) 2012. Loch Leven: 40 years of scientific research. Developments in Hydrobiology 218. Springer, Dordrecht.

Quality Assurance
A manual check was performed on all data to look for any errors in data entry or values reported outside expected ranges - any unusual values were then investigated and corrected where appropriate.

## Format of stored data

Data is stored in a single comma-separated values (CSV) file. Missing data values (when a measurement was not taken / recorded) are marked as NA (‘Not Available’) throughout, as used in a number of data standards.

## Column descriptions

All column names contain basic information on what is being measured, at what site and in what units. Table 2 (below) shows this information in more detail, giving full descriptions of everything in the dataset and should be seen as the core reference for what each column represents.

Table 2 Detailed information on column headings, giving descriptions of the determinand, site and units.

| Column Name | Description | Site | Units |
| --- | --- | --- | --- |
| Date | Sampling date | All | date |
| Week_of_Year | Week in the year | All | (1-52) |
| SRSiO2_mgSiO2/L_Site_RB1 | Concentration of soluble reactive silica of water sample (replicate 1) | Reed Bower | milligrams of silica / litre |
| SRSiO2_mgSiO2/L_Site_RB2 | Concentration of soluble reactive silica of water sample (replicate 2) | Reed Bower | milligrams of silica / litre |
| SRSiO2_mgSiO2/L_Site_L1 | Concentration of soluble reactive silica of water sample (replicate 1) | Sluices | milligrams of silica / litre |
| SRSiO2_mgSiO2/L_Site_L2 | Concentration of soluble reactive silica of water sample (replicate 2) | Sluices | milligrams of silica / litre |
| TSiO2_mgSiO2/L_Site_RB1 | Concentration of total diatom silica of water sample (replicate 1) | Reed Bower | milligrams of silica / litre |
| TSiO2_mgSiO2/L_Site_RB2 | Concentration of total diatom silica of water sample (replicate 2) | Reed Bower | milligrams of silica / litre |
| TSiO2_mgSiO2/L_Site_L1 | Concentration of total diatom silica of water sample (replicate 1) | Sluices | milligrams of silica / litre |
| TSiO2_mgSiO2/L_Site_L2 | Concentration of total diatom silica of water sample (replicate 2) | Sluices | milligrams of silica / litre |