# Concentrations of organochlorine pesticides (OCs), polychlorinated biphenyls (PCBs) and polybrominated diphenyl ethers (PBDEs) in sediments of the Forth estuary, 2020-2021, and north Aberdeenshire coastline, 2023, Scotland

## Overview

This dataset comprises the concentrations of organochlorine pesticides (OCs), polychlorinated biphenyls (PCBs), and polybrominated diphenyl ethers (PBDEs) measured in bed sediments collected from the Forth estuary in Scotland in 2020-2021, in addition to the north Aberdeenshire coastline in 2023. OCs and PBCs were analysed using gas chromatography-mass spectrometry (GC-MS). PBDEs were analysed using gas chromatography-tandem mass spectrometry (GC-MS/MS). These data were collected as part of the Natural Environment Research Council (NERC) funded project entitled Uptake of chemicals from legacy waste sites in coastal food webs and effects on higher predators (NERC reference NE/T003596/1).

## Data structure

The dataset comprises a csv file entitled “POPs_data_Forth_Aberdeenshire_sediment_2020_2023.csv”. The csv file has 72 columns. The first details the sample ID, the second is the sampling area, the third is the sampling site, and the fourth is the date of sampling. Columns 5-6 are the coordinates of the sampling points (longitude and latitude). Columns 7-72 are the concentrations of the analytes reported in ng g-1 wet weight (ww). Details of all compounds that were analysed are described in Table 1. Values for each analyte below the limit of detection have been assigned a value of zero.

## Sampling method

A total of 146 bed sediment surface samples were collected at a range of depths in the water column (2-57 m) in the Forth estuary from November 2020 to July 2021. Two sampling areas were selected, specifically the inner estuary channel (sites west of longitude -2.9) and the outer estuary channel (sites east of longitude -2.9). Samples were collected from the surface of the estuary bed using a 250 cm2 Van Veen grab sampler. Samples were stored in glass containers at -20 °C.

Samples were collected at three time periods: winter 2020-21 (n = 52; inner sites = 24; outer sites = 28; 03.11.20 - 03.03.21), spring 2021 (n = 53; inner = 27; outer = 26; 07.04.21 - 29.06.21), and summer 2021 (n = 41; inner sites sampled only due to logistical challenges; 14.07.21 – 23.07.21) from a total of 76 sites in the estuary and neighbouring coastal area (Figure 1). A further 9 samples were collected from sites along the North Aberdeenshire coastline in winter 2023 (21.02.23 - 22.02.23).

## Analytical methods

### POPs extraction

Sediment samples were thawed and ~2-4 g ww of each sample was accurately weighed, homogenised, and dried with anhydrous sodium sulfate. Samples were spiked with 13C-labelled recovery standards for PCBs (13C-PCB 153, 180, and 209, Cambridge Isotope Labs, USA), OCs (13C-α-HCH, Cambridge Isotope Labs, USA), and PBDEs (13C-PBDE 28, 47, and 153, Wellington Labs, Canada) and Soxhlet-extracted with DCM for 16 h. Extracts were dried using anhydrous sodium sulfate, reduced to a known volume on a parallel evaporator and filtered through a 0.45 µm PTFE syringe filter. Extracts were cleaned using automated size-exclusion liquid chromatography (SEC; Agilent 1200 series, Agilent, USA) using a DCM mobile phase. Following SEC clean-up, extracts were loaded onto a glass column packed with Al2O3 deactivated with 5% deionised water (w/w) and analytes were eluted using hexane. Cleaned extracts were concentrated to a known volume and split into two fractions; one fraction to be analysed for PCBs and OCs, and the other for PBDEs. Prior to instrumental analysis, 13C-labelled internal standards were added to each fraction: 13C-PCB 8, 167 and 194 (Cambridge Isotope Labs, USA) and 13C-HCB (Wellington Labs, Canada) to the PCB/OC fraction; 13C-PBDE 77 and 138 (Wellington Labs, Canada) to the PBDE fraction. A full procedural blank was carried out with each batch of samples extracted.

### POPs analysis, quality assurance and quality control

Sediment samples were analysed for a suite of 35 polychlorinated biphenyls (PCBs; di- to deca-PCBs), 25 polybrominated diphenyl ethers (PBDEs; tri- to octa-PBDE), and 7 organochlorine pesticides (OCs).

PCBs and OCs were analysed using an Agilent 7890B gas chromatograph coupled to an Agilent 5977B single-quadrupole mass spectrometer operating in electron ionisation mode. Samples were injected in splitless mode on to a Restek SGE HT8 analytical column (50 m length, 0.25 mm internal diameter, 0.22 µm film thickness). The inlet temperature was 60 °C, ramping at 600 °C min-1 to 325 °C. Following injection, the oven was held at 50 °C (2.5 min), followed by ramps to 200 °C (60 °C min-1), 235 °C (1.5 °C min-1), 240 °C (1 °C min-1, held for 12.5 min), 250 °C (1 °C min-1, held for 15 min), 280 °C (1 °C min-1, held for 5 min), 310 °C (40 °C min-1, held for 5 min). Helium was used as the carrier gas (1 mL min-1). The MS source temperature was 250 °C, with a quadrupole temperature of 150 °C. The analysis was carried out in single ion monitoring (SIM) mode.

PBDEs were analysed using an Agilent 7890B gas chromatograph coupled to an Agilent 7010B triple-quadrupole mass spectrometer operating in electron ionisation mode. 4 µL of sample was injected in splitless mode on to a Restek SGE HT8 analytical column (12 m length, 0.25 mm internal diameter, 0.22 µm film thickness). The inlet temperature was 90 °C, ramping at 600 °C min-1 to 400 °C. Following injection, the oven was held at 50 °C (4 min), followed by ramps to 270 °C (50 °C min-1), 290 °C (4 °C min-1), 320 °C (15 °C min-1, held for 6 min). Helium was used as the carrier gas (1 mL min-1). The MS source temperature was 250 °C, with a quadrupole temperature of 150 °C. The analysis was carried out in multiple reaction monitoring (MRM) mode.

Quantification of all analytes was carried out using 13C-labelled internal standards and calibration curves of unlabelled analyte standards (Chem Service & Cambridge Isotope Labs, both USA; Wellington Labs, Canada). Recoveries of 13C-labelled recovery standards ranged from 60-120%. Concentrations were recovery-corrected in order to take into account potential matrix effects (both suppression and enhancement) and potential losses in the analytical procedure. All concentrations were blank-corrected were necessary, and are reported in ng g-1 ww (wet weight). Mean limits of detection (LODs) ranged from 0.03-0.52 ng g-1 ww (median LOD 0.08 ng g-1), and were calculated from the mass of sample extracted, the dilution factor, and the lowest standard in the calibration curve.

### Table 1. Compounds analysed

| Class | Name/congener | Cl/Br number | CAS |
| --- | --- | --- | --- |
| OC | HCB | - | 118-74-1 |
|  | α-Lindane (α-HCH) | - | 319-84-6 |
|  | Lindane (γ-HCH) | - | 58-89-9 |
|  | Dieldrin (HEOD) | - | 60-57-1 |
|  | DDE | - | 72-55-9 |
|  | DDT | - | 50-29-3 |
|  | DDD (TDE) | - | 72-54-8 |
| PCB | PCB 8 | 2 | 34883-43-7 |
|  | PCB 18 | 3 | 37680-65-2 |
|  | PCB 28 | 3 | 7012-37-5 |
|  | PCB 29 | 3 | 15862-07-4 |
|  | PCB 31 | 3 | 16606-02-3 |
|  | PCB 52 | 4 | 35693-99-3 |
|  | PCB 77 | 4 | 32598-13-3 |
|  | PCB 101 | 5 | 37680-73-2 |
|  | PCB 105 | 5 | 32598-14-4 |
|  | PCB 114 | 5 | 74472-37-0 |
|  | PCB 118 | 5 | 31508-00-6 |
|  | PCB 123 | 5 | 65510-44-3 |
|  | PCB 126 | 5 | 57465-28-8 |
|  | PCB 128 | 6 | 38380-07-3 |
|  | PCB 138 | 6 | 35065-28-2 |
|  | PCB 141 | 6 | 52712-04-6 |
|  | PCB 149 | 6 | 38380-04-0 |
|  | PCB 153 | 6 | 35065-27-1 |
|  | PCB 156 | 6 | 38380-08-4 |
|  | PCB 157 | 6 | 69782-90-7 |
|  | PCB 163 | 6 | 74472-44-9 |
|  | PCB 167 | 6 | 52663-72-6 |
|  | PCB 169 | 6 | 32774-16-6 |
|  | PCB 170 | 7 | 35065-30-6 |
|  | PCB 171 | 7 | 52663-71-5 |
|  | PCB 180 | 7 | 35065-29-3 |
|  | PCB 183 | 7 | 52663-69-1 |
|  | PCB 187 | 7 | 52663-68-0 |
|  | PCB 189 | 7 | 39635-31-9 |
|  | PCB 194 | 8 | 35694-08-7 |
|  | PCB 199 | 8 | 52663-75-9 |
|  | PCB 201 | 8 | 40186-71-8 |
|  | PCB 205 | 8 | 74472-53-0 |
|  | PCB 206 | 9 | 40186-72-9 |
|  | PCB 209 | 10 | 2051-24-3 |
| PBDE | BDE 17 | 3 | 147217-75-2 |
|  | BDE 28 | 3 | 41318-75-6 |
|  | BDE 30 | 3 | 155999-95-4 |
|  | BDE 32 | 3 | 189084-60-4 |
|  | BDE 35 | 3 | 147217-80-9 |
|  | BDE 37 | 3 | 147217-81-0 |
|  | BDE 47 | 4 | 5436-43-1 |
|  | BDE 49 | 4 | 243982-82-3 |
|  | BDE 51 | 4 | 189084-57-9 |
|  | BDE 66 | 4 | 189084-61-5 |
|  | BDE 71 | 4 | 189084-62-6 |
|  | BDE 77 | 4 | 93703-48-1 |
|  | BDE 85 | 5 | 182346-21-0 |
|  | BDE 99 | 5 | 60348-60-9 |
|  | BDE 100 | 5 | 189084-64-8 |
|  | BDE 118 | 5 | 446254-80-4 |
|  | BDE 119 | 5 | 189084-66-0 |
|  | BDE 126 | 5 | 366791-32-4 |
|  | BDE 128 | 6 | 182677-28-7 |
|  | BDE 138 | 6 | 182677-30-1 |
|  | BDE 153 | 6 | 68631-49-2 |
|  | BDE 154 | 6 | 207122-15-4 |
|  | BDE 183 | 7 | 207122-16-5 |
|  | BDE 190 | 7 | 189084-68-2 |
|  | BDE 196 | 8 | 446255-39-6 |

### Table 2. Percentage recoveries of labelled compounds in sediments

| Compound | Mean (%) | SD | Min (%) | Max (%) |
| --- | --- | --- | --- | --- |
| 13C-PCB 153 | 83.0 | 16.4 | 60.0 | 120.0 |
| 13C-PCB 180 | 89.8 | 16.0 | 60.4 | 119.3 |
| 13C-PCB 209 | 77.0 | 14.7 | 60.1 | 119.9 |
| 13C-α-HCH | 80.5 | 19.7 | 60.0 | 120.0 |
| 13C-PBDE 28 | 88.0 | 12.8 | 63.2 | 118.5 |
| 13C-PBDE 47 | 98.4 | 14.7 | 62.9 | 119.9 |
| 13C-PBDE 153 | 100.1 | 13.6 | 60.4 | 119.8 |

### Table 3. Limits of detection (LODs)

| Analyte | LOD (ng g-1 ww) |
| --- | --- |
| BDE 30 | 0.03 |
| BDE 32 | 0.03 |
| BDE 17 | 0.03 |
| BDE 28 | 0.03 |
| BDE 35 | 0.03 |
| BDE 37 | 0.03 |
| BDE 51 | 0.03 |
| BDE 49 | 0.03 |
| BDE 71 | 0.03 |
| BDE 47 | 0.03 |
| BDE 66 | 0.03 |
| BDE 77 | 0.03 |
| BDE 100 | 0.03 |
| BDE 119 | 0.03 |
| BDE 99 | 0.03 |
| BDE 118 | 0.03 |
| BDE 85+126 | 0.07 |
| BDE 154 | 0.08 |
| BDE 153 | 0.14 |
| BDE 138 | 0.14 |
| BDE 183 | 0.07 |
| BDE 128 | 0.28 |
| BDE 190 | 0.28 |
| BDE 196 | 0.52 |
| PCB 8 | 0.08 |
| PCB 18 | 0.08 |
| PCB 29 | 0.08 |
| PCB 31 | 0.08 |
| PCB 28 | 0.08 |
| PCB 52 | 0.08 |
| PCB 101 | 0.08 |
| PCB 77 | 0.08 |
| PCB 149 | 0.08 |
| PCB 123 | 0.08 |
| PCB 118 | 0.08 |
| PCB 114 | 0.08 |
| PCB 153 | 0.08 |
| PCB 141 | 0.08 |
| PCB 105 | 0.08 |
| PCB 163 | 0.08 |
| PCB 138 | 0.08 |
| PCB 187 | 0.08 |
| PCB 183 | 0.08 |
| PCB 126 | 0.08 |
| PCB 128 | 0.08 |
| PCB 167 | 0.08 |
| PCB 171 | 0.08 |
| PCB 199 | 0.08 |
| PCB 156 | 0.08 |
| PCB 157 | 0.08 |
| PCB 180 | 0.08 |
| PCB 201 | 0.08 |
| PCB 170 | 0.08 |
| PCB 169 | 0.08 |
| PCB 189 | 0.08 |
| PCB 194 | 0.08 |
| PCB 205 | 0.08 |
| PCB 206 | 0.08 |
| PCB 209 | 0.08 |
| α-Lindane (α-HCH) | 0.08 |
| HCB | 0.08 |
| Lindane (γ-HCH) | 0.08 |
| DDE | 0.08 |
| Dieldrin (HEOD) | 0.29 |
| DDD (TDE) | 0.08 |
| DDT | 0.33 |