Physicochemical parameters of Chennai and Pondicherry water samples near antimicrobial manufacturing sites, India, 2023-2024

Physicochemical data from Pondicherry and Chennai water samples collected in 2023 and 2024 are detailed in the applicable data resources.

# Summary

These datasets contain information about the physicochemical properties of water samples taken from Pondicherry and Chennai in 2023 and 2024. Water samples were collected from river, lake, canal and tank water bodies in Pondicherry and Chennai, with locations selected due to their proximity to antimicrobial manufacturing sites. The physicochemical properties of each sample were analysed to determine water quality at each site and how this may be impacted by local antimicrobial pollution. Variables such as; temperature, oxidation reduction potential, salinity, pH, dissolved oxygen content, conductivity, resistivity, total dissolved solids, atmospheric pressure and seawater sigma were accounted for.

# File format

Data files are in CSV format (converted from Excel)

# File names and/or naming convention

- PhysicochemData_AMRWatch_ChennaiWater23 – sample collection in Chennai on 24/3/2023

- PhysicochemData_AMRWatch_ChennaiWater24 – sample collection in Chennai on 13/3/2024

- PhysicochemData_AMRWatch_PondicherryWater23 – sample collection in Pondicherry on 28/05/2023

- PhysiochemData_AMRWatch_PondicherryWater24 – sample collection in Pondicherry on 25/4/2024

# Nature and units of recorded values

Table 1. Description of variables

| Variable/Column header | Description | Units | Vocabulary URL (if applicable) |
| --- | --- | --- | --- |
| Location | Location of sample collection CSLx: Chennai Sample Location PSLx: Pondicherry Sample Location |  |  |
| Time | Time of sample collection (Indian Standard Time) |  |  |
| Co-ordinates | Coordinates of sample location (WGS 84) | Degrees N, degrees E |  |
| Proximity_to_nearest_effluent_discharge_point | Distance in km from the sample site to the nearest effluent discharge point. Site type e.g downstream, lake, tank or upstream, is also noted. | Kilometres (km) |  |
| Temperature_celsius | Temperature of water samples at collection | Celsius (C°) |  |
| pH | pH of water samples | pH |  |
| Oxidation_Reduction_Potential | Oxidation reduction potential of water samples | ORP (mVORP) |  |
| Dissolved_Oxygen_Percent | Percentage of dissolved oxygen in water samples | Percentage (%) |  |
| Dissolved_Oxygen_mgL | Amount of dissolved oxygen in water samples measured in mg/L | Mg/L |  |
| Conductivity | Conductivity of water samples | μs/cm |  |
| Resistivity | Resistivity measured in water samples | MΩ𝑐𝑚 |  |
| Total_Dissolved_Solids | Total dissolved solids in water samples | ppm / mg/L |  |
| Salinity_psu | Salinity of water samples | Psu |  |
| Atmospheric_Pressure | Atmospheric pressure of water samples | Psi |  |
| Seawater_Sigma | Density of seawater samples | Σt |  |

Missing values in the data resource(s) are indicated by 'N/A’

# Spatial coverage

Table 2. Location of Sample Sites

| Sample Site | Location | Coordinates |
| --- | --- | --- |
| CSL1 | Stagnant Tank CETP, Chennai, India | 12.692488, 80.1960082 |
| CSL2 | Buckingham Canal, Chennai, India | 12.688627, 80.198804 |
| CSL3 | Buckingham Canal, Chennai, India | 12.658584, 80.197216 |
| CSL4 | Buckingham Canal, Chennai, India | 12.623116, 80.187001 |
| CSL5 | Buckingham Canal (backwater lake), Chennai, India | 12.593453, 80.166419 |
| CSL6 | Upstream (Illalur), Chennai, India | 12.689754, 80.127312 |
| PSL1 | Sankarabarani River, Pondicherry, India | 12.005955, 79.677851 |
| PSL2 | Ossudu Lake, Pondicherry, India | 11.9479225, 79.7574303 |
| PSL3 | Gingee River, Pondicherry, India | 11.9047982, 79.7404181 |
| PSL4 | Thiruvandarkoil Lake, Pondicherry, India | 11.9205495, 79.656162 |
| PSL5 | Sedarapet Lake, Pondicherry, India | 11.992799, 79.7515867 |
| PSL6 | Gingee River, Pondicherry, India | 11.904298, 79.749307 |
| PSL7 | Sangarabarani River, Pondicherry, India | 11.8915291, 79.7705104 |
| PSL8 | Chunnambar River, Pondicherry, India | 11.8775832, 79.7943098 |
| PSL9 | Ousteri Lake, Pondicherry, India | 11.943263, 79.742901 |
| PSL10 | Uruvaiyar Tank, Pondicherry, India | 11.898703 , 79.748733 |
| PSL11 | Thirukanji Tank, Pondicherry, India | 11.8879299, 79.7644928 |
| PSL12 | Gingee River, Pondicherry, India | 12.021168, 79.631094 |

# Temporal coverage and resolution

The sampling was conducted annually. In 2023, Chennai sampling was conducted on 24/3/2023, and Pondicherry sampling was conducted on 28/05/2023. In 2024, Chennai sampling was conducted on 13/3/2024 and Pondicherry sampling on 25/4/2024.

# Collection/Generation/Transformation methods

Sample Collection and Processing

- Samples were collected following a standardized operating procedure mutually agreed on by the collaborating partners. For water samples, collection was performed at depths ranging from 1 to 5 meters using a sterile container attached to a pulley system. All samples were collected in triplicates to ensure reproducibility and statistical reliability.

- Samples were transported to the laboratory on the same day in cold storage containers to preserve their integrity. Upon arrival, the samples were processed immediately for both culture-based microbiological analyses and metagenomic DNA extraction, thereby maintaining sample freshness and minimizing potential degradation.

- Parameters were measured in situ using a Portable multiparameter Aquaprobe AP-200 with a GPS Aquameter (Aquaread AP 2000).

Data Handling and Quality Control

- Initial data entries were recorded manually on paper and subsequently digitized into Excel spreadsheets. No imputation or gap-filling procedures were applied to the dataset. In cases where certain parameters exhibited readings below the detection threshold, the measurement units automatically shifted to a lower metric scale. For example, dissolved oxygen concentrations were recorded in mg/L by default but were represented in µg/L for particularly low-concentration samples. For the sake of consistency and uniformity across the dataset, all such values were converted back to the default unit of mg/L.

Storage of Bacterial Isolates

- Bacterial isolates obtained from each sampling site were preserved as glycerol stocks and stored at −80°C to maintain viability for future analyses.

# Sampling regime

- Composite samples were created from three sampling points in each location. Control samples were collected from upstream sites selected to represent areas unaffected by pollution from manufacturing sources. All samples were transported in a refrigerated isothermal container and stored at −20 °C until analysis

- Duplicate samples were randomly collected as part of quality control.

# Quality control

- Due to a lack of water at the site because of warm conditions, no water samples were collected from PSL10 in 2023. The levels were similarly low the following year, so this site was excluded from water and physicochemical analysis.