S

Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP)

ERAMMP Supporting Information Metadata for:

Headwater stream erosion data from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023

Client Ref: Welsh Government / Contract C210/2016/2017

Version 1.0

Date: 07/01/2026

Version History

| Version | By | Date | Changes |
| --- | --- | --- | --- |
| 1.0 | AD & JRL | 07/01/2026 | published |

Document Info

| Series | Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP) |
| --- | --- |
| Title | ERAMMP Supporting Information Metadata for: Headwater stream erosion data from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023 |
| Client | Welsh Government |
| Client reference | C208/2021/2022 |
| Confidentiality, copyright and reproduction | © Crown Copyright 2026 This document is licensed under the Open Government Licence 3.0. |
| UKCEH contact details | Bronwen Williams UK Centre for Ecology & Hydrology (UKCEH) Environment Centre Wales, Deiniol Road, Bangor, Gwynedd, LL57 2UW 01248 374500 erammp@ceh.ac.uk |
| Corresponding author | Pete Scarlett, UKCEH pds@ceh.ac.uk |
| Authors | Scarlett, P.1, Bentley, L.1, Doeser, A.1, Emmett, B.A.1, Lucas, J.R.1, Monkman, G.1, Reinsch, S.1, Richardson-Jones, V.1, Williams, B.1, Wood, C.M.1 |
| Contributing authors & reviewers | 1UK Centre for Ecology & Hydrology (UKCEH), 2Welsh Government |
| How to cite (long) | Scarlett, P., Bentley, L., Doeser, A., Emmett, B.A., Lucas, J.R., Monkman, G., Reinsch, S., Richardson-Jones, V., Williams, B., Wood, C.M. (2026). ERAMMP Supporting Information Metadata for: Headwater stream erosion data from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023 (Report-117). UKCEH |
| How to cite (short) | Scarlett, P. et al. (2026). ERAMMP Supporting Information Metadata for: Headwater stream erosion data from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023 (Report-117). UKCEH |
| Approved by | Bridget Emmett (UKCEH) Oliver Massey (Welsh Government) |

Abbreviations

| AES | agri-environment schemes |
| --- | --- |
| CS | Countryside Survey |
| ERAMMP | Environment and Rural Affairs Monitoring & Modelling Programme |
| EU | European Union |
| GB | Great Britain |
| GMEP | Glastir Monitoring and Evaluation Programme |
| ITE | Institute of Terrestrial Ecology |
| LCM | Land cover map (followed by the year) |
| LMS | land management scheme |
| NFS | National Field Survey |
| RHS | River Habitat Survey |

Contents

1.	Overview	5

2.	Background Information	5

3.	Introduction to the GMEP Survey Design and its re-survey by ERAMMP	6

4.	Data Collection Methods	7

4.1	Headwater stream erosion survey background	7

4.2	Survey method	8

5.	Data Quality Assurance	8

6.	Notes	9

8.	Data Structure	10

8.1	File details:	10

8.2	Table columns:	10

9.	References	11

# Overview

| Spatial Extent | Terrestrial extent of Wales, UK |
| --- | --- |
| Temporal Extent | 2021–2023 |
| Summary | This data set consists of headwater erosion process data for sites across Wales between 2021 and 2023, collected as part of the Environment and Rural Affairs Monitoring and Modelling Programme (ERAMMP) National Field Survey. Data include the classification and extent of erosion features. Headwater stream survey data was collected from 101 x 1km squares across Wales, collected as part of the Environment and Rural Affairs Monitoring and Modelling Programme (ERAMMP) National Field Survey. Erosion features were identified in 99 surveyed headwater streams. ERAMMP follows an earlier monitoring programme, the Glastir Monitoring and Evaluation Programme (GMEP), which was set up by the Welsh Government in 2013 to monitor the effects of the Glastir agri-environment scheme on the environment and ran from 2013 to 2016. The field survey element for both GMEP and ERAMMP was based on a stratified random sampling design of 300 x 1km square sites across Wales (225 of which were resurveyed for ERAMMP in 2021-23, though not all squares contained headwater streams). It was managed by the UK Centre for Ecology & Hydrology. |
| File Name(s) | ERAMMP_STREAM_EROSION_2021_23.csv |
| File Format(s) | 1x .csv |

# Background Information

In Wales, funding from agri-environment schemes (AES) has been available since the early 1990s including Environmentally Sensitive Areas, the Habitat Scheme, the Woodland Grant Scheme, the Organic Farming Scheme, Tir Cymen, Tir Cynnal, Tir Gofal, followed by the Glastir scheme. A monitoring programme was set up by the Welsh Government in 2013 to monitor the national state and change of the countryside, along with effects of the Glastir scheme on the environment. This four-year rolling monitoring programme was called the “Glastir Monitoring and Evaluation Programme” (GMEP) and was conducted between 2013 and 2016. GMEP aimed to: (1) Evaluate the environmental effects of the Glastir sustainable land management scheme (LMS) (https://gov.wales/glastir); (2) Quantify environmental status and spatiotemporal environmental status trends across the Welsh countryside and; (3) Assess and predict outcomes for climate change mitigation, biodiversity, soil and water quality, woodland expansion and cultural landscapes (https://gmep.wales). The project was led by UKCEH. A major component of the work to deliver the first 2 aims was a National Field Survey (NFS) which was an expansion of a long-term integrated monitoring programme developed by UKCEH since the 1970s called Countryside Survey (Carey et al., 2008, Wood et al., 2018, Robinson et al., 2024).

Synthesis and analysis of the data derived from the NFS sought to identify how the Welsh environment is being impacted by drivers of change, such as land use, climate, and pollution, over and above Glastir interventions. The GMEP report was published in 2017 (Emmett et al. 2017) and data are summarized in Wood et al. 2021. Data are available from the Environmental Information Data Centre (https://eidc.ac.uk/) as part of the data series of the Glastir Monitoring and Evaluation Programme, Wales, 2013-2016.

The ’Environment and Rural Affairs Monitoring & Modelling Programme’ (ERAMMP) National Field Survey is the re-survey of GMEP locations. Data informs national trends analyses, and the effects of Glastir action bundles. Other elements of GMEP and ERAMMP including numerous expert reviews and integrated modelling is reported elsewhere (https://erammp.wales/).

# Introduction to the GMEP Survey Design and its re-survey by ERAMMP

In the Glastir Monitoring and Evaluation Programme, the NFS was designed to capture multiple measures and metrics, and to integrate across these metrics. Hence a full ecosystem-based approach is required where data operating at multiple scales are captured, where possible, at the same time in a single snap-shot visit.

For GMEP, a 4-year rolling survey was adopted to maximise the number of sites visited across the national spatial scale whilst at the same time monitoring year-on-year, such that spatial variation and temporal changes and trends can be detected cost-effectively (Emmett et al., 2013 & Wood et al. 2021). The first survey cycle dates from 2013-2016. The first partial re-survey cycle took place between 2021 and 2023, after being postponed from 2020 due to the COVID pandemic.

The rolling programme-based survey has two key components: the Wider Wales Component used for baseline estimation (Welsh national trends) and national reporting of Glastir action bundles (Targeted Component) which specifically links to the 9 objectives (of which 6 environmental outcomes are intended to be captured in the NFS) of the Glastir scheme. Across both components, integration of survey data is essential and therefore a common spatial unit of 1 km x 1 km square was adopted.

The selection of the sample of 1-km x 1-km squares for monitoring was accomplished following the same procedure as used for the Countryside Survey (CS) of Great Britain (GB) (as described below), which aims to provide robust estimates of indicators at national level across GB and constituent countries. Using the same methodology builds on previous knowledge and enabled integration of the two surveys in future analyses.

A total of 300 x 1 km2 squares were sampled over the 4-year GMEP NFS cycle. Half of these squares (n=150) were the Wider Wales squares, randomly sampled within strata defined according to the Land Classification of Great Britain (Bunce et al., 2007) – a derived classification of the landscape based on its topography, geology, climate and physical attributes. Environmental heterogeneity is minimized within each stratum of the Land Classification and is maximised between strata. The proportion of the 1-km x 1-km squares randomly sampled from within each stratum was proportional to the size of the stratum to allocate survey efforts best. The other half of the sampled squares were targeted specifically at Glastir priority areas (n=150). Glastir-targeted squares were randomly selected with a selection probability proportional to the ‘Glastir score’ of the square, derived from the Welsh Government’s scoring system of the Glastir Advanced target objective areas (Emmett et al., 2014). Any square randomly selected that contained more than 75% of urban land or that was more than 90% sea (defined by LCM2007 and the UK Census mean high tide data) was excluded.

The ERAMMP NFS re-survey covered 130 of the original 300 squares in 2021-22, and an additional 95 survey squares in 2023. The 130 squares in 2021-22 were selected from both the Wider Wales and Glastir-targeted squares to enable the evaluation of multiple Glastir interventions based on multiple environmental outcomes; the selection of these squares does not enable a robust evaluation of national trends without more Wider Wales squares being surveyed (Alison et al. 2021). Therefore, the 95 survey squares in 2023 were the remaining Wider Wales squares that had not been surveyed. A total of 225 squares were re-surveyed in ERAMMP between 2021 and 2023: 147 Wider Wales squares and 78 Glastir-targeted squares (Figure 1). Not all surveyed squares contained surveyable headwater streams. Table 1 contains an overview of squares containing headwater streams which were surveyed between 2021-23.

Table 1: Overview of squares containing headwater streams surveyed between 2021 and 2023, divided by square type and survey year. The number of squares containing headwater streams for which erosion features were identified is given in brackets.

|  | 2021–22 | 2023 | Total |
| --- | --- | --- | --- |
| Wider Wales squares | 28 (27) | 38 (38) | 66 (65) |
| Glastir-targeted squares | 35 (34) | 0 (0) | 35 (34) |
| total | 63 (61) | 38 (38) | 101 (99) |

| GMEP (2013-2016) | ERAMMP (2021-2023) |
| --- | --- |

Figure 1: Maps showing the distribution of 1-km x 1-km National Field Survey squares across Wales (plotted at 10 km to ensure data confidentiality). ERAMMP map shows all NFS squares surveyed in 2021-2023; the colour key indicates survey years for the botanical survey, headwater streams were visited in the same years.

# Data Collection Methods

## Headwater stream erosion survey background

The headwater stream erosion survey covers the same 500 m reach as the River Habitat Survey (RHS) (Scarlett and Wood, 2023). The erosion survey was typically conducted in tandem with the RHS, however, for more geomorphologically diverse sites, or for surveyors who were less experienced in this type of survey, the erosion survey was advised to be carried out separately to the RHS. Surveys were completed as the surveyor moved along the stream, not as a sweep-up survey. Multiple erosion processes can be recorded along the 500 m reach.

## Survey method

Erosion was assessed for the ERAMMP National Field Survey 2021–2023 using the ESRI ArcGIS Online tool, Survey123 on a field tablet. The software is described in more detail in the Stream Erosion survey handbook (Scarlett and Wood, 2023).

When a distinct area of erosion was identified, its length was recorded on the left or the right bank. As with RHS, banks are defined as left or right looking downstream. Four process types are recorded at part of the survey (Fluvial, Biological, Artificial and Other) with associated processes:

| PROCESS_TYPE | Process (options) | PROCESS (recorded) |
| --- | --- | --- |
| FLUVIAL | Full bank scour | FULL |
|  | Basal bank scour | BASAL |
|  | Stable cliff | STABLE |
| BIOLOGICAL | Tree fall | TREE |
|  | Burrowing | BURROWING |
|  | Bed scour | BED |
| ARTIFICIAL | Footpaths | FOOTPATHS |
|  | Access | ACCESS |
|  | Poaching | POACHING |
|  | Below structure | BELOW |
| OTHER | Runoff (field) actual | RUNF_ACT |
|  | Runoff (field) potential | RUNF_POT |
|  | Runoff (other) actual | RUNO_ACT |
|  | Runoff (other) potential | RUNO_POT |
|  | Tributary | TRIBUTARY |
|  | Other | Free text |

The length (in metres) of the area of erosion was measured through pacing; surveyors calibrated their paces to metres as part of the RHS training. A ranging pole may be used for small units of measurement.

When the length of a distinct process was recorded, the centre of the feature was stamped using the Survey123 tool, storing the coordinates of the location using the tablets GPS. The working of the GPS may be impacted by tree cover or topography. Due to the resolution of the GPS unit (usually <>5m) and the small size of streams recorded in the ERAMMP survey, it was not necessary to differentiate between left and right bank locations of erosion types where they occur adjacently.

A photograph of the erosion was taken with the field tablet after measurements were made. When the area of erosion being recorded was too large for a photograph to capture the whole length in detail, photographs were taken to show an example of the process. If more erosion features were captured in one photo, no additional photo was taken.

# Data Quality Assurance

Surveyors ensured that they were familiar with identifying the different types of erosion and the type and sub-type they are recorded under before going into the field.

Headwater stream erosion data were downloaded in csv files, checked and all changes were documented in an QA-issue log, followed up with the lead surveyor, and resolved as documented.

Department for Environment, Food and Rural Affairs (DEFRA) Joint Codes of Practice (JCoPR) were followed throughout.  The JCoPR sets out standards for the quality of science and the quality of research processes. This helps ensure the aims and approaches of research are robust. It also gives confidence that processes and procedures used to gather and interpret the results of research are appropriate, rigorous, repeatable and auditable.

# Notes

- To assess the national trend, only data originating from the Wider Wales square data population should be analysed as these are nationally representative.

- To assess Glastir impacts, data from Wider Wales and Glastir targeted squares should be analysed together and in conjunction with associated data on Glastir management, as both sets of squares contain sites with Glastir management option uptake and no Glastir uptake. However, information on the locations of Glastir uptake is not included in this dataset and must be requested from Welsh Government (data@gov.wales).

# Data Structure

## File details:

Survey data recorded from headwater streams in 99 x 1km squares between 2021 and 2023 using standardised recording methods (Scarlett and Wood, 2023). Dataset consists of 1 .csv file:

- ERAMMP_STREAM_EROSION_2021_23.csv, table containing details of the type, presence and extent of erosion processes over a 500m reach of headwater streams within the ERAMMP squares that were visited as part of the headwater stream erosion survey in 2021 and 2022 for which data could be collected. This reach is identical to that used in the River Habitat Survey described in the Headwater stream quality metrics from the Environment and Rural Affairs Monitoring and Modelling Programme, Wales, 2021-2023 dataset.  In all cases, the left and right bank are defined when facing downstream. Each erosion feature was photographed, and the name of the associated photograph is provided. For associated field methods, please consult the supplementary material (Scarlett and Wood, 2023). The table contains 11 columns of 622 rows.

## Table columns:

ERAMMP_STREAM_EROSION_2021_23:

| Colum Name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Text | None | 1km survey square – unique identifier for headwater stream |
| PROCESS_TYPE | Text | None | Classification of observed erosion process type as one of FLUVIAL, BIOLOGICAL, ARTIFICIAL or OTHER. |
| PROCESS | Text | None | Observed erosion process |
| EXTENT_LEFT | Number | Metres | Bank length in metres affected by presence of erosion process, on the left hand bank. |
| EXTENT_RIGHT | Number | Metres | Bank length in metres affected by presence of erosion process, on the right hand bank. |
| TOTAL_EXTENT | Number | Metres | Total bank length in metres affected by presence of erosion process. |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |
| SURVEY_YEAR | Number | None | Year of survey |

# References

Alison, J., Bowgen, K., Siriwardena, G., Williams, B., Reinsch, S., Garbutt, A., Emmett, B.A. (2021) Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP). ERAMMP Report-58: ERAMMP Square Selection Protocol 2021/2022. Report to Welsh Government (Contract C210/2016/2017) (UK Centre for Ecology & Hydrology Projects 06297 & 06810):

https://erammp.wales/sites/default/files/2022-11/58%20ERAMMP%20Report-58%20Square58%20%20Selection%20Protocol%202021%20v1.0_en.pdf

Bunce, R. G. H., Barr, C. J., Clarke, R. T., Howard, D. C., Scott, W. A. (2007) ITE Land Classification of Great Britain 2007. NERC-Environmental Information Data Centre doi: 10.5285/5f0605e4-aa2a-48ab-b47c-bf5510823e8f

Carey, P. D.; Wallis, S.; Chamberlain, P. M.; Cooper, A.; Emmett, B. A.; Maskell, L. C.; McCann, T.; Murphy, J.; Norton, L. R.; Reynolds, B.; Scott, W. A.; Simpson, I. C.; Smart, S. M.; Ullyett, J. M. (2008). Countryside Survey: UK Results from 2007 NERC/Centre for Ecology & Hydrology (CEH Project Number: C03259): https://nora.nerc.ac.uk/id/eprint/5191/1/N005191CR%20UK%20Results.pdf

Emmett, B. A. et al. (2013). An integrated ecological, social and physical approach to monitoring environmental change and land management effects: the Wales Axis II Monitoring and Evaluation Programme. Aspects of applied biology, 31-39.

Emmett, B.E., Abdalla, M., Anthony, S., Astbury, S., August, T., Barrett, G., Biggs, J., Botham, M., Bradley, D., Brown, M., Carter, H., Chadwick, D., Cigna, F., Cooper, D., Cooper, J., Cosby, BJ., Creer, S Cross, P., Edwards, F., Edwards, M., Evans, C., Ewald, N., Fitton, A., Garbutt, A., Grebby, S., Greene, S., Halfpenney, I., Hall, J., Harrison, S., Harrower, C., Henrys, P., Hobson, R., Hughes, S., Isaac, N., Jackson, B., Jarvis, S., Jones, D., Keith, A., Kelly, M., Korenko, J., Lallias, D., Leaver, D., Lebron, I., Malcolm, H., Maskell, L., McDonald, J., Moxley, J., Norton, L., O’Hare, M., Owen, A., Pereira, M., Peyton, J., Powney, G., Pywell, R., Rawlins, B., Robinson, D.A., Rorke, S., Rowland, C.,Roy, D., Scarlett, P., Scholefield, P., Scott, A., Scott, L., Scott, R., Siriwardena, G., Smart, S., Smith, P., Swetnam, R., Taylor, R., Tordoff, G., Van Breda, J., Vincent, H., Wagner, M., Waters, E., Watkins, J.,White, J., Williams, B., Wood, C. and Wright, S. (2014) Glastir Monitoring & Evaluation Programme. First Year Annual Report to Welsh Government (Contract reference: C147/2010/11). NERC/Centre for Ecology & Hydrology (CEH Project: NEC04780), pp.442

Emmett, B.E., Abdalla, M., Anthony, S., Astbury, S., August, T., Barrett, G., Beckman, B., Biggs, J., Botham, M., Bradley, D., Brown, M., Burden, A., Carter, H., Chadwick, D., Cigna, F., Collier, R., Cooper, D., Cooper, J., Cosby, B.J., Creer, S., Cross, P., Dadam, D., Edwards, F., Edwards, M., Evans, C., Ewald, N., Fitton, A., Garbutt, A., Giampieri, C., Gooday, R., Grebby, S., Greene, S., Halfpenney, I., Hall, J., Harrison, S., Harrower, C., Henrys, P., Hobson, R., Hughes, P., Hughes, S., Illian, J., Isaac, N., Jackson, B., Jarvis, S., Jones, D.L., Jones, P., Keith, A., Kelly, M., Kneebone, N., Korenko, J., Lallias, D., Leaver, D., Lebron, I., Malcolm, H., Maskell, L., McDonald, J., Moxley, J., Norton, L., O’Hare, M., Oliver, T., Owen, A., Parkhill, K.A., Pereira, M., Peyton, J., Pogson, M., Powney, G., Pritchard, N., Pritchard, S., Prochorskaite, A., Prosser, M., Pywell, R., Rawlins, B., Reuland, O., Richards, M., Robinson, D.A., Rorke, S., Rowland, C., Roy, D., Scarlett, P., Scholefield, P., Scott, A, Scott, L., Scott, R., Sharps, K., Siriwardena, G., Smart, S., Smith, G., Smith, P., Stopps, J., Swetnam, R., Taft, H., Taylor, R., Tebbs, E., Thomas, A., Todd-Jones, C., Tordoff, G., Turner, G., Van Breda, J., Vincent, H., Wagner, M., Waters, E., Walker-Springett, K., Wallace, H., Watkins, J., Webb, G., White, J., Whitworth, E., Williams, B., Williams, P., Wood, C. and Wright, S. (2017) Glastir Monitoring & Evaluation Programme. Final Report to Welsh Government. Contract reference: C147/2010/11. NERC/Centre for Ecology & Hydrology (CEH Projects: NEC04780/NEC05371/NEC05782)

Robinson, D.A., Bentley, L., Jones, L., Feeney, C., Garbutt, A., Tandy, S., Lebron, I., Thomas, A., Reinsch, S., Norton, L., Maskell, L., Wood, C., Henrys, P., Jarvis, S., Smart, S., Keith, A., Seaton, F., Skates, J., Higgins, S., Giuffrè, G., Emmett, B. A. (2024). Five decades' experience of long-term soil monitoring, and key design principles, to assist the EU soil health mission. European Journal of Soil Science, 75(5), e13570. https://doi.org/10.1111/ejss.13570

Scarlett, P.M. & Wood, C.M.  (2023). Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP). ERAMMP Report-90: Field-Survey Handbook (Procedures) – Stream Erosion 2022. Report to Welsh Government (Contract C210/2016/2017) (UK Centre for Ecology & Hydrology Projects 06297 & 06810): https://erammp.wales/sites/default/files/2023-07/90%20ERAMMP%20Doc-90%20Stream%20Erosion%20Field%20Handbook.pdf

Wood, C. M., Bunce, R. G. H., Norton, L. R., Maskell, L.  C., Smart, S. M., Scott, W. A., Henrys, P. A., Howard, D. C., Wright, S. M., Brown, M. J., Scott, R. J., Stuart, R. C., & Watkins, J. W. (2018). Ecological landscape elements: Long-term monitoring in Great Britain, the Countryside Survey 1978-2007 and beyond. Earth System Science Data, 10(2), 745–763. https://doi.org/10.5194/essd-10-745-2018

Wood, C. M., Alison, J., Botham, M. S., Burden, A., Edwards, F., Garbutt, R. A., George, P. B. L., Henrys, P. A., Hobson, R., Jarvis, S., Keenan, P., Keith, A. M., Lebron, I., Maskell, L. C., Norton, L. R., Robinson, D. A., Seaton, F. M., Scarlett, P., Siriwardena, G. M., Skates, J., Smart, S. M., Williams, B., and Emmett, B. A. (2021) Integrated ecological monitoring in Wales: the Glastir Monitoring and Evaluation Programme field survey, Earth Syst. Sci. Data, 13, 4155–4173, https://doi.org/10.5194/essd-13-4155-2021.

ERAMMP Programme Office

UKCEH Bangor

Environment Centre Wales

Deiniol Road

Bangor, Gwynedd

LL57 2UW

+ 44 (0)1248 374500

erammp@ceh.ac.uk

www.erammp.cymru

www.erammp.wales