Meteorological and soil data from Whim Bog, Scotland, UK, 2024-2025

Leeson, S.R., Deshpande, A.G., Levy, P.E., Harvey, D. and Jones, M.R.

Data collection

The meteorological data is collected from a 2 m mast on an ombrotrophic bog. A peatland nitrogen pollution experiment was established in 2002 at Whim Bog (55.76566o, -3.27155o, altitude: 316 m) near Edinburgh (Leith et al. 2004, Sheppard et al. 2004, Deshpande et al. 2024) and meteorological measurements started in 2003. Air temperature, relative humidity, rainfall, leaf wetness, wind speed, wind direction, net radiation, total solar radiation and photosynthetically active radiation (PAR) were measured over the bog. Soil volumetric water content is measured at one depth and temperature were measured at two depths in addition to water table depth. The instruments measure data every 20 seconds and the average is calculated over 15-minute intervals. All data is given in GMT (Greenwich Mean Time) from 1st January 2024 to 31st December 2025.

Quality Control

The data has been checked visually and any obvious errors introduced by instrument malfunctioning, datalogger programming issues and power cuts have been removed. Some gaps were also introduced during the time taken to replace faulty sensors.

Data structure

The dataset contains 736,321 measurements of 13 parameters. Data are provided in a CSV file with columns as below.

| Parameter | Unit | Height (m) | Manufacturer | Instrument | Description |
| --- | --- | --- | --- | --- | --- |
| Timestamp | - | - | - | - | 15-minute timestamps in the format dd/mm/yyyy hh:mm. |
| Rain | mm | 0.4 | Didcot Instruments | Tipping bucket | Total rainfall. |
| LWS | mV | 0.2 | Campbell scientific | 237-L | Leaf wetness. |
| AirT | ˚C | 1.5 | Vasiala | HMP60 | Air temperature. |
| RH | % | 1.5 | Vasiala | HMP60 | Relative humidity. |
| PAR | µmol m-2 s-1 | 1.5 | Skye Instruments | SKP215 | Photosynthetically active radiation. |
| Total_solar | W m-2 | 1.5 | Skye Instruments | SP1110 | Total solar radiation. |
| Net_rad | W m-2 | 1.0 | Kipp & Zonen | NRLite | Net radiation. |
| WS | m s-1 | 2.0 | Gill Instruments | Windsonic | Wind speed. |
| WD | ˚ | 2.0 | Gill Instruments | Windsonic | Wind direction. |
| Soil_VWC | % | -0.20 | Campbell scientific | CS616 | Soil volumetric water content near the surface (20 cm). |
| Soil_T1 | ˚C | -0.10 | Campbell scientific | 105E | Soil electrical conductivity near the surface (10 cm). |
| Soil_T2 | ˚C | -0.20 | Campbell scientific | 105E | Soil electrical conductivity at 20cm depth. |
| WTD | cm | -0.70 | Druck | PDCR1830 | Water table level measured at 70 cm depth. |

Funding source

The funding for establishing and operating this facility was provided by the UKCEH National Capability for UK Challenges Programme NE/Y006208/1.

References

Leith ID, Sheppard LJ, Fowler D, Cape JN, Jones M, Crossley A, et al. Quantifying dry NH3 deposition to an ombrotrophic bog from an automated NH3 field release system. Water, Air, & Soil Pollution: Focus. 2004;4(6):207–18.

Sheppard LJ, Crossley A, Leith ID, Hargreaves KJ, Carfrae JA, van Dijk N, et al. An automated wet deposition system to compare the effects of reduced and oxidised N on ombrotrophic bog species: practical considerations. Water, Air, & Soil Pollution: Focus. 2004 Dec;4(6):197–205.

Deshpande AG, Jones MR, Leeson SR, Harvey D, van Dijk N, Grenier M, Duarte F, Stephens A, Iwanicka A, Sheppard L.J, Sutton MA, Leith ID, Cape JN, Levy P. Ammonia concentration and deposition data from a peatland nitrogen pollution experiment, Whim Bog, Scotland, UK, 2002-2022. NERC EDS Environmental Information Data Centre. 2024. https://doi.org/10.5285/0cfcde23-753d-4bf0-81c6-13aea33b2b0f