Seasonal denitrification rates at Warton Bank & Old Hall salt marshes. Sep2024 - Jan2025

Author(s): Michael P. Perring*, UK Centre for Ecology and Hydrology (UKCEH); Dan Aberg*, Bangor University (BU); Paula Maria de la Barra UKCEH; Sophie Marshall-Potter, BU; Tree Oswald, BU; Lucy McMahon, Manchester Metropolitan University (MMU); Hannah Mossman, MMU; Joanna Harley, UKCEH; John Spill, UKCEH; Simon Oakley, UKCEH; Victor Ebuele, UKCEH; Inma Lebron, UKCEH; Susan Tandy, UKCEH; Annette Burden, UKCEH; Christian Dunn, BU; Angus Garbutt, UKCEH.

*: Joint first authors

## Summary

A key ecosystem service in coastal systems is remediation of nutrient pollution through sediment burial, vegetative uptake and microbial processing. Denitrification is a facultative anaerobic process where microbial activity transforms nitrate (NO3-), which in high concentrations can be environmentally harmful, into the environmentally benign dinitrogen gas (N2). Denitrification’s magnitude is considered particularly important in saltmarsh systems compared to other habitats, although an intermediate product, nitrous oxide (N2O), can also be given off and contribute to climate change.

Here, we improve the evidence base on denitrification, and potential relationships with purported drivers, through a seasonal study of denitrification rates in saltmarsh. Using a combination of classic vegetation survey techniques, core extraction, and subsequent laboratory processing, we aim to quantify relative variation in denitrification rates across environmental contexts.

In two saltmarshes, Old Hall in the Blackwater (southeast England) and Warton Bank in the Ribble (northwest England), we sampled every six to eight weeks from August/September 2024 to January 2025 to characterise seasonal denitrification and vegetation dynamics.

This data accompanies the technical report “Denitrification dynamics and relationships with potential drivers across English saltmarshes, seagrass beds, and mudflats: Capturing national variation in space and time”

Contents

Summary	1

Files included	2

Spatial and temporal coverage	3

Sampling regime and experimental design	3

Saltmarsh Vegetation Community	3

Saltmarsh Denitrification and Sediment Characterisation: Sample Collection for Analysis	4

Saltmarsh Porewater and Estuarine Water Tidal Sample Collection	4

Laboratory Analyses	4

Denitrification Dynamics: The Wetland Hydroperiod Simulator Approach	4

Denitrification Dynamics: Gas Analysis	6

Denitrification Dynamics: Calculation of Potential Denitrification Rates	6

Porewater and Seawater Samples	7

Organic Matter content and Bulk Density	7

## Files included

Eight data files and eight accompanying meta-data files are included. The meta-data files include the variable descriptions, units and other details.

| File names | Description |
| --- | --- |
| cover_diversity.csv | Derived measures from species_composition.csv. Including percentage cover of vegetation, litter and bare ground. Shannon diversity index for each quadrat |
| cover_diversity_meta.csv | Meta-data for cover_diversity.csv |
| denitrification.csv | Denitrification rate measurements and associated data |
| denitrification_meta.csv | Meta-data for denitrification.csv |
| loi.csv | Loss-on-ignition and bulk density measures |
| loi_meta.csv | Meta-data for loi.csv |
| plant_height.csv | Plant height. Measured from those species with cover >= 15 % per quadrat. Five vegetative shoots per species above cover threshold |
| plant_height_meta.csv | Meta-data for plant_height.csv |
| porewater.csv | Nutrient contents in soil porewater. Only measured in quadrats from 1 to 4 in each season, marsh and vegetation zone. |
| porewater_meta.csv | Meta-data for porewater.csv |
| quadrat.csv | Information from 6 quadrats taken per zone, saltmarsh and sampling event. |
| quadrat_meta.csv | Meta-data for quadrat.csv |
| seawater.csv | Nutrient contents in seawater. From one sample of seawater per saltmarsh and season. |
| seawater_meta.csv | Meta-data for seawater.csv |
| species_composition.csv | Percentage ground cover by plant genus |
| species_composition_meta.csv | Meta-data for species_composition.csv |

## Spatial and temporal coverage

The samples were taken from two salt marshes: Old Hall (0.839331° E, 51.772728° N; 0.842285° E, 51.773969° N) and Warton Bank (-2.937294° O, 53.730944° N; -2.917497° O, 53.737175°N).

The data includes four sampling events at each site between 29th August 2024 and 28th January 2025.

| Sampling Event (Label in figures) | Warton Bank, Ribble Estuary | Old Hall, Blackwater Estuary |
| --- | --- | --- |
| Seasonal Sample #1 (“Summer”) | 29th and 30th August 2024 | 3rd – 5th September 2024 |
| Seasonal Sample #2 (“Fall”) | 15th and 16th October 2024 | 22nd and 23rd October 2024 |
| Seasonal Sample #3 (“Early winter”) | 25th and 26th November 2024 | 10th December 2024 |
| Seasonal Sample #4 (“Late winter”) | 7th and 8th January 2025 | 28th January 2025 |

## Sampling regime and experimental design

Two salt marshes were sampled four times between 29th August 2024 and 28th January 2025. Six quadrats of 1 m2 were sampled at the low, high and middle vegetation zones during each visit. Notice that some visits needed to be spread out in consecutive days, but they still belong to the same sampling event. Vegetation zones were identified by observed dominant vegetation in an area: ‘pioneer/low’ (e.g. Spartina- or Salicornia-dominated), ‘middle’ (e.g. mixed communities of Atriplex, Armeria, Plantago, Limonium, and Puccinellia), and ‘high’ (e.g. Elymus-dominated) saltmarsh plant communities.

### Saltmarsh Vegetation Community

Thus, in each saltmarsh, we sampled three distinct elevation-related vegetation zones, aiming for pioneer to low, low-mid and upper saltmarsh communities to the extent that was practicable.

In each saltmarsh vegetation zone identified in the field, we surveyed six 1 x 1 m quadrats at random, but avoiding, to the extent possible, creek lines, nesting or roosting birds, feeding bird assemblages and saltpans. Through random sampling, taking account of these constraints, we tried to ensure unbiased characterisation of the vegetation and denitrification rates while capturing a representative sample of the vegetation community within a given zone.

Once the quadrat had been placed we recorded percentage cover of bare ground, litter, and live vegetation. In general, vegetation was identified to genus level although clearly identifiable species which we expected to play different functional roles were recorded separately e.g. the herbaceous Atriplex prostrata and woody Atriplex portulacoides. For those species with greater than or equal to 15 % cover, height was also recorded, on five randomly selected individuals per species. For current purposes, taking height and cover together could give an indication of biomass.

### Saltmarsh Denitrification and Sediment Characterisation: Sample Collection for Analysis

Within the 1 x 1m quadrats, we chose 2 locations of approximately the same aboveground plant community composition and biomass (estimated by eye) for saltmarsh denitrification core collection.

Two black plastic cores of 68 mm internal diameter, and 22 cm depth were hammered into the sediment, having removed any interfering vegetation first, leaving approximately 2 cm of the core showing above ground. Cores were then extracted with a post-hole spade, capped at both ends in black plastic, appropriately labelled and kept as cool as practicable (but not frozen) until transfer to Bangor University for laboratory analysis. In the field, they were kept cool in insulated plastic boxes, with ice packs subsequently added at the end of each day in the field, exchanged daily for fresh ice packs for the duration of sampling and transfer to Bangor University laboratory. Prior to analysis, cores were kept in a cool room at 3°C.

We used sediment extracts from these cores, after the denitrification assays described in the Laboratory analyses section, for characterisation of organic matter content and bulk density measures.

### Saltmarsh Porewater and Estuarine Water Tidal Sample Collection

In 4 of the 6 quadrats, we extracted porewater samples from one excavated core hole. We used Rhizon™ samplers of approximately 5 cm length, inserted at three depths – approximately 5, 10 and 15 cm. Each Rhizon™ was attached to a syringe to ensure a vacuum and draw for the porewater. Having waited a reasonable amount of time, given tidal and fieldwork constraints, but not less than 20 minutes, we collected individual samples into a Falcon tube, with the aim of having at least 2 ml of porewater solution per depth. We had difficulty extracting porewater samples from a number of locations; we take this into account in our analysis approach by integrating across depths to provide core-specific concentrations. Where porewater samples were collected, they were kept cool until such time they could be analysed at Bangor University laboratory i.e. in the same manner as for denitrification cores described above except that at the end of each sampling day they were placed in a refrigerator until transfer to the Bangor University laboratory.

In addition to the porewater, we also collected a tide water sample. These were sourced from a convenient tidal creek / main estuary channel location, approximately 1 to 2 hours either side of high water. These samples were used to characterise the nutrient environment of the saltmarsh, and we used this information when running denitrification samples through the Wetland Hydroperiod Simulator (see Laboratory analyses section).

## Laboratory Analyses

### Denitrification Dynamics: The Wetland Hydroperiod Simulator Approach

Denitrification rates from the whole core samples were analysed using the custom-built Wetland Hydroperiod Simulator (WHS; Figure 1). The WHS consists of a chamber linked to a water reservoir via a system of pipework. The water reservoir is fitted to a raising platform which can alter the level of water in the cores. This level is controlled by the WHS, using Raspberry Pi computing, coded to simulate the water level changes of a 24-hour neap tide.

Figure 1: The components of Bangor University’s Wetland Hydroperiod Simulator (WHS).

To calculate actual denitrification for one sample, two cores were taken per quadrat, one to undergo acetylene (C2H2) inhibition and one to act as a control.

Cores were placed in the chambers with a set volume of water (800 ml in a chamber and 1 L in a reservoir), calculated based on the depth of cores and tidal depth required. The nutrient (phosphate, NO3-, NO2-, and ammonium (NH4+)) and ion (chloride (Cl-), sodium (Na2+), potassium (K+), bromide (Br-), magnesium (Mg3+) and calcium (Ca2+)) composition of the water was artificially created, with amounts of nutrients and ions informed by the tidal samples collected for the coastal habitat cores being tested in any given run. In general, flood samples were used to inform conditions in the WHS, as we considered these to supply substrate for denitrification.

Chambers were sealed and acetylene chamber atmospheres were then altered to a 0.1 N atmosphere of acetylene. After the chambers were connected to the reservoirs, via the pipework, the water reservoirs for the acetylene chambers were also spiked to a 0.1 N acetylene atmosphere. The water in both the chambers and reservoirs was flushed with acetylene once filled. Once all chambers were connected, a 10 mL gas sample was taken and transferred to a 5 mL gas-tight glass container, this was referred to as a Time Zero sample (T0). The WHS then ran a full 24-hour tidal cycle. At the end of this cycle, a second gas sample was taken (T24). Temperature was recorded each time a gas sample was taken. Further, the headspace was manually pumped with a syringe prior to taking a gas sample, to avoid underestimating denitrification rates through gas being trapped in soil cores and to ensure that the air was homogenised to avoid density separation. In general, the climate conditions in the laboratory were kept as constant as possible from denitrification run to denitrification run. Given that, we can attribute differences in denitrification rate associated with the cores to their characteristics, including the vegetation community from which they were extracted, and the nutrient relationships of the estuarine water.

### Denitrification Dynamics: Gas Analysis

Gas samples collected were analysed by gas chromatography using a Varian model 450 gas chromatograph (GC) instrument, equipped with an electron capture detector (ECD) for N2O. Two mL of gas from the gas-tight glass containers (Exetainers®) containing the samples was injected via a 1041 on-column injector system, set at 40⁰C, onto a PoroPak QS (1.83 m x 3.18 mm) 80/100 column. The septum of this system was changed after approximately 500 injections. The column oven temperature was set to 40⁰C and the carrier gas, oxygen-free nitrogen, had a flow rate of 30 mL min-1. The temperature of the ECD was 340⁰C with a constant flow of 20 mL min-1 of oxygen-free nitrogen. Injection of the samples was achieved with a Combi PAL headspace auto-sampler (CTC Analytics, Zwingen, Switzerland) equipped with a 5 mL syringe and specially constructed trays for holding 50 individual 5.9 mL Exetainers®. N2O (retention time 3.26 minutes) was quantified by comparison of peak area with that of four standards of known concentration (0.3, 1.5, 5 and 40 ppm), prepared by BOC (an industrial gases company) and used in the preparation of a standard curve, which — according to standard laboratory protocol — was only accepted if the correlation coefficient (R2) value was greater than 0.98; indicating the strongest relationship between the variables.

### Denitrification Dynamics: Calculation of Potential Denitrification Rates

We used the following set of equations to calculate denitrification rate:

DR mg m-2s-1=TNPδt×(V×MS×Vmol)					Equation [1]

where:

- DR: Denitrification rate (mg N2O m-2 s-1)

- TNP: Total N2O produced (mg)

- δt: Change in time between first and second measurement (seconds (s))

- V: Volume of headspace in chamber (m3)

- M: Molecular weight of gas (mol)

- S: Area of core (m2)

- Vmol: Volume of a mol of gas at a given temperature (m3 mol-1)

where Vmol is given by:

Vmol=p×(R×K)							Equation [2]

- p: Pressure (kPa)

- R: Equal to ideal gas constant (8.314)

- K: Temperature (Kelvin)

where Total N2O produced (TNP) is given by:

TNP=(Ksp×m×W:Hs)+m 					Equation [3]

- Ksp: N2O solubility

- m: Mass of N2O in headspace (mg)

- W:Hs: Ratio of water to headspace

and, where mass of N2O in headspace (m) is given by:

m=Hs×NA×(AδC-CδC) 					Equation [4]

- m: Mass of N2O in headspace (mg)

- Hs: Headspace volume (ml)

- NA: Avogadro constant (6.022×10²³ mol⁻¹)

- AδC: Change in the gas concentration in acetylene samples: T0-T24

- CδC: Change in the gas concentration in control samples: T0-T24

### Porewater and Seawater Samples

Seawater and porewater samples were analysed using colorimetric based methods. Nitrate (NO3-) was measured using a Vanadium reduction followed by a Griess reaction. Ammonium (NH4+) was measured using a buffered indophenol method. Phosphate (PO43-) was measured using the molybdenum blue method.

Other seawater ion concentrations (Cl-, Na2+, K+, Br-, Mg3+ and Ca2+) were measured using a Metrohm 850 Professional IC. For anions, 20 μl of sample was injected onto a Metrosep A Supp 5 – 150/4.0 column using an 8mM Na2CO3, 0.2mM NaHCO3 eluent. For cations, 20 μl of sample was injected onto a Metrosep C 4 – 250/4.0 column using a 5mM HNO3/1mM Oxalic acid eluent. Both columns were fitted with guard columns (A Supp 19 guard 4.0 and C6 Guard 4.0). Seawater samples were diluted x50 to ensure ion levels were within the top standards. Ion concentrations were then used to generate seawater composition for each WHS run.

### Organic Matter content and Bulk Density

The Loss on Ignition (LoI) method was employed to determine the percentage of moisture and organic matter content in the sediment. Cleaned and labelled crucibles were weighed to four decimal places before being filled with homogenized sediment. The samples were dried in an oven at 105°C for 24 hours to remove water content (or until a constant weight was achieved), then cooled in a desiccator and reweighed to determine moisture loss. The dried samples were subsequently combusted in a muffle furnace at 550°C for 200 minutes to remove organic material (following initial trials to establish necessary timing). After cooling, the crucibles were reweighed to measure the mass of the remaining inorganic fraction. Moisture and organic content percentages were calculated using the following formulas:

Equations [5] and [6]

To determine soil bulk density, a 1 cm-thick disk was extracted from a 10 cm depth of the Control core. The soil sample was placed into a pre-weighed foil dish and dried in an oven at 105 °C for 48 hours, or until a constant weight was achieved, to remove all moisture. After drying, the sample was weighed again. Bulk density (g/cm³) was calculated by dividing the oven-dry mass of the soil by the sample's volume, which was determined from the known dimensions of the core sampler.