# Working Title

# R implementation of the Load Apportionment Model (RLAM) determining relative quantities of phosphorus entering rivers

# Summary

The R implementation of the Load Apportionment Model (RLAM) implements the Load Apportionment Model (LAM; Bowes et al., 2008), a novel and rapid method for determining relative quantities of phosphorus entering rivers from sewage treatment works and diffuse sources. A working Excel implementation was released, which is widely used routinely by a variety of organisations. However, the Excel-based application has limited functionality (e.g. manual input of data, site calibration partly automated only, no batch processing), is not very user-friendly, and offers limited protection against users modifying the model or misusing it. From a Quality Assurance perspective, it was therefore thought beneficial to create a version of LAM designed by the developers. It was decided to implement the model in the R language (as a suite of R scripts, which can be re-useable by specialists, e.g. for batch processing) linked to a user-friendly web-based front-end (R Shiny app) targeting non-specialists.

# Models

LAM (Bowes et al., 2008) is a novel and rapid method for determining relative quantities of phosphorus, either as total phosphorus (TP) or as soluble reactive phosphorus (SRP), entering rivers from continuous sources (largely from sewage treatment works’ effluent in the UK) and rain-related inputs from agricultural diffuse sources, septic tank soak-aways and remobilization of phosphorus stored within the river channel.

There are two ways phosphorus can enter the riverine system: via constant sources (CS) or via rain-related sources (RRS). The difference in timing of CS and RRS phosphorus delivery to a river produces clear differences in the relationship between [P] and flow. CS loads decrease in concentration with increasing river flow, due to dilution, whereas RRS loads increase with river flow. LAM is a simple model based on this fundamental difference in this concentration / flow relationship, which quantifies the respective contribution of CS and RRS to river phosphorus concentration, expressed as a power law function of flow.

The base equation is:

Equation 1: [P] = A.QB-1 + C.QD-1

The first term in Eq. 1 (A.QB-1) is interpreted as that part of the P concentration originating from CS, while the second term (C.QD-1) is that part originating from RRS.

In follow-up research, Bowes et al. (2014) showed that coefficient B should be constrained to zero, thus giving:

Equation 2: [P] = A.Q-1 + C.QD-1

It is this modified Equation 2, which is used in the RLAM Shiny app. Required inputs are simply a dataset of Q and [P] (as TP or SRP). The three coefficients A, C and are obtained from standard model fitting techniques. From this point, Eq. 2 is known and phosphorus concentrations and loads from both CS and RRS can be derived.

# File format

R, CSV, JPEG

# Files and folder structure

The following files and sub-folders should be placed in the main RLAM folder:

| Folder | Contents |
| --- | --- |
| Top level folder to where data is saved | RLAM_MAIN.R (RLAM launch script) RLAM_Functions.R (script with code for RLAM functions; called by RLAM_MAIN during launch) server.R (Shiny server script) ui.R (Shiny user interface script) |
| www | Sub-folder containing the UKCEH logo displayed in the RLAM user interface: UKCEH-Logo_Long_Positive_RGB.JPG |
| INPUT | Sub-folder containing the input files. Four files are provided as examples: Cut_Paley_Street.csv, Evenlode_Cassington_Mill.csv, Kennet_Woolhampton.csv, Thames_Sonning.csv. Users should copy their own input files here prior to start the app. |

# Input data

There should be one input file per site that is being modelled. They should be .csv files. No naming convention is enforced within the app but we would recommend a simple and explicit name like used for the example files (e.g. [river name]_{site name]). The app will use the file names as the site names (minus the .csv extension).

Input files include these columns:

- Date formatted as dd/mm/yyyy

- Q discharge in m3.s-1

- TP total phosphorus concentration in μg.l-1

- SRP suspended reactive phosphorus concentration in μg.l-1

Columns Date and Q are mandatory. Input files can have either TP, SRP, or both. Below is an example of input data:

| Date | Q | SRP | TP |
| --- | --- | --- | --- |
| 02/03/2009 | 0.256 | 315 | 450 |
| 10/03/2009 | 0.606 | 157 | 288 |
| 18/03/2009 | 0.236 | 333 | 464 |
| 23/03/2009 | 0.192 | 382 | 504 |
| … | … | … | … |

(Note: columns and rows do not have to be ordered as shown).

Users should ensure their data are of good quality. However, the RLAM app includes some limited automated checks. On starting, the code scans all files in the INPUT sub-folder. If a file is missing either Date, Q, or both TP and SRP (i.e. column missing altogether or header present but empty column), it is discarded and will not be available within the app. See section Running the app for more details.

# System requirements and installation

RLAM comprises a series of R scripts and uses the R shiny and ggplot2 packages. It was developed with R version 4.4.0 and further tested with version 4.4.1, and shiny version 1.9.1 on the Windows operating system. At a minimum RLAM requires the installation of R but can also run under R Studio, a more user-friendly environment than the standard R, which sits on top of R. Choice of R or R Studio is a matter of user preference and computer specifications. The standard R software can be downloaded from https://cran.r-project.org/ whilst the R Studio environment can be downloaded from https://posit.co (R should be installed before R Studio). Users should follow any installation instructions provided with this software. With R (or R Studio) installed, users should install the R shiny and ggplot2 packages. Shiny allows R developers to create user-friendly applications with a web-page front end running R code in the background. (for more details on Shiny, see the CRAN repository page at https://cran.r-project.org/web/packages/shiny/index.html and R Studio page at http://shiny.rstudio.com/). The ggplot2 package is used to generate the time series plot within the RLAM app.

# User Interface

Upon running the RLAM_MAIN.R script from R (or R Studio), a web page is automatically opened, which features the following components (white number in blue circle on screenshot):

1. Site selection drop-down menu (single selection)

2. Predicted variable selection drop-down menu (single selection)

3. Slider to select data subset range; current selection shown as blue line between grey circles; users can move either circle or the blue bar itself (moving both circles at the same time); in this example, the full dataset has 110 data-points and the selected subset includes data-points 41 to 92

4. Export button (plots and model results)

5. Plot of the discharge time series; the vertical red dash lines show the selected end data-points from the data subset selected using slider (see 3); in this example, the selected subset includes data-points 41 to 92

6. ‘Whole Dataset’ block; featuring model results using the whole input dataset (in this example, 110 data-points)

7. ‘Whole Dataset Autofit Coeffs’ sub-block; featuring automatically fitted model results: coefficients A, C, and D (first row), QCP and %ObsCSDom metrics (second row)

8. ‘Whole Dataset Autofit Coeffs’ sub-block; plot of modelled vs observed P variable (as selected in drop-down menu 2)

9. ‘Whole Dataset Autofit Coeffs’ sub-block; plot of P split between constant sources and rain-related sources

10. ‘Whole Dataset Custom Coeffs’ sub-block; users can enter a custom coefficient value in the input box and refit the model with that coefficient fixed (only one coefficient at a time); featuring similar outputs as items 7, 8 and 9 but based on user custom coefficients

11. ‘Data Subset’ block; featuring similar outputs as ‘Whole Dataset’ block but using the user-selected data subset (see item 4)

# Running the app

1)	Users should copy their input files in the INPUT sub-folder.

2)	In R or RStudio, run RLAM_MAIN.R script.

3)	The code scans the INPUT sub-folder to build a list of input files. First, only CSV files are retained. The code checks and retain only files having columns Date, Q, TP and/or SRP. The code then checks if any of these columns is empty, discarding files with one or more empty columns. The final list is used to populate the Site selection drop-down menu (item 1 on screenshot above).

4) On starting the app, the first file in the INPUT folder (alphabetical order) is selected by default. If only column TP or SRP is present in the input file, the app will only offer that option in the predicted variable drop-down menu (item 2 on screenshot); if both are present, both will be available to chose, with the first in alphabetical order selected by default (i.e. SRP).

5) Data from the selected file are loaded. Rows with missing or negative Q missing or negative are removed. Rows with negative TP or SRP are removed.

6) On starting, on changing the site selection (in drop-down menu 1) or changing the predicted variable (drop-down menu 2), the four models are fitted with the full dataset, and the Custom Coeffs are set as the Autofit Coeffs (i.e. all displays are the same).

7) Fitting models with a data subset (Data Subset block; item 11). Using the slider (item 3), users can select a data subset. The slider displays the selected data-point rank numbers, while on the hydrograph (item 5), matching vertical lines show the selection. The displays for the Data Subset block (item 11), i.e. the Autofit Coeffs and Custom Coeffs models are automatically re-set to the autofit coefficients for the new subset selection.

8) Fitting custom models. Both the Whole Dataset and Data Subset blocks provide an option to customise model coefficients in their respective Custom Coeffs sub-blocks. Coefficients are linked together so they cannot be changed independently. Instead, users enter one custom coefficient at a time in one of the three fields (A, C, D), then must re-fit the model by clicking on the corresponding button (e.g. enter custom coefficient in field A then click on Refit w fixed A button). Internally, this re-fits the model with the custom coefficient fixed and only the other two coefficients allowed to vary. However, when entering a custom coefficient, the plots are updated instantly; this permits to give a rough indication of the shape and positioning of the new modelled curves before hitting the re-fit button.

9) Exporting plots and model results. On clicking the Export Plots & Model Coefficients button (item 4), an OUTPUT sub-folder is automatically created if not already present at the same level as the INPUT sub-folder. The app automatically saves PDFs and CSVs with the following naming convention:

- Whole dataset and autofit coefficients models:

RLAM_REPORT_AUTOFIT_FULLDATA_[site name]_[variable name].pdf

RLAM_SPREADSHEET_AUTOFIT_FULLDATA_[site name]_[variable name].csv

- Whole dataset and custom coefficients:

RLAM_REPORT_CUSTOMFIT_FULLDATA_[site name]_[variable name].pdf

RLAM_SPREADSHEET_CUSTOMFIT_FULLDATA_[site name]_[variable name].csv

- Data subset and autofit coefficients:

RLAM_REPORT_AUTOFIT_SUBSET_[start data-point]_TO_[end data-point]_[site name]_[variable name].pdf (e.g. “RLAM_REPORT_AUTOFIT_SUBSET_37_TO_86_Cut_TP .pdf”)

RLAM_SPREADSHEET_AUTOFIT_SUBSET_[start data-point]_TO_[end data-point]_[site name]_[variable name].csv (e.g. “RLAM_SPREADSHEET_AUTOFIT_SUBSET_37_TO_86_Cut_TP.csv”)

- Data subset and customs coefficients:

RLAM_REPORT_CUSTOMFIT_SUBSET_[start data-point]_TO_[end data-point]_[site name]_[variable name].pdf

RLAM_SPREADSHEET_CUSTOMFIT_SUBSET_[start data-point]_TO_[end data-point]_[site name]_[variable name].csv

The code works out which files need to be exported. For example, if no custom model has been re-fitted, or no data subset has been used, only the “AUTOFIT_FULLDATA” PDF and CSV are saved. Files with custom coefficients are overwritten.

Each RLAM_REPORT_[..].pdf file includes a summary page with: selected site name; selected predicted variable (e.g. TP or SRP); type of model fitting (automated or custom coefficients); number of  observations used, start and end data-points; coefficients A, C, and D; Q Crossover−Point (QCP); percentage of time observations dominated by constant source. It also includes the hydrograph (item 5) and both model plots (items 8 and 9).

Each RLAM_SPREADSHEET_[…].csv include columns:

- Date formatted as dd/mm/yyyy

- Q discharge in m3.s-1

- MODELLED_CONCENTRATION_CS (concentration of TP/ SRP from constants sources) in μg.l-1

- MODELLED_CONCENTRATION_RRS (concentration of TP/ SRP from rain-related sources) in μg.l-1

- MODELLED_LOADS_CS in mg.s-1

- MODELLED_LOADS_RRS in mg.s-1

- MODELLED_LOADS_PER_DAY_CS in t.d-1 (tonne per day)

- MODELLED_LOADS_PER_DAY_RRS in t.d-1 (tonne per day)

# Miscellaneous

This program is distributed in the hope that it will be useful, but WITHOUT ANY WARRANTY; without even the implied warranty of MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.

# References

Bowes M.J., Helen P. Jarvie H.O., Naden P.S., Old G.H., Scarlett P.M., Roberts C., Armstrong L.K., Harman S.A., Wickham H.D., Collins A.L. (2014). Identifying priorities for nutrient mitigation using river concentration–flow relationships: The Thames basin, UK. Journal of Hydrology (517), 1-12. http://dx.doi.org/10.1016/j.jhydrol.2014.03.063

Bowes, M., Smith, J., Jarvie, H., & Neal, C. (2008). Modelling of phosphorus inputs to rivers from diffuse and point sources. Science of the Total Environment, 395(2-3), 125-138. https://doi.org/10.1016/j.scitotenv.2008.01.054