# Working Title

European Monitoring and Evaluation Program Model for the UK (EMEP4UK) annual air pollutant concentrations from domestic solid fuel burning in Scotland (2023)

# Summary

This dataset provides modelled annual average near-ground atmospheric composition for Scotland for the key air pollutants nitrogen dioxide, total nitrogen oxides, sulphur dioxide, and fine particulate matter for the year 2023. As well as total concentrations, the dataset includes concentration contributions from domestic solid fuel burning. The dataset was created with the European Monitoring and Evaluation Programme (EMEP) model version 5.5 and the WRF model version 4.4.2. It was generated under a Scottish Government project which also calculated emissions from domestic solid fuel burning in Scotland (Nemitz et al., 2026) and carried out measurements targeting the air pollution impacts of solid fuel burning in Scotland (Di Marco et al., 2026). Further details of the project are available in the final report, Nemitz et al. (2025).

## Model

The EMEP4UK rv5.5 model (Simpson et al., 2012; Vieno et al., 2016a, 2016b) is an atmospheric chemistry transport model (ACTM), derived from the EMEP MSC-W model rv5.5 (https://github.com/metno/emep-ctm), which simulates hourly to annual average atmospheric composition and deposition of various pollutants. The 3D meteorology is calculated using the Weather Research and Forecasting Model version 4.4.2 (WRF: https://www.mmm.ucar.edu/models/wrf).

The EMEP4UK and WRF model domain covers the EU with a resolution of 27 × 27 km2 with a nested domain covering Scotland with a resolution of 1 x 1 km2. The WRF model also uses intermediate domains at 9 x 9 km2 and 3 x 3 km2 resolutions. Only the Scottish model domain results are included here.

The WRF model (Skamarock et al., 2019) included data assimilation (Newtonian nudging) of the fifth-generation European Centre for Medium-Range Weather Forecasts (ECMWF) atmospheric reanalysis of the global climate (ERA5) at 0.25°x0.25° degrees resolution, every 6 hours (http://cds.climate.copernicus.eu/ ).

## Input data

Emissions from domestic combustion of solid fuel in Scotland were calculated by UKCEH based on new fuel use and emission factor data (Nemitz et al., 2026).

Anthropogenic emissions of NOx, NH3, SO2, primary fine particulates (PM2.5), primary coarse particulates (PMcoarse), CO and non-methane VOC for all other activity sectors in the UK are derived from the National Atmospheric Emission Inventory (NAEI, http://naei.defra.gov.uk) data for 2022, as data for 2023 were not available when the modelling was carried out. The EMEP emission estimates at a resolution of 0.1° × 0.1° provided by the Centre for Emission Inventories and Projections (CEIP, http://www.ceip.at/) were used for all non-UK emissions.

## Output data

Annual averaged EMEP4UK model data are included here. The base file contains total concentrations, whereas the scenario files contain the concentration contributions from specific domestic solid fuel types within Scotland or from all domestic solid fuel use in the UK outside Scotland. The contributions were calculated by subtracting output concentrations from a scenario run (using emissions excluding the relevant domestic solid fuel type) from the base concentrations (including all emissions).

The scenarios included are:

- nonScot_allSF – contribution from all UK domestic solid fuel burning outside Scotland

- Scot_wood – contribution from domestic wood burning in Scotland, including coffee logs

- Scot_coal – contribution from domestic coal burning in Scotland, including house coal, petroleum coke, anthracite, lignite and peat

- Scot_msf – contribution from domestic ‘manufactured solid fuel’ (MSF) burning in Scotland, including smokeless fuels

Post-processing was applied to the raw model output files to:

- Calculate annual averages by weighted average of monthly model outputs;

- Calculate NOx (total nitrogen oxides) from NO and NO2;

- Reduce the number of concentration variables; and

- Simplify file names.

## Model compiler

The WRF and EMEP4UK model FORTRAN codes were compiled with the gfortran compiler 11.5.0 20240719 (Red Hat 11.5.0-2). The calculations have been carried out using the UKCEH computer cluster POLAR (Rocky Linux version 9.6 (Blue Onyx)).

## Nature and Units of recorded values

Fine particulate matter has an aerodynamic diameter less than 2.5 µm.

Coarse particulate matter has an aerodynamic diameter between 2.5 µm and 10 µm.

Primary PM2.5 (SURF_ug_PPM25) is the primary component of fine particulate matter, which is emitted as particulates and dispersed within the model without chemical modification.

Total PM2.5 (SURF_ug_PM25_rh50) is defined as: PM2.5 = SO42- + NO3- (fine) + NH4+ + Sea salt (fine) + Mineral Dust (fine) + Secondary organic aerosols (fine) + primary PM2.5 + 0.27 NO3- (coarse) + PM water at relative humidity of 50 %.

NOX is defined as the sum of NO and NO2, converted from ppb to µg/m3 units using the molecular weight of NO2 to give ‘NOX as NO2’.

The netCDF files include the data variables defined in the following table.

| Species name | units | NetCDF variable name |
| --- | --- | --- |
| Near surface concentration of nitrogen dioxide | µg m-3 | SURF_ug_NO2 |
| Near surface concentration of nitrogen oxides (NO + NO2) | µg m-3 | SURF_ug_NOX |
| Particulate matter with aerodynamic diameter < 2.5μm, with particle water at 50 % relative humidity | µg m-3 | SURF_ug_PM25_rh50 |
| Near surface concentration of primary emitted fine particulate matter (contributions files only) | µg m-3 | SURF_ug_PPM25 |
| Near surface concentration of sulphur dioxide | µg m-3 | SURF_ug_SO2 |

## Quality Control

For the 2023 total concentrations a QA/QC has been carried out including comparison with routine concentration measurements. The QA/QC report is available on request but not included in this document. The domestic solid fuel contributions have also been compared to routine and campaign measurements (Di Marco et al., 2026). The comparisons showed that the total PM2.5 is an overestimate compared to routine measurements, possibly due to excessive particle-bound water, whereas the solid fuel contribution was well represented at three out of four measurement sites. The use of the content of this dataset is entirely at your own risk. We make no warranty or guarantee that the content is error free or fit for your intended use.

## Details of the netCDF data structure

An example of the 2023 output from the total concentration file showing the SURF_ug_PM25_rh50 data variable is as follows. The i, j variables give the projected coordinate values for grid cell centres, while the lat, lon variables give the corresponding latitude-longitude values.

netcdf base_concentration_2023 {

dimensions:

time = UNLIMITED ; // (1 currently)

j = 702 ;

i = 501 ;

variables:

int Polar_Stereographic ;

Polar_Stereographic:grid_mapping_name = "polar_stereographic" ;

Polar_Stereographic:straight_vertical_longitude_from_pole = 0. ;

Polar_Stereographic:latitude_of_projection_origin = 90. ;

Polar_Stereographic:scale_factor_at_projection_origin = 0.933012701892219 ;

Polar_Stereographic:numberofrecords = 1 ;

float SURF_ug_PM25_rh50(time, j, i) ;

SURF_ug_PM25_rh50:long_name = "SURF_ug_PM25_rh50" ;

SURF_ug_PM25_rh50:_FillValue = 9.96921e+36f ;

SURF_ug_PM25_rh50:numberofrecords = 1 ;

SURF_ug_PM25_rh50:class = "PM25_rh50" ;

SURF_ug_PM25_rh50:units = "ug/m3" ;

SURF_ug_PM25_rh50:current_date_first = 2023, 2, 1, 0 ;

SURF_ug_PM25_rh50:grid_mapping = "Polar_Stereographic" ;

SURF_ug_PM25_rh50:current_date_last = 2024, 1, 1, 0 ;

SURF_ug_PM25_rh50:coordinates = "lat lon" ;

SURF_ug_PM25_rh50:cell_methods = "time: mean" ;

float i(i) ;

i:standard_name = "projection_x_coordinate" ;

i:coord_axis = "x" ;

i:long_name = "EMEP grid x coordinate" ;

i:units = "km" ;

i:numberofrecords = 1 ;

float j(j) ;

j:standard_name = "projection_y_coordinate" ;

j:coord_axis = "y" ;

j:long_name = "EMEP grid y coordinate" ;

j:units = "km" ;

j:numberofrecords = 1 ;

float lat(j, i) ;

lat:long_name = "latitude" ;

lat:units = "degrees_north" ;

lat:standard_name = "latitude" ;

lat:numberofrecords = 1 ;

float lon(j, i) ;

lon:standard_name = "longitude" ;

lon:long_name = "longitude" ;

lon:units = "degrees_east" ;

lon:numberofrecords = 1 ;

double time(time) ;

time:long_name = "time at middle of period" ;

time:units = "days since 1900-1-1 0:0:0" ;

time:calendar = "standard" ;

time:cell_methods = "time: mean" ;

time:numberofrecords = 1 ;

# Spatial coverage

The data file extents were defined to cover the whole of Scotland.

The data is on a regular grid at 1 km x 1 km resolution defined in a polar stereographic projection, which is described in the following proj4 string: '+proj=stere +ellps=sphere +R=6370000 +lat_0=90.0 +lon_0=0.0 +x_0=0.0 +y_0=0.0 +units=km +k_0=0.9330127018922193 +no_defs +type=crs'. The bounding coordinates in longitude – latitude (spherical datum) are:

SW corner: lon -7.8334°, lat 54.4085°

SE corner: lon -0.3033, lat 54.7195

NE corner: lon -0.3724, lat 60.9568

NW corner: lon -9.5869, lat 60.5638

# Temporal coverage and resolution

The concentrations are annual averages for 2023, derived from monthly average EMEP output.

## Miscellaneous: Guidance for using the data

The model grid is regular in a polar stereographic projection (described in the ‘Spatial coverage’ section), you may need to define this projection in your application. This data set requires a NetCDF library which is, among others, readily available for R, Python, and FORTRAN.

## References

Di Marco, C., Nemitz, E., Cash, J., Mullinger, N. and Hood, C.: Measured air pollutant concentrations from solid fuel burning in Scotland, 2023-2024, NERC EDS Environmental Information Data Centre, https://doi.org/10.5285/e9f39fd7-d181-46d7-a8ef-5cc0c011b932, 2026.

Nemitz, E.; Di Marco, C. F.; Hood, C.; Vieno, M.; Pearson, C.; Carnell, E.; Cash, J.; Mullinger, N.; Tomlinson, S. 2025 The contribution of indoor domestic solid fuel burning to Scotland’s air pollution. Final report to Scottish Government. Edinburgh, UK Centre for Ecology & Hydrology, 89pp. https://nora.nerc.ac.uk/id/eprint/540833/

Nemitz, E., Pearson, C., Tomlinson, S., Carnell, E. and Hood, C.: Calculated air pollutant emissions from domestic solid fuel burning in Scotland, 2023, NERC EDS Environmental Information Data Centre, https://doi.org/10.5285/e82af684-6f84-4d2c-9ec7-dfa8c97b880f, 2026.

Simpson, D., Benedictow, A., Berge, H., Bergström, R., Emberson, L.D., Fagerli, H., Flechard, C.R., Hayman, G.D., Gauss, M., Jonson, J.E., et al. (2012). The EMEP MSC-W chemical transport model – technical description. Atmos. Chem. Phys. 12, 7825–7865.

Skamarock, W.C., Klemp, J.B., Dudhia, J., Gill, D.O., Liu, Z., Berner, J., Wang, W., Powers, J.G., Duda, M.G., Barker, D.M., et al. (2019). A Description of the Advanced Research WRF Model Version 4. UCAR/NCAR.

Vieno, M., Heal, M.R., Williams, M.L., Carnell, E.J., Nemitz, E., Stedman, J.R., and Reis, S. (2016a). The sensitivities of emissions reductions for the mitigation of UK PM2.5. Atmos. Chem. Phys. 16, 265–276.

Vieno, M., Heal, M.R., Twigg, M.M., MacKenzie, I.A., Braban, C.F., Lingard, J.J.N., Ritchie, S., Beck, R.C., Móring, A., Ots, R., et al. (2016b). The UK particulate matter air pollution episode of March–April 2014: more than Saharan dust. Environmental Research Letters 11, 044004.