Ammonia concentrations from high-elevation Himalayan forests and a Sri Lankan tropical forest (2022-2024)

Deshpande, A.G., Ellis, C.J., Pradhan, S.P., Sharma, S., Asad, S.A., Tshering, D., Wangchuk, K., Chatterjee, S., Weerakoon, B., Nissanka, S.P., Ganguly, D., Prabhashwara, T., Duarte, F., Stephens, A., Iwanicka, A., Sutton, M.A. and Jones, M.R.

Background

The South Asian region is a global hotspot of ammonia pollution with majority of the emissions originating from agriculture in the Indo-Gangetic plain. The biodiverse forests of the lower Himalayan range are extremely vulnerable to atmospheric ammonia, which is known to be highly detrimental to several plant species, especially non-vascular plants. Modelled estimates reveal that over 80% of Himalayan forests have already exceeded ammonia critical level for lichens and non-vascular plants (Ellis et al., 2022). The transport of ammonia into the Himalayan forests poses a massive risk for this biodiverse region. The actual ammonia concentrations within the Himalayan forests, however, are completely unknown since no field measurements have been conducted so far. In this study, field measurements of atmospheric ammonia were conducted under forest canopies in the Himalayas in India, Pakistan, Nepal and Bhutan and in a tropical forest in Sri Lanka between 2022 and 2024. This dataset provides the first ever empirical measurements of ammonia from South Asian forests and can be used to estimate ecological impact, inform policy decisions, validate atmospheric chemistry transport models and satellite data and design future air quality and ecological monitoring in the region.

Data collection

The study was conducted at 32 sites in the Himalayas in India (5 sites), Pakistan (11 sites), Nepal (10 sites) and Bhutan (6 sites) and one site in a tropical forest in Sri Lanka. Starting May 2022, triplicate sets of UKCEH Adapted Low-cost Passive High Absorption (ALPHA®) samplers (Tang et al., 2001) were deployed under the forest canopy at sites buffered by at least 500 m of continuous forest on all sides. The exposed samplers were replaced with fresh ones at the end of each month. The exposed samplers from India, Nepal, Bhutan and Sri Lanka were analysed at the National Centre for Sustainable Coastal Management, Chennai and samplers from Pakistan were analysed at UKCEH, Edinburgh using the salicylate method. Blank samplers for each period were also analysed. The analysis provides the concentration of ammonium ions absorbed by each sampler. This is further converted to atmospheric ammonia concentration using an uptake rate which determined annually in the UK by co-locating the ALPHA samplers with UKCEH DEnuder for Long-Term Atmospheric sampling (DELTA®) (Sutton et al., 2001) samplers at multiple UK sites. The monthly concentration at each site is calculated as the average of the concentration measured by each sampler in the triplicate set.

Quality Control

Obvious outliers are removed before calculating the average. Further quality assurance is performed based on comments from site operators regarding physical damage to the sampler, wet sampler, etc.

Data structure

The dataset contains 424 measurements of atmospheric ammonia concentration from 33 sites across South Asia. Data are provided in a CSV file with columns as below.

| Parameter | Unit | Description |
| --- | --- | --- |
| Country | - | Country in which the site is located. |
| Site | - | Local name of the site. |
| Month | - | Month over which the mean ammonia concentration was measured. |
| Latitude | Decimal degrees | Latitudinal coordinates of the site. |
| Longitude | Decimal degrees | Longitudinal coordinates of the site. |
| Elevation | m | Elevation above mean sea level. |
| NH3 | µg m-3 | Mean ammonia concentration measured using UKCEH ALPHA® samplers exposed over a month. |

Funding source

The funding for establishing this facility was provided by the UKRI GCRF South Asian Nitrogen Hub (NE/S009019/1).

References

Ellis, C.J., Steadman, C.E., Vieno, M., Chatterjee, S., Jones, M.R., Negi, S., Pandey, B.P., Rai, H., Tshering, D., Weerakoon, G., Wolseley, P., Reay, D., Sharma, S., Sutton, M., 2022. Estimating nitrogen risk to Himalayan forests using thresholds for lichen bioindicators. Biol. Conserv 265, 109401. https://doi.org/10.1016/j.biocon.2021.109401.

Sutton, M.A., Tang, Y.S., Miners, B., Fowler, D., 2001. A new diffusion denuder system for long-term, regional monitoring of atmospheric ammonia and ammonium. Water Air and Soil Pollution: Focus 1, 145–156.

Tang, Y.S., Cape, J.N., Sutton, M.A., 2001. Development and types of passive samplers for monitoring atmospheric NO2 and NH3 concentrations. ScientificWorldJournal 1, 513–529. https://doi.org/10.1100/tsw.2001.82.