Service Agreement for Climate station data from three common garden plantations along a thermal gradient in the Colombian Andes, 2019-2025

This collection of datasets was collected by weather stations installed to monitor common garden plantations along an elevational gradient in the Colombian Andes between 2018 and 2019. At each experimental site, 360 juvenile trees (1–2 years old) from 15 dominant Andean species (11 montane and 4 lowland species) were planted. For further details on common garden plantations, see Restrepo et al., (2025)

Experimental sites have been named after their mean annual temperatures (MAT) and are characterized as follows: 13.7 °C MAT site (99th percentile temperature, T99=21°C; annual precipitation=2774 mm; elevation=2452 m.a.s.l., latitude=5.513235 N and longitude=-75.67725 W ), 22.1°C MAT site (T99=33°C; annual precipitation=2045 mm; elevation=1326 m.a.s.l., latitude=5.641699 N and longitude=-75.685982 W), and 25.5°C MAT site; T99=38°C; annual precipitation=2298 mm; elevation=736 m.a.s.l., latitude=5,844349 N and longitude=-75.71034 W).

# Weather station setup

In October 2019, one weather station was installed at each common garden plantation to monitor environmental variation across sites. Each weather station was equipped with sensors to measure solar radiation (direct and diffuse), precipitation, air temperature and humidity, wind speed and direction and atmospheric visibility as proxy for foggy conditions. Sensor description is found below.

# Sensor descriptions for environmental measurements.

Solar radiation (direct and diffuse) was measured using a Delta-T Devices BF5 Sunshine Sensor. This sensor quantifies both direct and diffuse components of incoming solar radiation using an array of photodiodes and a shading pattern algorithm that separates total irradiance into direct beam and diffuse fractions. Measurements were recorded as electrical signals (mV) proportional to incident radiation.

Precipitation was measured using a Campbell Scientific TE525WS tipping-bucket rain gauge. Rainfall was recorded as discrete tipping events corresponding to a fixed water volume, which were converted into precipitation depth (mm).

Air temperature and relative humidity were measured using two types of sensors: a Campbell Scientific CS215 temperature and relative humidity probe (SDI-12 output), and a HOBO U23 Pro v2 External Temperature Data Logger. Both sensors measure temperature using a thermistor and relative humidity using a capacitive sensor. Each of the sensors was covered with a radiation shield, a protective, multi-plate housing unit that blocks solar radiation, minimizing heating and precipitation while allowing maximum airflow for accurate ambient temperature and humidity measurements. Dew point temperature was calculated internally by the HOBO data logger based on measured air temperature and relative humidity.

Wind speed and direction were measured using a Gill WindSonic1 ultrasonic anemometer. This sensor determines wind characteristics using ultrasonic pulses and time-of-flight differences caused by airflow. Wind speed is expressed in m s⁻¹ and wind direction in degrees (0–360° relative to true north).

Atmospheric visibility was measured using a the MiniOFS atmospheric visibility sensor. The sensor is sensitive for fog particles in a zone about 25 cm ahead of the location of the sensor who limit the visibility in the air. These particles are normally the microscopic water particles constituting fog, but they may also be raindrops. The sensor is heated to a few degrees above ambient temperature to keep moisture away. The sensor operates with a visibility range from 4 km – down to 20 meters and a wavelength of 850nm.

# Data acquisition system

Environmental variables were recorded using a Campbell Scientific CR1000X measurement and control datalogger installed at each experimental site. Sensor outputs were recorded as electrical or digital signals and processed according to manufacturer specifications.

A solar panel system was used to charge a 12 V battery that powered the data logger and sensor instrumentation..

# Quality control.

Weather stations were regularly inspected to ensure proper operation of sensors, power supply, and data logging systems. Sensor connections and instrumentation were checked, and data were downloaded periodically. Data collection and quality checks were conducted consistently to ensure reliability of measurements.

# Reported measured variables.

Two datasets are provided at 10-minute resolution: The first dataset contains meteorological variables recorded using the Campbell Scientific CR1000X datalogger (Table 1) and the second dataset corresponds to variables recorded with the HOBO U23-001 Pro v2 data logger (Table 2). Tables 1 and Table 2 provide detailed lineage information describing how each variable was generated on each dataset.

Table1. Description of variables for file: “met_station_colombian_andes_common_gardens_2019-2025.csv”

| Variable | Definition | Unit | Logical Range |
| --- | --- | --- | --- |
| Country and Site | Experimental locations where the trees were planted | NA | Caramanta, Tamesis and Puente Iglesias |
| Latitude and Longitude | Geographic coordinates that define a two-point position on Earth using the WGS84 reference ellipsoid. | decimal degrees | Caramanta (-75.678311N;5.513277 W) Tamesis (-75.685954N; 5.641678W) Puente Iglesias (-75.710442N; 5.844561W) |
| Elevation | The vertical distance of a location above mean sea level | meters above sea level | Caramanta 2516; Tamesis 1357 and Puente Iglesias 736 |
| Site_temperature | Mean annual temperature at experimental sites where trees were planted | degree Celsius | Caramanta 13,7; Tamesis 22,1 and Puente Iglesias 25.5 |
| Date_time | Date and time of measurements recorded at 10-minute intervals in hours, minutes, and seconds (HH:MM: SS) | NA | Day/month/year/ and time of day HH=0-24: MM= 0-60: SS= 0-60 |
| Tair_avg | Average air temperature measured over a ten-minute interval | degrees Celsius | Positive values |
| RH_avg | Relative humidity: Ratio of the amount of water vapour present in the air to the maximum amount the air can hold at a specific air temperature, indicating how close the air is to saturation. | percentaje | 0-100 |
| Rain_mm_Total | Total amount of precipitation (rainfall) received over a given at 10-minute intervals | mm | Positive values |
| Rdir | Direct radiation: Electrical signal (in millivolts, mV) from a sensor measuring incoming direct solar radiation. | mV | Positive values |
| Rdif | Diffuse radiation: Electrical signal (in millivolts, mV) representing scattered solar radiation reaching the sensor from the atmosphere. | mV | Positive values |
| WS_ms_Avg | Wind Speed: Horizontal speed of air movement measured at the sampling location. | meters/second | Positive values |
| WindDir_deg | Wind Direction: Direction from which the wind originates, expressed in degrees relative to true north (0–360°). 0° = North, 90° = East, 180° = South, 270° = West | degrees | 0-360 ° |
| Visibility_mV_Avg | Visibility: Maximum horizontal distance at which an object can be clearly observed. Is a measurement of fog and snow. | meters | Positive values |

Table 2. Description of variables for file: “airT_RH_dewT_colombian_andes_common_gardens_2020-2025.csv”

| Variable | Definition | Unit | Logical Range |
| --- | --- | --- | --- |
| Country and Site | Experimental locations where the trees were planted | NA | Caramanta, Tamesis and Puente Iglesias |
| Longitude and latitude | Geographic coordinates that define a two-point position on Earth using the WGS84 reference ellipsoid. | decimal degrees | Caramanta (-75.678311N; 5.513277W) Tamesis (-75.685954N; 5.641678W) Puente Iglesias (-75.710442N; 5.844561W) |
| Elevation | The vertical distance of a location above mean sea level | meters at the sea level | Caramanta 2516; Tamesis 1357 and Puente Iglesias736 |
| Site_temperature | Mean annual temperature at experimental sites where trees were planted | degree Celsius | Caramanta 13,7; Tamesis 22,1 and Puente Iglesias 25.5 |
| Date_time | Date and time of measurements recorded at 10-minute intervals in hours, minutes, and seconds (HH:MM: SS) | Day/month/year & HH=0-24: MM= 0-60: SS= 0-60 |  |
| Tair | Average air temperature measured over a ten-minute time interval. | degrees celsius | Positive values |
| RH | Relative humidity is measured directly by the sensor and reflects atmospheric moisture conditions at the measurement location. | percentaje | 0-100 |
| Dw_point | Dew point: The temperature at which air must be cooled to become saturated with water vapour reaching 100% RH and condensation begins | Degree celsius | Positive values |

# References

Restrepo, Z., González-caro, S., Hartley, I. P., Villegas, J. C., Meir, P., Sanchez, A., Carrascal, D. R., & Mercado, L. M. (2025). Heterogeneous thermal tolerance of dominant Andean montane tree species. Communications Earth & Environment, 1–11. https://doi.org/10.1038/s43247-025-02083-w