Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP)

ERAMMP Supporting Information Metadata for:

Soil erosion and disturbance assessment data from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023

Client Ref: Welsh Government / Contract C210/2016/2017

Version 1.0

Date: 06/02/2026

Version History

| Version | By | Date | Changes |
| --- | --- | --- | --- |
| 1.0 | JRL & CF | 06/02/2026 | published |

Document Info

| Series | Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP) |
| --- | --- |
| Title | ERAMMP Supporting Information Metadata for: Soil erosion and disturbance assessment data from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023 |
| Client | Welsh Government |
| Client reference | C208/2021/2022 |
| Confidentiality, copyright and reproduction | © Crown Copyright 2026 This document is licensed under the Open Government Licence 3.0. |
| UKCEH contact details | Bronwen Williams UK Centre for Ecology & Hydrology (UKCEH) Environment Centre Wales, Deiniol Road, Bangor, Gwynedd, LL57 2UW 01248 374500 erammp@ceh.ac.uk |
| Corresponding author | Chris Feeney, UKCEH chrfee@ceh.ac.uk |
| Authors | Feeney, C.1, Tye, A.2, Robinson, D.1, Lucas, J.R.1, Monkman, G.1, Reinsch, S.1, Richardson-Jones, V.1, Wood, C.M.1, Dhiedt, E.1, Horninge, L.3, Maskell, L.1, Moir, A.2, Norton, L.1, O’Neil, N., Roberts, M., Savage, C., Sayer, H., Williams, B.1, Emmett, B.A.1 |
| Contributing authors & reviewers | 1UK Centre for Ecology & Hydrology (UKCEH), 2British Geological Survey (BGS), 3National Trust for Scotland, 4Welsh Government |
| How to cite (long) | Feeney, C., Tye, A., Robinson, D., Lucas, J.R., Monkman, G., Reinsch, S., Richardson-Jones, V., Wood, C.M., Dhiedt, E., Horninge, L., Maskell, L., Moir, A., Norton, L., O’Neil, N., Roberts, M., Savage, C., Sayer, H., Williams, B., Emmett, B.A. (2026). ERAMMP Supporting Information Metadata for: Soil erosion and disturbance assessment data from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023 (Report-119). UKCEH |
| How to cite (short) | Feeney et al. (2026). ERAMMP Supporting Information Metadata for: Soil erosion and disturbance assessment data from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023 (Report-119). UKCEH |
| Approved by | Bridget Emmett (UKCEH) Oliver Massey (Welsh Government) |

Abbreviations

| AES | agri-environment schemes |
| --- | --- |
| APGB | Aerial Photography Great Britain |
| BGS | British Geological Survey |
| CS | Countryside Survey |
| DTM | Digital Terrain Model |
| EO | Earth Observation |
| ERAMMP | Environment and Rural Affairs Monitoring & Modelling Programme |
| EU | European Union |
| GB | Great Britain |
| GMEP | Glastir Monitoring and Evaluation Programme |
| LCM | Land cover map (followed by the year) |
| LMS | Land Management Scheme |
| NFS | National Field Survey |
| OS | Ordnance Survey |
| SED | Soil Erosion and Disturbance |

Contents

1.	Overview	5

2.	Background Information	5

3.	Introduction to the GMEP Survey Design and its re-survey by ERAMMP	6

4.	Data Collection Methods	7

4.1	Identification of features from aerial imagery	7

4.2	Field survey	8

5.	Data Processing and Analytical Methods	9

5.1	SED features from aerial images	9

5.2	SED features from field survey	9

5.3	SED drivers map	9

6.	Data Quality Assurance	12

7.	Notes	13

7.1	General notes	13

7.2	SED features from field survey	13

8.	Data Structure	14

8.1	File details:	14

8.2	Table columns	14

8.3	Relationship between datasets:	17

9.	References	18

10.	Appendix 1: Surveyor Key to Features	20

# Overview

| Spatial Extent | Terrestrial extent of Wales, UK |
| --- | --- |
| Temporal Extent | 2018, 2021–2023 |
| Summary | This dataset consists of polygon and polyline layers containing categorised soil erosion and disturbance features collected as part of the Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP) National Field Survey (NFS) between 2021-2023. Data include extent and categorised type of soil erosion and disturbance (SED) features from a desk-based survey of aerial imagery and mapped by field surveyors between 2021-2023. This dataset also includes a raster map of the primary groups of controls on SED feature density in Wales. ERAMMP NFS is a resurvey of all of the 300 locations from the initial Glastir Monitoring and Evaluation Programme (GMEP) monitoring program (300 1km squares) on a rolling annual basis, with the aim to resurvey all sites between 2021 and 2025. SED features were identified from aerial images of 261 squares. Between 2021-2023 ERAMMP NFS field surveyors surveyed zones within 225 squares for SED features. The initial monitoring program, GMEP, was set up by the Welsh Government in 2013 to report national trends and monitor the effects of the Glastir agri-environment scheme on the environment and ran from 2013 to 2016. The field survey element was based on a stratified random sampling design of 300 x 1km square sites across Wales, and was managed by the UK Centre for Ecology & Hydrology. |
| File Name(s) | ERAMMP_SOIL_EROSION_EO.csv ERAMMP_SOIL_EROSION_FIELD_SURVEY_21_23.csv ERAMMP_SOIL_EROSION_SURVEYED_SQUARES_21_23.csv Wales_soil_erosion_and_disturbance_zones.tif |
| File Format(s) | 3x .csv 1x .tif |

# Background Information

In Wales, funding from agri-environment schemes (AES) has been available since the early 1990s including Environmentally Sensitive Areas, the Habitat Scheme, the Woodland Grant Scheme, the Organic Farming Scheme, Tir Cymen, Tir Cynnal, Tir Gofal, followed by the Glastir scheme. A monitoring programme was set up by the Welsh Government in 2013 to monitor the national state and change of the countryside, along with effects of the Glastir scheme on the environment. This four-year rolling monitoring programme was called the “Glastir Monitoring and Evaluation Programme” (GMEP) and was conducted between 2013 and 2016. GMEP aimed to: (1) Evaluate the environmental effects of the Glastir sustainable land management scheme (LMS) (https://gov.wales/glastir); (2) Quantify environmental status and spatiotemporal environmental status trends across the Welsh countryside and; (3) Assess and predict outcomes for climate change mitigation, biodiversity, soil and water quality, woodland expansion and cultural landscapes (https://gmep.wales). The project was led by UKCEH. A major component of the work to deliver the first 2 aims was a National Field Survey (NFS) which was an expansion of a long-term integrated monitoring programme developed by UKCEH since the 1970s called Countryside Survey (Carey et al., 2008, Wood et al., 2018, Robinson et al., 2024).

Synthesis and analysis of the data derived from the NFS sought to identify how the Welsh environment is being impacted by drivers of change, such as land use, climate, and pollution, over and above Glastir interventions. The GMEP report was published in 2017 (Emmett et al. 2017) and data are summarized in Wood et al. 2021. Data are available from the Environmental Information Data Centre (https://eidc.ac.uk/) as part of the data series of the Glastir Monitoring and Evaluation Programme, Wales, 2013-2016.

The ’Environment and Rural Affairs Monitoring & Modelling Programme’ (ERAMMP) National Field Survey is the re-survey of GMEP locations. Data informs national trends analyses, and the effects of Glastir action bundles. Other elements of GMEP and ERAMMP including numerous expert reviews and integrated modelling is reported elsewhere (https://erammp.wales/).

Identifying the extent of soil erosion and disturbance (SED) features is an important first step to a statistical assessment of the vulnerability of the soils of Wales to physical degradation and reductions in their capacity to undertake normal soil function.

# Introduction to the GMEP Survey Design and its re-survey by ERAMMP

In the Glastir Monitoring and Evaluation Programme, the NFS was designed to capture multiple measures and metrics, and to integrate across these metrics. Hence a full ecosystem-based approach is required where data operating at multiple scales are captured, where possible, at the same time in a single snap-shot visit.

For GMEP, a 4-year rolling survey was adopted to maximise the number of sites visited across the national spatial scale whilst at the same time monitoring year-on-year, such that spatial variation and temporal changes and trends can be detected cost-effectively (Emmett et al., 2013 & Wood et al. 2021). The first survey cycle dates from 2013-2016. The first partial re-survey cycle took place between 2021 and 2023, after being postponed from 2020 due to the COVID pandemic.

The rolling programme-based survey has two key components: the Wider Wales Component used for baseline estimation (Welsh national trends) and national reporting of Glastir action bundles (Targeted Component) which specifically links to the 9 objectives (of which 6 environmental outcomes are intended to be captured in the NFS) of the Glastir scheme. Across both components, integration of survey data is essential and therefore a common spatial unit of 1 km x 1 km square was adopted.

The selection of the sample of 1-km x 1-km squares for monitoring was accomplished following the same procedure as used for the Countryside Survey (CS) of Great Britain (GB) (as described below), which aims to provide robust estimates of indicators at national level across GB and constituent countries. Using the same methodology builds on previous knowledge and enabled integration of the two surveys in future analyses.

A total of 300 x 1 km2 squares were sampled over the 4-year GMEP NFS cycle. Half of these squares (n=150) were the Wider Wales squares, randomly sampled within strata defined according to the Land Classification of Great Britain (Bunce et al., 2007) – a derived classification of the landscape based on its topography, geology, climate and physical attributes. Environmental heterogeneity is minimized within each stratum of the Land Classification and is maximised between strata. The proportion of the 1-km x 1-km squares randomly sampled from within each stratum was proportional to the size of the stratum to allocate survey efforts best. The other half of the sampled squares were targeted specifically at Glastir priority areas (n=150). Glastir-targeted squares were randomly selected with a selection probability proportional to the ‘Glastir score’ of the square, derived from the Welsh Government’s scoring system of the Glastir Advanced target objective areas (Emmett et al., 2014). Any square randomly selected that contained more than 75% of urban land or that was more than 90% sea (defined by LCM2007 and the UK Census mean high tide data) was excluded.

The ERAMMP NFS re-survey covered 130 of the original 300 squares in 2021-22, and an additional 95 survey squares in 2023. The 130 squares in 2021-22 were selected from both the Wider Wales and Glastir-targeted squares to enable the evaluation of multiple Glastir interventions based on multiple environmental outcomes; the selection of these squares does not enable a robust evaluation of national trends without more Wider Wales squares being surveyed (Alison et al. 2021). Therefore, the 95 survey squares in 2023 were the remaining Wider Wales squares that had not been surveyed. A total of 225 squares were re-surveyed in ERAMMP between 2021 and 2023: 147 Wider Wales squares and 78 Glastir-targeted squares (Figure 1).

| GMEP (2013-2016) | ERAMMP (2021-2023) |
| --- | --- |

Figure 1: Maps showing the distribution of 1-km x 1-km National Field Survey squares across Wales (plotted at 10 km to ensure data confidentiality). ERAMMP map shows all NFS squares surveyed in 2021-2023; the colour key indicates survey years for the botanical survey, soil erosion features were surveyed in the same years.

# Data Collection Methods

## Identification of features from aerial imagery

A desk-based soil erosion and disturbance (SED) survey was undertaken by the British Geological Survey (BGS) using high resolution aerial images to identify the extent of SED across 261 1 km x 1 km areas of Wales. These squares form part of the wider ERAMMP NFS. These were selected as part of the planning process for the next stage of the ERAMMP field survey (Alison et al., 2021).

Aerial photos were cut for the selected 1 km x 1 km squares. The aerial images used were the Aerial Photography Great Britain (APGB) high resolution aerial imagery licensed to BGS from Bluesky International Limited. The photos were the ‘True Orthorectified 25cm National Resolution’ dataset. Other datasets loaded into the GIS to help interpret the potential areas of erosion identified from the aerial imagery included OS maps and DTM derived landscape characteristics. Google Earth images were additionally used to help analyse potential erosion areas in squares where the resolution of the Bluesky aerial imagery was not sufficiently clear. An important further factor in the interpretation of potential features was the analyst’s knowledge of landscape features and agricultural practice. This expert knowledge is important in asking whether ‘would this process be likely here?’ and in classifying the nature of the soil erosion feature. Using these datasets, areas of SED were marked as polygons on the GIS which had their spatial extent calculated.

The time required for the photo analysis was approximately 2 to 2.5 hours for five 1 km x 1 km squares. However, squares with a large number of features, particularly those in this survey associated with lowland dairy farming, could take 40 minutes or more. When examining the aerial imagery, it was found that a good image resolution to use was obtained at 1:1250 (i.e. 1 cm on the screen represents 1250 cm in real life). When increasing resolution on the GIS beyond 1:1250, there was a tendency for the landscape to become a pixelated blur. With respect to the aerial imagery the specific dates of the photography were not easily identified (but are understood to be mainly from Spring 2018) so there may be questions of relevance if an up-to-date survey based on aerial images is required. However, for the purposes of this study, the time period of the photos was not considered a large negative, as it is not expected that large changes in the spatial land use and agricultural practice would take place. Thus, SED features are likely to be fairly consistent as arable and grassland will have similar practice and the landscape features such as slope will be constant.

## Field survey

During the ERAMMP NFS field survey, a subset of features was visited by field surveyors to validate the major features identified by the aerial survey (EO (aerial photography) features) and to record any features that might have been missed or were new. Surveyors were instructed to record within zones generated by drawing a 200m radius circles around ‘X-plots’: each square contains up to 5 plots (so called X-plots) where soils are sampled and vegetation is assessed (see Smart et al., 2023 for further details on locations of X-plots). Working within these 200m radii would maximise the chances of capturing a significant proportion of the features mapped from aerial images, while keeping field survey times tractable (Robinson et al., 2021). Survey teams were constrained to recording features on land where the survey had permission to record.

Surveyors were provided with a map showing polygons of SED features identified from aerial images as described above, along with the 200m radius circles around X-plots they were to survey within. Surveyors located features within these circles and confirmed their presence or absence, recording survey data using the ESRI ArcGIS Online tool, Survey123 on a field tablet. Any extra, easily visible, features within the 200m radius circles that the aerial survey might have missed were also recorded using the same methodology. Major features, typically covering an area of >10m2, were recorded. Tramlines in fields, road cuttings, grips on peat or field drainage channels were not recorded.

First-order estimates of the area of the eroded or disturbed soil event were made by marking the feature on the map on the tablet, either a satellite image or an OS 1:25000 map, including groups of features together to capture location and general extent. In some cases, visible EO (aerial photography) features were not traced by surveyors if the extent and location of the EO feature was judged to be correct. Features were categorised (see below) and photographed. Surveyors recorded the Broad Habitat (see Smart et al., 2023) in which features were located, along with details of land cultivation.

SED Features were categorised by surveyors into four main categories using a key (Appendix 1):

- Features that are erosion on organic / peat soils

- Features caused primarily by human or animal disturbance

- Features that are caused by gravity

- Features that are erosion on mineral soils

For further information on these categories, and more details on soil erosion field survey methodology see Robinson et al., 2021.

# Data Processing and Analytical Methods

## SED features from aerial images

Typographical errors were corrected, and capitalisations standardised in the LANDUSE and SLOPE columns. The DESCRIPTION column was cleaned to harmonise inconsistent data entry, and a CATEGORY column was created to categorise erosion features into four major category types (Table 1).

Table 1. The four major categories of SED and their sub-groupings derived from the aerial photo survey and used in the field survey.

| Peat & organo-mineral erosion features | Soil disturbance features | Scar or slip features | Mineral soil erosion features |
| --- | --- | --- | --- |
| Peat hags or peat erosion | Poaching or compaction by animals in fields or around feeders | Soil scar or slip | Rain Splash |
| Peat drainage ditch erosion | Poaching or compaction around gates | Soil creep / Terracettes | Sheet erosion |
| Peat pipes or tunnels present | Footpath disturbance or erosion | Scree | Rill |
|  | Substantial wheel ruts / machinery disturbance | Landslides or other mass movements | Gully |
|  | Tree root scars |  | Tillage |
|  |  |  | Riverbank erosion |
|  |  |  | Drainage ditch |
|  |  |  | Coastal |
|  |  |  | Soil pipes |
|  |  |  | Muddy outwash onto roads |
|  |  |  | Soil Erosion General |

## SED features from field survey

Surveys were merged for 2021-2023 into a single layer. During the survey, some EO (aerial photography) features assessed as ‘not visible’ by surveyors were grouped by surveyors under a single assessment record. These features were disaggregated into identical records, and each resulting row assigned the unique ERAMMP_ID.

## SED drivers map

In an effort to better understand the key landscape controls on SED feature density across Wales, regression tree modelling was applied. Regression tree modelling offers a powerful way of considering the interactions among multiple potential predictor variables to estimate a dependent variable – in this case, using factors such as land cover class, soil type, topography and rainfall to estimate the areal extent of SED features. Crucially, this type of model constructs a decision tree in which branches represent different combinations of predictor variables, and leaves represent mean predicted values for the dependent variable of interest. Potential predictor variables which were used in the regression tree modelling are summarised in Table 2.

Table 2. Potential SED predictors that were either already recorded for each ERAMMP square or were extracted. All methods of calculation were performed in R 4.4.1 (R Core Team, 2024).

| Predictor | Description | Method of calculation |
| --- | --- | --- |
| Broad habitat | Broad habitat classes recorded within each 1km survey square. | Modal class per square |
| Mean annual rainfall | Long-term mean annual rainfall (mm yr-1) for 1961-2017 based on the CHESS spatial dataset (Robinson et al., 2020) | Mean value determined per square |
| Soil type | DEFRA soil types described in Feeney et al. (2023), derived from linking soil series in the 1:250,000 scale NATMAP vector product to each of the 12 classes | Survey squares were intersected with the DEFRA soils classes map. The soil class with the single largest area was set as the soil class for the specific survey square. |
| Elevation | Mean elevation above mean sea level (m) based on the Intermap 25m DTM | Mean value of all 25m resolution pixels in a given 1km square |
| Local relief | Similar to elevation, but using the range rather than the mean | Range of all 25m resolution pixels in a given 1km square |

Here, the “rpart” package (version 4.1.23) in R (Therneau & Atkinson, 2023) was applied to implement the regression tree modelling. A training dataset was created by extracting a random sample of 80 % of the EO (aerial photography)-mapped SED dataset; the remaining 20 % was kept as testing data. An important weakness to regression tree models is their sensitivity to parameter settings and to the input dataset used to train the models. To mitigate these weaknesses, first, tree depth was controlled by optimising the complexity parameter value via 10-fold cross-validation with the “caret” package (version 6.0-94) in R (Kuhn, 2008), automating the usual step of “pruning” decision tree sub-nodes to reduce model over-fitting; second, an ensemble of 1,000 regression trees were constructed, each based on a unique combination of training and testing dataset. Regression tree accuracy was assessed by applying the model fitted to the training dataset to the testing dataset and calculating the root mean square error (RMSE), with the model corresponding to the lowest RMSE being selected. The nodes (or leaves) of this final decision tree represent what we describe as the SED driver groups: combinations of environmental factors that best predict different extents of SED.

The best-fit decision tree (Figure 2a) had only 3 significant predictors: elevation, habitat (land cover) and mean annual rainfall; relief and soil type were found to be not significant predictors of SED occurrence The decision tree results were therefore mapped for the whole of Wales using the following datasets:

- CHESS Rainfall, long-term average covering 1961-2017 (Robinson et al., 2020)

- UKCEH Land Cover Map, 25m raster for the year 2021 (Marston et al., 2022) Land cover map categories were converted to broad habitat classes as per Table 3.

- 25m digital terrain model by Intermap (Intermap, 2009)

All layers were harmonised to the same extent and grid resolution using the 1km Ordnance Survey grid tiles as a reference, yielding the map in Figure 2b. The group numbers correspond to the following names in supplement 9 of the technical annex to the Wales National Trends and Glastir Evaluation Report (Feeney et al., 2025):

- Agriculturally dominated

- Dry lowland bog and acid grassland

- Wet lowland bog and acid grassland

- Dry uplands

- Wet uplands

The 5 SED driver groups reflect a gradation in average SED feature density per 1km2, ranging from 1.22% for “Agriculturally dominated” to 27.44% for “Wet uplands”. “Soil disturbance” features are the dominant form of SED for groups 1 and 2, with a mix of “peat & organo-mineral” and “scar or slip” features characterising the other 3 groups. It is anticipated that the map will help with targeting land management recommendations to address different forms of SED across Wales and can be refined as future SED surveys are conducted.

Figure 2: a) Decision tree detailing the characteristics of each SED driver group. The percentages refer to the mean density of SED features per 1km grid; values in square brackets refer to the proportion (0-1 scale) of survey squares classes as a particular SED driver group. b) 1 km2 Ordnance Survey grids of Wales categorised into the 5 SED driver groups represented in the decision tree.

Table 3: Land cover map classes and their relationship to broad habitat classes

| LCM2021 target class number | LCM2021 target class | Broad habitat |
| --- | --- | --- |
| 1 | Broadleaved woodland | ‘Broadleaved, mixed and yew woodland’ |
| 2 | ‘Coniferous woodland’ | ‘Coniferous woodland’ |
| 3 | ‘Arable and horticulture’ | ‘Arable and horticulture’ |
| 4 | ‘Improved grassland’ | ‘Improved grassland’ |
| 5 | ‘Neutral grassland’ | ‘Neutral grassland’ |
| 6 | ‘Calcareous grassland’ | ‘Calcareous grassland’ |
| 7 | Acid grassland | ‘Acid grassland’ |
| 8 | ‘Fen, marsh and swamp’ | ‘Fen, marsh and swamp’ |
| 9 | Heather | ‘Dwarf shrub heath’ |
| 10 | Heather grassland |  |
| 11 | ‘Bog’ | ‘Bog’ |
| 12 | ‘Inland rock’ | ‘Inland rock’ |
| 13 | Saltwater | Saltwater |
| 14 | Freshwater | Freshwater |
| 15 | ‘Supra-littoral rock’ | ‘Supra-littoral rock’ |
| 16 | ‘Supra-littoral sediment’ | ‘Supra-littoral sediment’ |
| 17 | ‘Littoral rock’ | ‘Littoral rock’ |
| 18 | Littoral sediment | ‘Littoral sediment’ |
| 19 | Saltmarsh |  |
| 20 | Urban | ‘Built-up areas and gardens’ |
| 21 | Suburban |  |

# Data Quality Assurance

SED feature layers were checked for spatial integrity against a layer containing surveyed squares. Recorded SQ_ID was checked for referential integrity against SQ_ID of surveyed squares. Field survey data were checked for the EO (aerial photography) features expected to be reported on within a square. Issues arising from data checks were fed back to field teams to improve their recording.

# Notes

## General notes

- To assess the national trend, only data originating from the Wider Wales square data population should be analysed as these are nationally representative.

- To assess Glastir impacts, data from Wider Wales and Glastir targeted squares should be analysed together and in conjunction with associated data on Glastir management, as both sets of squares contain sites with Glastir management option uptake and no Glastir uptake. However, information on the locations of Glastir uptake is not included in this dataset and must be requested from Welsh Government (data@gov.wales).

- To assess the aerial extent of soil erosion and disturbance across Wales, the aerial image dataset is more comprehensive and more robust than the NFS field surveyed features (as whole features can be identified more easily from planform aerial images). The NFS field surveyed features provide a large-scale “ground-truthed” dataset and a valuable proof of concept for future ground-level surveys of soil condition.

## SED features from field survey

- This dataset contains visited SED features only. Features identified in the aerial survey could not be visited in the field survey if they occurred on land where survey teams did not have permission to record.

- Broad Habitat designations were assigned in the field by surveyors.

- 4 EO (aerial photography) features are described in two records in the NFS dataset.

- 185 features identified in the aerial survey and reidentified as visible in the field survey have no surveyor-mapped extents. Surveyors did not always re-map features identified in the aerial survey if location and extent of the aerially-mapped polygon matched what they could see in the field.

- As a first-order estimate of the area of SED features identified in the field survey, surveyors drew a polyline around the extent of the feature. The length of the polyline is provided in the NFS dataset.

- No line length is provided for events where erosion was recorded by surveyors as not present.

# Data Structure

## File details:

- ERAMMP_SOIL_EROSION_SURVEYED_SQUARES_21_23.csv, a table containing SQ_IDs of all survey squares, indicating whether a square was surveyed in the desk-based survey of soil erosion and disturbance identified from aerial images, and/or as part of the National Field Survey 2021-2023. The dataset contains 7 columns of 276 rows.

- ERAMMP_SOIL_EROSION_EO.csv, a table containing areas of soil erosion and disturbance identified from aerial images of 261 1km squares. SED features were identified in 252 squares. The layer was created by the BGS from a combination of APGB aerial photography, Ordnance Survey (OS) maps, Google Earth, Digital Terrain Model (DTM) derived landscape characteristics, and analyst knowledge of landscape features. The dataset contains 13 columns of 2580 rows.

- ERAMMP_SOIL_EROSION_FIELD_SURVEY_21_23.csv, a table containing data on soil disturbance and erosion features collected in the National Field Survey 2021-2023 from 225 1km squares. Features were recorded in 199 squares. The dataset contains 24 columns of 2036 rows.

- Wales_soil_erosion_and_disturbance_zones.tif, a geotiff (raster, 32-bit signed integer) containing a map of the primary groups of controls upon SED feature density in Wales, with classes assigned to all 1km OS grids across Wales. The dataset consists of 5 categories and uses the EPSG:27700 – OSGB36 / British National Grid coordinate reference system

## Table columns

ERAMMP_SOIL_EROSION_SURVEYED_SQUARES_21_23:

| Column name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Text | none | 1km survey square |
| EO_SURVEY | Number | none | Square surveyed in aerial photographic desk-based survey (1=Yes, 0=No) |
| NFS_SURVEY | Number | none | Square surveyed in NFS 2021–2023 (1=Yes, 0=No) |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| LAND_CLASSIFICATION_NUMBER | Number | none | Sampling framework category - ITE Land Class 2007 (Bunce et al., 2007). |
| SQ_TYPE | Text | none | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |

ERAMMP_SOIL_EROSION_EO:

| Column name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Text | none | 1km survey square |
| ERAMMP_ID | Text | none | Unique identifier of feature, a concatenation of SQ_ID and EO_FEATURE_ID |
| EO_FEATURE_ID | Number | none | Feature ID number assigned during desk-based aerial survey, unique within a square |
| LANDUSE | Text | none | Basic land use information determined from aerial imagery (<null>=no data) |
| SLOPE | Text | none | Basic topographic information recorded in relation to mapped features (<null>=no data) |
| DESCRIPTION | Text | none | Description of erosion feature |
| CATEGORY | Text | none | Category of erosion type |
| SHAPE_LENGTH_M | Number | m | Perimeter of feature |
| SHAPE_AREA_M2 | Number | m2 | Area of feature |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| LAND_CLASSIFICATION_NUMBER | Number | none | Sampling framework category - ITE Land Class 2007 (Bunce et al., 2007). |
| SQ_TYPE | Text | none | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |

ERAMMP_SOIL_EROSION_FIELD_SURVEY_21_23:

| Column name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Integer | none | 1km survey square – unique identifier |
| NFS_ID | String | none | Unique identifier of NFS record, of the form <ERAMMP_ID>_record number |
| ERAMMP_ID | String | none | Identifier of feature, an incrementing integer partitioned by SQ_ID. ERAMMP_IDs of the form <SQ_ID>_oid_<number> indicate features recorded in the field survey not present in ERAMMP_SOIL_EROSION_EO |
| EO_FEATURE | String | none | Was this feature identified in the desk-based aerial survey? (Yes, No) |
| EO_FEATURE_ID | String | none | If a feature identified in the aerial survey, feature ID number assigned by BGS, unique within a square (<null>= no data or not applicable) |
| EO_FEATURE_VISIBLE | String | none | If a feature identified in the desk-based aerial survey, was it visible in the field survey? (Yes, No. <null> = not applicable, not a feature identified in the desk-based aerial survey) |
| LENGTH_LINE_M | Integer | m | Line length of polyline drawn by surveyors (<null> if not applicable) |
| SURVEY_START_DATE | Date | none | Date and time of start of survey |
| FEATURE_CATEGORY | String | none | Main feature category (peat=peat & organo-mineral erosion features, soil=soil disturbance features from animals and humans, mass=scar or slip features on mineral soil, min=mineral soil erosion features, <null>= no data; unclassified or not applicable: not an erosion feature, not visible, no access) |
| ORGANIC_EROSION | String | none | Type of peat & organo-mineral erosion feature: Code Description Peat_hags_erosion = Peat hags or peat erosion Peat_drainage = Peat drainage ditch bank erosion Peat_pipes_tunnels = Peat pipes or tunnels present <null> = not applicable, feature belongs to a different category |
| ANIMAL_HUMAN_EROSION | String | none | Type of soil disturbance feature from animals and humans: Poaching_feeders = Poaching or compaction by animals in fields or around feeders Poaching_gates = Poaching or compaction around gates Footpath_damage = Footpath damage or erosion Wheel_ruts_machinery = Substantial wheel ruts / machinery damage Tree_roots = Tree root scars or erosion <null> = not applicable, feature belongs to a different category |
| MASS_MOVEMENT_EROSION | String | none | Type of scar or slip feature: Soil_scar = Soil scar or slip Soil_creep = Soil creep / terracettes Scree = Scree Landslides = Landslides or other mass movement <null> = not applicable, feature belongs to a different category |
| MINERAL_EROSION | String | none | Type of mineral soil erosion feature: Splash = Splash erosion Sheet = Sheet erosion Rill = Rill erosion Gully = Gully erosion Tillage = Tillage erosion Riverbank = River bank erosion Drainage = Drainage ditch erosion Coastal = Coastal erosion Soil_pipes = Soil pipes or tunnels present Muddy_outwash = Muddy outwash from fields visible <null> = not applicable, feature belongs to a different category |
| CULTIVATED | String | none | Is the land cultivated? (Yes, No, <null> = no data, may suggest not applicable to the habitat type) |
| FURROWS | String | none | If land is cultivated, and there are furrows, which way are the furrows pointed (across = pointed across slope, down = pointed downslope, <null> = not applicable: no furrows or not cultivated) |
| BROAD_HABITAT | String | none | The broad habitat (see Smart et al., 2023) the feature is in. |
| CROPS | String | none | Crop type on the land, arable or ley only. (<null> = not applicable) |
| CROPS_OTHER | String | none | If crop type is ‘other’, free text field to specify crop type. (<null> = not applicable) |
| OM_LAYER_CM | Integer | cm | Ad hoc measurement of organic matter horizon (O-horizon) depth consistent with the erosion feature. This represents extra information some surveyors entered but was not required. <null>=not measured |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| LAND_CLASSIFICATION_NUMBER | Integer | none | Sampling framework category - ITE Land Class 2007 (Bunce et al., 2007). |
| SQ_TYPE | String | none | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |
| SURVEY_YEAR | Integer | none | Year of survey |

## Relationship between datasets:

ERAMMP_ID is a unique ID in the ERAMMP_SOIL_EROSION_EO dataset. ERAMMP_ID can be used to match features between datasets.

# References

Alison, J., Bowgen, K., Siriwardena, G., Williams, B., Reinsch, S., Garbutt, A., Emmett, B.A. (2021) Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP). ERAMMP Report-58: ERAMMP Square Selection Protocol 2021/2022. Report to Welsh Government (Contract C210/2016/2017) (UK Centre for Ecology & Hydrology Projects 06297 & 06810):

https://erammp.wales/sites/default/files/2022-11/58%20ERAMMP%20Report-58%20Square58%20%20Selection%20Protocol%202021%20v1.0_en.pdf

Bunce, R. G. H., Barr, C. J., Clarke, R. T., Howard, D. C., Scott, W. A. (2007) ITE Land Classification of Great Britain 2007. NERC-Environmental Information Data Centre doi: 10.5285/5f0605e4-aa2a-48ab-b47c-bf5510823e8f

Carey, P. D.; Wallis, S.; Chamberlain, P. M.; Cooper, A.; Emmett, B. A.; Maskell, L. C.; McCann, T.; Murphy, J.; Norton, L. R.; Reynolds, B.; Scott, W. A.; Simpson, I. C.; Smart, S. M.; Ullyett, J. M. (2008). Countryside Survey: UK Results from 2007 NERC/Centre for Ecology & Hydrology (CEH Project Number: C03259): https://nora.nerc.ac.uk/id/eprint/5191/1/N005191CR%20UK%20Results.pdf

Emmett, B. A. et al. (2013). An integrated ecological, social and physical approach to monitoring environmental change and land management effects: the Wales Axis II Monitoring and Evaluation Programme. Aspects of applied biology, 31-39.

Emmett, B.E., Abdalla, M., Anthony, S., Astbury, S., August, T., Barrett, G., Biggs, J., Botham, M., Bradley, D., Brown, M., Carter, H., Chadwick, D., Cigna, F., Cooper, D., Cooper, J., Cosby, BJ., Creer, S Cross, P., Edwards, F., Edwards, M., Evans, C., Ewald, N., Fitton, A., Garbutt, A., Grebby, S., Greene, S., Halfpenney, I., Hall, J., Harrison, S., Harrower, C., Henrys, P., Hobson, R., Hughes, S., Isaac, N., Jackson, B., Jarvis, S., Jones, D., Keith, A., Kelly, M., Korenko, J., Lallias, D., Leaver, D., Lebron, I., Malcolm, H., Maskell, L., McDonald, J., Moxley, J., Norton, L., O’Hare, M., Owen, A., Pereira, M., Peyton, J., Powney, G., Pywell, R., Rawlins, B., Robinson, D.A., Rorke, S., Rowland, C.,Roy, D., Scarlett, P., Scholefield, P., Scott, A., Scott, L., Scott, R., Siriwardena, G., Smart, S., Smith, P., Swetnam, R., Taylor, R., Tordoff, G., Van Breda, J., Vincent, H., Wagner, M., Waters, E., Watkins, J.,White, J., Williams, B., Wood, C. and Wright, S. (2014) Glastir Monitoring & Evaluation Programme. First Year Annual Report to Welsh Government (Contract reference: C147/2010/11). NERC/Centre for Ecology & Hydrology (CEH Project: NEC04780), pp.442

Emmett, B.E., Abdalla, M., Anthony, S., Astbury, S., August, T., Barrett, G., Beckman, B., Biggs, J., Botham, M., Bradley, D., Brown, M., Burden, A., Carter, H., Chadwick, D., Cigna, F., Collier, R., Cooper, D., Cooper, J., Cosby, B.J., Creer, S., Cross, P., Dadam, D., Edwards, F., Edwards, M., Evans, C., Ewald, N., Fitton, A., Garbutt, A., Giampieri, C., Gooday, R., Grebby, S., Greene, S., Halfpenney, I., Hall, J., Harrison, S., Harrower, C., Henrys, P., Hobson, R., Hughes, P., Hughes, S., Illian, J., Isaac, N., Jackson, B., Jarvis, S., Jones, D.L., Jones, P., Keith, A., Kelly, M., Kneebone, N., Korenko, J., Lallias, D., Leaver, D., Lebron, I., Malcolm, H., Maskell, L., McDonald, J., Moxley, J., Norton, L., O’Hare, M., Oliver, T., Owen, A., Parkhill, K.A., Pereira, M., Peyton, J., Pogson, M., Powney, G., Pritchard, N., Pritchard, S., Prochorskaite, A., Prosser, M., Pywell, R., Rawlins, B., Reuland, O., Richards, M., Robinson, D.A., Rorke, S., Rowland, C., Roy, D., Scarlett, P., Scholefield, P., Scott, A, Scott, L., Scott, R., Sharps, K., Siriwardena, G., Smart, S., Smith, G., Smith, P., Stopps, J., Swetnam, R., Taft, H., Taylor, R., Tebbs, E., Thomas, A., Todd-Jones, C., Tordoff, G., Turner, G., Van Breda, J., Vincent, H., Wagner, M., Waters, E., Walker-Springett, K., Wallace, H., Watkins, J., Webb, G., White, J., Whitworth, E., Williams, B., Williams, P., Wood, C. and Wright, S. (2017) Glastir Monitoring & Evaluation Programme. Final Report to Welsh Government. Contract reference: C147/2010/11. NERC/Centre for Ecology & Hydrology (CEH Projects: NEC04780/NEC05371/NEC05782)

Feeney, C., Tye, A.M. & Robinson, D.A. (2025). Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP). ERAMMP Technical Annex105TA1S9: Wales National Trends and Glastir Evaluation, Supplement-9: Soil Erosion. Report to Welsh Government (Contract C208/2021/2022) (UK Centre for Ecology & Hydrology Project 08435)

Feeney, C. J., Robinson, D. A., Keith, A. M., Vigier, A., Bentley, L., Smith, R. P., Garbutt, A., Maskell, L. C., Norton, L., Wood, C. M., Cosby, B. J. and Emmett, B. A. (2023). Development of soil health benchmarks for managed and semi-natural landscapes. Science of the Total Environment, 886, 163973. doi:https://doi.org/10.1016/j.scitotenv.2023.163973

Intermap (2009). NEXTMap Britain digital terrain model [Dataset]. Technologies Inc.

Kuhn, M. (2008). Building Predictive Models in R Using the caret Package. 28. doi:10.18637/jss.v028.i05

Marston, C.; Rowland, C.S.; O'Neil, A.W.; Morton, R.D. (2022). Land Cover Map 2021 (25m rasterised land parcels, GB) [Dataset]. NERC EDS Environmental Information Data Centre. https://doi.org/10.5285/a1f85307-cad7-4e32-a445-84410efdfa70

R Core Team (2024). R: A Language and Environment for Statistical Computing. R Foundation for Statistical Computing, Vienna, Austria. https://www.R-project.org/

Robinson, D.A., Tye, A.M., Feeney, C.J., Fitos, E., Garbutt, R.A., Scarlett, P.M. & Wood, C.M. (2021). Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP). ERAMMP Document-71: Field-Survey Handbook (Procedures) - Soil Erosion and Damage Recording. For Welsh Government (Contract C210/2016/2017)(UK Centre for Ecology & Hydrology Projects 06297 & 06810): Report 71. Field-Survey Handbook (Procedures) - Soil Erosion and Damage Recording_v1.1.pdf

Robinson, D.A., Bentley, L., Jones, L., Feeney, C., Garbutt, A., Tandy, S., Lebron, I., Thomas, A., Reinsch, S., Norton, L., Maskell, L., Wood, C., Henrys, P., Jarvis, S., Smart, S., Keith, A., Seaton, F., Skates, J., Higgins, S., Giuffrè, G., Emmett, B. A. (2024). Five decades' experience of long-term soil monitoring, and key design principles, to assist the EU soil health mission. European Journal of Soil Science, 75(5), e13570. https://doi.org/10.1111/ejss.13570

Robinson, E.L.; Blyth, E.M.; Clark, D.B.; Comyn-Platt, E.; Rudd, A.C. (2020). Climate hydrology and ecology research support system meteorology dataset for Great Britain (1961-2017) [CHESS-met] [Dataset]. NERC Environmental Information Data Centre. https://doi.org/10.5285/2ab15bf0-ad08-415c-ba64-831168be7293

Smart, S.M., Wood, C.M., Maskell, L.C., Norton, L., Bunce, R. & Barr, C. (2023). Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP). ERAMMP Document-49: Field-Survey Handbook (Procedures) - Vegetation Plots. Report to Welsh Government (Contract C210/2016/2017)(UK Centre for Ecology & Hydrology Projects 06297 & 06810): ERAMMP Document-49 Field-Survey Handbook (Procedures) - Vegetation Plots v2.1.0.pdf

Therneau, T., & Atkinson, B. (2023). rpart: Recursive Partitioning and Regression Trees. Retrieved from https://CRAN.R-project.org/package=rpart

Wood, C. M., Bunce, R. G. H., Norton, L. R., Maskell, L. C., Smart, S. M., Scott, W. A., Henrys, P. A., Howard, D. C., Wright, S. M., Brown, M. J., Scott, R. J., Stuart, R. C., & Watkins, J. W. (2018). Ecological landscape elements: Long-term monitoring in Great Britain, the Countryside Survey 1978-2007 and beyond. Earth System Science Data, 10(2), 745–763. https://doi.org/10.5194/essd-10-745-2018

Wood, C. M., Alison, J., Botham, M. S., Burden, A., Edwards, F., Garbutt, R. A., George, P. B. L., Henrys, P. A., Hobson, R., Jarvis, S., Keenan, P., Keith, A. M., Lebron, I., Maskell, L. C., Norton, L. R., Robinson, D. A., Seaton, F. M., Scarlett, P., Siriwardena, G. M., Skates, J., Smart, S. M., Williams, B., and Emmett, B. A. (2021) Integrated ecological monitoring in Wales: the Glastir Monitoring and Evaluation Programme field survey, Earth Syst. Sci. Data, 13, 4155–4173, https://doi.org/10.5194/essd-13-4155-2021 .

# Appendix 1: Surveyor Key to Features

ERAMMP Soil erosion and disturbance key: Use the key in order to determine which category to report in: Start with 1 Peat; if not on peat then determine if there is 2 Soil Disturbance by animals or machines; if not disturbed, is the feature caused by gravity or 3 Mass Movement; if not, then it’s 4 Erosion, usually by water.

| 1) Peat: Are you on peat, if not go to soil disturbance |  |  |
| --- | --- | --- |
| Peat hags | Peat hag | Peat pipes |

| 2) Soil disturbance: Has machinery, human activity, animal activity or tree uprooting caused the disturbance, if not go to mass movement |  |  |
| --- | --- | --- |
| Poaching sheep | Machinery disturbance (not individual tramlines) | Footpath disturbance |
|  | R. Smith EA |  |
| Poaching horses | Gate or machinery disturbance from above | Tree root scars |

| 3) Mass movement: Has the soil moved by gravity or slipped, if not go to erosion |  |  |
| --- | --- | --- |
| Scar or slip | Creep or terracettes | Scree |
| Mass movement | 4) Erosion: usually by water in Wales |  |
| landslide | Sheet erosion | Rills less than 30 cm deep |
|  |  | R. Smith EA |
| Gullies more than 30 cm deep | Tillage erosion, soil thinning | Drainage ditch erosion |
|  |  | R. Smith EA |
| Muddy outwash | Riverbank erosion | Coastal erosion by waves |
| R. Smith EA |  |  |

Acknowledgement: Thanks to Richard Smith at the Environment Agency for the reuse of some of the photos (as marked).

ERAMMP Programme Office

UKCEH Bangor

Environment Centre Wales

Deiniol Road

Bangor, Gwynedd

LL57 2UW

+ 44 (0)1248 374500

erammp@ceh.ac.uk

www.erammp.cymru

www.erammp.wales