Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP)

ERAMMP Supporting Information Metadata for:

Topsoil physico-chemical metrics from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023

Client Ref: Welsh Government / Contract C210/2016/2017

Version 1.0

Date: 16/06/2025

Version History

| Version | By | Date | Changes |
| --- | --- | --- | --- |
| 1.0 | Reinsch, S. | 16/06/2025 |  |

Document Info

| Series | Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP) |
| --- | --- |
| Title | ERAMMP Supporting Information Metadata for: Topsoil physico-chemical metrics from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023 |
| Client | Welsh Government |
| Client reference | C208/2021/2022 |
| Confidentiality, copyright and reproduction | © Crown Copyright 2025 This document is licensed under the Open Government Licence 3.0. |
| UKCEH contact details | Bronwen Williams UK Centre for Ecology & Hydrology (UKCEH) Environment Centre Wales, Deiniol Road, Bangor, Gwynedd, LL57 2UW 01248 374500 erammp@ceh.ac.uk |
| Corresponding author | Sabine Reinsch, UKCEH sabrei@ceh.ac.uk |
| Authors | Reinsch, S.1, Bentley, L.1, Brentegani, M.1, Carter, H.1, Dhiedt, E.1, Ebuele, V.1, Emmett, B.A.1, Hunt, A.1, Keenan, P.1, Keith, A.1, Lebron, I.1, Lucas, J.R.1, Monkman, G.1, Pereira, M.G.1, Richardson-Jones, V.1, Rowe, R.1, Salisbury, E.1, Tandy, S.1, Wood, C.M.1, Ardeshir, D., Biddle, A.1, Boffey, C.2, Ceallaigh, C.1, Cope, S., Fothergill, C., Friend, R., Handley, J., Handley, T., Higgins, A., Horninge, L.3, Moir, A.4, Mullally, S., O’Neil, A.1, Savage, C.1, Scott, H., Shepherd, S.5, Ward, T.6, Winder, Jan, Winder, Jon, Robinson, D.A.1 |
| Contributing authors & reviewers | 1UK Centre for Ecology & Hydrology (UKCEH), 2Penny Anderson, 3National Trust for Scotland, 4British Geological Survey (BGS), 5ADAS, 6Severn Rivers Trust |
| How to cite (long) | Reinsch, S., Bentley, L., Brentegani, M., Carter, H., Dhiedt, E., Ebuele, V., Emmett, B.A., Hunt, A., Keenan, P., Keith, A., Lebron, I., Lucas, J.R., Monkman, G., Pereira, M.G., Richardson-Jones, V., Rowe, R., Salisbury, E., Tandy, S., Wood, C.M., Ardeshir, D., Biddle, A., Boffey, C., Ceallaigh, C., Cope, S., Fothergill, C., Friend, R., Handley, J., Handley, T., Higgins, A., Horninge, L., Moir, A., Mullally, S., O’Neil, A., Savage, C., Scott, H., Shepherd, S., Ward, T., Winder, Jan, Winder, Jon, Robinson, D.A. (2025). ERAMMP Supporting Information Metadata for: Topsoil physico-chemical metrics from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023 (Report-112). UKCEH |
| How to cite (short) | Reinsch, S. et al. (2025). ERAMMP Supporting Information Metadata for: Topsoil physico-chemical metrics from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023 (Report-112). UKCEH |
| Approved by | Bridget Emmett (UKCEH) Oliver Massey (Welsh Government) |

Abbreviations

| AES | agri-environment schemes |
| --- | --- |
| CS | Countryside Survey |
| DIW | deionised water |
| EC | electrical conductivity |
| ERAMMP | Environment and Rural Affairs Monitoring & Modelling Programme |
| ESA | Environmentally Sensitive Areas |
| EU | European Union |
| GB | Great Britain |
| GMEP | Glastir Monitoring and Evaluation Programme |
| LCM | Land cover map (followed by the year) |
| LMS | land management scheme |
| LOI | Loss-on-Ignition |
| NFS | National Field Survey |
| PD | particle density |
| QA | quality assurance |
| SOC | soil organic carbon |
| TG | Targeted |
| WW | Wider Wales |

Contents

1.	Overview	5

2.	Background Information	5

3.	Introduction to the GMEP Survey Design and its re-survey by ERAMMP	6

4.	Sample Collection Methods	7

5.	Laboratory Protocols and Analytical Methods	10

5.1	Soil sampling locations and general information	10

5.2	Land Class	10

5.3	Co-located soil and botanical information	10

5.4	UKCEH-CS soil carbon groups	11

5.5	Measured and derived soil metrics	11

5.6	Soil pH in deionized water and pH in calcium chloride	12

5.7	Soil solution Electrical Conductivity (EC)	12

5.8	Soil organic matter, Loss-on-Ignition (LOI) and derived carbon concentration, and CaCO3	12

5.9	Soil bulk density, stones & porosity of fine earth	13

5.10	Soil Olsen-phosphorous	14

5.11	Total Phosphorus in soil	15

5.12	Total soil organic carbon (SOC) and total nitrogen	15

6.	Data Quality Assurance	16

7.	Notes on ERAMMP soil data analysis	18

8.	Data Structure	19

9.	References	21

# Overview

| Spatial Extent | Terrestrial extent of Wales, UK |
| --- | --- |
| Temporal Extent | 2021–2023 |
| Summary | This dataset includes a range of physico-chemical properties measured from topsoil (0-15 cm) within a wide range of land use types across Wales, collected as part of the Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP) National Field Survey (NFS) between 2021 and 2023. The properties included are: soil organic matter (loss on ignition (LOI)), derived carbon concentration, total soil organic carbon (SOC), nitrogen, total soil phosphorous, Olsen-phosphorous (within improved land only), pH, electrical conductivity, soil bulk density of fine earth, fine earth volumetric water content, gravimetric water content, and soil group. ERAMMP is a resurvey of all the locations from the initial monitoring program (300 1km squares) on a rolling annual basis, with the aim to resurvey all sites between 2021 and 2025. Between 2021-2023 the ERAMMP National Field Survey resurveyed 224 of those 300 for soil monitoring. The initial monitoring program, GMEP, was set up by the Welsh Government in 2013 to report national trends and monitor the effects of the Glastir agri-environment scheme on the environment and ran from 2013 to 2016. The field survey element was based on a stratified random sampling design of 300 x 1km square sites across Wales and was managed by the UK Centre for Ecology & Hydrology (UKCEH). Many other variables are captured in the same locations in addition to the soil variables reported here. |
| File Name(s) | ERAMMP_SOIL_METRICS_2021_23.csv |
| File Format(s) | .csv |

# Background Information

In Wales, funding from agri-environment schemes (AES) has been available since the early 1990s including Environmentally Sensitive Areas (ESAs), the Habitat Scheme, the Woodland Grant Scheme, the Organic Farming Scheme, Tir Cymen, Tir Cynnal, Tir Gofal, followed by the Glastir scheme. A monitoring programme was set up by the Welsh Government in 2013 to monitor the national state and change of the countryside, along with effects of the Glastir scheme on the environment. This four-year rolling monitoring programme was called the “Glastir Monitoring and Evaluation Programme” (GMEP) and was conducted between 2013 and 2016. GMEP aimed to: (1) Evaluate the environmental effects of the Glastir sustainable land management scheme (LMS) (Welsh Government, 2020); (2) Quantify environmental status and spatiotemporal environmental status trends across the Welsh countryside and; (3) Assess and predict outcomes for climate change mitigation, biodiversity, soil and water quality, woodland expansion and cultural landscapes (https://gmep.wales). The project was led by UKCEH. A major component of the work to deliver the first 2 aims was a National Field Survey (NFS) which was an expansion of a long-term integrated monitoring programme developed by UKCEH since the 1970s called Countryside Survey (Carey et al., 2008, Wood et al., 2018, Robinson et al., 2024).

Synthesis and analysis of the data derived from the NFS sought to identify how the Welsh environment is being impacted by drivers of change, such as land use, climate, and pollution, over and above Glastir interventions. The GMEP report was published in 2017 (Emmett et al. 2017) and data are summarized in Wood et al. 2021. Data are available from the Environmental Information Data Centre (https://eidc.ac.uk/) as part of the data series of the Glastir Monitoring and Evaluation Programme, Wales, 2013-2016.

The ’Environment and Rural Affairs Monitoring & Modelling Programme’ (ERAMMP) National Field Survey is the re-survey of GMEP locations. Data informs national trends analyses, and the effects of Glastir action bundles. Other elements of GMEP and ERAMMP including numerous expert reviews and integrated modelling is reported elsewhere (https://erammp.wales/).

For soils, the national field survey (NFS) collected evidence to quantify national trends in soil quality and health, as well as reporting to Welsh Government on the impact of Glastir, fulfilling some of Wales reporting obligations to the European Union. The aim of the Glastir monitoring of the soil component was to establish a baseline for assessing the effectiveness of Glastir action bundles of management interventions in helping deliver improved soil health. This would help address the outcomes of interest related to climate change, biodiversity, soil and water quality and woodland expansion.

# Introduction to the GMEP Survey Design and its re-survey by ERAMMP

In the Glastir Monitoring and Evaluation Programme, the NFS was designed to capture multiple measures and metrics, and to integrate across these metrics. Hence a full ecosystem-based approach is required where data operating at multiple scales are captured, where possible, at the same time in a single snap-shot visit.

For GMEP, a 4-year rolling survey was adopted to maximise the number of sites visited across the national spatial scale whilst at the same time monitoring year-on-year, such that spatial variation and temporal changes and trends can be detected cost-effectively (Emmett et al., 2013 & Wood et al. 2021). The first survey cycle dates from 2013-2016. The first partial re-survey cycle took place between 2021 and 2023, after being postponed from 2020 due to the COVID pandemic.

The rolling programme-based survey has two key components: the Wider Wales Component used for baseline estimation (Welsh national trends) and national reporting of Glastir action bundles (Targeted Component) which specifically links to the 9 objectives (of which 6 environmental outcomes are intended to be captured in the NFS) of the Glastir scheme. Across both components, integration of survey data is essential and therefore a common spatial unit of 1 km x 1 km square was adopted.

The selection of the sample of 1-km x 1-km squares for monitoring was accomplished following the same procedure as used for the Countryside Survey (CS) of Great Britain (GB) (as described below), which aims to provide robust estimates of indicators at national level across GB and constituent countries. Using the same methodology builds on previous knowledge and enabled integration of the two surveys in future analyses.

A total of 300 x 1 km2 squares were sampled over the 4-year GMEP NFS cycle. Half of these squares (n=150) were the Wider Wales squares, randomly sampled within strata defined according to the Land Classification of Great Britain (Bunce et al., 2007) – a derived classification of the landscape based on its topography, geology, climate and physical attributes. Environmental heterogeneity is minimized within each stratum of the Land Classification and is maximised between strata. The proportion of the 1-km x 1-km squares randomly sampled from within each stratum was proportional to the size of the stratum to allocate survey efforts best. The other half of the sampled squares were targeted specifically at Glastir priority areas (n=150). Glastir-targeted squares were randomly selected with a selection probability proportional to the ‘Glastir score’ of the square, derived from the Welsh Government’s scoring system of the Glastir Advanced target objective areas (Emmett et al., 2014). Any square randomly selected that contained more than 75% of urban land or that was more than 90% sea (defined by LCM2007 and the UK Census mean high tide data) was excluded.

The ERAMMP NFS re-survey for soils covered 129 of the original 300 squares in 2021-22, and an additional 95 survey squares in 2023. The 129 squares in 2021-22 were selected from both the Wider Wales and Glastir-targeted squares to enable the evaluation of multiple Glastir interventions based on multiple environmental outcomes; the selection of these squares does not enable a robust evaluation of national trends without more Wider Wales squares being surveyed (Alison et al. 2021). Therefore, the 95 survey squares in 2023 were the remaining Wider Wales squares that had not been surveyed. A total of 224 squares were re-surveyed in ERAMMP between 2021 and 2023: 148 Wider Wales squares and 78 Glastir-targeted squares (Figure 1).

| GMEP (2013-2016) | ERAMMP (2021-2023) – botanical locations |
| --- | --- |

Figure 1: Maps showing the distribution of 1-km x 1-km National Field Survey squares across Wales (plotted at 10 km to ensure data confidentiality). ERAMMP map shows all NFS squares surveyed in 2021-2023; the colour key indicates survey years for the botanical survey. Soils were sampled between 2021 and 2022 from across the same squares.

# Sample Collection Methods

Within each of the 300 x 1-km2 NFS squares, a set of soil samples are taken from up to 5 pre-determined, randomly dispersed locations in conjunction with the botanical survey. The soil sample analysed for soil metrics is taken using a black plastic core (C-core), 15 cm long. After collection, the soil cores are refrigerated and stored until posted, usually within a couple of days, to laboratories at the UK Centre for Ecology & Hydrology, Bangor and Lancaster. For the full soil sampling protocol see Robinson et al. (2023). An overview of numbers of squares visited, and samples taken is shown in Table 1.

In GMEP all soil samples were collected simultaneously with vegetation surveys. However, in ERAMMP, soil samples taken in 2021 were collected by either the botanical survey team, mappers, or soil surveyors because of logistics. As a result, soil and vegetation data are not always co-located for 124 re-surveyed squares at the same time because of changes in land access permissions between survey team visits. However, robust plot-relocation methodology allows confident co-location of samples even if not sampled at the same time. In 2023, all soil cores were collected by the botanical team, directly co-located with the vegetation plots similar to GMEP. The ERAMMP sample flow for different analyses is shown in Figure 2.

Table 1. Overview of soil sampling information: NFS year and project; GMEP: Glastir Monitoring and Evaluation Programme (baseline survey of the Welsh landscape), ERAMMP: Environment and Rural Affairs Monitoring & Modelling Programme (first re-survey of the Welsh landscape); Sample corner:  Location of sampling point in relation to 2 m x 2 m vegetation quadrat each year; Number of squares visited each year and number of soil samples analysed.

| YEAR | PROJECT | SAMPLE CORNER | NUMBER OF SQUARES VISITED | NUMBER OF SOIL SAMPLES ANALYSED |
| --- | --- | --- | --- | --- |
| 2013 | GMEP year 1 | 15 cm to South | 60 | 300 |
| 2014 | GMEP year 2 | 15 cm to South | 90 | 450 |
| 2015 | GMEP year 3 | 15 cm to South | 75 | 375 |
| 2016 | GMEP year 4 | 15 cm to South | 75 | 376 |
| 2021 | ERAMMP year 1 | 15 cm to West | 124 | 530 |
| 2022 | ERAMMP year 2 | 15 cm to West | 5 | 20 |
| 2023 | ERAMMP year 3 | 15 cm to West | 95 | 357 |

Figure 2: ERAMMP soil core flow chart. Chemical cores are used for soil metric collection.

# Laboratory Protocols and Analytical Methods

## Soil sampling locations and general information

SQ_ID; WEB_ID; SQ_TYPE; PLOT_ID; ID; EASTING_10km; NORTHING_10km; COUNTRY; YEAR

ERAMMP survey locations are identified using 1-km x 1-km square IDs (SQ_ID, WEB_ID comparable to previous survey). GMEP squares were divided in two square types (SQ_TYPE) to analyse the Welsh national trend (WW = Wider Wales squares) or the impact of the Glastir agri-environment scheme (TG = Targeted).

Note: Care should be taken using the data for national trend (WW) analyses. The WW component only is a nationally representative sample, whereas the Glastir targeted (TG) sites represent a biased sample of Wales.

Within each survey square, up to five locations are surveyed for soils (PLOT_ID). If plots cannot sufficiently be re-located, another PLOT_ID may be created. Location information generalised to a 10 km grid to conform with confidentiality agreements (EASTING_10km; NORTHING_10km). A unique plot identifier, ID, is created from SQ_ID and PLOT_ID. COUNTRY is Wales (WAL) by default. YEAR defines the year of soil sampling.

## Land Class

LC07

ITE Land Class 2007 of Great Britain was used as the basis for stratifying the GMEP survey. Land classes are based on the major environmental gradients (climate, relief, geology) across the countryside (Bunce et al. 2007).

Table 2. ITE Land Classes (2007) for Wales as described in Bunce et al. 2007, LC07_NUM is the numeric equivalent to the 2007 Land Class description (LC07).

| LC07_NUM | COUNTRY | DESCRIPTION (LC07) |
| --- | --- | --- |
| 17w1 | Wales | Low mountain ridges / valley slopes, N Wales |
| 17w2 | Wales | Rounded mountains / scarps / upper valleys, mid / S Wales |
| 17w3 | Wales | Variable landforms of hills / low mountain, Wales |
| 5w | Wales | Shallow slopes / flood plains, Wales |
| 6w | Wales | Complex valley systems / table lands, Wales |
| 7w | Wales | Sea cliffs / hard coast, Wales |
| 15w | Wales | Flat river valleys / lower hill slopes, Wales |
| 18w | Wales | Upland valley sides / low mountains, Wales |

## Co-located soil and botanical information

PLOT_TYPE; BROAD_HABITAT_DESC

Soil samples are taken at vegetation X-Plots (Smart et al. 2023). Botanical Broad Habitats (as in Smart et al. 2023) are determined at the plot scale within a 14.14 m x 14.14 m (~200 m2) which are part of the botanical dataset. PLOT_TYPE = X by default. The main Broad Habitat assessment for soils is made in the wider polygon the X-plot falls into (BROAD_HABITAT_DESC).

## UKCEH-CS soil carbon groups

SOIL_LOI_CS_GROUP

UKCEH-CS (CS stands for Countryside Survey) grouped soils according to levels of soil organic matter determined by loss on ignition presented in the table below:

Table 3: Soil Loss-on-Ignition groups (1-4).

| Soil_LOI_CS_GROUP | LOI CATEGORY | Soil_GP_LOI_CS_1_4 |
| --- | --- | --- |
| Mineral | 0- >8% LOI (0-44 g C kg-1) | 1 |
| Humus mineral | 8 – >30% LOI (44-165 g C kg-1) | 2 |
| Organo-mineral | 30 – >60% LOI (165-330 g C kg-1) | 3 |
| Organic | 60 – 100% LOI (>330 g C kg-1) | 4 |

## Measured and derived soil metrics

One chemical (C-core), one physical (P-core), and one biological (B-core) core were collected per square (Fig. 2). These are sent to UKCEH laboratories. Cores are logged into the laboratory system. B-cores are frozen at -20oC for potential future analyses in the UKCEH Soil Bank. C-cores are the main soil cores processed to derive the metrics detailed below. Physical cores may be processed if there is not enough material from C-cores to determine all metrics. Measurements may be carried out on:

- Bulk soil (B) on C-cores with the notation: C_B_metric

- Fine Earth (FE) on C-cores with the notation: C_FE_metric

Basic measurements carried out in the labs include actual soil core length by measuring the indents at the top and the bottom of the soil core. The calculation considers the total possible length of the soil core of 15 cm:

C_B_SOILCORE_LENGTHCM_CS2007 (cm) =

15 + INDENT_TOP + INDENT_BOTTOM

The associated soil volume including stones is then calculated as:

C_B_SOILCORE_VOLUMECM3_CS2007 =

pi*2.5^2*C_B_SOILCORE_LENGTHCM_CS2007

The volume of the fine earth fraction without the volume of stones is calculated as below where BS_St is the mass of stones in the soil core assuming a particle density of stones of 2.65 g cm-3:

BS_St_Volume = BS_St / 2.65

BS_FE_Volume_CS2007 =

C_B_SOILCORE_VOLUMECM3_CS2007 - BS_St_Volume

Total soil (dry) mass is corrected for the fresh sub-sample taken for pH and EC measurements. Resulting reporting metrics C_FE_BULK_DENSITYGPERCM3 (g cm-3), see below.

## Soil pH in deionized water and pH in calcium chloride

C_B_PH_DIW; C_B_PH_CACL2

Soil pH measurements are carried out on a suspension of fresh field-moist bulk soil in deionised water and in 0.01 M CaCl2. The ratio of soil to water or CaCl2 is 1:2.5 by weight. This method is the one employed by the Soil Survey of England and Wales (Avery and Bascomb, 1974).

Quality Control

Two different internal standards of known pH values were included in each batch. Readings were acceptable only when the measured pH for the internal standards varied within ±2 standard deviations of the mean value. We included in each batch a suitable number of duplicated samples (about 10% of repetitions).

## Soil solution Electrical Conductivity (EC)

C_B_EC

We weight 10 g of field moist bulk soil in a beaker and add 25 mL of deionised water (DIW), stirred with a rod to produce a homogeneous suspension. Wait for half an hour, stir again with the rod, and measure the EC using an electrode and a conductivity meter (Jenway 4510).

Quality Control

Two different internal standards were included in each sample batch.  Readings were acceptable only when the measured pH for the internal standards varied within ±2 standard deviations of the mean value. We included in each batch a suitable number of duplicated samples (about 10% of repetitions).

## Soil organic matter, Loss-on-Ignition (LOI) and derived carbon concentration, and CaCO3

C_FE_LOI; C_FE_C_CONCLOI_GCPERKG; C_FE_CACO3

Hygroscopic water content, LOI and CaCO3 was measured on approximated 10 g air dried sub-sample taken after sieving to 2 mm. Samples were loaded into a ThermoGravimetric Analyser (LECO TGA701) and heated from 25°C to 1000°C in four steps. The first step is when the furnace reaches 105°C, at that point the temperature is kept constant for 3 h. The soil weight loss of the sample at that temperature compared to 25oC gives the hygroscopic water content (not reported here).

In the second step, samples are heated to 375°C and sustained at a constant temperature for a total of 15.45 h, the weight loss between 105°C and 375°C gives the Soil organic matter content as Loss-on-Ignition (LOI), associated fractions and particle density (PD). Calculating the particle density of a sample assumes a particle density of 1.4 g cm-3 for organic matter, and 2.7 g cm-3 for mineral (Ruehlmann (2020), Ruehlmann and Körschens (2020)):

C_FE_LOI (g per 100 g, %) = 100*(TGA_105-TGA_375)/(TGA_105)

In the third step samples were heated to 650°C and kept constant for 2 h, the weight loss between 375°C and 650°C is related to decomposition of clay mineral and other losses. In the final step samples were heated to 1000°C and kept constant for another 2 h, the weight loss between 650°C and 1000°C gives the CaCO3 content of the sample. The TGA instrument records the weight of the sample during the entire process and we exported at the end of the run the weight values for the sample at the beginning (initial soil weight), at 105°C after the 3.3 h, at 375°C at the end of the 15.45 h, at 650°C at the end of the 2 h, and at 1000C at the end of the run. The hygroscopic water content and LOI are calculated according to Emmett et al. (2010) (see above) and calcite is calculated according to the formula:

TEMP_Calcite (g per 100 g, (%)) = 100*(TGA_650 - TGA_1000)/(TGA_105)*2.27

The factor 2.27 is obtained from the chemical equation for the decomposition of CaCO3 with temperature:

CaCO3                 CaO + CO2

Where the weight loss of CaO3 is the molecular weight of the CaCO3 (100.0869) divided by the molecular weight of CO2 (44.01).

The CaCO3 concentration in each sample was corrected for variable CaCO3 recovery (95-100%, also see Quality control section below):

C_FE_CACO3 (g CaCO3 per 100g oven dry soil (%)) =

(TEMP_Calcite*100)/TGA_STANDARD

Soil carbon concentration in g C per 100 g soil of soil is derived from the LOI measurement and is determined by:

C_FE_C_CONCLOI_GCPER100G (g C per 100 g soil (%)) = 0.55*C_FE_LOI

The reason this is done is to have data that is compatible with legacy data from other surveys like Countryside Survey that emphasised the measurement of LOI. The algorithm used for the conversion from g C per 100g soil to g of C / kg soil and tonnes of C per hectare was the one used in 2007:

C_FE_C_CONCLOI_GCPERKG (g C per kg) =

C_FE_C_CONCLOI_GCPER100G * 10

Quality Control

Hygroscopic water content, LOI and calcite quality control checks were carried out using internal soil standards prepared in an identical manner to the sampled soils, also a sample of pure calcite was run with each run. Two different internal standards were included in each sample batch.  Batches were repeated for which the measured LOI for the internal standards varied by more than 2 standard deviations in either direction from the mean value generated historically for the internal standards. In addition, CaCO3 recovery was within 95-100%, if less than 95% it was repeated.

## Soil bulk density, stones & porosity of fine earth

C_FE_BULK_DENSITYGPERCM3

The bulk density of soil depends greatly on the mineral make up of soil and the degree of compaction. The particle density of quartz is around 2.65 g cm-3 but the (dry) bulk density of a mineral soil is normally about half that density, between 1.0 and 1.6 g cm-3. Soils high in organics and some friable clay may have a bulk density well below 1 g cm-3. Bulk density is a measure of the amount of soil per unit volume. It is therefore an excellent measure of available pore space in a soil and gives information on the physical status of the soil. Bulk density values are also essential when estimating soil carbon stocks, as they allow a conversion from %C to carbon per unit volume. Bulk density is determined from the C-core, which is 15 cm long with a radius of 2.55 cm.

The mass and volume of stones, C_FE_MASSSTONES (g) and C_FE_VOLSTONES (cm3), are derived from the total mass of stones in the measured subsample, and the potential mass of stones removed with bulk soil used for pH and EC measurements.

Dry bulk density is calculated using the following equation:

C_FE_BULK_DENSITYGPERCM3 (dry, g cm-3) =

(Dry weight core (105°C) (g) - stone weight (g))

(Core volume (cm-3) - stone volume (cm-3)

Fine earth volumetric and gravimetric water content when sampled

C_FE_VWC; C_FE_GWC_D; C_FE_GWC_W

Once the bulk density was calculated the volumetric water content of the fine earth fraction can be determined by multiplying the bulk density and the gravimetric water content of the fine earth:

C_FE_VWC (m3 m-3) = C_FE_GWC*C_FE_BULK_DENSITYGPERCM3

The gravimetric water content was calculated according to dry soil (D) and wet soil (W) with BS_W = free water in bulk soil, BS_BW = bound water in bulk soil, BS_FE = Fine earth part of the soil without bound water.

C_FE_GWC_D (g water per g oven dry soil) =

(BS_W + BS_BW) / BS_FE

C_FE_GWC_W (g water per g wet soil) =

(BS_W + BS_BW) / (BS_FE + (BS_W + BS_BW))

Quality Control

Quality control is achieved by using fix volume pre-cut sleeves for soil sampling and extensive training for soil surveyors.

## Soil Olsen-phosphorous

C_FE_POLSEN

Olsen-P was measured for improved land (arable and improved grassland) only (C_FE_POLSEN). Briefly, 2 g of air- dried soil samples were extracted in 40 mL Olsen’s reagent (0.5 M NaHCO3 at pH 8.5) for 30 min in a mechanical end-over-end shaker. The sample was then filtered through a Whatman 44 filter paper to separate the soil and the filtrate; the filtrate was kept for analysis.

The analysis was performed on a Seal Analytical AA3 segmented flow analyser. For this type of analysis samples are mixed in the flow channel with acidic ammonium molybdate and potassium antimony tartrate to form a complex with phosphate. This complex is reduced with ascorbic acid to develop a molybdenum blue colour. The reaction is temperature controlled to 40°C using a water bath to ensure uniform colour development. The developed colour is measured at 880 nm.

Olsen-P concentrations (mg kg-1) are calculated using a calibration curve, the mean of two extraction blanks is used to correct the data set and corrected for moisture content. The calibration range of this method is 0-5 mg L-1 phosphorus.

Quality Control

Two quality control reference samples, a duplicate sample and two blanks are run every 25 samples to ensure data quality. The final concentration is expressed mg kg-1 and it has been corrected for moisture content, the concentration of the blank and using a calibration curve of the standard.

## Total Phosphorus in soil

C_FE_PTOTAL

Total soil phosphorus (C_FE_PTOTAL) was measured by a H2O2/H2SO4 digestion followed by colorimetric analysis carried out on a Seal AQ2 discrete analyser. Briefly, ~0.4 g of air dried and ground (to 2 mm) soil samples were weighed into boiling tubes and then digested with digestion matrix. The digestion matrix contained sulphuric acid, hydrogen peroxide, selenium powder and lithium sulphate. This was repeated for all samples including reference samples and blanks. The tubes were then placed in a block digester and the temperature stepped to 250°C, held for 15 min to allow the mixture to stabilise and then further stepped to 400°C where the temperature was maintained for 2 h to complete the digestion.

Once digested, the samples and references were allowed to cool, then diluted to a total volume of 50 mL using ultrapure water and mixed. After allowing the silica residue to settle overnight the supernatant was collected and diluted a further 5 times and then measured calorimetrically using a SEAL AQ2 discrete analyser. Phosphorus concentrations were determined by reaction with acidic molybdate in the presence of antimony to form an antimony-phospho-molybdate complex. Ascorbic acid reduced this to the intensely blue phosphomolybdenum complex, measured spectrophotometrically at 880 nm.

Quality Control

Calibrations were run using standards, prepared from blank matrix digested in the same manner as the samples, using the range 0-2 mg L-1, with control standards analysed every ten samples to check precision and reproducibility throughout the run. Blanks and reference samples to assess the chemistry and the digest were run with each batch. The data is corrected for moisture content and reported in mg kg-1 phosphorus.

## Total soil organic carbon (SOC) and total nitrogen

C_FE_CTOTAL; C_FE_NTOTAL

Soil organic carbon develops soil structure, retains nutrients and water, whilst soils act as the major terrestrial carbon store buffering change in climate. The importance of soils to carbon mitigation is demonstrated by the calculation that a 1% loss of carbon from UK soils would be equivalent to the UK‘s annual fossil fuel emissions (PB13297, 2009).

Total soil organic carbon (SOC = C_FE_CTOTAL) and total nitrogen (C_FE_NTOTAL) are measured using the UKAS accredited method SOP3102, in UKCEH Lancaster. Samples were analysed using an Elementar Vario-EL elemental analyser (Elementaranalysensysteme GmbH, Hanau, Germany). The Vario EL is a fully automated analytical instrument working on the principle of oxidative combustion followed by thermal conductivity detection. Following combustion in the presence of excess oxygen the oxides of nitrogen and carbon flow through a reduction column which removes excess oxygen. Carbon is trapped on a column whilst nitrogen is carried to a detector. Carbon is then released from the trap and detected separately.

This analysis was carried out on ball milled soil samples. Samples were oven dried at 105°C (± 5°C) for a minimum of 3 h, cooled and sealed prior to weighing. The sample was weighed into a tin cup (10-20 mg ball milled sample for soil) on a 6-place micro-balance. The results are expressed in % notation.

Quality Control

Quality control is achieved by use of two in-house reference materials analysed with each batch of ~20 samples. The instrument’s calibration was checked on use using a working standard (Acetanilide) with a concentration of 71.1% total carbon and 10.4 % total nitrogen, and the sample data corrected (factored) against this value. Two of these standards were analysed at the beginning of every run, with every ten samples and again at the end of the run.

# Data Quality Assurance

Soil data collection and processing for UKCEHs large-scale field surveys has undergone major scrutiny. With data being collected as part of a rolling survey and thus, are spread over multiple years, a quality assurance (QA) pipeline was developed. This allows soils data QA in a standardised and timely manner. Version-controlled R-scripts and guidance documents were designed for laboratory staff and data analysts, covering different stages of the QA pipeline (see Figure 3):

- Lab data QA: both laboratories provided visually QA’ed data to the data analyst. For the 2023 dataset, Bangor labs compiled all lab data and run these through a primary QA pipeline. This step flags unexpected values, data gaps, deviations from expected relationships between parameters and reports on the stability of measurement standards. Results flagged in this process were investigated, corrected if appropriate, and commented on, before data were made available to the analyst team.

- Analyst data QA1: data provided by the lab were crossed checked with laboratory measurement records; queries and changes to the original dataset were documented. The final datasets for 2021/22 and 2023 were the input files for calculating derived soil metrics in step 3.

- Soil derived metrics: This step involves two R-scripts;

  - the “calculate_soil_derived_metrics_function.R” and

  - “Derive soil data.R”.

Where a) defines all soil metric calculations, b) is used to derive the soil metrics by reading in lab QA’ed soils data and applying the function to it. This helps protect the calculations from accidental changes through repeated use and provides a transparent record of the calculations. In addition, supporting metrics from other components of the field survey (Habitat and vegetation data) are joined to the soils dataset to facilitate subsequent QA steps.

- Analyst data QA2: An automated data analysis report (.HTML) is produced in the same form as in step 1, the Lab data QA, using the derived soil metrics output file. This data QA report examines patterns in derived metrics and explores expected relationships. The report also uses past data as point of comparison to identify potential outliers. At this stage, data analysists closely work with the labs to understand any outstanding data queries arising.

Throughout this workflow, any changes or updates made to the dataset are recorded against the data record and the QA step that necessitated the change, to ensure data processing is transparent and auditable.

Figure 3. Data QA pipeline. For the 2021/22 ERAMMP soils dataset, lab QA was not automated, and the main data QA was performed as part of the analyst QA step.

# Notes on ERAMMP soil data analysis

- GMEP and ERAMMP data can be combined to create a time series for the investigation of national trends and Glastir impact on soils. The reporting of national trends and Glastir impacts used the glmmTMB R package for these analyses.

- To assess the national trend, only soil samples originating from the Wider Wales square data population should be analysed as these are nationally representative.

- To assess Glastir impacts on soil metrices, data from Wider Wales and Glastir targeted squares should be analysed together and in conjunction with associated data on Glastir management, as both sets of squares contain sites with Glastir management option uptake and no Glastir uptake. However, information on the locations of Glastir uptake is not included in this dataset and must be requested from Welsh Government (data@gov.wales).

# Data Structure

For the 224 squares surveyed between 2021 and 2023 a total of 907 soil samples were taken. The dataset contains 25 columns and 907 rows. In the dataset, values of -9999 denote missing samples.

| COLUMN NAME | TYPE | UNIT | DESCRIPTION |
| --- | --- | --- | --- |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir targeted (TG) to be used only in conjunction with associated Glastir data |
| SQ_ID | Text | None | 1-km survey square - unique identifier |
| PLOT_TYPE | Text | None | Type of sampling plot (X = main vegetation plot) |
| PLOTNUM | Number | None | Plot number within 1km survey square |
| REPEAT_PLOT_ID | Text | None | Unique identifier of plot location across GMEP and ERAMMP surveys (and future surveys) |
| EASTING_10km | Number | Metre | Easting, BNG, plot location to nearest 10km grid square |
| NORTHING_10km | Number | Metre | Northing, BNG, plot location to nearest 10km grid square |
| LC07 | Text | None | ITE Land Classification code - see Bunce et al. (2007) |
| SURVEY_YEAR | Number | None | Years of survey: 2021 or 2022 |
| BROAD_HABITAT_DESC | Text | None | The broad habitat (see Smart et al., 2023) of the area surrounding the plot, where the broad habitat covers at least 400m2. This is compatible with previous habitat mapping data, as 400m2 was the minimum mappable area used historic field habitat mapping for the full 1-km square |
| C_FE_CTOTAL | Number | g C / 100 g of oven dry soil; % | Total soil carbon (0-15 cm) |
| C_FE_LOI | Number | g LOI / 100 g of oven dry soil; % | Soil organic matter content of fine earth (FE) as measured by Loss-on-Ignition |
| C_FE_CACO3 | Number | g CaCO3 / 100 g oven dry soil; % | Calcium carbonate content was derived from TGA measurements between 650oC and 1000oC; see method description for more details |
| C_FE_NTOTAL | Number | g N / 100 g of oven dry soil; % | Total soil nitrogen content of fine earth (FE, 0-15 cm) |
| C_FE_PTOTAL | Number | mg P / kg of oven dry soil | Total soil phosphorus content of fine earth (FE, 0-15 cm) |
| C_FE_POLSEN | Number | mg P / kg of oven dry soil | Olsen-phosphorus of fine earth (FE, 0-15 cm); for arable and improved grassland soils only |
| C_B_PH_DIW | Number | None | Soil pH in water measured on field-moist bulk soil (B) |
| C_B_PH_CACL2 | Number | None | Soil pH in Calcium Chloride (CaCl2) measured on field-moist bulk soil (B) |
| C_B_EC | Number | µS/cm at 25oC | Solution electrical conductivity measured on field-moist bulk soil (B) |
| C_FE_BULK_DENSITYGPERCM3 | Number | g soil / cm3 | Soil bulk density (0-15 cm) of fine earth (FE, excluding stones) |
| C_FE_C_CONCLOI_GCPERKG | Number | g C / kg of oven dry soil | Soil carbon concentration of fine earth (FE, 0-15 cm) |
| C_FE_VWC | Number | m3 / m3 | Volumetric water content of the fine earth fraction of soil (0-15 cm) |
| C_FE_GWC_D | Number | g water/g of oven dry soil | Gravimetric water content of the fine earth fraction per dry weight of soil |
| C_FE_GWC_W | Number | g water/g of field moist soil | Gravimetric water content of the fine earth fraction per wet weight of soil |
| SOIL_LOI_CS_GROUP | Category | None | UKCEH-Countryside Survey Carbon groups used in 2007 based on the levels of LOI |

# References

Alison, J., Bowgen, K., Siriwardena, G., Williams, B., Reinsch, S., Garbutt, A., Emmett, B.A. (2021) Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP). ERAMMP Report-58: ERAMMP Square Selection Protocol 2021/2022. Report to Welsh Government (Contract C210/2016/2017) (UK Centre for Ecology & Hydrology Projects 06297 & 06810):

https://erammp.wales/sites/default/files/2022-11/58%20ERAMMP%20Report-58%20Square58%20%20Selection%20Protocol%202021%20v1.0_en.pdf

Avery, B.W. and Bascomb, C.L.  1974.  Soil Survey Laboratory Methods.  Soil Survey Technical Monograph No. 6, Harpenden.

Bunce, R. G. H., Barr, C. J., Clarke, R. T., Howard, D. C., Scott, W. A. (2007) ITE Land Classification of Great Britain 2007. NERC-Environmental Information Data Centre doi: 10.5285/5f0605e4-aa2a-48ab-b47c-bf5510823e8f

Carey, P. D.; Wallis, S.; Chamberlain, P. M.; Cooper, A.; Emmett, B. A.; Maskell, L. C.; McCann, T.; Murphy, J.; Norton, L. R.; Reynolds, B.; Scott, W. A.; Simpson, I. C.; Smart, S. M.; Ullyett, J. M. (2008). Countryside Survey: UK Results from 2007 NERC/Centre for Ecology & Hydrology (CEH Project Number: C03259): https://nora.nerc.ac.uk/id/eprint/5191/1/N005191CR%20UK%20Results.pdf

Environmental Information Data Centre – Data Series for: Glastir Monitoring and Evaluation Programme, Wales, 2013-2016: https://catalogue.ceh.ac.uk/documents/5563266c-5bcf-4d1a-87d6-3e3ca72dc8a1 The data series holds three soils datasets:

- Robinson, D.A.; Astbury, S.; Barrett, G.; Burden, A.; Carter, H.; Emmett, B.A.; Garbutt, A.; Giampieri, C.; Hall, J.; Henrys, P.; Hughes, S.; Hunt, A.; Jarvis, S.; Jones, D.L.; Keenan, P.; Lebron, I.; Nunez, D.; Owen, A.; Patel, M.; Pereira, M.G.; Seaton, F.; Sharps, K.; Tanna, B.; Thompson, N.; Williams, B.; Wood, C.M. (2019). Topsoil physico-chemical properties from the Glastir Monitoring and Evaluation Programme, Wales 2013-2016. NERC Environmental Information Data Centre. https://doi.org/10.5285/0fa51dc6-1537-4ad6-9d06-e476c137ed09

- Keith, A.M.; Astbury, S.; Dodd, B.A.; Emmett, B.A.; Fitton, A.; Garbutt, A.; George, P.; Marshall, R.; Robinson, D.A.; Williams, B.; Wood, C.M. (2019). Topsoil meso-fauna data from the Glastir Monitoring and Evaluation Programme, Wales 2013-2014. NERC Environmental Information Data Centre. https://doi.org/10.5285/1c5cf317-2f03-4fef-b060-9eccbb4d9c21

- Lebron, I.; Seaton, F.; Barrett, G.; Alison, J.; Burden, A.; Emmett, B.A.; Garbutt, A.; Robinson, D.A.; Williams, B.; Wood, C.M. (2020). Topsoil particle size distribution from the Glastir Monitoring and Evaluation Programme, Wales 2013-2016. NERC Environmental Information Data Centre. https://doi.org/10.5285/d6c3cc3c-a7b7-48b2-9e61-d07454639656

Emmett, B.A.; Reynolds, B.; Chamberlain, P.M.; Rowe, E.; Spurgeon, D.; Brittain, S.A.; Frogbrook, Z.; Hughes, S.; Lawlor, A.J.; Poskitt, J.; Potter, E.; Robinson, D.A.; Scott, A.; Wood, C.; Woods, C. (2010) Countryside Survey: Soils Report from 2007. NERC/Centre for Ecology and Hydrology, 192pp. (CS Technical Report No. 9/07, CEH Project Number: C03259)

Emmett, B. A. et al. (2013). An integrated ecological, social and physical approach to monitoring environmental change and land management effects: the Wales Axis II Monitoring and Evaluation Programme. Aspects of applied biology, 31-39.

Emmett, B.E., Abdalla, M., Anthony, S., Astbury, S., August, T., Barrett, G., Biggs, J., Botham, M., Bradley, D., Brown, M., Carter, H., Chadwick, D., Cigna, F., Cooper, D., Cooper, J., Cosby, BJ., Creer, S Cross, P., Edwards, F., Edwards, M., Evans, C., Ewald, N., Fitton, A., Garbutt, A., Grebby, S., Greene, S., Halfpenney, I., Hall, J., Harrison, S., Harrower, C., Henrys, P., Hobson, R., Hughes, S., Isaac, N., Jackson, B., Jarvis, S., Jones, D., Keith, A., Kelly, M., Korenko, J., Lallias, D., Leaver, D., Lebron, I., Malcolm, H., Maskell, L., McDonald, J., Moxley, J., Norton, L., O’Hare, M., Owen, A., Pereira, M., Peyton, J., Powney, G., Pywell, R., Rawlins, B., Robinson, D.A., Rorke, S., Rowland, C.,Roy, D., Scarlett, P., Scholefield, P., Scott, A., Scott, L., Scott, R., Siriwardena, G., Smart, S., Smith, P., Swetnam, R., Taylor, R., Tordoff, G., Van Breda, J., Vincent, H., Wagner, M., Waters, E., Watkins, J.,White, J., Williams, B., Wood, C. and Wright, S. (2014) Glastir Monitoring & Evaluation Programme. First Year Annual Report to Welsh Government (Contract reference: C147/2010/11). NERC/Centre for Ecology & Hydrology (CEH Project: NEC04780), pp.442

Emmett, B.E., Abdalla, M., Anthony, S., Astbury, S., August, T., Barrett, G., Beckman, B., Biggs, J., Botham, M., Bradley, D., Brown, M., Burden, A., Carter, H., Chadwick, D., Cigna, F., Collier, R., Cooper, D., Cooper, J., Cosby, B.J., Creer, S., Cross, P., Dadam, D., Edwards, F., Edwards, M., Evans, C., Ewald, N., Fitton, A., Garbutt, A., Giampieri, C., Gooday, R., Grebby, S., Greene, S., Halfpenney, I., Hall, J., Harrison, S., Harrower, C., Henrys, P., Hobson, R., Hughes, P., Hughes, S., Illian, J., Isaac, N., Jackson, B., Jarvis, S., Jones, D.L., Jones, P., Keith, A., Kelly, M., Kneebone, N., Korenko, J., Lallias, D., Leaver, D., Lebron, I., Malcolm, H., Maskell, L., McDonald, J., Moxley, J., Norton, L., O’Hare, M., Oliver, T., Owen, A., Parkhill, K.A., Pereira, M., Peyton, J., Pogson, M., Powney, G., Pritchard, N., Pritchard, S., Prochorskaite, A., Prosser, M., Pywell, R., Rawlins, B., Reuland, O., Richards, M., Robinson, D.A., Rorke, S., Rowland, C., Roy, D., Scarlett, P., Scholefield, P., Scott, A, Scott, L., Scott, R., Sharps, K., Siriwardena, G., Smart, S., Smith, G., Smith, P., Stopps, J., Swetnam, R., Taft, H., Taylor, R., Tebbs, E., Thomas, A., Todd-Jones, C., Tordoff, G., Turner, G., Van Breda, J., Vincent, H., Wagner, M., Waters, E., Walker-Springett, K., Wallace, H., Watkins, J., Webb, G., White, J., Whitworth, E., Williams, B., Williams, P., Wood, C. and Wright, S. (2017) Glastir Monitoring & Evaluation Programme. Final Report to Welsh Government. Contract reference: C147/2010/11. NERC/Centre for Ecology & Hydrology (CEH Projects: NEC04780/NEC05371/NEC05782)

PB13297, 2009. Safeguarding our soils a strategy for England, Defra, Nobel House, 17 Smith Square, London SW1P 3JR

Robinson, D.A., Bentley, L., Jones, L., Feeney, C., Garbutt, A., Tandy, S., Lebron, I., Thomas, A., Reinsch, S., Norton, L., Maskell, L., Wood, C., Henrys, P., Jarvis, S., Smart, S., Keith, A., Seaton, F., Skates, J., Higgins, S., Giuffrè, G., Emmett, B. A. (2024). Five decades' experience of long-term soil monitoring, and key design principles, to assist the EU soil health mission. European Journal of Soil Science, 75(5), e13570. https://doi.org/10.1111/ejss.13570

Robinson, D.A., Emmett, B.A., Fitos, E., Keith, A. (2023) ERAMMP Document-51: Field-Survey Handbook (Procedures) - Soil Sampling. For Welsh Government (Contract C210/2016/2017) (UK Centre for Ecology & Hydrology Projects 06297 & 06810):

https://erammp.wales/sites/default/files/2023-06/ERAMMP%20Document-51%20Field-Survey%20Handbook%20%28Procedures%29%20-%20Soil%20Sampling%20v2.0.0.pdf

Ruehlmann, J., 2020. Soil particle density as affected by soil texture and soil organic matter: 1. Partitioning of SOM in conceptional fractions and derivation of a variable SOC to SOM conversion factor. Geoderma, p.114542.

Ruehlmann, J. and Körschens, M., 2020. Soil particle density as affected by soil texture and soil organic matter: 2. Predicting the effect of the mineral composition of particle-size fractions. Geoderma, p.114543.

Smart, S.M., Wood, C.M., Maskell, L.C., Norton, L., Bunce, R. & Barr, C. (2023). Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP). ERAMMP Document-49: Field-Survey Handbook (Procedures) - Vegetation Plots. Report to Welsh Government (Contract C210/2016/2017) (UK Centre for Ecology & Hydrology Projects 06297 & 06810):

https://erammp.wales/sites/default/files/2023-06/ERAMMP%20Document-49%20Field-Survey%20Handbook%20%28Procedures%29%20-%20Vegetation%20Plots%20v2.1.0.pdf

Wood, C. M., Bunce, R. G. H., Norton, L. R., Maskell, L. C., Smart, S. M., Scott, W. A., Henrys, P. A., Howard, D. C., Wright, S. M., Brown, M. J., Scott, R. J., Stuart, R. C., & Watkins, J. W. (2018). Ecological landscape elements: Long-term monitoring in Great Britain, the Countryside Survey 1978-2007 and beyond. Earth System Science Data, 10(2), 745–763. https://doi.org/10.5194/essd-10-745-2018

Wood, C. M., Alison, J., Botham, M. S., Burden, A., Edwards, F., Garbutt, R. A., George, P. B. L., Henrys, P. A., Hobson, R., Jarvis, S., Keenan, P., Keith, A. M., Lebron, I., Maskell, L. C., Norton, L. R., Robinson, D. A., Seaton, F. M., Scarlett, P., Siriwardena, G. M., Skates, J., Smart, S. M., Williams, B., and Emmett, B. A. (2021). Integrated ecological monitoring in Wales: the Glastir Monitoring and Evaluation Programme field survey. Earth System Science Data, 13, 4155–4173. https://doi.org/10.5194/essd-13-4155-2021

ERAMMP Programme Office

UKCEH Bangor

Environment Centre Wales

Deiniol Road

Bangor, Gwynedd

LL57 2UW

+ 44 (0)1248 374500

erammp@ceh.ac.uk

www.erammp.cymru

www.erammp.wales