| Topsoil visible and near-infrared (Vis-NIR) spectral data from the UKCEH Countryside Survey, Great Britain, 2018-2019 - EIDC |  |
| --- | --- |
| V.O.Ebuele, S. Tandy, M. Brentegani, I. Lebron, A. Tye, A. Keith, C. Hermansen, L. Wollesen de Jonge, M, Zaresourmanabad, D.A. Robinson |  |
| Client Ref: | Report |
| Issue number 1 15. May.2025 |  |

Contents

1.	Background information	3

2.	Introduction to the Countryside Survey Design and the rolling program	4

3.	Sample Collection Methods	6

4.	Metrics, Laboratory Protocols and Analytical Methods	7

4.1	Soil sampling locations and general information	7

4.2	Land Class	7

4.3	Countryside Survey Environmental Zones	10

4.4	Measured and derived soil metrics	11

4.5	Soil spectra measurement using the ASD AgriSpec spectrometer	11

4.6	Soil spectra measured using FOSS DS2500 spectrometer	12

5.	Data Quality Assurance	12

6.	Details of data structure	13

6.1	Details of soil spectra prediction	14

6.2	Open Soil Spectral Library (OSSL) software	14

6.3	Results Table	15

6.4	Comparison with the CS2018-19 laboratory metrics	17

6.5	Predicted (ASD AgricSpec and FOSS D2500) Vs. Observed (Laboratory Metrics) values	17

6.6	Evaluation of OSSL Model Performance in Predicting Soil Properties	19

7.	References and Further Reading	20

# Background information

Countryside Survey is a unique study or “audit” of the natural resources of the UK’s countryside. The Survey has been carried out at regular intervals since 1978. The countryside is sampled and studied using rigorous scientific methods, allowing us to compare results from 2019+ with those from previous surveys. In this way, we can detect the gradual and subtle changes that occur in the countryside over time. A series of reports have been published outlining the main findings for UK and individual countries. The most recent full survey was conducted in 2007 (Emmett et al., 2010) with components of the soils research published in Reynolds et al (2013) and Keith et al. (2015). This dataset marks the start of a new phase of Countryside Survey, which has adopted a rolling program of monitoring as of 2019.

The 2019 (and subsequent) surveys are supported by the Natural Environment Research Council (NERC) under the UK-SCAPE national capability program. It provides a science platform to address the overarching question:

What is the direction and magnitude of change in soil condition and function across the UK and how do multiple pressures interact to create the spatial and temporal patterns observed?

Soil health is fundamental for food security, public health and understanding the climate system. This national research platform will address a critical gap in our understanding of soil status and dynamics at the national scale by transforming previous surveys since 1978 (Keith et al. 2015) through a new rolling sampling programme to provide data and maps of stock and change in GB soils, taking primary measurements of topsoil condition which influence soil function and with co-located vegetation assessments.

The survey is designed to address research questions of relevance to the UK and internationally, with a particular focus to determine the direction and magnitude of soil change. Some of the questions we hope to address include:

- How do multiple pressures interact to change topsoil and vegetation condition and function?

- How is the topsoil carbon pool changing in response to multiple pressures?

- Are topsoil carbon to nitrogen ratios changing across habitats?

- Is soil pH changing in response to reduced atmospheric pollution or changes in land management practices?

- Is there any evidence for increased compaction due to intensive management practices?

- Has improved land seen any change in the levels of available phosphorus?

- How does change in vegetation, driven by land use change or chemical deposition, feedback on topsoil condition and function?

# Introduction to the Countryside Survey Design and the rolling program

In the Countryside Survey, the monitoring survey was designed in such a way as to capture multiple measures and metrics (Scott, 2008; Reynolds 2013), and to integrate across these metrics. Hence, a full ecosystem-based approach is required where data operating at multiple scales are captured, where possible, at the same time in a single snapshot visit. The original monitoring program has been reduced and currently focuses on soil and vegetation change.

For UKCEH-CS, a rolling survey has now been adopted that will repeat once a five-year cycle or when 500 squares have been completed. This is to identify change that occurs over that time period. The rolling programme is more practical and buffers the results against extreme years in terms of climate fluctuations. It ensures we maximise the number of sites visited across the national spatial scale whilst at the same time monitoring year-on-year, such that spatial variation and temporal changes and trends can be detected cost-effectively. The first survey cycle dates from 2019.

The selection of the sample of 1-km × 1-km squares for monitoring is based on the Countryside Survey of Great Britain (as described below), which aims to provide robust estimates of indicators at national and sub-national level across GB and constituent countries. All 256 squares selected in 1978  form part of this resampling, then a selection of 1998 and 2007 squares. The rationale for selecting the 1978 squares was to give a repeat sampling over the longest period to identify change. The Countryside Survey design is covered in detail in the accompanying documentation.

A total of 500 × 1-km2 squares, over the five-year cycle, were randomly sampled within strata defined according to the Land Classification of Great Britain (Bunce et. al., 2007) – a derived classification of the landscape based on its topography, geology, climate, and physical attributes. 100 of these squares will be visited on an annual basis.

Any square randomly selected that contained more than 75% of urban land or that was more than 90% sea (defined by LCM2007 and the UK Census mean high tide data) was excluded.

Figure 1. Map to show distribution of 1-km × 1-km survey squares across Great Britain for 2018 / 2019 (not shown to scale, to ensure data confidentiality).

# Sample Collection Methods

Within each 1-km × 1-km square, topsoil samples were taken from 5 pre-determined randomly dispersed locations (where possible) with the vegetation X-Plots. The soil sample analysed for soil metrics is taken using a black plastic core, 15 cm long, from a location co-incident with vegetation surveys (Table 1).

Sample Location: ‘As part of the 1978 survey, soil samples from the top approximately 0-15 cm were collected from a soil pit in the centre of each of the five Main X-Plots (200 m2) randomly located in each of the 256 1-km x 1-km squares. Maps were drawn to help future relocation. In 1990, permanent metal markers were placed immediately adjacent to the south corner of all plots, as mapped in 1978 (or according to an alternative detailed protocol if within-field placement was inappropriate) during a CS survey which did not involve soil sampling’ Topsoil samples were taken 15 cm from the corner of the 2 × 2 m vegetation quadrat accordingly.

Table 1. Location of sampling point in relation to 2 m × 2 m quadrat each year, Numbers of 1-km × 1-km squares visited, and numbers of squares attempted to visit, numbers of soil samples which were analysed.

| Year | Location of sampling point -corner of 2 by 2m quadrat | Number of squares visited | Number of soil samples analysed |
| --- | --- | --- | --- |
| 1978 | 15 cm to South | 256 | 1284 |
| 1998 | 15 cm to North | 569 | 1284 |
| 2007 | 15 cm to South | 591 | 2955 |
| 2018 2019 | 15 cm to West | 6 100 | 25 456 |

‘In 1998 and 2007, maps and markers were requested from the surveyors for all new or changed locations. Samples from 1998 and 2007 were therefore resampled 2-3 m apart between each survey.’ (Emmett et al., 2008). In 2019 plots were moved to the western corner of the inner 2 m × 2 m plot in the centre of the main plots. An analysis of exact plot locations is conducted to ensure plots were relocated accurately, using photos, field maps and GPS data.

After collection, the topsoil cores are refrigerated and stored until posted, usually within a couple of days, to laboratories at the UK Centre for Ecology & Hydrology, Bangor.

# Metrics, Laboratory Protocols and Analytical Methods

General field methods are described in the accompanying Soils Manual (Emmett et al. 2008). At each pre-defined location, five soil samples are taken, sent to UKCEH sites for processing or storage. Processed and/or frozen soil samples are stored in the UKCEH Soil Bank.

## Soil sampling locations and general information

SQUARE; PLOT; COUNTRY; YEAR

Long-term Countryside Survey locations are identified using 1-km × 1-km square IDs (SQUARE). Within each square, up to five locations are surveyed for plants and sampled for soils (PLOT). If plots cannot be sufficiently relocated, another PLOT_ID may be created. COUNTRY defines the location of each SQUARE within each of the three countries across Great Britain: England (ENG), Scotland (SCO) and Wales (WAL). YEAR defines the year of soil sampling.

## Land Class

LC07, LC07_NUM

ITE Land Class 2007 of Great Britain is used as the basis for stratifying the survey (Table 2). Land classes are based on the major environmental gradients (climate, relief, geology) across the countryside (Bunce et al. 2007). The strength of Countryside survey is the ability to report changes in soil condition according to vegetation classes, either through broad habitat or aggregate vegetation class in 45 ITE land classes.

Table 2: ITE Land Classes (2007) as described in Bunce et al. 2007; LC07_NUM is the numeric equivalent to the 2007 Land Class description (LC07).

| LC07_NUM | Description (LC07) |
| --- | --- |
| England |  |
| 1e | Flood plains/shallow valleys, S England |
| 2e | Low calcareous hills / variable lowlands, S England |
| 3e | Flat / gently undulating plains, E Anglia / S England |
| 4e | Flat coastal plains, E Anglia / S England |
| 5e | Shallow slopes / floodplains, S-W England |
| 6e | Complex valley systems / table lands, S-W England |
| 7e | Sea cliffs / hard coast, England |
| 8e | Estuarine / soft coast / tidal rivers, England |
| 9e | Almost flat plains, N Midlands, NE England |
| 10e | Gently rolling / almost flat plains, NE England / N Midlands |
| 11e | Flat plains / small river floodplains, E Midlands |
| 12e | Large river floodplains, flat plains, margins, E Anglia |
| 13e | Coastal plains / gently rolling low hills, NW England |
| 15e | Flat river valleys / lower hill slopes, NW England |
| 16e | Gently rolling low hills / flat river valleys, NW England |
| 17e | Upland valleys / rounded hill sides, England |
| 18e | Upland valley sides / Low mountains, N England |
| 19e | Upland valleys / plateaux, N England |
| 22e | Intermediate mountain tops / broad ridges, N England |
| 23e | High mountain summits / ridges, N England |
| 25e | Flat / gently undulating river valleys, N England |
| Scotland |  |
| 7s | Hard / mixed coasts, S-W Scotland |
| 13s | Coastal plains / soft coasts, S-W Scotland |
| 18s | Isolated hills / mountain summits, W Scotland |
| 19s | Upland valleys / low mountains, S Scotland |
| 21s | Low mountain slopes / upper river valleys, Highlands |
| 22s | Round mountains / broad upper ridges, S Scotland / Highlands |
| 23s | High mountain summits / ridges / valleys, Highlands |
| 24s | Steep valley sides / intermediate mountain tops, W Highlands |
| 25s | Undulating plains / gently sloping valleys, E Scotland |
| 26s | Flat plains / gently sloping lowlands, central & S Scotland |
| 27s | Low hills / undulating lowlands, Scotland except W |
| 28s | Shallow valleys / low hill plateaux, throughout Scotland |
| 29s | Inner rocky / mixed coasts / complex topography, W Scotland |
| 30s | Outer rockery / mixed coasts / low hills, W Scotland / Islands |
| 31s | Rocky / mixed coasts / low hills, N Scotland / Islands |
| 32s | Shallow hills / complex coastlines, N Scotland / Islands |
| Wales |  |
| 17w1 | Low mountain ridges / valley slopes, N Wales |
| 17w2 | Rounded mountains / scarps / upper valleys, mid / S Wales |
| 17w3 | Variable landforms of hills / low mountain, Wales |
| 5w | Shallow slopes / flood plains, Wales |
| 6w | Complex valley systems / table lands, Wales |
| 7w | Sea cliffs / hard coast, Wales |
| 15w | Flat river valleys / lower hill slopes, Wales |
| 18w | Upland valley sides / low mountains, Wales |

## Countryside Survey Environmental Zones

EZ_DESC_07

Environmental Zones are aggregations of ITE Land Classes (Table 3); these classes are derived from repeatable multivariate analysis of environmental data collected for each 1-km × 1-km square across GB. Thus, the classes, and hence the zones, are determined by combinations of environmental characteristics, not by just one or two. This means that the naming of classes (and zones) is not straightforward and cannot be achieved by reference to single parameters such as altitude (UKCEH Environmental Zones, 2013). Environmental Zone names were derived from an analysis of their average environmental characteristics and are namely:

Table 3: Environmental Zone categorization based on ITE Land Classes (2007).

| Environmental Zone 2007 | ITE Land Class Numbers (2007) | Environment Zone 2007 description |
| --- | --- | --- |
| EZ1 | 11e, 12e, 25e, 2e, 3e, 4e, 9e | Easterly Lowlands, England |
| EZ2 | 10e, 13e, 15e, 16e, 1e, 5e, 6e, 7e, 8e | Westerly Lowlands, England |
| EZ3 | 17e, 18e, 19e, 22e, 23e | Uplands, England |
| EZ4 | 13s, 25s, 26s, 27s, 7s | Lowlands, Scotland |
| EZ5 | 18s, 19s, 28s, 29s, 30s, 31s, 32s | Intermediate Uplands and Isles, Scotland |
| EZ6 | 21s, 22s, 23s, 24s | True Uplands, Scotland |
| EZ7 | (Northern Ireland, not included in this dataset) |  |
| EZ8 | 15w, 5w, 6w, 7w | Lowlands, Wales |
| EZ9 | 17w1, 17w2, 17w3, 18w | Uplands, Wales |

## Measured and derived soil metrics

Five soil cores were collected for up to five sampling locations per square; cores were sent to UKCEH laboratories. Cores were logged into the laboratory system. All spectra were measured on Fine Earth (FE) in the Laboratory. A detailed description of measured soil metrics is documented in Bentley et al. (2023).

## Soil spectra measurement using the ASD AgriSpec spectrometer

Visible and Near-Infrared (Vis-NIR) reflectance spectroscopy

Spectral reflectance (R) was measured, using an ASD AgriSpec spectrometer (Malvern Panalytical Inc., Boulder, Colorado, USA) on oven-dry soil that was deagglomerated and sieved to a size fraction of ≤2 mm, representing the fine earth. This instrument covers a full visible and near-infrared (Vis-NIR) spectral range from 350 nm to 2,500 nm (28,570 – 4000 cm⁻¹). The ASD AgriSpec was equipped with a MugLight accessory containing an internal broad-spectrum quartz tungsten halogen light source for spectral collection. The spectrometer also features one 512-element silicon detector array and two electrically cooled graded-index indium gallium arsenide (InGaAs) detector arrays. The device offers two spectral resolutions (3 nm at 700 nm and 6 nm at 1400/2100 nm) and operates within a temperature range of 0 to 40°C, as noted by Murad et al., (2023)

Spectral acquisition and processing

Spectra were acquired on the Fine Earth (FE) samples under laboratory conditions. The instrument and the MugLight (Analytical Spectral Devices, Boulder, CO) were powered on and allowed to warm up for 60 minutes to ensure system stabilisation. To ensure homogeneity, the bag containing the soil sample was thoroughly mixed to evenly mix the soil particles before scanning with the spectrometer. Approximately 20 grams of the soil were sub-sampled and transferred to the ASD AgriSpec sampling tray adapter, ensuring that the soil sample surface was smooth and if needed, a spatula was used to flatten the surface carefully, which was then attached to the MugLight. The soils were scanned from below using the MugLight. Reflectance (diffuse) measurements were taken in the spectral range from 350 nm to 2500 nm, with 1 nm increments, and were recorded in triplicate for each sample with a 180° rotation between measurements. Each spectrum was derived from the average of 25 internal measurements taken in about 2.5 seconds. Reflective measurements were recorded using the ASD’s proprietary Indico Pro software, while file conversions were performed with the View Spec Pro software.

The soil spectral data were initially stored in the instrument accessory PC using a proprietary format known as “ASD.” The three replicate spectra files collected per sample were then averaged to create a single spectra. The average raw “ASD” spectral data was then converted to the American Standard Code for Information Interchange (ASCII) format and saved as TXT files, which are tab delimited. These TXT files can be easily imported into external spreadsheet software, such as Excel, and subsequently converted to CSV (comma-separated values) files that contain the spectral data matrix. In these files, soil samples are organised in rows, while wavelengths are organised in columns; this is then fed to the prediction software.

Quality Control

A certified white reference disc standard panel with over 99% reflectance at all wavelengths (Spectralon®, Labsphere Inc., North Sutton, NH, USA) was used for white referencing. White referencing measurements were conducted at the beginning of the spectral readings and periodically throughout the session to ensure instrument calibration. These measurements were taken after every 20 scans. To avoid cross-contamination, sample holders (ASD AgricSpec tray adaptor) and the quartz window on the MugLight were thoroughly cleaned between samples. Triplicate readings were taken for each sample, and two internal quality control reference samples, Bangor reference H and M (BSH and BSM), were measured at the beginning and end of each session to ensure data quality.

## Soil spectra measured using FOSS DS2500 spectrometer

The soil samples were scanned with a FOSS DS2500 spectrometer, a laboratory-use-only instrument. The FOSS DS2500 NIR spectrometer covers a full visible and near-infrared (Vis-NIR) spectral range of 400 nm to 2,500 nm (25,000 – 4000 cm⁻¹) and weighs approximately 27 kg and measures 37.5 x 30 x 49 cm. It was equipped with an internal broad-spectrum quartz tungsten halogen light source. The spectrometer also includes one silicon detector (400 – 1100nm) and lead sulphide (1100 – 2500 nm) detector arrays. The device offers a spectral resolution of 0.5 nm and operates within a temperature range of 5 to 40°C.

The samples were added to a sample cup with a quartz window and automatically rotated during the measurements. During measurement, the soil surface diffuse reflectance data were collected at seven different positions, with each position measured four times. The 28 spectra for each soil sample were then averaged into one representative soil spectrum.

Quality Control

As a standard procedure, the software ISIScan Nova was used to perform hardware and performance tests on the instrument. Further, the instrument was checked against a FOSS white check sample.

# Data Quality Assurance

Soil data collection and processing for UKCEH’s large-scale field surveys has undergone major scrutiny. With the change from one-year highly intensive surveying to rolling surveys, data collection and processing is more continuous. It requires a robust system to check, and Quality Assure (QA) data on a regular basis. UKCEH laboratory staff and data scientists developed a data workflow to ensure the highest possible soil data standard using R-scripts and guidance documents.

# Details of data structure

High-resolution reflectance spectra were collected from re-quartered reserved Fine Earth (FE) soil samples collected during the 2018-2019 Countryside Survey (Bentley et al., 2023). Out of the 481 samples collected during the survey, only 451 were deemed suitable for diffuse reflectance spectroscopy because of sufficient sample mass remaining.

Diffuse reflectance spectra were collected from 451 samples using the ASD AgricSpec spectrometer and from 306 samples using the FOSS DS2500 spectrometer in the visible (350 to 780 nm) and near-infrared (780 to 2500 nm) regions, with spectral resolutions of 1 nm and 0.5 nm, respectively. The values are presented in reflectance and consist of two CSV files: one contains the average soil spectra (451 rows) determined using the ASD AgricSpec Spectrometer, while the other includes average soil spectra (306 rows) from the FOSS DS2500 spectrometer. The metadata summary of the dataset is provided in Table 4, and each row consists of a spectrum. (see CSV file ukceh_cs201819_soil_ Vis_NIR_spectra_ASD & ukceh_cs201819_soil_Vis-NIR_spectra_FOSS)

Table 4: Metadata summary for spectral data 2018 – 2019.

| COLUMN NAME | TYPE | UNIT | DESCRIPTION |
| --- | --- | --- | --- |
| SQ_ID | Text | None | 1-km survey square - unique identifier linked to UKCEH_CS2018 -19 data |
| PLOT | Number | None | Plot number for the soil sampling location (X-Plot) |
| YEAR | Number | None | Year of survey |
| Spectrum between 400 – 2499_5 nm | Number | nm | The column shows the reflectance data (FOSS D2500) in the Vis-NIR regions (400 – 2499_5 nm) with 0.5 nm resolution. The underscore symbol represents the decimal point. |
| Spectrum between 350 – 2500 nm | Number | nm | The column shows the reflectance data (ASD AgriSpec) in the Vis-NIR regions (350 – 2500 nm) with 1 nm resolution. |

## Details of soil spectra prediction

The prediction model in this study was developed for the Soil Spectroscopy for Global Good (SS4GG) project, a collaboration between the Woodwell Climate Research Centre, the OpenGeoHub Foundation, and the University of Florida, funded by the USDA National Institute of Food and Agriculture (NIFA) from 2021 to 2024 (Safanelli et al., 2025).

The “Open Soil Spectral Library” (OSSL) engine, developed by the SS4GG initiative, is an open-source project addressing soil data gaps and spectroscopy challenges. Building on (Rossel et al., 2016.) and (Shepherd et al., 2022.). It expands on the first global soil calibration library created by the World Agroforestry Centre (ICRAF) in collaboration with the International Soil Reference and Information Centre (ISRIC)

The Open Soil Spectral Library (OSSL) v1.2 is a digital asset and web service that integrates diverse datasets into a harmonised soil spectral library. It includes various Visible and Near-Infrared (Vis-NIR) libraries that cover the entire spectral range of 350 – 2500 nm (Safanelli et al., 2025), making it the most useful open-source predictive model for soil spectroscopy analysis. The Vis-NIR spectra represent the reflectance factor at each wavelength and were trimmed to this range with a 2 nm interval (Jones, 2020; Safanelli et al., 2025).

To improve the calibration and reliability of the prediction models in the OSSL database, the SS4GG utilised only selected harmonised soil properties with a minimum of 500 samples (Safanelli et al., 2025), with this approach ensuring strong data integrity. The SS4GG Team also reported that the Cubist Machine Learning algorithm with the OSSL Engine was the best model for estimating unknown spectral measurements. This has been corroborated by Safanelli et al., 2025, Sharififar et al., 2019, and Tang et al., 2020 who reported that the Cubist outperformed other machine learning algorithms in calibration effectiveness and output reliability. This suggests that the Open Soil Spectral Library, with the Cubist model, can offer better predictions, insights into uncertainty, and representation flags for a better understanding of results.

## Open Soil Spectral Library (OSSL) software

The OSSL Engine is a web platform for uploading Vis-NIR spectra (400 - 2500 nm) and receiving predictions with uncertainty estimations and representativeness flags. It features five model versions based on sample availability (Safanelli et al., 2025). There are two subsets: KSSL alone (kssl acronym), and the full OSSL database (ossl acronym). The models cover three spectral types: Vis-NIR (visnir acronym), MIR (mir acronym), and NIR from the NeoSpectra instrument (now known as ProxiScout) (nir.neospectra acronym) (Safanelli et al., 2025).

The OSSL model is recommended for spectral data predictions due to its representative Vis-NIR spectral database from most European countries (Safanelli et al., 2025). To estimate unknown spectral measurements, the SS4GG team suggests using the OSSL model, while KSSL models are preferable when the predicted spectra match the same instrument and model as the KSSL library.

## Results Table

The prediction dataset is provided in a CSV file and encompasses soil data for 451 and 307 samples measured using the ASD Agri Spec and FOSS DS2500 spectrometer, respectively. All samples were predicted using the OSSL Engine (visnir_cubist_ossl_na_v1.2 model), which has been calibrated with the full OSSL. Each sample is assigned a unique square ID, linking it to the measured soil data in the UKCEH EIDC portal (Bentley et al., 2023). The dataset includes 23 soil parameters related to harmonised soil properties within the OSSL engine, formatted in 23 columns and 757 rows for results from the two instruments. The metadata summary is given in Table 5. (See CSV file ukceh_cs201819_soil_Vis-NIR)

Table 5: Metadata summary for Open Soil Spectral Library (OSSL) prediction of soil data.

| COLUMN NAME | TYPE | UNIT | DESCRIPTION |
| --- | --- | --- | --- |
| SQ_ID | Number | None | 1-km survey square - unique identifier |
| PLOT_ID | Number | None | Plot number of samples within a square |
| EQ_ID | Text | None | Type of Instrument: ASD AgricSpec or FOSS D2500 |
| LC | Number and Text | None | ITE Land Classification number – see Bunce et al., 2007. |
| EZ_DESC | Text | Metres | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| COUNTRY | Text | None | GB country |
| YEAR | Number | None | Year of survey |
| BROAD HABITAT | Text | None | Large category for the ecological environment |
| ACIDITY | Number | cmol(+)/kg | Acidity BaCl2-TEA Extractable, pH 8.2 |
| Al_EXT | Number | cmol(+)/kg | Soil Aluminium, Extractable, KCl |
| BULK_DENSITY | Number | g/cm3 | Soil Bulk Density, <2mm fraction, Clod |
| C_TOTAL | Number | % | % of Total Soil Carbon NCS |
| CA_EXT | Number | cmol(+)/kg | Soil Extractable, Calcium NH4OAc_ |
| CACO3 | Number | % | Soil Carbonate content, <2mm Fraction |
| CEC | Number | cmol(+)/kg | Soil Cation Exchange Capacity, pH 7.0, NH4OAc, 2M KCl |
| COARSE | Number | % | % of Coarse Soil Fragments, Greater than 2mm |
| CLAY_TOTAL | Number | % | % of Total Soil Clay particles |
| EC | Number | dS/m | Soil Electrical Conductivity, 1:5 Soil-Water Suspension |
| K_EXT | Number | cmol(+)/kg | Soil Extractable, Potassium, NH4OAc, 2M KCl displacement |
| Mg_EXT | Number | cmol(+)/kg | Soil Extractable, Magnesium, NH4OAc, 2M KCl displacement |
| N_TOTAL | Number | % | % of Total Soil Nitrogen, NCS |
| Na_EXT | Number | cmol(+)/kg | Soil Extractable, Sodium NH4OAc, 2M KCl displacement |
| ORGANIC C_TOTAL | Number | % | % of Total Soil Organic Carbon, without CaCO3, S prep |
| PO4_ OLSEN | Number | mg/kg | Soil Extractable Phosphorus, Olsen |
| pH_CACl2 | Number | None | pH, 1:2 Soil-CaCl2 Suspension_ |
| pH | Number | None | pH, 1:1 Soil-Water Suspension |
| S_TOTAL | Number | % | % of Total Soil Sulphur, Total NCS |
| SAND_TOTAL | Number | % | % of Total Soil Sand particles, S prep |
| SILT_TOTAL | Number | % | % of Total Soil Silt particles, S prep_ |
| WR_15 BAR | Number | % | % of Water Retention, 15 Bar (1500 kPa) |
| WR_1/3 BAR | Number | % | % of Water Retention, 1/3 Bar (33 kPa) |

## Comparison with the CS2018-19 laboratory metrics

Linear regression analysis and statistical tests were conducted to evaluate the effectiveness of the OSSL model in predicting soil matrices and their relationship with the measured soil data reported by Bentley et al. (2023). The analysis focused on key parameters, including Soil Bulk Density (BULK_DENSITY), Total Carbon (C_TOTAL), Carbonate Content (CaCO3 (%)), Electrical Conductivity (EC), Total Nitrogen (N_TOTAL), Total Soil Organic Carbon (Organic C_TOTAL), Extractable Phosphorus (PO4 OLSEN), pH in CaCl2, pH in Water and Particle-size data (Sand, Silt, and Clay). The Particle-size data were only available for 313 samples with organic matter content (% LOI) of less than 40 %. The dataset (see Table 6) includes eight columns and eleven rows, enabling the assessment of predictive performance and trend identification, particularly the alignment of predicted values with actual values. It also evaluates the statistical significance of differences between predictions and true values.

Qualitative description of the NIR spectral data

Reflectance signals usually arise from vibrations in the bonds between carbon (C), nitrogen (N), hydrogen (H), oxygen (O), phosphorus (P), and sulphur (S) atoms. In the near-infrared (NIR) range (780 – 2500 nm), three distinct absorption characteristics can be seen in soil spectra, typically occurring around 1400 nm, 1900 nm, and 2200 nm. These absorption characteristics are mainly linked to the sensitive stretching and bending vibrations of functional groups such as NH, CH, CO, and CN. These groups are recognised for producing significant overtones essential for the quantitative assessment of important soil properties, including soil carbon, organic matter, clay minerals, and water content (Ben Dor et al., 2015).

## Predicted (ASD AgricSpec and FOSS D2500) Vs. Observed (Laboratory Metrics) values

The comparative results indicated that the OSSL Cubist model provided reasonable predictions for five out of the twelve soil parameters analysed for both instrument outputs. Notably, the model performed well in predicting BULK_DENSITY (ASD: R² = 0.43; FOSS; R² = 0.33),  C_TOTAL (ASD: R² = 0.69: FOSS; R² = 0.76 ), CaCO3 (ASD: R² = 0.53: FOSS: R² = 0.22 ), N_TOTAL (ASD: R² = 0.59; FOSS: R² = 0.68), ORGANIC C_TOTAL (ASD: R² = 0.67: FOSS: R² = 0.75). The FOSS instrument output also provided better predictions for CLAY (R² = 0.42), pH_CaCl2 (R² = 0.77) and pH (R² = 0.71) compared to R² = 0.011,  R² = 0.12, and  R² = 0.19 for CLAY,  pH_CaCl2 and PH for the ASD instruments, respectively (see Table 6).

In terms of prediction accuracy for both instruments, the FOSS data set seems to give better predictions than the ASD instrument data, as indicated by the slightly higher R2 values for most of the soil parameters compared. It should be noted, however, that the ASD instrument data regression analysis was conducted with a sample size of between 308 – 451 soil samples, compared to 246 – 306 soil samples used for the FOSS instrument data regression analysis. (see Table 6).

Although both datasets were derived from the same set of samples, the difference in predictive performance between the smaller FOSS dataset (n =306) and the larger ASD Agric Spec dataset ( n = 451) may be due to differences in data quality or measurement precision between the two instruments. However, the better results from the smaller dataset could also have been caused by overfitting or reduced variability in the smaller dataset.

Both instrument spectra data, however, performed poorly in predicting PO₄-OLSEN, EC, SAND_TOTAL and SILT_TOTAL, which suggests that the OSSL Cubist model was unable to predict these soil parameters with acceptable accuracy.

Table 6. Summary of regression analysis comparison between Open Soil Spectral Library (OSSL) prediction and CS laboratory metrics. Statistics are reported for 12 soil properties for the two Vis_NIR instruments. n = number of samples; p value ; R2: Coefficient of determination; r; RMSE: Root mean square error

|  |  | FOSS DS2500 |  |  |  |  | ASD AgricSpec |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil Property | n | p < 0.05 | R2 | r | RMSE | n | p < 0.05 | R2 | r | RMSE |
| BULK _DENSITY | 306 | <0.001 | 0.33 | 0.57 | 0.29 | 451 | <0.001 | 0.43 | 0.66 | 0.50 |
| C_TOTAL | 305 | <0.001 | 0.76 | 0.86 | 27.39 | 449 | <0.001 | 0.69 | 0.83 | 10.96 |
| CACO3 | 306 | <0.001 | 0.22 | 0.46 | 181.92 | 451 | <0.001 | 0.53 | 0.73 | 21.08 |
| CLAY_TOTAL | 246 | <0.001 | 0.42 | 0.65 | 11.07 | 308 | 0.064 | 0.011 | 0.04 | 20.68 |
| EC | 451 | 0.249 | 0.044 | -0.07 | 0.22 | 451 | 0.861 | 0.01 | -0.01 | 1.26 |
| N_TOTAL | 305 | <0.001 | 0.68 | 0.82 | 0.51 | 451 | <0.001 | 0.59 | 0.77 | 0.46 |
| ORGANIC C_TOTAL | 306 | <0.001 | 0.75 | 0.87 | 10.01 | 451 | <0.001 | 0.67 | 0.82 | 23.36 |
| PO4_OLSEN | 294 | <0.001 | 0.18 | 0.43 | 29.11 | 435 | 0.878 | 0 | 0.01 | 23441 |
| pH_CaCl2 | 306 | <0.001 | 0.77 | 0.88 | 0.73 | 451 | 0.021 | 0.12 | 0.11 | 2.94 |
| pH H2O | 306 | <0.001 | 0.71 | 0.84 | 0.72 | 451 | <0.001 | 0.19 | 0.43 | 1.81 |
| SAND_TOTAL | 246 | <0.001 | 0.22 | 0.47 | 26.62 | 308 | <0.001 | 0.036 | 0.19 | 36.57 |
| SILT_TOTAL | 246 | <0.001 | 0.08 | 0.28 | 17.91 | 308 | 0.465 | 0.002 | 0.04 | 23.42 |

## Evaluation of OSSL Model Performance in Predicting Soil Properties

These results (Table 6) indicate that the Vis-NIR spectral similarities approach alone may be insufficient for accurately predicting these particular soil properties, likely due to the inherent limitations of instrumental spectral methods in capturing certain chemical and physical characteristics.

The unsatisfactory performance of most soil properties in table 6 may be attributed to the characteristics of the unknown spectra, the standard operating procedures (SOP) of the instruments, and spectral variations in the ASD and FOSS instruments. These instruments can be highly sensitive to environmental and operational conditions, leading to inconsistencies in the spectral data that ultimately impact model accuracy. As noted in previous research, Vis-NIR is especially vulnerable to discrepancies between instruments (Rossel et al., 2016; Safanelli et al., 2025). This difference was also observed in the model prediction representation flags from the OSSL Output, where all the ASD data points were poorly represented, while all the FOSS data points were well represented. Additionally, Safanelli and his team pointed out that the origin and quality of the original datasets included in the Vis-NIR calibration set can also significantly impact the model's effectiveness.

Additionally, the calibration performance of the OSSL model, especially in the Vis-NIR region, has been reported to be inconsistent concerning soil properties such as bulk density, coarse fragments, electrical conductivity, and phosphorus (Olsen P). This finding indicates that relying solely on spectral variation as the only predictive feature within the OSSL engine may have limitations. Our results support these findings, as all five soil properties displayed poor model performance, although Clay (FOSS instrument) and Bulk density (ASD instrument) yielded relatively better results.(Safanelli et al., 2025).

Previous studies have shown that the OSSL Cubist model can predict soil properties such as total carbon, organic carbon, carbonate, and nitrogen in the Vis-NIR region with some level of accuracy. Our findings further support this assertion, as these properties ranked among those displaying the highest predictive performance in our analysis (Safanelli et al., 2025; Shepherd et al., 2022.; Tang et al., 2020).

This project acknowledges funding from the European Union’s Horizon Europe research and innovation program under grant agreement No.-101086179; and funding from the UK Research and Innovation (UKRI) under the UK government’s Horizon Europe funding guarantee [grant number 10053484] as part of AI4SoilHealth.

Disclaimer: Work funded by the European Union. Views and opinions expressed are however those of the author(s) only and do not necessarily reflect those of the European Union or the Research Executive Agency. Neither the European Union nor the granting authority can be held responsible for them.

# References and Further Reading

Allen, S. E. 1989. Chemical Analysis of Ecological Materials, 2nd ed., Blackwell Scientific Publications.

Avery, B.W. and Bascomb, C.L.  1974.  Soil Survey Laboratory Methods.  Soil Survey Technical Monograph No. 6, Harpenden.

Avery, B.W., 1990. Soils of the British Isles. CAB International.

Dor, E. B., Ong, C., & Lau, I. C. (2015). Reflectance measurements of soils in the laboratory: Standards and protocols. Geoderma, 245, 112-124.

https://doi.org/10.1016/j.geoderma.2015.01.002

Bentley, L.; Reinsch, S.; Alison, J.; Andrews, C.; Brentegani, M.; Chetiu, N.; Dart, S.; Dhiedt, E.; Emmett, B.A.; Fitos, E.; Garbutt, R.A.; Gray, A.; Henrys, P.A.; Hunt, A.; Keenan, P.O.; Keith, A.M.; Koblizek, E.; Lebron, I.; Millani Lopes Mazzetto, J.; Pallett, D.W.; Pereira, M.G.; Pinder, A.; Risser, H.; Rose, R.J.; Rowe, R.L.; Scarlett, P.; Seaton, F.; Smart, S.M.; Towill, J.; Wagner, M.; Williams, B.; Wood, C.M.; Robinson, D.A. (2023). Topsoil physico-chemical properties from the UKCEH Countryside Survey, Great Britain, 2018-2019. NERC EDS Environmental Information Data Centre.

Black, H.I.J., Parekh, N.R., Chaplow, J.S., Monson, F., Watkins, J., Creamer, R., Potter, E.D., Poskitt, J.M., Rowland, P., Ainsworth, G. and Hornung, M., 2003. Assessing soil biodiversity across Great Britain: national trends in the occurrence of heterotrophic bacteria and invertebrates in soil. Journal of Environmental Management, 67(3), pp.255-266.

Bunce, R.G.H., Barr, C.J., Clarke, R.T., Howard, D.C. & Lane, A.M.J. (1996) ITE Merlewood Land Classification of Great Britain. Journal of Biogeography, 23, 625-634

Bunce, R.G.H., Barr, C.J., Whittaker, H.A. (1981), Land Classes in Great Britain: Preliminary Descriptions for Users of the Merlewood Method of Land Classification.  Merlewood Research and Development Paper No. 86

http://nora.nerc.ac.uk/5898/1/R%26D0862.pdf

Bunce, R.G.H., Barr, C.J., Clarke, R.T., Howard, D.C., Scott, W.A. (2007). ITE Land Classification of Great Britain 2007. NERC Environmental Information Data Centre. http://doi.org/10.5285/5f0605e4-aa2a-48ab-b47c-bf5510823e8f

Bunce, R. G. H., Barr, C. J., Clarke, R. T., Howard, D. C., Scott, W. A. (2007) ITE Land Classification of Great Britain 2007. NERC-Environmental Information Data Centre doi:10.5285/5f0605e4-aa2a-48ab-b47c-bf5510823e8f

Carey, P.D., Wallis, S., Chamberlain, P.M., Cooper, A., Emmett, B.A., Maskell, L.C., McCann, T., Murphy, J., Norton, L.R., Reynolds, B., Scott, W.A., Simpson, I.C., Smart, S.M., Ullyett, J.M.. 2008 Countryside Survey: UK Results from 2007. NERC/Centre for Ecology & Hydrology, 105pp. (CEH Project Number: C03259).

Countryside Survey Website: General overview of Countryside Survey project with links to many relevant documents and methodologies. http://www.countrysidesurvey.org.uk/

Emmett, B.A., Reynolds, B., Chamberlain, P.M., Rowe, E., Spurgeon, D., Brittain, S.A., Frogbrook, Z., Hughes, S., Lawlor, A.J., Poskitt, J., Potter, E., Robinson, D.A., Scott, A., Wood, C., Woods, C.. 2010 Countryside Survey: Soils Report from 2007. NERC/Centre for Ecology and Hydrology, 192pp. (CS Technical Report No. 9/07, CEH Project Number: C03259)

Emmett, B. A., Frogbrook, Z. L., Chamberlain, P. M., Griffiths, R, Pickup, R., Poskitt, J., Reynolds, B., Rowe, E., Rowland, P., Wilson, J., Wood, C. M.. 2008 Countryside Survey. Soils Manual. NERC/Centre for Ecology & Hydrology, 180pp. (CS Technical Report No.3/07, CEH Project Number: C03259) http://nora.nerc.ac.uk/5201/1/CS_UK_2007_TR3%5B1%5D.pdf

Griffiths, R.I., Thomson, B.C., James, P., Bell, T., Bailey, M. and Whiteley, A.S., 2011. The bacterial biogeography of British soils. Environmental microbiology, 13(6), pp.1642-1654.	 https://sfamjournals.onlinelibrary.wiley.com/doi/full/10.1111/j.1462-2920.2011.02480.x

Jackson, M.L., 1958. Soil Chemical Analysis. Prentice Hall, London.

Jones, A., Fernandez-Ugalde, O., & Scarpa, S. (2020). Lucas 2015 topsoil survey. Presentation of dataset and results, EUR, 30332(10.2760), 616084.

Keith, A.M., Griffiths, R.I., Henrys, P.A., Hughes, S., Lebron, I., Maskell, L.C., Ogle, S.M., Robinson, D.A., Rowe, E.C., Smart, S.M. and Spurgeon, D., (2015). Monitoring soil natural capital and ecosystem services by using large‐scale survey data. Soil Ecosystems Services, pp.127-155. https://doi.org/10.2136/2015.soilecosystemsservices.2014.0070

Murad, M. O. F., Ackerson, J., Tolles, C., Meissner, K., Morgan, C. L., & Ge, Y. (2023). Estimating soil organic carbon content at variable moisture contents using a low-cost spectrometer. Geoderma, 440, 116723.

Norton, L.R., Maskell, L.C., Smart, S.S., Dunbar, M.J., Emmett, B.A., Carey, P.D., Williams, P., Crowe, A., Chandler, K., Scott, W.A. and Wood, C.M., 2012. Measuring stock and change in the GB countryside for policy–key findings and developments from the Countryside Survey 2007 field survey. Journal of environmental management, 113, pp.117-127.

https://nora.nerc.ac.uk/id/eprint/19534/1/N019534PP.pdf

PB13297, 2009. Safeguarding our soils a strategy for England, Defra, Nobel House, 17 Smith Square, London SW1P 3JR

Reynolds, B., Chamberlain, P.M., Poskitt, J., Woods, C., Scott, W.A., Rowe, E.C., Robinson, D.A., Frogbrook, Z.L., Keith, A.M., Henrys, P.A. and Black, H.I.J., 2013. Countryside Survey: National “Soil Change” 1978–2007 for Topsoils in Great Britain—acidity, carbon, and total nitrogen status. Vadose Zone Journal, 12(2).

Rossel, R. V., Behrens, T., Ben-Dor, E., Brown, D. J., Demattê, J. A. M., Shepherd, K. D., & Ji, W. (2016). A global spectral library to characterize the world's soil. Earth-Science Reviews, 155, 198-230.

Rowe, E.C., Emmett, B.A., Frogbrook, Z.L., Robinson, D.A. and Hughes, S., 2012. Nitrogen deposition and climate effects on soil nitrogen availability: influences of habitat type and soil characteristics. Science of the Total Environment, 434, pp.62-70. https://nora.nerc.ac.uk/id/eprint/16026/1/N016026PP.pdf

Ruehlmann, J., 2020. Soil particle density as affected by soil texture and soil organic matter: 1. Partitioning of SOM in conceptional fractions and derivation of a variable SOC to SOM conversion factor. Geoderma, p.114542.

Ruehlmann, J. and Körschens, M., 2020. Soil particle density as affected by soil texture and soil organic matter: 2. Predicting the effect of the mineral composition of particle-size fractions. Geoderma, p.114543.

Scott W.A. 2008. Statistical Report CS Technical Report No.4/07, Centre for Ecology and Hydrology, (Natural Environment Research Council)

Safanelli, J. L., Hengl, T., Parente, L. L., Minarik, R., Bloom, D. E., Todd-Brown, K., Gholizadeh, A., Mendes, W. de S., & Sanderman, J. (2025). Open Soil Spectral Library (OSSL): Building reproducible soil calibration models through open development and community engagement. PLOS ONE, 20(1), e0296545.

https://doi.org/10.1371/journal.pone.0296545

Sharififar, A., Singh, K., Jones, E., Ginting, F. I., & Minasny, B. (2019). Evaluating a low-cost portable NIR spectrometer for the prediction of soil organic and total carbon using different calibration models. Soil Use and Management, 35(4), 607–616.

https://doi.org/10.1111/sum.12537

Shepherd, K. D., Ferguson, R., Hoover, D., van Egmond, F., Sanderman, J., & Ge, Y. (2022). A global soil spectral calibration library and estimation service. Soil Security, 7, 100061.

https://www.sciencedirect.com/science/article/pii/S2667006222000284

Tang, Y., Jones, E., & Minasny, B. (2020). Evaluating low-cost portable near infrared sensors for rapid analysis of soils from Southeastern Australia. Geoderma Regional, 20. https://doi.org/10.1016/j.geodrs.2019.e00240

UK Centre for Ecology & Hydrology, Countryside Survey Environmental Zones, 2013 https://catalogue.ceh.ac.uk/documents/0cfd454a-d035-416c-80dc-803c65470ea2

UKCEH Soil Bank (uksoilbank@ceh.ac.uk):

https://catalogue.ceh.ac.uk/documents/d5a7b276-3fbc-4d12-b816-0f74a4692a33

Role Definitions

| V.O. Ebuele | Methodology, Investigation (ASD AgricSpec experiments); Formal Analysis Writing – Original Draft |
| --- | --- |
| D.A. Robinson | Supervision, Conceptualisation, Funding Acquisition, Review and Editing |
| S. Tandy | Supervision, Review, and Editing |
| I.L. Robinson | Supervision, Review and Editing |
| M. Brentegani | Preparation of soil samples for analysis |
| A. Tye | Methodology, loan of the ASD Agric Spec instrument |
| A. Keith | Review and Editing |
| C. Hermansen | Methodology, Investigation (FOSS D2500 experiments), Formal Analysis Writing – Original Draft |
| L. Wollesen de Jonge | Supervision, Review, and Editing |
| M. Zaresourmanabad | Formal Analysis of Raw Spectra Data |

Edinburgh

Lancaster

Bangor

Wallingford

Contact

enquiries@ceh.ac.uk

@UK_CEH

ceh.ac.uk

____

Bangor
UK Centre for Ecology & Hydrology
Environment Centre Wales
Deiniol Road
Bangor
Gwynedd LL57 2UW

+44 (0)1248 374500

Lancaster
UK Centre for Ecology & Hydrology
Lancaster Environment Centre
Library Avenue
Bailrigg
Lancaster LA1 4AP

+44 (0)1524 595800

Wallingford (Headquarters)
UK Centre for Ecology & Hydrology
Maclean Building
Benson Lane
Crowmarsh Gifford
Wallingford
Oxfordshire  OX10 8BB

+44 (0)1491 838800

Edinburgh
UK Centre for Ecology & Hydrology
Bush Estate
Penicuik
Midlothian EH26 0QB

+44 (0)131 4454343