# Supporting documentation to dataset: Plastic litter data collected in transect surveys in Santa Cruz, Galápagos,2023

# Summary

This dataset presents information on litter collected in 40 transects surveys conducted in rural, urban and protected areas on Santa Cruz island, Galápagos. Litter was collected between 8 and 18 July 2023 and data was subsequently analysed at the University of Exeter laboratory. Each transect survey covered an area of 200m2.

The dataset includes characteristics of the surveyed transects, location information, the number and weight of litter items collected by category and subcategory, data on human movement and the polymer type, colour and texture of litter. The data were collected to explore alternatives for reducing inland plastic waste through a circular economy approach. This work is part of a NERC funded project (NE/V005448/1), that aims to reduce plastic in the Galápagos Islands and the Eastern Pacific.

# File format, file name and structure

The dataset consists of a three CSV file entitled “Database_transects_general_info”, “Database_transects_count_weight” and “Database_transects_polymer_ID”.

The CSV file: “Database_transects_general_info” contains 18 columns. Column A (“Transect_number”) corresponds to the transect identifier.  Column B (“Date_of_litter_collection”) records the day on which litter was collected. Column C (“Transect_side”) indicates the side of the street or road surveyed (west, east, north or south). Column D and E provide the geographic coordinates of the transect (latitude and longitude). Column F (“Weather_conditons”) describes the weather conditions at the time of litter collection, columns G to Q define of the surveyed area and Column R (“Human_movement_number”) records the number of people, bikes, motorbikes and cars passing through a fixed point.  Additional details for selected variables are provided in Table 1.

Table 1. Description of variables in “Database_transects_general_info”.

| Variable/Column | Description | Units |
| --- | --- | --- |
| Transect_number | There are 40 transects in the dataset, each identified by a unique code starting from 1. | - |
| Date_of_litter_collection | It consists in the day on which litter was collected | dd/mm/yyyy |
| Transect_side | The midpoint of the transect was located at the kerbside. The side of the road was randomly selected, and could be east, west, north or south, depending on the road direction. This column indicates the side selected. | - |
| Coordinates_Y_Latitude | Latitude geographic coordinate expressed in decimal degrees (WGS84) | ° |
| Coordinates_X_Longitude | Longitude geographic coordinate expressed in decimal degrees (WGS84) | ° |
| Weather_conditons | Represents the weather conditions during the transect survey (cloudy, few clouds, light wind, moderate wind, light rain and sunny) | - |
| Columns G to Q | Defines the characteristics surveyed areas, which are presence of “Houses”, “Natural_area_abundant_vegetation_in_the_surrondings”, “Recreation_services”, “Farmland”, “Industries”, “Services_shops”, “Unmanaged_waste”, “Bike_path”, “Construction_site”, “Pavement” and “Highway”. (1 = present; 0 = absent) | - |
| Human_movement_number | Number of people, bikes, motorbikes and cars passing through a fixed point in the transect for a period of 19 hours. | Number |

The CSV file: “Database_transects_count_weight” consists of 5 columns. Column A (“Transect_number”) corresponds to the transect identifier.  Column B (“Category”) indicates the category under which litter was classified, there are in total 12 categories (construction and demolition material, e-waste, fishing gear, food plastic, glass, metal, organics, others, other plastics, paper, personal care products and plastic fragments). Column C (“Subcategory”) corresponds to the subcategories within each category. Column D (“Quantity_number”) records the number of items found, and Column E (“Weight_g”) records the total weight of the items. Additional details for selected variables are provided in Table 2.

Table 2. Description of variables in “Database_transects_count_weight”.

| Variable/Column | Description | Units |
| --- | --- | --- |
| Transect_number | There are 40 transects in the dataset, each identified by a unique code starting from 1. | - |
| Category | The category under which litter was classified, there are in total 12 categories (construction and demolition material, e-waste, fishing gear, food plastic, glass, metal, organics, others, other plastics, paper, personal care products and plastic fragments) | - |
| Subcategory | the subcategories within each category | - |
| Quantity_number | Corresponds to the number of items found in each transect for each category and subcategory | number |
| Weight_g | Corresponds to the weight of items found in each transect for each category and subcategory | g |

The CSV file: “Database_transects_polymer_ID” consists of 12 columns. Column A (“Transect_number”) corresponds to the transect identifier. Column C (“Category”) indicates the category under which the litter was classified, a total of 11 categories are included (construction and demolition material, e-waste, fishing gear, food plastic, glass, metal, organics, other plastics, paper, personal care products and plastic fragments). Column D (“Subcategory”) corresponds to the subcategories within each category. Column E and F record the texture and colour of the plastic. The remaining columns are labelled as follows: “PlasTELL_ID”, “PlasTELL_score", “FTIR_ATR_ID”, “FTIR-ATR_ID_Abbrev”, “FTIR_ATR_score”, and "Match_ID_Y_N”. The description and units of selected variables are provided bellow:

Table 3. Description of variables in “Database_transects_polymer_ID”.

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Transect_number | There are 40 transects in the dataset, each identified by a unique code starting from 1. | - |
| Sample_date | It consists in the day on which litter was collected | dd/mm/yyyy |
| Category | The category under which the litter was classified, a total of 11 categories are included (construction and demolition material, e-waste, fishing gear, food plastic, glass, metal, organics, other plastics, paper, personal care products and plastic fragments) | - |
| Subcategory | The subcategories within each category | - |
| Texture | Texture of the plastic (hard, medium, soft) | - |
| Colour | Colour of the plastic | - |
| PlasTELL_ID | Corresponds to the abbreviated polymer name identified by a portable plastic identification device (PlasTell NIR plastics scanner) | - |
| PlasTELL_score | The score represents the confidence of the polymer type identified based on the reference database. For this database it ranges from 0.3 to 3.75. | dimensionless |
| FTIR_ATR_ID | Corresponds to the full polymer name identified through Fourier transform infrared (FT-IR) spectrometer with an attenuated total reflection (ATR) attachment. | - |
| FTIR_ATR_ID_Abbrev | Corresponds to the abbreviated polymer name identified through Fourier transform infrared (FT-IR) spectrometer with an attenuated total reflection (ATR) attachment. | - |
| FTIR_ATR_score | Score obtained during polymer identification. Values closer to 1 indicate a better match between the sample spectrum and the reference library. | Dimensionless |
| Match_ID_Y_N | Indicates whether the PlasTELL polymer identification matches the FTIR-ATR result (Y=Yes, N=No). | - |

Polymer types not identified by PlasTELL are labeled as NI, while missing data values are identified by NA.

# Spatial coverage

Data were collected from 40 transects on Santa Cruz Island located in the Galápagos Archipelago, in Ecuador, South America. The location of each transect can be found in the CSV file: “Database_transects_general_info” in the column D (Coordinates_Y_Latitude) and E (Coordinates_X_Longitude).

# Temporal coverage and resolution

Data were collected between 08/07/2023 to the 18/07/2023, with a single sampling event conducted at each transect. Polymer analysis was performed after the collection of plastics between August 2023 and May 2024.

# Sampling, collection and analysis

Litter was collected in 40 transects selected randomly using the geoprocessing analysis tool “create random points” in ArgGIS Pro 3.0.1.: ten transects per population count categories (high, medium-high, medium and low derived from the LandScan dataset: https://landscan.ornl.gov/). Transects consisted of a 100mx2m area, where all the visible litter was collected, classified, counted and weighed.

Then the polymer identification was performed with two instruments:  Spectrum Two PerkinElmer Fourier transform infrared (FT-IR) spectrometer with an attenuated total reflection (ATR) (measuring in a wavelength range from 650 to 4000 to cm-1 and a resolution 4cm-1) and a portable plastic identification device (PlasTell NIR plastics scanner, desktop version, Matoha Instrumentation Ltd.) equipped with a transmittance-reflectance NIR spectroscopy, 1550-1950 nm - 2NIR lamps built-in.

Human movement was recorded using Pyro Evo sensors, which are passive infrared sensors. These sensors were place in a fixed point in the transect surveys for 19 hours.

# Quality control

All equipment (FTIR-ATR and PlasTELL) was thoroughly cleaned before each use to avoid contamination and ensure accurate polymer identification. Polymer identification was conducted at the University of Exeter laboratories under controlled conditions.  The database was carefully reviewed to identify and correct any errors.

# Miscellaneous

This data is used in a manuscript which is in the process to be published.