# Loch Leven monitoring data (water quality, chemistry, crustacean zooplankton), 2025

# Supporting Information

## Abstract

This is an annual publication of monitoring data from Loch Leven, Scotland, including data on water quality, water chemistry and counts of crustacean & zooplankton, as well as supporting environmental data recorded in the field. The data were collected as part of the long-term monitoring programme of this lowland loch, and monitoring is generally performed every 2 weeks.

This programme began in 1968 and is still underway at the time of writing. Sampling and analyses have been conducted by staff from various former institutions of the Natural Environment Research Council (NERC), and most-recently by the NERC / UK Centre for Ecology & Hydrology This document describes the nature of the data, sampling and analysis methods used to obtain them, the data processing techniques used and the format of the output file. The data collection and processing was carried out under project UK-SCAPE WP7 LEVEN.

## Sampling regime

In general, sampling has been conducted on a fortnightly basis. On each sampling occasion, two main sites were visited. These are the Reed Bower and Sluices sites (see Table 1, below). The Reed Bower site is only accessible by boat, whereas the Sluices site is either accessed by boat, or when conditions don't allow, a representative sample is taken on foot from the nearby lake outlet. Although data is recorded as ‘Outlet’ rather than ‘Sluices’ when the boat isn’t used, these values have all been referenced as ‘Sluices’ (L / Sl8) in this dataset for ease of use.

Several circumstances can prevent sampling on occasion. These include resource constraints, bad weather, or ice cover. When this has prevented safe use of a boat, only the Sluices site could be sampled and in extreme cases, neither site can be visited.

Measurements were made either in the field at the time of sampling or, later, in a laboratory using samples collected in the field. The details of each type of measurement are detailed in this document.

## List of Sites

Table 1 and Figure 1 show a list of the sampling sites represented in the dataset by their corresponding Site IDs. Sites are recorded in the data using the last value in the column name, after the last underscore. These codes are used throughout the long-term monitoring programme at Loch Leven and refer to the Harbour (Site_H), Reed Bower (Site_RB) and the Sluices (Site_L), with some alternative IDs used in some data shown in Table 1.

Table 1 List of sampling sites represented in the dataset, showing their ID, name and location (as UK National Grid Easting and Northing, epsg:27700).

| Site ID |  | Site Name | Easting | Northing |
| --- | --- | --- | --- | --- |
| H |  | Harbour | 312251 | 701670 |
| RB / RB5 |  | Reed Bower | 313568 | 701141 |
| L / Sl8 |  | Sluices / Outflow | 317004 | 699380 |

Figure 1 Map of Loch Leven showing the location of the three monitoring sites, with names / IDs.

## Sampling, measurement and analysis methods

Measurements of conductivity, pH, temperature and Secchi depth were conducted in situ. Each of these were generally measured at every site sampled, except Secchi depth, which was measured at Reed Bower. For the remainder of determinands water samples were taken at each site and analysed later.

Conductivity, pH and temperature were measured using hand-held electronic probes.

Water clarity was measured as Secchi depth, which is the depth to which a Secchi disc (a disc of 25 cm diameter, divided into quadrants, which are alternately black and white) can be lowered before it is no longer visible.

In general, on each sampling occasion, duplicate 2 L water samples were collected from each site. At the Reed Bower site, an integrated sample was collected using a PVC tube with an internal diameter of 25 mm, which was weighted at one end. This tube was lowered into the water until the weighted end was within 0.25 m of the lake bottom, filling as it was lowered. The weighted end was then lifted out of the lake while ensuring that the other end of the tube remained stationary. This ensured that the sample was collected from the entire water column. At the Sluices site, water was collected using a bottle that was held 10 cm below the surface.

On return to the laboratory, samples were shaken and sub-samples of filtered (using Whatman® GF/C filters, within 6 hours of collection) or unfiltered water were taken from each duplicate site sample for soluble reactive phosphorus (SRP) and total soluble phosphorus (TSP; filtered and frozen), soluble reactive silicate (filtered and stored in fridge), total diatom silica (unfiltered and stored in fridge) and total phosphorus (TP; unfiltered and frozen) analyses. Samples for chlorophylla analysis were prepared for each site by filtering 400 ml of lake water through a GF/C (Whatman®) grade filter. The filter was stored (frozen) in a 15 ml centrifuge tube until analysis.

SRP concentrations were determined following the method of Murphy & Riley (1962). This method uses a reagent of ammonium molybdate, potassium antimony tartrate, and ascorbic acid in 1 M sulphuric acid, which reacts with the phosphate ion to form a phospho-molybdenum blue complex. Concentration was determined by measuring the absorption of red light (882 nm) and comparing this against known standards.

TP concentrations were determined on unfiltered samples, which were digested using a solution of sulphuric acid and potassium persulphate that converted all forms of phosphorus to SRP. This was then measured in a similar way to that described above. The method used was as described for TP by Wetzel and Likens (2000), with an added acidification step (0.1 ml of 30% H2SO4 was added to the samples before addition of persulfate). TSP is determined in the same way as TP but using a filtered sample.

Soluble reactive silica and total diatom silica concentrations in water samples were determined according to the colorimetric method outlined in Golterman et al. (1978), Paasche (1980) and Kirka (2000). The method uses acidic ammonium molybdate to form a silicomolybdate complex that is reduced with ascorbic acid to produce blue-coloured solution. Oxalic acid was added to eliminate the interference from any phospho-molybdate complex formation. Concentrations were determined by measuring the absorbance at 810 nm and comparing this against know standards using a Perkin Elmer Lambda 365 spectrophotometer. Prior to the analysis, 1 mL of each unfiltered sample was digested by adding sodium carbonate solution (Na2CO3) and heating at 120oC for 30 minutes. The ‘gentle’ digestion of unfiltered samples using Na2CO3 converts the opaline silica of diatom frustules to a soluble form that can then be measured using the method for soluble reactive silica. The digestion is not considered strong enough to include any suspended sand particles from the sediments. NB silica data has been added for 2025, but not for previous years. A silica dataset that aligns to previous data publications is earmarked for publication on EIDC soon.

Weather conditions (cloud cover, wind direction and wind force) are recorded at Reed Bower, when possible. The wind force value is calculated using the ‘Effects at sea’ description in Table 2. The recorded value in the dataset corresponds to the first column, and associated values can be ascertained from this.

Table 2 Wind Force lookup table with associated wind speeds and descriptions.

| No. | Knots | Mph | Description | Effects at sea |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | Calm | Sea like a mirror |
| 1 | 1-3 | 1-3 | Light air | Ripples but no foam crests |
| 2 | 4-6 | 4-7 | Light breeze | Small wavelets |
| 3 | 7-10 | 8-12 | Gentle breeze | Large wavelets; crests not breaking |
| 4 | 11-16 | 13-18 | Moderate wind | Numerous whitecaps; waves 1-4ft high |
| 5 | 17-21 | 19-24 | Fresh wind | Many whitecaps, some spray; waves 4-8 ft high |
| 6 | 22-27 | 25-31 | Strong wind | Whitecaps everywhere; larger waves 8-13 ft high |
| 7 | 26-33 | 32-38 | V. strong wind | White foam from waves is blown in streaks; waves 143-20 ft high |
| 8 | 34-40 | 39-46 | Gale | Edges of wave crests break into spindrift |
| 9 | 41-47 | 47-54 | Severe gale | High waves; sea begins to roll; spray reduces visibility; 20 ft waves |
| 10 | 48-55 | 55-63 | Storm | V. high waves 2—30 ft; blowing foam gives sea white appearance |
| 11 | 56-63 | 64-72 | Severe storm | Exceptionally high waves; 30-45 ft high |

## Sampling, measurement and analysis – References

Golterman, H.L., Clymo, R.S., Ohnstad, M.A.M. (1978). Methods for Physical and Chemical Analysis of Fresh Waters. IBP Handbook No. 8. Blackwell, Oxford.

Kirka A. 2000. Loch Leven Field Notes. CEH Edinburgh (unpublished).

Murphy, J. & Riley, J.P. (1962). A modified single solution method for the determination of phosphate in natural waters. Analytica Chimica Acta 27, 31-36.

Paasche E. (1980). Silicon content of five marine plankton diatom species measured with a rapid filter method. Limnol. Oceanogr. Vol. 25, No.3, pp 474-480.

Wetzel, R.G. & Likens, G.E. (2000). Inorganic nutrients: nitrogen, phosphorus and other nutrients. Limnological Analyses. Springer-Verlag, New York.

## Sampling, measurement and analysis - Further reading

Additional information can be gained from a special issue of the journal Hydrobiologia, and a report written for Scottish Natural Heritage.

Dudley, B.J., Spears, B.M., Carvalho, L., Gunn, I.D.M. & May, L., 2012. Water quality monitoring at Loch Leven 2008-2010 – Report of results. Scottish Natural Heritage Commissioned Report No. 511

Spears, B. M. & May, L. (editors) 2012. Loch Leven: 40 years of scientific research. Developments in Hydrobiology 218. Springer, Dordrecht.

## Crustacean & Zooplankton

The dataset comprises densities of crustacean zooplankton collected from two separate locations (Reed Bower and Sluices) at Loch Leven by staff at based in Edinburgh, as part of their long-term monitoring programme of the lake. The dataset shows the number of individuals per litre (ind/l) of the following crustacean zooplankton taxa recorded in each sample: Daphnia hyalina; Eudiaptomus gracilis (adults + copepodites I-V); Eudiaptomus nauplii; Cyclopidae (i.e. total number of Cyclopidae (including Cyclops abyssorum and C. vicinus)  adults and copepodites I-V); Cyclops abyssorum (adults + copepodites I-V); Cyclops vicinus (adults + copepodites I-V); Cyclopidae nauplii; Bosmina longirostris; Leptodora kindtii; Bythotrephes longimanus; Chydoridae.

Values of zero indicate that the species was not recorded in the sample at the sampling and analytical resolution undertaken, but does not necessarily imply complete absence from the loch. NAs indicate missing values.

NB As of the 2022 dataset, a number of column names have been slightly changed, although the order remains the same. This was due to a number of small errors being spotted, updating names to more current usage and consistency of punctuation. Two new taxa were recorded and added as columns at the end of the dataset for both sites – Ceriodaphnia sp. and Polyphemus pediculus.

At the Reed Bower sample site, crustacean zooplankton samples were collected using a 4 m angled net haul (mesh size 120 µm). At the Sluices sample site, crustacean zooplankton samples were collected by passing 30 litres of surface water through a net (mesh size 120 µm). Immediately after collection, the crustacean zooplankton samples were preserved in 4% formaldehyde solution.

In the laboratory, each sample was placed in a glass vessel and made up to a final volume of 250 ml with distilled water. The sample was thoroughly mixed, to distribute the animals randomly, and then sub-sampled with a Stempel pipette (volume 5 ml). The animals present in each subsample were identified (Dussart and Defaye 1995; Einsle 1996; Flöβner and Kraus, 1986; Harding and Smith 1974; Scourfield and Harding 1966) and counted under a low power binocular microscope. In all cases, four sub-samples were counted. The preserved freshwater crustacean zooplankton were, generally, identified to species level, wherever practical to do so. No specimen was identified beyond the level justified by its condition of preservation or stage of maturity (as recommended in the appropriate key). The sub-sample counts were converted to numbers of individuals per litre (ind/l) using appropriate multiplication factors. After counting, the samples were reconstituted and archived.

## Crustacean & Zooplankton – References

Dussart, B. H. & Defaye, D. (1995). Copepoda: Introduction to the Copepoda. Guides to the identification of the microinvertebrates of the continental waters of the world (H.J.F. Dumont, Ed.). Vol 7. SPB Publishing, Amsterdam.

Einsle, U. (1996). Copepoda: Cyclopoida: Genera Cyclops, Megacyclops, Acanthocyclops. Guides to the identification of the microinvertebrates of the continental waters of the world (H.J.F. Dumont, Ed.). Vol 10. SPB Publishing, Amsterdam.

Flöβner, D. & Kraus, K. (1986). On the taxonomy of the Daphnia hyalina-galeata complex (Crustacea:Cladocera). Hydrobiologia, 137, 97-115.

Harding, J. P. & Smith, W. A., (1974). A key to the British freshwater cyclopoid and calanoid copepods. Scientific Publications of the Freshwater Biological Association, No. 18.

Scourfield, D.J. & Harding, J.P. (1966). A key to the British species of freshwater Cladocera. Scientific Publications of the Freshwater Biological Association, No. 5.

## Data processing

An import script written in R was used to process the data before entering into a database. The script makes some decisions in order to process the data as sensibly as possible. They are detailed here:

Wind Force
Occasionally multiple values are input in the format '4-5', but the first value is taken for the database.

Probe Replicate Values
Applies to conductivity, pH and DO values (and corresponding temperatures) taken from the Hach probe*. Recorded values are included if they fall within Median ± MAD (median absolute deviation); this method means if only 1 or 2 replicates are taken, none are thrown out. It also means that if 3 replicates are taken and 2 are the same value, the other value is always thrown out. Details on MAD can be found here: https://en.wikipedia.org/wiki/Median_absolute_deviation

NB pH values are converted to 10^pH before the median / MAD are calculated and the resulting mean value is log10 transformed back again. A mean is then taken of the remaining values to give a single value - pH / DO concentration are rounded to 2dp and conductivity, DO % and all temperatures are rounded to 1dp.

*As of 21/12/2022, the Hach probe is now a new model, with the HQ30d being replaced by the HQ4300. Inter-calibration was performed to maintain consistency in measurements. Column names now refer to the HQ4300, but column order is unaffected.

Quality Assurance
Both an automated and manual check was performed on all data to look for any errors in data entry or values reported outside the expected range. Any unusual values were then investigated and corrected where appropriate.

## Format of stored data

Data is stored in a single comma-separated values (CSV) file. This file has columns which explain the determinand including site conditions, lake chemistry and crustacean / zooplankton counts, with units / sites shown where relevant. Missing data values (when a reading wasn’t taken / recorded) are marked as NA (‘Not Available’) throughout, as used in a number of data standards.

## Column descriptions

All column names contain basic information on what is being measured, at what site and in what units. Table 3 (below) shows this information in more detail, giving full descriptions of everything in the dataset and should be seen as the core reference for what each column represents.

NB As of the 2023 dataset, columns have been made internally consistent and more encoding-friendly. All references to litres are now ‘L’,  µ is now ‘u’ and ° is now ‘deg’. Additionally, Ceriodaphnia_sp. are now recorded. All these changes are reflected in Table 3.

Table 3 Detailed information on column headings, giving descriptions of the determinand, site and units.

| Column Name | Description | Site | Units |
| --- | --- | --- | --- |
| Date | Sampling date | All | date |
| Week_of_Year | Week in the year | All | (1-52) |
| Water_Level_cm_Site_H | Distance between the top of the harbour wall and the loch level | Harbour | centimetres |
| Cloud_cover_Eighths_Site_RB | Cloud cover (i.e. 8 = totally overcast) | Reed Bower | eighths |
| Wind_Direction_Site_RB | Main direction that wind is travelling from | Reed Bower | compass points |
| Wind_Force_Beaufort_scale_Site_RB | Wind force | Reed Bower | Beaufort scale |
| Depth_m_Site_RB | Depth of water at sampling site (NB not sampling depth) | Reed Bower | metres |
| Secchi_m_Site_RB | Secchi depth at sampling site | Reed Bower | metres |
| Conductivity_uS/cm_Hach_HQ4300_Site_RB | Conductivity of water sample | Reed Bower | microsiemens / centimetre |
| Temperature_degC_Hach_ HQ4300_(cond)_Site_RB | Temperature of water sample for conductivity measurement | Reed Bower | degrees Celsius |
| Conductivity_uS/cm_Hach_ HQ4300_Site_L | Conductivity of water sample | Sluices | microsiemens / centimetre |
| Temperature_degC_Hach_ HQ4300_(cond)_Site_L | Temperature of water sample for conductivity measurement | Sluices | degrees Celsius |
| DO_mg/L_Hach_ HQ4300_Site_RB | Concentration of dissolved oxygen of water sample | Reed Bower | micrograms / litre |
| DO_%_Hach_ HQ4300_Site_RB | Percentage of dissolved oxygen of water sample | Reed Bower | percentage |
| Temperature_degC_Hach_ HQ4300_(DO)_Site_RB | Temperature of water sample for dissolved oxygen measurement | Reed Bower | degrees celsius |
| DO_mg/L_Hach_ HQ4300_Site_L | Concentration of dissolved oxygen of water sample | Sluices | micrograms / litre |
| DO_%_Hach_ HQ4300_Site_L | Concentration of dissolved oxygen of water sample (percentage) | Sluices | percentage |
| Temperature_degC_Hach_ HQ4300_(DO)_Site_L | Temperature of water sample for dissolved oxygen measurement | Sluices | degrees celsius |
| pH_Hach_ HQ4300_Site_RB | pH of water sample | Reed Bower | pH |
| Temperature_degC_Hach_ HQ4300_(pH)_Site_RB | Temperature of water sample for pH measurement | Reed Bower | degrees celsius |
| pH_Hach_ HQ4300_Site_L | pH of water sample | Sluices | pH |
| Temperature_degC_Hach_ HQ4300_(pH)_Site_L | Temperature of water sample for pH measurement | Sluices | degrees celsius |
| TP_ugP/L_Site_RB1 | Concentration of total phosphorus of water sample (replicate 1) | Reed Bower | micrograms of phosphorus / litre |
| TP_ugP/L_Site_RB2 | Concentration of total phosphorus of water sample (replicate 2) | Reed Bower | micrograms of phosphorus / litre |
| TP_ugP/L_Site_L1 | Concentration of total phosphorus of water sample (replicate 1) | Sluices | micrograms of phosphorus / litre |
| TP_ugP/L_Site_L2 | Concentration of total phosphorus of water sample (replicate 2) | Sluices | micrograms of phosphorus / litre |
| SRP_ugP/L_Site_RB1 | Concentration of soluble reactive phosphorus of water sample (replicate 1) | Reed Bower | micrograms of phosphorus / litre |
| SRP_ugP/L_Site_RB2 | Concentration of soluble reactive phosphorus of water sample (replicate 2) | Reed Bower | micrograms of phosphorus / litre |
| SRP_ugP/L_Site_L1 | Concentration of soluble reactive phosphorus of water sample (replicate 1) | Sluices | micrograms of phosphorus / litre |
| SRP_ugP/L_Site_L2 | Concentration of soluble reactive phosphorus of water sample (replicate 2) | Sluices | micrograms of phosphorus / litre |
| TSP_ugP/L_Site_RB1 | Concentration of total soluble phosphorus of water sample (replicate 1) | Reed Bower | micrograms of phosphorus / litre |
| TSP_ugP/L_Site_RB2 | Concentration of total soluble phosphorus of water sample (replicate 2) | Reed Bower | micrograms of phosphorus / litre |
| TSP_ugP/L_Site_L1 | Concentration of total soluble phosphorus of water sample (replicate 1) | Sluices | micrograms of phosphorus / litre |
| TSP_ugP/L_Site_L2 | Concentration of total soluble phosphorus of water sample (replicate 2) | Sluices | micrograms of phosphorus / litre |
| chla_ug/L_Site_RB1 | Concentration of chlorophyll a of water sample (replicate 1) | Reed Bower | micrograms / litre |
| chla_ug/L_Site_RB2 | Concentration of chlorophyll a of water sample (replicate 2) | Reed Bower | micrograms / litre |
| chla_ug/L_Site_L1 | Concentration of chlorophyll a of water sample (replicate 1) | Sluices | micrograms / litre |
| chla_ug/L_Site_L2 | Concentration of chlorophyll a of water sample (replicate 2) | Sluices | micrograms / litre |
| Daphnia_hyalina_ind/L_Site_RB | Density of Daphnia hyalina individuals in water sample | Reed Bower | individuals / litre |
| Daphnia_hyalina_ind/L_Site_L | Density of Daphnia hyalina individuals in water sample | Sluices | individuals / litre |
| Eudiaptomus_gracilis_(adults_+_copepodites_I-V)_ind/L_Site_RB | Density of Eudiaptomus gracilis individuals in water sample | Reed Bower | individuals / litre |
| Eudiaptomus_gracilis_(adults_+_copepodites_I-V)_ind/L_Site_L | Density of Eudiaptomus gracilis individuals in water sample | Sluices | individuals / litre |
| Eudiaptomus_nauplii_ind/L_Site_RB | Density of Eudiaptomus nauplii individuals in water sample | Reed Bower | individuals / litre |
| Eudiaptomus_nauplii_ind/L_Site_L | Density of Eudiaptomus nauplii individuals in water sample | Sluices | individuals / litre |
| Cyclopidae_(total_no._of_Cyclopidae_adults_&_copepodites_I-V_incl_C._abyssorum_&_C._vicinus)_ind/L_Site_RB | Density of Cyclopidae individuals in water sample | Reed Bower | individuals / litre |
| Cyclopidae _(total_no._of_Cyclopidae_adults_&_copepodites_I-V_incl._C._abyssorum_&_C._vicinus)_ind/L_Site_L | Density of Cyclopidae individuals in water sample | Sluices | individuals / litre |
| Cyclops_abyssorum_(adults_+_copepodites_I-V)_ind/L_Site_RB | Density of Cyclops abyssorum individuals in water sample | Reed Bower | individuals / litre |
| Cyclops_abyssorum_(adults_+_copepodites_I-V)_ind/L_Site_L | Density of Cyclops abyssorum individuals in water sample | Sluices | individuals / litre |
| Cyclops_vicinus_(adults_+_copepodites_I-V)_ind/L_Site_RB | Density of Cyclops vicinus individuals in water sample | Reed Bower | individuals / litre |
| Cyclops_vicinus_(adults_+_copepodites_I-V)_ind/L_Site_L | Density of Cyclops vicinus individuals in water sample | Sluices | individuals / litre |
| Cyclopidae_nauplii_ind/L_Site_RB | Density of Cyclopidae nauplii individuals in water sample | Reed Bower | individuals / litre |
| Cyclopidae_nauplii_ind_/L_Site_L | Density of Cyclopidae nauplii individuals in water sample | Sluices | individuals / litre |
| Bosmina_longirostris_ind/L_Site_RB | Density of Bosmina longirostris individuals in water sample | Reed Bower | individuals / litre |
| Bosmina_longirostris_ind/L_Site_L | Density of Bosmina longirostris individuals in water sample | Sluices | individuals / litre |
| Leptodora_kindtii_ind/L_Site_RB | Density of Leptodora kindtii individuals in water sample | Reed Bower | individuals / litre |
| Leptodora_kindtii_ind/L_Site_L | Density of Leptodora kindtii individuals in water sample | Sluices | individuals / litre |
| Bythotrephes_longimanus_ind/L_Site_RB | Density of Bythotrephes longimanus individuals in water sample | Reed Bower | individuals / litre |
| Bythotrephes_longimanus_ind/L_Site_L | Density of Bythotrephes longimanus individuals in water sample | Sluices | individuals / litre |
| Chydoridae_ind/L_Site_RB | Density of Chydoridae individuals in water sample | Reed Bower | individuals / litre |
| Chydoridae_ind/L_Site_L | Density of Chydoridae individuals in water sample | Sluices | individuals / litre |
| Ceriodaphnia_sp._ind/L_Site_RB | Density of Ceriodaphnia sp. individuals in water sample | Reed Bower | individuals / litre |
| Ceriodaphnia_sp._ind/L_Site_L | Density of Ceriodaphnia sp. individuals in water sample | Sluices | individuals / litre |
| Polyphemus_pediculus_ind/L_Site_RB | Density of Polyphemus pediculus individuals in water sample | Reed Bower | individuals / litre |
| Polyphemus_pediculus_ind/L_Site_L | Density of Polyphemus pediculus individuals in water sample | Sluices | individuals / litre |
| SRSi_mgSi/L_Site_RB1 | Concentration of soluble reactive silica of water sample (replicate 1) | Reed Bower | milligrams of silica / litre |
| SRSi_mgSi/L_Site_RB2 | Concentration of soluble reactive silica of water sample (replicate 2) | Reed Bower | milligrams of silica / litre |
| SRSi_mgSi/L_Site_L1 | Concentration of soluble reactive silica of water sample (replicate 1) | Sluices | milligrams of silica / litre |
| SRSi_mgSi/L_Site_L2 | Concentration of soluble reactive silica of water sample (replicate 2) | Sluices | milligrams of silica / litre |
| TSi_mgSi/L_Site_RB1 | Concentration of total diatom silica of water sample (replicate 1) | Reed Bower | milligrams of silica / litre |
| TSi_mgSi/L_Site_RB2 | Concentration of total diatom silica of water sample (replicate 2) | Reed Bower | milligrams of silica / litre |
| TSi_mgSi/L_Site_L1 | Concentration of total diatom silica of water sample (replicate 1) | Sluices | milligrams of silica / litre |
| TSi_mgSi/L_Site_L2 | Concentration of total diatom silica of water sample (replicate 2) | Sluices | milligrams of silica / litre |