# Title

Daily peat surface motion data from 51 locations in England (2022 – 2025)

# Summary

This dataset comprises daily peat surface level, water table depth, soil temperature and relative humidity. Collected as part of the England Peat Map project to quantify peat surface motion across the range of peat habitats across England. Instruments were installed between April 2022 and September 2023. Data presented are the mean of 2 hourly raw data presented as a daily average. The dataset contains data from 51 peatland sites across England, collected using fixed point photography between April 2022 and December 2025.

# File format

csv

# File names and/or naming convention

England_peat_surface_motion_2022-2025.csv

# Nature and units of recorded values

Table 1. Description of variables in England_peat_surface_motion_2022-2025.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| id_data | Unique row ID number |  |
| camera_name | Name of camera location in the format PeatCam_nnn |  |
| date_now | Date of measurement | DD/MM/YYYY |
| air_temperature | Mean daily temperature (measured inside the camera unit) | oC |
| air_pressure | Mean daily atmospheric pressure (measured inside the camera unit, but approximately analogous to external atmospheric pressure) | hPa |
| relative_humidity | Mean daily relative humidity (measured inside the camera unit, but approximately analogous to external relative humidity) | % |
| soil_temp | Soil temperature at 10 cm below the soil surface | oC |
| wtd_cm | Calibrated water table depth calculated from water pressure. 0 is the soil surface, negative values are water levels below the surface. | cm |
| peat_motion_cm | Peat surface motion relative to an arbitrary 0 (usually at time 0). Missing data represented by NA | cm |

Table 2 gives the location and broad peat condition for each measurement location. This can be copied into an Excel file if the location is needed for further analysis.

Table 2: Site locations and peat condition

| Camera_name | Camera_region | Latitude | Longitude | Co-ordinate reference system | Peat condition |
| --- | --- | --- | --- | --- | --- |
| PeatCam_001 | Humberhead | 53.45704 | -0.91411 | WGS84 | Cropland deep drained |
| PeatCam_002 | Humberhead | 53.45921 | -0.90859 | WGS84 | Cropland deep drained |
| PeatCam_003 | Humberhead | 53.45676 | -0.91053 | WGS84 | Cropland deep drained |
| PeatCam_004 | North Pennines | 54.69595 | -2.38767 | WGS84 | Calluna dominated bog |
| PeatCam_005 | North Pennines | 54.70206 | -2.20507 | WGS84 | Calluna dominated bog |
| PeatCam_006 | North Pennines | 54.70919 | -2.21424 | WGS84 | Eroded bog |
| PeatCam_007 | North Pennines | 54.46054 | -1.90964 | WGS84 | Molinia dominated bog |
| PeatCam_008 | North Pennines | 54.45928 | -1.9227 | WGS84 | Molinia dominated bog |
| PeatCam_010 | East Anglian Fens | 52.33139 | 0.2245 | WGS84 | Cropland deep drained |
| PeatCam_011 | East Anglian Fens | 52.48587 | -0.18964 | WGS84 | Grassland intermediate drained |
| PeatCam_012 | East Anglian Fens | 52.32218 | 0.230662 | WGS84 | Cropland deep drained |
| PeatCam_013 | Somerset Levels | 51.17815 | -2.82711 | WGS84 | Grassland shallow drained |
| PeatCam_014 | Somerset Levels | 51.18448 | -2.82882 | WGS84 | Grassland shallow drained |
| PeatCam_015 | Somerset Levels | 51.10921 | -2.86116 | WGS84 | Tall vegetation dominated fen |
| PeatCam_016 | Somerset Levels | 51.02397 | -2.906 | WGS84 | Grassland shallow drained |
| PeatCam_017 | Somerset Levels | 51.17531 | -2.89708 | WGS84 | Grassland shallow drained |
| PeatCam_018 | North Pennines | 54.69378 | -2.39237 | WGS84 | Eroded bog |
| PeatCam_019 | Lake District | 54.30453 | -2.61233 | WGS84 | Scrub dominated bog |
| PeatCam_020 | North Pennines | 54.69644 | -2.15072 | WGS84 | Molinia dominated bog |
| PeatCam_021 | North Pennines | 54.69566 | -2.15899 | WGS84 | Eroded bog |
| PeatCam_022 | Lake District | 54.30309 | -2.61265 | WGS84 | Forest deep drained |
| PeatCam_023 | Kent | 51.24581 | 1.343079 | WGS84 | Tall vegetation dominated fen |
| PeatCam_024 | Kent | 51.24452 | 1.34502 | WGS84 | Tall vegetation dominated fen |
| PeatCam_025 | West Midlands | 52.91647 | -2.76039 | WGS84 | Dry grass/scrub dominated bog |
| PeatCam_026 | West Midlands | 52.91933 | -2.76526 | WGS84 | Sphagnum dominated bog |
| PeatCam_027 | West Midlands | 52.77962 | -2.34932 | WGS84 | Tall vegetation dominated fen |
| PeatCam_028 | West Midlands | 52.77572 | -2.33029 | WGS84 | Grassland shallow drained |
| PeatCam_029 | Norfolk | 52.38137 | 1.048159 | WGS84 | Grassland shallow drained |
| PeatCam_030 | Norfolk | 52.67764 | 1.502556 | WGS84 | Tall vegetation dominated fen |
| PeatCam_031 | Norfolk | 52.38004 | 1.048639 | WGS84 | Wet woodland |
| PeatCam_032 | Norfolk | 52.68334 | 1.50258 | WGS84 | Wet woodland |
| PeatCam_033 | East Anglian Fens | 52.54955 | 0.515002 | WGS84 | Grassland intermediate drained |
| PeatCam_034 | Norfolk | 52.53902 | 1.511829 | WGS84 | Grassland shallow drained |
| PeatCam_035 | Norfolk | 52.67961 | 1.498161 | WGS84 | Short-vegetation dominated fen |
| PeatCam_036 | Norfolk | 52.69367 | 1.452777 | WGS84 | Wet woodland |
| PeatCam_037 | Norfolk | 52.6966 | 1.454906 | WGS84 | Short-vegetation dominated fen |
| PeatCam_038 | Norfolk | 52.71888 | 1.508334 | WGS84 | Short-vegetation dominated fen |
| PeatCam_039 | Norfolk | 52.75242 | 1.496604 | WGS84 | Grassland deep drained |
| PeatCam_040 | Norfolk | 52.72109 | 1.511803 | WGS84 | Wet woodland |
| PeatCam_041 | Norfolk | 52.78206 | 1.570131 | WGS84 | Scrub-dominated fen |
| PeatCam_042 | Lake District | 54.23027 | -2.83539 | WGS84 | Molinia dominated bog |
| PeatCam_043 | Lake District | 54.25034 | -2.82742 | WGS84 | Molinia dominated bog |
| PeatCam_045 | Norfolk | 52.6045 | 1.453971 | WGS84 | Short-vegetation dominated fen |
| PeatCam_046 | Norfolk | 52.59366 | 1.468797 | WGS84 | Grassland shallow drained |
| PeatCam_047 | Border Mires | 55.01993 | -2.52816 | WGS84 | Molinia dominated bog |
| PeatCam_048 | Border Mires | 55.02148 | -2.52746 | WGS84 | Sphagnum dominated bog |
| PeatCam_049 | Border Mires | 55.02251 | -2.52169 | WGS84 | Calluna dominated bog |
| PeatCam_050 | Border Mires | 55.10532 | -2.48902 | WGS84 | Calluna dominated bog |
| PeatCam_051 | Border Mires | 55.10991 | -2.48458 | WGS84 | Calluna dominated bog |
| PeatCam_052 | Border Mires | 55.1677 | -2.46017 | WGS84 | Sphagnum dominated bog |
| PeatCam_053 | Border Mires | 55.16759 | -2.46269 | WGS84 | Eriophorum dominated bog |

# Spatial coverage

England wide

Bounding box co-ordinates: southwest 49.968006, -5.976563, northest 55.518525, 1.801758

CRS: WGS84

# Temporal coverage and resolution

April 2022 – Dec 2025

Daily average data from 2 hourly monitoring values.

# Collection/Generation/Transformation methods

Peat surface motion was measured using 2 hourly image collection against a fixed point marker (see (Evans et al. 2021) for more details).  A camera on a floating platform took pictures of a fixed point marker, and image analysis code converted these images into a time series of vertical motion. Water table depth was measured using a calibrated pressure sensor that had been laboratory calibrated at depths of 200, 150, 100 and 50 cm prior to installation. Soil temperature was measured at 10 cm depth.

Images and data were processed daily and appended to master files by site number. Automated QA removed images that were of insufficient sharpness (due to precipitation or mist / fog), and updated step changes following site visits, and manual QA checks were carried out by project staff.

# Quality control

Periods of missing data could relate to a number of factors including: lack of mobile signal for telemetry of data (though these gaps were infilled where data was stored locally), battery failure, and flooding of instruments.

Manual QA detected where there were unusual data records (unusually high or low motion values) and these were checked against the original images to determine the correct value. One particular issue relates to the freeze-thaw action in the soil, which in a few cases dislodged the measurement platform leading to a step in the peat motion time series. These were manually corrected.

# References

Evans, C. D., N. Callaghan, A. Jaya, A. Grinham, S. Sjogersten, S. E. Page, M. E. Harrison, K. Kusin, L. K. Kho, M. Ledger, S. Evers, Z. Mitchell, J. Williamson, A. D. Radbourne, and A. J. Jovani-Sancho. 2021. A Novel Low-Cost, High-Resolution Camera System for Measuring Peat Subsidence and Water Table Dynamics. Frontiers in Environmental Science 9.