Hare Moss peatland stream water chemistry data from the MERLIN project, 2024-2025 – Supporting information

Authors: Alanna Grant, Linda Armstrong, Anna Belcher, Ezra Kitson, Alice MacSporran, Rebecca McKenzie, David Nicholls, Amy Pickard, Bryan Spears

Data Overview	2

Project objectives	2

Sampling Locations	2

Collection of samples	3

In-field collection	3

Sample storage	3

Sample analysis	4

DOC and TdN	4

UV/Vis – SUVA	4

Particulate organic carbon	4

Total phosphorous	5

Soluble reactive phosphorous	6

Greenhouse gases	6

Nature and units of recorded values	7

Dataset completeness	8

Acknowledgements	9

References	10

# Data Overview

## Project objectives

The MERLIN project aims to upscale and transform the restoration of freshwater-related ecosystems across Europe by generating a framework for the proficient implementation of nature-based solutions suited for immediate replication. This dataset comprises the first two years of water quality data from Hare Moss, a peatland in the Forth catchment which is part of Case Study 17. The objective of this dataset is to implement a control-intervention approach to characterise differences in the baseline period before restoration occurs.

## Sampling Locations

Fortnightly sampling was carried out at two sites between January 2024 and December 2025 (Table 1). Both locations were situated along Hare Burn: one upstream and one downstream of the planned restoration area. Hare Burn is strongly affected by intensive peat extraction immediately upstream of the monitoring area, which contributed to marked erosion at the upstream site. Evidence of this included bank collapse and the deposition of sand throughout the two-year sampling period, continuing trends observed during the 2022–2023 monitoring (Grant, McKenzie, Pickard, Roberts and Spears, 2025).

A third site was monitored on Black Burn, close to the Auchencorth Research Facility. This location served as the control site because of its proximity to the restoration area and its long-term water-quality dataset, which has been collected since 2006 and reported in several previous publications (e.g. Dinsmore et al., 2013).

Table 1 – Peatland sampling locations

| Site | Site code | Latitude | Longitude |
| --- | --- | --- | --- |
| Black Burn | AUCH | 55.794318 | -3.248223 |
| Hare Burn Upstream | HARE US | 55.794768 | -3.270693 |
| Hare Burn Downstream | HARE DS | 55.801449 | -3.259603 |

Figure 1. Map showing the locations of the three sampling sites included in the dataset. Grid reference: NT 21245 56644

# Collection of samples

## In-field collection

Water samples for total phosphorous (TP), soluble reactive phosphorous (SRP), and dissolved organic carbon/total dissolved nitrogen (DOC/TdN) were collected from as close to the middle of the stream as possible, using a 60 ml polypropylene syringe that had been rinsed with stream water three times prior to sampling. TP samples were syringed directly into sterile 15 ml centrifuge tubes up to the 13 ml mark to allow for expansion on freezing. SRP and DOC samples were filtered through a 25 mm Cytiva Whatman™ GD/X 0.45 µm Cellulose Acetate (CA) syringe filter. The filter was first primed before use by pushing through several drops of stream water (this was not collected). Water was filtered into a sterile 15 ml centrifuge tube for SRP analysis. An HCl acid-washed 30 ml HDPE Nalgene® bottle was rinsed with filtered water before being filled to the top, leaving as little headspace as possible, for DOC, TdN and UV/Vis analysis. The same filter was used for both samples unless it became clogged. Unfiltered water for particulate organic carbon analysis (POC) was collected directly from the stream into an acid-washed 250 ml HDPE Nalgene® bottle which had been pre-rinsed with stream water three times. The 30 ml and 250 ml HDPE Nalgene® bottles were washed before use using 10% hydrochloric acid (HCl) and then triple rinsed with deionised Milli-Q water (18.2 Mῼ resistivity). In addition to these four samples, a spare filtered sample was taken in a 15 ml sterile centrifuge tube for possible problems or future analysis. All sample bottles were labelled with the name of the project, site code, and sampling date.

Dissolved greenhouse gas samples were taken in duplicate using the headspace method. Samples were collected using two 200 ml syringes fitted with a 3-way valve, with 100 ml of water and 100 ml of air being drawn into the syringe. The syringes were then submerged in the stream and shaken for 60 seconds to equilibrate the gas concentrations. The air sample was then extracted from the 200 ml syringe into a dry 100 ml syringe and injected into a 22 ml Agilent® glass Gas Chromatography (GC) vial, using 25G and 19G needles. The needles were then removed, creating a seal. Ambient air was drawn into the 100 ml syringe and injected into a third 22 ml G GC vial using the same method.

The physicochemical parameters of pH, electrical conductivity, dissolved oxygen (DO), and temperature were measured using a HACH® HQ4300 Portable 3-channel multimeter with a PHC20101 Intellical pH electrode, CDC40101 Intellical conductivity electrode, and LDO10101 Intellical dissolved oxygen (DO) sensor. The pH electrode was calibrated in the lab using pH 4, 7, and 10 buffer solutions before every use. The electrode bulb was checked for fouling and bubbles before readings were taken. The conductivity electrode was calibrated using 1413 µS/cm standard solution before every use.

## Sample storage

Immediately upon return to the lab TP, SRP, and the spare filtered samples were placed in the freezer at -18°C until analysis in large batches could be undertaken. DOC and POC samples were kept at 4°C. These samples were analysed within a week or as soon as possible. Greenhouse gas samples were stored at room temperature until analysis could be undertaken in large batches.

# Sample analysis

## DOC and TdN

Samples collected for DOC and TdN analysis were analysed using a Shimadzu TOC-LCPH analyser with an added TNM L unit and ASI-L autosampler. This analyser uses the 680°C combustion catalytic oxidation method which utilises acidification and sparging. By sparging samples to which a small amount of acid has been added, the inorganic carbon in the sample is converted to carbon dioxide and removed. The remaining carbon is measured and reported as non-purgeable organic carbon (NPOC). NPOC is taken as DOC.

Filtered samples and standards were transferred to 40 mL glass vials and analysed using the NPOC/TN method. NPOC standards of 5, 10, and 20 mg L-1 were prepared from the 100 mg L-1 potassium hydrogen phthalate stock solution and used as quality control standards at the beginning and end of each run. TdN standards of 2, 5, and 10 mg L-1 were prepared from the stock 50 mg L-1 potassium nitrate standard and ran at the beginning and end of each run. Three Milli-Q blanks were run at the beginning and end of each run, as well as between standard types and samples.

To verify the Shimadzu TOC-L analyser was operating correctly, calibration curves for NPOC (0-100 mg L-1) and TN (0-50 mg L-1) were generated periodically. Re-calibration occurred when the carrier gas was changed, or after the analyser was serviced.

The instrument method was set to measure all samples twice and perform a third measurement if the coefficient of variation of the first two peak areas was >2%. The outlier of the three measurements was excluded, and the mean of the remaining two measurements was then calculated and reported.

## UV/Vis – SUVA

Absorbance was measured using a PerkinElmer UV/Vis Lambda 365 spectrophotometer. Refrigerated samples were decanted into glass vials and placed on an autosampler where they were introduced to a quartz cuvette with a 10 mm path length.

Absorbance was measured in 1 nm increments from 230 to 800 nm. Samples were blank corrected with a deionised water standard. Specific UV absorbance was recorded at 254 nm (SUVA254). This indicates the relative aromaticity of the DOC. SUVA254 was calculated as the absorbance (A) at λ= 254 nm normalised to a 1 m path length (l) divided by the DOC concentration in mg L-1 after Weishaar et al. (2003) (Equation 1).

Equation 1

𝑆𝑈𝑉𝐴254 = (𝐴/𝑙)/𝐷𝑂𝐶

## Particulate organic carbon

For POC analysis, 45 mm 0.7 µm GF/F filters were pre-combusted at 450 °C for 8 hours and weighed using a four decimal place Mettler AE 163 scale. 250 mL of unfiltered water was then filtered through the pre-combusted filter using a Nalgene® vacuum filtration system to capture the particulate carbon. The filter was then dried for 3 hours at 105 °C and reweighed before combustion at 375 °C for 16 hours. The filter was then weighed a final time and POC concentration (mg L-1 ) was calculated by determining the loss on ignition (LOI). A conversion factor of 0.58 (Pribyl, 2010) was applied to the LOI value, which assumes that 58% of organic matter comprises C.

## Total phosphorous

SEAL AQ2 and AQ400 method

Samples collected up until 10/10/2024 were analysed for total phosphorous using a SEAL AQ2 Discrete analyser. Frozen samples for total phosphorous were first thawed at room temperature before undergoing digestion by adding 0.5 ml potassium persulfate (K2S2O8) and 0.1 ml of 31% sulfuric acid (H2SO4) to convert all forms of P into orthophosphate. All standards and blanks were digested in the same way. Samples, standards, and blanks were then autoclaved for 30 minutes at 121°C.

P concentrations were determined according to the acid-molybdenum-blue colorimetric method. Colorimetric determination of orthophosphate in water relies on the reaction of orthophosphate with ammonium molybdate ((NH4)6Mo7O24H2O) and potassium antimony tartrate (PAT; C8H4K2O12Sb2) in acid solution (H2SO4). The yellow phospho-molybdate complex formed during the reaction is reduced with ascorbic acid (C6H8O6) to a more stable blue complex. The intensity of the blue colour is proportional to the concentration of orthophosphate present in the solution.

At the beginning of each run the analyser auto-calibrated using a 0.2 mg L-1 P top standard which was prepared from a 100 mg L-1 P stock standard solution (0.2195 g potassium dihydrogen orthophosphate in 500 ml deionised water). The generated calibration curve was checked at the end of each run to ensure accuracy. In addition to the 0.2 mg L-1 P standard, the same 100 mg L-1 P stock solution was used to make 0.1, 0.05, and 0.025 mg L-1 P reference standards which were used at the beginning and the end of each run to ensure the analyser was operating correctly. Digested de-ionised water was used for blanks before and after the standards as well as throughout the run after every 15-20 samples.

This same method was used for samples collected between 30/05/2025 and 11/12/2025 but were analysed on a SEAL AQ400 discrete analyser, a newer replacement for the AQ2.

SEAL AA3 method

Samples collected between 24/10/2024 and 16/05/2025 inclusive were analysed using a SEAL AA3 HR Auto Analyser due to breakdown of the AQ2 analyser.  Frozen samples for total phosphorous were first thawed at room temperature before undergoing digestion by adding 0.15 g potassium persulfate (K2S2O8) and 1 ml of 1 N sulfuric acid (H2SO4) to convert all forms of P into orthophosphate. All standards and blanks were digested in the same way. Samples, standards, and blanks were then autoclaved for 45 minutes at 121°C.

At the beginning of each run the analyser was calibrated using a 0.01 - 1 mg L-1 P range which was prepared from a 1000 mg L-1 P stock standard solution (4.3937 g potassium dihydrogen orthophosphate in 1000 ml deionised water). The generated calibration curve was checked at the end of each run to ensure accuracy.

In addition to the calibration, reference standards 0.163 and 0.815 mg L-1 P were used at the beginning and end of every run to ensure the analyser was operating correctly with an additional reference standard 0.326 mg L-1 P run periodically throughout the run every 10 samples. Reference standards were prepared from 1000 mg L-1 PO4 stock standard solution (1.4330 g potassium dihydrogen orthophosphate in 1000 ml deionised water). In addition to this, certified reference materials provided by LGC standards were run at the beginning and end of each run. De-ionised water was used for blanks before and after the standards as well as throughout the run after every 10 samples.

## Soluble reactive phosphorous

SEAL AQ2 and AQ400 method

Samples collected for SRP analysis were first thawed from frozen and then analysed using a SEAL AQ2 and AQ400 Discrete analysers according to the same acid-molybdenum-blue colorimetric method as outlined above. Samples, blanks, and standards did not undergo acid digestion.

SEAL AA3 method

At the beginning of each run the analyser was calibrated was calibrated in the same way as outlined above. The generated calibration curve was checked at the end of each run to ensure accuracy.

In addition to the calibration, reference standards 0.481 mg L-1 P were used at the beginning, end and periodically every 5 samples to ensure the analyser was operating correctly. Reference standards were prepared from 1000 mg L-1 PO4 stock standard solution (1.4330 g potassium dihydrogen orthophosphate in 1000ml deionised water). In addition to this, certified reference materials provided by LGC standards were run at the beginning and end of each run. De-ionised water was used for blanks before and after the standards as well as throughout the run after every 5 samples

## Greenhouse gases

Dissolved greenhouse gas samples were analysed on an Agilent 7890B gas chromatograph (GC), fitted with a micro electron capture detector (μECD) for N2O analysis and a flame ionisation detector (FID) for CH4 and CO2 analysis. Concentrations were calculated from peak areas, and calibrated against mixed gas standards of known concentrations (Table 2). The results were given in ppmv and then converted to µg L using Henry’s law (Dawson et al., 1995) The average of the two duplicate samples from each site was averaged to give a final value.

Table 2 - Methane (CH4), carbon dioxide (CO2) and nitrous oxide (N2O) concentrations in the mixed gas standards used for the 4-point calibration of GHG samples analysed on the Agilent 7890B gas chromatograph. Concentrations given in parts per million per volume (ppmv).

| Standard | CH4 (ppmv) | CO2 (ppmv) | N2O (ppmv) |
| --- | --- | --- | --- |
| 1 | 1.12 | 202 | 0.208 |
| 2 | 1.60 | 355 | 0.296 |
| 3 | 4.10 | 693 | 0.507 |
| 4 | 98.2 | 5253 | 1.04 |

# Nature and units of recorded values

The dataset shows the sampling information in columns A to C, the physicochemical data collected in the field in columns D to K, and results from laboratory analysis in columns L to V. The description and units associated with these columns are outlined below in Table 3.

Table 3. Column heading descriptions and unit details for MERLIN 2022-2023 dataset

| Column heading | Description | Decimal places | Unit |
| --- | --- | --- | --- |
| Site_Name | Assigned name of the site |  |  |
| Site_Code | Assigned site codes |  |  |
| Date | Date of site visit and sample collection |  | dd/mm/yy |
| Time | Time of sample collection, 24-hour format |  | hh:ss |
| pH | Water pH | 2 |  |
| pH_Temp | Water temperature recorded by pH probe | 2 | °C |
| DO | Dissolved oxygen in water | 2 | mg L-1 |
| DO_perc | Percentage of dissolved oxygen in water | 1 | % |
| DO_Temp | Water temperature recorded by DO probe | 2 | °C |
| Cond | Water conductivity | 2 | µS/cm |
| Cond_Temp | Water temperature recorded by conductivity probe | 2 | °C |
| Temp_av | Average water temperature of three probe readings | 2 | °C |
| DOC | Dissolved Organic Carbon | 2 | mg L-1 |
| TdN | Total dissolved Nitrogen | 3 | mg L-1 |
| C_N | Carbon to Nitrogen ratio | 3 |  |
| Abs_254 | Absorbance at 254 nm | 4 | nm |
| SUVA | Specific Ultraviolet Absorbance | 4 |  |
| TP | Total Phosphorous | 3 | mg L-1 |
| SRP | Soluble Reactive Phosphorous | 3 | mg L-1 |
| CH4 | Dissolved methane | 3 | µg/L CH4 |
| CH4_C | Dissolved carbon as methane | 3 | µg L⁻¹ CH₄-C |
| CO2 | Dissolved carbon dioxide | 3 | mg L-1 CO2 |
| CO2_C | Dissolved carbon as carbon dioxide | 3 | mg L-1 CO2_C |
| N2O | Dissolved nitrous oxide | 3 | µg L⁻¹ N2O |
| N2O_N | Dissolved nitrogen as nitrous oxide | 3 | µg L⁻¹ N2O_N |
| POC | Particulate Organic Carbon | 2 | mg L-1 |

# Dataset completeness

On occasion, failure of field instruments or laboratory analysers resulted in missed or incorrect observations or results. Table 4 outlines the reasoning for any excluded results or observations from the dataset. Excluded results are defined by NA in the data file.

Table 4. Table of excluded data from the dataset

| Date collected | Site code | Determinants | Reason for exclusion |
| --- | --- | --- | --- |
| 14/03/2024 | AUCH | Abs_254 | Instrument error |
| 14/03/2024 | AUCH | SUVA | No absorbance result to calculate SUVA |
| 14/03/2024 | AUCH | CO2 | Instrument error |
| 14/03/2024 | AUCH | CO2_C | Instrument error |
| 11/04/2024 | HARE US | CO2 | Instrument error |
| 11/04/2024 | HARE US | CO2_C | Instrument error |
| 25/04/2024 | AUCH | CO2 | Instrument error |
| 25/04/2024 | AUCH | CO2_C | Instrument error |
| 09/05/2024 | HARE DS | CO2 | Instrument error |
| 09/05/2024 | HARE DS | CO2_C | Instrument error |
| 02/08/2024 | AUCH | CO2 | Instrument error |
| 02/08/2024 | AUCH | CO2_C | Instrument error |
| 02/08/2024 | HARE DS | CO2 | Instrument error |
| 02/08/2024 | HARE DS | CO2_C | Instrument error |
| 02/08/2024 | HARE US | CO2 | Instrument error |
| 02/08/2024 | HARE US | CO2_C | Instrument error |
| 29/08/2024 | AUCH | CO2 | Instrument error |
| 29/08/2024 | AUCH | CO2_C | Instrument error |
| 29/08/2024 | HARE DS | CO2 | Instrument error |
| 29/08/2024 | HARE DS | CO2_C | Instrument error |
| 29/08/2024 | HARE US | CO2 | Instrument error |
| 29/08/2024 | HARE US | CO2_C | Instrument error |
| 05/12/2024 | HARE DS | SRP | Missing sample |
| 05/12/2024 | HARE US | SRP | Missing sample |
| 21/03/2025 | HARE US | Abs_254 | Presence of ferric hydroxide colloids in sample caused abnormally high results |
| 21/03/2025 | HARE US | SUVA | No absorbance result to calculate SUVA |
| 03/04/2025 | HARE US | Abs_254 | Presence of ferric hydroxide colloids in sample caused abnormally high results |
| 03/04/2025 | HARE US | SUVA | No absorbance result to calculate SUVA |
| 16/04/2025 | HARE US | Abs_254 | Presence of ferric hydroxide colloids in sample caused abnormally high results |
| 16/04/2025 | HARE US | SUVA | No absorbance result to calculate SUVA |
| 16/05/2025 | HARE US | Abs_254 | Presence of ferric hydroxide colloids in sample caused abnormally high results |
| 16/05/2025 | HARE US | SUVA | No absorbance result to calculate SUVA |
| 10/07/2025 | AUCH | CO2 | Instrument error |
| 10/07/2025 | AUCH | CO2_C | Instrument error |
| 10/07/2025 | HARE DS | CO2 | Instrument error |
| 10/07/2025 | HARE DS | CO2_C | Instrument error |
| 10/07/2025 | HARE US | CO2 | Instrument error |
| 10/07/2025 | HARE US | CO2_C | Instrument error |
| 23/07/2025 | AUCH | CO2 | Instrument error |
| 23/07/2025 | AUCH | CO2_C | Instrument error |
| 23/07/2025 | HARE DS | CO2 | Instrument error |
| 23/07/2025 | HARE DS | CO2_C | Instrument error |
| 23/07/2025 | HARE US | CO2 | Instrument error |
| 23/07/2025 | HARE US | CO2_C | Instrument error |
| 07/08/2025 | AUCH | CO2 | Instrument error |
| 07/08/2025 | AUCH | CO2_C | Instrument error |
| 07/08/2025 | HARE DS | CO2 | Instrument error |
| 07/08/2025 | HARE DS | CO2_C | Instrument error |
| 07/08/2025 | HARE US | CO2 | Instrument error |
| 07/08/2025 | HARE US | CO2_C | Instrument error |
| 13/11/2025 | HARE US | CO2 | Instrument error |
| 13/11/2025 | HARE US | CO2_C | Instrument error |
| 28/11/2025 | HARE DS | CH4 | Sample error |
| 28/11/2025 | HARE DS | CH4_C | Sample error |

# Acknowledgements

MERLIN is a "research and innovation action" funded under the European Commission’s Horizon 2020 programme under grant agreement No 101036337.

The Hare Moss monitoring was partially supported by NERC, through the UKCEH National Capability for UK Challenges Programme NE/Y006208/1.

# References

Dawson, J. J. C., Hope, D., Cresser, M. S., Billett, M. F., (1995). Downstream Changes in Free Carbon-Dioxide in an Upland Catchment from Northeastern Scotland. Journal of Environmental Quality, 24(4), 699-706. https://doi.org/10.2134/jeq1995.00472425002400040022x

Dinsmore, K. J., Billett, M. F., Dyson, K. E. (2013). Five year record of aquatic carbon and greenhouse gas concentrations from Auchencorth Moss. NERC Environmental Information Data Centre. https://doi.org/10.5285/3f0820a7-a8c8-4dd7-a058-8db79ba9c7fe

Pribyl, D. W. (2010). A critical review of the conventional SOC to SOM conversion factor. Geoderma, 156(3-4), 75-83. https://doi.org/10.1016/j.geoderma.2010.02.003

Weishaar, J. L., Aiken, G. R., Bergamaschi, B. A., Fram, M. S., Fujii, R., & Mopper, K. (2003). Evaluation of Specific Ultraviolet Absorbance as an Indicator of the Chemical Composition and Reactivity of Dissolved Organic Carbon. Environment Science and Technology, 37(20), 4702-4708. https://doi.org/10.1021/es030360x