Hydrodynamic data from embankment edges of Kumirmari island, Indian Ganges-Brahmaputra-Meghna (GBM) Delta, July-August 2022

Description

Hydrodynamics were measured at the base of embankments and foreshores at three locations around Kumirmari island, Indian GBM delta in July-August 2022. Tidal inundation, current velocity, and wave orbital velocity were measured using B4+ Mini Buoys (low-cost bottom mounted floats). Wave heights and energy were measured using RBRSolo3 D | wave sensors. Water depth in front of embankments were measured using a Garmin Aquamap device. Sediment properties were also gathered at each hydrodynamic sensor deployment. The purpose of the survey was to determine the level of hydrological forcing around the island, to provide guidance on coastal flood and erosion protection planning.

Files and class names

| File(s) | Column | Description |
| --- | --- | --- |
| Coordinates/Surveys.gpkg | Site | Site in which data was gathered around Kumirmari island (East, South, or West). |
|  | Foreshore | Nature of the embankment (constructed from ‘concrete’, ‘mud’, or showing a ‘breach’), and whether it was flanked by mangrove and saltmarsh habitat (‘veg’). |
|  | Position | Position at which either a Mini Buoy deployment, RBRSolo deployment, or sediment sampling was done on the embankment. ‘Base’ refers to the seaward-facing toe of the embankment. ‘Bottom’ refers to the lowest point on the foreshore. |
|  | MiniBuoyID | ID of the Mini Buoy deployed at that location. |
|  | RBRSoloID | ID of the RBRSolo deployed at that location. |
| Aquamap/Depths.gpkg | ID | Identifier of each sounding. |
|  | Depth | Depth of the water. Units are in m. |
| MiniBuoy/Hydrodynamics.csv | Site | Site in which data was gathered around Kumirmari island (East, South, or West). |
|  | Foreshore | Nature of the embankment (constructed from ‘concrete’, ‘mud’, or showing a ‘breach’), and whether it was flanked by mangrove and saltmarsh habitat (‘veg’). |
|  | Position | Position at which either a Mini Buoy deployment, RBRSolo deployment, or sediment sampling was done on the embankment. ‘Base’ refers to the seaward-facing toe of the embankment. ‘Bottom’ refers to the lowest point on the foreshore. |
|  | Date | Date and time stamp of each record in the format yyyy-mm-dd hh:mm:ss. |
|  | FullDay | Logical statement denoting whether a day contains a complete data set (‘True’ or ‘False’). |
|  | MiniBuoyID | ID of the Mini Buoy deployed at that location. |
|  | Tilt | Mean tilt from horizonal. Units in degrees. |
|  | Event | ID of each inundation event. |
|  | Tide | Phase of the tide that is either incoming (‘Flood’) or outgoing (‘Ebb’). |
|  | Status | Inundation status, denoted as non-inundated (‘N’), partially inundated (‘P’), or fully inundated (‘F’). |
|  | CurrentVelocity | Mean current velocity. Units in m/s. |
|  | WaveOrbitalVelocity | Mean wave orbital velocity. Units in m/s. |
| MiniBuoy/Raw/East/MB15.csv MiniBuoy/Raw/East/MB17.csv MiniBuoy/Raw/East/MB18.csv MiniBuoy/Raw/East/MB19.csv MiniBuoy/Raw/East/MB25.csv MiniBuoy/Raw/South/MB01.csv MiniBuoy/Raw/South/MB02.csv MiniBuoy/Raw/South/MB20.csv MiniBuoy/Raw/South/MB21.csv MiniBuoy/Raw/South/MB22.csv MiniBuoy/Raw/South/MB23.csv MiniBuoy/Raw/South/MB24.csv MiniBuoy/Raw/West/MB04.csv MiniBuoy/Raw/West/MB06.csv MiniBuoy/Raw/West/MB07.csv MiniBuoy/Raw/West/MB08.csv MiniBuoy/Raw/West/MB09.csv MiniBuoy/Raw/West/MB10.csv MiniBuoy/Raw/West/MB11.csv MiniBuoy/Raw/West/MB12.csv MiniBuoy/Raw/West/MB13.csv | DATA | Date and time stamp of each record in the format yyyy-mm-dd hh:mm:ss. |
|  | G | Gravity-compensated acceleration values measured along the y-axis. Units in g-force (G). |
| MiniBuoy/Metadata | Code Name ID Site | Mini Buoy code Mini Buoy ID Mini Buoy secondary ID Location (East, South and West) |
| RBRSolo/East_210628.csv RBRSolo/South_210627.csv RBRSolo/West_210629.csv | Time | Start date and time of each sampling burst in the format yyyy-mm-dd hh:mm:ss. |
|  | Burst | ID of each burst event. |
|  | Depth | Average water depth per burst. Units in m. |
|  | Significant_wave_height | Significant wave height per burst. Units in m. |
|  | Significant_wave_period | Significant wave period per burst. Units in s. |
|  | 1/10_wave_height | 1/10 wave height per burst. Units in m. |
|  | 1/10_wave_period | 1/10 wave period per burst. Units in s. |
|  | Maximum_wave_height | Maximum wave height per burst. Units in m. |
|  | Maximum_wave_period | Maximum wave period per burst. Units in s. |
|  | Average_wave_height | Average wave height per burst. Units in m. |
|  | Average_wave_period | Average wave period per burst. Units in s. |
|  | Wave energy | Wave energy per burst. Units in J/m2. |
| SoilProperties/SoilProperties.csv | Site | Site in which data was gathered around Kumirmari island (East, South, or West). |
|  | Foreshore | Nature of the embankment (constructed from ‘concrete’, ‘mud’, or showing a ‘breach’), and whether it was flanked by mangrove and saltmarsh habitat (‘veg’). |
|  | Position | Position at which either a Mini Buoy deployment, RBRSolo deployment, or sediment sampling was done on the embankment. ‘Base’ refers to the seaward-facing toe of the embankment. ‘Bottom’ refers to the lowest point on the foreshore. |
|  | MiniBuoyID | ID of the Mini Buoy deployed at that location. |
|  | BulkDensity | Bulk density of the soil. Units in g/cm3. |
|  | OrganicMatterContent | Organic matter content of the soil. Units in % of wet weight. |
|  | WaterContent | Water content of the soil. Units in % of wet weight. |
|  | SandContent | Sand content of the soil. Units in %. |
|  | SiltContent | Silt content of the soil. Units in %. |
|  | ClayContent | Clay content of the soil. Units in %. |
|  | Conductivity | Electrical conductivity of the soil. Units in μS/cm. |
|  | Salinity | Salinity content of the soil. Units relate to the Practical Salinity Scale 1978 (Fofonoff & Millard 1983). |

Spatial coverage

- Caerlaverock saltmarsh, Solway Firth, Scotland (54°58'05"N, 3°31'58"W)

- Black Scar, Wales (51°45′54′′ N, 4°26′12′′ W)

- Percut Sei Tuan, Indonesia (3°43′22′′ N, 98°46′15′′ E)

- Skinflats, Scotland (56°03′22′′ N, 3°43′60′′ W)

Temporal coverage

2020-09-15    to    2020-09-24

Collection methods

1. CalibrationS2

Data to build a calibration between hydrodynamics (inundation duration, current velocity, and wave orbital velocity) and acceleration was gathered at Percut Sei Tuan, Indonesia (3°43′22′′ N, 98°46′15′′ E), Caerlaverock, Scotland (54°58′05′′ N, 3°31′58′′ W), Black Scar, Wales (51°45′54′′ N, 4°26′12′′ W), and Skinflats, Scotland (56°03′22′′ N, 3°43′60′′ W) for three designs: the ‘B4’, ‘B4+’, and ‘Pendant’.

Acceleration

B4 and B4+ buoys sampled y-axis acceleration at 1 Hz, whilst Pendant buoys sampled at 0.1 Hz on the x-axis. All Mini Buoys were deployed within 5 m of calibrating sensors.

Inundation duration

For the B4, pressure data was gathered using an upward-facing Nortek Aquadopp 300 m current meter. Both the B4+ and Pendant loggers were calibrated against pressure data gathered from an In-Situ Rugged TROLL 100 pressure sensor mounted upright above the ground. Pressure sensors logged every minute.

Current velocity

The B4 buoy was calibrated using an upward-facing Nortek Aquadopp 300 m. ‘Diagnostic Mode’ was used to measure in 1,025 sample bursts every hour at 1Hz (internal sampling rate was 23 Hz). The measurement head was located 20 cm above the sediment surface, and blanking distance set to 35 cm. The B4+ buoy was calibrated using an upward-facing Acoustic Doppler Velocimeter (Nortek Vector), hereafter referred to as ADV. Height of the ADV pressure sensor and velocity cell being measured above the bed was 4 and 30 cm respectively. Velocities were recorded at a rate of 16 Hz in 13-minute bursts every 30 minutes. Nominal velocity range was set to 0.3 m/s and ‘Surf Zone’ default settings were used. The Pendant design was calibrated using an upward-facing Nortek ECO taking velocity readings every 2 minutes. The lower sampling bin was used for surveying velocities nearest the ground.

Wave orbital velocity

For the B4+ only, wave orbital velocities were measured using an ADV. Height of the ADV pressure sensor and velocity cell being measured above the bed at Black Scar was 4 and 30 cm respectively. For the ADV deployed at Caerlaverock, the pressure sensor and velocity cell were 9 and 32 cm above the tidal flat respectively. Velocities were recorded at a rate of 16 Hz in 13-minute bursts every 30 minutes at Black Scar and 1Hz frequency in 20-minute bursts every 1 hour at Caerlaverock. Nominal velocity range was set to 0.3 m/s and ‘Surf Zone’ default settings were used.

2. CalibrationS3

Data to build a calibration between wave orbital velocity and acceleration was gathered at Caerlaverock, Scotland (54°58′05′′ N, 3°31′58′′ W) and Black Scar, Wales (51°45′54′′ N, 4°26′12′′ W).

Acceleration

B4+ Mini Buoys sampled y-axis acceleration at 1 Hz within 5 m of an ADV.

Velocity

Wave orbital velocities were measured using an ADV. Height of the ADV pressure sensor and velocity cell being measured above the bed at Black Scar was 4 and 30 cm respectively. For the ADV deployed at Caerlaverock, the pressure sensor and velocity cell were 9 and 32 cm above the tidal flat respectively. Velocities were recorded at a rate of 16 Hz in 13-minute bursts every 30 minutes at Black Scar and 1Hz frequency in 20-minute bursts every 1 hour at Caerlaverock. Nominal velocity range was set to 0.3 m/s and ‘Surf Zone’ default settings were used.

3. Development

Nine B4+ Mini Buoys, each sampling acceleration values along the y-axis at 1 Hz, were deployed adjacent to a water level (pressure) logger (In-Situ Rugged TROLL 100) on the tidal flat in front of Caerlaverock saltmarsh, Solway Firth, Scotland (54°58’05”N, 3°31’58”W) between the 15th and 24th of September 2020. Prior to deployment and after retrieving them, the loggers were inserted into a bucket of water for 1 minute to determine logger position in the tube. Each logger was fitted with different weights of ballast and different tether lengths. The pressure logger was set to sample at 1-minute intervals. Unfortunately, two loggers were lost during the deployment.

Data processing

1. Calibration S2

Acceleration

For current velocity calibrations, mean acceleration was calculated across 1-minute windows for the B4 and B4+ Mini Buoy designs, and 2-minute windows for the Pendant Mini Buoy design. In all other cases, no processing of acceleration data was done.

Inundation

Pressure sensors were corrected for atmospheric pressure and were converted to water level according to manufacturer’s instructions. The water level was corrected for the height of the pressure sensors above the bed and set to start at the base of the Mini Buoy using a Leica RTK-GPS (B4+ and Pendant) or Leica Optical Level (B4) accurate to below 2 cm. A linear interpolation was used to predict water levels to match the sampling frequencies of the Mini Buoys. All negative water level values were converted to 0. An inundation status was then assigned for when the Mini Buoy was emersed (N), partially inundated (P), or fully submerged (F).

Current velocity

For current velocity calibrations, mean current velocity was calculated across 1-minute windows for the B4 and B4+ Mini Buoy designs, and 2-minute windows for the Pendant Mini Buoy design.

Wave orbital velocity

Wave dimensional spectra per burst duration were calculated from the ADV velocity and pressure data. Prior to analysis, velocity data was filtered to remove values with an average beam correlation value of <75% indicating partial inundation. Bursts that had >5% of missing data were also removed. Rotated velocity vectors aligned with the main flow direction, were calculated from the x, y, and z velocity values recorded by the ADV. Nominal number of output frequencies were 780 and 1,000 for Black Scar and Caerlaverock sites respectively. Low frequency cut-off was set to 0.067 Hz, maximum value of factor scaling pressure to waves was 200, and minimum spectral level for computing direction and spreading was 0.03 m2/Hz.

2. Calibration S3

Acceleration

No processing was done to the acceleration data.

Velocity

Mean beam correlation of all three receiver arms was calculated.

3. Development

Mean acceleration values were taken for that period and compared to determine sensor movement in the tube. Pressure from tidal inundation was corrected for atmospheric pressure by interpolating during non-inundated periods and subtracting the difference from the pressure data. Water levels were calculated, and interpolated to 1 second intervals to match the Mini Buoy data. Elevation difference between the tidal flat, pressure logger, and Mini Buoy were corrected for using GPS (Leica Viva rtkGPS) to determine water levels at each Mini Buoy.