Multi-hazard impact datasets of a synthetic future urban context

# Summary

This dataset includes hazard data from 10 cities (Istanbul, Nablus, Chattogram, Cox’s Bazaar, Nairobi, Nakuru, Quito, Kokhana, Rapti, and Darussalam) generated by a computational process, yielding eight impact metrics for a given future urban scenario. Each dataset represents the potential impacts of the case study area within the cities mentioned above. The computed metrics are:

- Number of workers unemployed,

- Number of children with no Access to education

- Number of households with no Access to hospital

- Number of individuals with no Access to hospital

- Number of households displaced

- Number of homeless individuals

- Population displacement

- Number of casualties

The computation is done in https://webapp.tomorrowscities.org, which hosts the Tomorrow’s Cities Decision Support Environment Web Application.

The process involved the integration of three primary components:

- Hazard Models: Physics-based simulations or probabilistic models for relevant hazards (e.g., earthquakes, floods, landslides) specific to the project location.

- Future Exposure Scenarios: Spatially explicit projections of future urban growth, land use, and building typologies, co-developed with local stakeholders and planning authorities.

- Vulnerability Functions: Engineering-based curves defining the susceptibility of proposed infrastructure to specific hazard intensities.

# File format

Impact metric results are all in “.geojson” format.

# File names and/or naming convention

List the file names of the data files (including the file extension).

The files are named under the convention provided below:

“Cityname_scenarioname_hazardtype_exposuretype.geojson”

For example, for Istanbul, for the youth group scenario, for earthquake hazard impacts on buildings are named as:

“istanbul_youth_earthquake_buildings.geojson”

# Collection/Generation/Transformation methods

The dataset was produced using the Tomorrow’s Cities Decision Support Environment’s (TCDSE) computational engine, designed to assess disaster risk in future urban contexts. The process comprised two distinct stages:

1. Data Collection (Inputs):

- Future Exposure Dataset: Buildings, roads and power network, households and individuals that represent a future urban context, generated based on land use plans, socio-demographic trends through a bespoke exposure generation algorithm.  Related datasets and production methodology are provided in the documentation with reference number “cdfea06f-d47c-4967-99d4-cc71bddea45d” in EIDC repository. Data is in geospatial and tabular formats.

- Hazard Data: Physics-based hazard intensity maps (e.g., peak ground acceleration for earthquakes, flood depth raster) were obtained from hazard modelling teams validated against historical events. Related datasets and production methodology are provided in the documentation with reference number “dd5c8a28-a82a-4b73-981c-dd1ead32df26” in EIDC repository. Data is in geospatial format.

- Vulnerability Data: Fragility functions and vulnerability curves were collated from engineering literature and adapted to local building typologies. Vulnerability data includes parameters describing the probability of damage for a given exposure type as a function of hazard intensity. Data is in tabular format.

2. Impact modelling:

- Three datasets are integrated into the online platform https://webapp.tomorrowscities.org in which potential losses and impact metrics are calculated. While damage states are computed based on vulnerability functions, for impact modelling for eight different metrics, bespoke computational processes are utilised as described below:

  - Metric 1: Number of workers unemployed

It denotes the number of individuals who lost their jobs based on any of the following conditions:

    - The associated workplace is damaged beyond a threshold

    - The associated workplace has lost electricity

    - The associated workplace cannot be reached from the building the individual lives in via the transportation network.

  - Metric 2: Number of children with no access to education**

Similar to the first metrics but individual here refers to individuals associated with a school. This metric takes place when any of the following conditions hold:

    - The school is damaged beyond a threshold

    - The school has lost electricity

    - The school cannot be reached from the building where the individual lives via the transportation network.

  - Metric 3: Number of households with no access to hospital**

The number of households that lost their access to their associated hospital. Access is lost when any of the following conditions hold:

    - The damage state of the associated hospital is beyond a threshold

    - The associated hospital is inaccessible via the transportation network

    - The associated hospital has no electricity

  - Metric 4: Number of individuals with no access to hospital**

This metric is very similar to metric 3, except that the individual-hospital association is used. In this metric, the individual-hospital association is built upon the household-hospital association. An individual is said to lose his/her access to the hospital when any of the conditions hold:

    - The damage state of the associated hospital is beyond a threshold

    - The associated hospital is inaccessible via transportation network

    - The associated hospital has no electricity

  - Metric 5: Number of households displaced**

It is direct result of building damage state. If a building is damaged, then the households in it are also damaged.

  - Metric 6: Number of homeless individuals**

It is derived from metric 5.

  - Metric 7: Population displacement**

An individual is assumed to be displaced when any of the following conditions hold:

    - At least X of the following assets either suffer direct damage, cannot be reached via the transportation network (applies to all except residential buildings), or have no electricity:

    - The individual's residence, workplace, school, or associated hospital

    - Where X is the population displacement consensus parameter, which can be adjusted in the settings with a default value of 2.

  - Metric 8: Casualty

This metric is calculated at the building level, derived directly from each building's damage state and the number of residents in that building.

# Units of recorded values

In this section, the file contents are described to provide detailed information about what they include as attributes and about the definitions of the attributes.

The impact metrics in TCDSE are assigned to each building because the buildings' damage states are the source of social impacts. The attributes and their description are given in Table 1. Please note, not all files include all of the metrics listed as it varies for the different sites.

Table 1: Description of variables for building-level impacts data

| Field | Type | Description |
| --- | --- | --- |
| zoneid | Real | Identification number of the land use zone, that the building is located in |
| bldid | Integer | Identification number of the building |
| specialfac | Real | Identification number for building, representing whether it is a special facility or not (e.g. hospital, school) |
| repvalue | Real | Replacement value (in USD) for the building, in case it is damaged and needs to be repaired/retrofitted/reconstructed |
| expstr | String | Exposure string that represents a concatenated text the lateral load resisting system, construction code level, height and occupation type of the building. |
| fptarea | Real | Footprint area of the building in square meters |
| occbld | String | Occupation type of the building (e.g. commercial, residential) |
| lrstype | String | Lateral load resisting system of the building (e.g. Adobe, Reinforced Concrete) |
| Codelevel | String | Construction code level of the building that represents the level of construction quality of the building. Mostly associated with the construction year of the building as it gives an opinion on the regulations that the building was relying on. |
| Nstoreys | Integer | Number of storeys of the building. |
| nhouse | Real | Number of households in the building |
| residents | Real | Number of individuals living in the building |
| freqincome | String | Average income level of the household |
| ds | Integer | Damage state of the building (e.g. collapsed, heavy damage, moderate damage, slight damage) |
| metric1 | Integer | Number of workers unemployed in the building |
| metric2 | Integer | Number of children with no Access to education in the building |
| metric3 | Integer | Number of households with no Access to hospital in the building |
| metric4 | Integer | Number of individuals with no Access to hospital in the building |
| metric5 | Integer | Number of households displaced in the building |
| metric6 | Integer | Number of homeless individuals in the building |
| metric7 | Integer | Population displacement in the building |
| metric8 | Integer | Number of casualties in the building |
| node_id | Integer | Most approximate road node to the building |
| hospital_access | Boolean | Whether there is an access to hospital or not following the hazard occurrence. |
| has_power | Boolean | Whether there is loss of electricity power in the building, following the hazard occurrence |

Table 2: Description of variables for fragility function data for power network (nodes)

| Field | Description |
| --- | --- |
| vuln_string | The field that describes the power node (the facility that connects power lines and transmits energy) characteristics. |
| med_Slight | Intensity measure value that leads to slight damage |
| med_Moderate | Intensity measure value that leads to moderate damage |
| med_Extensive | Intensity measure value that leads to extensive damage |
| med_Complete | Intensity measure value that leads to complete damage |
| beta_Slight | Standard deviation for the slight damage |
| beta_Moderate | Standard deviation for the moderate damage |
| beta_Extensive | Standard deviation for the extensive damage |
| beta_Complete | Standard deviation for the complete damage |
| description | Specific description of the structure and vulnerability level if needed. |

Table 3: Description of variables for fragility function data for road network (bridges)

| Field | Description |
| --- | --- |
| vuln_string | The field that describes the road node (bridge in TCDSE computational procedures) characteristics. |
| med_slight | Intensity measure value that leads to slight damage |
| med_moderate | Intensity measure value that leads to moderate damage |
| med_extensive | Intensity measure value that leads to extensive damage |
| med_complete | Intensity measure value that leads to complete damage |
| dispersion | A statistical parameter that quantifies the uncertainty or variability in the bridge's performance during an earthquake. Applies same for all the damage states. |

Table 4: Description of variables for fragility function data for buildings against flood hazard

| Field | Description |
| --- | --- |
| expstr | Exposure string that represents a concatenated text the lateral load resisting system, construction code level, height and occupation type of the building. |
| hw0 | Probability of complete malfunction of the building, when exposed to 0 m flood depth |
| hw0_5 | Probability of complete malfunction of the building, when exposed to 0,5 m flood depth |
| hw1 | Probability of complete malfunction of the building, when exposed to 1 m flood depth |
| hw1_5 | Probability of complete malfunction of the building, when exposed to 1,5 m flood depth |
| hw2 | Probability of complete malfunction of the building, when exposed to 2m flood depth |
| hw3 | Probability of complete malfunction of the building, when exposed to 3 m flood depth |
| hw4 | Probability of complete malfunction of the building, when exposed to 4 m flood depth |
| hw5 | Probability of complete malfunction of the building, when exposed to 5 m flood depth |
| hw6 | Probability of complete malfunction of the building, when exposed to 6 m flood depth |

Table 5: Description of variables for fragility function data for buildings against earthquake hazard

| Field | Description |
| --- | --- |
| expstr | Exposure string that represents a concatenated text the lateral load resisting system, construction code level, height and occupation type of the building. |
| minperiod | The minimum fundamental period ($T$) of the building (in seconds) |
| maxperiod | The maximum fundamental period ($T$) of the building (in seconds) |
| muds1_g | earthquake intensity (in g) at which there is a 50% probability of reaching or exceeding Damage State 1 |
| muds2_g | earthquake intensity (in g) at which there is a 50% probability of reaching or exceeding Damage State 2 |
| muds3_g | earthquake intensity (in g) at which there is a 50% probability of reaching or exceeding Damage State 3 |
| muds4_g | earthquake intensity (in g) at which there is a 50% probability of reaching or exceeding Damage State 4 |
| sigmads1 | Quantifies the uncertainty in the prediction for Damage State 1. |
| sigmads2 | Quantifies the uncertainty in the prediction for Damage State 2 |
| sigmads3 | Quantifies the uncertainty in the prediction for Damage State 3 |
| sigmads4 | Quantifies the uncertainty in the prediction for Damage State 4 |
| dirds1 | The ratio of repair cost to replacement cost associated with that damage state 1. |
| dirds2 | The ratio of repair cost to replacement cost associated with that damage state 2. |
| dirds3 | The ratio of repair cost to replacement cost associated with that damage state 3. |
| dirds4 | The ratio of repair cost to replacement cost associated with that damage state 4. |
| remark | Any additional note or remark to describe the building vulnerability characteristics. |
| Real_or_dummy_values | States if real values or dummy value for exception handling |

# Spatial coverage

# Different case study areas in Istanbul, Nablus, Chattogram, Cox’s Bazaar, Nairobi, Nakuru, Quito, Kokhana, Rapti and Darussalam in approximately 500 ha of areas.

# Quality control

The spatial topological errors were removed using specific topological debugging software modules provided in QGIS software.

# References

1) Cremen, G., Galasso, C., McCloskey, J., Barcena, A., Creed, M., Filippi, M. E., Gentile, R., Jenkins, L. T., Kalaycioglu, M., Mentese, E. Y., Muthusamy, M., Tarbali, K., & Trogrlić, R. Š. (2023). A state-of-the-art decision-support environment for risk-sensitive and pro-poor urban planning and design in Tomorrow's cities. International Journal of Disaster Risk Reduction, 85, Article 103400. https://doi.org/10.1016/j.ijdrr.2022.103400

2) Cremen, G., Galasso, C., & McCloskey, J. (2022). A simulation-based framework for earthquake risk-informed and people-centered decision making on future urban planning. Earth's Future, 10, e2021EF002388. https://doi.org/10.1029/2021EF002388