Merged LiDAR based Digital Terrain Model (DTM) and Digital Surface Model (DSM) for England and Wales

Supplementary information

Barbedo R. , Chambers L., Tso C.M. , Cooper H. , Smith R. , Barr A., Samreen F. , Brown M. , Rudd A. , Fry M.

17.3.2026

Contents

Description	4

Spatial Reference	4

1.	Details of data structure	5

1.1	File structure	5

1.2	Data attributes	5

2.	Collection/generation methods	6

2.1	Source data	6

(1) England data: Data access from Environmental Agency (EA)	6

(2) Wales data: Data access from the Natural Resources Wales (NRW)	7

2.2	Generation method	9

2.3	Quality Control	10

Description of automatic / manual QC checks	10

Methods of quality control, e.g. identifying missing/incorrect values	10

Explanation of quality codes	10

Factors affecting the data (e.g. reasons for missing data, or incorrect values)	10

Final dataset merging criteria	14

Necessary transformations	14

Validation with source data	14

3.	Other useful information	16

3.1	Potential uses of LiDAR data	16

3.2	Example use cases and applications	16

3.3	How to use the data?	17

3.4	Example Jupyter notebooks	17

Contact		19

Partners		19

# Description

This high resolution (1m) spatial dataset contains information about the elevation and terrain of England and Wales. The main dataset includes a digital terrain model (DTM) and a digital surface model (DSM) obtained by merging LiDAR-based, 1m DTMs and DSMs published by the Environmental Agency in 2023 (England) and Natural Resources Wales in 2024 (Wales) respectively that are available as individual files for small regions to create a unified offering of DTM and DSM for England and Wales for ease of use, in particular for hydrological analyses such as catchment boundary definition.

The DTM represents the topographic model (height) of the bare earth, while the DSM includes the height of features on the bare earth such as buildings or vegetation (if present). The DSM and DTM files are large (>200GB) as cloud-optimised GeoTIFF (COG) files. COGs are specialized raster image files designed for efficient cloud-native workflows, allowing software to stream and read specific, small portions of a large file via HTTP GET range requests without downloading the entire dataset. To aid its use and analysis, links to python scripts and Jupyter notebooks are included in this dataset for subsetting the DTM and DSM to catchments, and for terrain, hydrological, and canopy analysis.

The work was supported by the Natural Environment Research Council under the Flood and Drought Research Infrastructure (FDRI). An overview of the FDRI project is available on the web at https://fdri.org.uk.

### Spatial Reference

Coordinate System: OSGB 1936/British National Grid (EPSG:27700)

The COG files are also available at the EIDC data store: - https://catalogue.ceh.ac.uk/datastore/eidchub/34443359-64c0-4909-9f27-bb5d47f1153f/merged_engwal_dtm.tif - https://catalogue.ceh.ac.uk/datastore/eidchub/34443359-64c0-4909-9f27-bb5d47f1153f/merged_engwal_dsm.tif Links to example Jupyter notebooks to help you use the data (see section 3.4): - https://doi.org/10.5281/zenodo.21297014 or https://github.com/NERC-CEH/dtm-analysis

# Details of data structure

## File structure

| File name | Description | Size |
| --- | --- | --- |
| merged_engwal_dtm.tif | Merged England and Wales digital terrain model | 254 GB |
| merged_engwal_dsm.tif | Merged England and Wales digital surface model | 289 GB |

## Data attributes

The dataset has the following attributes

| Attribute | Description | Units |
| --- | --- | --- |
| Elevation | Height | Integer millimetres (To convert the data to metres, divide the values by 1,000.) |

# Collection/generation methods

## Source data

### (1) England data: Data access from Environmental Agency (EA)

The LIDAR Composite DTM, a raster elevation model, covers over 95% of England with a spatial resolution of 1 meter. This comprehensive dataset was produced by the Environment Agency in 2022 and is a result of merging data from their Time Stamped archive and National LIDAR Programme. The compilation process involved using the latest and best-resolution data from repeat surveys, with bilinear interpolation applied to resampled data before merging.

The LIDAR Composites includes surveys conducted between June 6, 2000, and April 2, 2022. The model is generated from the last return LIDAR signal. The data is available for download in GeoTiff format, segmented into 5km grids.

For more detailed information and data access, you can visit the Environment Agency's data portal (https://environment.data.gov.uk/dataset/13787b9a-26a4-4775-8523-806d13af58fc). Data is available via a number of mechanisms, but this dataset was accessed as a bulk download after contacting the EA to request access. They are a series of small files in zip folders including the following:

- 1m lidar data for the UK

- DTM

- FZ_DSM (first return)

- LZ_DSM (last return)

Metadata

| Date downloaded | 16/07/2024 |
| --- | --- |
| Data types | Zipped directories of .tif files and associated metadata |
| Number of files (zipped directories) | DTM: 5877 FZ_DSM: 5735 LZ_DSM: 5875 |

The main content extracted was the *.tif images. The other file contents have not been extracted.

### (2) Wales data: Data access from the Natural Resources Wales (NRW)

The Natural Resources Wales composite dataset contains digital elevation data derived from surveys conducted between 2020 and 2023. The surveys produced datasets at various resolutions, such as 25cm, 50cm, 1m, and 2m gridded products (point cloud data was not made available). These are available as both terrain models (representing ground level) and surface models (representing object heights like vehicles, buildings, and vegetation).

The source files were downloaded from the Data Map Wales, currently at: https://datamap.gov.wales/maps/lidar-data-download/. 2 precisions of data were available. Links are provided for purposes of audit (these may not persist in future).

- DTM and DSM (integer, not scaled, i.e. less precision)

https://dmwproductionblob.blob.core.windows.net/cogs/wg_lidar_2020_22_dtm_16bit_cog.tif

https://dmwproductionblob.blob.core.windows.net/cogs/wg_lidar_2020_22_dsm_16bit_cog.tif

- DTM and DSM (float, higher accuracy, 10x larger file size)

https://dmwproductionblob.blob.core.windows.net/cogs/wg_lidar_1m_dsm_32bit_cog.tif

https://dmwproductionblob.blob.core.windows.net/cogs/wales_lidar_dtm_1m_32bit_cog.tif

Metadata

| Date downloaded | 01/08/2024 |
| --- | --- |
| Data types | *.tif files |
| Filenames | wg_lidar_2020_22_dtm_16bit_cog.tif wg_lidar_2020_22_dsm_16bit_cog.tif wg_lidar_1m_dsm_32bit_cog.tif wales_lidar_dtm_1m_32bit_cog.tif wales_lidar_dsm_1m_hillshade_cog.tif wg_lidar_2020_22_dtm_hillshade_cog.tif |

Figure 1 Coverage of the merged DTM file. White denotes valid data. This image is produced by upscaling the raster by 1000 times.

Figure 2 Coverage of the merged DSM file. White denotes valid data. This image is produced by upscaling the raster by 1000 times.

## Generation method

Scripts based on open source geospatial software GDAL was used to merge the England tiles and then merge the result with the existing Wales raster as one cloud-optimised GeoTIFF file to produce this dataset..

Such processing is both storage- and memory-intensive, thus providing the merged dataset as a community is highly desirable. It requires:

GDAL command line utility

A machine or compute node with high‑RAM configuration (64 GB was used)

~4TB of storage

## Quality Control

### Description of automatic / manual QC checks

The merged dataset was manually evaluated for consistency and was compared against source data. Please refer to the data sources metadata to view their respective QC procedure.

### Methods of quality control, e.g. identifying missing/incorrect values

For Figure 1 and Figure 2, we run the gdal command line utility to identify missing values.

### Explanation of quality codes

N/A.

### Factors affecting the data (e.g. reasons for missing data, or incorrect values)

There is some overlap between the datasets for England and Wales due to the scanning being done on different dates. After some investigation it was found that the differences of height exist mainly in:

Rivers: 		Due to different water levels

Quarries/Mines: 	Due to material removal

Woodlands: 	Due to plant life growing

Figure 3 Maps showing overlapping areas of the England and Wales sources data. The number of pixels in the overlapping area are 1,493,392,073 for DTM and 827,016,258 for DSM.

|  | - Legend: |
| --- | --- |

Figure 4 For the DTM, the differences were mainly along rivers, likely due to differences in water levels on the dates the images were captured. Image below around the confluence between rivers Severn and Vyrnwy.

Figure 5 For the DTM data, some small differences were found around mining areas, most definitely due to difference in dates as well. The image on the left is taken at mining area around River Severn, while the image on the right shows the difference between England and Wales datasets.

|  | - Legend: |
| --- | --- |

Figure 6 For DSM data, differences were encountered in rivers and mining areas, as well as in areas with denser vegetation types. The latter reflects the vegetation growth differences since the England and Wales LiDAR images are taken on different dates. Image below around the confluence between rivers Severn and Vyrnwy.

Figure 7 Woodland area around Stanford. The image on the left shows the satellite image, while the image on the right shows the differences between England and Wales DSM datasets.

### Final dataset merging criteria

To better capture the topography of DSM and DTM in the overlapping area, the merging decision logic used was:

- For DTM, as it is desired to capture the raw ground layer, the minimum value between the two datasets was used, in a pixel-by-pixel manner.

- For DSM, as it is desired to capture the vegetation in its maximum growth, the maximum value between the two datasets was used, in a pixel-by-pixel manner.

### Necessary transformations

To drastically reduce file size, the data are stored as integers. To achieve this, the original measurements in metres are converted to millimetres and saved as integer values.

### Validation with source data

To validate the merged dataset, we compared it (downsampled to 5 m resolution) against the corresponding Environment Agency (EA) source data covering the NRFA Chess catchment (ID: 41028). The comparison used the four 5 km EA tiles that together cover the TQ21 10 km grid square. More than 3 million pixels were compared. The differences between the FDRI and EA datasets for the digital terrain model (DTM) and digital surface model (DSM), respectively, are summarised below:

- Mean difference: +0.000 m (DTM) and −0.000 m (DSM)

- Standard deviation of differences: 0.034 m (DTM) and 0.400 m (DSM)

- RMSE: 0.034 m (DTM) and 0.400 m (DSM)

- MAE: 0.018 m (DTM) and 0.183 m (DSM)

These results demonstrate excellent agreement between this FDRI product and the original EA source data, with negligible bias and only minor differences introduced through the resampling and processing workflow.

Figure 8 Comparison of the FDRI (this dataset) and Environment Agency (EA) digital terrain models (DTMs) and digital surface models (DSMs) for the TQ21 10 km grid square, showing excellent correspondence between the two datasets.

# Other useful information

## Potential uses of LiDAR data

Potential applications in hydrology includes:

- Creating catchment boundaries, especially for flat areas where this is difficult

- Understanding overland flow pathways in small catchments or fields

- Developing drainage network including small streams and ditches to improve modelling

- Developing metrics or catchment descriptors to quantify connectivity of areas within catchments to drainage network, leading to better estimates of overland vs. channel flow

## Example use cases and applications

These applications generally rely on WhiteboxTools, which is available as a python package as well as other formats. Additional features are supported by the Leafmap python package.

Visualizing the Area with Leafmap

Use Leafmap to display the DSM or DTM and any additional layers.

Draw or upload a shapefile to define the specific area for the drainage network analysis or specify the area using coordinates.

DEM Preprocessing for Hydrological Analysis

Employ WhiteboxTools within the notebook to preprocess the DEM, which includes filling sinks/depressions and removing spurious peaks.

This preprocessing ensures a more accurate representation of water flow paths.

Extracting the Drainage Network

Use WhiteboxTools' hydrological analysis functions to delineate the drainage network. This includes identifying flow directions, flow accumulation, and stream network extraction.

Adjust parameters to ensure smaller streams and ditches are included in the model, which is critical for detailed analysis in smaller catchments.

Refining and Analysing the Drainage Network:

With the initial drainage network delineated, use Geopandas to refine the network, possibly integrating it with existing water body data to improve accuracy.

Analyse the drainage network to identify key hydrological features such as watershed boundaries, confluence points, and potential areas of water accumulation.

Visualizing and Interacting with the Results:

Utilize Leafmap to dynamically visualize the developed drainage network over the base DEM.

Interact with the map to explore different aspects of the drainage network, adjusting visualization settings as needed for clarity.

Sharing Insights:

The planner exports the maps and analytical results for presentations or reports.

The dataset can be easily used within Jupyter notebooks and, with its comprehensive documentation and visualizations, can be shared with stakeholders to support collaborative decision-making.

## How to use the data?

You can download the data files in their entirety from the EIDC dataset landing page. However, because the merged LiDAR files for the entirety of England and Wales are very large (each exceeding 200 GB), downloading and processing them on a personal computer may be challenging.

To make it easier to subset and use the data for your area of interest, EIDC also provides programmatic access to the dataset's Cloud Optimized GeoTIFF (COG) files from its data store. See the next section for examples to use this service.

Link to EIDC data store: - https://catalogue.ceh.ac.uk/datastore/eidchub/34443359-64c0-4909-9f27-bb5d47f1153f/merged_engwal_dtm.tif - https://catalogue.ceh.ac.uk/datastore/eidchub/34443359-64c0-4909-9f27-bb5d47f1153f/merged_engwal_dsm.tif

## Example Jupyter notebooks

We provide example Jupyter notebooks to aid the use of this dataset. These notebooks rely on the whitebox python package. Additionally, leafmap can be used to create interactive maps.

- Basic setting: The first notebook explains ways to subset, manipulate, and check the merged LiDAR datasets. For example, how to crop the data with a user-supplied shape file that defines the area of interest.

Terrain analysis: Following the first notebook, this section illustrates the calculation of terrain features such as slope, slope direction, as well as profile curvature and plan curvature.

Hydrological analysis: This notebook showcases common hydrological analysis that is based on terrain features, such as deriving a river network.

Canopy analysis: This notebook illustrates the analysis of canopy height, which is typically defined as the difference between the DSM and the DTM.

Link to example notebooks: https://doi.org/10.5281/zenodo.21297014 or https://github.com/NERC-CEH/dtm-analysis

Contact

fdri@ceh.ac.uk

Partners

Funder:

UKRI-NERC (£38 million)

Lead partner:

UK Centre for Ecology & Hydrology

Other partners:

British Geological Survey

Imperial College

University of Bristol