Agricultural emission scenarios for Northern Ireland, 2020-2027, under multiple CO2-equivalence metrics

Recent past and projected agricultural greenhouse gas emissions for Northern Ireland (2020-2027) reported under a range of CO2-equivalence metrics, and spreadsheet tool to demonstrate calculation of these emissions.

# Summary

# This dataset contains greenhouse gas emission estimates for agricultural greenhouse gas emissions in Northern Ireland, aggregating agricultural carbon dioxide, methane and nitrous oxide to report as different CO2-equivalent emissions. Past emissions are taken from the 2022 Northern Ireland Greenhouse Gas Inventory (NAEI, 2024). Projected emissions (2023-2027 emissions projected at time of project) were modelled by the Department of Agriculture, Environment and Rural Affairs, Northern Ireland (DAERA) to contribute to the Northern Ireland Climate Action Plan 2023-2027 (DAERA, 2025). Projections were generated for 4 different scenarios assuming different levels of success in rolling out agricultural emission reductions and their efficacy: ‘Headwinds’, ‘Central’, ‘Tailwinds’ and ‘Tailwinds + complete adoption of methane supressing feed additives’; from least to most successful in agricultural greenhouse gas emission reduction. Total emissions were reported under the 100-year Global Warming Potential, the 20-year Global Warming Potential, the 100-year Global temperature Change Potential and 'GWP*', to explore implications of what ‘net-zero’ emissions could look like under different metrics.

# Further information at https://www.cabidigitallibrary.org/doi/epdf/10.31220/agriRxiv.2025.00321  (Lynch, 2025)

# File format

CSV of data

.xlsx calculation spreadsheet

# File names and/or naming convention

NIAgGHGmetrics_MainResults.csv

NIAgGHGmetrics_Calculations.xlsx

# Nature and units of recorded values

Table 1. Description of variables in NIAgGHGmetrics_MainResults.csv.

| Variable/Column header | Description | Units | Vocabulary URL (if applicable) |
| --- | --- | --- | --- |
| Year | Year of emissions being reported | n/a | n/a |
| CH4(kt)_H | Annual agricultural methane emissions in ‘headwinds’ scenario of less successful implementation of agricultural emission reductions | kt |  |
| CH4(kt)_C | Annual agricultural methane emissions in ‘central’ scenario of intermediate success in implementation of agricultural emission reductions | kt |  |
| CH4(kt)_T | Annual agricultural methane emissions in ‘tailwinds’ scenario of more successful implementation of agricultural emission reductions | kt |  |
| CH4(kt)_T+100%feedadd | Annual agricultural methane emissions in ‘Tailwinds + 100% feed additives’ scenario: more successful implementation of agricultural emission reductions in line with tailwinds scenario, plus complete adoption of methane suppressing feed additives for ruminants | kt |  |
| CH4(ktGWP100)_H | Annual agricultural methane emissions reported using the 100 year Global Warming Potential in ‘headwinds’ scenario of less successful implementation of agricultural emission reductions | kt CO2 equivalent |  |
| CH4(ktGWP100)_C | Annual agricultural methane emissions reported using the 100 year Global Warming Potential in ‘central’ scenario of intermediate success in implementation of agricultural emission reductions | kt CO2 equivalent |  |
| CH4(ktGWP100)_T | Annual agricultural methane emissions reported using the 100 year Global Warming Potential in ‘tailwinds’ scenario of more successful implementation of agricultural emission reductions | kt CO2 equivalent |  |
| CH4(ktGWP100)_T+100%feedadd | Annual agricultural methane emissions reported using the 100 year Global Warming Potential in ‘Tailwinds + 100% feed additives’ scenario: more successful implementation of agricultural emission reductions in line with tailwinds scenario, plus complete adoption of methane suppressing feed additives for ruminants | kt CO2 equivalent |  |
| CH4(ktGWP20)_H | Annual agricultural methane emissions reported using the 20 year Global Warming Potential in ‘headwinds’ scenario of less successful implementation of agricultural emission reductions | kt CO2 equivalent |  |
| CH4(ktGWP20)_C | Annual agricultural methane emissions reported using the 20 year Global Warming Potential in ‘central’ scenario of intermediate success in implementation of agricultural emission reductions | kt CO2 equivalent |  |
| CH4(ktGWP20)_T | Annual agricultural methane emissions reported using the 20 year Global Warming Potential in ‘tailwinds’ scenario of more successful implementation of agricultural emission reductions | kt CO2 equivalent |  |
| CH4(ktGWP20)_T+100%feedadd | Annual agricultural methane emissions reported using the 20 year Global Warming Potential in ‘Tailwinds + 100% feed additives’ scenario: more successful implementation of agricultural emission reductions in line with tailwinds scenario, plus complete adoption of methane suppressing feed additives for ruminants | kt CO2 equivalent |  |
| CH4(ktGTP100)_H | Annual agricultural methane emissions reported using the 100 year Global Temperature change Potential in ‘headwinds’ scenario of less successful implementation of agricultural emission reductions | kt CO2 equivalent |  |
| CH4(ktGTP100)_C | Annual agricultural methane emissions reported using the 100 year Global Temperature change Potential in ‘central’ scenario of intermediate success in implementation of agricultural emission reductions | kt CO2 equivalent |  |
| CH4(ktGTP100)_T | Annual agricultural methane emissions reported using the 100 year Global Temperature change Potential in ‘tailwinds’ scenario of more successful implementation of agricultural emission reductions | kt CO2 equivalent |  |
| CH4(ktGTP100)_T+100%feedadd | Annual agricultural methane emissions reported using the 100 year Global Temperature change Potential in ‘Tailwinds + 100% feed additives’ scenario: more successful implementation of agricultural emission reductions in line with tailwinds scenario, plus complete adoption of methane suppressing feed additives for ruminants | kt CO2 equivalent |  |
| CH4(ktGWP*inc)_H | Annual agricultural methane emissions reported using GWP* to show annual change in warming caused by methane being reported in ‘headwinds’ scenario of less successful implementation of agricultural emission reductions | kt CO2 warming equivalent |  |
| CH4(ktGWP*inc)_C | Annual agricultural methane emissions reported using GWP* to show annual change in warming caused by methane being reported in ‘central’ scenario of intermediate success in implementation of agricultural emission reductions | kt CO2 warming equivalent |  |
| CH4(ktGWP*inc)_T | Annual agricultural methane emissions reported using GWP* to show annual change in warming caused by methane being reported in ‘tailwinds’ scenario of more successful implementation of agricultural emission reductions | kt CO2 warming equivalent |  |
| CH4(ktGWP*inc)_T+100%feedadd | Annual agricultural methane emissions reported using GWP* to show annual change in warming caused by methane being reported in ‘Tailwinds + 100% feed additives’ scenario: more successful implementation of agricultural emission reductions in line with tailwinds scenario, plus complete adoption of methane suppressing feed additives for ruminants | kt CO2 warming equivalent |  |
| CH4(ktGWP*avoid)_H | Annual agricultural methane emissions reported using GWP* to show near-term avoidable warming caused by methane being reported in ‘headwinds’ scenario of less successful implementation of agricultural emission reductions | kt CO2 warming equivalent |  |
| CH4(ktGWP*avoid)_C | Annual agricultural methane emissions reported using GWP* to show near-term avoidable warming caused by methane being reported in ‘central’ scenario of intermediate success in implementation of agricultural emission reductions | kt CO2 warming equivalent |  |
| CH4(ktGWP*avoid)_T | Annual agricultural methane emissions reported using GWP* to show near-term avoidable warming caused by methane being reported in ‘tailwinds’ scenario of more successful implementation of agricultural emission reductions | kt CO2 warming equivalent |  |
| CH4(ktGWP*avoid)_T+100%feedadd | Annual agricultural methane emissions reported using GWP* to show near-term avoidable warming caused by methane being reported in ‘Tailwinds + 100% feed additives’ scenario: more successful implementation of agricultural emission reductions in line with tailwinds scenario, plus complete adoption of methane suppressing feed additives for ruminants | kt CO2 warming equivalent |  |
| GHG(ktGWP100)_H | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using the 100 year Global Warming Potential in ‘headwinds’ scenario of less successful implementation of agricultural emission reductions | kt CO2 equivalent |  |
| GHG(ktGWP100)_C | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using the 100 year Global Warming Potential in ‘central’ scenario of intermediate success in implementation of agricultural emission reductions | kt CO2 equivalent |  |
| GHG(ktGWP100)_T | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using the 100 year Global Warming Potential in ‘tailwinds’ scenario of more successful implementation of agricultural emission reductions | kt CO2 equivalent |  |
| GHG(ktGWP100)_T+100%feedadd | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using the 100 year Global Warming Potential in ‘Tailwinds + 100% feed additives’ scenario: more successful implementation of agricultural emission reductions in line with tailwinds scenario, plus complete adoption of methane suppressing feed additives for ruminants | kt CO2 equivalent |  |
| GHG(ktGWP20)_H | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using the 20 year Global Warming Potential in ‘headwinds’ scenario of less successful implementation of agricultural emission reductions | kt CO2 equivalent |  |
| GHG(ktGWP20)_C | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using the 20 year Global Warming Potential in ‘central’ scenario of intermediate success in implementation of agricultural emission reductions | kt CO2 equivalent |  |
| GHG(ktGWP20)_T | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using the 20 year Global Warming Potential in ‘tailwinds’ scenario of more successful implementation of agricultural emission reductions | kt CO2 equivalent |  |
| GHG(ktGWP20)_T+100%feedadd | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using the 20 year Global Warming Potential in ‘Tailwinds + 100% feed additives’ scenario: more successful implementation of agricultural emission reductions in line with tailwinds scenario, plus complete adoption of methane suppressing feed additives for ruminants | kt CO2 equivalent |  |
| GHG(ktGTP100)_H | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using the 100 year Global Temperature change Potential in ‘headwinds’ scenario of less successful implementation of agricultural emission reductions | kt CO2 equivalent |  |
| GHG(ktGTP100)_C | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using the 100 year Global Temperature change Potential in ‘central’ scenario of intermediate success in implementation of agricultural emission reductions | kt CO2 equivalent |  |
| GHG(ktGTP100)_T | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using the 100 year Global Temperature change Potential in ‘tailwinds’ scenario of more successful implementation of agricultural emission reductions | kt CO2 equivalent |  |
| GHG(ktGTP100)_T+100%feedadd | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using the 100 year Global Temperature change Potential in ‘Tailwinds + 100% feed additives’ scenario: more successful implementation of agricultural emission reductions in line with tailwinds scenario, plus complete adoption of methane suppressing feed additives for ruminants | kt CO2 equivalent |  |
| GHG(ktGWP*inc)_H | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using GWP* to show annual change in warming caused by emissions being reported in ‘headwinds’ scenario of less successful implementation of agricultural emission reductions | kt CO2 warming equivalent |  |
| GHG(ktGWP*inc)_C | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using GWP* to show annual change in warming caused by emissions being reported in ‘central’ scenario of intermediate success in implementation of agricultural emission reductions | kt CO2 warming equivalent |  |
| GHG(ktGWP*inc)_T | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using GWP* to show annual change in warming caused by emissions being reported in ‘tailwinds’ scenario of more successful implementation of agricultural emission reductions | kt CO2 warming equivalent |  |
| GHG(ktGWP*inc)_T+100%feedadd | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using GWP* to show annual change in warming caused by emissions being reported in ‘Tailwinds + 100% feed additives’ scenario: more successful implementation of agricultural emission reductions in line with tailwinds scenario, plus complete adoption of methane suppressing feed additives for ruminants | kt CO2 warming equivalent |  |
| GHG(ktGWP*avoid)_H | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using GWP* to show near-term avoidable warming caused by emissions being reported in ‘headwinds’ scenario of less successful implementation of agricultural emission reductions | kt CO2 warming equivalent |  |
| GHG(ktGWP*avoid)_C | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using GWP* to show near-term avoidable warming caused by emissions being reported in ‘central’ scenario of intermediate success in implementation of agricultural emission reductions | kt CO2 warming equivalent |  |
| GHG(ktGWP*avoid)_T | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using GWP* to show near-term avoidable warming caused by emissions being reported in ‘tailwinds’ scenario of more successful implementation of agricultural emission reductions | kt CO2 warming equivalent |  |
| GHG(ktGWP*avoid)_T+100%feedadd | Combined agricultural carbon dioxide, methane and nitrous oxide emissions reported using GWP* to show near-term avoidable warming caused by emissions being reported in ‘Tailwinds + 100% feed additives’ scenario: more successful implementation of agricultural emission reductions in line with tailwinds scenario, plus complete adoption of methane suppressing feed additives for ruminants | kt CO2 warming equivalent |  |

# Spatial coverage
Study is based on domestic agricultural greenhouse gas emissions in Northern Ireland only

# Temporal coverage and resolution
Reports annual emissions from the national inventory 2020-2022, and projected emissions 2023-2027.

# Collection/Generation/Transformation methods

Past emissions are taken from the 2022 Northern Ireland Greenhouse Gas Inventory (NAEI, 2024). Projected emissions (2023-2027 emissions projected at time of project) were modelled by the Department of Agriculture, Environment and Rural Affairs, Northern Ireland (DAERA) to contribute to the Northern Ireland Climate Action Plan 2023-2027 (DAERA, 2025). All emissions data (past and projected) are collected separately for individual carbon dioxide, methane and nitrous oxide, with the purpose of the project to transform and aggregate under different carbon dioxide equivalence methods. Reporting using the 100-year Global Warming Potential (GWP100), 20-year Global Warming Potential (GWP20) and 100-year Global Temperature change Potential (GTP) follows Forster et al (2021), the  and ‘GWP*’ method follows Lynch et al (2020), with an additional GWP* application to show ‘near-term avoidable warming’ following Hickey and Allen (2024).

Supplementary spreadsheet calculation tool NIAgGHGmetrics_Calculations.xlsx included with this data deposit shows how calculations are undertaken.

Further information provided in Lynch (2025).

Quality control

Data reviewed by author during project and re-interrogated as part of preparation of manuscript Lynch (2025). Supplementary spreadsheet calculation tool NIAgGHGmetrics_Calculations.xlsx included with this data deposit shows all methods for further interrogation.

# References

Department of Agriculture, Environment and Rural Affairs; DAERA (2025) Northern Ireland Climate Action Plan [Draft]; https://www.daera-ni.gov.uk/sites/default/files/2025-07/Draft%20Climate%20Action%20Plan%202023-2027.pdf

Forster, P., T. Storelvmo, K. Armour, W. Collins, J.-L. Dufresne, D. Frame, D.J. Lunt, T. Mauritsen, M.D. Palmer, M. Watanabe, M. Wild, and H. Zhang, (2021) The Earth’s Energy Budget, Climate Feedbacks, and Climate Sensitivity. In Climate Change 2021: The Physical Science Basis. Contribution of Working Group I to the Sixth Assessment Report of the Intergovernmental Panel on Climate Change [Masson-Delmotte, V., P. Zhai, A. Pirani, S.L. Connors, C. Péan, S. Berger, N.  Caud, Y. Chen, L. Goldfarb, M.I. Gomis, M. Huang, K. Leitzell, E. Lonnoy, J.B.R. Matthews, T.K. Maycock, T. Waterfield, O. Yelekçi, R. Yu, and B. Zhou (eds.)]. Cambridge University Press, Cambridge, United Kingdom and New York, NY, USA, pp. 923–1054, doi:10.1017/9781009157896.009.

Hickey, C., Allen, M., (2024) Economics of enhanced methane oxidation relative to carbon dioxide removal. Environ. Res. Lett. 19, 064043. https://doi.org/10.1088/1748-9326/ad4898

Lynch, J., Cain, M., Pierrehumbert, R. and Allen, M., (2020) Demonstrating GWP*: a means of reporting warming-equivalent emissions that captures the contrasting impacts of short-and long-lived climate pollutants. Environmental Research Letters, 15(4)

Lynch, J. (2025) What does the large range in potential methane ‘CO2 equivalent’ evaluation imply for climatepolicy targets? An initial exploration in the context of agricultural emission scenarios forNorthern Ireland. agriRxiv 2025 (2025): 20250195097. https://www.cabidigitallibrary.org/doi/10.31220/agriRxiv.2025.00321

NAEI, National Atmospheric Emissions Inventory (2024) Greenhouse Gas Inventories for England, Scotland, Wales & Northern Ireland: 1990-2022; https://naei.energysecurity.gov.uk/reports/greenhouse-gas-inventories-england-scotland-wales-northern-ireland-1990-2022