Supporting Documentation:  Flammability metrics from cone calorimetry for common wildfire fuels sampled in Great Britain from 2021-2023

Belcher, C.M., Baker, S.J.1,, Clay, G.D., Crawford, A.J.1, Doerr, S.H.2, Elliott, A.1,, Ivison, K.4, Kettridge, N.4, Little, K.4, Morison, J.I.L, Nikonovas, T.2, Orpin, A.4, Tatkiewicz, W.1 & Vitali, R.1

Overview & aims

This dataset contains flammability metrics for fuel types that are commonly involved in wildfires in the UK.  Flammability data were obtained by cone calorimetry and include measures of ignitability (time to ignition), peak heat release rate, total heat release, and effective heat of combustion.  The dataset contains the results of 1634 tests on 833 fuel samples, with 801 of those being tested in duplicate.  Samples were obtained between January 2021 and October 2023 from 22 sites across Great Britain.  Calorimetry data are matched to weather data obtained at times of sampling, and fuel moisture contents obtained by weighing samples before and after drying.  The data will be used to assess comparative fire risk between vegetation types, and to refine inputs for fire behaviour modelling.

The dataset addresses a key research question (RQ 3.1) of the UKFDRS project: “How variable is the core flammability and energy content of key UK land cover types in time and space?”  Its potential to inform work on fire behaviour modeling will also contribute to answering RQ 3.2: “How does fire behaviour vary in key UK land covers across the seasons?”

Data structure & variables

The single CSV file contains 1634 cases (rows), where each represents one fuel sample subjected to cone calorimetry.  It includes the following 16 variables (columns):

Date:  The date on which the sample was collected.

Site:  A site code identifying the site from which the sample was collected.  See Table 1 (below) for full names and coordinates of sites.

Fuel type:  Whether standing vegetation or litter.

Species:  The plant species from which the samples originate.  Where the fuel type is litter, this means the species forming the majority of the tree cover, but not necessarily the only species from which the sample originates.

Status:  Whether the fuel sampled was live or dead (for standing vegetation only).

Replicate:  An identifier (‘A’ or ‘B’) to differentiate the two subsamples that were subjected to cone calorimetry from each sample.

Temperature (°C):  The air temperature recorded at the time and place of sampling.

RH (%):  The relative humidity recorded at the time and place of sampling.

Wind Speed (m s-1):  The wind speed recorded at the time and place of sampling.

Fuel Moisture (%):  The moisture content of the sample, expressed as a percentage of dry mass.  There is a single value for every pair of replicates, as each sample had one subsample taken for moisture analysis and two for calorimetric analysis.

Duff moisture (%):  The moisture content of the duff underlying a litter sample, expressed as a percentage of dry mass.

Ignition:  Whether the sample ignited to produce flaming combustion (indicated by ‘1’) or not (indicated by ‘0’).

TTI (s):  Time to ignition, defined as the time elapsed between exposing the sample beneath the heating element and the beginning of flaming combustion.

PHRR (kW m-2):  Peak heat release rate, the maximum rate of heat release attained during the test.

THR (MJ m-2):  Total heat release, the cumulative heat release per unit area (i.e. horizontal area) of the fuel sample.

EHC (MJ kg-1):  Effective heat of combustion, the cumulative heat release per unit mass of fuel consumed.

Missing values are left as blank cells.  Cells are also blank where the variable is not relevant to the particular case (such as time to ignition for a sample that did not ignite).

Methods

The samples were collected from 22 sites across Great Britain, as listed in Table 1.  The general sampling regime included: (1) fortnightly sampling of lowland heath vegetation from March 2021 to February 2022; (2) monthly sampling of forest plantation litter from July 2022 to July 2023; and (3) monthly sampling of upland heaths and moorlands from July 2022 to July 2023, with sampling occasionally prevented by weather conditions or travel restrictions.  As data for low fuel moisture conditions is of particular importance, further sampling of lowland heaths was undertaken on days without precipitation, and additional sampling at one site (Woodbury Common) focused on periods of dry weather from May to October 2023.  Whether dead and/or live fuel samples were present varied seasonally according to species, and a small number of collected samples were lost during processing, or contained insufficient material to use two replicates for flammability testing.  Weather data were recorded on the majority of sampling occasions; this depended on the equipment available at the time and location and is therefore not consistent across the dataset.  Some individual weather data, fuel moisture values, and TTI values and are missing due to human error.

Shrubs were sampled by cutting approximately the top 15 cm of the canopy, and non-woody plants by taking the whole aboveground plant.  Each litter sample was collected at a single point, and the duff samples from immediately beneath.  Weather data were recorded with a Kestrel 5400FW or 3550FW Fire Weather Meter (Kestrel instruments, USA).  Moisture content was preserved by sealing samples in zip-lock polythene bags (litter and vegetation) or polythene tubes (duff).  Laboratory processing began typically 24 hours after collection, excepting lowland heath vegetation which was tested on the day of collection.

Table 1: The site name, latitude and longitude, for each site code used in the dataset.

| Site code | Site name | Latitude | Longitude |
| --- | --- | --- | --- |
| Scootmore | Scootmore Forest | 57.43° N | 3.39° W |
| Danby_W | Danby West | 54.41° N | 0.92° W |
| Danby_E | Danby East | 54.41° N | 0.90° W |
| Goathland | Goathland Moor East | 54.37° N | 0.68° W |
| Wykeham | Wykeham Forest | 54.28° N | 0.58° W |
| Marsden | Marsden Moor | 53.55° N | 1.92° W |
| Hope_Bog | Hope Woodlands (Bog) | 53.44° N | 1.84° W |
| Hope_Heath | Hope Woodlands (Heath) | 53.42° N | 1.85° W |
| Hope_AG | Hope Woodlands (Acid Grassland) | 53.41° N | 1.85° W |
| STANTA_Heath | Stanford Training Area (Heath) | 52.54° N | 0.76° E |
| STANTA_AG | Stanford Training Area (Acid Grassland) | 52.51° N | 0.74° E |
| Thetford | Thetford Forest | 52.45° N | 0.66° E |
| Talybont | Talybont Forest | 51.89° N | 3.32° W |
| Holnicote_AG | Holnicote (Acid Grassland) | 51.18° N | 3.60° W |
| Holnicote_Heath | Holnicote (Heath) | 51.17° N | 3.61° W |
| Alice_Holt | Alice Holt Forest | 51.17° N | 0.84° W |
| Thursley_Forest | Thursley Common (Forest) | 51.16° N | 0.72° W |
| Thursley_Heath | Thursley Common (Heath) | 51.16° N | 0.71° W |
| Hardown | Hardown Hill | 50.74° N | 2.84° W |
| Duddle | Duddle Heath | 50.73° N | 2.37° W |
| Woodbury | Woodbury Common | 50.68° N | 3.36° W |
| Haldon | Haldon Forest Park | 50.65° N | 3.58° W |

Each sample was divided to provide two subsamples for flammability testing and one for moisture content measurement.  Moisture content was obtained by weighing subsamples (c. 20 g), then placing them in a drying cabinet at 40-50° C until they reached constant weight.  The flammability subsamples were analysed in an iCone plus cone calorimeter (Fire Testing Technology Ltd, East Grinstead, UK) following ASTM standard E1354 (ASTM International, 2016).  The weighed subsample is filled into a perforated steel cylinder of 375 cm3, and placed  25 mm beneath the heating element, set to 750° C, to achieve a heat flux of 50 kW m−2 at the surface of the sample.  The apparatus maintains an electric arc 15 mm above the surface to ignite the pyrolysate.  Time to ignition (TTI) is recorded manually, and the sample remains in place until flaming ceases, and the test time is defined as ending 20 s after cessation of flaming.  Non-ignition after 300 s (240 s for lowland heaths) results in the test being discontinued, as the fuel is considered non-ignitable in any realistic scenario.   PHRR, THR and EHC are calculated automatically and recorded after the test is complete.

Detailed methods are given by Belcher et al. (2025) and Crawford et al. (2025).

Quality control

The fuel type, species and status recorded in the field were confirmed at the lab prior to processing. Weather, fuel moisture and calorimetric data were screened for quality upon compilation of the data. Some weather data that had been recorded ambiguously (e.g. lacking units) were removed from the data set.

References

ASTM International (2016) E1354: Standard Test Method for Heat and Visible Smoke Release Rates for Materials and Products Using an Oxygen Consumption Calorimeter. ASTM International, West Conshohocken, PA.

Belcher CM, Vitali R, Nikonovas T, Little K, Elliott A, Baker SJ, Crawford AJ, Doerr SH, Kettridge N & Clay GD (2025) A flammability phenology for dry mixed heaths and its implications for modelling fire behaviour. International Journal of Wildland Fire 34: WF24123.

Crawford AJ, Belcher CM, Morison JIL, Doerr SH, Kettridge N & Clay GD (2025) Fuel moisture and flammability of leaf litter in British forest plantations and their implications for wildfire risk. Forestry: An International Journal of Forest Research, cpaf029.

University of Exeter

Swansea University

University of Manchester

University of Birmingham

Forest Research