Topography, orthoimagery, topographic change, predicted hydraulic flow, and bedload transport rates of the Bislak River, the Philippines

Summary

For the Bislak River, the Philippines, this dataset contains a set of gridded rasters of: (i) topography and orthoimagery acquired in 2014, 2019 and 2020; (ii) Digital Elevation Models (DEMs) of Difference; (iii) predicted depth, velocity and shear stress for 10, 50 and 100 year return period flood scenarios; and (iv) calculated bedload transport for 10, 50 and 100 year return period flood scenarios. Topography and orthoimagery were acquired from airborne Light Detection and Ranging (LiDAR) surveys. DEMs of Difference were generated using Geomorphic Change Detection software. Depth, velocity and shear stress were predicted using HEC-RAS two-dimensional hydraulic model simulations. Shear stress predictions were combined with median (D50) grain size observations to compute bedload transport rates for four different river patterns (meandering, wandering, braided, deltaic). For the 50-year return period flood event, bedload transport rates were also calculated for the D16 and D84 grain size. Geomorphic change detection was used to identify geomorphic change which took place between 2014-2019 and 2019-2020. All geospatial data are in WGS 1984 UTM Zone 51N. This data was created as part of a numerical, two-dimensional hydraulic modelling investigation into using two-dimensional HEC-RAS modelling to estimate predict patterns of flood inundation and bedload transport for the Bislak River Philippines under different flood scenarios (10-, 50- 100-year flood events). These data were produced as part of NERC Project NE/S003312/1, led in the UK by Richard Williams, University of Glasgow.

Repeat Topographic Surveys

Data type:.tif

Units: m

Collection/generation methods:

An initial topographic survey of the Bislak River was completed in 2014, as reported by Paringit and Pascua (2015). Subsequent topographic surveys were commissioned in 2019 and 2020 (Quick et al., 2025) to provide repeat datasets. Airborne LiDAR surveyed dry topography, with near-vertical aerial imagery also collected. Field measurements of water depth and bed elevation were made at selected locations along the wet channels, using a Global Navigation Satellite System (GNSS) and echo sounding. A spectrally based depth retrieval method (Legleiter et al., 2009) using the Optical River Bathymetry Toolkit, ORByT (Legleiter, 2021) was then applied to model river bathymetry in shallow areas of the wetted channel. The topography of deeper areas of the wetted channel was directly mapped from RTK-GNSS positioned echo sounding. High-resolution digital elevation models were generated by fusing data from the different survey techniques to obtain a continuous topographic representation (Legleiter and Harrison, 2019).

DEM Errors:

Each survey technique that was used to generate a DEM product is associated with a specific error. To assess the quality of the DEM surfaces, errors were calculated by comparing validation data to co-located elevations in the interpolated DEM products (Table. 1). The error magnitudes are consistently lowest for the LiDAR-DEM and highest for the Bathymetry-DEM and OEM-DEM. Consequently, the DEM surfaces are represented more accurately for the dry topography than the wetted channel; but this is to be expected, given that the dry parts of river systems are usually mapped with greater detail than the wetted parts (Kasvi et al., 2019).

| DEM | Survey technique | Validation points | Mean (m) | Mean Absolute Error (m) | Standard Deviation (m) | Root Mean Square Error (m) |
| --- | --- | --- | --- | --- | --- | --- |
| 2019 | LiDAR-DEM | 12099 | 0.0101 | 0.0338 | 0.0433 | 0.0445 |
|  | OEM-DEM | 37059 | 0.0062 | 0.2277 | 0.3001 | 0.3002 |
|  | Bathymetry-DEM | 20683 | -0.0138 | 0.1614 | 0.2497 | 0.2501 |
| 2020 | LiDAR-DEM | 11393 | 0.0236 | 0.0366 | 0.0545 | 0.0594 |
|  | OEM-DEM | 21882 | -0.0121 | 0.2073 | 0.2961 | 0.2963 |
|  | Bathymetry-DEM | 40572 | -0.0351 | 0.1667 | 0.2783 | 0.2815 |

Table 1: Errors associated with different survey techniques against validation data for the 2019 DEM and 2020 DEM.

Bislak River Depth, Velocity, and Shear stress rasters

Data type:.tif

Units: Depth (m), Velocity ( m s⁻¹), Shear stress (Nm-2)

Generation method:

HEC-RAS 2D software was used to predict inundation patterns (depth, velocity, shear stress). To represent the topographic complexity of the model domain, a 2D grid cell size of 3 m was used in the active channel, with 5 m cells on the floodplain. Manning’s roughness was uniformly set to 0.05 based on calibration with an observed flood event (Typhoon Ineng). The Typhoon Ineng calibration dataset can be found at:  https://researchdata.gla.ac.uk/1781/. Hydrographs were specified as the input boundary condition, with the downstream boundary set to the normal depth. For 2D modelling of gradually varied flows, the full St Venant shallow water equations were used. The unsteady flow function in HEC-RAS was applied to simulate a hydrograph that was ramped up to the desired discharge over 10 hours of simulation time and then run for a further 14 hours at a constant discharge to achieve a steady state. Flood events with return intervals of 10-, 50-, and 100-years were simulated across both the 2019 and 2020 Bislak DEMs (Quick et al., 2025). Water depth (>100 mm), velocity, and shear stress predictions were exported.

Bislak River Geomorphic Change Detection (GCD) rasters

Data type:.tif

Units: m

Generation method:

GCD software was used to subtract two DEMs with consistent projections and datums to produce  DEMs of Difference (DoD) (Quick et al., 2025). To investigate geomorphic change for the Bislak River over a single (2019–2020) and multiple (2014–2019) years, the Bislak DEMs (2014, 2019, and 2020) were clipped to the active channel area. The active channel area was identified by scrutinising the 2014, 2019 and 2020 DEMs alongside high resolution corresponding satellite imagery (for 2014) and aerial imagery acquired at the same time as LiDAR (for 2019 and 2020) (active channel area can be found at:  https://researchdata.gla.ac.uk/1781/. The Bislak DEMs were formed from a fusion of topographic data collected from different survey techniques (e.g. LiDAR, echo sounding and spectrally based depth mapping). Each survey technique has its own associated errors, which are often spatially autocorrelated. Errors in the different data sets may not be independent. For each DEM, an error mask was defined for each survey technique, and appropriate vertical errors were assigned from GNSS checkpoint observations (Table 1). The error mask can be found at  https://researchdata.gla.ac.uk/1781/. Geomorphic change detection was calculated using a probabilistic thresholding approach, which removed systematic bias through filtering elevation changes based on the confidence that the detected change is real (e.g. similar changes in elevation are observed in nearby cells; Wheaton et al., 2010). Sediment budgets were calculated using a confidence interval of 95%.

Bislak River Bedload transport rate rasters

Data type:.tif

Units: g/s1

Collection/generation methods:

Bislak River Bedload transport rate rasters were generated using the shear stress rasters exported from HECRAS. Shear stress ratsers were split into river morphology types (meandering, wandering, braided, deltaic). River morphology shape files for the Bislak River can be found at  https://researchdata.gla.ac.uk/1781/.. Bedload transport rate calculations were preformed using raster calculator on ARC GIS.

Unit bedload transport rate, gb, by mass was calculated using a generalised form of the Meyer-Peter and Müller relation:

(1)

where τ is bed shear stress (N.m-2), τc is critical shear stress (N.m-2), g is acceleration due to gravity (m.s-2), 𝜌 is the density of water (kg.m-3) and 𝑅 is the hydraulic radius (m). R is calculated as A/P, where A is wetted area (m2) and P is wetted perimeter (m) of a grid cell face. S is the slope of the energy grade line (m/m), calculated between the centres of adjacent downstream cells. The constant c is set to 1 for convenience and k was taken as 1.5 (Huang, 2010).

Critical shear stress, 𝜏𝑐, was calculated as:

(2)

where 𝜃𝑐 is the dimensionless critical shear stress, D is a representative particle size (m), and 𝜌𝑠 is particle density (kg.m-3). Soulsby and Whitehouse’s (1997) approximation of the Shields (1936) curve (equation 3) was used to calculate 𝜃𝑐 where 𝐷∗ is the dimensionless grain-size (equation 4):

(3)

(4)

where ss is 𝜌𝑠/𝜌 and v is kinematic viscosity at a specific water temperature (m2.s-1). Three representative particle sizes, D, were used, the D16, D50, and D84 of the active layer (Hoey and Ferguson, 1994) calculated by combining the grain size distributions of five bulk surface and five subsurface samples distributed along the study area.

References:

- Hoey, T and R. Ferguson. Numerical simulation of downstream fining by selective transport in gravel bed rivers: Models development and illustration. Water Resources Research. 30, 7, 2251-2260 (1994)

- Huang, H.Q. Reformulation of the bed load equation of Meyer-Peter and Muller in light of the linearity theory for alluvial channel flow. Water Resources Research, 46, W09533 (2010)

- Kasvi, E., J. Salmela, E. Lotsari, T. Kumpula, and S.N. Lane, Comparison of remote sensing based approaches for mapping bathymetry of shallow, clear water rivers. Geomorphology, 333, 180–197 (2019)

- Legleiter, C. J. & Harrison, L. R. Remote sensing of river bathymetry: evaluating a range of sensors, platforms, and algorithms on the Upper Sacramento River, California, USA. Water Resour. Res. 55, 2142–2169 (2019).

- Legleiter, C. J. The optical river bathymetry toolkit. River Res. Appl. https://doi.org/10.1002/rra.3773 (2021).

- Legleiter, C. J., Roberts, D. A. & Lawrence, R. L. Spectrally based remote sensing of river bathymetry. Earth Surf. Process. Landf. 34, 1039–1059 (2009).

- Paringit, E. C. & Pascua, C. LiDAR Surveys and Flood Mapping of Bacarra River (University of the Philippines Training Center for Applied Geodesy and Photogrammetry, Quezon City, 2017).

- Quick, L., Williams, R. D., Boothroyd, R. J., Hoey, T. B., Tolentino, P. L. M., MacDonell, C., Guardian, E., Reyes, J., Sabillo, C., Perez, J., & David, C. P. C. (2025). Author Correction: Confined and mined: anthropogenic river modification as a driver of flood risk change. Npj Natural Hazards, 2(1), 1–1. https://doi.org/10.1038/s44304-024-00051-6

- Soulsby, R.L. and Whitehouse, R.J.S. Threshold of Sediment Motaion in Coastal Environments: In Pacific Coasts and Ports '97: Proceedings of the 13th Australasian Coastal and Ocean Engineering Conference and the 6th Australasian Port and Harbour Conference, 1, 1, 1, 1997, 145-150 (1997)

- Wheaton, J. M., Brasington, J., Darby, S. E. & Sear, D. A. Accounting for uncertainty in DEMs from repeat topographic surveys: improved sediment budgets. Earth Surf. Process. Landf. 35, 136–156 (2010).