National scale maps of parent material properties, terrain and soil natural capital units at 50 metre resolution for Great Britain, 2020

|  | Supporting Documentation 1 April 2026 |
| --- | --- |

Contents

Acknowledgements	4

List of abbreviations	5

Third-party datasets	7

Data generation methods	8

Background & rationale for dataset development	8

Natural Capital Pedotope classification	9

Methods & generation of component maps	11

Textural composition	12

Physico-chemical parent material characteristics	12

Subsurface drainage controls	12

Geomorphon landform classes	13

Applying smoothing filters to blur linework	14

Generation of additional terrain layers for evaluation	15

Topographic wetness index (TWI)	15

Ridge-top & valley-bottom flatness indices (MRRTFI & MRVBFI)	16

USGS geomorphic landscapes	16

Nature of recorded values	19

Natural Capital Pedotopes	19

Textural composition	21

Physico-chemical parent material characteristics	23

Subsurface drainage controls	25

Geomorphon landform classes	27

Additional terrain maps included in the data cube	29

Quality control	30

Dataset structure checks	30

Evaluation & limitations	30

Northern Ireland	32

Details of data structure	33

References	34

Appendix 1: ESB class translation table	36

Appendix 2: Map Evaluation Results	45

Correspondence matrix for subsurface drainage characteristics.	45

Correspondence matrix for soil textural characteristics.	46

Correspondence matrix for soil depth, CaCO3 contents and presence of alluvial or coastal parent materials.	47

Landforms with summarised surface drainage statistics	48

MRRTFI and MRVBFI summarised by NCP	49

USGS geomorphic landscapes summarised per NCP	50

UKCEH LCM 2023 classes summarised per NCP	51

# Acknowledgements

Funding for generating this dataset was supported by the Natural Environment Research Council (NERC) under research programme NE/W005050/1 AgZero+: Towards sustainable, climate-neutral farming. AgZero+ is an initiative jointly supported by NERC and the Biotechnology and Biological Sciences Research Council (BBSRC).

Any product derived from or incorporating the Data must be accompanied by an acknowledgment of the source by using the following statements: Contains data derived from British Geological Survey materials [Soil Parent Material Dataset © UKRI 2021] and [Depth to Groundwater Dataset © NERC 2014]. Contains data derived from OS data (OS Terrain 50) © Crown Copyright [and database right] (2022). Contains derived data from soil organic carbon concentration maps in the EcoDataCube database © Copyright OpenGeoHub & CVUT Prague & mundialis & Terrasigna & MultiOne 2020-2022. Contains data on quarry and dump locations derived from OpenStreetMap © OpenStreetMap contributors (2022). The GB peatlands map contains data derived separately for Wales, England and Scotland (2017 © Crown copyright: Welsh Government, © Natural Resources Wales and Database Right. All rights Reserved; Derived in part from British Geological Survey geology data 1:50,000 scale. BGS © UKRI, NATMAP © Cranfield University, © James Hutton Institute, and OS data © Crown copyright and database right).

# List of abbreviations

| Abbreviation | Description |
| --- | --- |
| ALERT | Agricultural Land Environmental Risk and opportunity Tool |
| BGS | British Geological Survey |
| CaCO3 | Calcium Carbonate |
| CS | Countryside Survey |
| DEM / DTM | Digital Elevation Model / Digital Terrain Model |
| DSM | Digital Soil Map |
| EA | Environment Agency |
| ESB | European Soil Bureau (parent material classes) |
| GB | Great Britain |
| GIS | Geographical Information System |
| GRASS | Geographic Resources Analysis Support System |
| HOST | Hydrology of Soil Types |
| JHI | James Hutton Institute |
| LCM | Land Cover Map |
| MNAW | Moving Neighbourhood Analysis Window |
| MRRTFI | Multi-Resolution Ridge Top Flatness Index |
| MRVBFI | Multi-Resolution Valley Bottom Flatness Index |
| NATMAP | The National Soil Map (for England & Wales) |
| NCP | Natural Capital Pedotope |
| OS | Ordnance Survey |
| PMM | Parent Material Model |
| R | The R Project for Statistical Computing |
| SAGA | System for Automated Geoscientific Analyses |
| SOC / SOM | Soil Organic Carbon / Soil Organic Matter |
| TWI | Topographic Wetness Index |
| UK | United Kingdom |
| UKCEH | UK Centre for Ecology & Hydrology |
| UKSO | UK Soil Observatory |
| USDA | United States Department of Agriculture |
| USGS | United States Geological Survey |
| WRB | World Reference Base (global soil classification system) |

# Third-party datasets

- OpenGeoHub – soil organic carbon concentration maps at 0 & 30 cm for the years, 2000, 2004, 2008, 2012, 2016 & 2020: https://stac.ecodatacube.eu/oc_iso.10694.1995.wpct/collection.json?.language=en [last accessed 22/01/2026].

- Open Street Map: locations of dumps/landfills and quarries rasterised to 30m resolution https://zenodo.org/records/6653917 [last accessed 22/01/2026] (Witjes, 2022).

- Peatlands map for Great Britain (Evans, et al., 2017), shared internally by Jennifer Williamson. Contains data derived separately for Wales, England and Scotland (© Crown copyright: Welsh Government, © Natural Resources Wales and Database Right. All rights Reserved; Derived in part from British Geological Survey geology data 1:50,000 scale. BGS © UKRI, NATMAP © Cranfield University, © James Hutton Institute, and OS data © Crown copyright and database right).

- The Parent Material Model (ver 6.1) by the BGS: https://www.bgs.ac.uk/datasets/soil-parent-material-model/ [last accessed 22/01/2026] (Lawley, 2021).

- The National Depth to Groundwater Dataset (1:50,000 scale) by the BGS: https://www.bgs.ac.uk/datasets/depth-to-groundwater/ [last accessed 22/01/2026] (McKenzie, 2014).

- The OS Terrain 50 DTM to derive the 10 landform classes: https://www.ordnancesurvey.co.uk/products/os-terrain-50 [last accessed 22/01/2026]. This layer was also used to derive several other terrain classes/indices that were used to evaluate our 5 main maps.

Additionally, national soils map data were used to evaluate the spatial distributions of 3 of the main component layers:

- NATMAP vector: https://www.landis.org.uk/data/nmvector.cfm [last accessed 22/01/2026].

- NATMAP topsoil texture: https://www.landis.org.uk/data/nmtopsoiltexture.cfm [last accessed 22/01/2026].

- National Soil Map of Scotland, including derived soil texture map: https://www.hutton.ac.uk/soil-maps/ [last accessed 22/01/2026].

- HOST map of the UK https://www.ceh.ac.uk/data/hydrology-soil-types-1km-grid [last accessed 22/01/2026] (Boorman, Hollis, & Lilly, 1995).

A modified vector layer was developed to mask out areas of open water from all of our final raster map layers. This modified vector layer consisted of the OS Boundary High Water Mark line layer (https://docs.os.uk/osngd/data-structure/administrative-and-statistical-units/boundaries/boundary-high-water-mark [last accessed 22/01/2026]), which was polygonised and lastly overlain by inland water body polygons included in the “Spatial inventory of UK waterbodies” dataset (Taylor, 2021 https://catalogue.ceh.ac.uk/documents/b6b92ce3-dcd7-4f0b-8e43-e937ddf1d4eb [last accessed 22/01/2026]), using GIS software.

We also compared the Natural Capital Pedotope map against unpublished 1:10,000 scale soil map data available from all 1 km grid squares sampled during the 1990 Countryside Survey.

# Data generation methods

## Background & rationale for dataset development

Soils support humanity and life on Earth by supporting a multitude of functions and processes within ecosystems including the provision of biomass and food, water and climate regulation, cultural services, and support for habitats across the terrestrial biosphere. Soils can be categorised based on key physico-chemical characteristics that determine the kinds of ecosystem services and functions a particular soil provides, as well as susceptibilities to degradation from environmental pressures. Mapping different types of soil thus aids in our understanding of the provision of soil-derived ecosystem services and areas that are particularly sensitive to environmental change over space and time.

Soil surveying to map soil types across Great Britain had been carried out decades ago. However, there are a number of characteristics of the currently available map products that may provide challenges to some users:

- Soils in Scotland have been mapped separately by what is now the James Hutton Institute to soils in England & Wales by what is now Cranfield University. This means that there isn’t a single harmonised soil type map for the country, though it is possible to translate the categories of both maps to the World Reference Base (WRB) classification system or USDA soil taxonomy. A version of the WRB map is available to view from UKSO: https://www.ukso.org/ [last accessed 22/01/2026].

- The existing maps represent soil associations – a high-level classification that group smaller units (Soil Series), consisting of hundreds of unique categories. Soil Associations and the Series that make up these are named after places where the particular soil type was first recognised, and so their names convey little if any information on the structure, function and broader characteristics of the soils themselves. Soilscapes represents an important example of aggregating Soil Associations into something more meaningful to lay audiences, though this does not cover Scotland. A 12-class soil type map for all of Great Britain has been introduced (Feeney, et al., 2023; Feeney, Robinson, & Emmett, 2025), with a viewable version for England & Wales being accessible (in 250 m resolution raster format) via the EA’s ALERT site: https://experience.arcgis.com/experience/fd8e7ede2548409f965275da8e8d35f7/ [last accessed 22/01/2026].

- National scale maps are only available at 1:250,000 scale or coarser and are intended for large regional to national-scale use. While that isn’t a problem in itself, it does make these maps unsuitable for some finer scale uses (e.g. at individual field scales). Higher resolution maps are available for specific regions, but not everywhere, meaning one cannot simply aggregate all fine-resolution map tiles into a single unified map for the whole country.

- Available soil maps reflect observations recorded in the mid-20th Century. Although we would not expect soil types to change significantly from year to year, it’s possible that new landfill and extractive industrial sites such as quarries, mines and spoil heaps have been established. Similarly, organic matter changes will mean that some soils which were organic decades ago may be mineral now, or vice versa if a wetland, heathland or woodland has been restored. Nationwide soil surveying for the purposes of mapping soil types is time consuming and expensive. Thus, we need a new approach that allows for soil characteristics like organic matter content and occurrence of landfill and extractive industrial sites which can change at multi-decadal scales to be updated more readily over time.

Additionally, while soil types reflect the amalgamation of several key properties (e.g. depth, texture, acidity, wetness and organic matter content), it may be more useful for some purposes to have spatial information on these different components separately. For instance, knowing the spatial distribution of soils rich in calcium carbonates could help to train machine learning models of land cover classification to delineate calcareous grasslands from other habitats more accurately. Soil texture and wetness also underpin multiple environmental opportunities that could be mapped for farms using the e-Planner tool: https://e-planner.ceh.ac.uk/About [last accessed 22/01/2026]. The aim here, therefore, was to develop a data cube of soil parent material and terrain information with the ambition of overcoming these limitations (i.e. harmonised, simpler soil classes, higher spatial resolution, and increased updatability).

## Natural Capital Pedotope classification

We define 13 soil categories in our classification system (which we term “Natural Capital Pedotopes” (NCPs) to avoid confusion with terms like “soil type” or “soil group” used by soil surveyors). We adopt the term “pedotope” to refer to a homogenous pedological unit within the landscape, using criteria that encompass soil texture, organic matter content, profile depth, drainage, and whether the soil has been significantly modified by alluvial or coastal processes, or otherwise altered substantially by human activity (e.g. through extraction of mineral resources or dumping of waste materials).

Assignment of the 13 NCP categories is illustrated by the decision tree below. This decision tree was used to categorise an NCP in each 50 x 50-metre grid cell (excluding areas with no data e.g. open water), applying a simple set of rules to classify soils based on information presented in the 4 component map layers that were developed from parent material, topographic and other spatial datasets.

Decision tree to assign NCPs to different raster grid cells. Blue downward branches indicate the condition is met; red upward branches indicate that a different condition applies. The various coloured boxes match the NCPs presented in the corresponding map in Nature of recorded values.

## Methods & generation of component maps

To develop a new soil type map, we chose to apply a simple decision tree to existing spatial datasets to generate 4 component layers. These component layers include: textural composition (including whether the soil is light, medium or heavy textured; peat; organic, but too shallow to be considered peat; and modified or removed), subsurface drainage controls (permeability of parent material classes and water table depth), physico-chemical parent material properties (deep vs shallow; calcareous vs non-calcareous; alluvial and coastal parent materials), and landforms based on the geomorphon classification system (including flat, peak, ridge, shoulder, spur, slope, hollow, footslope, valley and pit classes). The existing soil classification system for England & Wales defines soil types based predominantly on parent material characteristics. Thus, we chose to use the BGS Parent Material Model (PMM) for the bulk of our map construction, using information to inform 3 of our 4 component map layers. We also sourced available maps for individual soil types, including a harmonised peat map for all of Great Britain to represent the “Peat” class, and maps of quarry and dump sites from Open Street Map to represent the “Significantly disturbed or excavated soils” class. The Ordnance Survey (OS) Terrain 50 Digital Terrain Model (DTM) was used to classify landforms. A complete list of datasets used in our map construction is shown below.

| Dataset, including provider & format | Purpose |
| --- | --- |
| BGS PMM 1:50k vector map | - Depth to bedrock to separate shallow mineral soils from other groups - CaCO3 rank to separate calcareous from non-calcareous shallow mineral soils - Soil group to assign light, medium & heavy soils (note: there is a “Soil Texture” variable in the PMM with similar information, but we do not use that variable here) - ESB class to identify alluvial & marine PM groups and to classify soil substrate into permeable & impermeable categories |
| BGS Depth to Groundwater 50m raster model | - Identify shallow vs deep groundwater areas (2 m threshold). This was combined with the map of permeable vs impermeable areas to determine likely soil drainage behaviour |
| GB peatlands map; amalgamated from peat maps produced for each of the 3 GB nations separately (rasterised to 50 m). Methods reference for GB peat mapping (Evans, et al., 2017). | - Identify peat soils |
| Ordnance Survey (OS) Terrain 50 DTM (50 m raster digital terrain model) | - Used to define landforms based on the geomorphons classification (Jasiewicz & Stepinski, 2013) |
| Maps of quarry and dump locations in 2020 from Open Street Map (30m raster maps) | - Merged to highlight the known extent of significantly disturbed or excavated soils |
| Maps of soil organic carbon concentration at 0 & 30 cm for 2000, 2004, 2008, 2012, 2016 & 2020 from EcoDataCube (30 m raster maps) | - Aggregated into 0-30 cm topsoil profiles and used to identify organic soils (>20 % of maximum SOC concentration) |

BGS vector data and the GB peatlands map were rasterised to 50 m resolution. The 30 m raster datasets from Open Street Map and EcoDataCube were resampled to 50 m resolution, using the nearest neighbour and bilinear methods for integer and continuous raster layers, respectively. All layers were aligned to the OS Terrain 50 DTM of Great Britain which also underpins the BGS depth to groundwater model. The derivation of the 5 soil parent material and terrain maps of our dataset are detailed below.

### Textural composition

The BGS PMM contains information on the likely dominant soil texture characteristics, based on the underlying parent material and extensive soil profile sampling (Lawley, 2021). Not including “NA”, there are 25 soil texture classes in the “Soil Group” variable, and the PMM also includes an aggregation of these into 6 categories. Here, we take the aggregation a step further, placing these 6 aggregate classes into 1 of 3 top-level texture groups (light, medium or heavy). “Lightest soils” are classed as “light”; “Heaviest soils” as “heavy”; the 3 groups containing “Medium” in the name (“Medium and/to Light”, “Medium soils” and “Medium and/to Heavy”) were classed as “medium” soils. The “Mixed or Organic” group, as the name suggests, contains organic, chalky, alluvial and other soils of various textures. As “medium” is the most common texture group and is intermediate between light and heavy in terms of clay content, we have put this group into the “medium” top level texture class (assuming soils here are classed as mineral, which we elaborate on below).

Several maps of SOC content exist with UK-wide coverage. To our knowledge, only EcoDataCube provide maps at a spatial resolution (30 m) that is comparable to our other datasets. Predicting SOC concentration is challenging, owing to the bimodal and right-skewed distribution of SOC contents at national-scale (mostly mineral soils with a main peak of low carbon concentrations, and a significant minority of carbon-rich soils, particularly peats, represented by a smaller secondary peak in SOC concentrations). We know from Countryside Survey (CS) that SOC ranges ~0-550 g/kg across GB, yet the EcoDataCube maps seldom reach or exceed ~110 g/kg here. Despite this caveat, the EcoDataCube maps do appear to compare closely with CS-derived maps in terms of predicting the spatial distribution of relative high and low areas of SOC content across the country. Normally, we would identify organic soils as containing >20 % soil organic matter (SOM); equivalent to 110 g/kg SOC (or 20 % of the maximum SOC level). Here, we chose to identify 20% of the maximum SOC value for GB in each year that has been mapped by EcoDataCube (2000, 2004, 2008, 2012, 2016 & 2020). Grid cells are classed as organo-mineral if SOC exceeded these 20 % thresholds in all years.

Lastly, we updated our map by overlaying the GB peatlands map, which identifies all grid cells with “Peat” soil, followed by maps of dumps and quarries from Open Street Map (Witjes, 2022) to represent “Significantly disturbed or excavated soils”.

### Physico-chemical parent material characteristics

Shallow soils are defined by the “Depth” component of the BGS PMM (Lawley, 2021). If “Depth” is “SHALLOW” and the “CaCO3 Rank” in the PMM is either “HIGH” or “VARIABLE {HIGH)”, then the soil is classed as “Shallow calcareous soil”; shallow soils overlying other CaCO3 rank areas are classed “Shallow non-calcareous soil”. Where applicable, grid cells that have already been classified as light, medium or heavy clay are superseded by the 2 shallow groups.

Additionally, alluvial and coastal soils can be identified based on parent material with clear riverine or marine/coastal origins. A complete series of ESB class that have been identified as either alluvial or coastal in settings can be found in Appendix 1: ESB class translation table.

### Subsurface drainage controls

Light and medium soils can be sub-divided further based on their drainage behaviour. Soil drainage is affected by several factors, chiefly the permeability of the soil profile itself and the underlying parent material substrate, depth to groundwater, topographic convergence of surface runoff, and climate. The Hydrology of Soil Types (HOST) classification represents soils in terms of their drainage behaviour and receptiveness to rainfall. The HOST system is ultimately built on 3 physical settings that describe soil drainage (Boorman, Hollis, & Lilly, 1995), illustrated below. Here, we mapped these 3 physical settings using the BGS Depth to Groundwater model & European Soil Bureau (ESB) PM classes recorded in the PMM. The ~200 PM classes were categorised as either permeable or impermeable (see Appendix 1: ESB class translation table) based on the following rules:

- If the parent material is permeable & overlies groundwater >2 m deep = class (i)

- If the parent material is permeable & overlies shallow groundwater (≤2 m) = class (ii)

- If the parent material is impermeable, irrespective of groundwater depth = class (iii)

The 3 primary physical settings underpinning all 29 HOST soil classes (adapted from Boorman, Hollis, & Lilly, 1995).

### Geomorphon landform classes

Landforms were classified using the geomorphon classification system by Jasiewicz and Stepinski (2013) using the r.geomorphon GRASS GIS tool. The OS 50 m DTM was used as the input layer using a search distance (outer search radius) of 10 cells (equivalent to a 500 m radius) and, following recommended settings for national-scale landform mapping at 30 m resolution in Poland (Jasiewicz & Stepinski, 2013), a flatness threshold of 1 and flatness distance of 0 m. Additionally, the skip distance (inner search distance) was set to 1 cell to reduce noise from nearby cells that may represent erroneous elevation values or that may not give additional information in terms of elevation difference. This creates an annulus comprising cells that lie between the skip and search distances; a conceptual illustration is shown below. The r.geomorphon tool works using a moving neighbourhood analysis window which loops across all grid cells within the domain of interest. Once completed, the end result is a raster map consisting of 10 unique integer codes that correspond to a unique landform category, numbered from 1 (flat) going clockwise to 10 (pit).

Schematic showing how the analysis area (blue) is determined by the r.geomorphon tool given a target cell; image taken from a blog by Tania Lopez-Cantu from 22nd March 2023: https://www.esri.com/arcgis-blog/products/spatial-analyst/analytics/classify-terrain-with-the-new-geomorphon-landforms-tool [last accessed 22/01/2026]; b) The 10 landform classes generated by the r.geomorphon tool based on relative elevation comparisons with cells within the annulus between the search and skip distances; image taken from r.geomorphon GRASS GIS manual page: https://grass.osgeo.org/grass-stable/manuals/r.geomorphon.html [last accessed 22/01/2026].

### Applying smoothing filters to blur linework

To mitigate concerns surrounding the linework of the high-resolution BGS parent material data (including obscuring sensitive boundary information between neighbouring map units and blurring artificial abrupt changes resulting from mapping of different tiles), a moving neighbourhood analysis window was applied to all component layer maps except for the geomorphons map (where a version of this was already applied as part of map generation). This was implemented using the r.neighbors GRASS GIS tool using a circular neighbourhood with a 21-grid cell diameter (note, in this tool, the diameter includes the target cell, so in essence, this is equivalent to specifying a search radius of 10 cells, or 500 m at 50 m grid resolution). Each 50 m grid cell was updated to include the majority class within an approximate 1 km2 search area. This resulted in airbrushing out many of the thinner, more linear features from the “raw” component layer maps, as well as rounding many of the sharper edges between map units. In a final post-processing step, the OS Boundary High Water Mark was polygonised and overlain by the spatial inventory of UK waterbodies (Taylor, 2021); then, all raster layers (including the 4 additional terrain layers described below) were clipped to this combined vector layer to mask out areas of open water.

## Generation of additional terrain layers for evaluation

### Topographic wetness index (TWI)

The geomorphons layer is used to characterise soil drainage based on likely surface flow / wetness patterns linked to the catenary sequence concept illustrated below. Specifically:

- Shoulders, spurs & slopes are assumed to be more likely to exhibit free drainage at the surface, as runoff will generally have higher velocity.

- Hollows, footslopes & flats are assumed to be likely hotspots of surface waterlogging, as runoff will generally have lower velocity.

Basic 2D cross-section of a catenary sequence to illustrate the likely surface drainage behaviours shown by different geomorphons.

The other 4 geomorphon classes meanwhile are used to indicate whether the soil profile is likely to be shallow (i.e. no deeper than 50 cm; we assume this includes peaks & ridges where soils are generally thinner) or susceptible to riverine flooding (valleys, plus pits which seem to most often occur in proximity to cells classed as valleys).

To explore differences in the likely surface wetness characteristics, topographic wetness index (TWI) was calculated from the OS Terrain 50 DTM using the “r.topidx” tool from GRASS GIS. TWI means and standard deviations were computed for each of the 10 geomorphon classes and compared to visually assess differences in relative surface drainage behaviour.

### Ridge-top & valley-bottom flatness indices (MRRTFI & MRVBFI)

Peat soils and alluvial and coastal soils in particular will more likely than not occur on flat areas of land; the latter exclusively in valley bottom areas, while the former will occur in both valley bottom and ridge top areas. Moreover, soils situated in gentler and flatter areas of land will more likely show impeded surface drainage behaviour compared to soils on steeper sloping areas of land.

The multi-resolution ridge top flatness index (MRRTFI) and multi-resolution valley bottom flatness index (MRVBFI) can be derived using basic information on slope and elevation (Gallant & Dowling, 2003). Here, we applied the “Multiresolution index of valley bottom flatness (mrvbf)” tool from SAGA GIS to the OS Terrain 50 DTM to calculate both indices. The initial threshold for slope was set to 10, as recommended for a DEM of 50 m resolution, while the other parameter settings (upness and lowness percentile thresholds for elevation, shape parameter for slope, shape parameter for percentile elevation, and maximum resolution) were left unchanged, as recommended by the developers of the indices following their own testing (Gallant & Dowling, 2003).

Values of 0.5-1.5 for MRVBFI and MRRTFI reflect the steepest and smallest valley bottoms and ridge tops, respectively; MRVBFI and MRRTFI values >1.5 reflect flatter, larger valley bottom and ridge top areas, respectively (Gallant & Dowling, 2003). In our evaluation of the NCP map, we would expect to see alluvial and coastal soils with MRVBFI values >0.5 with similar patterns for both indices for peat soils. Soils with free drainage would be expected to have lower values than soils with moderate or poor drainage.

### USGS geomorphic landscapes

Large-scale mapping of geomorphic landscapes has been used by the United States Geological Survey (USGS) to support the delineation of soils and terrestrial ecosystems including at global and continental scales, such as for Africa and the contiguous United States (Sayre, Comer, Warner, & Cress, 2009). Originally, classification of geomorphic landscapes required information on slope, local relief and profile type (Hammond, 1964), but this was later adapted into the Missouri Resource Assessment Partnership (MoRAP) method so that only the first 2 variables were required (True, Gordon, & Diamond, 2000).

Using the MoRAP method, slope is classified as gently sloping when more than 50 percent of the area in a 1-km2 neighbourhood has slope less than 8 percent, otherwise the area is considered moderately sloping. Local relief, which is the difference between the maximum and minimum elevation in a neighbourhood, is classed into 6 groups: 0-15 m, 16-30 m, 31-90 m, 91-150 m, 151-400 m & >400 m.

The USGS geomorphic landscape classes are derived by combining slope and local relief to create 9 landform classes:

- Flat plains (gently sloping and local relief ≤ 15 m)

- Smooth plains (gently sloping and 15 m < local relief ≤ 30 m)

- Irregular plains (gently sloping and 30 m < local relief ≤ 90 m)

- Escarpments (gently sloping and local relief > 90 m)

- Low hills (not gently sloping and local relief ≤ 30 m)

- Hills (not gently sloping and 30 m < local relief ≤ 90 m)

- Breaks/Foothills (not gently sloping and 90 m < local relief ≤ 150 m)

- Low mountains (not gently sloping and 151 m < local relief < 400 m)

- High mountains (not gently sloping and local relief > 400 m)

However, because some of these landform classes are very rare in Great Britain, we have modified the scheme, including editing the names slightly to reflect where some of these landforms occur. For instance, a lot of “foothills” and “mountains” occur along the coastline, so it is more appropriate to consider these as low or high cliffs, with comparable features inland likelier representing “true” foothills and mountains:

- Flat plains (gently sloping and local relief ≤ 15 m)

- Smooth plains (gently sloping and 15 m < local relief ≤ 30 m)

- Irregular plains & escarpments (gently sloping and local relief > 30 m)

- Hills (not gently sloping and local relief ≤ 90 m)

- Foothills & low cliffs (not gently sloping and 90 m < local relief ≤ 150 m)

- Mountains & high cliffs (not gently sloping and local relief > 150 m)

Slope was calculated in percentage using the “Slope” tool in QGIS 3.36.1 [Maidenhead]. Percentage slope was then converted into a binary categorical layer using the “reclassify by table” tool in QGIS. If slope was less than 8, the cell was given a value of 1; greater than or equal to 8, 2; all other cells = 0.

Slope classes and relief were calculated in QGIS using the “r.neighbors” tool from the GRASS GIS toolbox. A circular neighbourhood of size 11 (i.e. a radius of 11 cells of 50×50 m, including the central cell), giving an approximate area of 1 km2. Mode was set as the method for determining which neighbourhoods were majority “gentle slopes” from the slope class map, while range was set as the method to determine relief on the same local neighbourhood basis from the elevation map.

Landforms were classified using a series of conditional “ifelse” statements in R 4.3.1 based on the 6 rules listed above. The final version of the map is shown on the next page. Pie charts were produced to illustrate the proportional make-up of the USGS geomorphic landscapes for each of the 13 NCP classes – see Appendix 2: Map Evaluation Results.

# Nature of recorded values

For the NCPs and component maps used in the decision tree, each of the 5 raster GeoTIFF files consists of a series of integer values that correspond to various soil parent material or terrain characteristics. The 4 other terrain maps for evaluation consist of 1 integer & 3 float maps. Details are outlined below, along with suggested colour schemes intended to maximise the accessibility for colourblind users.

## Natural Capital Pedotopes

Filename: “natural_capital_pedotopes.tif”

| Integer | Name | Hexcode colour |
| --- | --- | --- |
| 1 | Light free-draining soil | #009292 |
| 2 | Light moderate to poorly draining soil | #004949 |
| 3 | Medium free-draining soil | #b6dbff |
| 4 | Medium moderately draining soil | #6db6ff |
| 5 | Medium poorly draining soil | #006ddb |
| 6 | Heavy clay soil | #924900 |
| 7 | Shallow calcareous soil | #24ff24 |
| 8 | Shallow non-calcareous soil | #ffff6d |
| 9 | Alluvial and coastal soils | #db6d00 |
| 10 | Organo-mineral free-draining soil | #b66dff |
| 11 | Organo-mineral moderate to poorly draining soil | #490092 |
| 12 | Peat | #920000 |
| 13 | Significantly disturbed or excavated soils | #d9d9d9 |

## Textural composition

Filename: “textural_composition.tif”

| Integer | Name | Hexcode colour |
| --- | --- | --- |
| 1 | Light | #009292 |
| 2 | Medium | #6db6ff |
| 3 | Heavy | #924900 |
| 4 | Organic | #b66dff |
| 5 | Peat | #920000 |
| 6 | Modified or removed | #d9d9d9 |

## Physico-chemical parent material characteristics

Filename: “physicochemical_properties.tif”

| Integer | Name | Hexcode colour |
| --- | --- | --- |
| 1 | Deep non-calcareous parent material | #b6dbff |
| 2 | Deep calcareous parent material | #006ddb |
| 3 | Shallow non-calcareous parent material | #ffff6d |
| 4 | Shallow calcareous parent material | #24ff24 |
| 5 | Alluvial & coastal parent material | #db6d00 |

## Subsurface drainage controls

Filename: “subsurface_drainage.tif”

| Integer | Name | Hexcode colour |
| --- | --- | --- |
| 1 | Freely draining (permeable PM over groundwater deeper than 2m) | #2b83ba |
| 2 | Moderately draining (permeable PM over groundwater 2m deep) | #ffffbf |
| 3 | Poorly draining (impermeable PM; groundwater depth not relevant) | #d7191c |

## Geomorphon landform classes

Filename: “geomorphons.tif”

| Integer | Name | Hexcode colour |
| --- | --- | --- |
| 1 | Flat | #d9d9d9 |
| 2 | Peak | #ff7f00 |
| 3 | Ridge | #ffc34c |
| 4 | Shoulder | #ffee99 |
| 5 | Spur | #ffffcc |
| 6 | Slope | #ccffff |
| 7 | Hollow | #99edff |
| 8 | Footslope | #4cc3ff |
| 9 | Valley | #007fff |
| 10 | Pit | #000000 |

## Additional terrain maps included in the data cube

As outlined in Generation of additional terrain layers for evaluation, an additional 4 maps were derived from the OS Terrain 50 DTM for evaluation purposes. These are also included for transparency and for potential wider use by users for their applications. Details on the nature of recorded values for these maps are outlined below.

USGS Geomorphic Landscapes:

Filename: “usgs_geomorphic_landscapes.tif”

| Integer | Name | Description |
| --- | --- | --- |
| 1 | Flat plains | Slope ≤8 % & local relief ≤15 m |
| 2 | Smooth plains | Slope ≤8 % & local relief = 15-30 m |
| 3 | Irregular plains & escarpments | Slope ≤8 % & local relief >30 m |
| 4 | Hills | Slope >8 % & local relief ≤90 m |
| 5 | Foothills & low cliffs | Slope >8 % & local relief = 90-150 m |
| 6 | Mountains & high cliffs | Slope >8 % & local relief >150 m |

TWI (filename = “twi.tif”):

Continuous surface with values upwards of 0, and larger values indicating greater degrees of topographic wetness. TWI is unitless and represents a relative metric for a given landscape, hence, specific thresholds of what constitutes “high” or “low” values do not exist.

MRVBFI & MRRTFI (filenames = “mrvbf.tif” & “mrrtf.tif”, respectively):

Continuous surface with values upwards of 0, and larger values indicating greater degrees of valley bottom flatness for MRVBF or ridge-top flatness for MRRTF. Like TWI, these maps are also unitless and intended to provide relative indications of the degrees of valley bottom / ridge top flatness. However, values ranging from 0.5-1.5 have been recommended for setting thresholds between flat and not flat areas (Gallant & Dowling, 2003).

# Quality control

## Dataset structure checks

The following checks were performed prior to dataset submission:

| Check | Verified? (Y/N) |
| --- | --- |
| All layers share the correct spatial resolution (50 m), extent, and projected to the OSGB36 / British National Grid coordinate reference system? | Y: - All layers stacked in R, which would not be possible if the datasets were not exactly aligned in terms of resolution, extent or CRS. - Each layer’s spatial extent is: xmin = 0.000, ymin = 0.00, xmax = 660000.000; ymax = 1230000.000 - CRS is British National Grid - Grid resolution is 50 m |
| All rasters are correctly formatted with clearly defined “No Data” values? | Y: - For categorical rasters with integers: - Unsigned integer 32-bit - No Data set to 255 - For continuous rasters: - Float 32-bit - No Data set to -9999 |
| All layers can be loaded into multiple platforms (testing QGIS, ArcMap, R)? | Y: All 3 platforms tested across >1 machine |
| All values are positive and include only expected ranges? | Y: - NCP layer = 1-13 - Textural composition = 1-6 - Subsurface drainage = 1-3 - Parent material properties = 1-5 - Geomorphons = 1-10 - USGS geomorphic landscapes = 1-6 - TWI: all values >0; higher values occur in areas of expected flow convergence - MRVBFI: all values >0; higher values occur in expected valley bottom areas based on visual assessment of DEM - MRRTFI: all values >0; higher values occur in expected ridge top areas based on visual assessment of DEM |

## Evaluation & limitations

We have compared our soil parent material and terrain layers against a number of independent datasets (Appendix 2: Map Evaluation Results). These include a map of the 3 groundwater/aquifer settings represented in the UK HOST map (1 km raster), as well as a textural groups map and physicochemical properties map, both produced from the 2 national soil maps (NATMAP for England & Wales and the National Soil Map of Scotland) that have been drawn at a 1:250,000 scale and rasterised to 250 m resolution.

Overall results of the comparison with third-party reference data:

- Subsurface drainage: overall agreement = 63.4 %; Cohen’s Kappa = 0.33

- Textural composition: overall agreement = 53.2 %; Cohen’s Kappa = 0.37

- Physicochemical properties: overall agreement = 72.1 %; Cohen’s Kappa = 0.35

The Cohen’s Kappa results indicate “fair agreement” across the board, which is relatively low. Low agreement does not necessarily mean poor accuracy because unavoidable factors, such as differences in native spatial resolution and how characteristics like drainage behaviour and the boundary between “shallow” and “deep” soils are defined, will compromise comparability.

TWI averages and standard deviations have been summarised for each of the 10 geomorphon landform classes to highlight the role differing landforms play in shaping surface drainage behaviour. As expected, shoulders, spurs and slopes (where we would expect to see generally freer drainage) have smaller TWI values relative to hollows, footslopes and flats (where we would expect to see greater levels of flow convergence and thus, waterlogging risk in wet conditions).

Lastly, the proportional make up of larger-scale geomorphic landscape types and aggregations of the UK Land Cover Map (LCM) classes for the year 2023 (the most recent dataset on record) were summarised for each of the 13 NCPs. The 25 m rasterised land parcels for GB dataset was used here (Morton, Marston, O'Neil, & Rowland, 2024), with cells resampled to 50 m using the nearest neighbour method to align with all other dataset layers. Most of the mineral soil categories are dominated by irregular plains and escarpments, and typically two thirds to three quarters of each of these NCPs are taken up by arable or improved grassland. The more SOM-rich soil categories are dominated by mountains & high cliffs and foothills & low cliffs, and support much greater proportions of wetlands, acid grassland, heathland & bracken and coniferous woodland habitats than their mineral counterparts. Some notable additional details include:

- Alluvial & coastal soils and soils with more impeded drainage commonly occur on flat plains and support most saltmarsh & near-shore habitats.

- The 2 shallow mineral soil NCPs are the most commonly occurring on hills, with a clear split in terms of the habitats these NCPs support: shallow calcareous soils support the bulk of the neutral & calcareous grasslands, while non-calcareous soils support far more acid grassland as well as heathland & bracken.

- Significantly disturbed or excavated soils are classed as inland rock >50 % of the time, while the most common class associated with the peat NCP is wetlands, expectations which the LCM developers described in both instances (Morton, Marston, O'Neil, & Rowland, 2024).

Several limitations should be noted here:

- At 50 m resolution, the map does come close to representing soils at field scale, but ideally, an even finer resolution map would be better. Additionally, the use of a moving neighbourhood smoothing algorithm with a 1 km diameter circle blurs out some detail at sub-field scale. Thus, this map may be better suited for comparing between fields and at larger domains such as river catchments up to full GB-scale.

- It is not possible to translate our NCPs directly into international classification systems such as the USDA & WRB, limiting international comparison with soils maps of other countries.

- Peats have stricter thresholds of depth and SOM content in Scotland (≥60 % SOM over at least 50 cm in the upper soil profile) than for England & Wales (≥35 % SOM over at least 40 cm in the upper soil profile). Thus, to make this a truly “unified map” of soil types, a consistent GB-wide definition ought to be applied. The easiest approach in theory would be to adopt the England & Wales definition across GB, meaning that some organo-mineral soils in Scotland would become peats. However, because digital soil maps do not adequately capture SOM levels in organic soils very well, this is not currently practical.

- Soils that have been artificially drained for agriculture, or sealed by urbanisation are not included here, but this applies to some existing soil maps too. Spatially explicit data on artificial drainage is lacking across GB; 50 m resolution data on urban extent is too coarse to isolate gardens, while several road networks may be too narrow to be captured adequately.

- As a map built from several other maps, it is inevitable that limitations and other artefacts will have permeated into our final dataset. Where possible, we have tried to use data that are specific to the UK context only, to try to ensure the produced map layers are as representative and as accurate as possible. But in some cases, international datasets with UK coverage (e.g. Open Street Map and EcoDataCube) were the best available options to us.

Ultimately, NATMAP and the National Soils Map of Scotland remain the official authoritative maps of British soils. What we have undertaken here is to assess the possibilities and challenges to building a soil type map from other existing spatial datasets. This does limit the possible ways we can delineate soils, and some soil categories will be at odds with the official existing maps. For instance, soils <30 cm thick are considered shallow in NATMAP, yet in our map, the depth cut-off is 50 cm because that is the threshold applied in the PMM. This does introduce some uncertainty, particularly regarding peat, which is meant to consist of ≥35 % SOM over at least 40 cm in the upper soil profile for England & Wales; ≥60 % SOM in the upper 50 cm in Scotland. With 50 cm marking the cutoff between shallow and deep in our classification, it is difficult to determine which areas of peat may be more accurately described as organo-mineral soils instead; there are approximately 7,100 km2 (~26 %) of peat affected, overlying shallow parent material and/or either a peak or ridge where deeper soil profile development may be limited. Additionally, whilst we have done our best to represent the main controls upon soil drainage (parent material permeability and groundwater characteristics, combined with topographic features that are likely to promote or hinder drainage), other factors that impact drainage (including climate, among others mentioned above) have not been accounted for in the component datasets we used.

## Northern Ireland

Due to a lack of accessible data on soil parent material properties, it has not been possible to include Northern Ireland in this dataset. At present, it is only possible to produce the terrain variables (geomorphons, USGS landscapes, TWI, MRVBFI & MRRTFI), based on the Ordnance Survey of Northern Ireland (OSNI) 50 m DTM. Under a future project, we hope we will be able to release a fully UK-wide dataset.

# Details of data structure

The dataset consists of 9 single-band GeoTIFF raster files that can be loaded in GIS software, such as ArcMap and QGIS, or via scripts written in open-source programming languages such as R and Python. Each of these raster layers has the following structure:

- File format: categorical/integer rasters = single-band, 32-bit unsigned integer raster; continuous/float rasters = single-band, 32-bit floating point

- Spatial extent: xmin = 0.000; ymin = 0.000; xmax = 660000.000; ymax = 1230000.000

- Grid resolution: 50 metres

- Coordinate Reference System: EPSG: 27700 (British National Grid)

- Temporal extent: 2020. Our dataset is considered to be representative of soils and landscapes as of the year 2020, based on the maps of soil organic carbon content and locations of quarries and landfills we used. However, it should be stressed that several soil properties, including texture, depth, calcium carbonate contents and drainage reflect much longer-term conditions.

For further details on each of the map layers, please refer to Data generation methods and Nature of recorded values.

# References

Boorman, D. B., Hollis, J. M., & Lilly, A. (1995). Hydrology of soil types: a hydrologically based classification of the soils of the United Kingdom. Wallingford: Institute of Hydrology. Retrieved from https://nora.nerc.ac.uk/id/eprint/7369/

Evans, C., Artz, R., Moxley, J., Smyth, M.-A., Taylor, E., Archer, N., . . . Potts, J. (2017). Implementation of an Emissions Inventory for UK Peatlands. Report to the Department for Business, Energy and Industrial Strategy. Bangor: Centre for Ecology and Hydrology. Retrieved from https://uk-air.defra.gov.uk/library/reports?report_id=980

Feeney, C. J., Robinson, D. A., & Emmett, B. A. (2025). Response to commentary by Hollis et al. on “The development of soil health benchmarks for managed and semi-natural landscapes”. Science of the Total Environment, 984, 179711. Retrieved from https://doi.org/10.1016/j.scitotenv.2025.179711

Feeney, C. J., Robinson, D. A., Keith, A. M., Vigier, A., Bentley, L., Smith, R. P., . . . Emmett, B. A. (2023). Development of soil health benchmarks for managed and semi-natural landscapes. Science of the Total Environment, 886, 163973. Retrieved from https://doi.org/10.1016/j.scitotenv.2023.163973

Gallant, J. C., & Dowling, T. I. (2003). A multiresolution index of valley bottom flatness for mapping depositional areas. Water Resources Research, 39(12), 1347. Retrieved from https://doi.org/10.1029/2002WR001426

Hammond, E. H. (1964). Analysis of Properties in Land Form Geography - An Application to Broad-Scale Land Form Mapping. Annals of the Association of American Geographers, 54(1), 11-19.

Jasiewicz, J., & Stepinski, T. J. (2013). Geomorphons—a pattern recognition approach to classification and mapping of landforms. Geomorphology, 182, 147-156. Retrieved from https://doi.org/10.1016/j.geomorph.2012.11.005

Lawley, R. (2021). User Guide: Soil Parent Material Dataset. Keyworth, Nottingham: British Geological Survey.

McKenzie, A. A. (2014). User Guide for the British Geological Survey National Depth to Groundwater Dataset. Keyworth, Nottingham: British Geological Survey.

Morton, R. D., Marston, C. G., O'Neil, A. W., & Rowland, C. S. (2024, May 17). Land Cover Map 2023 (25m rasterised land parcels, GB). Retrieved from NERC EDS Environmental Information Data Centre: https://doi.org/10.5285/ab10ea4a-1788-4d25-a6df-f1aff829dfff

Sayre, R., Comer, P., Warner, H., & Cress, J. (2009). A new map of standardized terrestrial ecosystems of the conterminous United States. Reston, Virginia, USA: U. S. Geological Survey. Retrieved from https://pubs.usgs.gov/pp/1768/

Taylor, P. J. (2021). Spatial inventory of UK waterbodies. Retrieved from NERC EDS Environmental Information Data Centre: https://doi.org/10.5285/b6b92ce3-dcd7-4f0b-8e43-e937ddf1d4eb

True, D., Gordon, T., & Diamond, D. (2000). How the size of a sliding window impacts the generation of landforms. Columbia, Missouri, USA: Missouri Resources Assessment Partnership.

Witjes, M. (2022, June 16). OpenStreetMap+ Land Use / Land Cover classes and administrative regions of Europe (0.1) [Data set]. Retrieved from Zenodo: https://zenodo.org/records/6653917

# Appendix 1: ESB class translation table

| ESB name | Main group | Likely permeability | Alluvial or coastal? |
| --- | --- | --- | --- |
| ACID AND ULTRABASIC PLUTONIC ROCKS | Igneous rocks | Impermeable | No |
| ACID PLUTONIC ROCKS AND TUFF | Igneous rocks | Impermeable | No |
| ACID REGIONAL METAMORPHIC ROCKS | Metamorphic rocks | Impermeable | No |
| ACID TO INTERMEDIATE ROCKS | Igneous rocks | Impermeable | No |
| ACID TO INTERMEDIATE VOLCANIC ROCKS | Igneous rocks | Impermeable | No |
| AMPHIBOLITE | Metamorphic rocks | Impermeable | No |
| ANDESITE | Igneous rocks | Impermeable | No |
| ANDESITE AND BASALT | Igneous rocks | Impermeable | No |
| ANHYDRITE | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| ARGILLITE-SLATE | Metamorphic rocks | Impermeable | No |
| BASALT | Igneous rocks | Impermeable | No |
| BASALT AND ANDESITE | Igneous rocks | Impermeable | No |
| BASALT AND GABBRO | Igneous rocks | Impermeable | No |
| BASALT AND SANDSTONE | Igneous rocks | Impermeable | No |
| BASALT AND TUFF | Igneous rocks | Impermeable | No |
| BASIC AND ULTRABASIC PLUTONIC ROCKS | Igneous rocks | Impermeable | No |
| BASIC PLUTONIC ROCKS | Igneous rocks | Impermeable | No |
| BASIC REGIONAL METAMORPHIC ROCKS | Igneous rocks | Impermeable | No |
| BASIC TO ULTRABASIC VOLCANIC ROCKS | Igneous rocks | Impermeable | No |
| BASIC TO ULTRABASIC VOLCANIC ROCKS AND CLASTIC ROCKS | Igneous rocks | Impermeable | No |
| BRECCIA | Consolidated clastic sedimentary rocks | Permeable | No |
| BRECCIA AND SANDSTONE | Consolidated clastic sedimentary rocks | Permeable | No |
| CALCAREOUS METAMORPHIC ROCKS AND CALCSILICATES | Metamorphic rocks | Impermeable | No |
| CALCAREOUS METAMORPHIC ROCKS AND CLASTIC ROCKS | Metamorphic rocks | Impermeable | No |
| CALCAREOUS METAMORPHIC ROCKS AND MUDSTONES | Metamorphic rocks | Impermeable | No |
| CALCAREOUS REGIONAL METAMORPHIC ROCKS | Metamorphic rocks | Impermeable | No |
| CALCAREOUS ROCKS | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| CALCAREOUS ROCKS AND CLASTIC ROCKS | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| CALCAREOUS ROCKS AND METAMORPHIC ROCKS | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| CALCAREOUS ROCKS AND MUDSTONE | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| CALCAREOUS ROCKS AND SANDSTONE | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| CALCSILICATE | Metamorphic rocks | Impermeable | No |
| CALCSILICATES AND CALCAREOUS METAMORPHIC ROCKS | Metamorphic rocks | Impermeable | No |
| CALCSILICATES AND MUDSTONES | Metamorphic rocks | Impermeable | No |
| CALCSILICATES AND PSAMMITE | Metamorphic rocks | Impermeable | No |
| CALCSILICATES AND TUFF | Metamorphic rocks | Impermeable | No |
| CHALK | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| CHERT | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| CLASTIC ROCKS | Consolidated clastic sedimentary rocks | Permeable | No |
| CLASTIC ROCKS WITH CALCAREOUS METAMORPHIC ROCKS | Consolidated clastic sedimentary rocks | Permeable | No |
| CLASTIC ROCKS WITH CALCSILICATE ROCKS | Consolidated clastic sedimentary rocks | Permeable | No |
| CLASTIC ROCKS WITH CHEMICALLY PRECIPITATED ROCKS | Consolidated clastic sedimentary rocks | Permeable | No |
| CLASTIC ROCKS WITH DOLOMITE | Consolidated clastic sedimentary rocks | Permeable | No |
| CLASTIC ROCKS WITH LIMESTONE | Consolidated clastic sedimentary rocks | Permeable | No |
| CLASTIC ROCKS WITH METAMORPHIC ROCKS | Consolidated clastic sedimentary rocks | Permeable | No |
| CLASTIC ROCKS WITH PLUTONIC BASIC ROCKS | Consolidated clastic sedimentary rocks | Permeable | No |
| CLASTIC ROCKS WITH TUFFS | Consolidated clastic sedimentary rocks | Permeable | No |
| CLAY WITH FLINTS | Consolidated clastic sedimentary rocks | Impermeable | No |
| CLAYSTONE/MUDSTONE | Consolidated clastic sedimentary rocks | Impermeable | No |
| COAL | Organic materials | Impermeable | No |
| COLLUVIUM | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | No |
| CONGLOMERATE | Consolidated clastic sedimentary rocks | Permeable | No |
| CONGLOMERATE AND SANDSTONE | Consolidated clastic sedimentary rocks | Permeable | No |
| CONTACT METAMORPHIC ROCKS | Metamorphic rocks | Impermeable | No |
| CONTACT METAMORPHIC ROCKS AND QUARTZITE | Metamorphic rocks | Impermeable | No |
| DACITE | Igneous rocks | Impermeable | No |
| DACITE AND TUFF | Igneous rocks | Impermeable | No |
| DIORITE | Igneous rocks | Impermeable | No |
| DIORITE AND ANDESITE | Igneous rocks | Impermeable | No |
| DIORITE AND BASIC VOLCANIC ROCKS | Igneous rocks | Impermeable | No |
| DIORITE AND GABBRO | Igneous rocks | Impermeable | No |
| DIORITE AND GRANITE | Igneous rocks | Impermeable | No |
| DIORITE AND GRANODIORITE | Igneous rocks | Impermeable | No |
| DIORITE AND SYENITE | Igneous rocks | Impermeable | No |
| DOLOMITE AND CALCAREOUS ROCKS | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| DOLOMITE AND LIMESTONE | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| DOLOMITE AND MUDSTONE | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| DOLOMITE AND PSAMMITE | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| DOLOMITE AND SANDSTONE | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| DOLOSTONE | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| EOLIAN SAND | Eolian deposits | Permeable | No |
| FERRUGINOUS SANDSTONE | Consolidated clastic sedimentary rocks | Permeable | No |
| FLOODPLAIN CLAY, SAND/GRAVEL | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| FLOODPLAIN SAND/GRAVEL | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| FLUVIAL CLAYS AND SILTS | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| FLUVIAL CLAYS, SILTS SANDS AND GRAVEL | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| FLUVIAL SANDS AND GRAVELS | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| GABBRO | Igneous rocks | Impermeable | No |
| GABBRO AND BASALT | Igneous rocks | Impermeable | No |
| GABBRO AND BASIC VOLCANIC ROCKS | Igneous rocks | Impermeable | No |
| GABBRO AND SANDSTONE | Igneous rocks | Impermeable | No |
| GABBRO AND ULTRABASIC PLUTONIC ROCKS | Igneous rocks | Impermeable | No |
| GLACIAL TILL | Unconsolidated glacial deposits / glacial drift | Impermeable | No |
| GLACIOFLUVIAL AND GLACIOLACUSTRINE DEPOSITS | Unconsolidated glacial deposits / glacial drift | Permeable | No |
| GLACIOFLUVIAL DEPOSITS | Unconsolidated glacial deposits / glacial drift | Permeable | No |
| GLACIOLACUSTRINE | Unconsolidated glacial deposits / glacial drift | Impermeable | No |
| GNEISS | Metamorphic rocks | Impermeable | No |
| GRANITE | Igneous rocks | Impermeable | No |
| GRANITE AND GABBRO | Igneous rocks | Impermeable | No |
| GRANODIORITE | Igneous rocks | Impermeable | No |
| GRANODIORITE AND ANDESITE | Igneous rocks | Impermeable | No |
| GRANODIORITE AND DIORITE | Igneous rocks | Impermeable | No |
| GREENSCHIST | Metamorphic rocks | Impermeable | No |
| GREYWACKE | Consolidated clastic sedimentary rocks | Permeable | No |
| GYPSUM | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| HALITE | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| HOLOCENE SILT | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | No |
| HORNFELS | Metamorphic rocks | Impermeable | No |
| LACUSTRINE SAND AND SILT | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | No |
| LAKE DEPOSITS | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | No |
| LAKE SAND | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | No |
| LAKE SILT | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | No |
| LAMPROPHYRE | Igneous rocks | Impermeable | No |
| LIMESTONE | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| LIMESTONE AND CLASTIC ROCKS | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| LIMESTONE AND CONGLOMERATE | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| LIMESTONE AND DOLOMITE | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| LOAM LOESS | Eolian deposits | Permeable | No |
| LOAMY AND SANDY LOESS | Eolian deposits | Permeable | No |
| MARINE AND ESTUARINE SANDS | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| MARINE AND ESTUARINE SANDS AND SILTS | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| METAMORPHIC ROCKS | Metamorphic rocks | Impermeable | No |
| METAMORPHIC ROCKS AND CALCAREOUS METAMORPHIC ROCKS | Metamorphic rocks | Impermeable | No |
| MICA SCHIST | Metamorphic rocks | Impermeable | No |
| MORAINIC DEPOSITS | Unconsolidated glacial deposits / glacial drift | Permeable | No |
| MORANIC AND GLACIOFLUVIAL DEPOSITS | Unconsolidated glacial deposits / glacial drift | Permeable | No |
| MUDSTONE AND BRECCIA | Consolidated clastic sedimentary rocks | Impermeable | No |
| MUDSTONE AND CALCSILICATE ROCKS | Consolidated clastic sedimentary rocks | Impermeable | No |
| MUDSTONE AND CONGLOMERATE | Consolidated clastic sedimentary rocks | Impermeable | No |
| MUDSTONE AND CONTACT METAMORPHIC ROCKS | Consolidated clastic sedimentary rocks | Impermeable | No |
| MUDSTONE AND DOLOMITE | Consolidated clastic sedimentary rocks | Impermeable | No |
| MUDSTONE AND METAMORPHIC ROCKS | Consolidated clastic sedimentary rocks | Impermeable | No |
| MUDSTONE AND REGIONAL METAMORPHIC ROCKS CALCAREOUS | Consolidated clastic sedimentary rocks | Impermeable | No |
| MUDSTONE AND REGIONAL METAMORPHIC ROCKS ULTRABASIC | Consolidated clastic sedimentary rocks | Impermeable | No |
| MUDSTONE AND SANDSTONE | Consolidated clastic sedimentary rocks | Impermeable | No |
| MUDSTONE AND TUFF | Consolidated clastic sedimentary rocks | Impermeable | No |
| MYLONITE | Metamorphic rocks | Impermeable | No |
| NA | NA | NA | NA |
| OUTWASH SAND | Unconsolidated glacial deposits / glacial drift | Permeable | No |
| OUTWASH SAND AND OUTWASH GRAVEL | Unconsolidated glacial deposits / glacial drift | Permeable | No |
| OVERBANK DEPOSITS | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| PEAT | Organic materials | Impermeable | No |
| PEAT AND GLACIOLACUSTRINE DEPOSITS | Organic materials | Impermeable | No |
| PEGMATITE | Igneous rocks | Impermeable | No |
| PELITE, ARGILLITE OR LUTITE (MUDROCKS) | Consolidated clastic sedimentary rocks | Impermeable | No |
| PERIDOTITE | Igneous rocks | Impermeable | No |
| PREQUATERNARY MARINE/ESTUARINE CLAY/SILT | Pre-Quaternary Consolidated Clay/silt | Impermeable | No |
| PREQUATERNARY MARINE/ESTUARINE SAND | Pre-Quaternary Consolidated Clay/silt | Impermeable | No |
| PREQUATERNARY MARINE/ESTUARINE SAND AND SILT | Pre-Quaternary Consolidated Clay/silt | Impermeable | No |
| PSAMMITE AND CALCSILICATE ROCKS | Consolidated clastic sedimentary rocks | Permeable | No |
| PSAMMITE OR ARENITE | Consolidated clastic sedimentary rocks | Permeable | No |
| PSEPHITE OR RUDITE | Consolidated clastic sedimentary rocks | Permeable | No |
| PYROCLASTIC ROCKS | Igneous rocks | Impermeable | No |
| PYROCLASTIC ROCKS AND SANDSTONE | Igneous rocks | Impermeable | No |
| QUARTZITE | Consolidated clastic sedimentary rocks | Impermeable | No |
| QUARTZITE AND CLASTIC ROCKS | Consolidated clastic sedimentary rocks | Permeable | No |
| QUARTZITE AND CONGLOMERATE | Consolidated clastic sedimentary rocks | Permeable | No |
| QUARTZITE AND MUDSTONE | Consolidated clastic sedimentary rocks | Impermeable | No |
| QUARTZITE AND PSAMMITE | Consolidated clastic sedimentary rocks | Impermeable | No |
| QUATERNARY MARINE/ESTUARINE CLAY/SILT | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Impermeable | Yes |
| QUATERNARY MARINE/ESTUARINE SAND | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| QUATERNARY MARINE/ESTUARINE SAND AND SILT | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| RESIDUAL CLAY | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | No |
| RESIDUAL CLAY AND LOAMY LOESS | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | No |
| RESIDUAL LOAM | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | No |
| RHYOLITE | Igneous rocks | Impermeable | No |
| RIVER CLAY AND SILT | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| RIVER LOAM | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| RIVER TERRACE SAND/GRAVEL | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| RIVERINE CLAY AND FLOODPLAIN SANDS AND GRAVEL | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| RIVERINE CLAY AND FLUVIAL SANDS AND GRAVEL | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| SAND LOESS | Eolian deposits | Permeable | No |
| SANDSTONE | Consolidated clastic sedimentary rocks | Permeable | No |
| SANDSTONE AND BRECCIA | Consolidated clastic sedimentary rocks | Permeable | No |
| SANDSTONE AND CONGLOMERATE | Consolidated clastic sedimentary rocks | Permeable | No |
| SANDSTONE AND FERRUGINOUS SANDSTONE | Consolidated clastic sedimentary rocks | Permeable | No |
| SANDSTONE AND GYPSUM | Consolidated clastic sedimentary rocks | Permeable | No |
| SANDSTONE AND MUDSTONE | Consolidated clastic sedimentary rocks | Permeable | No |
| SCHIST AND GRANITE | Metamorphic rocks | Impermeable | No |
| SCHIST AND PSAMMITE | Metamorphic rocks | Impermeable | No |
| SEDIMENTARY ROCKS (CHEMICAL OR BIOGENIC ORIGIN) | Sedimentary rocks (chemically precipitated, evaporated, or of organogenic or biogenic origin) | Permeable | No |
| SERPENTINITE | Metamorphic rocks | Impermeable | No |
| SERPENTINITE AND GABBRO | Metamorphic rocks | Impermeable | No |
| SILTSTONE | Consolidated clastic sedimentary rocks | Impermeable | No |
| SILTSTONE AND BASALT | Consolidated clastic sedimentary rocks | Impermeable | No |
| SILTSTONE AND CHERT | Consolidated clastic sedimentary rocks | Impermeable | No |
| SILTSTONE AND GYPSUM | Consolidated clastic sedimentary rocks | Impermeable | No |
| SILTSTONE AND HALITE | Consolidated clastic sedimentary rocks | Impermeable | No |
| SILTSTONE AND LIMESTONE | Consolidated clastic sedimentary rocks | Impermeable | No |
| SILTSTONE AND TUFF | Consolidated clastic sedimentary rocks | Impermeable | No |
| SLATE AND LIMESTONE | Metamorphic rocks | Impermeable | No |
| SLIME AND OOZE | Organic materials | Impermeable | No |
| SLOPE DEPOSITS | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | No |
| SYENITE | Igneous rocks | Impermeable | No |
| TALUS | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | No |
| TECTONIC BRECCIA | Metamorphic rocks | Permeable | No |
| TERRACE CLAY | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| TERRACE CLAY AND LOAMY LOESS | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| TRACHYTE | Igneous rocks | Impermeable | No |
| TUFF | Igneous rocks | Impermeable | No |
| TUFF AND BASALT | Igneous rocks | Impermeable | No |
| TUFF AND LIMESTONE | Igneous rocks | Permeable | No |
| ULTRABASIC PLUTONIC ROCKS | Igneous rocks | Impermeable | No |
| UNCONSOLIDATED FLUVIAL AND GLACIGENIC DEPOSITS | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| UNCONSOLIDATED FLUVIAL DEPOSITS | Unconsolidated deposits (alluvium, weathering residuum and Slope deposits) | Permeable | Yes |
| UNCONSOLIDATED GLACIAL DEPOSITS | Unconsolidated glacial deposits / glacial drift | Permeable | No |
| VOLCANIC ASH | Igneous rocks | Permeable | No |
| VOLCANIC BRECCIA | Igneous rocks | Permeable | No |
| WEAKLY METAMORPHIC | Metamorphic rocks | Impermeable | No |
| WEAKLY METAMORPHIC ROCKS AND SANDSTONE | Metamorphic rocks | Permeable | No |

# Appendix 2: Map Evaluation Results

## Correspondence matrix for subsurface drainage characteristics.

Column names from HOST 1km map of the UK; row names from the Subsurface Drainage layer in this dataset. Values are areas in km2.

| Our data | Reference data |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Permeable over deep (>2m) groundwater | Permeable over shallow (≤2m) groundwater | Impermeable; no significant groundwater present | Total | User’s accuracy (%) |
| Permeable over deep (>2m) groundwater | 33078 | 1266 | 18032 | 52376 | 63.2 |
| Permeable over shallow (≤2m) groundwater | 8893 | 9472 | 6781 | 25146 | 37.7 |
| Impermeable; no significant groundwater present | 37192 | 10353 | 100464 | 148009 | 67.9 |
| Total | 79163 | 21091 | 125277 | 225531 |  |
| Producer’s accuracy (%) | 41.8 | 44.9 | 80.2 |  |  |
| Overall accuracy (%) | 63.4 |  |  |  |  |

Cohen’s Kappa = 0.33

## Correspondence matrix for soil textural characteristics.

Column names from a combined map of NATMAP topsoil texture and soil texture according to the National Soil Map of Scotland (NSMS); row names from the Textural Composition layer in this dataset. Values are areas in km2.

| Our data | Reference data |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Light | Medium or mixed | Heavy | Organic | Peat | Total | User’s accuracy (%) |
| Light | 23138 | 5428 | 652 | 898 | 1765 | 31881 | 72.6 |
| Medium or mixed | 23903 | 62639 | 19661 | 2903 | 373 | 109480 | 57.2 |
| Heavy | 2332 | 7857 | 5564 | 78 | 32 | 15864 | 35.1 |
| Organic | 9567 | 3030 | 382 | 6126 | 21923 | 41028 | 14.9 |
| Peat | 1541 | 473 | 1254 | 1354 | 22387 | 27009 | 82.9 |
| Total | 60482 | 79428 | 27513 | 11360 | 46481 | 225263 |  |
| Producer’s accuracy (%) | 38.3 | 78.9 | 20.2 | 53.9 | 48.2 |  |  |
| Overall accuracy (%) | 53.2 |  |  |  |  |  |  |

Cohen’s Kappa = 0.37

## Correspondence matrix for soil depth, CaCO3 contents and presence of alluvial or coastal parent materials.

Column names from major physicochemical parent material groups derived from NATMAP and the NSMS; row names from the Physicochemical properties layer in this dataset. Values are areas in km2.

| Our data | Reference data |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Deep non-calcareous | Deep calcareous | Shallow non-calcareous | Shallow calcareous | Alluvial and coastal | Total | User’s accuracy (%) |
| Deep non-calcareous | 96870 | 8133 | 474 | 1213 | 2837 | 109527 | 88.4 |
| Deep calcareous | 2656 | 2605 | 15 | 5370 | 96 | 10742 | 24.3 |
| Shallow non-calcareous | 10277 | 74 | 309 | 17 | 116 | 10792 | 2.9 |
| Shallow calcareous | 1817 | 589 | 258 | 1830 | 28 | 4521 | 40.5 |
| Alluvial and coastal | 7847 | 802 | 12 | 180 | 8904 | 17745 | 50.2 |
| Total | 119466 | 12203 | 1067 | 8610 | 11980 | 153327 |  |
| Producer’s accuracy (%) | 81.1 | 21.3 | 28.9 | 21.3 | 74.3 |  |  |
| Overall accuracy (%) | 72.1 |  |  |  |  |  |  |

Cohen’s Kappa = 0.35

## Landforms with summarised surface drainage statistics

## MRRTFI and MRVBFI summarised by NCP

Flatness indices are strongly right skewed, so medians are presented here instead of means. Graph plotted on a logarithmic scale so that flatness indices for the organo-mineral NCPs (<10-4) are visible (all other NCPs show values >10-2). Threshold value to be considered flat (for either valley bottom or ridge top terrains) is displayed as a dotted line.

## USGS geomorphic landscapes summarised per NCP

## UKCEH LCM 2023 classes summarised per NCP

For more information 
please contact:

[Email us]

Enquiries@ceh.ac.uk

[Learn more]

ceh.ac.uk

[Follow us]

@uk_ceh

Most maps use colours taken from the “paletteMartin” discrete colour scheme in R’s paletteer package; for the geomorphon landforms map, we applied an 8-step blue to orange scheme with grey and black colours for classes 1 & 10, respectively – see: https://r-charts.com/color-palettes/#discrete [last accessed 22/01/2026]. Specific colour pairings with categories were trialled in QGIS using viewer tools that simulate the various forms of colour-blindness. While we have tried to accommodate colour blindness as best as possible (particularly the more common forms of deuteranopia and protanopia), the 13 categories in the NCP map makes this challenging. Thus, we accept that some categories may prove difficult to differentiate when viewing this map, especially for persons with tritanopia or achromatopsia. Other maps with ≤6 categories should be much easier to interpret, including for most forms of colour blindness. Ultimately, the colour schemes chosen here are suggestions which you do not have to follow yourself, and we are open to alternative suggestions to improve accessibility.