Supporting Document

Surface temperature, surface oxygen, water clarity, water chemistry and phytoplankton chlorophyll a data from Windermere North Basin, UK, 2025.  Feuchtmayr, H., Alemany, A., Carter, H.T., Dodd, B.A., Hill, I., Keenan, P.O., Le Quesne, K., McShane, G., Moorhouse, H., Rhodes, G., Thackeray, S.J. (2026)

Sampling Regime and Collection Methods

This is part of an ongoing monitoring dataset of surface temperature, surface oxygen, water clarity, water chemistry and phytoplankton chlorophyll a from fortnightly sampling at the North Basin of Windermere in Cumbria, England, that began in 1945 for some variables. The data have been collected by the UK Centre for Ecology & Hydrology (UKCEH). The data available to download comprise surface temperature (TEMP) in degree Celsius, surface oxygen saturation (OXYG) in % air-saturation, Secchi depth (SECC) in metres, alkalinity (ALKA) in µg per litre as CaCO3 and pH. Total ammoniacal nitrogen (NH4N), total oxidised nitrogen (TON), soluble reactive phosphate (PO4P), total phosphorus (TOTP), dissolved reactive silicon expressed as SiO2 (SIO2) and phytoplankton chlorophyll a (TOCA) are all given in µg per litre. Water samples are based on a sample integrated from 0 to 7 m. Water chemistry values below the detection limit are indicated by a < sign in the column titled ‘sign(if<LOD)’. Please beware that the limits of detection for NH4N, TON, TOTP and SIO2 are reviewed annually and can fluctuate. Measurements are made from a boat at a marked location (buoy) at the deepest part of the basin. When it was not possible to visit the buoy, samples were taken from the shore along with surface temperature and oxygen measurements (from 2023 onwards). Water samples were not integrated on these occasions, marked as Flag 2. All data are from January 2025 until the end of 2025.

Quality Control

Please note that the data presented here are raw data that have not yet been quality controlled and calibrated. However, data for TEMP, OXYG, SECC, ALKA, pH, PO4P and TOCA has been double entered by different people into excel from field notebooks or laboratory paper record sheets and validated by a database manager. The database manager checked all data visually (line graphs). Missing data are due to weather conditions, equipment limitations, or staff shortage.

Data structure

Data are provided in a comma separated values file with columns as below.

| Column name | Description |
| --- | --- |
| sdate | Date of measurement |
| variable | Surface temperature (TEMP) in degree Celsius, surface oxygen saturation (OXYG) in % air-saturation, Secchi depth (SECC) in metres, alkalinity (ALKA) in µg CaCO3 per litre, pH, total ammoniacal nitrogen (NH4N) in µg N per litre, total oxidised nitrogen (TON) in µg N per litre, soluble reactive phosphate (PO4P) in µg P per litre, total phosphorus (TOTP) in µg P per litre, dissolved reactive silicon (SIO2) in µg per litre, phytoplankton chlorophyll a (TOCA) in µg per litre |
| value | Values for each variable |
| sign(if<LOD) | If values are below detection limit, there is a < sign |
| flag | If flag = 1 then sample was taken on the lake at the marked location (buoy). If flag = 2 then sample was taken from the shore as a visit to the buoy was not possible |

Methods

Water temperature and dissolved oxygen are measured with a YSI ProODO handheld Instrument (Xylem Analytics UK). The ProODO measures dissolved oxygen in mg per litre (DO) and saturation is calculated based on Mortimer (1981). The probe is calibrated for DO % on the day of sampling and must reach between 98% and 102% to ‘pass’. Further calibrations: biweekly single point calibration of DO % in water saturated air, and bimonthly single point calibration of DO % in air saturated water. Secchi depth is recorded by lowering a Secchi disk into the water on a measured and labelled rope.

For alkalinity and pH, the integrated water sample is sealed with a glass-stoppered bottle to prevent atmospheric exchange and measured via Gran titration with a combination electrode (Radiometer Electrode GK2401C on a Radiometer PHM64). The electrode is calibrated before each sampling occasion.

Total ammoniacal nitrogen, i.e. ammonium nitrogen (NH₄-N) and ammonia nitrogen (NH3-N), concentrations are measured colourimetrically using a SEAL AQ2 discrete analyser following filtration through a Whatman GF/C filter. The reaction is carried out under alkaline conditions, which ensures that any ammonium ions are converted to ammonia and are measured in the following reaction. Ammonia reacts with hypochlorite ions (from sodium dichloroisocyanurate) and salicylate in the presence of sodium nitroprusside, at a pH of approximately 12.6, to form a blue-green reaction product (related to indophenol blue). This is measured spectrophotometrically at 670 nm. A calibration is produced by automatic dilution of a single stock solution of 2.0 mg/l NH3-N; concentrations are obtained using the calibration curve. Control standards of 1.0mg/l NH3-N are analysed every 10 samples which, in addition to selected sample duplicates, are used to check precision. Certified reference materials containing NH4-N are used to check accuracy. The validated range is 0 – 2.0 mg/l. The method is UKAS accredited.

Total oxidised nitrogen (TON) concentrations are measured colorimetrically using a Seal AQ2 discrete analyser following filtration through a Whatman GF/C filter. Nitrate is reduced to nitrite by hydrazine under alkaline conditions, using cupric ion as a catalyst. The reduced nitrate, together with any nitrite originally present, reacts with sulphanilamide to form a diazonium compound which, in dilute phosphoric acid, couples with N-1-naphthylethylene diamine dihydrochloride to form a reddish-purple azo dye. This dye is measured at 546 nm. Calibration is produced by automatic dilution of a single stock solution of 2.0 mg/l N; concentrations are obtained using the calibration curve within the range 0 – 2.0mg/l. Control standards of 1.0mg/l N are analysed every 10 samples. The method is UKAS accredited.

Total Phosphorus (TP) using a potassium persulphate (K2S2O8) digestion and subsequent molybdenum blue colorimetric analysis was carried out using a Shimadzu UV-150-02 double beam spectrophotometer.

Unfiltered samples were digested in a matrix of K2S2O8 and 1N sulphuric acid.  These were then placed in an autoclave under heat and pressure to complete the digestion procedure.

Once digested the phosphorus in the samples is in the form of ortho-phosphate which was determined using a molybdenum blue chemistry with the addition of ascorbic acid to control the colour production. The concentration of ortho-phosphate, and thereby total phosphorus, related to the absorbance measurements obtained at 880 nm using the UV-150-02 spectrophotometer.

Calibrations were run using standards prepared and digested in the same manner as the samples in a range of 0 - 0.2 mg/L of P.  Selected samples were repeated to check for precision and reference samples, diluted from a certified reference material, were run alongside samples to check for accuracy.  The data was blank corrected.

To determine dissolved reactive silicon, a Seal AQ2 analyser was used to measure silicon (as SiO2), with a colorimetric method in filtered samples.  The reactive silica in the sample combined with ammonium molybdate, which was then reduced with ascorbic acid to form a silicomolybenum blue complex.  The blue complex was measured spectrophotometrically at 880nm. The calibration range of the method was 0-5mg/L of SiO2, with blanks run to assess carry over and background readings.  Quality control samples at concentrations of 2.0mg/L and 4.0mg/L were run at the start of analysis and then ever 20 subsequent samples.

To determine phytoplankton chlorophyll a, a known volume of the integrated water sample is filtered through a Whatman GF/C filter. The pigments are extracted using a boiling methanol extraction as described by Talling (1974) and analysed spectrophotometrically.

Soluble reactive phosphate was measured following filtration through a Whatman GF/C filter and measured colorimetrically (molybdenum blue colorimetric analysis) according to Mackereth et al (1978). A calibration along with an AQC sample (to check for accuracy) is run before each sampling occasion and blanks are used for every lake sample.

Funding

Data collection at and validation for North Basin of Windermere was supported by NERC, through the UKCEH National Capability for UK Challenges Programme NE/Y006208/1.

References

Mackereth F.J.H., Heron J., Talling J.F. (1978) Water Analysis: Some Revised Methods for Limnologists. Vol 36. Freshwater Biological Association Publ.

Mortimer C.H. (1981) The oxygen content of air-saturated fresh waters over ranges of temperature and atmospheric pressure of limnological interest. Internationale Vereinigung Für Theoretische Und Angewandte Limnologie: Mitteilungen, 22(1), 1–23. https://doi.org/10.1080/05384680.1981.11904000

Talling J.F. (1974) In standing waters. In: Vollenweider R.A. (Ed.), A Manual on Methods for Measuring Primary Production in Aquatic Ecosystems. Blackwell Publishing. Oxford.