# Raw Imaging Survey Data

This dataset contains RAW nighttime imagery recorded using a DSLR camera during a survey of artificial light in nighttime environments across Exeter, UK between February and May 2023. This dataset includes the CR2 imagery. The DSLR camera was setup on a tripod at each location. HDR imagery was taken in each cardinal direction, as well as a final photo towards zenith. The locations were chosen via randomly selecting roads through stratified random sampling of built area ratio and foliage coverage present within land usage parcels. We made each draw in a round-robin manner in quadrants of the city to ensure that they were also spatially well distributed. This data was collected by fieldworkers at the University of Exeter (primarily Sam Morrell and Maisy Inston) to process into flux-calibrated 360 degree maps of the lighting environment across the city.

# Summary

This is a collection of photos that document in-situ observations of light conditions on Exeter streets, as part of a greater study measuring artificial light at night (ALAN) using various methods.

# File format

CSV and CR2 formats. CR2 format images contain the raw pixel counts measured from the CMOS sensor, through the Bayer matrix, on the DLSR camera.

# File names and/or naming convention

- 12 folders formatted YYYY-MM-DD

  - Each folder contains a variable number of image files, formatted IMG_XXXX.CR2.

  - Additionally, each folder contains a pdf file formatted YYYY-MM-DD_log.pdf

- 1 folder labelled “gps”

  - Containing 16 files with the naming convention “YYMMDD00.LOG”

- 1 file named “3EV_Measurements.csv”

- 1 file named “Draws.csv”

- 1 file named “Locations.csv”

- 1 file named “Measurements.csv”

# Nature and units of recorded values

The raw imagery in the dataset is made up of the RAW (.CR2) image. The imagery has the following specifications

- Resolution: 26.2 megapixel

- Focal length: 8mm

Table 1. Nature of variables in observing logs (e.g., 2023-03-17.pdf)

| Variable/Column header | Description | Units | Vocabulary URL (if applicable) |
| --- | --- | --- | --- |
| Image No. | A unique number that identifies each image | None | http://onto.nerc.ac.uk/CAST/16 |
| Direction (headers labelled “North”, “East”, “South”, “West”, “Sky”) | The direction in which the photos were taken from each location | Cardinal directions, or “Sky” |  |
| Exposure Length (“Short”, “Medium”, “Long”) | The exposure length of each photo |  |  |
| Date/Time | The time (in 24 hour format) the sampling was completed in each location | Hour and minute; UTC or BST depending on the observation date |  |
| Location | The sampling location described in 3 ways | -A short description such as a road or area name -Location ID, e.g. “SE-CPM” -what3words location identifier, e.g., “origin doctor credit” |  |

# Spatial coverage

City of Exeter, Devon, England, United Kingdom

The table below provides time and location data for camera campaign observations. This includes location_id, latitude & longitude (in WGS 84), date, and time (in UTC) the observations were completed in each location.

| location_id | lat | lon | date | time_finished_utc |
| --- | --- | --- | --- | --- |
| SE-HFD | 50.694251 | -3.46551 | 22/02/2023 | 19:53:00 |
| SE-SHL | 50.688861 | -3.46602 | 22/02/2023 | 20:10:00 |
| SE-ASH | 50.687406 | -3.46895 | 22/02/2023 | 20:30:00 |
| SE-SAN | 50.69188 | -3.47968 | 22/02/2023 | 20:51:00 |
| SE-DAA | 50.694494 | -3.4784 | 22/02/2023 | 21:20:00 |
| SE-SAV | 50.694844 | -3.47606 | 22/02/2023 | 21:36:00 |
| SE-NPE | 50.693793 | -3.48406 | 22/02/2023 | 21:52:00 |
| SE-NPW | 50.694494 | -3.48696 | 22/02/2023 | 22:05:00 |
| SE-MBD | 50.69948 | -3.48227 | 22/02/2023 | 22:19:00 |
| SE-GHL | 50.696353 | -3.49679 | 22/02/2023 | 22:35:00 |
| SE-ALB | 50.704708 | -3.47776 | 22/02/2023 | 22:56:00 |
| SE-NWC | 50.707646 | -3.48015 | 22/02/2023 | 23:13:00 |
| SE-BDW | 50.713872 | -3.46884 | 22/02/2023 | 23:44:00 |
| SE-BKB | 50.710692 | -3.47028 | 23/02/2023 | 00:03:00 |
| SE-SGR | 50.71274 | -3.4624 | 23/02/2023 | 00:28:00 |
| SE-CWR | 50.690936 | -3.50628 | 23/02/2023 | 19:52:00 |
| SW-MBR | 50.692823 | -3.51764 | 23/02/2023 | 20:11:00 |
| SW-MPR | 50.698159 | -3.52377 | 23/02/2023 | 20:29:00 |
| SW-BHW | 50.699911 | -3.52356 | 23/02/2023 | 20:48:00 |
| SW-ARR | 50.703792 | -3.52556 | 23/02/2023 | 21:09:00 |
| SW-TRS | 50.707296 | -3.5296 | 23/02/2023 | 21:29:00 |
| SW-ATR | 50.708616 | -3.53606 | 23/02/2023 | 21:41:00 |
| SW-ALP | 50.705733 | -3.53782 | 23/02/2023 | 22:09:00 |
| SW-PWL | 50.700854 | -3.5325 | 23/02/2023 | 22:34:00 |
| SW-TCP | 50.697054 | -3.52807 | 23/02/2023 | 22:47:00 |
| SW-ARC | 50.697 | -3.5373 | 23/02/2023 | 23:02:00 |
| SW-TSS | 50.701636 | -3.54284 | 23/02/2023 | 23:31:00 |
| SW-WTL | 50.715812 | -3.53287 | 23/02/2023 | 23:50:00 |
| SW-EBN | 50.718534 | -3.53815 | 24/02/2023 | 00:05:00 |
| NW-RCL | 50.731309 | -3.54407 | 24/02/2023 | 00:22:00 |
| NW-EXC | 50.724868 | -3.55332 | 24/02/2023 | 19:49:00 |
| NW-CLL | 50.727347 | -3.55455 | 24/02/2023 | 20:11:00 |
| NW-BUD | 50.731067 | -3.55349 | 24/02/2023 | 20:29:00 |
| NW-OBC | 50.730123 | -3.54816 | 24/02/2023 | 21:17:00 |
| NW-KNW | 50.73298 | -3.55012 | 24/02/2023 | 21:40:00 |
| NW-SAR | 50.74209 | -3.55268 | 24/02/2023 | 22:00:00 |
| NW-CWR | 50.746698 | -3.5477 | 24/02/2023 | 22:25:00 |
| NW-WFL | 50.747291 | -3.54625 | 24/02/2023 | 22:35:00 |
| NW-WHP | 50.744974 | -3.52605 | 24/02/2023 | 22:54:00 |
| NW-RBL | 50.739125 | -3.51749 | 24/02/2023 | 23:19:00 |
| NW-SYA | 50.736187 | -3.51779 | 24/02/2023 | 23:33:00 |
| NW-BEA | 50.737805 | -3.52107 | 24/02/2023 | 23:47:00 |
| NW-OWL | 50.733654 | -3.5261 | 25/02/2023 | 00:04:00 |
| NW-PWE | 50.73263 | -3.53419 | 25/02/2023 | 00:18:00 |
| NW-PWW | 50.732064 | -3.53781 | 25/02/2023 | 00:35:00 |
| NE-MPR | 50.736187 | -3.50939 | 10/03/2023 | 19:58:00 |
| NE-PRR | 50.734732 | -3.50505 | 10/03/2023 | 20:27:00 |
| NE-MLR | 50.739853 | -3.50305 | 10/03/2023 | 20:54:00 |
| NE-SML | 50.738344 | -3.49333 | 10/03/2023 | 21:16:00 |
| NE-CEN | 50.740257 | -3.49223 | 10/03/2023 | 21:32:00 |
| NE-HLW | 50.740742 | -3.4843 | 10/03/2023 | 21:57:00 |
| NE-FIN | 50.740095 | -3.48183 | 10/03/2023 | 22:18:00 |
| NE-HLE | 50.740661 | -3.47565 | 10/03/2023 | 22:38:00 |
| NE-BRI | 50.741281 | -3.47655 | 10/03/2023 | 22:54:00 |
| NE-PCL | 50.740985 | -3.46492 | 10/03/2023 | 23:10:00 |
| SE-CPM | 50.709721 | -3.4768 | 17/03/2023 | 20:38:00 |
| SE-DPR | 50.713252 | -3.47578 | 17/03/2023 | 21:07:00 |
| SE-RDL | 50.715112 | -3.4814 | 17/03/2023 | 21:39:00 |
| SE-KES | 50.716863 | -3.46807 | 17/03/2023 | 22:07:00 |
| SE-HAR | 50.715839 | -3.46884 | 17/03/2023 | 22:33:00 |
| SE-SRV | 50.714896 | -3.46573 | 17/03/2023 | 22:57:00 |
| SE-CTR | 50.715893 | -3.49146 | 17/03/2023 | 23:46:00 |
| SE-PYM | 50.71681 | -3.49627 | 18/03/2023 | 00:04:00 |
| SE-MLT | 50.721176 | -3.50637 | 18/03/2023 | 00:24:00 |
| SE-IKA | 50.708293 | -3.47708 | 23/03/2023 | 20:36:00 |
| SW-CBB | 50.703442 | -3.54616 | 23/03/2023 | 21:26:00 |
| SW-DRW | 50.707754 | -3.56093 | 23/03/2023 | 22:02:00 |
| SW-IDL | 50.70964 | -3.56359 | 23/03/2023 | 22:30:00 |
| SW-POH | 50.710719 | -3.56474 | 23/03/2023 | 22:59:00 |
| SW-NHC | 50.708724 | -3.54442 | 23/03/2023 | 23:22:00 |
| SW-CWH | 50.710503 | -3.55247 | 23/03/2023 | 23:56:00 |
| SW-PNG | 50.711069 | -3.54173 | 24/03/2023 | 00:17:00 |
| SW-QUR | 50.711446 | -3.53909 | 24/03/2023 | 00:28:00 |
| NW-BGV | 50.735999 | -3.54271 | 24/03/2023 | 20:12:00 |
| NW-ELG | 50.734382 | -3.54241 | 24/03/2023 | 20:35:00 |
| NW-TFC | 50.731579 | -3.54139 | 24/03/2023 | 20:49:00 |
| NW-NNN | 50.732953 | -3.54177 | 24/03/2023 | 21:05:00 |
| NW-CLY | 50.737373 | -3.54186 | 24/03/2023 | 21:24:00 |
| NW-NNS | 50.729369 | -3.53866 | 24/03/2023 | 21:43:00 |
| NW-BOB | 50.726781 | -3.53543 | 24/03/2023 | 22:09:00 |
| NW-RMC | 50.725569 | -3.537 | 24/03/2023 | 22:27:00 |
| NW-ACR | 50.723547 | -3.52345 | 24/03/2023 | 22:48:00 |
| NW-DRP | 50.724221 | -3.52056 | 24/03/2023 | 23:05:00 |
| NW-GSR | 50.724868 | -3.51527 | 24/03/2023 | 23:30:00 |
| NW-BBR | 50.729018 | -3.52034 | 24/03/2023 | 23:44:00 |
| NW-ELS | 50.730932 | -3.51579 | 25/03/2023 | 00:05:00 |
| NW-PRR | 50.732549 | -3.5117 | 25/03/2023 | 00:15:00 |
| NW-PHR | 50.730393 | -3.51246 | 25/03/2023 | 00:28:00 |
| NE-AVR | 50.726296 | -3.46483 | 04/04/2023 | 20:33:00 |
| NE-FRR | 50.724814 | -3.47297 | 04/04/2023 | 20:51:00 |
| NE-EPW | 50.727428 | -3.47212 | 04/04/2023 | 21:08:00 |
| NE-AMD | 50.726889 | -3.46982 | 04/04/2023 | 21:31:00 |
| NE-COL | 50.730959 | -3.46913 | 04/04/2023 | 21:55:00 |
| NE-GHL | 50.732306 | -3.46645 | 04/04/2023 | 22:12:00 |
| NE-TBW | 50.734436 | -3.46445 | 04/04/2023 | 22:36:00 |
| NE-LGL | 50.739233 | -3.4633 | 04/04/2023 | 22:55:00 |
| NE-SNS | 50.734274 | -3.48089 | 04/04/2023 | 23:21:00 |
| SE-BWR | 50.705463 | -3.4984 | 06/04/2023 | 20:50:00 |
| SE-SCL | 50.704223 | -3.50053 | 06/04/2023 | 21:05:00 |
| SE-LKL | 50.700693 | -3.50219 | 06/04/2023 | 21:16:00 |
| SE-TSR | 50.705921 | -3.50309 | 06/04/2023 | 21:46:00 |
| SE-EDC | 50.706164 | -3.50487 | 06/04/2023 | 21:56:00 |
| SE-HTR | 50.710665 | -3.50407 | 06/04/2023 | 22:12:00 |
| SE-MSS | 50.713629 | -3.50914 | 06/04/2023 | 22:30:00 |
| SW-TRE | 50.714007 | -3.51864 | 06/04/2023 | 22:54:00 |
| SW-TRW | 50.717079 | -3.52482 | 06/04/2023 | 23:14:00 |
| SW-HSR | 50.719774 | -3.52844 | 06/04/2023 | 23:26:00 |
| SW-IWR | 50.719073 | -3.55634 | 08/05/2023 | 21:51:00 |
| SW-CLA | 50.717025 | -3.55494 | 08/05/2023 | 22:04:00 |
| SW-MVR | 50.717375 | -3.54957 | 08/05/2023 | 22:17:00 |
| SW-COW | 50.714734 | -3.54761 | 08/05/2023 | 22:32:00 |
| SW-CHR | 50.716028 | -3.54075 | 08/05/2023 | 22:47:00 |
| SW-BWS | 50.718912 | -3.54382 | 08/05/2023 | 23:01:00 |
| SW-ABS | 50.719181 | -3.54224 | 08/05/2023 | 23:12:00 |
| SW-LCS | 50.719289 | -3.5321 | 08/05/2023 | 23:26:00 |
| NE-HRL | 50.732145 | -3.4817 | 28/05/2023 | 22:14:00 |
| NE-SUM | 50.735163 | -3.48937 | 28/05/2023 | 22:32:00 |
| NE-KEN | 50.733465 | -3.49444 | 28/05/2023 | 22:44:00 |
| NE-HLG | 50.727455 | -3.50049 | 28/05/2023 | 22:57:00 |
| NE-HNE | 50.726916 | -3.50258 | 28/05/2023 | 23:11:00 |
| NE-HSE | 50.725892 | -3.50309 | 28/05/2023 | 23:25:00 |

Additionally, EV3 measurements were taken at 14 of the same locations.

| location_id | lat | lon | date | time_finished_utc |
| --- | --- | --- | --- | --- |
| NE-PRR | 50.7347 | -3.50505 | 10/03/2023 | 20:35:00 |
| SE-SRV | 50.7149 | -3.46573 | 17/03/2023 | 23:02:00 |
| SE-IKA | 50.7083 | -3.47708 | 23/03/2023 | 20:40:00 |
| SW-CBB | 50.7034 | -3.54616 | 23/03/2023 | 21:33:00 |
| SW-DRW | 50.7078 | -3.56093 | 23/03/2023 | 22:10:00 |
| SW-IDL | 50.7096 | -3.56359 | 23/03/2023 | 22:46:00 |
| NW-BGV | 50.736 | -3.54271 | 24/03/2023 | 20:19:00 |
| NW-BOB | 50.7268 | -3.53543 | 24/03/2023 | 22:18:00 |
| NW-DRP | 50.7242 | -3.52056 | 24/03/2023 | 23:11:00 |
| NW-BBR | 50.729 | -3.52034 | 24/03/2023 | 23:50:00 |
| NE-SNS | 50.7343 | -3.48089 | 04/04/2023 | 23:29:00 |
| SE-LKL | 50.7007 | -3.50219 | 06/04/2023 | 21:21:00 |
| NE-HNE | 50.7269 | -3.50258 | 28/05/2023 | 23:16:00 |
| NE-HSE | 50.7259 | -3.50309 | 28/05/2023 | 23:29:00 |

# Temporal coverage and resolution

12 nights of data collection from 22 February to 28 May 2023. During each night of data collection, photos were taken in roughly 10-15 locations over the course of a few hours. In each location, 15 photos were taken over the course of several minutes. See spatial coverage table above for all dates and times measurements were taken.

# Collection/Generation/Transformation methods

The method for capturing the imagery was intended to capture the lighting environment in each location. Hence, we placed the camera at the same height, completely level with the ground (based on measurements from spirit levels) and facing the north cardinal direction. Via the exposure bracketing function on the camera, we captured high-dynamic range imagery of the scene with a 1EV bracket. For particularly dark or bright locations, where the longest photo was still underexposed or the shortest photo overexposed, we also captured a 3EV bracket version of the sample. We repeated this process for each of the cardinal directions (turning clockwise) before finally aligning the camera with zenith and taking sky imagery. At the end of each location we recorded the image numbers and timings in the log. For each photo, an accompanying JPEG was automatically generated by the camera.

At the end of each night, the data raw data were copied directly from the camera on to our data storage (SharePoint) and backup (Research Data Storage) solutions, along with the log and GPS tracks.

# Experimental design/sampling regime

We designed the experiment to sample the lighting environments across the city, hence we performed random stratified sampling of locations along roadways within the bounds of Exeter. We stratified by the building area ratio and foliage cover, and ensured the points be well spatially distributed by sampling each of the quadrants of the city in a round-robin fashion. Each point was sampled following the procedure detailed in the ‘Data Collection` section.

Some points, such as the points in Ikea’s car park were replicated due to 16/03/2023 being taken in poor weather, and the photos having to be deleted at the request of the Ikea security team (17/03/2023). Some other drawn locations were rejected due to being on private land or otherwise inaccessible – our reasoning for not sampling them has been noted in the ‘Notes’ column of `Draws.csv`.   Others were accepted and sampled exactly one time.

# Fieldwork and/or laboratory instrumentation

| Variable/Column header | Description | Units | Vocabulary URL (if applicable) |
| --- | --- | --- | --- |
| Camera | Canon 6D Mk II 49305300829 | Make/Model& Serial No. | Specifications & Features - Canon EOS 6D Mark II - Canon UK |
| Lens | Sigma EXDG 8mm Fisheye 10198550 | Make/Model & Serial No. | 8mm F3.5 EX DG CIRCULAR FISHEYE ¦ LENSES ¦ PRODUCTS ¦ Sigma |

# Calibration steps and values

No calibration was required for the raw imaging survey data presented in this dataset.

# Quality control

- The procedure for sampling each location was defined ahead of time and repeated verbatim for each of the sampled locations.

- Where adverse fieldwork conditions prematurely stopped fieldwork on a given evening, it has been noted in the corresponding fieldwork log.

- For each location, we ensured checked that the camera was not over or under-exposing. To mitigate this, in areas where this did happen, we performed additional high-dynamic range photography with a larger 3EV bracket, as opposed to the 1EV bracket on the normal camera.

- Drawn locations for which either could not safely access, or were on private land, have been rejected from the final sample, with the reasoning for doing so noted in the `Notes` column of `Draws.csv`.