Hydro-morphodynamic simulations of dam impacts in Andean Rivers of the Amazon Basin

eridynas computes hydro-morphodynamic changes based on physical and empirical relations (see Nicholas, 2025 for more detail). The output files contain the results from applying the eridynas model to idealized Andean foreland rivers. We ran simulations for six 100-km long exploratory models of 1350 years, including a 450-year spin-up (hindcasting) period and 900-year forecasting period for a control scenario without a dam and several dam scenarios. The repository holds the model results of the Andea Foreland River simulations (Amazon River Basin). Outputs for the six river models are provided for: (1) a reference simulation without dam impacts in csv-format including an initial spin-up period (900 output files each with 100 for the spin-up), (2) simulations with upstream dam impacts (1-4 scenarios) in csv-format (800 output files each), (3) three Python postprocessing scripts.

The model output files each represent the hydrodynamic and geomorphic characteristics of the river-floodplain model at specific output intervals. Each output-file spans 4.5 morphological years driven by alternating 4.5-year long high and 4.5-year long low flow event. The hindcast period (spin-up) are the output files with the names 001-100. Here, the output interval is every 4.5 years at the peak of the high flow and the trough of the low flow event (001 is year 4.5 at high flow, 002 is year 9 at low flow, etc.). The output-files with the names 101-900 represent the 900 years of the reference and dam scenarios saved at denser time intervals of 1.125 morphological years.

The structure of the output files is as follows: the first two lines represent the input parameterization of the model simulations. The remaining lines are the output values per grid cell. To run the model, users can download the executable that comes with the publication of Nicholas (2025).

The python files are used to plot the relative bed elevation maps of from the output file (eRiDynaS_plotter_rel_loop.py), to extract the channel and floodplain cells (eRiDynaS_ctype_extract.py) and to extract the entrenchment area, defined as reworked floodplain cells that were transformed into channel and back to floodplain (eRiDynaS_trench_total.py). Those files are meant as example files on how to extract meaningful data from the output files.

A. P. Nicholas, Modelling river and floodplain evolution using physics-based and reduced-complexity approaches. Earth Surface Processes Landforms 50 (2025).