# The degradation, microplastic generation, and effect of mulch films exposed above and below ground for 1 year in a field experiment on soil properties, North Wales, UK

# Summary

This dataset was generated based on a 12-month field experiment exposing 7 types of plastic mulch films to natural weathering (above ground) or soil (below ground). Below ground films were either in fine or coarse mesh bags (filled with 20g soil and 5x5 cm plastic piece) to allow only microfauna or micro-, meso-, and macrofauna interaction. The aim of this experiment was to identify the degradation rates of mulch films under UK weather conditions, their microplastic generation rates, and their effect on soil properties and microbial communities (microbial data has been deposited with NCBI: PRJNA1229197). We recorded the following data:

Plastic:

- Film thickness

- Film surface roughness

- Chemical spectra (FTIR)

Soil:

- Moisture at 5 cm depth

- Temperature at 5 cm depth

- Surface temperature

- Microplastic concentration

- pH and electrical conductivity

- Nutrients (ammonium, nitrate, phosphate, DOC)

Environmental data:

- Air temperature

- Precipitation

- Wind speed

- Solar radiation

# File format

All files are in .csv format.

# File names and/or naming convention

| File name | File content |
| --- | --- |
| SoilMoistureTemperature.csv | Soil moisture and temperature at 5 cm depth |
| SoilSurfaceTemp.csv | Soil surface temperature |
| SoilProperties.csv | Soil properties (pH, EC, ammonium, nitrate, phosphate, DOC) |
| Microplastic.csv | Number of polymer particles in the size range of 20-500 µm |
| PlasticProperties.csv | Plastic mulch film thickness and surface roughness pre and post exposure |
| FTIRspectra.csv | FTIR spectra of mulch films |
| Weather.csv | Environmental data including air temperature, precipitation, wind speed, and solar radiation |

# Nature and units of recorded values

Table 1. Description of variables in SoilMoistureTemperature.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Date | Date of sampling | Dd/mm/yyyy |
| Time | Time of sampling | Hh:mm:ss |
| Moisture | Soil moisture at 5 cm depth | % |
| Temperature | Soil temperature at 5 cm depth | °C |

Table 2. Description of variables in SoilSurfaceTemp.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Date | Date of sampling | Dd/mm/yyyy |
| Time | Time of sampling | Hh:mm:ss |
| Treatment | Refers to sensor being placed on soil surface either on “Control” (bare soil, no mulch film) or “LDPE” (sensor under LDPE mulch film) plots | n/a |
| Temperature | Soil surface temperature | °C |

Table 3. Description of variables in SoilProperties.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Date | Sampling interval (months) | n/a |
| Mesh | Refers to either fine or coarse mesh bags | n/a |
| Treatment | Refers to plastic mulch film type: Control (no mulch film), Synthetic, Biodegradable, Bio Ctrl A, Bio A1, Bio A2, Bio Ctrl B, Bio B1, Bio B2 | n/a |
| pH | Soil pH | n/a |
| EC | Soil electrical conductivity | µS/cm |
| Ammonium | Soil available ammonium-N concentration | Mg/g |
| Nitrate | Soil available nitrate-N concentration | Mg/g |
| Phosphate | Soil available phosphate-P concentration | Mg/g |
| DOC | Soil dissolved organic carbon concentration | Mg/g |

Table 4. Description of variables in Microplastic.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Method | Refers to the extraction method used for either conventional polymers (PE) or biodegradable polymers (Bio) | n/a |
| Treatment | Refers to plastic mulch film type: Control (no mulch film), Synthetic, Biodegradable, Bio Ctrl A, Bio A1, Bio A2, Bio Ctrl B, Bio B1, Bio B2; and mesh bag (fine or coarse) | n/a |
| Particles | Number of polymer particles | Particles / 10mg |

Table 5. Description of variables in PlasticProperties.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Type | Refers to plastic mulch film type: Control (no mulch film), Synthetic, Biodegradable, Bio A, Bio A1, Bio A2, Bio B, Bio B1, Bio B2 | n/a |
| Method | Refers to exposure method: Control (new films), UV (above ground exposure 12 months), Fine (soil burial 12 months in fine mesh bag), coarse (soil burial 12 months in coarse mesh bag) | n/a |
| Roughness | Surface roughness of mulch films | µm |
| Thickness | Film thickness | µm |

Table 6. Description of variables in FTIRspectra.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Type | Refers to plastic mulch film type: Control (no mulch film), Synthetic, Biodegradable, Bio A, Bio A1, Bio A2, Bio B, Bio B1, Bio B2 | n/a |
| Method | Refers to exposure method: Control (new films), UV (above ground), Fine (soil burial in fine mesh bag), coarse (soil burial in coarse mesh bag) | n/a |
| Time | Refers to exposure time (4, 8, 12 months) and control (no exposure) | n/a |
| 656-3992 | Column headers are the wavenumber intervals (cm-1), and values for each sample are expressed in absorbance (0-1) | n/a |

Table 7. Description of variables in Weather.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Date | Date of sampling | Dd/mm/yyyy |
| Time | Time of sampling | Hh:mm |
| Air_temperature | Air temperature | °C |
| Precipitation | Precipitation | Mm |
| Solar_radiation | Solar radiation | W/m2 |
| Wind_speed | Wind speed | Km/h |

# Spatial coverage

The field experiment was performed at the Henfaes Research Centre, Abergwyngregyn, North Wales, UK.

# Temporal coverage and resolution

The experiment started on 29/10/2021 and ended on 26/10/2022.

Plastic:

- Film thickness – measured once after 12 months exposure at the end of the experiment

- Film surface roughness – measured once after 12 months exposure at the end of the experiment

- Chemical spectra (FTIR) – measured every 4 months for a duration of 12 months

Soil:

- Moisture at 5 cm depth – recorded continuously in 30 min intervals

- Temperature at 5 cm depth – recorded continuously in 30 min intervals

- Surface temperature – recorded continuously in 1h intervals

- Microplastic concentration – measured once at the end of the experiment

- pH and electrical conductivity – measured every 4 months for a duration of 12 months

- Nutrients (ammonium, nitrate, phosphate, DOC) - measured every 4 months for a duration of 12 months

Environmental data:

- Air temperature – measured continuously in 1h intervals

- Precipitation– measured continuously in 1h intervals

- Wind speed– measured continuously in 1h intervals

- Solar radiation– measured continuously in 1h intervals

# Collection/Generation/Transformation methods

Plastic:

- Film thickness: Measured with a thickness gauge. Data recorded in paper notebook, transferred to Excel file, then exported as csv file.

- Film surface roughness: Measured with a surface roughness tester. Data recorded in paper notebook, transferred to Excel file, then exported as csv file.

- Chemical spectra (FTIR): Data was recorded using Cary 630 (Agilent) ATR-FTIR, exported as individual spectra csv files. Data was then combined for each plastic type.

Soil:

- Moisture at 5 cm depth: Data was recorded using TDT-12 Acclima sensors buried at 10 cm soil depth. Data was recorded on datasnaps, exported as Excel file, then exported as csv file.

- Temperature at 5 cm depth: Data was recorded using TDT-12 Acclima sensors buried at 10 cm soil depth. Data was recorded on datasnaps, exported as Excel file, then exported as csv file.

- Surface temperature: Data was recorded using iButtons placed on the soil surface. Data was exported as csv file.

- Microplastic concentration: Samples were analysed using 8700 LDIR chemical imaging. Data was exported as Excel files, then exported as csv files.

- pH and electrical conductivity: Soil was analysed in a 1:2.5 soil:distilled water solution using standard electrodes. Data recorded in paper notebook, then transferred to Excel file. Excel file exported as csv file.

- Nutrients (ammonium, nitrate, phosphate, DOC): Soil was analysed in a 1:5 soil:0.5 M K2SO4 solution using colorimetric analysis for ammonium and nitrate, and a TOC analyser for DOC. Data was exported as Excel files from respective instruments, then exported as csv file.

Environmental data:

- Air temperature: Data was recorded by weather station on site, exported as Excel file, then exported as csv file.

- Precipitation: Data was recorded by weather station on site, exported as Excel file, then exported as csv file.

- Wind speed: Data was recorded by weather station on site, exported as Excel file, then exported as csv file.

- Solar radiation: Data was recorded by weather station on site, exported as Excel file, then exported as csv file.

# Experimental design/sampling regime

The study was undertaken in an arable field located at the Henfaes Research Centre, Abergwyngregyn, North Wales, UK (53◦14’19.4’’N, 4◦00’54’’W). The experimental area has a temperate oceanic climate, no known history of agricultural plastic application, and was previously under a rotation of grass (Lolium perenne L.) and wheat (Triticum aestivum). The soil is classed as a freely draining sandy clay loam textured Eutric Cambisol. Environmental data was recorded in 30-minute intervals by a meteorological station on site. Soil surface temperature was recorded hourly using iButtons (DS1922L Thermochron Data Logger, Measurement Systems Ltd., UK) placed on bare soil (n = 3) and under the LDPE PMF surface (n = 3) to determine the average soil surface temperature. Soil moisture and temperature at 5 cm depth were recorded in 30-minute intervals using TDT sensors (n = 12) connected to a data logger (SDI-12, Acclima, USA).

To assess the above ground degradation of PMFs, films were cut into 0.25 ×4 m strips and stretched taut over rotavated soil seed beds in a randomised block design (n =4 per film type), secured by burying 5 cm of the edges in the surrounding soil, and weighed down with soil across the film surface in 1 m intervals to reduce mechanical stress from wind. Films were laid out slightly dome-shaped to allow water run-off and reduce splashback from precipitation. All films were sampled monthly over the course of 12 months, by removing a 10 ×10 cm piece of exposed film from the end of each row and burying the newly created end with soil to reduce the likelihood of mechanical damage introduced by the sampling, subsequently resulting in shortened rows of PMF throughout the experiment. Samples were cleaned with ultrapure water and stored in paper bags.

To assess the below ground degradation of PMFs in soil and their potential impact on soil biological and chemical properties, PMFs were cut into 5 ×5 cm pieces (n =4 per film type) and horizontally placed into 10 ×10 cm nylon mesh bags, surrounded by two layers of soil (ca. 5 mm on each side of the PMF piece). The soil was previously collected from the study site, air dried, then brought to a moisture content of 44 %, and weighed out to 20 ± 0.1 g per mesh bag. Control treatments contained only soil. Mesh bags were sealed using a heat gun and stored in aluminium foil at 4 ◦C overnight before burying them in the field, adjacent to the area designated for the above ground degradation.

Two different nylon mesh sizes were used to assess the impact of micro-, meso-, and macrofauna on the degradation process.  The fine mesh (100 µm mesh size, 32 % opening) allowed for only microfauna to enter the bags, whereas the coarse mesh (5000 µm mesh size, 72 % opening) allowed micro-, meso-, and macrofauna to interact with the contents of the bags, from hereon referred to as fine and coarse mesh, respectively. Mesh was sourced from Normesh Ltd. (UK), and nylon was selected as an inert material with low likelihood of affecting surrounding soil properties, which would be a risk if using metal. None of the PMF materials used in this study contained nylon, and the control mesh bags allowed for a creation of a baseline. Mesh bags were buried horizontally in the soil at 5 cm depth in a randomised block design for fine and coarse mesh bags respectively. Mesh bags were sampled in 4-month intervals over a 12-month period by excavating them from the soil. The PMF pieces were removed and cleaned using ultrapure water, subsequently stored in paper bags.

For sampling frequency refer to section “Temporal coverage and resolution”.

# Fieldwork and/or laboratory instrumentation

Plastic:

- Film thickness: Thickness gauge, Mututoyo, Japan

- Film surface roughness: Surftest SJ-210, Mututoyo, Japan

- Chemical spectra (FTIR): Cary 630 ATR-FTIR, Agilent

Soil:

- Moisture at 5 cm depth: TDT-12 soil moisture and temperature sensors, Acclima

- Temperature at 5 cm depth: TDT-12 soil moisture and temperature sensors, Acclima

- Surface temperature: iButtons (DS1922L Thermochron DataLogger, Measurement Systems, UK)

- Microplastic concentration: Microplastic was extracted according to Ren et al (2025) and Li et al (2023), and analysed using chemical imaging 8700 LDIR, Agilent.

- pH and electrical conductivity: Jenway 3510 pH meter and Jenway 4520 conductivity meter

- Nutrients (ammonium, nitrate, phosphate): BioTek Powerwave XS, Agilent

- Nutrient (DOC): Multi N/C Series TOC/TN analyser, Analytik-Jena

Environmental data:

- Air temperature: Weather station on site

- Precipitation: Weather station on site

- Wind speed: Weather station on site

- Solar radiation: Weather station on site

# Calibration steps and values

All analyses have been carried out alongside regular blanks and internal standards applicable for each instrument.

# Quality control

Data was curated and missing values were replaced with n/a.

# Miscellaneous

Published paper: https://doi.org/10.1016/j.jhazmat.2025.139871

NCBI accession number: PRJNA1229197

# References

Ren, S., Graf, M., Wang, K., Zhang, J., Zhang, H., Liu, X., et al., 2025. Separation and identification of conventional microplastics from farmland soils. J Vis Exp 217, e67064. https://doi.org/10.3791/67064.

Li, S., Ding, F., Flury, M., Wang, J., 2023. Dynamics of macroplastics and microplastics formed by biodegradable mulch film in an agricultural field. Sci Total Environ 894, 164674. https://doi.org/10.1016/j.scitotenv.2023.164674.