Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP)

ERAMMP Supporting Information Metadata for:

Headwater stream quality metrics from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023

Client Ref: Welsh Government / Contract C210/2016/2017

Version 1.0

Date: 20/01/2026

Version History

| Version | By | Date | Changes |
| --- | --- | --- | --- |
| 1.0 | AD & JRL | 20/01/2026 | published |

Document Info

| Series | Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP) |
| --- | --- |
| Title | ERAMMP Supporting Information Metadata for: Headwater stream quality metrics from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023 |
| Client | Welsh Government |
| Client reference | C208/2021/2022 |
| Confidentiality, copyright and reproduction | © Crown Copyright 2026 This document is licensed under the Open Government Licence 3.0. |
| UKCEH contact details | Bronwen Williams UK Centre for Ecology & Hydrology (UKCEH) Environment Centre Wales, Deiniol Road, Bangor, Gwynedd, LL57 2UW 01248 374500 erammp@ceh.ac.uk |
| Corresponding author | Anna Doeser, UKCEH anndoe@ceh.ac.uk |
| Authors | Doeser, A.1, Scarlett, P.1, Bentley, L.1, Emmett, B.A.1, Lucas, J.R.1, Monkman, G.1, Reinsch, S.1, Richardson-Jones, V.1, Williams, B.1, Wood, C.M.1 |
| Contributing authors & reviewers | 1UK Centre for Ecology & Hydrology (UKCEH), 2Welsh Government |
| How to cite (long) | Doeser, A., Scarlett, P., Bentley, L., Emmett, B.A., Lucas, J.R., Monkman, G., Reinsch, S., Richardson-Jones, V., Williams, B., Wood, C.M. (2026). ERAMMP Supporting Information Metadata for: Headwater stream quality metrics from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023 (Report-116). UKCEH |
| How to cite (short) | Doeser, A. et al. (2026). ERAMMP Supporting Information Metadata for: Headwater stream quality metrics from the Environment and Rural Affairs Monitoring & Modelling Programme, Wales, 2021–2023 (Report-116). UKCEH |
| Approved by | Bridget Emmett (UKCEH) Oliver Massey (Welsh Government) |

Abbreviations

| AES | agri-environment schemes |
| --- | --- |
| ASPT | Average Score Per Taxon |
| AWIC | Acid Water Indicator Community |
| BMWP | Biological Monitoring Working Party |
| BNG | British National Grid |
| CCI | Community conservation index |
| CS | Countryside Survey |
| DAM | Diatom acidification score |
| DEHLI | Drought Effect of Habitat Loss on Invertebrates |
| ERAMMP | Environment and Rural Affairs Monitoring & Modelling Programme |
| EU | European Union |
| GB | Great Britain |
| GMEP | Glastir Monitoring and Evaluation Programme |
| HMS | habitat modification score |
| HQA | Habitat Quality Assessment |
| ITE | Institute of Terrestrial Ecology |
| LIFE | Lotic-invertebrate Index for Flow Evaluation |
| LCM | Land cover map (followed by the year) |
| LMS | land management scheme |
| NFS | National Field Survey |
| PSI | Proportion of Sediment-sensitive Invertebrates |
| RHS | River Habitat Survey |
| TDI | Trophic Diatom Index |
| TDN | Total Dissolved Nitrogen |
| TRS | Trophic Ranking Score |
| WFD | Water Framework Directive |
| WHPT | Walley, Hawkes, Paisley & Trigg [score] |

Contents

1.	Overview	5

2.	Background Information	5

3.	Introduction to the GMEP Survey Design and its re-survey by ERAMMP	6

4.	Field Data Collection Methods	7

5.	Analytical Methods	8

5.1	Diatoms	8

5.2	Invertebrates	8

5.3	Water chemistry	9

5.4	River Waterbody Catchment	10

6.	Data Quality Assurance	10

7.	Notes	10

8.	Data Structure	11

8.1	File details:	11

8.2	Table columns:	14

8.3	Relationship between tables:	41

9.	References	42

# Overview

| Spatial Extent | Terrestrial extent of Wales, UK |
| --- | --- |
| Temporal Extent | 2021–2023 |
| Summary | This data set consists of headwater quality data for sites across Wales between 2021 and 2023), collected as part of the Environment and Rural Affairs Monitoring and Modelling Programme (ERAMMP) National Field Survey. Data include water chemistry and ecological quality metrics, River Habitat Survey data (including information on substrates, flow types, aquatic vegetation, channel and bank materials, modifications and natural features, riparian land use, artificial features, spot check records, tree cover and associated features and Habitat Modification and Habitat Quality indices), macro-invertebrate taxonomic and abundance data, diatom taxonomic and abundance data, headwater quality indices derived from both the invertebrate and diatom communities and associated environmental data. Headwater streams were surveyed within a set of 101 x 1km squares across Wales, collected as part of the Environment and Rural Affairs Monitoring and Modelling Programme (ERAMMP) National Field Survey. ERAMMP follows an earlier monitoring programme, the Glastir Monitoring and Evaluation Programme (GMEP), which was set up by the Welsh Government in 2013 to monitor the effects of the Glastir agri-environment scheme on the environment and ran from 2013 to 2016. The field survey element for both GMEP and ERAMMP was based on a stratified random sampling design of 300 x 1km square sites across Wales (225 of which were resurveyed for ERAMMP in 2021-23, though not all squares contained headwater streams). It was managed by the UK Centre for Ecology & Hydrology. |
| File Name(s) | ERAMMP_STREAM_DIATOM_ENV_2021_23.csv ERAMMP_STREAM_DIATOMS_2021_23.csv ERAMMP_STREAM_DIATOM_INDICES_2021_23.csv ERAMMP_STREAM_INVERT_ENV_2021_23.csv ERAMMP_STREAM_INVERTS_2021_23.csv ERAMMP_STREAM_INVERT_INDICES_2021_23.csv ERAMMP_STREAM_WATER_CHEMISTRY_2021_23.csv ERAMMP_STREAM_RHS_MAIN_2021_23.csv ERAMMP_STREAM_RHS_BANK_2021_23.csv ERAMMP_STREAM_RHS_SPOTCHECK_2021_23.csv ERAMMP_STREAM_RHS_MODIFICATION_SCORES_2021_23.csv ERAMMP_STREAM_RHS_QUALITY_SCORES_2021_23.csv GMEP_HEADWATERS_BY_WFD.csv |
| File Format(s) | 13 x .csv |

# Background Information

In Wales, funding from agri-environment schemes (AES) has been available since the early 1990s including Environmentally Sensitive Areas, the Habitat Scheme, the Woodland Grant Scheme, the Organic Farming Scheme, Tir Cymen, Tir Cynnal, Tir Gofal, followed by the Glastir scheme. A monitoring programme was set up by the Welsh Government in 2013 to monitor the national state and change of the countryside, along with effects of the Glastir scheme on the environment. This four-year rolling monitoring programme was called the “Glastir Monitoring and Evaluation Programme” (GMEP) and was conducted between 2013 and 2016. GMEP aimed to: (1) Evaluate the environmental effects of the Glastir sustainable land management scheme (LMS) (https://gov.wales/glastir); (2) Quantify environmental status and spatiotemporal environmental status trends across the Welsh countryside and; (3) Assess and predict outcomes for climate change mitigation, biodiversity, soil and water quality, woodland expansion and cultural landscapes (https://gmep.wales). The project was led by UKCEH. A major component of the work to deliver the first 2 aims was a National Field Survey (NFS) which was an expansion of a long-term integrated monitoring programme developed by UKCEH since the 1970s called Countryside Survey (Carey et al., 2008, Wood et al., 2018, Robinson et al., 2024).

Synthesis and analysis of the data derived from the NFS sought to identify how the Welsh environment is being impacted by drivers of change, such as land use, climate, and pollution, over and above Glastir interventions. The GMEP report was published in 2017 (Emmett et al. 2017) and data are summarized in Wood et al. 2021. Data are available from the Environmental Information Data Centre (https://eidc.ac.uk/) as part of the data series of the Glastir Monitoring and Evaluation Programme, Wales, 2013-2016.

The ’Environment and Rural Affairs Monitoring & Modelling Programme’ (ERAMMP) National Field Survey is the re-survey of GMEP locations. Data informs national trends analyses, and the effects of Glastir action bundles. Other elements of GMEP and ERAMMP including numerous expert reviews and integrated modelling is reported elsewhere (https://erammp.wales/).

# Introduction to the GMEP Survey Design and its re-survey by ERAMMP

In the Glastir Monitoring and Evaluation Programme, the NFS was designed to capture multiple measures and metrics, and to integrate across these metrics. Hence a full ecosystem-based approach is required where data operating at multiple scales are captured, where possible, at the same time in a single snap-shot visit.

For GMEP, a 4-year rolling survey was adopted to maximise the number of sites visited across the national spatial scale whilst at the same time monitoring year-on-year, such that spatial variation and temporal changes and trends can be detected cost-effectively (Emmett et al., 2013 & Wood et al. 2021). The first survey cycle dates from 2013-2016. The first partial re-survey cycle took place between 2021 and 2023, after being postponed from 2020 due to the COVID pandemic.

The rolling programme-based survey has two key components: the Wider Wales Component used for baseline estimation (Welsh national trends) and national reporting of Glastir action bundles (Targeted Component) which specifically links to the 9 objectives (of which 6 environmental outcomes are intended to be captured in the NFS) of the Glastir scheme. Across both components, integration of survey data is essential and therefore a common spatial unit of 1 km x 1 km square was adopted.

The selection of the sample of 1-km x 1-km squares for monitoring was accomplished following the same procedure as used for the Countryside Survey (CS) of Great Britain (GB) (as described below), which aims to provide robust estimates of indicators at national level across GB and constituent countries. Using the same methodology builds on previous knowledge and enabled integration of the two surveys in future analyses.

A total of 300 x 1 km2 squares were sampled over the 4-year GMEP NFS cycle. Half of these squares (n=150) were the Wider Wales squares, randomly sampled within strata defined according to the Land Classification of Great Britain (Bunce et al., 2007) – a derived classification of the landscape based on its topography, geology, climate and physical attributes. Environmental heterogeneity is minimized within each stratum of the Land Classification and is maximised between strata. The proportion of the 1-km x 1-km squares randomly sampled from within each stratum was proportional to the size of the stratum to allocate survey efforts best. The other half of the sampled squares were targeted specifically at Glastir priority areas (n=150). Glastir-targeted squares were randomly selected with a selection probability proportional to the ‘Glastir score’ of the square, derived from the Welsh Government’s scoring system of the Glastir Advanced target objective areas (Emmett et al., 2014). Any square randomly selected that contained more than 75% of urban land or that was more than 90% sea (defined by LCM2007 and the UK Census mean high tide data) was excluded.

The ERAMMP NFS re-survey covered 130 of the original 300 squares in 2021-22, and an additional 95 survey squares in 2023. The 130 squares in 2021-22 were selected from both the Wider Wales and Glastir-targeted squares to enable the evaluation of multiple Glastir interventions based on multiple environmental outcomes; the selection of these squares does not enable a robust evaluation of national trends without more Wider Wales squares being surveyed (Alison et al. 2021). Therefore, the 95 survey squares in 2023 were the remaining Wider Wales squares that had not been surveyed. A total of 225 squares were re-surveyed in ERAMMP between 2021 and 2023: 147 Wider Wales squares and 78 Glastir-targeted squares (Figure 1). Not all surveyed squares contained surveyable headwater streams. Table 1 contains an overview of squares containing headwater streams which were surveyed between 2021-23.

Table 1: Overview of squares containing headwater streams surveyed between 2021 and 2023, divided by square type and survey year.

|  | 2021–22 | 2023 | Total |
| --- | --- | --- | --- |
| Wider Wales squares | 28 | 38 | 66 |
| Glastir-targeted squares | 35 | 0 | 35 |
| total | 63 | 38 | 101 |

| GMEP (2013-2016) | ERAMMP (2021-2023) |
| --- | --- |

Figure 1: Maps showing the distribution of 1-km x 1-km National Field Survey squares across Wales (plotted at 10 km to ensure data confidentiality). ERAMMP map shows all NFS squares surveyed in 2021-2023; the colour key indicates survey years for the botanical survey, headwater streams were visited in the same years.

# Field Data Collection Methods

Data were collected according to standardised field methods. Please see the Field-Survey Handbook (Scarlett et al., 2021), included as supplementary information, for full details.

# Analytical Methods

## Diatoms

Sample analysis

Samples were digested using hydrogen peroxide to remove organic matter and finally mounted on slides using the mountant Naphrax.  At least 300 valves on each slide were identified to the highest resolution possible using a Nikon BX40 microscope with 100x oil immersion objective with phase contrast. The primary floras and identification guides used were Lange-Bertalot et al. (2017) and Jüttner et al. (2026). Samples were analysed by Bowburn Consultancy, Durham (http://www.bowburn-consultancy.co.uk/).  Diatom counts are presented as total valve count per taxa observed on the prepared slide.

Calculation of Diatom metrics

Water quality indices of number of scoring taxa (NTAXA), trophic diatom index (TDI4), diatom acidification (DAM), and % motile were derived using the methods and references provided in the data table descriptions.  The indices were derived from diatom species data and water chemistry data collected in the field. Environmental properties of the site should be taken into consideration when interpreting indices.

## Invertebrates

Samples were analysed by APEM Ltd (https://www.apemltd.co.uk/) using the following methods:

Sorting

Preservative and silt are washed from the samples in a fume cupboard by rinsing repeatedly with tap water The samples are split into smaller portions to enable thorough washing. A series of sieves, reducing in coarseness, can also be used.

The samples are finally sieved through a 500µm sieve (mandatory). Samples with a large quantity of filamentous algae or plant material are floated in a bucket of water so the trapped invertebrates fall to the bottom.

Small portions of the samples are placed into a water filled sorting tray, marked with a grid to act as an aid, and systematically scanned for invertebrates. Large pieces of substrate, sticks etc are removed and the invertebrates are then removed using forceps or tweezers. Examples of all taxa are placed into vials for Quality Assurance. Specimens may be identified straight away or separated into different taxon groups for more detailed investigation. Large clumps of vegetation are broken down to ensure that no specimens are overlooked.

After the first sort, the sample is then disturbed and/or rotated to expose previously hidden taxa. The process is repeated until all necessary taxa have been removed from the sample.

Identification

Macro invertebrates removed from the sample are identified to species level where possible, including Caddis and Diptera pupae but with the exception of Oligochaeta, Chironomidae, Simuliidae and Hydracarina. The majority of identification work is conducted using dissection microscopes with high power microscopes used for examination of small specimens or specific parts of specimens.

Terrestrial and aerial stages of aquatic species, terrestrial species and samples which were dead when collected are not counted. Invertebrates which have become fragmented are only counted as a record if the thorax and abdomen are present. If only the posterior, abdomen or head are present the species is not recorded.

Abundance

Though the abundances of taxa are not necessary for the Biological Monitoring Working Party (BMWP) scoring system, they are recorded for use in other indices and environmental diagnostics. Only free-living individuals are counted. Colonies are counted as one individual.

Very abundant taxa are recorded by distributing the sample evenly, counting the specimens in a portion of the tray using the grid lines and calculating the total by proportions.

Quality Control for Invertebrate Sorting

Quality control is carried out by the re-analysis of one randomly selected sample per twenty by a different analyst.

Calculation of invertebrate metrics

Indices calculated include Biological Monitoring Working Party score (BMWP), Lotic-invertebrate Index for Flow Evaluation (LIFE) indices, community conservation index (CCI scores), the fraction of Oligochaetes or Chironomidea, the fraction of Ephemeroptera (mayflies), Plecoptera (stoneflies), and Tricoptera (caddisflies), Proportion of Sediment-sensitive Invertebrates (PSI) and Walley, Hawkes, Paisley & Trigg (WHPT) scored. All indices were calculated based on the observed community using the methods and references provided in the data table.

## Water chemistry

Bottled water samples were sent to the laboratories at the UK Centre for Ecology and Hydrology, Lancaster and were analysed according to accredited methods as below:

Phosphate (PO4-P)

PO₄-P concentrations were measured colorimetrically using a Seal Analytical AQ2 discrete analyser. [PO₄-P] was determined by reaction with acidic molybdate in the presence of antimony to form an antimony-phospho-molybdate complex. Ascorbic acid reduces this to the intensely blue phosphomolybdenum complex, measured spectrophotometrically at 880nm.

Calibration is produced by automatic dilution of a single stock solution of 0.2 mg/l PO₄-P; concentrations are obtained using the calibration curve within the range 0 – 0.2mg/l. Control standards of 0.1 mg/l PO₄-P are analysed every 10 samples.

Total dissolved nitrogen (TDN)

A Skalar Formacs CA16 analyser with an attached ND25 is used to measure Total dissolved nitrogen (TDN) in water samples.

TDN is measured by combustion at 900°C with a cobalt chromium catalyst which converts all nitrogen to nitric oxide.  The nitric oxide is measure by a chemiluminescent reaction with ozone.

The calibration range of the instrument is 0-4 mg/L for nitrogen.  Samples with values over these are diluted within range using 18.2MΩ carbon free water.

Alkalinity

Alkalinity was determined by the UKAS accredited laboratories at the UK Centre for Ecology & Hydrology (UKCEH) Lancaster using a standard operating procedure for alkalinity in waters.  Alkalinity is determined using the Mettler Toledo DL53 titrator which performs analyses automatically using predefined methods.  A complete titration method comprises sample dilution, dispensing of acid, stirring and waiting times, the actual titration, the calculation of results and a report.

Reagents and material used include: standard buffer solutions from Fisher Scientific of pH 4 and pH 7, electrode filling solution made with potassium chloride solution (4M) saturated with silver chloride from Fisher Scientific, 1.0M Hydrochloric acid, 0.02M Hydrochloric acid, 1000 mg/l stock solution as calcium carbonate, 20 mg/l (400 ue/l) QC standard as calcium carbonate, deionised water.

Alkalinity (usually) reflects the activity of calcium carbonate so results are reported as milligrams per litre of calcium carbonate (mg/l CaCO3).

## River Waterbody Catchment

Headwater streams were spatially located within Water Framework Directive (WFD) River Waterbody Catchments using the intersection between sampling points and the Water Framework Directive (WFD) River Waterbody Catchments Cycle 3 spatial layer (Natural Resources Wales, 2023).

# Data Quality Assurance

All data used in these analyses have been subject to quality assurance. This includes:

- Manual inspection and verification of reported survey locations against historic reports and photographs.

- Inspection of all raw data and derived variables against known ecological limits and historic reporting

- Confirmation of data continuity and integrity across survey strands

- Independent quality assurance of taxonomic ID carried out by subcontractors.

- Inspection of taxonomic data for errors and synonyms

In addition, surveyor training and procedures are reviewed on an annual basis. Reporting methods and methods used in the derivation of all reporting variables are reviewed for each reporting cycle. All changes made to the data are recorded to maintain a robust audit trail.

The laboratories at the UK Centre for Ecology & Hydrology (UKCEH) Lancaster are UKAS (United Kingdom Accreditation Service, https://www.ukas.com/) accredited.

# Notes

- To assess the national trend, only data originating from the Wider Wales square data population should be analysed as these are nationally representative.

- To assess Glastir impacts, data from Wider Wales and Glastir targeted squares should be analysed together and in conjunction with associated data on Glastir management, as both sets of squares contain sites with Glastir management option uptake and no Glastir uptake. However, information on the locations of Glastir uptake is not included in this dataset and must be requested from Welsh Government (data@gov.wales).

# Data Structure

## File details:

Survey data recorded from headwater streams in 101 x 1km squares between 2021 and 2023 using standardised recording methods (Scarlett et al., 2021). Dataset consists of 13 .csv files:

Diatom data

- ERAMMP_STREAM_DIATOM_ENV_2021_23.csv, table containing environmental data associated with the diatom survey for each headwater stream visited between 2021 and 2023 as part of the ERAMMP survey for which contributing data could be collected. Environmental data was collected using the DARES application and consists of the required attributes to predict the values of water quality indices based on the diatom community under pristine water quality conditions. This includes information on channel shading, channel habitat type, water clarity, bed stability, spate occurrence, the occurrence of photosynthetic taxa and dominant photosynthetic taxa. The table contains 49 columns of 86 rows.

- ERAMMP_STREAM_DIATOMS_2021_23.csv, table containing taxonomic and abundance information for the diatom communities for each headwater stream visited between 2021 and 2023 as part of the ERAMMP survey for which a diatom sample could be collected. The table contains 10 columns of 2552 rows.

- ERAMMP_STREAM_DIATOM_INDICES_2021_23.csv, table containing water quality indices calculated from the diatom community and water chemistry data for each headwater stream visited between 2021 and 2023 as part of the ERAMMP survey for which contributing data could be collected. Indices calculated include % Motility, Trophic Diatom Index (TDI) and diatom acidification score (DAM). In the case of DAM and TDI, we provided the index as calculated based on the observed community, the expected index if the relevant headwater was in pristine condition, and the observed index as a proportion of the expected score. Where relevant, a citation has been provided detailing the method used to calculate each index. Overall headwater status based on an assessment of diatom-derived water quality indices is also provided, as either Bad, Poor, Moderate or Good. The table contains 31 columns of 86 rows.

Macro-invertebrate data

- ERAMMP_STREAM_INVERT_ENV_2021_23.csv, table containing the environmental data associated with the macro-invertebrate survey for each headwater stream visited between 2021 and 2023 as part of the ERAMMP survey for which contributing data could be collected. Environmental data was collected using the RIVPACS application and consists of the required attributes to predict the values of water quality indices based on the macro-invertebrate community under pristine water quality conditions. This includes information on channel shape, water velocity, sediment type, water pH, conductivity and other channel characteristics. For associated field methods, please see Scarlett et al., 2021. The table contains 28 columns of 91 rows.

- ERAMMP_STREAM_INVERTS_2021_23.csv, table containing taxonomic and abundance information for the macro-invertebrate communities for each headwater stream visited between 2021 and 2023 as part of the ERAMMP survey for which a macro-invertebrate community sample could be collected. The table contains 8 columns of 3851 rows.

- ERAMMP_STREAM_INVERT_INDICES_2021_23.csv, table containing stream quality indices calculated from the macro-invertebrate community for each headwater stream visited between 2021 and 2023 as part of the ERAMMP survey for which contributing data could be collected. Indices calculated include Biological Monitoring Working Party score (BMWP), Lotic-invertebrate Index for Flow Evaluation (LIFE) indices,  community conservation index (CCI scores), the fraction of Oligochaetes or Chironomidea, the fraction of Ephemeroptera (mayflies), Plecoptera (stoneflies), and Tricoptera (caddisflies), Proportion of Sediment-sensitive Invertebrates (PSI) and Walley, Hawkes, Paisley & Trigg (WHPT) scored. All indices are as calculated based on the observed community. Where relevant, a citation has been provided detailing the method used to calculate each index and to aid subsequent interpretation. The table contains 29 columns of 90 rows.

Water Chemistry

- ERAMMP_STREAM_WATER_CHEMISTRY_2021_23.csv, table containing the water chemistry data for each headwater stream visited between 2021 and 2023 as part of the ERAMMP survey for which contributing data could be collected. Collected data include water pH, gran alkalinity, non-purgeable organic carbon and calcium concentration. The table contains 12 columns of 86 rows.

River Habitat Survey

- ERAMMP_STREAM_RHS_MAIN_2021_23.csv, table containing the main part of the sweep-data from the river habitat survey (RHS) survey for each headwater stream visited between 2021 and 2023 as part of the ERAMMP survey for which contributing data could be collected. RHS is a method designed to characterise and assess, in broad terms, the physical structure of freshwater streams and rivers. RHS is carried out along a standard 500m length of river channel. The sweep-up data represents an assessment of the features along the whole 500m length of the survey. In this table we record channel and bank features, artificial features, trees and associated features, channel dimensions, features of special interest, nuisance species, and overall characteristics including major impacts (e.g. dams, drought etc), evidence of recent management and animals. In all cases, the left and right bank are defined when facing downstream. For associated field methods, and further detail on the definitions of variables in this dataset please see Environment Agency (2022) and Scarlett et al. (2021). The table contains site details, and sections A, B, C, D, J, K, L, M, N, O, P and Q of the River Habitat Survey Form (Environment Agency, 2022). The table contains 157 columns of 99 rows.

- ERAMMP_STREAM_RHS_BANK_2021_23.csv, table containing additional sweep-up data from the River Habitat Survey (RHS) survey for each headwater stream visited between 2021 and 2023 as part of the ERAMMP survey for which contributing data could be collected. In this table we record land uses and bank profiles. In all cases, the left and right bank are defined when facing downstream. For associated field methods, and further detail on the definitions of variables in this dataset please see Environment Agency (2022) and Scarlett et al. (2021). The table contains site details, and sections H and I of the River Habitat Survey Form (Environment Agency, 2022). The table contains 41 columns of 198 rows.

- ERAMMP_STREAM_RHS_SPOTCHECK_2021_23.csv, table containing the River Habitat Survey (RHS) spot-check data for each headwater stream visited between 2021 and 2023 as part of the ERAMMP survey for which contributing data could be collected. RHS is carried out along a standard 500m length of river channel. Each headwater site was assessed at ten spot-check sites along a 500m length (ie approximately 50m apart) where possible. Each spot check site considers the predominant channel, bank and river corridor features in a 10m wide transect across the river.  Specifically, at each spot-check site information is collected on bank material, channel substrate, bank features, flow type, channel modifications, land use and vegetation structure. In all cases, the left and right bank are defined when facing downstream.  For associated field methods, please see Environment Agency (2022) and Scarlett et al. (2021). The table contains 48 columns of 1089 rows.

- ERAMMP_STREAM_RHS_MODIFICATION_SCORES_2021_23.csv, table containing the habitat modification score (HMS) and habitat modification class for each headwater stream visited between 2021 and 2023 as part of the ERAMMP survey for which contributing data could be collected. The sub-scores that comprise the final HMS score are also provided. These scores are derived from data collected as part of the River Habitat Survey (RHS).  Where relevant, a citation has been provided detailing the method used to calculate each index and to aid subsequent interpretation. For associated field methods, please see Environment Agency (2022) and Scarlett et al. (2021). The table contains 18 columns of 99 rows.

- ERAMMP_STREAM_RHS_QUALITY_SCORES_2021_23.csv, table containing the Habitat Quality Assessment scores (HQA) for each headwater stream visited between 2021 and 2023 as part of the ERAMMP survey for which contributing data could be collected. The sub-scores that comprise the final HQA score are also provided. These scores are derived from data collected as part of the River Habitat Survey (RHS).  Where relevant, a citation has been provided detailing the method used to calculate each index and to aid subsequent interpretation. For associated field methods, please see Environment Agency (2022) and Scarlett et al. (2021). The table contains 17 columns of 99 rows.

River Waterbody Catchments

- GMEP_HEADWATERS_BY_WFD.csv, table containing the WFD River Waterbody Catchments which headwater streams fall within. The table includes all headwater streams surveyed in the GMEP field survey. In cases where the headwater stream is located outside the major catchments in a coastal area, no catchment is assigned. The table contains 3 columns of 169 rows.

## Table columns:

Diatom data

In all tables, null values in fields indicate no data for this sample.

ERAMMP_STREAM_DIATOM_ENV_2021_23:

| Colum Name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Text | None | 1km survey square – unique identifier for headwater stream |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |
| SUBSTRATUM | Number | None | Substrate or material from which the sample was taken. Cobbles/boulders = 1, emergent macrophyte = 2, submerged macrophyte = 3, manmade structure = 4 |
| NO_ALGAE_COBBLES | Number | None | Number of cobbles smothered with algae collected at the sample site |
| MACROPHYTE_TAXON | Text | None | Macrophyte taxon from which diatom sample was taken. Only recorded where SUBSTRATUM was 2 or 3. Sites where recording macrophyte taxa was not applicable are NA. |
| SAMPLE_DATE | Date | None | Date of sample collection |
| SAMPLE_TIME | Text | hh:mm:ss | Time of sample collection |
| SHADE_LB_NONE | Number | % | Percentage of channel at the survey site not affected by shading from the left bank and associated structures, estimated for when the sun is directly overhead (i.e. 12 noon). When facing downstream, the left bank in on the left-hand side. |
| SHADE_LB_BROKEN | Number | % | Percentage of channel at the survey site with broken shade from the left bank and associated structures, where some direct sunlight hits the water surface in the shade-affected area when the sun is directly overhead (i.e. 12 noon). When facing downstream, the left bank in on the left-hand side. |
| SHADE_LB_DENSE | Number | % | Percentage of channel at the survey site with dense shade from the left bank and associated structures, where 5% or less of the shade-affected area receives direct sunlight when the sun is directly overhead (i.e. 12 noon). When facing downstream, the left bank in on the left-hand side. |
| SHADE_RB_NONE | Number | % | Percentage of channel at the survey site not affected by shading from the right bank and associated structures, estimated for when the sun is directly overhead (i.e. 12 noon). When facing downstream, the right bank in on the right-hand side. |
| SHADE_RB_BROKEN | Number | % | Percentage of channel at the survey site with broken shade from the right bank and associated structures, where some direct sunlight hits the water surface in the shade-affected area when the sun is directly overhead (i.e. 12 noon). When facing downstream, the right bank in on the right-hand side. |
| SHADE_RB_DENSE | Number | % | Percentage of channel at the survey site with dense shade from the right bank and associated structures, where 5% or less of the shade-affected area receives direct sunlight when the sun is directly overhead (i.e. 12 noon). When facing downstream, the right bank in on the right-hand side. |
| HABITAT_GLIDE | Number | % | Percentage of channel at survey site area recorded as glide habitat - smooth flow |
| HABITAT_POOL | Number | % | Percentage of channel at survey site area recorded as pool habitat - A distinct natural feature of deeper water |
| HABITAT_RIFFLE | Number | % | Percentage of channel at survey site area recorded as riffle habitat - Shallow, fast-flowing water with a distinctly disturbed surface |
| HABITAT_RUN | Number | % | Percentage of channel at survey site area recorded as run habitat - Fast-moving water with rippled surface |
| HABITAT_TORRENT | Number | % | Percentage of channel at survey site area recorded as torrent habitat |
| HABITAT_CASCADE | Number | % | Percentage of channel at survey site area recorded as cascade habitat - Chute flow occurring over boulder substrates or bedrock outcrops |
| HABITAT_RAPID | Number | % | Percentage of channel at survey site area recorded as rapid habitat - White-water with broken standing waves |
| HABITAT_BEDROCK | Number | % | Percentage of channel at survey site area recorded as exposed bedrock habitat |
| HABITAT_MAT_ISLAND | Number | % | Percentage of channel at survey site area recorded as mature island habitat -Permanent mid-channel feature with surface at or above bankfull height. |
| HABITAT_TRICKLE | Number | % | Percentage of channel at survey site area recorded as trickle habitat |
| WATER_CLRTY_CLEAR | Number | % | Percentage of water clarity recorded as clear - where channel substrate is clearly visible at all depths, as are macrophyte species. |
| WATER_CLRTY_SLIGHT | Number | % | Percentage of water clarity recorded as slightly turbid, following surveyor judgement |
| WATER_CLRTY_MODERATE | Number | % | Percentage of water clarity recorded as moderately turbid, following surveyor judgement |
| WATER_CLRTY_HIGH | Number | % | Percentage of water clarity recorded as highly turbid, following surveyor judgement |
| BED_STBL_SOLID | Number | % | The percentage of the river bed where bed stability is Solid/firmly bedded - e.g. bedrock/compacted clay, increased flow has little effect |
| BED_STBL_STABLE | Number | % | The percentage of the river bed where bed stability is Stable - e.g. boulders/pebbles/gravel, unlikely to be significantly altered by increased flows |
| BED_STBL_UNSTABLE | Number | % | The percentage of the river bed where bed stability is Unstable – e.g. gravel/sand/silt/mud, likely to be dislodged by increased flows |
| BED_STBL_SOFT | Number | % | The percentage of the river bed where bed stability is Soft/sinking, - e.g. deep silt/mud, makes channel unwadeable, bank stick penetrates easily into substrate. |
| SPATE_3_DAYS | Number | None | Whether time since last spate flow is estimated as < 3 days. YES = -1, NO = 0 |
| SPATE_3_7_DAYS | Number | None | Whether time since last spate flow is estimated as 3-7 days. YES = -1, NO = 0 |
| SPATE_7_14_DAYS | Number | None | Whether time since last spate flow is estimated as > 7-14 days. YES = -1, NO = 0 |
| SPATE_14_DAYS | Number | None | Whether time since last spate flow is estimated as > 14 days. YES = -1, NO = 0 |
| SPATE_UNKNOWN | Number | None | Whether time since last spate flow is estimated as unknown. YES = -1, NO = 0 |
| O_PHOTOSYN_FIL_ALG | Number | % | Percentage of sample site covered by filamentous algae |
| O_PHOTOSYN_O_ALG | Number | % | Percentage of sample site covered by other algal species |
| O_PHOTOSYN_BRYO | Number | % | Percentage of sample site covered by bryophyte species |
| O_PHOTOSYN_HPLANTS | Number | % | Percentage of sample site covered by higher plant species |
| DOM_TAX_FIL_ALG | Text | None | Dominant filamentous algae, where present. Where present, but taxa could not be identified "Unknown" is recorded. Where record is not applicable no data was recorded. |
| DOM_TAX_O_ALG | Text | None | Dominant non filamentous algae, where present. Where present, but taxa could not be identified "Unknown" is recorded. Where record is not applicable no data was recorded. |
| DOM_TAX_BRYO | Text | None | Dominant bryophyte taxa, where present. Where present, but taxa could not be identified "Unknown" is recorded. Where record is not applicable no data was recorded. |
| DOM_TAX_HPLANTS | Text | None | Dominant higher plant taxa, where present. Where present, but taxa could not be identified "Unknown" is recorded. Where record is not applicable no data was recorded. |
| COMMENTS | Text | None | Comments made by the field surveyor about data collection |
| SURVEY_YEAR | Text | None | Year of survey |

ERAMMP_STREAM_DIATOMS_2021_23:

| Colum Name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Text | None | 1km survey square – unique identifier for headwater stream |
| GENUS | Text | None | Genus of diatom |
| SPECIES | Text | None | Species of diatom |
| INFRASPECIES_TAXON | Text | None | Infra-species taxon of diatom, where relevant |
| COUNT | Number | None | Count of this taxon observed in the sample. |
| SURVEY_YEAR | Number | None | Year that sample was collected from the stream |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |

ERAMMP_STREAM_DIATOM_INDICES_2021_23:

| Colum Name | Type | Unit | Description |
| --- | --- | --- | --- |
| SAMPLE | Number | None | Unique identifier for diatom sample |
| SQ_ID | Text | None | 1km survey square – unique identifier for headwater stream |
| SAMPLE_DATE | Date | None | Date of diatom sample collection. |
| NTAXA | Number | None | The number of scoring taxa in the derivation of the TDI score, which is indicative of organic pollution. Method for the derivation of the TDI score is given in Kelly, M. G. & Whitton, B. A. (1995). Missing data is null. |
| TDI4 | Number | None | Observed TDI (Trophic diatom index) is used in the calculation of EQR_TDI and STATUS_TDI which are indicative of organic pollution. For the full method see Kelly & Whitton (1995), Kelly et al., (2014), and WFD-UKTAG (2014). Missing data is null. |
| DAM | Number | None | Observed DAM (diatom acidification metric) is used in the calculation of EQR_DAM and STATUS_DAM. It is a score derived from the diatom community indicative of acidification. For the full method used in deriving the score, see Juggins, S. & Kelly, M. (2012) Development of a Water Framework Directive-compatible metric for assessing acidification in UK and Irish rivers. Report SC070034/TR2. Environment Agency, Bristol. Missing data is null. |
| MOTILE | Number | % | The percentage of motile taxa in the diatom sample, which is typically thought to be indicative of the level of siltation. See Juggins & Kelly (2012). Missing data is null. |
| PH | Number | pH unit | pH of headwater water sample. Missing data is null. |
| GRAN_ALKALINITY_UEQ_L | Number | µeq per L | Gran alkalinity of headwater water sample. Missing data is null. |
| ALKALINITY_AS_MG_L | Number | mg per L | Alkalinity of headwater water sample in milligrams per litre (derived from gran alkalinity). Missing data is null. |
| CONDUCTIVITY_US_CM | Number | µS per cm | Conductivity of headwater water sample. Missing data is null. |
| CA_FILTERED_MG_L | Number | mg per L | Calcium concentration of headwater water sample measured in milligrams per litre. Missing data is null. |
| CA_F_MEQ_L | Number | meq per L | Calcium concentration of headwater water sample measured in milliequivalents per litre (derived from CA_FILTERED_MG_L). Missing data is null. |
| ESTIMATED_CA_MEQ_L | Number | meq per L | Calcium in milliequivalents per litre derived from conductivity of the water sample. This was how calcium content was estimated in GMEP. It is used in the estimation of expected DAM for EDAM_GMEP_METHOD only, for the purpose of direct comparison with GMEP results. Where ESTIMATED_CA produced a negative value, a value of 20 meq/L was assumed for subsequent calculations (EDAM_GMEP_METHOD). Missing data is null. |
| NPOC_MG_L | Number | mg per L | Non-purgeable organic carbon in headwater sample. Missing data is null. |
| ETDI | Number | None | Expected Trophic diatom index for the headwater stream under pristine conditions, derived from environmental data collected at the site using based on the DARLEQ software TDI version 4 as described in Kelly, M. G. & Whitton, B. A. (1995) Trophic diatom index - a new index for monitoring eutrophication in rivers Journal of Applied Phycology, 7, 433-444. DARLEQ: Diatom Assessment of River and Lake Ecological Quality, version 2.0 User guide, and River Phytobenthos UKTAG method statement. Dec 2014. ISBN - 978-1-906934-58-3. Missing data is null. |
| EQR_TDI | Number | None | Ecological Quality Ratio for the trophic diatom index. This is calculated as EQR_TDI = (100- TDI4) / (100 - eTDI). Missing data is null. |
| STATUS_TDI | Text | None | Classification of water quality as High, Good, Moderate, Poor or Bad based on EQR_TDI. Thresholds for the categories are: High/Good = 0.80, Good/Moderate = 0.60, Moderate/Poor = 0.40, Poor/Bad = 0.20. If the calculation of EQR_TDI failed due to unusual water chemistry data or gave a spurious value as an output, expert opinion was used to infer a probable status based on available data and this is presented in brackets, e.g. (High). Missing data is null. |
| EDAM | Number | None | Expected Diatom acidification metric for the headwater stream under pristine conditions, derived from environmental data collected at the site. This calculation used measured Ca values. Missing data is null. |
| EQR_DAM | Number | None | Ecological Quality Ratio for the diatom acidification metric. This is calculated as EQR_DAM = DAM/eDAM. Missing data is null. |
| STATUS_DAM | Text | None | Classification of water quality as High, Good, Moderate, Poor or Bad based on EQR_DAM. Thresholds for the categories are: High/Good = 0.81, Good/Moderate = 0.65, Moderate/Poor = 0.44, Poor/Bad = 0.22. If the calculation of EQR_DAM failed due to unusual water chemistry data or gave a spurious value as an output, expert opinion was used to infer a probable status based on available data and this is presented in brackets, e.g. (High). Missing data is null. |
| FINAL_STATUS | Text | None | Overall headwater status following diatom assessment, equal to the lowest status as determined by EQR_TDI or EQR_DAM. Missing data is null. |
| EDAM_GMEP_METHOD | Number | None | Expected Diatom acidification metric for the headwater stream under pristine conditions, derived from environmental data collected at the site. This calculation used estimated Ca values. Where ESTIMATED_CA was a negative value, a value of 20 meq/L was assumed. Missing data is null. |
| EQR_DAM_GMEP_METHOD | Number | None | Ecological Quality Ratio for the diatom acidification metric. This is calcluated as EQR_DAM = DAM/eDAM_GMEP_METHOD. Missing data is null. |
| STATUS_DAM_GMEP_METHOD | Text | None | Classification of water quality as High, Good, Moderate, Poor or Bad based on EQR_DAM_GMEP_METHOD. Thresholds for the categories are: High/Good = 0.81, Good/Moderate = 0.65, Moderate/Poor = 0.44, Poor/Bad = 0.22. If the calcuation of EQR_DAM_GMEP_METHOD failed due to unusual water chemistry data or gave a spurious value as an output, expert opinion was used to infer a probable status based on available data and this is presented in brackets, e.g. (High). Missing data is null. |
| NOTES | Text | None | Comments relating to the analysis of the diatom community and the calculation of NTAXA, MOTILE, TDI4 and DAM, and any anomalies noted in subsequent calculations. |
| SURVEY_YEAR | Number | None | Year of survey |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |

Macro-invertebrate data

In all tables, null values in fields indicate no data for this sample.

ERAMMP_STREAM_INVERT_ENV_2021_23

| Colum Name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Text | None | 1km survey square – unique identifier for headwater stream |
| SAMPLE_DATE | Date | None | Date of sample collection |
| SAMPLE_TIME | Text | hh:mm:ss | Time of sample collection in h:mm:ss |
| METHOD | Number | None | Sampling method used. Kick/Sweep = 1, Kick/Sweep and Search = 6, 15 second Kick/Sweep = 18. |
| SAMPLE_PROP_HABITATS | Number | None | Whether sampling was proportional to habitat occurrence, where YES = -1 and NO = 0. |
| SPH_COMMENT | Text | None | If sampling was not proportional to habitat occurrence, comments by the surveyor on why the sampling was not proportional to habitats are recorded here |
| PHOTO_TAKEN | Number | None | Whether a photo was taken of the site in the field, where YES = -1 and NO = 0. |
| WATER_WIDTH_M | Number | metres (m) | Width of water in the sampling area |
| DEPTH1_CM | Number | cm | Water depth at the sampling point at 1/4 of the way across the width of the water. |
| DEPTH2_CM | Number | cm | Water depth at the sampling point at 1/2 of the way across the width of the water. |
| DEPTH3_CM | Number | cm | Water depth at the sampling point at 3/4 of the way across the width of the water. |
| SURF_VEL_CAT | Number | None | Estimated surface velocity in the main headwater channel. <=10 cm/s = 1, >10-25 cm/s = 2, >50-100 cm/s = 3, >100 cm/s = 4 |
| BOULDER_COBBLE | Number | % | Substrate at the sampling site - percentage of the water width covered by boulders or cobbles (longest axis >64mm). |
| PEBBLE | Number | % | Substrate at the sampling site - percentage of the water width covered by pebbles (longest axis >2-64 mm) . |
| SAND | Number | % | Substrate at the sampling site - percentage of the water width covered by sand (longest axis 0.0625 - 2mm). |
| SILT_CLAY | Number | % | Substrate at the sampling site - percentage of the water width covered by silt or clay (longest axis 0.004 - 0.0625mm). |
| ROCK_PAVEMT | Number | % | Percentage of the width of the water at the sampling site with rock pavement. |
| MOSS | Number | % | Percentage of the width of the water at the sampling site with moss. |
| FILAM_ALGAE | Number | % | Percentage of the width of the water at the sampling site with filamentous algae. |
| HIGHER_PLANTS | Number | % | Percentage of the width of the water at the sampling site with higher plants. |
| PH | Number | None | Water sample pH |
| CONDUCTIVITY_US_CM | Number | µS per cm | Water sample conductivity |
| STREET_LIGHT | Number | None | Whether street lights are present within 20m of the sample site where YES = 1 and NO = 0 |
| SURVEY_YEAR | Number | None | Year of survey |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |

ERAMMP_STREAM_INVERTS_2021_23

| Colum Name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Text | None | 1km survey square – unique identifier for headwater stream |
| TAXA | Text | None | Binomial species name for macro-invertebrate taxa |
| COUNT | Number | None | Number individuals of this taxon observed in the sample. |
| SURVEY_YEAR | Number | None | Year that sample was collected from the stream |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |

ERAMMP_STREAM_INVERT_INDICES_2021_23

| Colum Name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Text | None | 1km survey square – unique identifier for headwater stream |
| SAMPLE_DATE | Date | None | Date of sample collection. |
| TOTAL_NUMBER_OF_TAXA_IN_SAMPLE | Number | None | Number of distinct macro-invertebrate taxa in sample |
| TOTAL_ABUNDANCE_IN_SAMPLE | Number | None | Number of individuals across all taxa in sample |
| BMWP_SCORE | Number | None | This is the Biological Monitoring Working Party (BMWP) score observed for the headwater stream. BMWP score is indicative of organic pollution where higher values indicate low rates of organic pollution. (Armitage et al., 1983). |
| ASPT | Number | None | Average Score Per Taxon (ASPT) for Biological Monitoring Working Party (BMWP) observed in the sample, which is calculated as ASPT = BMWP_SCORE / N_TAXA. ASPT is indicative of organic pollution where higher values indicate low rates of organic pollution. (Armitage et al., 1983). |
| BMWP_N_TAXA | Number | None | The number of taxa present (N_TAXA) in the sample for which can be scored for BMWP. These are taxa which can be scored for BMWP, and whose presence and abundance is indicative of the level of organic pollution. This number is, by definition, less than or equal to TOTAL_NUMBER_OF_TAXA_IN_SAMPLE. (Armitage et al., 1983). |
| LIFE_SCORE_FAMILY | Number | None | This is the observed Lotic-invertebrate Index for Flow Evaluation (LIFE) score, derived from taxonomic information at the family level and recognised associations with flow. This score is indicative of the level of flow degradation in lotic systems, where a higher score indicated a faster flowing system. (Extence et al., 1999) |
| LIFE_SCORE_SPECIES | Number | None | This is the observed Lotic-invertebrate Index for Flow Evaluation (LIFE) score, derived from taxonomic information at the species level and recognised associations with flow. This score is indicative of the level of flow degradation in lotic systems, where a higher score indicated a faster flowing system. (Extence et al., 1999) |
| LIFE_GROUPS_1_2 | Number | None | This is the count of the number of individuals present in Lotic-invertebrate Index for Flow Evaluation (LIFE) groups 1 (Rapid) or 2 (Moderate/fast) which are groups of species typically associated with flow conditions of >100 cm per s and 20-100 cm per s respectively. Taxa in these groups are associated with higher LIFE scores. (Extence et al., 1999) |
| LIFE_GROUPS_3_6 | Number | None | This is the count of the number of individuals present in Lotic-invertebrate Index for Flow Evaluation (LIFE) groups 3 (Slow/sluggish), 4 (Flowing / standing), 5 (Standing) or 6 (Drought resistant) which are groups of species typically associated with flow conditions of <20 cm per s where there is flow, or no flow where there is standing water. Taxa in these groups are associated with lower LIFE scores. (Extence et al., 1999) |
| CCI_SCORE | Number | None | This is the observed community conservation index (CCI), based on the Conservation Score of taxa present in the sample. A high score is indicative of high conservation value (which may be driven by the presence of high species richness, or the presence of endangered or nationally rare species). CCI is calculated from mean taxon conservation scores for the sample, multiplied by the community score, which is the highest conservation score present in the sample. (Chadd & Extence, 2004) |
| F_OLIGO_CHIRO | Number | % | Percentage of the total observed taxa which are Oligochaetes or Chironomidea. Oligochaetes and Chironomidea are generally an indicator of organic pollution and low water quality. A high F_OLIGO_CHIRO is indicative of low water quality and low water pollution. |
| F_EPT | Number | % | Percentage of total observed taxa which are Ephemeroptera (mayflies), Plecoptera (stoneflies), and Tricoptera (caddisflies) - taxa which are highly sensitive to water pollution. A high F_EPT is indicative of high water quality and low water pollution. |
| PSI_SPECIES_SCORE | Number | None | Observed Proportion of Sediment-sensitive Invertebrates (PSI) score derived using species level classifications, which provides a proxy to describe the extent to which the surface of river beds are composed of, or covered by, fine sediments. Scores range from 0 to 100 where 0 represents highly sedimented and 100 represents minimally sedimented conditions. (Extence et al., 2013) |
| SPECIES_PSI_INTERPRETATION | Text | None | Observed Proportion of Sediment-sensitive Invertebrates (PSI) classifications derived using species level PSI scores, describing the extent to which the surface of river beds are composed of, or covered by, fine sediments. Classifications range from Heavily sedimented (0-20), Sedimented (21-40), Moderately Sedimented (41-60), Slightly sedimented (61-80) to minimally sedimented/unsedimented (81-100). (Extence et al., 2013) |
| PSI_FAMILY_SCORE | Number | None | Observed Proportion of Sediment-sensitive Invertebrates (PSI) score derived using family level classifications, which provides a proxy to describe the extent to which the surface of river beds are composed of, or covered by, fine sediments. Scores range from 0 to 100 where 0 represents highly sedimented and 100 represents minimally sedimented conditions. (Extence et al., 2013) |
| FAMILY_PSI_INTERPRETATION | Text | None | Observed Proportion of Sediment-sensitive Invertebrates (PSI) classifications derived using family level PSI scores, describing the extent to which the surface of river beds are composed of, or covered by, fine sediments. Classifications range from Heavily sedimented (0-20), Sedimented (21-40), Moderately Sedimented (41-60), Slightly sedimented (61-80) to minimally sedimented/unsedimented (81-100). (Extence et al., 2013) |
| WHPT_ASPT | Number | None | Observed scores for the Walley, Hawkes, Paisley & Trigg (WHPT) Average score per taxon (ASPT) is an indicator of general degradation (including organic pollution) of river habitats. A high score is generally indicative of less degradation. (WFD-UKTAG, 2021) |
| WHPT_N_TAXA | Number | None | Observed scores for the Walley, Hawkes, Paisley & Trigg (WHPT) number of scoring taxa (N_TAXA) is an indicator of general degradation (including organic pollution) of river habitats. A high score is generally indicative of less degradation. (WFD-UKTAG, 2021) |
| DEHLI | Number | None | Drought Effect of Habitat Loss on Invertebrates (observed score for each site), which quantifies the effects of drought on instream macroinvertebrate communities. This is calculated by assigning weights to taxa on the basis of their likelihood of associating with key stages of channel drying. (See Chadd et al., 2017) |
| TOTAL_ABUNDANCE_AWIC | Number | None | Total abundance of species present in the sample that may be scored for AWIC (Acid Water Indicator Community) (WFD-UKTAG, 2013). Missing data, e.g. where no scoring indicator taxa were present, is null. |
| AWIC_N_TAXA | Number | None | Number of species present in the sample that may be scored for AWIC (Acid Water Indicator Community) (WFD-UKTAG, 2013). Missing data, e.g. where no scoring indicator taxa were present, is null. |
| AWIC_SCORE | Number | None | Average score per taxon for AWIC index (WFD-UKTAG, 2013). Missing data, e.g. where no scoring indicator taxa were present, is null. |
| SURVEY_YEAR | Number | None | Year that sample was collected from the stream |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |

Water Chemistry

In all tables, null values in fields indicate no data for this sample.

ERAMMP_STREAM_WATER_CHEMISTRY_2021_23:

| Colum Name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Text | None | 1km survey square – unique identifier for headwater stream |
| PH | Number | pH unit | pH of water sample. |
| GRAN_ALKALINITY_MICROEQ_L | Number | meq per L | Gran alkalinity of water sample. |
| CONDUCTIVITY_US_CM | Number | µS per cm | Conductivity of water sample at 25 degrees Celcius |
| NPOC_MGL_1 | Number | mg per L | Non-purgeable organic carbon of water sample. |
| CA_FORMATTED_RESULT_FILTERED_WATER_PPM | Number | parts per million (ppm) | Calcium concentration of water sample. |
| COMMENTS | Text | None | Comments on freshwater chemistry. |
| SURVEY_YEAR | Number | None | Year of survey. |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |

River Habitat Survey

In all tables, null values in fields indicate no data for this sample.

ERAMMP_STREAM_RHS_MAIN_2021_23:

| Colum Name | Type | Unit | Description | RHS Section |
| --- | --- | --- | --- | --- |
| SQ_ID | Text | None | 1km survey square – unique identifier for headwater stream | Field survey details |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |  |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |  |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |  |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |  |
| SURVEY_YEAR | Number | None | Year of survey |  |
| SAMPLE_DATE | Date | None | Survey date |  |
| PART_OF_RIV_OR_ART | Text | None | Is the site part of a river or an artificial channel, where R = River and A = Artificial channel. |  |
| ADV_COND | Text | None | Are adverse conditions affecting the survey? YES = Y, NO = N. Details of adverse conditions encountered are recorded in ADV_CON. |  |
| ADV_CON | Text | None | Description of adverse conditions affecting the survey, where relevant. Fields are null where no adverse conditions were recorded. |  |
| BED_VIS | Text | None | Whether the river bed is visible, where: 0 = barely or partially obscured (0-33% visible) P = Partially obscured (33-95% visible) E = Entirely visible (>95% visible). |  |
| SURVFROM | Text | None | Surveyed from left bank (L) and/or right bank (R) and/or channel (C). Where multiple letters are present (e.g. LRC) the survey was conducted from all respective vantage points. null = Missing Value C = Channel only L = Left bank only LC = Left bank and channel only LR = Left bank and right bank only LRC = Left bank, right bank and channel R = Right bank only RC = Right bank and channel only |  |
| SPOTCHCK | Text | None | Whether spotcheck 1 is at the upstream or downstream extreme of the survey area, where U = Upstream and D = Downstream. |  |
| VALLEY_FORM | Number | None | Valley form referring to the shape of the whole valley landform within which the survey site is situated, where: 1: Shallow vee - Overall valley side slopes <30° from floor to top (horizon). 2: Deep vee - Overall valley side slopes >30° – 80° from floor to top (horizon). 3: Gorge - Overall valley side slopes >30° – 80° from floor to top (horizon). 4: Concave/bowl - Gently curving slopes that do not have a distinct glaciated U-shape. 5: Asymmetrical valley - Valley sides are different, shallow on one side and steep on the other. 6: No obvious valley sides - No obvious valley sides in near- to mid-horizon. 7: U shaped valley - Steep valley sides rising from a flat valley floor that characterises a glacial valley. null: Missing value | B. Predominant valley form |
| VALLEY_TERR | Text | None | Whether natural terraces (features on a river valley floor forming steps or breaks in slope produced as the river erodes downwards) are present, where Y = YES and N = NO |  |
| VALLEY_FLAT | Text | None | Whether there is a distinct flat valley bottom present into which water would spill during floods, where Y = YES and N = NO |  |
| NO_RIFFLES | Number | None | Number of riffles present. For a full definition of riffle habitat, see Environment Agency (2022). | C. Number of riffles, pools and point bars |
| NO_POOLS | Number | None | Number of pools present. For a full definition of riffle habitat, see Environment Agency (2022). |  |
| NO_UNVEG_PBARS | Number | None | Number of unvegetated point bars present. For a full definition of riffle habitat, see Environment Agency (2022). |  |
| NO_VEG_PBARS | Number | None | Number of vegetated point bars present. For a full definition of riffle habitat, see Environment Agency (2022). |  |
| W_LC_S_NO_DS | Number | None | Spot check number upstream of weir/dam, where null = no data | D. Number of artificial features |
| W_LC_S_NO_US | Number | None | Spot check number downstream of weir/dam, where null = no data |  |
| WEIRSMAJOR | Number | None | Number of major weirs |  |
| WEIRSMINOR | Number | None | Number of minor weirs |  |
| WEIRSLUICESMAJ | Number | None | Number of major sluices |  |
| WEIRSLUICESINT | Number | None | Number of intermediate sluices |  |
| WEIRSLUICESMIN | Number | None | Number of minor sluices |  |
| WEIR_HT | Number | Meters | Weir height (cm), where null = no data |  |
| CULVERTS | Number | None | Number of culverts |  |
| BRIDGEMAJ | Number | None | Number of major bridges |  |
| BRIDGEINT | Number | None | Number of intermediate bridges |  |
| BRIDGEMIN | Number | None | Number of minor bridges |  |
| OUTINT_MAJ | Number | None | Number of major outfalls |  |
| OUTINT_INT | Number | None | Number of intermediate outfalls |  |
| OUTINT_MIN | Number | None | Number of minor outfalls |  |
| FORDS_MAJ | Number | None | Number of major fords |  |
| FORDS_INT | Number | None | Number of intermediate fords |  |
| FORDS_MIN | Number | None | Number of minor fords |  |
| DEFLECT_MAJ | Number | None | Number of major deflectors |  |
| DEFLECT_INT | Number | None | Number of intermediate deflectors |  |
| DEFLECT_MIN | Number | None | Number of minor deflectors |  |
| REVETMAJ | Number | None | Number of major revetments |  |
| REVETINT | Number | None | Number of intermediate revetments |  |
| REVETMIN | Number | None | Number of minor revetments |  |
| OTHERMAJ | Number | None | Number of other major structures >=25 of bank length. |  |
| OTHERINT | Number | None | Number of other intermediate structures 10-25m of bank length |  |
| OTHERMIN | Number | None | Number of other minor structures <10m of bank length |  |
| OTH_STATE | Text | None | Free text column for description of other features |  |
| CH_REALIGN | Text | None | Percentage of channel obviously realigned, where percentages are reported as one of >75%, 75-51%, 50-33%, 33-26%, 25-11%, 10-1% or 0%. These thresholds were chosen to be compatible with more widely used RHS reporting thresholds (No,Yes <33% of site,Yes >=33%), whilst providing more contextual information. |  |
| CH_OVER_DEEP | Text | None | Percentage of channel obviously over-deepened, where percentages are reported as one of >75%, 75-51%, 50-33%, 33-26%, 25-11%, 10-1% or 0%. These thresholds were chosen to be compatible with more widely used RHS reporting thresholds (No,Yes <33% of site,Yes >=33%), whilst providing more contextual information. |  |
| WAT_IMP_DAM | Text | None | Percentage of channel water impounded by dam, where percentages are reported as one of >75%, 75-51%, 50-33%, 33-26%, 25-11%, 10-1% or 0%. These thresholds were chosen to be compatible with more widely used RHS reporting thresholds, whilst providing more contextual information. |  |
| ARTIF_NO | Number | None | Artificial features present? -1 = YES, 0 = NO. |  |
| TREES_L | Number | None | Extent of Trees (left bank) where: 0 = None 1= Isolated/scattered 2= Regularly spaced, single 3= Occasional clumps 4= Semi-continuous 5=Continuous | J. Extent of trees and associated features |
| TREES_R | Number | None | Extent of Trees (right bank) where: 0 = None 1= Isolated/scattered 2= Regularly spaced, single 3= Occasional clumps 4= Semi-continuous 5=Continuous |  |
| SH_CHANNEL | Text | None | Extent of features relating to tree extent: Shading of Channel, where 0=None, P=Present, E=Extensive >=33%, null = missing value |  |
| OVERHANG | Text | None | Extent of features relating to tree extent: Overhanging Boughs, where 0=None, P=Present, E=Extensive >=33%, null = missing value |  |
| EX_B_ROOTS | Text | None | Extent of features relating to tree extent: Exposed bankside roots, where 0=None, P=Present, E=Extensive >=33%, null = missing value |  |
| UW_T_ROOTS | Text | None | Extent of features relating to tree extent: Underwater Tree Roots, where 0=None, P=Present, E=Extensive >=33%, null = missing value |  |
| FALLEN_TREES | Text | None | Extent of features relating to tree extent: Fallen Trees, where 0=None, P=Present, E=Extensive >=33%, null = missing value |  |
| C_WOODY_DEB | Text | None | Extent of features relating to tree extent: Coarse Woody Debris, where 0=None, P=Present, E=Extensive >=33%, null = missing value |  |
| WATERFALL1 | Text | None | Extent of channel and bank features: Free fall flow (waterfall) which separates from rock (Feature of bedrock channels), where 0=None, P=Present, E=Extensive >=33%, null=missing value. | K. Extent of channel and bank features |
| CASCADES | Text | None | Extent of channel and bank features: cascades - Chute flow occurring over boulder substrates or bedrock outcrops, where 0=None, P=Present, E=Extensive >=33%, null=missing value. |  |
| RAPIDS | Text | None | Extent of channel and bank features: rapids - White-water with broken standing waves, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| RIFFLES | Text | None | Extent of channel and bank features: riffles - Shallow, fast-flowing water with a distinctly disturbed surface, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| RUNS | Text | None | Extent of channel and bank features: runs - Fast-moving water with rippled surface, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| BOILS | Text | None | Extent of channel and bank features: boils - upwelling flow, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| GLIDES | Text | None | Extent of channel and bank features: glides - Smooth flow, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| POOLS | Text | None | Extent of channel and bank features: pools - A distinct natural feature of deeper water, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| NOFLOW | Text | None | Extent of channel and bank features: No flow (dry), where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| MARGINAL_DEADWATER | Text | None | Extent of channel and bank features: Marginal deadwater, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| ERODING_CLIFF | Text | None | Extent of channel and bank features: Eroding cliff (s), where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| STABLE_CLIFF | Text | None | Extent of channel and bank features: Stable cliff (s), where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| EXPOSED_BEDROCK | Text | None | Extent of channel and bank features: Bedrock outcropping/protruding above water level, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| EXPOSED_BOULDER | Text | None | Extent of channel and bank features: Boulders outcropping/protruding above water level, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| VEG_BED_BOULDER | Text | None | Extent of channel and bank features: Vegetated bedrock/boulders, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| UNVEG_MCB | Text | None | Extent of channel and bank features: Unvegetated mid-channel bars - A distinctive depositional feature composed of unconsolidated material, where 0=None, P=Present, E=Extensive >=33%, - null=missing value |  |
| VEG_MCB | Text | None | Extent of channel and bank features: Vegetated mid-channel bars - A distinctive depositional feature, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| MATURE_ISLAND | Text | None | Extent of channel and bank features: Mature islands - Permanent mid-channel feature with surface at or above bankfull height, where 0=None, P=Present, E=Extensive >=33%, null=missing value. |  |
| UNVEG_SB | Text | None | Extent of channel and bank features: Unvegetated side-bar - A distinctive depositional feature, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| VEG_SB | Text | None | Extent of channel and bank features: Vegetated side bar - A distinctive depositional feature, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| UNVEG_PB | Text | None | Extent of channel and bank features: Unvegetated point bar, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| VEG_PB | Text | None | Extent of channel and bank features: Vegetated point bar, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| DISC_SILT_DEP | Text | None | Extent of channel and bank features: Discrete unvegetated silt deposits - A discrete silt deposit either underwater or on the margins, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| DISC_SAND_DEP | Text | None | Extent of channel and bank features: Discrete unvegetated sand deposits - A discrete sand deposit either underwater or on the margins, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| DISC_GRAVEL_DEP | Text | None | Extent of channel and bank features: Discrete unvegetated gravel deposits - A discrete unvegetated gravel deposit, where 0=None, P=Present, E=Extensive >=33%, null=missing value |  |
| SBARS_OPPO | Text | None | Are side bars opposite one another or do they alternate between banks, or both? Opposite, Alternative or Both, null=missing data |  |
| BKTOP_HT_LEFT | Number | Meters | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): Embanked height left bank (m) | L. Channel dimensions |
| BKTOP_HT_RIGHT | Number | Meters | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): Banktop height right bank (m). Banktop = first major break in slope above which cultivation or development is possible. |  |
| BANKFULL_LEFT | Text | None | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): Is banktop height also bankfull height (Y or N) left bank. Bankfull = point where river first spills on to floodplain. Banktop = first major break in slope above which cultivation or development is possible. |  |
| BANKFULL_RIGHT | Text | None | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): Is banktop height also bankfull height (Y or N) on right bank? Banktop = first major break in slope above which cultivation or development is possible. Bankfull = point where river first spills on to floodplain. |  |
| EMBANKED_HT_LEFT | Number | Meters | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): Embanked height left bank (m) |  |
| EMBANKED_HT_RIGHT | Number | Meters | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): Embanked height right bank (m) |  |
| BKFULL_WIDTH | Number | Meters | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): Bankfull width (m). Bankfull = point where river first spills on to floodplain. |  |
| WATER_WIDTH | Number | Meters | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): Water width (m). Water width is the distance across the wetted perimeter of the channel. |  |
| WATER_DEPTH | Number | Meters | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): Water depth (m). Water depth is the maximum depth measured in mid-channel. |  |
| TRASH_HEIGHT | Number | Meters | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): If trashline lower than banktop, height above water (metres) null=no value |  |
| TRASH_WIDTH | Number | Meters | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): If trashline lower than banktop, width from bank to bank (metres) null=no value |  |
| BED_MATERIAL | Number | None | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): Bed material code: 1 = consolidated 2 = unconsolidated 3 = unknown |  |
| MEASURE_LOCATION | Text | None | Channel dimensions (measured at one location on a straight uniform section, preferably across a riffle): Location of measurement. Pool (POL), Riffle (RIF), Run or glide (RUN), Other (OTH) |  |
| MEASURE_OTHER | Number | None | Channel dimensions: Free text field to describe Other location of measurement (SM = Smooth, NP = No Perceptible, flow type where measurements not taken at a riffle, DRY, null=NA) |  |
| SP_NO | Number | None | Are special features present (0= NO, -1= YES) | M. Features of special interest |
| BRAIDED_CHANNEL | Text | None | Naturally divided course of a river, characterised by at least two channels. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| SIDE_CHANNEL | Text | None | Permanent channel running parallel, and connected, to the main channel. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| WATERFALL6 | Text | None | Uninterrupted free-fall flow >5m high. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| WATERFALL4 | Text | None | Uninterrupted free-fall flow <5m high. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| NAT_CASCADE | Text | None | Natural cascade(s). (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| V_LARGE_BOULDER | Text | None | Very large boulders (>1m). (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| DEBRIS_DAM | Text | None | Log jam of woody debris creating an obstruction across the channel and ponding back water. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| LEAFY_DEBRIS | Text | None | Significant accumulations of twigs and leaf litter along channel edge. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| FRINGE | Text | None | Fringing reeds which extend at least half-way to bank top. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| QUAKING_BANK | Text | None | Quaking bank(s). (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| SINK_HOLE | Text | None | Sink hole. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| BACKWATER | Text | None | Backwater(s). (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| FLOOD_BOULDER_DEP | Text | None | Floodplain boulder deposits (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| WATER_MEADOW | Text | None | Floodplain meadows, often associated with chalk streams, traditionally deliberately flooded. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| FEN | Text | None | Mainly lowland wetlands fed by base-rich groundwater, dominated by sedge vegetation. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| BOG | Text | None | Area of rain-fed acid peatland. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| CARR | Text | None | Wet woodland composed of trees such as willow and alder, often at the edge of other wetlands. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| MARSH | Text | None | Fluvial flooded/impeded drainage habitats often dominated by rushes. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| FLUSH | Text | None | Groundwater seepage or springs producing a localised wet area. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| NAT_OPEN_WATER | Text | None | Natural open water. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| SP_OTHER | Text | None | Special feature other. null=missing value |  |
| CHOKED | Text | None | Is >33% of the channel choked with vegetation (Y or N) | N. Choked channel |
| NNPS_NO | Number | None | Nuisance species present? (0= NO, -1= YES) | O. Notable nuisance plant species |
| HOGWEED_FACE | Text | None | Notable nuisance plant species Giant hogweed bankface. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| HOGWEED_TOP | Text | None | Notable nuisance plant species Giant hogweed banktop to 50m. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| HIMALAYAN_B_FACE | Text | None | Notable nuisance plant species Himalayan balsam bankface. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| HIMALAYAN_B_TOP | Text | None | Notable nuisance plant species Himalayan balsam banktop to 50m. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| JAP_KNOTWEED_FACE | Text | None | Notable nuisance plant species Japanese knotweed bankface. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| JAP_KNOTWEED_TOP | Text | None | Notable nuisance plant species Japanese knotweed banktop to 50m. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| NNPS_OTHER | Text | None | Free text column for description of additional nuisance plant species |  |
| OTHER_FACE | Text | None | Notable nuisance plant species Other bankface. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| OTHER_TOP | Text | None | Notable nuisance plant species Other banktop to 50m. (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| OV_CHAR_NONE | Number | None | Impacts, Recent management, Animals recorded? (0= NO, -1= YES) | P. Overall characteristics |
| MAJ_IMP1-6 | Text | None | Major Impacts recorded during survey: landfill - tipping - litter - sewage - pollution - drought (DRO) - abstraction - mill - dam - road (ROA) - rail - industry - housing mining - quarrying - overdeepening (OVE) - afforestation (AFF) - fisheries management – silting (SIL) - waterlogging - hydroelectric power |  |
| L_MANAGE1-6 | Text | None | Records of Evidence of Recent Management: dredging - bank mowing - weed cutting - enhancement - river rehabilitation - gravel extraction - other (OTHER) |  |
| ANIMALS1-6 | Text | None | Sightings of mammals, birds, insects and other taxa of interest: otter - mink - water vole - kingfisher (KIN) - dipper - grey wagtail (G_W) - sand martin - heron - dragonflies/damselflies (D_F) |  |
| LBANK_F | Number | Meters | Length of bank which is fenced left bank |  |
| RBANK_F | Number | Meters | Length of bank which is fenced right bank |  |
| ALDERS | Text | None | Alders present? (0=None, P=Present, E=Extensive >=33%, null=missing value) | Q. Alders |
| DIS_ALDERS | Text | None | Diseased Alders present? (0=None, P=Present, E=Extensive >=33%, null=missing value) |  |
| COMMENTS | Text | None | Additional notes |  |
| MEMO | Text | None | Additional notes |  |

ERAMMP_STREAM_RHS_BANK_2021_23:

| Colum Name | Type | Unit | Description | RHS Section |
| --- | --- | --- | --- | --- |
| SQ_ID | Text | None | 1km survey square – unique identifier for headwater stream | Field survey details |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |  |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |  |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |  |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |  |
| SAMPLE_DATE | Date | None | Survey date |  |
| SURVEY_YEAR | Number | None | Year of survey |  |
| BANK | Text | None | Left (L) or right (R) bank |  |
| BROAD_SN | Text | None | Landuse within 50m of bank-top: Broadleaved/mixed woodland (semi-natural) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) | H. Land use within 50m of banktop (500m sweep up) |
| BROAD_PL | Text | None | Landuse within 50m of bank-top: Broadleaved/mixed plantation (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| CONIF | Text | None | Landuse within 50m of bank-top: Coniferous plantation (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| CONIF_PL | Text | None | Landuse within 50m of bank-top: Coniferous woodland semi-natural (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| SCRUB | Text | None | Landuse within 50m of bank-top: Scrub/shrubs (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| ORCHARD | Text | None | Landuse within 50m of bank-top: Orchard (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| WETLAND | Text | None | Landuse within 50m of bank-top: Wetland (e.g. bog, march, fen) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| HEATH | Text | None | Landuse within 50m of bank-top: Moorland/heath (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| OPEN_WATER_ART | Text | None | Landuse within 50m of bank-top: Artificial Open water (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| OPEN_WATER_NAT | Text | None | Landuse within 50m of bank-top: Natural Open water (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| R_PASTURE | Text | None | Landuse within 50m of bank-top: Rough unimproved grassland/pasture (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| IMP_GRASS | Text | None | Landuse within 50m of bank-top: Improved/semi-improved grassland (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| HERBS | Text | None | Landuse within 50m of bank-top: Tall herb/rank vegetation (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| ROCKSCREE | Text | None | Landuse within 50m of bank-top: Rock/scree/sand dunes (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| URBAN | Text | None | Landuse within 50m of bank-top: Suburban/urban development (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| TILLED | Text | None | Landuse within 50m of bank-top: Tilled land (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| IRRIG | Text | None | Landuse within 50m of bank-top: Irrigated land on the left bank (0=None, P=Present < 33%, E=Extensive >=33%, -9 = missing value) |  |
| PARK | Text | None | Landuse within 50m of bank-top: Parkland or gardens (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| NOTVIS | Text | None | Landuse within 50m of bank-top: Not visible (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| VERT_U | Text | None | Bank profile: Vertical/undercut (natural/unmodified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) | I. Bank profile (sweep up) |
| VERT_T | Text | None | Bank profile: Vertical with toe (natural/unmodified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| STEEP | Text | None | Bank profile: Steep (>45 degrees) (natural/unmodified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| GENTLE | Text | None | Bank profile: Gentle (<45 degrees) (natural/unmodified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| COMPOSITE | Text | None | Bank profile: Composite (natural/unmodified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| NAT_BERM | Text | None | Bank profile: Natural berm (natural/unmodified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| RESECTIONED | Text | None | Bank profile: Resectioned (reprofiled) (artificial/modified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| REINFORCED_WB | Text | None | Bank profile: Reinforced - whole bank (artificial/modified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| REINFORCED_TOP | Text | None | Bank profile: Reinforced - top only (artificial/modified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| REINFORCED_TOE | Text | None | Bank profile: Reinforced - toe only (artificial/modified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| ART_TWO | Text | None | Bank profile: Artificial two-stage (artificial/modified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| POACHED | Text | None | Bank profile: Poached bank (artificial/modified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| EMBANK | Text | None | Bank profile: Embanked (artificial/modified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |
| SB_EMBANK | Text | None | Bank profile: Set-back embankment (artificial/modified profile) (0=None, P=Present < 33%, E=Extensive >=33%, null=missing value) |  |

ERAMMP_STREAM_RHS_SPOTCHECK_2021_23:

| Colum Name | Type | Unit | Description | RHS Section |
| --- | --- | --- | --- | --- |
| SQ_ID | Text | None | 1km survey square – unique identifier for headwater stream | Field survey details |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |  |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |  |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |  |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |  |
| SAMPLE_DATE | Date | None | Survey date |  |
| SURVEY_YEAR | Number | None | Year of survey |  |
| SPOT | Text | None | Spot-check number (C) 1-10 (C11= channel substrate not occurring as predominant in spot-checks, but present in >1% of whole site. For channel vegetation, a summary of the 500m including types not occurring in spot checks and any type not recorded at spot checks but present at >1% of the whole RHS site). |  |
| LB_MAT | Text | None | Left Bank Material at spot site where: NV = Not Visible , BE = Bedrock , BO = Boulder, CO = Cobble, GS = Gravel/Sand, EA = Earth, PE = Peat, CL = Sticky Clay, CC = Concrete, SP = Sheet Piling, WP = Wood Piling, GA = Gabion, BR = Brick/laid Stone, RR = Rip-rap, TD = Tipped Debris, FA = Fabric, BI = Bio-engineering materials, null=Missing value. | E. Physical attributes - spot-checks |
| LB_MD1 | Text | None | Left Bank Modification at spot site where: NK = Not Known, NO = None, RS = Resectioned, RI = Reinforced, PC = Poached, PC(B) = Poached(Bare), BM = Artificial Berm, EM = Embanked, null=Missing value |  |
| LB_MD2 | Text | None | Second Left Bank Modification at spot site where: NK = Not Known, NO = None, RS = Resectioned, RI = Reinforced, PC = Poached, PC(B) = Poached(Bare), BM = Artificial Berm, EM = Embanked, null=Missing value |  |
| LB_FE1 | Text | None | Left Marginal and Bank Features where: NV = Not Visible, NO = None, EC = Eroding Cliff, SC = Stable Cliff, PB = Unvegetated Point Bar, VP = Vegetated Point Bar, SB = Unvegetated Side Bar, VS = Vegetated Side Bar, NB = Natural berm, null=Missing value |  |
| LB_FE2 | Text | None | Secondary Left Bank Features where: NV = Not Visible, NO = None, EC = Eroding Cliff, SC = Stable Cliff, PB = Unvegetated Point Bar, VP = Vegetated Point Bar, SB = Unvegetated Side Bar, VS = Vegetated Side Bar, NB = Natural berm, null=Missing value |  |
| CH_SUB | Text | None | Channel Substrate where: NV = Not Visible, BE = Bedrock, BO = Boulder, CO = Cobble, GP = Gravel/Pebble, SA = Sand, SI = Silt, CL = Clay, PE = Peat, EA = Earth, AR = Artificial, null=Missing value |  |
| C_SUB1 | Text | None | Channel substrate not occurring as predominant in spot-checks but present in >1% of whole site 1 where: NV = Not Visible, NO = None, BE = Bedrock, BO = Boulder, CO = Cobble, GP = Gravel/Pebble, SA = Sand, SI = Silt, CL = Clay , PE = Peat, EA = Earth, AR = Artificial, null=Missing value |  |
| C_SUB2 | Text | None | Channel substrate not occurring as predominant in spot-checks but present in >1% of whole site 2 where: NV = Not Visible, NO = None, BE = bedrock, BO = Boulder, CO = Cobble, GP = Gravel/Pebble, SA = Sand, SI = Silt, CL = Clay , PE = Peat, EA = Earth, AR = Artificial, null=Missing value |  |
| C_SUB3 | Text | None | Channel substrate not occurring as predominant in spot-checks but present in >1% of whole site 3 where: NV = Not Visible, NO = None, BE = Bedrock, BO = Boulder, CO = Cobble, GP = Gravel/Pebble, SA = Sand, SI = Silt, CL = Clay , PE = Peat, EA = Earth, AR = Artificial, null=Missing value |  |
| CH_FLW | Text | None | Predominant Flow Type at spot site where: NV = Not Visible, FF = Free Fall, CH = Chute, BW = Broken Standing Wave, UW = Unbroken standing Wave, CF = Chaotic Flow, RP = Rippled, UP = Upwelling, SM = Smooth, NP = No Perceptible Flow, DR = Dry (no flow), null=Missing value |  |
| CH_MD1 | Text | None | Channel Modification(s) at spot site where: NK = Not Known, NO = None, CV = Culverted, RS = Resectioned, RI = Reinforced, DA = Dam/Weir/Sluice, FO = Ford (man-made), null=Missing value |  |
| CH_MD2 | Text | None | Second Channel Modification(s) at spot site where: NK = Not Known, NO = None, CV = Culverted, RS = Resectioned, RI = Reinforced, DA = Dam/Weir/Sluice, FO = Ford (man-made), null=Missing value |  |
| CH_FE1 | Text | None | Channel Feature(s) at spot site, where: NV = Not Visible, NO = None, EB = Exposed Bedrock, RO = Exposed Boulders, VR = Vegetated Rock, MB = Unvegetated mid-channel bar, VB = Vegetated mid-channel bar, MI = Mature Island, TR = Trash (urban debris), null=Missing value |  |
| CH_FE2 | Text | None | Second Channel Feature(s) at spot site, where: NV = Not Visible, NO = None, EB = Exposed Bedrock, RO = Exposed Boulders, VR = Vegetated Rock, MB = Unvegetated mid-channel bar, VB = Vegetated mid-channel bar, MI = Mature Island, TR = Trash (urban debris), null=Missing value |  |
| CH_WET | Text | None | Braided rivers only - number of wet channels. null indicates a missing value. |  |
| CH_DRY | Text | None | Braided rivers only - number of dry channels. null indicates a missing value. |  |
| RB_MAT | Text | None | Right Bank Material where: NV = Not Visible, BE = Bedrock, BO = Boulder, CO = Cobble, GS = Gravel/Sand, EA = Earth, PE = Peat, CL = Sticky Clay, CC = Concrete, SP = Sheet Piling, WP = Wood Piling, GA = Gabion, BR = Brick/laid Stone, RR = Rip-rap, TD = Tipped Debris, FA = Fabric, BI = Bio-engineering materials, null=Missing value |  |
| RB_MD1 | Text | None | Right Bank Modification where: NK = Not Known, NO = None, RS = Resectioned, RI = Reinforced, PC = Poached, PC(B) = Poached(Bare), BM = Artificial Berm, EM = Embanked, null=Missing value |  |
| RB_MD2 | Text | None | Second Right Bank Modification where: NK = Not Known, NO = None, RS = Resectioned, RI = Reinforced, PC = Poached, PC(B) = Poached(Bare), BM = Artificial Berm, EM = Embanked, null=Missing value |  |
| RB_FE1 | Text | None | Right Bank Features where: NV = Not Visible, NO = None, EC = Eroding Cliff , SC = Stable Cliff, PB = Unvegetated Point Bar, VP = Vegetated Point Bar, SB = Unvegetated Side Bar, VS = Vegetated Side Bar, NB = Natural berm, null=Missing value |  |
| RB_FE2 | Text | None | Secondary Right Bank Features where: NV = Not Visible, NO = None, EC = Eroding Cliff , SC = Stable Cliff, PB = Unvegetated Point Bar, VP = Vegetated Point Bar, SB = Unvegetated Side Bar, VS = Vegetated Side Bar, NB = Natural berm, null=Missing value |  |
| USE5_L | Text | None | Land use within 5m of bank top (Left) where: BL = Broadleaf/mixed woodland (semi-natural), BP = Broadleaf/mixed plantation, CW = Coniferous woodland (semi-natural), CP = Coniferous plantation, SH = Scrub & shrubs, OR = Orchard, WL = Wetland (e.g. bog, marsh, fen), MH = Moorland/heath, AW = Artificial open water, OW = Natural open water, RP = Rough unimproved grassland/pasture, IG = Improved/semi-improved grassland, TH = Tall herb/rank vegetation, RD = Rock, scree or sand dunes, SU = Suburban/urban development, TL = Tilled land, IL = Irrigated land, PG = Parkland or gardens, NV = Not visible, null=Missing value | F. Banktop land-use and vegetation |
| L_BTOP | Text | None | Left bank-top vegetation structure where: B = BARE, U = UNIFORM, S = SIMPLE, C = COMPLEX, NV = NOT VISIBLE. The left bank is on the lefthand side when facing downstream. null=Missing value |  |
| L_BFAC | Text | None | Left bank face vegetation structure where: B = BARE, U = UNIFORM, S = SIMPLE, C = COMPLEX, NV = NOT VISIBLE. The left bank is on the lefthand side when facing downstream. null=Missing value |  |
| R_BFAC | Text | None | Right bank face vegetation structure where: B = BARE, U = UNIFORM, S = SIMPLE, C = COMPLEX, NV = NOT VISIBLE. The left bank is on the lefthand side when facing downstream. null=Missing value |  |
| R_BTOP | Text | None | Right bank-top vegetation structure where: B = BARE, U = UNIFORM, S = SIMPLE, C = COMPLEX, NV = NOT VISIBLE. The left bank is on the lefthand side when facing downstream. null=Missing value |  |
| USE5_R | Text | None | Land use within 5m of bank top (Right) where: BL = Broadleaf/mixed woodland (semi-natural), BP = Broadleaf/mixed plantation, CW = Coniferous woodland (semi-natural), CP = Coniferous plantation, SH = Scrub & shrubs, OR = Orchard, WL = Wetland (e.g. bog, marsh, fen), MH = Moorland/heath, AW = Artificial open water, OW = Natural open water, RP = Rough unimproved grassland/pasture, IG = Improved/semi-improved grassland, TH = Tall herb/rank vegetation, RD = Rock, scree or sand dunes, SU = Suburban/urban development, TL = Tilled land, IL = Irrigated land, PG = Parkland or gardens, NV = Not visible, null=Missing value |  |
| VEG_NONE | Number | None | Whether there is no channel vegetation. Channel vegetation is not present = -1, Channel vegetation is present = 0, null=missing value | G. Channel vegetation types - spot-checks |
| CV_B_L | Text | None | Bryophytes/lichens (% range of cover). NV = Not Visible, NO = None, null=Missing value |  |
| CV_HER | Text | None | Emergent broad-leaved herbs (% range of cover). NV = Not Visible, NO = None, null=Missing value |  |
| CV_REE | Text | None | Emergent reeds/sedges/rushes (% range of cover). NV = Not Visible, NO = None, null=Missing value |  |
| CV_FOL | Text | None | Floating-leaved (rooted) (% range of cover). NV = Not Visible, NO = None, null=Missing value |  |
| CV_FRF | Text | None | Free-floating (% range of cover). NV = Not Visible, NO = None, null=Missing value |  |
| CV_AMP | Text | None | Amphibious (% range of cover). NV = Not Visible, NO = None, null=Missing value |  |
| CV_SBT | Text | None | Submerged broad-and fine/linear leaved (% range of cover). NV = Not Visible, NO = None, null=Missing value |  |
| CV_SBL | Text | None | Submerged broad-leaved (% range of cover). NV = Not Visible, NO = None, null=Missing value |  |
| CV_FLL | Text | None | Submerged fine/linear-leaved (% range of cover). NV = Not Visible, NO = None, null=Missing value |  |
| CV_FIA | Text | None | Filamentous Algae (% range of cover). NV = Not Visible, NO = None, null=Missing value |  |
| CV_FIN | Text | None | Submerged fine-leaved (% range of cover). NV = Not Visible, NO = None, null=Missing value |  |
| CV_LL | Text | None | Submerged linear-leaved (% range of cover). NV = Not Visible, NO = None, null=Missing value |  |

ERAMMP_STREAM_RHS_MODIFICATION_SCORES_2021_23:

| Colum Name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Text | None | Unique identifier for ERAMMP survey square |
| SAMPLE_DATE | Date | None | Survey date |
| HABITAT_MODIFICATION_SCORE | Number | None | Habitat Modification Score |
| HABITAT_MODIFICATION_CLASS | Number | None | Habitat Modification Class |
| CULVERTS | Number | None | Habitat Modification Score Subsection Culverts |
| BANK_BED_REINFORCEMENT | Number | None | Habitat Modification Score Subsection Bank & Bed Reinforcement |
| BANK_BED_RESECTIONING | Number | None | Habitat Modification Score Subsection Bank & Bed Resectioning |
| BERMS_AND_EMBANKMENT | Number | None | Habitat Modification Score Subsection Berms and Embankment |
| WEIRS_DAMS_SLUICES | Number | None | Habitat Modification Score Weirs/Dams/Sluices |
| BRIDGES | Number | None | Habitat Modification Score Subsection Bridges |
| POACHING | Number | None | Habitat Modification Score Subsection Poaching |
| FORDS | Number | None | Habitat Modification Score Subsection Fords |
| OUTFALLS_DEFLECTORS | Number | None | Habitat Modification Score Subsection Outfalls & Deflectors |
| SURVEY_YEAR | Number | None | Year of survey. |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |

ERAMMP_STREAM_RHS_QUALITY_SCORES_2021_23:

| Colum Name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Text | None | Unique identifier for ERAMMP survey square |
| SAMPLE_DATE | Date | None | Survey date |
| HABITAT_QUALITY | Number | None | Habitat Quality Score |
| FLOWS | Number | None | Habitat Quality Score Subsection Flows |
| CHANNEL_SUBSTRATES | Number | None | Habitat Quality Score Subsection Channel substrates |
| CHANNEL_FEATURES | Number | None | Habitat Quality Score Subsection Channel features |
| BANK_FEATURES | Number | None | Habitat Quality Score Subsection Bank Features |
| BANK_VEG_STRUCTURE | Number | None | Habitat Quality Score Subsection Bank vegetation structure |
| CHANNEL_VEG | Number | None | Habitat Quality Score Subsection Channel vegetation |
| LAND_USE | Number | None | Habitat Quality Score Subsection Land use |
| TREES_ASSOCIATED_FEATURES | Number | None | Habitat Quality Score Subsection Trees and associated features |
| SPECIAL_FEATURES | Number | None | Habitat Quality Score Subsection Special features |
| SURVEY_YEAR | Number | None | Year of survey. |
| EASTING_10km | Number | Metre | Easting of survey square at a 10km resolution (OSGB 1936, metres) |
| NORTHING_10km | Number | Metre | Northing of survey square at a 10km resolution (OSGB 1936, metres) |
| LAND_CLASSIFICATION_NUMBER | Number | None | ITE Land Classification number – see Bunce et al., 2007. |
| SQ_TYPE | Text | None | Wider Wales (WW) for national trends, or Glastir Targeted (TG). Data from Glastir Targeted squares are to be used only in conjunction with associated Glastir data and data from Wider Wales squares for Glastir assessment. |

River Waterbody Catchment

GMEP_HEADWATERS_BY_WFD:

Null values in fields indicate that headwater stream is outside the major catchments.

| Colum Name | Type | Unit | Description |
| --- | --- | --- | --- |
| SQ_ID | Text | None | Unique identifier for ERAMMP survey square |
| WATERBODY_ID | Text | None | Unique identifier for Waterbody Catchment – from Water Framework Directive (WFD) River Waterbody Catchments Cycle 3 dataset |
| WB_NAME | Text | None | Waterbody Catchment name – from Water Framework Directive (WFD) River Waterbody Catchments Cycle 3 dataset |

## Relationship between tables:

SQ_ID is a unique identifier for each headwater stream.

The following tables contain one row per headwater stream:

- ERAMMP_STREAM_DIATOM_ENV_2021_23.csv

- ERAMMP_STREAM_DIATOM_INDICES_2021_23.csv

- ERAMMP_STREAM_INVERT_ENV_2021_23.csv

- ERAMMP_STREAM_INVERT_INDICES_2021_23.csv

- ERAMMP_STREAM_WATER_CHEMISTRY_2021_23.csv

- ERAMMP_STREAM_RHS_MAIN_2021_23.csv

- ERAMMP_STREAM_RHS_MODIFICATION_SCORES_2021_23.csv

- ERAMMP_STREAM_RHS_QUALITY_SCORES_2021_23.csv

- GMEP_HEADWATERS_BY_WFD.csv

The following tables contain multiple rows per headwater stream:

- ERAMMP_STREAM_DIATOMS_2021_23.csv

- ERAMMP_STREAM_INVERTS_2021_23.csv

- ERAMMP_STREAM_RHS_BANK_2021_23.csv

- ERAMMP_STREAM_RHS_SPOTCHECK_2021_23.csv

# References

Alison, J., Bowgen, K., Siriwardena, G., Williams, B., Reinsch, S., Garbutt, A., Emmett, B.A. (2021) Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP). ERAMMP Report-58: ERAMMP Square Selection Protocol 2021/2022. Report to Welsh Government (Contract C210/2016/2017) (UK Centre for Ecology & Hydrology Projects 06297 & 06810):

https://erammp.wales/sites/default/files/2022-11/58%20ERAMMP%20Report-58%20Square58%20%20Selection%20Protocol%202021%20v1.0_en.pdf

Armitage, P. D., Moss, D., Wright, J. F. & Furse, M. T. (1983) The performance of a new biological water quality score system based on macroinvertebrates over a wide range of unpolluted running-water sites. Water Research, 17, 333-347. https://doi.org/10.1016/0043-1354(83)90188-4

Bunce, R. G. H., Barr, C. J., Clarke, R. T., Howard, D. C., Scott, W. A. (2007) ITE Land Classification of Great Britain 2007. NERC-Environmental Information Data Centre doi: 10.5285/5f0605e4-aa2a-48ab-b47c-bf5510823e8f

Carey, P. D.; Wallis, S.; Chamberlain, P. M.; Cooper, A.; Emmett, B. A.; Maskell, L. C.; McCann, T.; Murphy, J.; Norton, L. R.; Reynolds, B.; Scott, W. A.; Simpson, I. C.; Smart, S. M.; Ullyett, J. M. (2008). Countryside Survey: UK Results from 2007 NERC/Centre for Ecology & Hydrology (CEH Project Number: C03259): https://nora.nerc.ac.uk/id/eprint/5191/1/N005191CR%20UK%20Results.pdf

Chadd, R. & Extence, C. (2004) The conservation of freshwater macroinvertebrate populations: a community-based classification scheme. Aquatic Conservation: Marine and Freshwater Ecosystems, 14, 597-624. https://doi.org/10.1002/aqc.630

Chadd, R. P., England, J. A., Constable, D., Dunbar, M. J., Extence, C. A., Leeming, D. J., Murray-Bligh, J. A., Wood, P. J. (2017). An index to track the ecological effects of drought development and recovery on riverine invertebrate communities. Ecological Indicators, 82, 344-356. https://doi.org/10.1016/j.ecolind.2017.06.058.

Emmett, B. A. et al. (2013). An integrated ecological, social and physical approach to monitoring environmental change and land management effects: the Wales Axis II Monitoring and Evaluation Programme. Aspects of applied biology, 31-39.

Emmett, B.E., Abdalla, M., Anthony, S., Astbury, S., August, T., Barrett, G., Biggs, J., Botham, M., Bradley, D., Brown, M., Carter, H., Chadwick, D., Cigna, F., Cooper, D., Cooper, J., Cosby, BJ., Creer, S Cross, P., Edwards, F., Edwards, M., Evans, C., Ewald, N., Fitton, A., Garbutt, A., Grebby, S., Greene, S., Halfpenney, I., Hall, J., Harrison, S., Harrower, C., Henrys, P., Hobson, R., Hughes, S., Isaac, N., Jackson, B., Jarvis, S., Jones, D., Keith, A., Kelly, M., Korenko, J., Lallias, D., Leaver, D., Lebron, I., Malcolm, H., Maskell, L., McDonald, J., Moxley, J., Norton, L., O’Hare, M., Owen, A., Pereira, M., Peyton, J., Powney, G., Pywell, R., Rawlins, B., Robinson, D.A., Rorke, S., Rowland, C.,Roy, D., Scarlett, P., Scholefield, P., Scott, A., Scott, L., Scott, R., Siriwardena, G., Smart, S., Smith, P., Swetnam, R., Taylor, R., Tordoff, G., Van Breda, J., Vincent, H., Wagner, M., Waters, E., Watkins, J.,White, J., Williams, B., Wood, C. and Wright, S. (2014) Glastir Monitoring & Evaluation Programme. First Year Annual Report to Welsh Government (Contract reference: C147/2010/11). NERC/Centre for Ecology & Hydrology (CEH Project: NEC04780), pp.442

Emmett, B.E., Abdalla, M., Anthony, S., Astbury, S., August, T., Barrett, G., Beckman, B., Biggs, J., Botham, M., Bradley, D., Brown, M., Burden, A., Carter, H., Chadwick, D., Cigna, F., Collier, R., Cooper, D., Cooper, J., Cosby, B.J., Creer, S., Cross, P., Dadam, D., Edwards, F., Edwards, M., Evans, C., Ewald, N., Fitton, A., Garbutt, A., Giampieri, C., Gooday, R., Grebby, S., Greene, S., Halfpenney, I., Hall, J., Harrison, S., Harrower, C., Henrys, P., Hobson, R., Hughes, P., Hughes, S., Illian, J., Isaac, N., Jackson, B., Jarvis, S., Jones, D.L., Jones, P., Keith, A., Kelly, M., Kneebone, N., Korenko, J., Lallias, D., Leaver, D., Lebron, I., Malcolm, H., Maskell, L., McDonald, J., Moxley, J., Norton, L., O’Hare, M., Oliver, T., Owen, A., Parkhill, K.A., Pereira, M., Peyton, J., Pogson, M., Powney, G., Pritchard, N., Pritchard, S., Prochorskaite, A., Prosser, M., Pywell, R., Rawlins, B., Reuland, O., Richards, M., Robinson, D.A., Rorke, S., Rowland, C., Roy, D., Scarlett, P., Scholefield, P., Scott, A, Scott, L., Scott, R., Sharps, K., Siriwardena, G., Smart, S., Smith, G., Smith, P., Stopps, J., Swetnam, R., Taft, H., Taylor, R., Tebbs, E., Thomas, A., Todd-Jones, C., Tordoff, G., Turner, G., Van Breda, J., Vincent, H., Wagner, M., Waters, E., Walker-Springett, K., Wallace, H., Watkins, J., Webb, G., White, J., Whitworth, E., Williams, B., Williams, P., Wood, C. and Wright, S. (2017) Glastir Monitoring & Evaluation Programme. Final Report to Welsh Government. Contract reference: C147/2010/11. NERC/Centre for Ecology & Hydrology (CEH Projects: NEC04780/NEC05371/NEC05782)

Environment Agency (2022) River Habitat Survey in Britain and Ireland. Field Survey Guidance Manual: 2003 Version (2022 reprint): https://assets.publishing.service.gov.uk/media/62dff4138fa8f564a21dcd5e/RHS-manual-2003_2022-reprint-LIT-1758.pdf

Extence, C.A., Balbi, D.M. & Chadd, R.P. (1999) River flow indexing using British benthic macroinvertebrates: a framework for setting hydroecological objectives. Regulated Rivers- Research & Management, 15, 543-574. https://doi.org/10.1002/(SICI)1099-1646(199911/12)15:6<545::AID-RRR561>3.0.CO;2-W

Extence, C.A., Chadd, R.P., England, J., Dunbar, M.J., Wood, P.J., Taylor, E.D. (2013) The assessment of fine sediment accumulation in rivers using macro-invertebrate community response. River Research And Applications, 29 (1) 1- 133. https://doi.org/10.1002/rra.1569

Juggins, S. & Kelly, M. (2012) Development of a Water Framework Directive-compatible metric for assessing acidification in UK and Irish rivers. Report SC070034/TR2. Environment Agency, Bristol.

Jüttner I., Carter C., Cox E.J., Ector L., Jones V., Kelly M.G., Kennedy B., Mann D.G., Turner J. A., Van de Vijver B., Wetzel C.E., Williams D.M. (2026) Freshwater Diatom Flora of Britain and Ireland. Amgueddfa Cymru - National Museum Wales. Available online at https://naturalhistory.museumwales.ac.uk/diatoms.
[ Accessed:  22 January 2026 ]

Kelly, M. G. & Whitton, B. A. (1995) Trophic diatom index - a new index for monitoring eutrophication in rivers. Journal of Applied Phycology, 7, 433-444. https://doi.org/10.1007/BF00003802

Kelly, M.G., Juggins, S., Bennion, H., Burgess, A., Yallop, M., Hirst, H., Jamieson, J., Guthrie, R., Rippey, B. (2014) DARLEQ: Diatom Assessment of River and Lake Ecological Quality, version 2.0 User Guide. The DARLEQ Consortium.

Lange-Bertalot, H., Hofmann, G. Werum, M. & Cantonati, M. (2017).  Freshwater Diatoms of Central Europe: Over 800 species Used in Ecological Assessment.  Edited by M. Cantonati, M.G. Kelly & H. Lange-Bertalot.  Koeltz Botanical Books, Schmitten-Oberreifenberg.

Natural Resources Wales (2023). Water Framework Directive (WFD) River Waterbody Catchments Cycle 3 [Dataset]. DataMapWales. https://datamap.gov.wales/layers/geonode:nrw_wfd_river_waterbody_catchments_c3_baseline_classification

Robinson, D.A., Bentley, L., Jones, L., Feeney, C., Garbutt, A., Tandy, S., Lebron, I., Thomas, A., Reinsch, S., Norton, L., Maskell, L., Wood, C., Henrys, P., Jarvis, S., Smart, S., Keith, A., Seaton, F., Skates, J., Higgins, S., Giuffrè, G., Emmett, B. A. (2024). Five decades' experience of long-term soil monitoring, and key design principles, to assist the EU soil health mission. European Journal of Soil Science, 75(5), e13570. https://doi.org/10.1111/ejss.13570

Scarlett, P., Edwards, F., Ewald, N., Kelly, M., Garbutt, R.A., Gunn, I., O’Hare, M., Vincent, H. & Webb, G. (2021). Environment and Rural Affairs Monitoring & Modelling Programme (ERAMMP). ERAMMP Report-50: Field-Survey Handbook (Procedures) – Freshwater 2021. Report to Welsh Government (Contract C210/2016/2017) (UK Centre for Ecology & Hydrology Projects 06297 & 06810): https://erammp.wales/sites/default/files/2023-07/50-ERAMMP-Doc-50-Freshwater-Field-Handbook-2021-en.pdf

Water Framework Directive – United Kingdom Technical Advisory Group (WFD-UKTAG) (2013). Final recommendations on new and updated biological standards. Annex 5 Rivers Invertebrates, WFD–AWIC: https://www.wfduk.org/sites/default/files/Media/UKTAG%20Final%20recommendations%20on%20biological%20stds_20131030.PDF

Water Framework Directive – United Kingdom Technical Advisory Group (WFD-UKTAG) (2014). UKTAG River Assessment Method Macrophytes and Phytobenthos: Phytobenthos – Diatoms for Assessing River and Lake Ecological Quality (River DARLEQ2). ISBN - 978-1-906934-58-3.

Water Framework Directive – United Kingdom Technical Advisory Group (WFD-UKTAG) (2021). UKTAG River Assessment Method Benthic Invertebrate Fauna: Invertebrates (General Degradation): Walley, Hawkes, Paisley & Trigg (WHPT) metric in River Invertebrate Classification Tool (RICT). ISBN: 978-1-84911-483-7

Wood, C. M., Bunce, R. G. H., Norton, L. R., Maskell, L. C., Smart, S. M., Scott, W. A., Henrys, P. A., Howard, D. C., Wright, S. M., Brown, M. J., Scott, R. J., Stuart, R. C., & Watkins, J. W. (2018). Ecological landscape elements: Long-term monitoring in Great Britain, the Countryside Survey 1978-2007 and beyond. Earth System Science Data, 10(2), 745–763. https://doi.org/10.5194/essd-10-745-2018

Wood, C. M., Alison, J., Botham, M. S., Burden, A., Edwards, F., Garbutt, R. A., George, P. B. L., Henrys, P. A., Hobson, R., Jarvis, S., Keenan, P., Keith, A. M., Lebron, I., Maskell, L. C., Norton, L. R., Robinson, D. A., Seaton, F. M., Scarlett, P., Siriwardena, G. M., Skates, J., Smart, S. M., Williams, B., and Emmett, B. A. (2021) Integrated ecological monitoring in Wales: the Glastir Monitoring and Evaluation Programme field survey, Earth System Science Data, 13, 4155–4173, https://doi.org/10.5194/essd-13-4155-2021

ERAMMP Programme Office

UKCEH Bangor

Environment Centre Wales

Deiniol Road

Bangor, Gwynedd

LL57 2UW

+ 44 (0)1248 374500

erammp@ceh.ac.uk

www.erammp.cymru

www.erammp.wales