# Nitrogen emissions-related datasets for South Asia: atmospheric deposition, fertiliser application, human and livestock populations, hydrological characteristics, urban land-use, 1980-2035.

This dataset consists of inputs of ammonium (NH4+), nitrate (NO3-) and phosphorus (P) to soil from the use of synthetic fertilisers, atmospheric deposition of reduced (NHx) and oxidised (NOy) nitrogen (N), three hydrological characteristics (drainage direction, accumulated catchment area, coastal outflow location), urban land use, human population and head counts of livestock in eight broad categories. The domain covers the South Asian Association for Regional Cooperation (SAARC) and represents data used and/or produced within the South Asia Nitrogen Hub project (SANH). These gridded spatio-temporal data are available for four current or near-future scenarios (see below for details) at a spatial resolution of 0.1˚ x 0.1˚.

# Summary

The data provided here were produced as part of the South Asia Nitrogen Hub project (SANH – Global Challenge Research Fund, Grant Ref. Number NE/S009019/2) during 2022 and 2023. The spatio-temporal nature of the data allows for integrated modelling across long time-series under four scenarios specific to the SANH project; ‘Baseline’ (Low Urea emission factor scenario: 1980 – 2020), ‘CLE’ (Current legislation scenario for 2021 – 2035), ‘MTFR’ (Maximum Technically Feasible Reduction scenario for 2021 – 2035) and MTFR+DIET (Dietary change scenario for 2021 – 2035).

Spatial datasets of nutrient inputs from fertiliser (NH4+, NO3- and P) for 2015 were constructed from crop data (Monfreda et al., 2008; Ramankutty et al., 2008), fertiliser use (FAOSTAT Database – FAO, 2023) and composition (Nishina et al., 2017; FAOSTAT Database – FAO, 2023), crop planting dates (Sacks et al., 2010), fertiliser placement timings (Nishina et al., 2017) and crop-specific fertiliser application rates (Cui et al., 2021). Further spatial information regarding crop distributions and fertiliser use were incorporated for India specifically (IARI, 2016), which was available through the SANH programme. Historic (1980 to 2014 and 2016 to 2020) and projected (2021 to 2035) fertiliser usage was taken from the FAOSTAT Database (FAO, 2023) and The Future of Food and Agriculture (FAO, 2022) respectively, plus other scenario conditions (see Methods).

Atmospheric N deposition data for 2015 was obtained from the European Monitoring and Evaluation Programme Meteorological Synthesizing Centre-West (EMEP MSC-W) model output, as outlined in Wang et al. (2025). Pre-2015 N deposition was informed by The Inter-Sectoral Impact Model Intercomparison Project (Tian et al., 2018) while post-2015 data were informed by scenario parameters.

Hydrological characteristics are assumed static through time and are represented by one surface per variable to be used in any year of analysis. Grid-cell drainage outflow direction is based on the Dominant River Tracing (DRT) flow direction data (Wu et al., 2011; Wu et al., 2012). These are used to calculate the upstream catchment area to every grid-cell including the grid-cell itself. Similarly, drainage directions are used to detect coastal outflow cells which indicate whether a coastal grid-cell drains into the sea, or inland. In combination with gridded freshwater flow/flux data, this dataset can be used to calculate the flow/flux of freshwater entering coastal, estuarine or marine waters.

Urban land use is assumed constant in time and is extracted from the 2015 land cover classification product from the European Space Agency (ESA) Climate Change Initiative (CCI) (Copernicus Climate Change Service, 2019).

Human population data for 2015 were obtained from the Gridded Population of the World dataset (GPW) (CIESIN, 2018). Data from several sources were combined to derive a consistent time-series: pre-2015 population estimates were obtained from the History Database of the Global Environment (HYDE, v 3.2.1) (Goldewijk et al., 2017) while post-2015 data were obtained from The Future of Food and Agriculture (FAO, 2022). Similarly, spatial livestock populations for a base year of 2010 were sourced from the Gridded Livestock of the World (GLW3) (Gilbert et al., 2018), then these data were perturbed using  pre-2010 population data from the FAOSTAT Database (FAO, 2023) and post-2010 data from The Future of Food and Agriculture (FAO, 2022).

N.B. It is important to note that for these data:

- Human and livestock population time series are the same for the CLE, MTFR and MTFR+DIET scenarios (i.e. 2021 onwards) and are included for all scenarios for completeness.

- Hydrological characteristics and urban land use do not change across any scenario (and are fixed in time) and are included for all scenarios for completeness.

File format

All data are provided in NetCDF format (.nc).

# File naming convention

The filenames are structured as follows;

xxxx_yyyyyyyy_aaaa_bbbb.nc

Where ‘xxxx’ is the scenario name the data belongs to, ‘yyyyyyyy’ is the grouping the data belongs to, ‘aaaa’ is the start year of the temporal coverage and ‘bbbb’ is the end year of the temporal coverage.

Available options for the filename components are summarised in Table 1 below.

Table 1. Summary of available options for the filename components. It should be noted that for filename structure, the Baseline scenario is named ‘BASE’ and the MTFR+DIET scenario is named ‘DIET’.

| Filename component | xxxx | yyyyyyyy | aaaa | bbbb |
| --- | --- | --- | --- | --- |
| Description | Scenario name | Data type | Start year | End year |
| Available options | - BASE - CLE - MTFR - DIET | - Population - Fertiliser - Deposition - Hydrology - urban_lu | - 1980 (BASE) - 2021 (CLE, MTFR and DIET) - Fixed (hydrological characteristics or urban land use only). | - 2020 (BASE) - 2035 (CLE, MTFR and DIET) - Fixed (hydrological characteristics or urban land use only). |

For each of these data types, the following variables are available:

- Population: human population ("human_popn"), Buffalo head count ("buffalo"), Cattle head count ("cattle"), Chicken head count ("chicken"), Duck head count ("duck"), Goat head count ("goat"), Horse head count ("horse"), Pig head count ("pig") and Sheep head count ("sheep").

- Fertiliser: Ammonium (NH4+) to soil from synthetic fertiliser use (“NH4N_fert”), Nitrate (NO3-) to soil from synthetic fertiliser use (“NO3N_fert”) and Phosphorous (P) to soil from synthetic fertiliser use (“P_fert”).

- Deposition: Reduced nitrogen atmospheric deposition ("NHx_dep") and oxidised nitrogen atmospheric deposition ("NOy_dep").

- Hydrology: Grid-cell catchment area (“catchment_area”), coastal outflow locations (“coastal_outflow”) and drainage direction (“drainage_dir”).

- urban_lu: Urban land use fraction (“urban_frac”).

# Nature and units of recorded values

Units per variable listed above plus other attributes of the .nc data (no-data values are given as NA):

| Variable name | Description | Units | Units name .nc file |
| --- | --- | --- | --- |
| human_popn | number of persons per grid-cell per year | Number of people | Npeople |
| buffalo, cattle, chicken, duck, goat, horse, pig and sheep | total head of livestock per grid-cell per year | Livestock number | Nlivestock |
| NH4N_fert, NO3N_fert and P_fert | tonnes N or P per grid-cell per month from the use of fertilisers | Tonnes | tonnes |
| NHx_dep and NOy_dep | grammes N from atmospheric deposition, reduced or oxidised | Grammes N per m2 per month | gNm2 |
| catchment_area | number of grid-cells draining to each cell, including the cell itself | Number of 0.1° x 0.1° grid-cells | Ncells |
| coastal_outflow | indicates whether a coastal (land) grid-cell drains to the sea | To the sea (1) or inland (0) | Coastalflowindicator |
| drainage_dir | the D8 outflow drainage direction (E = 1, SE = 2, S = 4, SW = 8, W = 16, NW = 32, N = 64, NE = 128) | Unitless | D8 |
| urban_frac | fraction of grid-cell that is urban land use (0 ≤ urban_frac ≤ 1) | Proportion of 0 (zero) to 1 (complete urban coverage) | cellfraction |
| time | days from 01-01-1900 | days | days since 01-01-1900 |
| lat / lon | Latitude / longitude | degrees | degrees_north / degrees_east |

Spatial coverage
The coordinate reference system is the World Geodetic System 1984 (WGS84).

The domain is: xmin = 60˚, xmax = 100˚, ymin = -2˚, ymax = 41˚, and the spatial resolution is 0.1˚ x 0.1˚.

# Temporal coverage and resolution

Scenarios are provided for the following years:

- Baseline: 1980 to 2020

- CLE, MTFR & MTFR+DIET: 2021 to 2035

The temporal resolution of available data types is summarised in Table 2 below.

Table 2. Summary of temporal metadata for each dataset. All data are provided for 4 scenarios, but for hydrological characteristics and urban datasets these scenarios are all identical.

| Data type | Variable | Temporal range | Temporal resolution | Notes |
| --- | --- | --- | --- | --- |
| Fertiliser nutrient inputs | NH4N_fert NO3N_fert P_fert | 1980 – 2035 | Monthly |  |
| Atmospheric deposition | NHx_dep NOy_dep | 1980 – 2035 | Monthly |  |
| Hydrological characteristics | drainage_dir catchment_area coastal_outflow | Representative of any year | NA |  |
| Urban land use | urban_frac | Representative of any year | NA |  |
| Human population | human_popn | 1980 – 2035 | Annual | CLE, MTFR & MTFR+DIET scenario datasets are identical. |

## Collection/Generation/Transformation methods

Scenarios: Time-series data produced for the SANH project are available for 4 scenarios. These scenarios are defined as follows:

- Baseline: The key characteristic of the Baseline scenario is a reduction in the emission factor (EF) of NH3 from Urea fertiliser (from 21% of N in Urea as NH3-N as per IPCC, to 13.5%) which principally affects emissions to air and therefore atmospheric deposition of N.

- CLE: key changes include emissions projections in line with current legislation in the ECLIPSEv6 emissions database, India-specific emissions projections, projections for human and livestock populations and projections for fertiliser use.

- MTFR: key changes include emissions projections under maximum feasible reduction in the ECLIPSEv6 emissions database, India-specific emissions projections, 100% uptake of fertiliser incorporation to soil (90% reduction in NH3 emissions), adjustments to the N-flow informed by UNECE (2015) (e.g. reduction in N excreted and emission reductions from housing and storage), plus projections for human and livestock populations and projections for fertiliser use (identical to CLE). Further adjustments (reductions) to fertiliser projections were made to reflect increased N retention in the soil after incorporation.

- MTFR+DIET: key changes include adjustment of food consumption data to a recommended daily intake of 2500 kcal per person, projections of consumption of different food groups, plus projections for human and livestock populations and projections for fertiliser use (identical to MTFR). Further adjustments to fertiliser projections were made to match the change in crop areas.

Note that the hydrological characteristics and urban land use datasets are constant across all four scenarios. For more detail, see below.

Nutrient inputs from fertiliser: Inputs of N (both NH4+ and NO3-) and P from the use of synthetic fertilisers were calculated for each scenario using a combination of datasets as follows:

- Baseline: for a given year up to 2020: fertiliser use (“agricultural”) per country per fertiliser type was obtained from FAOSTAT (FAO, 2023) and combined with N and P content as a fraction (FAO, 2023), to estimate total N/P applied per country per year. Data from Nishina et al. (2017) was also linked with FAOSTAT fertiliser data to obtain the fraction of each N fertiliser as NH4+ and NO3-, per fertiliser type per year. An estimated mean application rate of N/P to crops was derived using the total N/P used per country per year, together with the total area of the crop distributions of 32 crops in 2010 (Monfreda et al., 2008; Ramankutty et al., 2008). For each of the 32 crop distributions, an N application rate was applied either from crop-specific fertiliser application rates of 26 crops (Cui et al., 2021) or using the pre-calculated country mean application rate for that year. Crop-specific fertiliser application rates were based on spatially distributed data from Cui et al. (2021), which were further gap-filled via inverse-distance weighting to ensure full coverage against crop distributions. Spatial fertiliser application timing, given as day of year, for 25 crops (SAGE, Sacks et al., 2010), with 6 crops having two application dates in some locations, were applied to all 32 crops. As per Nishina et al. (2017), crop application dates d from Sacks et al. (2010) were further split to d-7 and d+38 in a 70:30 ratio of the total application rate. The resulting day of year N/P application per crop data is converted to monthly and summed across all crop types and, as the type of fertiliser to type of crop is not known, scaled to match the annual total N/P use as determined in FAOSTAT (FAO, 2023). The country level split of NH4+ and NO3- is then applied to the N fertiliser data. Finally, in the case of India, specific information on state-level usage of fertiliser by type, state-level crop distribution by type and some crop-specific fertiliser application rates were incorporated (IARI, 2016). For nitrogen emissions-related datasets used by the South Asia Hydrological model (HMF-SA), 10% of nitrogen fertiliser from runoff was assumed to reach freshwaters.

- CLE: fertiliser projections of total N/P were taken from FAO (2022), meaning that fertiliser type information from 2021 to 2035 was fixed as per data in 2020, per country per year. For nitrogen emissions-related datasets used by the South Asia Hydrological model (HMF-SA), 10% of nitrogen fertiliser from runoff was assumed to reach freshwaters.

- MTFR: further adjustments to national total fertiliser usage as projected in CLE were made to consider a) increased retention of N in spread manures and b) increased retention of N from improved placement of fertiliser products into the soil. Firstly, additional N applied via manures via better manure management, compared to the baseline, is removed from the projected usage of N from fertiliser, to maintain the total. Secondly, placement of synthetic fertiliser was deemed to be done entirely by deep injection, a method that reduces NH3 losses via volatilisation from the surface by up to 90% (UNECE, 2015). An assumed consequence of reduced NH3 loss via deep injection is the reduced need to use as much synthetic N to begin with and therefore N saved was further subtracted from the total N use per country per year. For nitrogen emissions-related datasets used by the South Asia Hydrological model (HMF-SA), 15% of nitrogen fertiliser from runoff was assumed to reach freshwaters.

- MTFR+DIET: here an adjustment of food consumption data to a recommended daily intake of 2500 kcal per person, divided among 14 food groups (Willet et al., 2019) was undertaken. For this scenario, food consumption was adjusted to meet total recommended caloric intake of 2500 kcal - the consumption of pulses, vegetables, and fruits was increased to meet levels recommended by the EAT-Lancet Commission (Willet et al., 2019), with meat and dairy consumption unchanged. Non-millet cereals were reduced, with cereal consumption reductions of up to 30% (Afghanistan) to 60% (Bangladesh). Millet and sorghum consumption was increased from 12.5% to 25% of consumed cereals, to reflect a substitution of healthier whole-grain cereals for white rice. Dietary change was then applied to crop area and N fertiliser requirements. Crop areas of cereals (millet, rice, barley, canary seed, wheat, maize, oats, and sorghum) and pulses (beans, green beans, broad beans, lentils, chickpeas, cowpeas, peas, pigeon peas and green peas) were scaled in line with recommendations at the total domain area, not the country level, due to potentially unrealistic changes in crop areas. N fertiliser application (as calculated in the MTFR scenario) was further adjusted to reflect the altered areas of crops in each country. For nitrogen emissions-related datasets used by the South Asia Hydrological model (HMF-SA), 15% of nitrogen fertiliser from runoff was assumed to reach freshwaters.

Atmospheric N deposition:

Table 2. Sources of emissions data for different pollutants and scenarios for atmospheric deposition modelling. CLE = Current Legislation, MTFR = Maximum Technically Feasible Reductions, MTFR+Diet = MTFR with changes in dietary consumption.

| Component & Model | N Pollutant(s) or Inputs | Baseline | CLE | MTFR | MTFR+DIET |
| --- | --- | --- | --- | --- | --- |
| Atmosphere EMEP model | NH3 (agricultural) | India: Wang et al. (2025), Tomlinson et al., (2026) | India: This study, Wang et al. (2025). | India: This study, Wang et al. (2025). | India: This study Wang et al. (2025). |
|  |  | Other domain nations: Wang et al. (2025), Tomlinson et al., (2026) | Other domain nations: This study, Wang et al. (2025). | Other domain nations: This study, Wang et al. (2025).) | Other domain nations: This study, Wang et al. (2025). |
|  |  | Globe: EDGARv6.1 | Globe: Relative changes from ECLIPSEv6 - CLE | Globe: Relative changes from ECLIPSEv6 - CLE (no further changes) | Globe: Relative changes from ECLIPSEv6 - CLE (no further changes) |
|  | Non-NH3 (all sectors) NH3 (non-agricultural) | India: SMoG | India: SMoG | India: Relative changes from ECLIPSEv6 - MTFR | India: Relative changes from ECLIPSEv6 - MTFR |
|  |  | Other domain nations: EDGARv6.1 | Other domain nations: Relative changes from ECLIPSEv6 - CLE | Other domain nations: Relative changes from ECLIPSEv6 - MTFR | Other domain nations: Relative changes from ECLIPSEv6 - MTFR |
|  |  | Globe: EDGARv6.1 | Globe: Relative changes from ECLIPSEv6 - CLE (no further changes) | Globe: Relative changes from ECLIPSEv6 - CLE (no further changes) | Globe: Relative changes from ECLIPSEv6 - CLE (no further changes) |

- Baseline: Atmospheric deposition for 2015 was generated via the EMEP atmospheric model (see Wang et al., 2025). The anthropogenic emission inventory (NOx, SO2, CO, NMVOCs, PM, and NH3) for the 2015 base run were based upon the EDGARv6.1 (Crippa et al., 2018; Janssens-Maenhout et al., 2019). However, across South Asian countries emissions of agricultural NH3 were recalculated by using updated data, including detailed crop data, livestock distribution maps and sub-national activity information. Furthermore, for India specifically, emissions of non-NH3 air pollutants (NOx, SO2, CO, NMVOCs, PM) were replaced with a national inventory (‘SMOG’: Venkataraman et al., 2018; Sadavarte & Venkataraman, 2014; Pandey et al., 2014). More information about this 2015 baseline emission inventory was detailed in Wang et al. (2025) and Tomlinson et al. (2026). For data from 1980 to 2014, gridded data from ISIMIP3b dataset were used to rescale 2015 EMEP model output data. EMEP model output for total N deposition, for both reduced (NHx) and oxidised (NOy) N, was extracted on a monthly time resolution and reprojected to 0.1˚ x 0.1˚ (g N m-2). Total N deposition data from ISIMIP, for both reduced (NHx) and oxidised (NOy) N, were extracted on a monthly basis and summarised to national totals of N deposition per month per year per country. These data were converted to relative scaling factors (with ISIMIP 2015 data as the index year) and applied to the EMEP model output.

For data from 2016 to 2035, a combination of new EMEP model runs for 2030 and interpolation/extrapolation was used. New emissions surfaces for NOx, particulate matter, NH3, SO2 and VOCs in 2030 were made under conditions of the 3 scenarios, including:

- CLE: The only change to emission source strengths was the assumption of the complete uptake of neem-coated Urea. The new emission factor for NH3 from Urea fertiliser is 15.6% of N emitted as NH3, a 13% increase on the baseline (Bhatia et al., 2023; Mark A. Sutton, personal communication, 2024) (already reduced from 21% to 13.8%, see Tomlinson et al., 2026). Sectoral aggregations were very similar in both EDGAR and ECLIPSE, though aviation, offroad machinery and road transport were aggregated to one ‘transport’ sector in both instances, to avoid some discrepancies due to underlying methods. In some instances, where an EDGAR emission estimate for a given pollutant, sector and country were an order of magnitude larger than the equivalent ECLIPSE estimate (due to the methodological inputs of either dataset), plus the ECLIPSE relative change from the baseline to CLE was more than 3 (i.e. a large predicted relative change in emissions), the resultant scaling factor was reset to 2. This avoided very large changes to the EDGAR data, and was predominantly limited to NH3, NMVOCs and SO2 in energy production, domestic combustion and industry in a small number of countries (<0.5% of outcomes). Cell level relative changes were not applicable due to incoherent spatial distributions and emission estimates between EDGARv6.1 and ECLIPSEv6. For the emissions of non-NH3 pollutants specifically in India, future projections of NOx, CO, SO2, NMVOCs and PM were provided by the SM0G inventory at ‘conservative’ and ‘ambitious’ levels (Venkataraman et al., 2018; Sharma et al., 2023), which were translated to CLE ambition sector-by-sector (Venkataraman, personal communication, 2022). For both NHx and NOy deposition estimates from the EMEP model, cell values between the 2015 EMEP output and the 2030 MTFR scenario output were linearly interpolated, and then extrapolated out to 2035, to provide the adequate time series.

- MTFR: Emissions changes were informed as per Table 2 and adjustments were made to the baseline N-flow approach outlined in Tomlinson et al. (2026) to reflect a suite of technological interventions in the livestock management sector. These included the following measures/reductions at each stage of the N-flow, informed by UNECE (2015) except where stated:

  - Livestock diets. A 10 % reduction in N excreta per animal was assumed to account for improved dietary intake (Adrian Leip, personal communication, 2023).

  - Housing emissions: Pig & poultry: An 80 % reduction to housing NH3 emissions for pig and poultry.

  - All other livestock (not pig and poultry): A 33 % NH3 emission reduction in housing emissions for all other livestock (not pig and poultry).

  - Storage emissions:  Slurry Storage: A 90 % reduction to NH3 emissions from the storage of slurries. N2 emissions from the storage of slurries remains unchanged.

  - Solid storage: A 90 % reduction in NH3 emissions associated with the storage of solid manures.

  - Spreading emissions: A 90 % reduction in NH3 emissions associated with the spreading of manures/slurries is assumed to reflect the immediate incorporation of all manures/slurries applied.

  - With regards the application of synthetic fertilisers, further adjustments to national total usage as projected in CLE were made to consider a) increased retention of N in spread manures and b) increased retention of N from improved placement of fertiliser products into the soil.  This additional N applied via manures, compared to the baseline, is removed from the projected usage of N from mineral fertiliser (FAO, 2018), to maintain that total. This additional N in spread manures displaced 1.5% (India) to 37% (Afghanistan and Bhutan) of synthetic N used country-1. Placement of synthetic fertiliser was deemed to be done entirely by deep injection, a method that reduces NH3 losses via volatilisation from the surface by up to 90% (UNECE, 2015). An assumed consequence of reduced NH3 loss via deep injection is the reduced need to use as much synthetic N to begin with – i.e. less fertiliser is required to provide the same TAN. To estimate this, NH3 emissions from fertiliser with deep injection were subtracted from NH3 emissions from fertiliser without deep injection (fertiliser type-1 country-1), and the difference was deemed to be N saved and subtracted from the country total N use (following the reduction for N retention in manures).

  - For the emissions of non-NH3 pollutants specifically in India, future projections of NOx, CO, SO2, NMVOCs and PM were not available in the SMoG inventory for an MTFR scenario, so data regarding relative sectoral change from ECLIPSEv6 was used (Table 2).

  - For both NHx and NOy deposition estimates from the EMEP model, cell values between the 2015 EMEP output and the 2030 MTFR scenario output were linearly interpolated, and then extrapolated out to 2035, to provide the adequate time series.

- MTFR+DIET: Dietary change, outlined above, was applied to crop area and N fertiliser requirements. Crop areas of cereals (millet, rice, barley, canary seed, wheat, maize, oats, and sorghum) and pulses (beans, green beans, broad beans, lentils, chickpeas, cowpeas, peas, pigeon peas and green peas) were scaled in line with recommendations at the total domain area, not the country level, due to potentially unrealistic changes in crop areas. N fertiliser application (as calculated in the MTFR scenario) was further adjusted to reflect the altered areas of crops in each country. Finally, crop residue emissions were scaled in line with changes to crop areas. In total, N fertiliser use was reduced by ~22% across the whole domain, in comparison to the MTFR scenario

Hydrological characteristics:  The drainage direction dataset (drainage_dir) consists of outflow drainage directions based on  the Dominant River Tracing (DRT) flow directions dataset (Wu et al., 2012; Wu et al., 2011) –  these data were extended to fit with the SANH domain and include hand corrections in areas such as the Bangladesh delta. The drainage directions use the D8 method (Jenson & Domingue, 1988), which assumes eight drainage directions (see Units section). Note that a flow direction value is not available for every pixel in some semi-arid regions (e.g. endorheic basins in Afghanistan). Such grid-cells are considered hydrological sinks, for which upstream rivers drain to evaporating lakes or wet areas instead of flowing downstream.  The catchment area dataset (catchment_area) represents the number of upstream 0.1° × 0.1° grid cells draining to each cell including the cell itself. This dataset can be useful to identify the location of, and area draining to, gauged/ungauged catchments and is derived from the drainage direction grid using commonly available GIS tools.  The coastal outflow dataset (coastal_outflow) identifies coastal land grid-cells for which outflows go to the sea rather than inland – this dataset can be combined with published spatial grids of SANH freshwater flows/fluxes to enable users to estimate flows/fluxes to sea and is consistent with the drainage direction grid.

Note that all three hydrological datasets are assumed constant in time and do not change for different scenarios.

Urban land use: Global land cover for the year 2015 was obtained from the European Space Agency (ESA) Climate Change Initiative (CCI). These data were aggregated from 0.0028˚ x 0.0028˚ (~300m) to a 0.1˚ x 0.1˚ grid and converted from a discrete cell classification to a fraction of total cell area. Urban land cover was extracted using the “Urban” class, which was originally defined using the United Nations Food and Agriculture Organization’s (UN FAO) Land Cover Classification System (LCCS), and includes both the Global Human Settlement Layer and the Global Urban Footprint.

Note that urban land-use is assumed constant in time and does not change for different scenarios.

Human population: For data from 1980 to 2014, gridded population data (0.083˚ x 0.083˚) from the HYDE3.2.1 dataset were used (aggregated to 0.1˚ x 0.1˚). Available years in that dataset were 1980, 1990 and annually from 2000 to 2014; cell values were linearly interpolated for missing years. HYDE data were converted into scaling values, per cell, with HYDE 2015 data as the index year (for which all scaling values = 1). The resulting gridded scaling data were applied to the GPW data of 2015. For data from 2016 to 2035, FAO projections (FAO, 2022) from 2012 to 2050 were used (country total). Relevant available years in that dataset were 2012, 2030 and 2035; country total population was linearly interpolated between values. The data was converted to scaling values, per country, with 2015 as the index year. The same scaling value was used for every cell in a country within the domain. Original methods of data generation are outlined in the references in the Summary section. With regards the South Asia Hydrological model (HMF-SA), sewage treatment was increased in the MTFR and MTFR+DIET scenarios so as to reduce the proportion of untreated sewage by 50% in line with SDG6.3: “halving the proportion of untreated wastewater”, and 80% of ammonium in treated sewage was recovered (i.e. it was removed before reaching freshwaters).  Furthermore, in the MTFR+DIET scenario human nitrogen emissions in sewage were reduced by 15%.

Livestock populations: For data from 1980 to 2009, FAOSTAT data (FAO, 2023) was obtained for "Buffaloes", "Cattle", "Chickens", "Ducks", "Goats", "Horses", "Pigs" and "Sheep", which represented total population per annum. The data were converted to scaling values, per country, with 2010 as the index year. The same scaling value was used for every cell in a country, for a given livestock category, within the domain. This method was applied from 2011 to 2015. For data from 2016 to 2035, FAO projections from 2012 to 2050 were used (country total). Relevant available years in that dataset were 2012, 2030 and 2035; total livestock populations per country per category were linearly interpolated between values. The data was converted to scaling values, per country and per livestock type, with 2015 as the index year. The same scaling value was used for every cell in a country within the domain. As chickens, ducks and horses did not exist in the projection data, “poultry” was used to represent chickens and ducks, and buffalo were used to represent horses. Original methods of data generation are outlined in the references in the Summary section.

Note that in the Future of Food and Agriculture (FAO, 2022) data, there were no future projection data available for Bhutan so the projections for the “Rest of South Asia” region was used to infill missing values.

# Quality control

- Data were checked for extreme values and presence/locations of NA values.

- Population and fertiliser use totals were summed for a subset of countries and sense checked against original scaling values.

- The atmospheric model output underwent its own QAQC procedure. See https://github.com/NERC-CEH/EMEP_QAQC for details.

# Miscellaneous

Crop distributions: The “Earthstat” data collection is openly available, licensed under the Creative Commons Attribution 4.0 International License.

Dominant River Tracing (DRT) flow direction data is openly available, licensed under the Creative Commons Attribution 4.0 International License.

Fertiliser application rates: The data in Cui et al. (2021) are licensed under a Creative Commons Attribution 4.0 International License (CC BY 4.0).

Fertiliser composition and placement: The data in Nishina et al. (2017) are licensed under the Creative Commons Attribution 3.0 License.

Human Population: The “Gridded Population of the World” (GPW) data collection is openly available, licensed under the Creative Commons Attribution 4.0 International License.

Historic N deposition: The Inter-Sectoral Impact Model Intercomparison Project (ISIMP) data are licensed under a Creative Commons Attribution 4.0 International License (CC BY 4.0).

Livestock: The "Gridded Livestock of the World" (GLW) datasets are licensed under a Creative Commons Attribution 4.0 International License (CC BY 4.0).

Urban land use: The land cover data produced by the European Space Agency (ESA) Climate Change Initiative (CCI) (Copernicus Climate Change Service, 2019) are licensed under a Creative Commons Attribution 4.0 International License (CC BY 4.0).

# Marine modelling with NEMO-ERSEM

Note regarding marine model used: Data produced by the EMEP atmospheric chemistry transport model and the South Asia Hydrological model (HMF-SA) was used by a marine model called NEMO-ERSEM. Inputs from the open ocean were included by imposing nitrogen concentration at the open boundary of the model equal to the climatological cycle described in the World Ocean Atlas 2018. Direct emissions of nitrogen in the marine environment from maritime and marine activities were neglected because negligible at the scale of the model.

# References

Bhatia, A., Cowan, N. J., Drewer, J., Tomer, R., Kumar, V., Sharma, S., Paul, A., Jain, N., Kumar, S., Jha, G., and Singh, R.: The impact of different fertiliser management options and cultivars on nitrogen use efficiency and yield for rice cropping in the Indo-Gangetic Plain: Two seasons of methane, nitrous oxide and ammonia emissions, Agr. Ecosyst. Environ., 355, 108593, https://doi.org/10.1016/j.agee.2023.108593, 2023.

Center for International Earth Science Information Network (CIESIN): Documentation for the Gridded Population of the 520 World, Version 4 (GPWv4), Revision 11 Data Sets. Palisades NY: NASA Socioeconomic Data and Applications Center (SEDAC), published by Columbia University. https://doi.org/10.7927/H45Q4T5F, 2018.

Copernicus Climate Change Service, Climate Data Store: Land cover classification gridded maps from 1992 to present derived from satellite observation. Copernicus Climate Change Service (C3S) Climate Data Store (CDS). DOI: 10.24381/cds.006f2c9a, 2019.

Crippa, M., Guizzardi, D., Muntean, M., Schaaf, E., Dentener, F., van Aardenne, J. A., Monni, S., Doering, U., Olivier, J. G. J., Pagliari, V., and Janssens-Maenhout, G.: Gridded emissions of air pollutants for the period 1970–2012 within EDGAR v4.3.2, Earth Syst. Sci. Data, 10, 1987–2013, https://doi.org/10.5194/essd-10-1987-2018, 2018.

Cui, X. et al. Global mapping of crop-specific emission factors highlights hotspots of nitrous oxide mitigation. Nature Food 2, 886–893, https://doi.org/10.1038/s43016-021-00384-9, 2021.

Food and Agriculture Organization of the United Nations (FAO). The future of food and agriculture – Drivers and triggers for transformation. The Future of Food and Agriculture, no. 3. [Rome], 2022.

Food and Agriculture Organization of the United Nations (FAO). FAOSTAT Statistical Database. [Rome]: FAO, 2023.

Gilbert, M., Nicolas, G., Cinardi, G, van Boeckel, T.P., Vanwambeke, S.O., William Wint, G.R., and Robinson, T.P.: Global distribution data for cattle, buffaloes, horses, sheep, goats, pigs, chickens and ducks in 2010. Sci Data 5, 180227, https://doi.org/10.1038/sdata.2018.227, 2018.

Indian Agriculture Research Institute (IARI): Project completion report submitted by Indian Agriculture Research Institute to 585 BASF, India. (unpublished), 2016.

Janssens-Maenhout, G., Crippa, M., Guizzardi, D., Muntean, M., Schaaf, E., Dentener, F., Bergamaschi, P., Pagliari, V., Olivier, J. G. J., Peters, J. A. H. W., van Aardenne, J. A., Monni, S., Doering, U., Petrescu, A. M. R., Solazzo, E., and Oreggioni, G. D.: EDGAR v4.3.2 Global Atlas of the three major greenhouse gas emissions for the period 1970–2012, Earth Syst. Sci. Data, 11, 959–1002, https://doi.org/10.5194/essd-11-959-2019, 2019.

Jenson, S. K., & Domingue, J. O.: Extracting topographic structure from digital elevation data for geographic information-system analysis. Photogrammetric Engineering and Remote Sensing, 54, 1593-1600, 1988.

Klein Goldewijk, K., Beusen, A., Doelman, J., and Stehfest, E.: Anthropogenic land use estimates for the Holocene; HYDE 3.2, Earth System Science Data, 9, 927-953, 2017.

Klimont, Z., Kupiainen, K., Heyes, C., Purohit, P., Cofala, J., Rafaj, P., Borken-Kleefeld, J. and Schöpp, W.: Global anthropogenic emissions of particulate matter including black carbon. Atmos. Chem. Phys., 17, 8681-8723. https://doi.org/10.5194/acp-17-8681-2017, 2017.

Monfreda, C., Ramankutty, N., and Foley, J.A.: Farming the planet: 2. Geographic distribution of crop areas, yields, physiological types, and net primary production in the year 2000, Global Biogeochem. Cycles, 22, GB1022, https://doi:10.1029/2007GB002947, 2008.

Nishina, K., Ito, A., Hanasaki, N., and Hayashi, S.: Reconstruction of spatially detailed global map of NH4+ and NO3− application in synthetic nitrogen fertilizer. Earth Syst. Sci. Data, 9, 149–162, https://doi.org/10.5194/essd-9-149-2017, 2017.

Pandey, A., Sadavarte, P., Rao, A.B., and Venkataraman, C.: Trends in multi-pollutant emissions from a technology-linked inventory for India: II. Residential, agricultural and informal industry sectors. Atmospheric Environment, 99, 341-352, https://doi.org/10.1016/j.atmosenv.2014.09.080, 2014.

Ramankutty, N., Evan, A.T., Monfreda, C., and Foley, J.A.: Farming the planet: 1. Geographic distribution of global agricultural lands in the year 2000, Global Biogeochemical Cycles 22, GB1003, https://doi:10.1029/2007GB002952, 2008.

Sacks, W.J., Deryng, D., Foley, J.A., and Ramankutty, N.: Crop planting dates: An analysis of global patterns. Global Ecology and Biogeography, 19(5), 607-620, https://doi:10.1111/j.1466-8238.2010.00551.x, 2010.

Sadavarte, P., and Venkataraman, C.: Trends in multi-pollutant emissions from a technology-linked inventory for India: I. Industry and transport sectors. Atmospheric Environment, 99, 353-364. https://doi.org/10.1016/j.atmosenv.2014.09.081, 2014.

Sharma, A., Venkataraman, C., Muduchuru, K., Singh, V., Kesarkar A., Ghosh, S., and Dey, S.: Aerosol radiative feedback enhances particulate pollution over India: A process understanding. Atmospheric Environment, 298, 119609. https://doi.org/10.1016/j.atmosenv.2023.119609, 2023.

Tian, H., Yang, J., Lu, C., Xu, R., et al.: The Global N2O Model Intercomparison Project, Bulletin of the American Meteorological Society, 99(6), 1231-1251, https://doi.org/10.1175/BAMS-D-17-0212.1, 2018.

Tomlinson, S.J., Carnell, E.J., Pearson, C., Sutton, M.A., Jain, N., and Dragosits, U.: A framework for gridded estimates of ammonia emissions from agriculture in South Asia, Earth System Science Data, 18(3), https://doi.org/10.5194/essd-18-2349-2026, 2026.

United Nations Economic Commission for Europe (UNECE), Convention on Long-range Transboundary Air Pollution.: Framework Code for Good Agricultural Practice for Reducing Ammonia Emissions. ECE/EB.AIR/129 published by the European Commission, 2015.

Venkataraman, C., Brauer, M., Tibrewal, K., Sadavarte, P., Ma, Q., Cohen, A., Chaliyakunnel, S., Frostad, J., Klimont, Z., Martin, R.V. and Millet, D.B.: Source influence on emission pathways and ambient PM 2.5 pollution over India (2015–2050). Atmospheric Chemistry and Physics, 18(11), 8017-8039, 2018.

Wang, Y., et al.: Response of South Asia PM2.5 pollution to ammonia emission changes and associated impacts on human health, Environment International, 195, 109207, https://doi.org/10.1016/j.envint.2024.109207, 2025.

Willett W, Rockstrom J, Loken B, Springmann M, Lang T, Vermeulen S, et al. Food in the Anthropocene: the EAT-Lancet Commission on healthy diets from sustainable food systems. Lancet. 2019; 393(10170): 447-92, https://doi.org/10.1016/S0140-6736(18)31788-4, 2019.

Wu, H., Kimball, J. S., Mantua, N., & Stanford, J.: Automated upscaling of river networks for macroscale hydrological modeling. Water Resources Research, 47(3). https://doi.org/https://doi.org/10.1029/2009WR008871, 2011.

Wu, H., Kimball, J. S., Li, H., Huang, M., Leung, L. R., & Adler, R. F.: A new global river network database for macroscale hydrologic modeling. Water Resources Research, 48(9). https://doi.org/https://doi.org/10.1029/2012WR012313, 2012.

Crops distributions used from Earthstat, as named in Earthstat: barley, bean, broadbean, cassava, castor,  chickpea, cotton, cowpea, greenbean, groundnut, lentil, linseed, maize, millet, oats, pea, pigeonpea, potato, pulsenes, rapeseed, rice, rye, sesame, sorghum, soybean, stringbean, sugarbeet, sugarcane, sunflower, sweetpotato, wheat, yam

BARLEY, BARLEY2, CASSAVA, COTTON, FRUITS, GROUNDNUT, MAIZE, MAIZE2, MILLET, OILPALM, OTHER, POTATO, RAPESEED, RICE, RICE2, RYE, SORGHUM, SORGHUM2, SOYBEAN, SUGARBEET, SUGARCANE, SUNFLOWER, SWEETPOTATO, VEGETABLES, WHEAT, WHEAT2

Barley, Barley.Winter, Cassava, Cotton, Groundnuts, Maize, Maize.2, Millet, Oats, Oats.Winter, Potatoes, Pulses, Rapeseed.Winter, Rice, Rice.2, Rye.Winter, Sorghum, Sorghum.2, Soybeans, Sugarbeet, Sunfower, Sweet.Potatoes, Wheat, Wheat.Winter, Yams