# Supporting Documentation: Soil Water Assessment Tool (SWAT) modelling: River Derwent, Yorkshire

Dr Janet Richardson, Edge Hill University

# Summary

Erosion risk mapping showing soil erosion potential (tonnes/yr) using the Soil Water Assessment Tool (SWAT) for the Yorkshire River Derwent, UK. The modelled data includes 29 years of weather data (1990-2019). Outputs were validated using SUFI2. This dataset builds on previous modelling using SCIMAP (Richardson et al. 2019).

# Collection / Generation methods

The Soil and Water Assessment Tool (developed by the USDA Agricultural Service; link to SWAT website) was used to simulate streamflow and sediment transport. SWAT is a physically based hydrological and water quality model designed to estimate the impacts of underlying physical factors such as geology and climate, as well as land management practices in complex watersheds. SWAT divides the catchment area into sub-catchments and each of them is further divided into hydrological response units (HRUs), which are defined as areas of land with the same soil, land use and slope that are assumed to behave similarly in the model (i.e., each HRU contributes a unique runoff to the total catchment) (Neitsch et al. 2005). The hydrological routines within SWAT account for precipitation, evaporation, infiltration, plant uptake, surface flow, lateral flow, percolation, channel transmission losses, channel routing and shallow aquifer and deep aquifer recharge. SWAT uses the Soil Conservation Service curve number method (Mishra et al. 2003) to simulate the generation of surface runoff on a daily time-step and the Universal Soil Loss Equation (USLE) (Wischmeier and Smith, 1965) to simulate erosion. Surface runoff can also be simulated at a sub-hourly timestep within the model, but this was not considered for the multi-year simulations undertaken here. The data reported relates to the USLE aspect of the model, as this is the most important for understanding the erosion risk across the catchment and the sediment budget.

The River Derwent catchment was delineated in QSWAT (QSWAT3, version 1.6.0) up to the outlet at the Barmby Barrage. An Ordnance Survey 5-m digital elevation model was resampled to a 20m resolution. A 10m resolution land cover map (LCM 2019) from the Centre for Ecology and Hydrology (CEH) was used for land-use data and was transformed into the land-use codes used within SWAT and resampled to 20m resolution. Soil data were provided by a 10km resolution map of EU soils (Pangos et al. 2012) for the study area and was resampled to 20m resolution.

There are 12 major soil classes represented in the European Soils Map in the River Derwent (Table 1). For analysis, the coverage of peat was removed from the catchment (<0.5% coverage), this is because SWAT cannot process organic material, and relies on the percentages of sand, silt, and clay, with other factors to understand how the soil will respond to a hydrological event.

Table 1 – Soil Types within the River Derwent Catchment

| Soil Code | Soil Type |
| --- | --- |
| GB 8 | Calcaro-Eutric Cambisol |
| GB 14 | Stagno-Eutric Gleysol |
| GB 27 | Stagno-Gleyic Luvisol |
| GB 13 | Stagno-Dystric Gleysol |
| GB 15 | Humic Gleysol |
| GB 61 | Placic Podzol |
| GB 5 | Eutric Cambisol |
| GB16 | Mollic Gleysol |
| GB 30 | Orthic Luvisol |
| GB 12 | Rendzina |
| GB 49 | Gleyo-Eutric Fluvisol |
| GB 23 | Chromic Luvisol |

The European Soils Map was used as the basis for SWAT modelling due to the availability of detailed soil hydrological and physico-chemical properties soil profiles to depth. Due to the large area of the Derwent catchment with relatively low slope, spatial slope data were classified into four ranges in the model (0-0.5%, 0.5-1% and > 1%). The catchment was finally defined by 24 sub-catchments, the HRUs were not categorised (thereby removing certain areas) to ensure the whole catchment was modelled, this resulted in 2984 HRUs due to the complex nature of the catchment.

Weather data including solar radiation, wind speed, relative humidity and maximum and minimum air temperature were obtained from four Met Office weather stations at Flyindales, High Mowthorpe, Kirkby Misperton and Scarborough (Open access MIDAS dataset through the Centre For Environmental Data Analysis: https://catalogue.ceda.ac.uk/uuid/dbd451271eb04662beade68da43546e1/). Data from the years 1990 - 2019 were downloaded and used in the model run. Outputs presented represent the average across the years modelled.

For the initial model run to occur the following steps are undertaken:

- Updating SWAT databases to include user soil information for soils in Derwent Catchment

- Collating weather data and transforming into SWAT input files (~30 years of data for 4 sites)

- Projection of data sources to ensure they are in the same grid co-ordinates

- Resampling of data to 20 m resolution

- Uploading DEM, soil and land-use data to SWAT

- Delineation of hydrological response units

- Integration of weather data to catchment

- Initial run of SWAT model to produce results

After step 8, calibration can occur and observed flow data is integrated.

### Calibration and validation (Quality Control)

Flow gauges across the catchment will be used to calibrate the model and are available from the Environment Agency. The SWAT calibration and uncertainty program (SWAT-CUP) (Abbaspoir et al. 2007) was used for model calibration and validation using a multiple regression system via the SUFI2 method.

The model was calibrated for the years 1992 – 2013 and validated for the years 2013 – 2019. These data periods cover wet years such as 2007, 2012 and 2015/6.

Calibration used the lowest flow gauge site of Buttercrambe which had data coverage for the entire period modelling. Calibration of the model for six variables related to runoff, groundwater infiltration rates and soil properties (SUFI2 using CN2, Alpha BF, GW Delay, GWQMN, Sol_K and Sol_AWC) resulted in a r2 value of 0.58 and a Nash Sutcliffe Score of 0.49. Further calibration could be achieved by integrating in additional values. Calibration resulted in similar peak flows being achieved, important for erosion risk and yield studies. For the data time frame and size of the Derwent catchment, these calibration values were deemed sufficient.

# Nature and units of recorded values

Table 2 shows the attribute table data and explanation of units.

Table 2 – Attribute table of SWAT shapefile

| Column Name | Description | Units |
| --- | --- | --- |
| Subbasin | Numbering associated to the sub-basins of the Derwent, as delineated by SWAT. | Number |
| LANDUSE | Land-use categories used in SWAT. | Landuse Code; Land use. FRSD; Deciduous Forest. FRSE; Evergreen Forest. AGRL; Generic. RNGE; Grasslands/Herbaceous. RNGB; Range Shrubland. RNGE; Range Shrubland. WATR; Grasslands/Herbaceous. WATR; Water. WETN; Emergent/Herbaceous Wetlands. URHD; Urban High Density. URML; Urban Medium Density. |
| SOIL Info | Soil classifications used in SWAT. | Soil Code; Soil Type. GB 8; Calcaro-Eutric Cambisol. GB 14; Stagno-Eutric Gleysol. GB 27; Stagno-Gleyic Luvisol. GB 13; Stagno-Dystric Gleysol. GB 15; Humic Gleysol. GB 61; Placic Podzol. GB 5; Eutric Cambisol. GB16; Mollic Gleysol. GB 30; Orthic Luvisol. GB 12; Rendzina. GB 49; Gleyo-Eutric Fluvisol. GB 23; Chromic Luvisol. |
| SLOPE_BAND | Slope bands classified during SWAT modelling. | % |
| Area_ha | Area of polygons | Ha |
| HRUGIS | Numbering of hydrological response units delineated by model. | Number |
| SEDth | Annual modelled sediment yield. | Tonnes/yr |
| Shape_Leng | Length of shape | Metres |
| Shape_Area | Area of Shape | Metres |

# Quality control

Modelled results were calibrated and validated using flow gauges in the River Derwent catchment using SUFI2 and SWAT-CUP. Calibration of the model for six variables related to runoff, groundwater infiltration rates and soil properties (CN2, Alpha BF, GW Delay, GWQMN, Sol_K and Sol_AWC) resulted in a r2 value of 0.58 and a Nash Sutcliffe Score of 0.4.

# Details of data structure

The modelling output is shown as a shapefile for the River Derwent Catchment. Table 2 details the information for the attribute table and relevant units.

# References

Abbaspour, K.C., Vejdani, M., Haghighat, S., 2007. SWAT-CUP Calibration and Uncertainty Programs for SWAT. Modsim 2007: International Congress on Modelling and Simulation 1603-1609.

Mishra, S K and Singh, V P. 2003. “Soil conservation service curve number (SCS-CN) methodology”. Dordrecht: Kluwer Academic Publishers. ISBN 1-4020-1132-6.

Neitsch, S.L., Arnold, J.G., Kiniry, J.R., Williams, J.R. and King, K.W., 2005. Soil and water assessment tool (SWAT) theoretical documentation. Blackland Research Center, Texas Agricultural Experiment Station, Temple, Texas (BRC Report 02-05).

Panagos P., Van Liedekerke M., Jones A., Montanarella L., “European Soil Data Centre: Response to European policy support and public data requirements”; (2012) Land Use Policy, 29 (2), pp. 329-338. doi:10.1016/j.landusepol.2011.07.003

Richardson, J.C., Hodgson, D.M., Kay, P., Aston, B.J. and Walker, A.C., 2019. Muddying the picture? Forecasting particulate sources and dispersal patterns in managed catchments. Frontiers in Earth Science, 7, p.277.

Wischmeier, W.H. and Smith, D.D. (1965) Predicting rainfall-erosion losses from cropland east of the Rocky Mountains : guide for selection of practices for soil and water conservation. The USDA Agricultural Handbook No. 282, Washington, D.C.