# Title

UK Blanket Bog Water Logger Data 2021-2023

# Summary

This dataset contains the logger data from water loggers installed in five UK blanket bog sites: Moor House, Upper Teesdale, England. Flow Country, Forsinard Flows sites Crocach and Nam Breac, Scotland. Flow Country Braemore and Langwell Estate, Scotland.  Migneint, Snowdonia, Wales. Within each site, loggers are placed at locations identified as near natural, under restoration and drained. The dataset contains recordings converted into water table depth from each logger from 2021 to 2023.

# File format

.csv

# File names and/or naming convention

water_logger_blanket_bog_peatlands_uk_2021_2023.csv

# Nature and units of recorded values

Table 1. Location, logger ID, peat depth and altitude of each logger used in file water_logger_blanket_bog_peatlands_uk_2021_2023

| Site | Region | Logger ID | Peat Depth / cm | Location | Altitude / m |
| --- | --- | --- | --- | --- | --- |
| FORSINARD FLOWS CROCACH AND NAM BRAC (FF)* | Damaged (DAM) | FFDAMA | 355 | N58° 22.624’ W004° 00.487’ | 187 |
|  |  | FFDAMB | 415 | N55° 22.631 W004° 00.486’ | 187 |
|  |  | FFDAMC | 385 | N58° 22.641’ W004° 00.489’ | 187 |
|  | Near Natural (NN) | FFNND | 345 | N58° 23.565’ W004° 00.109’ | 184 |
|  |  | FFNNE | 320 | N58° 23.556’ W004° 00.109’ | 183 |
|  |  | FFNNF | 315 | N58° 23.576’ W004° 00.105’ | 185 |
|  | Under Restoration (RES) | FFRESG | 90 | N58° 23.281’ W003° 59.820’ | 176 |
|  |  | FFRESH | 172 | N58° 23.283’ W003° 59.839 | 177 |
|  |  | FFRESI | 80 | N58° 23.276’ W003° 59.806’ | 174 |
| BRAEMORE | Near Natural (NN) | BraemoreNN1 | 382 | N58 15’ 40.6’’ W003 40’ 26.3’’ | 296 |
|  |  | BraemoreNN2 | 410 | N58° 15’ 40.5’’ W003 40’ 27.8’’ | 296 |
|  |  | BraemoreBaro2 | 382 | 58° 15’ 40.5’’N 003° 40’ 26.3’’W | 294 |
| LANGWELL (MORVEN, SCARABEN, STRATHY, WAG) | Damaged (DAM) | MorvenDAM1 | 105 | 58° 12’ 42.5’’N 003° 42’ 56.3’’W | 265 |
|  |  | MorvenDAM2 | 115 | 58° 12’ 44.8’’N 003° 43’ 00.1’’W | 265 |
|  | Under restoration (RES) | ScarabenRES1 | 378 | N58° 12’ 06.0’’N 003° 33’ 31.6’’ | 233 |
|  |  | ScarabenRES2 | 382 | N58° 12’ 06.8’’N 003° 33’ 32.6’’ | 235 |
|  |  | StrathyRES1 | 123 | N58° 11’ 33.0’’N 003° 34’ 43.9’’ | 143 |
|  |  | StrathyRES2 | 111 | N58° 11’ 32.1’’N 003° 34’ 45.1’’ | 140 |
|  |  | StrathyBaro1 | 111 | N58° 11’ 33.2’’N 003° 34’ 45.2’’W | 140 |
|  |  | WagRES1 | 224 | N58° 12’ 24.4’’N W003 41’ 10.5’’ | 268 |
|  |  | WagRES2 | 205 | N58° 12’ 25.6’’N 003° 41’ 10.8’’W | 268 |
| MIGNEINT (Mig) | Near natural (NN) | MigNN1 | 406 | 52° 58’ 15.7’’N 003° 49’ 00.8’’W | 458 |
|  | Damaged (DAM) | MigDAM1 | 153 | 52° 58’ 09.5’’N 003° 49’ 00.5’’W | 459 |
|  |  | MigDAM2 | 142 | 52° 58’ 09.4’’N 003° 49’ 00.6’’W | 455 |
|  | Under Restoration (RES) | MigRES1 | 191 | 52° 58’ 10.6’’N 003° 48’ 57.2’’W | 456 |
|  |  | MigRES2 | 198 | 52° 58’ 10.6’’N 003° 48’ 57.2’’W | 455 |
|  |  | MigBaro1 | 198 | 52° 58’ 10.6’’N 003° 48’ 57.1’’W | 455 |
| MOOR HOUSE (MH) | Near natural (NN) | MHNN1 | 190 | 54° 41’ 38.6’’N 002° 22’ 32.2’’W | 556 |
|  |  | MHNN2 | 124 | 54° 41’ 38.1’’N 002° 22’ 30.1’’W | 554 |
|  |  | Baro1 | 124 | 54° 41’ 38.1N 002° 22’ 30.0’’W | 555 |
|  | Damaged (DAM) | MHDAM1 | 144 | 54° 41’ 55.5’’N 002 22’ 56.8’’W | 550 |
|  |  | MHDAM2 | 186 | 54° 41’ 53.3’’N 002° 22’ 56.6’’W | 550 |
|  | Under restoration (RES) | MHRES1 | 212 | 54° 41’ 31.4’’N 002° 22’ 56.2’’W | 561 |
|  |  | MHRES2 | 242 | 54° 41’ 30.1’’N 002° 23’ 02.5’’W | 565 |

*Forsinard Flows Barologger data provided by RSPB using their logger. Unknown location.

Data table codes and adjustment calculations: Contained within the data sheet the loggers are listed by site code explained in the table above. The temperature and pressure of the logger and associated barologger are listed in mmHg.

Table 2. Description of variables in water_logger_blanket_bog_peatlands_uk_2021_2023.csv

| Variable/Column header | Description | Units | Vocabulary URL (if applicable) |
| --- | --- | --- | --- |
| ID | Peatland location name and health status code |  |  |
| Site | Peatland location name |  |  |
| Region | Health status as defined above |  |  |
| Date_time |  |  |  |
| Temperature_Logger | Temperature of the loggers | oC |  |
| Temperature_Baro | Temperature of the barologger | oC |  |
| Pressure_Logger | Pressure measurements of loggers | mmHg |  |
| Pressure_Baro | Pressure measurements of the barologger | mmHg |  |
| Pressure_Baro_Adj | Logger pressure adjusted for altitude. Atmospheric pressure decreases ~ -0.0875 mmHg per m in altitude (calculated manually using https://www.sensorsone.com/altitude-pressure-units-conversion/) | mmHg |  |
| mmH2O | mmHg to mmH20 using constant 13.5951. | mmH20 |  |
| A | Dipwell lip to logger mount | mm |  |
| B | Dipwell lip to peat surface | mm |  |
| C | Length of logger wire to bottom of sensor | mm |  |
| E | C - B + A | mm |  |
| WTD | WTD = E – mmH2O | mm |  |

# Spatial coverage
Locations of each logger provided in the Table 1.

# Temporal coverage and resolution
The data was collected between 2021 and 2023 on an hourly basis.

# Collection/Generation/Transformation methods

Installation and data collection: Each In-situ Rugged Troll 100 data logger is hung from a nail or screw at the top of a perforated PVC dipwell tube that is cut to size depending on the peat depth. To prevent peat encroachment into the tube, the tube is covered by a tight. A wire mesh is secured to the top. The tube height above the peat surface is recoded as well as the length of the wire from eye to base of the logger.  An additional barologger is also installed in each site, apart from Forsinard Flows where the RSPB’s barologger data was used.

Data is collected by manually downloading using a docking station using In-situ software.

Data was processed using script located at https://github.com/NBellGroup/water-loggers/tree/main