# Supporting documentation to dataset: Characterization data of recyclable waste on Santa Cruz Island, Galápagos, 2024

# Summary

This dataset presents information on waste characterized from the recycling flow on Santa Cruz island, Galápagos. Characterization was conducted between 19 and 25 November 2024. The process included recyclable waste classification, counting, weighing and polymer identification.  The dataset contains the category and subcategory assigned to each item, the identified polymer type and additional comments, including brand information when available. The data were collected to explore alternatives for reducing inland plastic waste through a circular economy approach. This work is part of a NERC funded project (NE/V005448/1), that aims to reduce plastic in the Galápagos Islands and the Eastern Pacific.

# File format, file name and structure

The dataset consists of a two CSV file entitled “Database_recycling_all” and “Database_recycling_polymer_ID”.

The CSV file: “Database_recycling_all” contains 6 columns. Column A (“Sample_no”) identifies the recyclable waste sample characterized.  Column B (“Sample_date”) records the date on which waste characterization was conducted. Column C (“Category”), indicates the classification category assigned to each item, a total of 9 categories are included (construction and demolition material, food plastic, glass, metal, other, other plastic, paper, personal care products and plastic fragments). Column D (“Subcategory”) corresponds to the subcategories within each category. Column E (“Quantity_number”) records the number of items identified, and Column F (“Weight_g”) records the total weight of the items in grams. Additional details for selected variables are provided in Table 1

Table 1. Description of variables in “Database_recycling_all”.

| Variable/Column | Description | Units |
| --- | --- | --- |
| Sample_no | Four samples were collected and characterize over three sampling days (sample_date). Samples R1 and R2 were collected the same day (19/11/2024) and consisted of a total of 8 bags of recyclable waste. Sample R3 was collected on the 22/11/2024, consisting of 8 bags of recyclable waste while sample R4 was taken the 25/11/2024 and also consisted of 8 bags. For each sample, the collected bags were thoroughly mixed and one subsample was then extracted for characterization. | - |
| Quantity_number | Corresponds to the number of items found in each sample for each category and subcategory | number |
| Weight_g | Corresponds to the weight of items found in each sample for each category and subcategory | g |

The CSV file: “Database_recycling_polymer_ID” consists of 6 columns. Column A (“Sample_no”) corresponds to the recyclable waste sample characterized, consistent with the CSV file: “Database_recycling_all”. Column B (“Item_No”), identifies the specific item for which the polymer type was determined. Column C (“Category”) and column D (“Subcategory”), indicate the classification assigned to each item, following the same categorization system used in “Database_recycling_all”.  Column E (“PlasTELL_ID”) records the abbreviated polymer name identified by a portable plastic identification device (PlasTell NIR plastics scanner). Column F (“Comments”) identifies any relevant information marked in the waste packaging.

Polymer types not identified by PlasTELL are labeled as NI, while missing data values are identified by NA.

# Spatial coverage, temporal coverage and resolution

Recycling waste from Santa Cruz Island located in the Galápagos Archipelago was characterized over three sampling days (19/11/2024, 22/11/2024, 25/11/2024).  The sampling covered the four waste collection routes on the island. Polymer analysis was conducted following waste characterization on the same day.

# Sampling, collection and analysis

Waste characterization was performed at the municipal recycling centre. On the island, four waste collection routes collect both recyclable waste and waste destined directly for the landfill. To characterize the recyclable waste, 2 bags per route were randomly selected by the municipality staff, resulting in a total of 8 bags per sampling event. The 8 bags were weighed and then emptied onto a tarpaulin. The material was thoroughly mixed and divided in four quadrants, three quadrants were discarded and the remaining quadrant was characterised. Characterization involved sorting the material into predefined categories, counting individual items and weighing each category.  Hazardous and sanitary risk waste was removed prior to characterization. Plastics were separated and the polymer type identified using a portable plastic identification device (PlasTell NIR plastics scanner, desktop version, Matoha Instrumentation Ltd.) equipped with a transmittance-reflectance NIR spectroscopy, 1550-1950 nm - 2NIR lamps built-in.

# Quality control

All equipment was thoroughly cleaned before each use to avoid contamination and ensure accurate polymer identification.  The database was carefully reviewed to identify and correct any errors.

# Miscellaneous

This data is used in a manuscript which is in the process to be published.