Groundwater levels, salinity, pH, and rainfall on the Indian Ocean island of Karainagar, Sri Lanka, 2019–2024

Inthirakumaran Karthiga1,2, Ponnambalam Rameshwaran2, Balachandran Ketheesan1, and Nadarajah Sriskandarajah3

1Department of Civil Engineering, Faculty of Engineering, University of Jaffna, Kilinochchi, Sri Lanka

2UK Centre for Ecology and Hydrology, Wallingford, Oxfordshire, OX10 8BB, UK

3Division of Environmental Communication, Department of Urban and Rural Development, Swedish University of Agricultural Sciences, Uppsala, Sweden

## Summary

This dataset includes measurements of groundwater levels, salinity, and pH collected from 36 dug wells, along with rainfall data, on the Indian Ocean Island of Karainagar, Sri Lanka, from 2019 to 2024. Groundwater levels and well dimensions (kerb height, diameter/width, and bottom elevation) were measured using a measuring tape. The locations of the dug wells and ground elevations relative to Mean Sea Level (MSL) were determined using a Global Positioning System (GPS) and land surveys. Salinity (measured as electrical conductivity, EC) and pH were recorded using a Hach HQ40D multi-parameter meter. Daily rainfall data were obtained from records maintained by the Agrarian Services Centre (ASC) in Karainagar. Further details of this research are provided in Karthiga et al. (2022) and Karthiga et al. (2023).

Figure 1 Locations of thirty-six dug wells (K1 to K36) and the Agrarian Services Centre (ASC) rain gauge on Karainagar Island, along with the island’s location in Northern Sri Lanka (OpenStreetMap).

## The Study Area

The study area, Karainagar - previously known as Karaitivu (tivu meaning "island"), is one of seven islands located off the coast of Jaffna in northern Sri Lanka. This rectangular island is bounded on three sides by the Palk Strait, while the fourth side faces the Jaffna Lagoon and is connected to the mainland by a causeway (Figure 1). Karainagar covers an area of approximately 22 km², including inland water bodies, and measures about 6 km from north to south and about 4 km from east to west. Its geographic coordinates range from 9.702° to 9.768° N latitude and from 79.847° to 79.904° E longitude.

## Methodology

Thirty-six dug wells, labelled K1 to K36 (Figure 1), used for agricultural, household, Kovil (Temple), and public purposes, were inventoried. This facilitated the monitoring of groundwater levels and key physicochemical parameters, specifically salinity (measured as electrical conductivity, EC) and pH, on Karainagar Island. In the  pilot phase of the study, which began in November 2019, only 20 dug wells were included (Anojan et al., 2020). An additional 16 wells were incorporated in April 2020, bringing the total to 36 representative dug wells. Photographs of all 36 dug wells are provided in Appendix A.

Groundwater levels and dug well dimensions—including kerb height, diameter/width, and bottom elevation—were measured using a measuring tape. Figure 2 illustrates the parameters used to describe the dug well dimensions. The locations of the dug wells and ground elevations, referenced to Mean Sea Level (MSL), were determined using a Global Positioning System (GPS) and land surveys. Electrical conductivity (EC) and pH were measured using a Hach HQ40D multi-parameter meter. Daily rainfall time series data were obtained from the records of the Agrarian Services Centre (ASC) in Karainagar. The ASC has its own procedures for the continuous monitoring and processing of rainfall data. The measured rainfall on a given day refers to the amount of precipitation recorded over a 24-hour period, from 9 a.m. on that day to 9 a.m. the following day.

Figure 2 Schematic of a dug well.

## Data

The water year is defined as a 12-month period beginning in October and ending in September of the following year (e.g., October 1, 2021, to September 30, 2022, corresponds to water year 2021–22). This timeframe aligns with the onset of the main, or Maha, rainy season and thus the primary cropping season. The dataset includes all data on groundwater levels, EC, pH, and rainfall over five water years: 2019–20, 2020–21, 2021–22, 2022–23, and 2023–24. All dug wells were monitored weekly during the main rainy season and monthly during the dry season in the water years 2020–21 and 2021–22.

Thirty-six dug well locations and their dimensions, purposes, and water usages, along with time series data for groundwater levels, EC, pH, and rainfall for Karainagar, are provided in CSV format. Table 1 shows the file naming conventions. Table 2 provides the units or descriptions of these variables. Tables 3 and 4 present the purposes of the dug wells and their current water usage, respectively.

Table 1. File naming convention.

| File type | Format | Name of files |
| --- | --- | --- |
| K1 to K36 Dug Wells | CSV | Karainagar_K1_to_K36_Dug_Wells.csv |
| Groundwater Level | CSV | Karainagar_K1_to_K36_Dug_Wells_Groundwater_Level_2019_2024.csv |
| Salinity (EC) | CSV | Karainagar_K1_to_K36_Dug_Wells_EC_2019_2024.csv |
| pH | CSV | Karainagar_K1_to_K36_Dug_Wells_pH_2019_2024.csv |
| ASC Rain Gauge | CSV | Karainagar_ASC_Rain_Gauge.csv |
| Rainfall | CSV | Karainagar_ASC_Rainfall_2019_2024.csv |

Table 2. Variables and units/descriptions.

| Variable | Unit/Descriptions |
| --- | --- |
| Dug well number | K1 to K36 |
| Dug well location: Latitude | Decimal Degrees |
| Dug well location: Longitude | Decimal Degrees |
| Ground elevation from MSL | meter (m) |
| Dug well bottom elevation from MSL | meter (m) |
| Dug well kerb height | meter (m) |
| Dug well shape | Round or Square |
| Dug well diameter/width | meter (m) |
| Purpose of the dug well | Listed in Table 3 |
| Current water usage | Listed in Table 4 |
| Groundwater level from MSL | meter (m) |
| Salinity (EC) | milliSiemens per centimetre (mS cm-1) |
| pH | Range (0 – 14) |
| Rain gauge | Nylex 250 mm rain gauge at ASC |
| Rain gauge location: Latitude | Decimal Degrees |
| Rain gauge location: Longitude | Decimal Degrees |
| Daily rain gauge measurement time | Time |
| Daily rainfall | millimetre per day (mm day-1) |
| Missing values | N/A |

Table 3. Purpose of the dug well.

| Dug well purpose | Definition |
| --- | --- |
| Agricultural dug well | An agricultural dug well primarily supplies water for farming activities, such as livestock husbandry and paddy/crop irrigation during periods of insufficient rainfall. Some wells are also used for home gardening. |
| Household dug well | A household dug well is a private water source that provides water for everyday household activities, including drinking, cooking, washing, sanitation, and other essential tasks. The usage of dug well water for drinking and cooking is heavily determined by its water quality, particularly its salinity. |
| Kovil dug well | A Kovil dug well is associated with a Hindu temple (Kovil) and is generally a freshwater well located within or near the temple premises. Besides its ritual and spiritual roles, it serves as a reliable source for human consumption for surrounding communities, especially in areas where household or public wells are saline. |
| Public dug well | A public dug well is a communal well intended to be accessible to the public. It is often managed by local authorities, community organisations, or families, and serves various purposes, primarily providing drinking water and meeting other needs of the community. |

Table 4. Current water usage.

| Current water usage | Definition |
| --- | --- |
| Agriculture use | Agricultural water use refers to water used in farming and agricultural activities, including irrigating crops (such as paddy) and raising livestock. |
| Domestic use (potable water for human consumption only) | Domestic water use (potable water for human consumption only) refers to safe water suitable for drinking and/or cooking. |
| Domestic use (potable water for all household needs) | Domestic water use (potable water for all household needs) covers all the water needs of individuals and families in a household that includes water for drinking, cooking, washing, sanitation, watering gardens, and other routine household tasks. |
| Domestic use (non-potable water) | Domestic water use (non-potable water) refers to the water that is not suitable for drinking and cooking but can be used for other activities including water for washing, sanitation, watering gardens, and other routine household tasks. |
| Kovil use | Kovil (Hindu temple) water use refers to the water used for ritual purification, offerings, or practical purposes by priests and worshippers. |
| Laundry use | Laundry water use refers to the water used during the traditional handwashing of clothes. |
| No Usage | No usage of water refers to water that is no longer being used or effectively managed due to neglect or disuse, which can result from a variety of factors, including abandonment, out migration, lack of maintenance, poor water quality, and economic or social factors. |

These spatiotemporal groundwater data, along with rainfall records, are provided to facilitate a comprehensive understanding of seasonal groundwater dynamics on Karainagar Island. The dataset aims to support future research, inform governance efforts and planning for sustainable water resource management, particularly given the island's vulnerability to water stress and limited freshwater availability.

## Example Figures

This Section offers one set of graphs, as example, drawn from data documented here. Figure 3a shows the variation in daily rainfall (mm day⁻¹) across Karainagar during the 2021–22 water year. Figures 3b and 3c present the variations in groundwater level relative to MSL (m) and electrical conductivity (mS cm⁻¹), respectively, for all 36 sampled dug wells over the same period.

Figure 3 (a) Daily Rainfall (mm day⁻¹), (b) Groundwater Level relative to MSL (m), and (c) Electrical Conductivity EC (mS cm⁻¹) from 36 dug wells during the 2021–22 water year.

## Acknowledgements

This research work was supported by the Accelerating Higher Education Expansion and Development (AHEAD-R2–DOR6) Operation of the Ministry of Higher Education, funded by the World Bank, for the Water Security Project at the University of Jaffna. Ponnambalam Rameshwaran was supported by the Natural Environment Research Council (NERC, UK), as part of the NC-International Programme (NE/X006247/1).

## References

Anojan, S. R., Wijetunga, S., & Sriskandarajah, N. (2020). Community Participation in Village Scale Groundwater Monitoring and Management: A Case Study in Karainagar Island of Jaffna. Proceedings of the Undergraduate Research Symposium on Agricultural Engineering & Green Technology 2020, Department of Agricultural Engineering, Faculty of Agriculture, University of Ruhuna, Sri Lanka., 22.

Karthiga, I., Rameshwaran, P., Ketheesan, B., & Sriskandarajah, N. (2022). Aquifer monitoring and community engagement in groundwater management for water security in northern Sri Lanka. 7th Annual Research Symposium, The International Water Conference 2022, 14-16 of December 2022. National Water Supply and Drainage Board, Sri Lanka, 256-263. https://cewas.nwsdb.lk/wp-content/uploads/2022/12/7th-annual-Research-symposium-proceeding-book.pdf

Karthiga, I., Rameshwaran, P., Ketheesan, B., & Sriskandarajah, N. (2023). Groundwater mapping and locally engaged water governance in a small island terrain: Case study of Karainagar island, Northern Sri Lanka. World Water Policy, 9(3), 456-480. https://doi.org/https://doi.org/10.1002/wwp2.12112

## Appendix A: Photographs of all 36 dug wells

| K1 | K2 | K3 | K4 |
| --- | --- | --- | --- |
| K5 | K6 | K7 | K8 |
| K9 | K10 | K11 | K12 |
| K13 | K14 | K15 | K16 |
| K17 | K18 | K19 | K20 |
| K21 | K22 | K23 | K24 |
| K25 | K26 | K27 | K28 |
| K29 | K30 | K31 | K32 |
| K33 | K34 | K35 | K36 |