Development and calibration data of the B4+ Mini Buoy design

Description

This dataset contains acceleration data for seven 'Mini Buoys' (low-cost open-source bottom-mounted float sensors for measuring inundation duration, current velocity, and wave orbital velocity from acceleration data) and local water levels used in a pilot study to determine the effect of weight and tether length on the sensitivity of the B4+ Mini Buoy to detecting hydrodynamics.

Additional datasets provided are used for deriving near-bottom wave orbital velocities using two methods: linear wave theory and direct velocity measurements. Both methods were then used to compare calibrations between velocity and acceleration data gathered by low-cost and open-source bottom-mounted floats called 'Mini Buoys'. The dataset contains water velocity data gathered by Acoustic Doppler Velocimeters, and acceleration data gathered from the B4+ Mini Buoy design.

Furthermore, a dataset is provided for Mini Buoy calibration, which uses the drag-tilt principle to infer hydrodynamics from acceleration data. The dataset contains acceleration data gathered from three 'Mini Buoy' designs ('B4', 'B4+', and 'Pendant'), alongside water level, current velocity, and wave orbital velocity measured from a range of commercially-available sensors, in order to build calibration curves for inferring hydrodynamics from acceleration data.

Nine B4+ Mini Buoys, each sampling acceleration values along the y-axis at 1 Hz, were deployed adjacent to a water level (pressure) logger (In-Situ Rugged TROLL 100) on the tidal flat in front of Caerlaverock saltmarsh, Solway Firth, Scotland (54°58'05"N, 3°31'58"W) between the 15th and 24th of September 2020. Prior to deployment and after retrieving them, the loggers were inserted into a bucket of water for 1 minute to determine logger position in the tube. Each logger was fitted with different weights of ballast and different tether lengths. The pressure logger was set to sample at 1-minute intervals. Unfortunately, two loggers were lost during the deployment. Data to build a calibration between wave orbital velocity and acceleration was gathered at Caerlaverock, Scotland (54°58′05′′ N, 3°31′58′′ W) and Black Scar, Wales (51°45′54′′ N, 4°26′12′′ W). Data to build a calibration between hydrodynamics (inundation duration, current velocity, and wave orbital velocity) and acceleration was gathered at Percut Sei Tuan, Indonesia (3°43′22′′ N, 98°46′15′′ E), Caerlaverock, Scotland (54°58′05′′ N, 3°31′58′′ W), Black Scar, Wales (51°45′54′′ N, 4°26′12′′ W), and Skinflats, Scotland (56°03′22′′ N, 3°43′60′′ W) for the three designs: the 'B4', 'B4+', and 'Pendant'.

Files and class names

1. CalibrationS2

| File(s) | Column | Description |
| --- | --- | --- |
| B4/Inundation/MB09.csv B4/Inundation/MB11.csv B4/Inundation/MB12.csv B4/Inundation/MB14.csv B4+/Inundation/MB14.csv B4+/Inundation/MB15.csv B4+/Inundation/MB16.csv Pendant/ Inundation/MB03.csv Pendant/ Inundation/MB04.csv Pendant/ Inundation/MB19.csv | Date | Date and time stamp of each record in the format yyyy-mm-dd hh:mm:ss. |
|  | Acceleration | Gravity-compensated acceleration values. Units in g-force (G). |
|  | WaterLevel | Water level values above the tidal flat. Units in metres (m). |
|  | Status | Inundation status of the Mini Buoy classified as either non-inundated (N), partially inundated (P), or fully submerged (F). |
| B4/Currents/MB01.csv Pendant/Currents/MB01.csv | Date | Date and time stamp of each record in the format yyyy-mm-dd hh:mm:ss. |
|  | Acceleration | Gravity-compensated acceleration values. Units in g-force (G). |
|  | Velocity | Integrated current velocity values. Units in metres per second (m/s). |
| B4+/Currents/MB14.csv | Date | Date and time stamp of each record in the format yyyy-mm-dd hh:mm:ss. |
|  | Acceleration | Gravity-compensated acceleration values. Units in g-force (G). |
|  | Velocity | Integrated current velocity values. Units in metres per second (m/s). |
|  | Beam | Integrated beam correlation. Units as a percentage (%). |
| B4+/Waves/Waves.csv | Code | Code denoting the study area as either Black Scar (BS) or Caerlaverock (CA). |
|  | Date | Date and time stamp of each record in the format yyyy-mm-dd hh:mm:ss. |
|  | Burst | Burst ID denoting a period of continuous wave orbital velocity sampling. |
|  | Acceleration | Gravity-compensated acceleration values. Units in g-force (G). |
|  | WaveOrbVel | Integrated peak wave orbital velocity values. Units in metres per second (m/s). |
| B4+/Test/CI/MB01.csv B4+/Test/TB/MB17.csv | Date | Date and time stamp of each record in the format yyyy-mm-dd hh:mm:ss. |
|  | Acceleration | Gravity-compensated acceleration values. Units in g-force (G). |

2. CalibrationS3

| File(s) | Column | Description |
| --- | --- | --- |
| ADV/BS.csv ADV/CA.csv | Date | Date and time stamp of each record in the format yyyy-mm-dd hh:mm:ss. |
|  | Burst | Burst ID denoting a period of continuous wave orbital velocity sampling. |
|  | Xvel | Water velocity in the horizontal (x-axis) direction. Units in m/s. |
|  | Yvel | Water velocity in the horizontal (y-axis) direction. Units in m/s. |
|  | Zvel | Water velocity in the vertical (z-axis) direction. Units in m/s. |
|  | MeanBeamCorr | Mean beam correlation from all three ADV receiver arms. Units as a percentage (%). |
| B4+/BS.csv B4+/CA.csv | Date | Date and time stamp of each record in the format yyyy-mm-dd hh:mm:ss. |
|  | Yacc | Gravity-compensated acceleration values. Units in g-force (G). |

3. Development1

| File(s) | Column | Description |
| --- | --- | --- |
| Acceleration/MB02.csv Acceleration/MB03.csv Acceleration/MB04.csv Acceleration/MB05.csv Acceleration/MB06.csv Acceleration/MB07.csv Acceleration/MB08.csv | Date | Date and time stamp of each record in the format yyyy-mm-dd hh:mm:ss. |
|  | Yacc | Gravity-compensated acceleration values. Units in g-force (G). |
| WaterLevel.csv | Date | Date and time stamp of each record in the format yyyy-mm-dd hh:mm:ss. |
|  | WaterLevel_m | Water level values above the tidal flat. Units in metres (m). |

Collection methods

Data to build a calibration between hydrodynamics (inundation duration, current velocity, and wave orbital velocity) and acceleration was gathered at Percut Sei Tuan, Indonesia (3°43′22′′ N, 98°46′15′′ E), Caerlaverock, Scotland (54°58′05′′ N, 3°31′58′′ W), Black Scar, Wales (51°45′54′′ N, 4°26′12′′ W), and Skinflats, Scotland (56°03′22′′ N, 3°43′60′′ W) for three designs: the ‘B4’, ‘B4+’, and ‘Pendant’.

Acceleration

B4 and B4+ buoys sampled y-axis acceleration at 1 Hz, whilst Pendant buoys sampled at 0.1 Hz on the x-axis. All Mini Buoys were deployed within 5 m of calibrating sensors.

Inundation duration

For the B4, pressure data was gathered using an upward-facing Nortek Aquadopp 300 m current meter. Both the B4+ and Pendant loggers were calibrated against pressure data gathered from an In-Situ Rugged TROLL 100 pressure sensor mounted upright above the ground. Pressure sensors logged every minute.

Current velocity

The B4 buoy was calibrated using an upward-facing Nortek Aquadopp 300 m. ‘Diagnostic Mode’ was used to measure in 1,025 sample bursts every hour at 1Hz (internal sampling rate was 23 Hz). The measurement head was located 20 cm above the sediment surface, and blanking distance set to 35 cm. The B4+ buoy was calibrated using an upward-facing Acoustic Doppler Velocimeter (Nortek Vector), hereafter referred to as ADV. Height of the ADV pressure sensor and velocity cell being measured above the bed was 4 and 30 cm respectively. Velocities were recorded at a rate of 16 Hz in 13-minute bursts every 30 minutes. Nominal velocity range was set to 0.3 m/s and ‘Surf Zone’ default settings were used. The Pendant design was calibrated using an upward-facing Nortek ECO taking velocity readings every 2 minutes. The lower sampling bin was used for surveying velocities nearest the ground.

Wave orbital velocity

For the B4+ only, wave orbital velocities were measured using an ADV. Height of the ADV pressure sensor and velocity cell being measured above the bed at Black Scar was 4 and 30 cm respectively. For the ADV deployed at Caerlaverock, the pressure sensor and velocity cell were 9 and 32 cm above the tidal flat respectively. Velocities were recorded at a rate of 16 Hz in 13-minute bursts every 30 minutes at Black Scar and 1Hz frequency in 20-minute bursts every 1 hour at Caerlaverock. Nominal velocity range was set to 0.3 m/s and ‘Surf Zone’ default settings were used.

Data processing

Acceleration

For current velocity calibrations, mean acceleration was calculated across 1-minute windows for the B4 and B4+ Mini Buoy designs, and 2-minute windows for the Pendant Mini Buoy design. In all other cases, no processing of acceleration data was done.

Inundation

Pressure sensors were corrected for atmospheric pressure and were converted to water level according to manufacturer’s instructions. The water level was corrected for the height of the pressure sensors above the bed and set to start at the base of the Mini Buoy using a Leica RTK-GPS (B4+ and Pendant) or Leica Optical Level (B4) accurate to below 2 cm. A linear interpolation was used to predict water levels to match the sampling frequencies of the Mini Buoys. All negative water level values were converted to 0. An inundation status was then assigned for when the Mini Buoy was emersed (N), partially inundated (P), or fully submerged (F).

Current velocity

For current velocity calibrations, mean current velocity was calculated across 1-minute windows for the B4 and B4+ Mini Buoy designs, and 2-minute windows for the Pendant Mini Buoy design.

Wave orbital velocity

Wave dimensional spectra per burst duration were calculated from the ADV velocity and pressure data. Prior to analysis, velocity data was filtered to remove values with an average beam correlation value of <75% indicating partial inundation. Bursts that had >5% of missing data were also removed. Rotated velocity vectors aligned with the main flow direction, were calculated from the x, y, and z velocity values recorded by the ADV. Nominal number of output frequencies were 780 and 1,000 for Black Scar and Caerlaverock sites respectively. Low frequency cut-off was set to 0.067 Hz, maximum value of factor scaling pressure to waves was 200, and minimum spectral level for computing direction and spreading was 0.03 m2/Hz.