# Microplastic agricultural soil survey and inter-laboratory comparison across China, Egypt, India, Sri Lanka, and Vietnam, 2022-2025

# Summary

This dataset was generated during the period of 2022-2025. This dataset entails two components: 1) an agricultural soil survey on fields covered with plastic mulch film to determine drivers of microplastic enrichment and identify microplastic types in China, Egypt, India, Sri Lanka, and Vietnam; 2) an inter-laboratory comparison of microplastic quantification and identification across Australia, China, Egypt, India, Sri Lanka, UK, and Vietnam. We recorded the following data:

Part 1 – Soil Survey

- Site specific metadata on plastic use and farm practices

- Environmental data (precipitation, temperature)

- Soil data (pH, EC, nutrients, classification, texture)

- Microplastic quantification

- Microplastic identification

- Metadata for each participating laboratory

Part 2 – Inter-laboratory standard comparison

- Metadata for each participating laboratory

- Polymer identification of known polymer standards

- Microplastic recovery rates of known reference standard mediums

# File format

All files are in .csv format.

# File names and/or naming convention

List the file names of the data files (including the file extension).

| File name | File content |
| --- | --- |
| SurveyQuestionnaire.csv | Metadata of survey sites covered in the soil survey. |
| SurveySoilProperties.csv | Soil properties of soils analysed in the soil survey. |
| SurveyMPQuantification.csv | Microplastic particle numbers of soil analysed in the soil survey. |
| SurveyMPIdentification.csv | Microplastic identification of particles detected in soil in the soil survey. |
| StandardPolymerID.csv | Identification of standard polymer particles provided to each laboratory. |
| StandardPolymerLibraryComparisonUK.csv | Comparison of standard polymer identification results after analysis of the same sample on the same instrument with the original and updated reference library (performed in the UK only). |
| StandardMPQuantification.csv | MP quantification from standard reference materials provided to each laboratory. |
| LaboratoryMetadata.csv | Metadata for each participating laboratory. |

# Nature and units of recorded values

Include information about the structure of the data (variable name(s), description, units).

Table 1. Description of variables in SurveyQuestionnaire.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Site_Code | Numerical identifier for each sampling location | n/a |
| Country | Name of the country | n/a |
| Crop | Recent crop types grown on sampled fields | n/a |
| Tillage | Cultivation practice(s) used on site – Yes refers to practice is being used; No refers to practice not being used. Multiple practices can be used on the same site. | Presence/absence |
| Surface_cultivation | Cultivation practice(s) used on site – Yes refers to practice is being used; No refers to practice not being used. Multiple practices can be used on the same site. | Presence/absence |
| Plough | Cultivation practice(s) used on site – Yes refers to practice is being used; No refers to practice not being used. Multiple practices can be used on the same site. | Presence/absence |
| Harrow | Cultivation practice(s) used on site – Yes refers to practice is being used; No refers to practice not being used. Multiple practices can be used on the same site. | Presence/absence |
| Rotavator | Cultivation practice(s) used on site – Yes refers to practice is being used; No refers to practice not being used. Multiple practices can be used on the same site. | Presence/absence |
| Max_cultivation_depth | Maximum cultivation depth of applied practices | cm |
| Soil_classification | Soil classification according to WRB [1] | n/a |
| Sand | Soil type – Yes refers to soil type present; No refers to soil type not present. Multiple soil types can be present on the same site. | Yes/no |
| Loam | Soil type – Yes refers to soil type present; No refers to soil type not present. Multiple soil types can be present on the same site. | Yes/no |
| Silt | Soil type – Yes refers to soil type present; No refers to soil type not present. Multiple soil types can be present on the same site. | Yes/no |
| Clay | Soil type – Yes refers to soil type present; No refers to soil type not present. Multiple soil types can be present on the same site. | Yes/no |
| Annual_average_rainfall | Average annual precipitation | mm/y |
| Annual_average_air_temperature | Average annual air temperature | °C |
| PMF_use_duration | Duration of plastic mulch film (PMF) use | years |
| PMF_type | PMF plastic type | n/a |
| Ploughing | Description of PMF end-of-life (EOL) routes – ploughing PMF into the soil after use. Yes means this practice is used; No means this practice isn’t used. Multiple EOL routes can apply to the same site. | Yes/no |
| Burning | Description of PMF end-of-life (EOL) routes – burning PMF on site after use. Yes means this practice is used; No means this practice isn’t used. Multiple EOL routes can apply to the same site. | Yes/no |
| Waste_collection | Description of PMF end-of-life (EOL) routes – PMF is collected after use. Yes means this practice is used; No means this practice isn’t used. Multiple EOL routes can apply to the same site. | Yes/no |
| Selling | Description of PMF end-of-life (EOL) routes – PMF is sold after use. Yes means this practice is used; No means this practice isn’t used. Multiple EOL routes can apply to the same site. | Yes/no |
| Littering | Description of PMF end-of-life (EOL) routes – PMF is collected and littered/dumped after use. Yes means this practice is used; No means this practice isn’t used. Multiple EOL routes can apply to the same site. | Yes/no |
| Recycling | Description of PMF end-of-life (EOL) routes – PMF is collected and recycled after use. Yes means this practice is used; No means this practice isn’t used. Multiple EOL routes can apply to the same site. | Yes/no |
| Storage | Description of PMF end-of-life (EOL) routes – PMF is collected and stored on site after use. Yes means this practice is used; No means this practice isn’t used. Multiple EOL routes can apply to the same site. | Yes/no |
| Reuse | Description of PMF end-of-life (EOL) routes – PMF is re-used. Yes means this practice is used; No means this practice isn’t used. Multiple EOL routes can apply to the same site. | Yes/no |
| Coated_fertiliser_use | Use of coated fertilisers on site | Yes/no |
| Coated_fertiliser_use_duration | Duration of coated fertiliser use | years |
| Compost_use | Use of compost on site | Yes/no |
| Compost_quantity_per_year | Compost quantity applied to site per year | Kg/ha |
| Cow_manure | Type of compost applied at site. Multiple types can apply to the same site. Yes means compost type is used on site, no means compost type isn’t used on site. | Yes/no |
| Chicken_manure | Type of compost applied at site. Multiple types can apply to the same site. Yes means compost type is used on site, no means compost type isn’t used on site. | Yes/no |
| Pig_manure | Type of compost applied at site. Multiple types can apply to the same site. Yes means compost type is used on site, no means compost type isn’t used on site. | Yes/no |
| Sheep_manure | Type of compost applied at site. Multiple types can apply to the same site. Yes means compost type is used on site, no means compost type isn’t used on site. | Yes/no |
| Goat_manure | Type of compost applied at site. Multiple types can apply to the same site. Yes means compost type is used on site, no means compost type isn’t used on site. | Yes/no |
| Microbial_Organic_fertiliser | Type of compost applied at site. Multiple types can apply to the same site. Yes means compost type is used on site, no means compost type isn’t used on site. | Yes/no |
| Commercial_fertiliser | Type of compost applied at site. Multiple types can apply to the same site. Yes means compost type is used on site, no means compost type isn’t used on site. | Yes/no |
| Compost_use_duration | Duration of compost use on site | years |
| Sewage_sludge_use | Use of sewage sludge on site | Yes/no |
| Sewage_sludge_quantity_per_year | Quantity of sewage sludge applied on site per year | Kg/ha |
| Sewage_sludge_use_duration | Duration of sewage sludge use on site | years |
| Irrigation_pipes | Use of other plastics on site. Multiple other plastic categories can apply per site. | Yes/no |
| Nets | Use of other plastics on site. Multiple other plastic categories can apply per site. | Yes/no |
| Greenhouses | Use of other plastics on site. Multiple other plastic categories can apply per site. | Yes/no |
| Polytunnels | Use of other plastics on site. Multiple other plastic categories can apply per site. | Yes/no |
| Packaging | Use of other plastics on site. Multiple other plastic categories can apply per site. | Yes/no |
| Drums | Use of other plastics on site. Multiple other plastic categories can apply per site. | Yes/no |
| Barriers | Use of other plastics on site. Multiple other plastic categories can apply per site. | Yes/no |
| Crates | Use of other plastics on site. Multiple other plastic categories can apply per site. | Yes/no |

Table 2. Description of variables in SurveySoilProperties.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Site | Numerical identifier for each sampling location | n/a |
| Country | Name of the country | n/a |
| Depth | Sampling depth of analysed sample: 10 = 0-10cm; 20 = 10-20cm; 30=20-30cm. | cm |
| pH | Soil pH | n/a |
| EC | Soil electrical conductivity | µS/cm |
| NH4 | Soil ammonium concentration on dry weight basis | Mg/kg |
| NO3 | Soil nitrate concentration on dry weight basis | Mg/kg |
| DOC | Soil dissolved organic carbon concentration on dry weight basis | Mg/kg |
| PO4 | Soil phosphate concentration on dry weight basis | Mg/kg |

Table 3. Description of variables in SurveyMPQuantification.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Site | Numerical identifier for each sampling location | n/a |
| Country | Name of the country | n/a |
| Depth | Sampling depth of analysed sample: 10 = 0-10cm; 20 = 10-20cm; 30=20-30cm. | cm |
| Z | Robust Z transformed relative enrichment of MP particles in soil. | n/a |
| MP_number | Raw MP particle number per gram of dry soil | g-1 |

Table 4. Description of variables in SurveyMPIdentification.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Code | Numerical identifier for each sampling location | n/a |
| Country | Name of the country | n/a |
| Depth | Sampling depth of analysed sample: 10 = 0-10cm; 20 = 10-20cm; 30=20-30cm. | cm |
| Quality | Match quality of sample spectra to reference library spectra | % |
| ID | MP particle identification via IR spectroscopy | n/a |

Table 5. Description of variables in StandardPolymerID.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Country | Name of the country | n/a |
| Sample | Number of the standard sample | n/a |
| Particle | Number of the analysed particle | n/a |
| ID | Identification of particle via infrared spectroscopy | n/a |
| MatchQuality | Match quality of sample spectra to reference library spectra | % |

Table 6. Description of variables in StandardPolymerLibraryComparisonUK.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Library | Version of reference library used | n/a |
| Sample | Number of the standard sample | n/a |
| Particle | Number of the analysed particle | n/a |
| ID | Identification of particle via infrared spectroscopy | n/a |
| MatchQuality | Match quality of sample spectra to reference library spectra | % |

Table 7. Description of variables in StandardMPQuantificaation.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Country | Name of the country | n/a |
| Sample | Name of the reference sample. - PE only 0.04%: Equivalent number of PE particles to samples spiked with 0.04% w/w PE - Blank: Blank samples, no medium added, only reagents - BT 0%: Bentonite clay without added PE. - BT 0.04%: Bentonite clay with 0.04% w/w PE added - SD 0%: Silicon dioxide without added PE - SD 0.04% Silicon dioxide with 0.04% w/w PE added - Soil 0%: Soil representative of country without PE - Soil 0.04%: Soil representative of country with 0.04% w/w PE added | n/a |
| Rep | Number of sample replicate | n/a |
| Particle_number | Number of fluorescent (suspected MP) particles per g dry soil | g-1 |

Table 8. Description of variables in LaboratoryMetadata.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| Country | Name of country | n/a |
| Instrument_manufacturer | Metadata for infrared spectroscopy: Instrument manufacturer | n/a |
| Instrument_type | Metadata for infrared spectroscopy: Instrument type | n/a |
| Instrument_model | Metadata for infrared spectroscopy: Instrument model | n/a |
| Location | Metadata for infrared spectroscopy: Location of analysis | n/a |
| Library | Metadata for infrared spectroscopy: Reference library used | n/a |
| Soil_weight | Weight of dry soil analysed for MP quantification and identification | g |
| Soil_classification | Metadata for soil reference standards: Soil classification | n/a |
| Soil_texture | Metadata for soil reference standards: Soil texture | n/a |
| Membrane_area | Metadata for fluorescent microscopy: Area of filter membrane analysed for particle quantification | % |
| Image_size | Metadata for fluorescent microscopy: Size of individual images taken of membrane | Mm x mm |
| Membrane_d | Metadata for fluorescent microscopy: Membrane diameter | mm |
| Real_membrane_d | Metadata for fluorescent microscopy: Diameter of membrane area that contains particles after filtration | mm |
| Real_membrane_a | Metadata for fluorescent microscopy: Area of membrane that contains particles after filtration | Mm2 |
| Membrane_type | Metadata for fluorescent microscopy: Type pf membrane | n/a |
| Pore_size | Metadata for fluorescent microscopy: Membrane pore size | µm |
| Membrane_manufacturer | Metadata for fluorescent microscopy: Manufacturer/supplier of membranes | n/a |
| Membrane_product_number | Metadata for fluorescent microscopy: Product number of membranes | n/a |
| Filtration_system_manufacturer | Metadata for fluorescent microscopy: Manufacturer of filtration system | n/a |
| Vacuum_pump_manufacturer | Metadata for fluorescent microscopy: Manufacturer of vacuum pump | n/a |
| Vacuum_pump_type | Metadata for fluorescent microscopy: Type pf vacuum pump | n/a |
| Microscope_manufacturer | Metadata for fluorescent microscopy: Fluorescent microscope manufacturer | n/a |
| Microscope_model | Metadata for fluorescent microscopy: Fluorescent microscope type | n/a |
| Excitation_wavelength | Metadata for fluorescent microscopy: Excitation wavelength | Nm |
| Emission_wavelength | Metadata for fluorescent microscopy: Emission wavelength | Nm |
| Location | Metadata for fluorescent microscopy: Place of analysis | n/a |
| Nile_Red_manufacturer | Metadata for chemicals: Nile red manufacturer/supplier | n/a |
| Nile_red_product_number | Metadata for chemicals: Nile red product number | n/a |
| Nile_red_purity | Metadata for chemicals: Nile red purity | % |
| Nile_red_CAS_number | Metadata for chemicals: Nile red CAS number | n/a |
| Polyethylene_manufacturer | Metadata for chemicals: PE manufacturer/supplier – PE used for inter-lab comparison tests | n/a |
| Polyethylene_product_number | Metadata for chemicals: PE product number – PE used for inter-lab comparison tests | n/a |
| Polyethylene_CAS_number | Metadata for chemicals: PE CAS number – PE used for inter-lab comparison tests | n/a |
| SD_manufacturer | Metadata for chemicals: Silicon dioxide manufacturer/supplier – used for inter-lab comparison tests | n/a |
| SD_product_number | Metadata for chemicals: Silicon dioxide product number – used for inter-lab comparison tests | n/a |
| SD_CAS_number | Metadata for chemicals: Silicon dioxide CAS number – used for inter-lab comparison tests | n/a |
| BT_manufacturer | Metadata for chemicals: Bentonite clay manufacturer/supplier – used for inter-lab comparison tests | n/a |
| BT_product_number | Metadata for chemicals: Bentonite clay product number – used for inter-lab comparison tests | n/a |
| BT_CAS_number | Metadata for chemicals: Bentonite clay CAS number – used for inter-lab comparison tests | n/a |
| NaOH_manufacturer | Metadata for chemicals: NaOH manufacturer/supplier – used for MP extractions | n/a |
| NaOH_product_number | Metadata for chemicals: NaOH product number – used for MP extractions | n/a |
| NaOH_CAS_number | Metadata for chemicals: NaOH CAS number – used for MP extractions | n/a |
| NaCl_manufacturer | Metadata for chemicals: NaCl manufacturer/supplier – used for MP extractions | n/a |
| NaCl_product_number | Metadata for chemicals: NaCl product number – used for MP extractions | n/a |
| NaCl_CAS_number | Metadata for chemicals: NaCl CAS number – used for MP extractions | n/a |

Spatial coverage
[CONDITIONAL – Required for spatial data resources]

The soil survey was conducted in China, Egypt, India, Sri Lanka, and Vietnam. The locations of participating farm can not be disclosed due to privacy and identification protection. The regions in which sampling took place are shown in Fig. 1.

The inter-laboratory comparison was conducted in Australia, China, Egypt, India, Sri Lanka, UK, and Vietnam.

Figure 1. Sample site distribution within a) China, b) Egypt, c) India, d) Sri Lanka, and e) Vietnam. Blue areas indicate regions/provinces where sampling sites (n = 32-40) were located. This figure was sourced from Graf et al., under review.

# Temporal coverage and resolution
[CONDITIONAL – Required if applicable]

The soil survey was carried out between 2022 – 2025. Each location was visited once, during which soil samples for analysis were collected and farm specific data acquired.

# Collection/Generation/Transformation methods

Part 1 – Soil Survey:

- pH and electrical conductivity: Soil samples (n=1) were analysed in a 1:2.5 soil:distilled water solution using standard electrodes. Data recorded in paper notebook, then transferred to Excel file. Excel file exported as csv file.

- Nutrients (ammonium, nitrate, DOC, phosphate): Soil samples (n=1) were analysed in a 1:5 soil:0.5 M K2SO4 solution (ammonium, nitrate, DOC) using colorimetric analysis for ammonium and nitrate, and a TOC analyser for DOC. Soil samples were analysed in a 1:5 soil:0.5M acetic acid or 0.5M sodium bicarbonate solution (phosphate) using colorimetric analysis. Data was exported as Excel files from respective instruments, then exported as csv file.

- Microplastic quantification: Microplastics were extracted from soil samples in each country following a standardised protocol[2]. Briefly, soil samples were dried at 40 °C for min. 24 h, larger aggregates were broken up, and plant material was removed before weighing out 5 ± 0.05 g of dry soil into a clean glass beaker. For density separation, 230 ml saturated NaCl (density 1.2 g cm-3) were added to the beaker, the solution was homogenised for 30 min using a magnetic stirring plate, and left to stand overnight. Following this, the supernatant was decanted into a new glass beaker, and 4 M NaOH was added to a fixed volume of 500 ml, after which the solution was homogenised and left to stand overnight to facilitate digestion of biologically-derived organic material. The supernatant was then decanted into a new glass beaker and 250 µl of Nile Red solution added, homogenised, and incubated in the dark at room temperature for 30 min before vacuum filtration onto a suitable membrane. Membranes were stored in glass petri dishes covered in aluminium foil and analysed within 24 h. Particle quantification on membranes was carried out using fluorescent microscopy within each country. Between 5 – 100 % of the membrane area were analysed, depending on country, and resulting images were processed using ImageJ (v. 1.54f). Briefly, a Macro was customised for each country, scaling images accordingly, applying a suitable colour threshold with the default thresholding method, adjusting the brightness level to a suitable setting (according to the average best fit of 10 randomly selected images), and exporting particles’ Feret’s diameter. Data was filtered to only include particles with Feret’s diameter of 20 – 500 µm, and scaled to an equivalent particle number of 100% membrane area (= 5 g dry soil). Total particle number was then divided by 5 to express particles g-1 dry soil.

- Microplastic identification: Complimentary to the particle quantification, fluorescent particles (i.e. suspected MPs) were also analysed using IR spectroscopy within each country. Particle spectra were matched against instrument-specific or open-source libraries with an average match quality for polymers of 40.6% (China), 80.8% (Egypt), 70.6% (India), 85.7% (Vietnam), and 84.7% (Sri Lanka). China, Egypt, India, and Vietnam manually selected individual particles for analysis, resulting in an average of 10 particles analysed across these countries due to resource constraints and instrument availability. Data for China are only available for 0 – 10 cm depth. Sri Lankan samples were analysed in the UK, utilising an automated analysis of particles per subsample (analysing approx. 1% of total sample volume). A match quality threshold of ≥ 80% was applied to the Sri Lankan dataset only, according to manufacturer’s guidance. Due to no clear manufacturers’ instructions and the already low manual particle count per sample in the other countries, no match quality threshold was applied.

- Environmental data: Precipitation and air temperature data was sourced from the nearest weather station for each location.

- Farm metadata: Data regarding farm practices was collected directly from participating farmers, written on paper, entered into an Excel file, exported as a csv file.

Part 2 – Inter-laboratory standard comparison

- Microplastic quantification: The standards consisted of three matrices: silicon dioxide (SD; quartz sand), bentonite clay (BT), and country-representative soil; all with and without 0.04% (w/w) polyethylene (PE; 34-50 μm particle size). SD and BT were selected to represent two commonly found soil textures (sand, clay), and soil was selected representative of each country’s prevalent soil types. The soil was collected by each laboratory and spiked with PE (provided by UK) to 0.04% (w/w). The UK also determined the number of PE particles in the mass required to achieve the 0.04% (w/w) concentration in 5 g matrix to estimate the average particle number per sample and therefore estimate maximum particle number recoverable. All standards were analysed in triplicates alongside blanks (no matrix, only reagents), following a standardised extraction and analysis protocol [2]. The same methodology was applied as described above.

- Microplastic identification: The standards consisted of 9 individual polymer types (PET, PP, PTFE, PU, PC, PE, PS, PA, PVC) with different size ranges (Table 9). Each country identified 10 randomly selected particles per standard. The analysis was carried out blind, i.e. participating countries (apart from UK) were unaware of what polymer types were included in each standard vial. Sri Lanka had no capability to analyse polymer standards via IR spectroscopy and was therefore excluded from this analysis. India was unable to analyse ‘standard 3’ (PTFE) due to sample loss. In addition to individual standards, each laboratory received a homogenised polymer mixture containing the same nine polymers in known weight proportions (Table 10). Each laboratory analysed three replicates of the mixed polymer standard, selecting 50 random particles per replicate. India was unable to analyse Mix 1 and 2 due to sample loss, therefore only Mix 3 is included.

Table 9. Material specifications of individual polymer standards analysed via IR spectroscopy in each country. Average and median particle sizes were determined by the UK using the LDIR 8700 chemical imaging system’s reported diameter (n = 10). This table was sourced from Graf et al., under review.

| Standard # | Polymer type | Supplier | CAS | Product number | Measured average particle size ± SD (µm) | Measured median particle size (µm) | Particle type and shape | Colour | Weight per vial (mg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PET | Goodfellow | 25038-59-9 | ES30-PD-000132 | 130 ± 112 | 97 | Powder - circular | White | 1 |
| 2 | PP | Parchvale Ltd. | 9003-07-0 | Glissawax 14205 | 13 ± 23 | 5 | Powder – circular | White | 0.5 |
| 3 | PTFE | Goodfellow | 9002-84-0 | FP30-FB-000122 | 121 ± 266 | 12 | Fibre – elongated | Brown | 0.5 |
| 4 | PU | Goodfellow | 9009-54-5 | UR30-GL-000101 | 2150 ± 948 | 2160 | Granule – fragment | Clear | 20 |
| 5 | PC | Goodfellow | 25766-59-0 | CT30-GL-000111 | 261 ± 469 | 38.5 | Granule – fragment | Red | 2 |
| 6 | PE | Sigma Aldrich | 9002-88-4 | 434272 | 22 ± 37 | 8 | Powder – circular | White | 0.5 |
| 7 | PS | Sigma Aldrich | 9003-53-6 | 441147 | 764 ± 968 | 88 | Granule – fragment | Clear | 2 |
| 8 | PA | Goodfellow | 25038-54-5 | AM30-PD-000155 | 31 ± 42 | 14 | Powder – circular | White | 0.5 |
| 9 | PVC | Sigma Aldrich | 9002-86-2 | 189588 | 194 ± 107 | 224 | Powder - circular | White | 1 |

Table 10. Composition of polymer mix standard (analysed in triplicates: Mix 1-3) analysed via IR spectroscopy in each country. This table was sourced from Graf et al., under review.

| Polymer type | Weight added (mg) | Equivalent of total weight (% w/w) |
| --- | --- | --- |
| PET | 0.5 | 2.04 |
| PP | 0.15 | 0.61 |
| PTFE | 0.25 | 1.02 |
| PU | 20 | 81.80 |
| PC | 1.25 | 5.11 |
| PE | 0.15 | 0.61 |
| PS | 1.5 | 6.13 |
| PA | 0.15 | 0.61 |
| PVC | 0.5 | 2.04 |

# Experimental design/sampling regime
[CONDITIONAL - required for monitoring/experimental data]

A soil survey was carried out in China, Egypt, India, Sri Lanka, and Vietnam, from 2022 – 2024. Sites were selected according to the following criteria: i) LDPE PMF use at the site; ii) good spatial separation to capture representative PMF-use areas within each country (where possible); iii) reasonable distance away from waste management facilities or industrial areas.

Soil samples from three depths (0 – 10, 10 – 20, 20 – 30 cm) were collected at 40 sites per country, resulting in a total of 120 soil samples per country with no replicates. Sampling followed a standard protocol and was conducted in the middle of fields, avoiding field edges and areas with plastic waste accumulation, in a W-sampling pattern, collecting 5 samples per soil depth, which were homogenised into one composite sample per soil depth[2]. Soil samples were collected, stored, and processed using non-plastic implements to minimise contamination.

For the inter-laboratory standard comparison, all samples were produced in the UK and shipped to participating countries.

# Fieldwork and/or laboratory instrumentation
[CONDITIONAL - required for experiments where instruments used]

Detailed information on instruments and consumables used during the sampling and analysis process for each country are included in the file LaboratoryMetadata.csv.

# Calibration steps and values
[CONDITIONAL – required where instruments have been used]

All analyses have been carried out alongside regular blanks and internal standards applicable for each instrument.

# Quality control

Data was curated and missing values were replaced with n/a.

# References

[1]	IUSS Working Group WRB, 2022. World Reference Base for Soil Resources. International soil classification system for naming soils and creating legends for soil maps. 4th edition. Vienna

[2]	Ren S, Graf M, Wang K, Zhang J, Zhang H, Liu X, Li J, Zhu T, Ren K, Sun Y, Qi R, Collins BI, Xu L, Jiang X, Cui J, Ding F, Yan C, Liu X, Jones DL, Chadwick DR, 2025. Separation and Identification of Conventional Microplastics from Farmland Soils. Journal of Visualized Experiments 217, e67064. https://doi.org/10.3791/67064