# Sub-daily actual evapotranspiration data for 45 monitoring sites (2014-2024) [COSMOS-UK]

Hollie M. Cooper, David Crowhurst, Alex M.J. Cumming, Jonathan Evans, Tim Howson, Ross Morrison, Andrew Retter, Richard Smith, Simon Stanley, Philip Vincent and Matthew Fry

UK Centre for Ecology & Hydrology (UKCEH), Wallingford, UK

FOR DATA CENTRE USE ONLY

These data are available under the terms of the [licence].

As a condition of using these data you must include the following citation:

[Authors] [Date]. [Dataset title] NERC EDS Environmental Information Data Centre. (Dataset). https://doi.org/10.5285/[Dataset_ID]

Further information is available from https://doi.org/10.5285/[Dataset_ID]

Table of contents

1	Overview	3

1.1	Evapotranspiration	3

1.2	This dataset	3

2	Parameters	4

3	File information	6

3.1	File format and naming	6

3.2	File contents	6

4	Spatial coverage	7

5	Temporal coverage and resolution	11

6	Data acquisition and processing	13

6.1	Experimental design/sampling regime	13

6.2	Collection, calibration and maintenance methods	14

6.3	Data processing methods	14

6.4	Quality control	17

6.5	Additional calculations	18

7	Acknowledgements	18

8	References	19

9	Appendix A – Additional COSMOS-UK site measurements	22

10	Appendix B – Anemometer firmware	24

# Overview

## Evapotranspiration

Evapotranspiration (ET) is the combined process of evaporation (from soil, surfaces and open water) and transpiration (by plants), which forms a key part of the global water cycle. ET is an important factor in understanding hydrological processes (e.g. soil moisture dynamics) and sustainable water management, particularly in agriculture.

## This dataset

This dataset contains derived land surface-atmosphere exchanges of sensible heat (H) and latent heat (LE) from available COSMOS-UK environmental monitoring sites located across the UK. These heat, energy and momentum flux data are available alongside the hydrometeorological and soil physics data, available here:

- The COSMOS-UK website and API - https://cosmos.ceh.ac.uk/data-home-page

- Published data in the Environmental Information Data Centre (EIDC) -https://doi.org/10.5285/06d7c463-298c-4c7e-a4c3-55d003aa91cb

- Measurement list - Appendix A – Additional COSMOS-UK site measurements

Fluxes are calculated from high frequency measurements using the eddy covariance technique, and estimated latent heat (LE, W m2) and actual evapotranspiration (ETa, mm) are derived from these (see Section 6). These flux data are available from COSMOS-UK sites where:

- The site has a 3D sonic anemometer measuring high frequency (20 Hz) wind components and sonic temperature

- There are no related instrumentation faults

- The site has a mobile network telemetry system

- There are no telemetry faults (including where slower data can be transferred but faster data cannot).

This dataset consists of:

- Site metadata (1 CSV file)

- Flux and ancillary data

  - One file for processed variables (1 CSV file per site)

  - One file for a subset of processed and quality controlled variables (1 CSV file per site)

- Dynamic metadata

  - Land cover change (Table 5 and Table 6)

  - Anemometer changes (Table 8)

- Dictionaries of data variables (CSV files)

# Parameters

This dataset contains fluxes and ancillary data derived from fast anemometer measurements, as shown in Table 1. Two levels of data are provided:

- Level 1 (L1) variables represent raw data, including those produced by EddyPro® Software v7.0.9 (LI-COR Biosciences, 2022) (see Section 6.3).  These have been provided to to preserve the original observations and calculations for full traceability, understanding and reproducibility of downstream processing.

- Level 2 (L2) variables have been further quality controlled (QC’d) following EddyPro and/or post-processing methods as described in Section 6. Many hydrological users may be principally interested in the estimated evapotranspiration (ET) from QC’d LE.

Each COSMOS-UK site offers a variety of other hydrometeorological and soil physics measurements; these are listed in Appendix A – Additional COSMOS-UK site measurements.

Table 1 Variables that are available in this dataset.

| Dataset | Variable | Units | Brief description |
| --- | --- | --- | --- |
| Both raw and QC’d 30 minute data (L1 & L2) | SITE_ID | - | Unique five character identifier for site |
|  | DATE_TIME | - | Datetime in YYYY-MM-DDTHH:MM:SSZ |
| QC’d 30 minute data (L2) | H | W m-2 | QC’d corrected sensible heat flux |
|  | H_FLAG | - | EddyPro H quality flag, 0=good, 1=suspicious, 2=bad |
|  | LE | W m-2 | QC’d estimated latent heat |
|  | ET | mm | Estimated evapotranspiration from QC’d LE |
|  | TAU | kg m-1 s-2 | QC’d corrected momentum flux |
|  | TAU_FLAG | - | EddyPro TAU quality flag, 0=good, 1=suspicious, 2=bad |
|  | SHF | W m-2 | Average soil heat flux |
| Raw 30 minute data (L1) | H | W m-2 | Raw sensible heat flux (corrected by EddyPro) |
|  | UN_H | W m-2 | Raw sensible heat flux (uncorrected by EddyPro) |
|  | H_STRG | W m-2 | Estimated storage sensible heat flux |
|  | H_SCF | - | Spectral correction factor for sensible heat flux |
|  | LAMBDA | MJ kg-1 | Derived latent heat of vaporisation |
|  | VPD | Pa | Ambient water vapour pressure deficit |
|  | TDEW | K | Ambient dew point temperature |
|  | WS_MAX | m s-1 | Maximum instantaneous wind speed |
|  | U_STAR | m s-1 | Friction velocity |
|  | L | m | Monin-Obukhov length |
|  | X_Z_D_L | - | Monin-Obukhov stability parameter ((z-d)/L) |
|  | BOWEN_RATIO | - | Sensible heat flux to latent heat flux ratio (Bowen ratio) |
|  | TAU | kg m-1 s-2 | Raw momentum flux (corrected by EddyPro) |
|  | UN_TAU | kg m-1 s-2 | Raw momentum flux (uncorrected by EddyPro) |
|  | TAU_SCF | - | Spectral correction factor for momentum heat flux |
|  | X_PEAK | m | Distance providing the highest contribution to turbulent fluxes |
|  | X_OFFSET | m | Distance providing <1% contribution to turbulent fluxes |
|  | X_90 | m | Distance providing 90% (cumulative) contribution to turbulent fluxes |
|  | TS | K | Mean temperature of ambient air measured by the anemometer |
|  | LE | W m-2 | Raw estimated latent heat |
|  | ET | mm | Estimated evapotranspiration |
|  | PE | mm | Potential evapotranspiration |

Full descriptions of each variable in this dataset are provided in the files:

“COSMOS-UK_FastFlux_L1_SH_2014-2024_Metadata.csv” and

“COSMOS-UK_FastFlux_L2_SH_2014-2024_Metadata.csv”

The column descriptions for these dictionary files are listed in Table 2.

Table 2 Columns in “COSMOS-UK_FastFlux_<level>_SH_2014-2024_Metadata.csv” metadata file.

| Column | Description |
| --- | --- |
| VARIABLE_ID | The ID used as column name in data file |
| VARIABLE_NAME | Name of variable |
| RESOLUTION | Time resolution of data |
| UNIT | Variable units where applicable |
| AGGREGATION | How the data has been aggregated over measured period |

# File information

## File format and naming

Data and metadata are provided in CSV files.

These files are listed in Table 3, and further details are provided in the sections below.

Table 3 Summary of the files included in this dataset.

|  | Filename | Summary |
| --- | --- | --- |
| Data files |  |  |
| 1. | COSMOS-UK_<site id>_FastFlux_L1_SH_<site start year>-2024.csv | Level 1 data files for each site: Flux data which has been processed by EddyPro. |
| 2. | COSMOS-UK_<site id>_FastFlux_L2_SH_<site start year>-2024.csv | Level 2 data files for each site: Flux data from EddyPro which has undergone post-processing. |
| Metadata files |  |  |
| 3. | COSMOS-UK_SiteMetadata_2014-2024.csv | Metadata about each of the COSMOS-UK sites. |
| 4. | COSMOS-UK_FastFlux_L1_SH_2014-2024_Metadata.csv | Metadata about the Level 1 (L1) data files. |
| 5. | COSMOS-UK_FastFlux_L2_SH_2014-2024_Metadata.csv | Metadata about the Level 2 (L2) data files. |

## File contents

The variables available in this dataset are provided in the supporting metadata files, as described above,

“COSMOS-UK_FastFlux_L1_SH_2014-2024_Metadata.csv” and

“COSMOS-UK_FastFlux_L2_SH_2014-2024_Metadata.csv”

Missing data due to instrumentation faults, telemetry issues, or error are indicated by “-9999”.

# Spatial coverage

This dataset includes 45 sites across the UK, as shown in Figure 1. Sites cover a variety of soil types and a range of natural and managed land covers. Full site metadata are included in the file,

“COSMOS-UK_SiteMetadata_2014-2024.csv”

Changing site land cover is further described in Section 6.3.

Figure 1 Map of COSMOS-UK sites.

The column descriptions for the site metadata file are listed in Table 4.

Table 4 Details of columns within the site metadata file.

| Column name | Unit | Description |
| --- | --- | --- |
| SITE_NAME | - | Full site name. |
| SITE_ID | - | Unique five character identifier for site. |
| START_DATE | - | Start date for site’s flux data in format DD/MM/YYYY HH:mm |
| END_DATE | - | End date for site’s flux data in format DD/MM/YYYY HH:mm |
| DATE_INSTALLED | - | Site’s deployment date in format DD/MM/YYYY. |
| DATE_DECOMMISSIONED | - | For sites no longer active, the end date in format DD/MM/YYYY. This is left blank if site is still active. |
| EASTING | m | Site coordinate. Coord ref system: OSGB36, EPSG SRID code: 27700. |
| NORTHING | m | Site coordinate. Coord ref system: OSGB36, EPSG SRID code: 27700. |
| EAST_NORTH_PROJECTION | - | EPSG SRID code: 27700 provided for reference. |
| LATITUDE | degrees | Site coordinate. Coord ref system: WGS84, EPSG SRID code: 4326. |
| LONGITUDE | degrees | Site coordinate. Coord ref system: WGS84, EPSG SRID code: 4326. |
| LAT_LONG_PROJECTION | - | EPSG SRID code: 4326 provided for reference. |
| ALTITUDE | m | Altitude of the site above sea level. |
| SOIL_TYPE | - | Informal soil type description derived from site inspection and soil sampling. |
| LAND_COVER | - | Main land cover as observed at the site. |
| BULK_DENSITY | g cm-3 | Mass per unit volume derived from soil samples taken from the site. |
| BULK_DENSITY_SD | g cm-3 | Standard deviation of bulk density across soil samples taken from site. |
| SOIL_ORGANIC_CARBON | g / g | Organic carbon content within dry soil mass, derived from soil samples taken from the site. |
| SOIL_ORGANIC_CARBON_SD | g / g | Standard deviation of soil organic carbon across soil samples taken from site. |
| LATTICE_WATER | g / g | Lattice and bound water. Water integrated into mineral structures within the soil. |
| LATTICE_WATER_SD | g / g | Standard deviation of lattice water across soil samples taken from site. |

Sites have cameras (PhenoCams) which provide a sub-daily view of the instruments and surrounding area throughout time, offering insight into site conditions including the growth stage of a crop and periods of standing water (Figure 2). Red, green, blue (RGB) data are extracted from the images, providing additional information about the site’s land cover. These data are available from Stanley et al. (2025).

Figure 2 Top: PhenoCam image from Fincham (FINCH) with the arable crop senescing; bottom: The Lizard (LIZRD) where standing water is present for a significant portion of the year.

# Temporal coverage and resolution

This dataset consists of available data from COSMOS-UK sites from 24-04-2014 to 31-07-2024. Sites were installed, altered and removed at different points during this time. Full site metadata are included in the file,

“COSMOS-UK_SiteMetadata_2014-2024.csv”

Data beyond 31-07-2024 will be made available in future publications.

All data are provided at 30 minute resolution. Flux data variables processed by EddyPro are provided as 30 minute averages of the raw 20 Hz data.

Brief site data completeness for chosen Level 1 flux variables in this dataset is shown in Figure 3. Data completeness for calculated evapotranspiration after quality control is shown is Figure 4.

Figure 3 Completeness of Level 1 flux data shown by combined availability of example variables (H, TAU, TS and U_STAR).

Figure 4 Data completeness for Level 2 evapotranspiration.

# Data acquisition and processing

## Experimental design/sampling regime

Most COSMOS-UK sites include a three-dimensional sonic anemometer (WindMaster, Gill Instruments Limited, UK) installed at a measurement height of 2.6 m, as shown in Figure 5. Each anemometer measures the three velocity components of atmospheric turbulence (m s-1) and sonic temperature (°C) at 20 Hz resolution, which are recorded on the site’s data logger and telemetered to UKCEH.

Figure 5 COSMOS-UK site instrumentation (Cooper et al., 2021). Note that measurements are made within a maintained compound where the land cover might not be representative of the surrounding area.

Sensible heat (H) was derived from anemometer-thermometer measurements and used in combination with co-located radiometer and soil heat flux measurements to estimate latent heat (LE) as a residual from the surface energy balance (Crowhurst et al., 2019; Dirmeyer et al., 2021). Incoming and outgoing short- and long-wave radiation (W m−2), and the subsequently derived net radiation, are measured using an NR01 four-component net radiometer (Hukseflux Thermal Sensors B.V., the Netherlands). Two HFP01-SC sensors (Hukseflux Thermal Sensors B.V., Delft, the Netherlands) each buried at a depth of 0.03 m are used to measure soil heat flux (W m−2). Further details for the measurements of all co-located 30 minute hydrometeorological and soil physics data are available in Stanley et al. (2025) and Cooper et al. (2021).

## Collection, calibration and maintenance methods

Monitoring data from the COSMOS-UK sites are telemetered as binary files to UKCEH and stored on a secure server. Raw data are processed (see Section 6.3), saved as CSV files, merged with processing outputs, quality controlled, and stored as 30 min timeseries data in an Oracle database.

Sites receive routine and responsive maintenance and instrument calibration following manufacturer recommendations to ensure optimum capture of high-quality data. Sensor metadata, including installation heights and firmware, are recorded and stored in an Oracle database.

## Data processing methods

To efficiently and accurately process COSMOS-UK flux data, EddyPro® Software v7.0.9 (LI-COR Biosciences, 2022) was used for all sites.

EddyPro processing requires and/or utilises the following site and sensor (meta)data to produce processed fluxes:

- Site geography: latitude, longitude, altitude (m)

- Site canopy height (m)

- Anemometer metadata: embedded software version, height, north alignment and offset

- Co-located Biomet data: air temperature (°C), relative humidity (%), barometric pressure (kPa), soil heat flux (W m-2), net radiation (W m-2), and shortwave incoming radiation (W m-2)

As canopy height was not measured at individual sites, estimated heights based on land cover were used to create a set of templates as shown in Table 5. Canopy height and leaf area index (LAI, closely related to ET) for grass templates were estimated from Byrne et al. (2005), and a wheat template for arable sites was created based on height data from the Agriculture and Horticulture Development Board and LAI from Liu et al. (2020).

Sonic anemometer heights remained static at a fixed measurement height of 2.6 m.

Table 5 Canopy height templates for each relevant land cover type.

|  | Wheat template (for arable sites) |  |  |  |  | Meadow grass template |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Code | Month | LAI | Crop height (m) |  | Code | Month | LAI | Crop height (m) |  |
|  | WH | January | 0.2 | 0.04 |  | MG | January | 1 | 0.11 |  |
|  |  | February | 0.4 | 0.07 |  |  | February | 1 | 0.11 |  |
|  |  | March | 1 | 0.09 |  |  | March | 1 | 0.11 |  |
|  |  | April | 2 | 0.2 |  |  | April | 1.3 | 0.15 |  |
|  |  | May | 4.5 | 0.35 |  |  | May | 1.3 | 0.15 |  |
|  |  | June | 5.3 | 0.79 |  |  | June | 1.8 | 0.21 |  |
|  |  | July | 2 | 0.79 |  |  | July | 1.8 | 0.21 |  |
|  |  | August | 0 | 0.05 |  |  | August | 1.8 | 0.21 |  |
|  |  | September | 0.5 | 0.07 |  |  | September | 1.5 | 0.17 |  |
|  |  | October | 0.5 | 0.07 |  |  | October | 1.3 | 0.15 |  |
|  |  | November | 0 | 0 |  |  | November | 1.2 | 0.14 |  |
|  |  | December | 0 | 0 |  |  | December | 1 | 0.11 |  |
|  | Cut grass template |  |  |  |  | Grazed grass template |  |  |  |  |
|  | Code | Month | LAI | Crop height (m) |  | Code | Month | LAI | Crop height (m) |  |
|  | CG | January | 0.5 | 0.05 |  | GG | January | 1 | 0.05 |  |
|  |  | February | 0.5 | 0.05 |  |  | February | 1 | 0.05 |  |
|  |  | March | 0.5 | 0.05 |  |  | March | 1 | 0.05 |  |
|  |  | April | 0.5 | 0.05 |  |  | April | 1 | 0.07 |  |
|  |  | May | 0.5 | 0.05 |  |  | May | 1 | 0.07 |  |
|  |  | June | 0.5 | 0.05 |  |  | June | 1 | 0.09 |  |
|  |  | July | 0.5 | 0.05 |  |  | July | 1 | 0.09 |  |
|  |  | August | 0.5 | 0.05 |  |  | August | 1 | 0.09 |  |
|  |  | September | 0.5 | 0.05 |  |  | September | 1 | 0.08 |  |
|  |  | October | 0.5 | 0.05 |  |  | October | 1 | 0.07 |  |
|  |  | November | 0.5 | 0.05 |  |  | November | 1 | 0.06 |  |
|  |  | December | 0.5 | 0.05 |  |  | December | 1 | 0.05 |  |

Each site’s land cover, and any changes to this, is recorded in Table 6. These land cover classifications are taken from UKCEH Land Cover Map (Rowland et al., 2017). The most appropriate land cover template (from Table 5) was chosen for each site and is listed in Table 6.

Table 6 Site land cover throughout deployment.

| Site ID | Land cover | Land cover template code | Land cover change date | Relevant site notes |
| --- | --- | --- | --- | --- |
| BALRD | Arable and horticulture | WH | - | - |
| BICKL | Improved grassland | MG | - | - |
| BUNNY | Arable and horticulture | WH | - | - |
| CARDT | Improved grassland | CG | - | - |
| CHIMN | Improved grassland | MG | - | - |
| CHOBH | Heather grassland | MG | - | - |
| COCHN | Improved grassland | GG | - | - |
| COCLP | Arable and horticulture | WH | - | - |
| CRICH | Improved grassland | GG | 10/05/2019 00:00 | - |
|  | Arable and horticulture | WH | - | - |
| EASTB | Improved grassland | GG | - | - |
| ELMST | Arable and horticulture | WH | - | - |
| EUSTN | Improved grassland | GG | - | - |
| FINCH | Arable and horticulture | WH | - | - |
| FIVET | Arable and horticulture | MG | - | - |
| GLENS | Heather | MG | - | - |
| GLENW | Improved grassland | GG | - | - |
| HADLW | Improved grassland | GG | - | - |
| HARTW | Improved grassland | GG | - | - |
| HENFS | Acid grassland | GG | - | - |
| HILLB | Improved grassland | GG | - | - |
| HLACY | Arable and horticulture | WH | - | - |
| HOLLN | Improved grassland | GG | - | Steep south-facing sloped site |
| HYBRY | Calcareous grassland | MG | - | - |
| LIZRD | Heather grassland | MG | - | Compound and surrounding area frequently experiences standing water |
| LODTN | Arable and horticulture | WH | - | - |
| LULLN | Calcareous grassland | MG | - | - |
| MOORH | Acid grassland | GG | - | - |
| MOREM | Improved grassland | GG | - | - |
| MORLY | Arable and horticulture | WH | - | - |
| NWYKE | Improved grassland | GG | 09/09/2019 00:00 | - |
|  | Arable and horticulture | WH | - | - |
| PLYNL | Acid grassland | GG | - | - |
| PORTN | Improved grassland | GG | - | - |
| RDMER | Arable and horticulture | WH | - | - |
| REDHL | Arable and horticulture | MG | - | Vineyard |
| RISEH | Improved grassland | MG | - | - |
| ROTHD | Arable and horticulture | WH | - | - |
| SHEEP | Improved grassland | GG | 03/10/2019 00:00 | - |
|  | Arable and horticulture | WH | - | - |
| SPENF | Arable and horticulture | WH | - | - |
| STGHT | Arable and horticulture | WH | - | - |
| STIPS | Improved grassland | MG | - | - |
| SYDLG | Improved grassland | MG | - | - |
| TADHM | Improved grassland | WH | - | Compound and surrounding area occasionally inundated with water |
| WADDN | Improved grassland | GG | - | - |
| WIMPL | Arable and horticulture | WH | - | - |
| WRTTL | Improved grassland | GG | - | - |

Anemometer firmware for each site, including when this changed, is listed in Appendix B – Anemometer firmware.

## Quality control

This dataset has undergone basic quality control (QC) for its first full release; the team plans to expand and diversify the QC methods for future versions as part of a single collated COSMOS-UK dataset. Users of the data are encouraged to contact the authors with questions, feedback, or observations regarding the dataset.

Quality of raw measurements is ensured, in part, through routine and responsive instrument maintenance and calibration, as issues with sensors are avoided or resolved as soon as possible.

EddyPro performs recommended raw flux data corrections, statistical analysis, and spectral analysis and corrections following methodologies widely adopted by the eddy covariance research community. Corrections and QC include: “w boost” bug for Gill WindMasters; angle-of-attack for Gill anemometers (Nakai et al., 2006; Van Der Molen et al., 2004); axis rotations for tilt (Wilczak et al., 2001); flux quality flags (Mauder & Foken, 2006); flux footprint estimation (Kormann & Meixner, 2001); spike count removal and statistical tests (Vickers & Mahrt, 1997); random uncertainty estimation (Finkelstein & Sims, 2001); and spectral correction factors (Ibrom et al., 2007; Massman, 2004; Moncrieff et al., 1997). The “w boost” bug correction was only applied to periods of time where a site’s anemometer was running the affected firmware version. For more detailed information regarding EddyPro calculations, please see the EddyPro support documentation and the Cooper et al. (2024) dataset from which much of the processing pipeline for this dataset was based.

EddyPro outputs for a site are merged and undergo the following post-processing quality checks:

- Poor quality H and TAU flags as identified by EddyPro

- Automated spike checks (following Papale et al. (2006))

- Automated range checks

- Manual visual check of the data

Poor quality data have been replaced in the Level 2 data with a value of ‘-9999’; raw unchanged data are provided in the Level 1 data files.

## Additional calculations

Derived variables in this dataset are calculated from QC’d measurements of radiation and soil heat flux. Variables derived from the calculated fluxes include:

  - LE (W m-2)

LE=RN-SHF-H

Equation 1 Latent heat (LE, W m-2) calculated from net radiation (RN, W m-2), the average of available soil heat flux (SHF, W m-2), and sensible heat (H, W m-2).

  - ET (mm)

ET=LE / λ / 1000

Equation 2 Evapotranspiration (ET, mm) calculated from latent heat (LE, W m-2) and the latent heat of vaporisation (λ, MJ kg-1).

Potential evapotranspiration (PE) is available in this dataset at 30-minute resolution. PE is derived using the Penman-Monteith method (FAO 56, Allen et al. (1998)), from the 30-minute resolution hydrometeorological data recorded at each COSMOS-UK monitoring site. The variables used to calculate this are: net radiation, soil heat flux, air temperature, relative humidity, wind speed, and atmospheric pressure. The derived PE values are the direct outputs of the calculation and have not been adjusted or constrained beyond the Penman-Monteith formulation. More information about the input variables can be found in the COSMOS-UK supporting documentation (Stanley et al., 2025).

# Acknowledgements

The authors gratefully acknowledge the COSMOS-UK team, the ENTRAIN team, and site hosts who have made this dataset possible.

Work on this dataset was funded by the Natural Environment Research Council under the NC-UK (NE/Y006208/1) and ENTRAIN (NE/S016244/1, NE/S016244/2) projects.

# References

Agriculture and Horticulture Development Board. (n.d.). Measurement of stem extension and stem reserves in wheat. AHDB.Org.Uk.

Allen, R. G., Pereira, L. S., Raes, D., & Smith, M. (1998). FAO Irrigation and Drainage Paper 56. In Crop evapotranspiration - Guidelines for computing crop water requirements.

Byrne, K. A., Kiely, G., & Leahy, P. (2005). CO2 fluxes in adjacent new and permanent temperate grasslands. Agricultural and Forest Meteorology, 135(1–4), 82–92. https://doi.org/10.1016/j.agrformet.2005.10.005

Cooper, H. M., Bennett, E., Blake, J., Blyth, E., Boorman, D., Cooper, E., Evans, J., Fry, M., Jenkins, A., Morrison, R., Rylett, D., Stanley, S., Szczykulska, M., Trill, E., Antoniou, V., Askquith-Ellis, A., Ball, L., Brooks, M., Clarke, M. A., … Winterbourn, B. (2021). COSMOS-UK: national soil moisture and hydrometeorology data for environmental science research. Earth System Science Data, 13(4), 1737–1757. https://doi.org/10.5194/essd-13-1737-2021

Cooper, H. M., Bodo, A., Burden, A., Callaghan, N., Crabtree, D. E., Cumming, A. M. J., D’Acunha, B., Evans, C. D., Journeaux, K. L., Jovani-Sancho, A. J., McNamara, N. P., Oakley, S., Rylett, D., Scarlett, P., Thornton, J., Winterbourn, J. B., Worrall, F., & Morrison, R. (2024). Meteorology, soil physics, and eddy covariance measurements of carbon dioxide, energy, and water exchange from a distributed network of sites across England and Wales, 2018-2023. NERC EDS Environmental Information Data Centre. https://doi.org/https://doi.org/10.5285/06d7c463-298c-4c7e-a4c3-55d003aa91cb

Crowhurst, D., Morrison, R., Evans, J., Cooper, H., Cumming, A., Fry, M., & Boorman, D. (2019). Actual evaporation data system for COSMOS-UK. In Hydro-JULES: Next Generation Land Surface and Hydrological Predictions, The Royal Society, London. https://nora.nerc.ac.uk/id/eprint/525105

Dirmeyer, P. A., Balsamo, G., Blyth, E. M., Morrison, R., & Cooper, H. M. (2021). Land‐Atmosphere Interactions Exacerbated the Drought and Heatwave Over Northern Europe During Summer 2018. AGU Advances, 2(2). https://doi.org/10.1029/2020AV000283

Finkelstein, P. L., & Sims, P. F. (2001). Sampling error in eddy correlation flux measurements. Journal of Geophysical Research: Atmospheres, 106(D4), 3503–3509. https://doi.org/10.1029/2000JD900731

Ibrom, A., Dellwik, E., Flyvbjerg, H., Jensen, N. O., & Pilegaard, K. (2007). Strong low-pass filtering effects on water vapour flux measurements with closed-path eddy correlation systems. Agricultural and Forest Meteorology, 147(3–4), 140–156. https://doi.org/10.1016/J.AGRFORMET.2007.07.007

Kormann, R., & Meixner, F. X. (2001). An analytical footprint model for non-neutral stratification. Boundary-Layer Meteorology, 99(2), 207–224. https://doi.org/10.1023/A:1018991015119

Liu, Y., Su, L., Wang, Q., Zhang, J., Shan, Y., & Deng, M. (2020). Comprehensive and quantitative analysis of growth characteristics of winter wheat in China based on growing degree days. Advances in Agronomy, 159, 237–273. https://doi.org/10.1016/bs.agron.2019.07.007

Massman, W. J. (2004). Concerning the measurement of atmospheric trace gas fluxes with open- and closed-path eddy covariance system: The WPL terms and spectral attenuation. In X. Lee, W. J. Massman, & B. E. Law (Eds.), Handbook of micrometeorology: a guide for surface flux measurements (pp. 133–160). Kluwer Academic.

Mauder, M., & Foken, T. (2006). Impact of post-field data processing on eddy covariance flux estimates and energy balance closure. Meteorologische Zeitschrift, 15(6), 597–609. https://doi.org/10.1127/0941-2948/2006/0167

Moncrieff, J. B., Massheder, J. M., De Bruin, H., Elbers, J., Friborg, T., Heusinkveld, B., Kabat, P., Scott, S., Soegaard, H., & Verhoef, A. (1997). A system to measure surface fluxes of momentum, sensible heat, water vapour and carbon dioxide. Journal of Hydrology, 188–189(1–4), 589–611. https://doi.org/10.1016/S0022-1694(96)03194-0

Nakai, T., Van Der Molen, M. K., Gash, J. H. C., & Kodama, Y. (2006). Correction of sonic anemometer angle of attack errors. Agricultural and Forest Meteorology, 136(1–2), 19–30. https://doi.org/10.1016/J.AGRFORMET.2006.01.006

Papale, D., Reichstein, M., Aubinet, M., Canfora, E., Bernhofer, C., Kutsch, W., Longdoz, B., Rambal, S., Valentini, R., Vesala, T., & Yakir, D. (2006). Towards a standardized processing of Net Ecosystem Exchange measured with eddy covariance technique: Algorithms and uncertainty estimation. Biogeosciences, 3(4), 571–583. https://doi.org/10.5194/BG-3-571-2006

Rowland, C. S., Morton, R. D., Carrasco, L. ., McShane, G., O’Neil, A. W., & Wood, C. M. (2017). Land Cover Map 2015. NERC Environmental Information Data Centre. https://doi.org/https://doi.org/10.5285/6c6c9203-7333-4d96-88ab-78925e7a4e73

Stanley, S., Antoniou, V., Askquith-Ellis, A., Ball, L., Bennett, E. S., Blake, J. R., Boorman, D. B., Brookes, M., Clarke, M. A., Cooper, H. M., Cowan, N. J., Cumming, A., Evans, J. G., Farrand, P., Fry, M., Harvey, D., Houghton-Carr, H., Howson, T., Jimenez-Arranz, G., … Winterbourn, J. B. (2025). Daily and sub-daily hydrometeorological and soil moisture data (2013-2024) [COSMOS-UK] - EIDC. Https://Doi.Org/10.5285/2dce161d-2fab-47bb-9fe6-38e7ed1ae18a. https://doi.org/https://doi.org/10.5285/2dce161d-2fab-47bb-9fe6-38e7ed1ae18a

Van Der Molen, M. K., Gash, J. H. C., & Elbers, J. A. (2004). Sonic anemometer (co)sine response and flux measurement: II. The effect of introducing an angle of attack dependent calibration. Agricultural and Forest Meteorology, 122(1–2), 95–109. https://doi.org/10.1016/J.AGRFORMET.2003.09.003

Vickers, D., & Mahrt, L. (1997). Quality Control and flux sampling problems for tower and aircraft data. Journal of Atmopsheric and Oceanic Technology, 14(3), 512–526.

Wilczak, J. M., Oncley, S. P., & Stage, S. a. (2001). Sonic anemometer tilt correction algorithms. Boundary-Layer Meteorology, 99(1), 127–150. https://doi.org/10.1023/A:1018966204465

# Appendix A – Additional COSMOS-UK site measurements

The additional measurements available from COSMOS-UK sites are listed in Table 7 and described in full in Stanley et al. (2025): doi.org/10.5285/2dce161d-2fab-47bb-9fe6-38e7ed1ae18a.

Table 7 Hydrometeorological and soil physics measurements available from COSMOS-UK sites.

| Dataset | Variable | Units | Brief description |
| --- | --- | --- | --- |
| COSMOS-UK 2025: 30 minute data | LWIN | W m-2 | Incoming longwave radiation |
|  | LWOUT | W m-2 | Outgoing longwave radiation |
|  | SWIN | W m-2 | Incoming shortwave radiation |
|  | SWOUT | W m-2 | Outgoing shortwave radiation |
|  | RN | W m-2 | Net radiation |
|  | PRECIP | Mm | Precipitation from Pluvio |
|  | PRECIP_TIPPING | Mm | Precipitation from tipping bucket |
|  | PRECIP_RAINE | Mm | Precipitation from rain[e] |
|  | PA | hPa | Atmospheric Pressure |
|  | TA | °C | Air Temperature |
|  | WS | W m-2 | Wind speed |
|  | WD | Degrees | Wind direction |
|  | Q | g m-3 | Absolute Humidity |
|  | RH | % | Relative Humidity |
|  | SNOW_DEPTH | Mm | Snow depth |
|  | UX | m s-1 | X component of wind speed |
|  | UY | m s-1 | Y component of wind speed |
|  | UZ | m s-1 | Z component of wind speed |
|  | G1 | W m-2 | Heat flux 1 |
|  | G2 | W m-2 | Heat flux 2 |
|  | TDT1_TSOIL TDT2_TSOIL | °C | Point soil temperature at 10 cm* |
|  | TDT1_VWC TDT2_VWC | % | Point soil moisture at 10 cm* |
|  | TDT3_TSOIL TDT4_TSOIL | °C | Point soil temperature at 5 cm* |
|  | TDT3_VWC TDT4_VWC | % | Point soil moisture at 5 cm* |
|  | TDT5_TSOIL TDT6_TSOIL | °C | Point soil temperature at 15 cm* |
|  | TDT5_VWC TDT6_VWC | % | Point soil moisture at 15 cm* |
|  | TDT7_TSOIL TDT8_TSOIL | °C | Point soil temperature at 25 cm* |
|  | TDT7_VWC TDT8_VWC | % | Point soil moisture at 25 cm* |
|  | TDT9_TSOIL TDT10_TSOIL | °C | Point soil temperature at 50 cm* |
|  | TDT9_VWC TDT10_VWC | % | Point soil moisture at 50 cm* |
|  | STP_TSOIL2 | °C | Point soil temperature at depth 2 cm |
|  | STP_TSOIL5 | °C | Soil temperature profile at depth 5 cm |
|  | STP_TSOIL10 | °C | Soil temperature profile at depth 10 cm |
|  | STP_TSOIL20 | °C | Soil temperature profile at depth 20 cm |
|  | STP_TSOIL50 | °C | Soil temperature profile at depth 50 cm |
| COSMOS-UK 2025: Daily data | COSMOS_VWC | % | Field-scale soil moisture from CRNS |
|  | CTS_MOD_CORR | Count | Corrected moderated counts from CRNS |
|  | D86_75m | Cm | Effective depth of CRNS (D86 at 75 m) |
|  | SNOW | - | Snow days |
|  | SNOW_DEPTH | Mm | Snow depth |
|  | SWE | Mm | Snow water equivalent from CRNS |
|  | ALBEDO | - | Albedo |
|  | PE | Mm | Potential evapotranspiration |
|  | GCC | - | Green colour content from PhenoCam image |

* See full instrumentation and installation details in Stanley et al. (2025). Please note that since publication of Stanley et al. (2025), the radiometer data used in this evapotranspiration dataset have been updated to account for a sensor issue at the Riseholme (RISEH) site.

# Appendix B – Anemometer firmware

Anemometer firmware for each site, including when this changed, is listed in Table 8. Firmware is recorded to ensure any bugs are accounted and corrected for (see Section 6.4).

Table 8 Anemometer firmware changes (± 1 day).

| Site ID | Start date | Firmware version |
| --- | --- | --- |
| BALRD | 15/05/2014 17:30 | 2329 601 01 |
| BALRD | 03/08/2017 10:30 | 2329 701 01 |
| BALRD | 22/09/2021 00:00 | 2329 700 01 |
| BICKL | 28/01/2015 17:00 | 2329 600 |
| BICKL | 14/02/2019 00:00 | 2329 701 01 |
| BUNNY | 27/01/2015 00:30 | 2329 600 |
| BUNNY | 29/11/2017 10:00 | 2329 601 01 |
| BUNNY | 11/09/2019 00:00 | 2329 701 01 |
| CARDT | 24/06/2015 10:00 | 2329 600 01 |
| CARDT | 01/06/2021 10:00 | 2329 701 |
| CGARW | 29/06/2016 11:00 | 2329 700 01 |
| CGARW | 10/08/2021 09:00 | 2329 600 01 |
| CGARW | 25/01/2024 11:30 | 2329 700 01 |
| CHIMN | 12/02/2020 00:00 | 2329 701 01 |
| CHOBH | 24/02/2015 14:00 | 2329 601 01 |
| CHOBH | 17/08/2017 09:43 | 2329 601 |
| CHOBH | 17/08/2017 09:43 | 2329 701 01 |
| COCHN | 23/08/2017 00:00 | 2329 700 01 |
| COCLP | 21/11/2014 14:30 | 2329 600 |
| COCLP | 31/10/2019 12:00 | 2329 700 01 |
| CRICH | 02/12/2014 15:00 | 2329 600 |
| CRICH | 03/09/2019 00:00 | 2329 601 03 |
| EASTB | 13/08/2014 11:30 | 2329 600 01 |
| EASTB | 25/08/2021 09:00 | 2329 700 01 |
| ELMST | 11/08/2016 15:30 | 2329 700 01 |
| ELMST | 14/07/2021 09:00 | 2329 701 01 |
| EUSTN | 31/03/2016 16:00 | 2329 700 01 |
| EUSTN | 29/04/2021 12:00 | 2329 600 01 |
| FINCH | 07/06/2017 00:00 | 2329 601 |
| FINCH | 07/06/2017 00:00 | 2329 701 01 |
| FINCH | 18/05/2023 12:30 | 2329 700 01 |
| FIVET | 26/06/2018 15:30 | 2329 601 |
| FIVET | 26/06/2018 15:30 | 2329 701 01 |
| GISBN | 15/08/2014 12:00 | 2329 600 01 |
| GISBN | 29/08/2019 12:50 | 2329 600 |
| GISBN | 29/08/2019 12:50 | 2329 701 01 |
| GLENS | 14/05/2014 11:30 | 2329 600 |
| GLENS | 18/06/2019 15:00 | 2329 701 01 |
| GLENW | 15/06/2016 15:00 | 2329 700 01 |
| GLENW | 05/08/2021 09:00 | 2329 701 01 |
| HADLW | 27/10/2016 14:30 | 2329 700 01 |
| HADLW | 21/07/2021 09:00 | 2329 701 01 |
| HARTW | 20/05/2014 13:30 | 2329 600 |
| HARTW | 29/03/2018 10:00 | 2329 601 01 |
| HARTW | 11/11/2020 00:00 | 2329 701 01 |
| HENFS | 17/12/2015 13:00 | 2329 600 01 |
| HENFS | 25/06/2020 10:00 | 2329 701 01 |
| HILLB | 14/06/2016 17:00 | 2329 700 01 |
| HILLB | 29/09/2020 12:00 | 2329 701 01 |
| HILLB | 04/08/2021 09:00 | 2329 700 01 |
| HLACY | 11/04/2018 00:00 | 2329 700 01 |
| HLACY | 23/05/2023 10:30 | 2329 600 01 |
| HOLLN | 25/03/2014 12:30 | 2329 600 01 |
| HOLLN | 20/05/2021 09:00 | 2329 701 |
| HYBRY | 16/08/2017 12:30 | 2329 700 01 |
| HYBRY | 08/06/2022 09:00 | 2329 701 01 |
| LIZRD | 17/10/2014 12:00 | 2329 600 |
| LIZRD | 25/05/2019 00:00 | 2329 701 01 |
| LODTN | 26/04/2016 15:00 | 2329 700 01 |
| LODTN | 28/04/2022 12:00 | 2329 600 01 |
| LULLN | 16/12/2014 15:30 | 2329 600 01 |
| LULLN | 18/06/2020 00:00 | 2329 701 01 |
| MOORH | 04/12/2014 11:30 | 2329 600 |
| MOORH | 18/09/2019 00:00 | 2329 701 01 |
| MOORH | 13/05/2021 09:00 | 2329 700 01 |
| MOREM | 16/11/2018 12:30 | 2329 701 01 |
| MORLY | 14/05/2014 05:00 | 2329 600 |
| MORLY | 24/09/2019 11:00 | 2329 701 01 |
| NWYKE | 16/10/2014 09:00 | 2329 600 |
| NWYKE | 05/12/2018 09:30 | 2329 600 01 |
| NWYKE | 11/05/2022 09:00 | 2329 701 01 |
| PLYNL | 05/11/2014 12:30 | 2329 600 |
| PLYNL | 03/12/2015 12:00 | 2329 601 03 |
| PLYNL | 16/11/2017 16:00 | 2329 601 01 |
| PLYNL | 19/09/2019 11:00 | 2329 701 01 |
| PORTN | 18/12/2014 10:30 | 2329 601 01 |
| PORTN | 08/08/2017 10:00 | 2329 701 01 |
| PORTN | 08/11/2022 00:00 | 2329 700 01 |
| RDMER | 10/02/2015 10:30 | 2329 600 01 |
| REDHL | 18/02/2016 16:00 | 2329 600 01 |
| REDHL | 20/07/2021 09:00 | 2329 601 01 |
| REDHL | 20/07/2021 09:00 | 2329 701 |
| RISEH | 04/05/2016 16:00 | 2329 700 01 |
| ROTHD | 25/07/2014 08:00 | 2329 600 01 |
| ROTHD | 23/10/2019 17:30 | 2329 701 01 |
| SHEEP | 11/02/2020 00:00 | 2329 701 01 |
| SOURH | 19/11/2014 11:00 | 2329 600 01 |
| SOURH | 07/10/2020 00:00 | 2329 701 01 |
| SPENF | 23/11/2016 10:00 | 2329 701 01 |
| SPENF | 18/05/2021 09:00 | 2329 701 |
| SPENF | 01/09/2022 14:00 | 2329 701 01 |
| STGHT | 19/08/2015 06:30 | 2329 600 01 |
| STGHT | 22/10/2020 12:00 | 2329 701 01 |
| STIPS | 06/11/2014 15:00 | 2329 600 |
| STIPS | 12/07/2018 11:30 | 2329 701 01 |
| SYDLG | 28/11/2018 12:30 | 2329 701 01 |
| TADHM | 14/10/2014 12:00 | 2329 601 01 |
| TADHM | 11/12/2019 00:00 | 2329 701 01 |
| WADDN | 13/02/2020 00:00 | 2329 701 01 |
| WIMPL | 10/09/2019 00:00 | 2329 701 01 |
| WRTTL | 04/07/2017 00:00 | 2329 701 01 |
| WRTTL | 20/07/2022 09:00 | 2329 700 01 |
| WRTTL | 19/07/2023 12:30 | 2329 600 01 |