# Supporting documentation to “LiDAR-derived canopy height model and digital terrain model of Sabah Biodiversity Experiment, Malaysia, 2020”

This dataset contains raster layers at 1-m resolution representing the canopy height model (CHM) and digital terrain model (DTM) of the Sabah Biodiversity Experiment (SBE), Sabah, Malaysia. CHM and DTM were created from airborne LiDAR point cloud data collected at SBE in 2020. The data were collected to assess the effectiveness of different restoration interventions for speeding up the recovery of logged tropical forests in SE Asia.

# File format

GeoTIFF

# File names and/or naming convention

sbe_chm_2020.tiff – Canopy height model in meters above ground level

sbe_dtm_2020.tiff – Digital terrain model in meters above sea level

# Spatial coverage

Spatial extent: Sabah Biodiversity Experiment (SBE), Sabah, Malaysia
Bounds: 570250m, 574000m, 560750m, 564250m (xmin, xmax, ymin, ymax)

Spatial resolution: 1 meter

Coordinate reference system: WGS 84 / UTM zone 50N (EPSG:32650)

# Temporal coverage and resolution

Temporal coverage: March 2020

Temporal resolution: n/a

# Collection/Generation/Transformation methods

Canopy height model (CHM) and digital terrain model (DTM) were generated using the LiDAR processing pipeline described in Fischer et al. (2024). Specifically, ground points were classified using the LASground function in LAStools to create the DTM, while the CHM was generated using a locally adaptive spike-free algorithm.

# Fieldwork and/or laboratory instrumentation

Airborne LiDAR data were acquired using a RIEGL scanner mounted on a helicopter flown at 250 m above ground.

# Quality control

To ensure quality control we followed the steps described in Fischer et al. (2024), including (1) removing any areas affected by interference with low clouds or water bodies on the ground, (2) any areas with <2 LiDAR pulses per m2, (3) any areas where scan angles exceeded 20°.

# References

Fischer et al. (2024). Robust characterisation of forest structure from airborne laser scanning – A systematic assessment and sample workflow for ecologists. Methods in Ecology and Evolution, 15, 1873–1888 (https://doi.org/10.1111/2041-210X.14416)