Grid-to-Grid model simulations of daily anthropogenically influenced river flows for England and natural flows for Great Britain, 1961-2021

Ponnambalam Rameshwaran, Victoria A. Bell, Helen N. Davies and Mark D. Rhodes-Smith

UK Centre for Ecology and Hydrology, Wallingford, Oxfordshire, OX10 8BB, UK

## Summary

These datasets provide Grid-to-Grid (G2G) hydrological model estimates of daily mean river flow (m3s-1) on a 1 km x 1 km grid. The river flows provided here are generated using the G2G model for Great Britain driven by observation-based meteorological data. They provide spatially consistent simulations of natural and anthropogenically influenced daily mean river flows. The natural flows are for the period 1st January 1961 to 31st December 2021, the observed-anthropogenically influenced flows are for 1st January 1999 to 31st December 2014 and the mean-anthropogenically influenced flows are for 1st January 1961 to 31st December 2021. A complementary dataset provides G2G model estimates of river flows across Great Britain when driven by UKCP18 Regional (12 km) climate projections (Dataset Record: CS-NOW: Gridded future projections of natural and artificially influenced river flows (1980 to 2080)).

The G2G hydrological model spatial river flow simulations are available as follows:

- 1 km × 1 km gridded daily mean river flows (m3s-1) for two spatial regions:

- Natural river flows across the whole of Great Britain, excluding Orkney and Shetland islands (Figure 1a)

- Anthropogenically influenced river flows across the whole of England (Figure 1b)

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 1. (a) Natural flow domain (b) anthropogenically influenced flow domain and (c) locations of 626 catchments with gauging stations

## Hydrological Model - Grid-to-Grid (G2G)

The Grid-to-Grid model (G2G: (Bell et al., 2009)) is a grid-based, spatially distributed hydrological model which has recently been enhanced to take account of recorded monthly abstractions and annual discharges (Rameshwaran et al., 2022). Figure 2 provides a visual schematic of the overall approach for the anthropogenically influenced flow simulations.

.

Figure 2. Schematic of the overall approach for the anthropogenically influenced flow simulations (Rameshwaran et al., 2022).

While in practice, hydrological modelling has been undertaken for mainland Britain, model estimates for anthropogenically influenced catchments are only available for English catchments, albeit with a modest overlap of the Welsh border (Figure 1c). This is because, to date, abstraction and discharge data are only available for England (Rameshwaran et al., 2022).

## Anthropogenic Influence Data

The anthropogenic influence data (licences/consents/returns) used here have been sourced from the UK Environment Agency (EA) under licence. Specifically, monthly surface and groundwater abstraction data were obtained from the EA’s NALD database for 1999 to 2014, and annual discharges (2017) and Hands-off-Flow (HoF) conditions were obtained from the EA’s Water Resources GIS (WRGIS) database. The impact of other anthropogenic influences, such as reservoir impoundments and releases, is not considered here, and the G2G does not explicitly simulate lakes and reservoirs or abstractions from them.

Rameshwaran et al. (2022) outlined the methodology used for the conversion of these point abstraction and discharge data into monthly 1 km × 1 km grids across England for each of the 57 primary uses (see Table 1 in Rameshwaran et al. (2022)). These 1 km resolution data are now available on CEDA (Dataset Record: Gridded actual groundwater, surface water and tidal water abstraction, discharge and Hands-off Flow datasets for England (1999 to 2014) together with a dedicated data document (Rameshwaran, Bell, Davies, Sadler, Beverton, & Thornton, 2025). Pragmatic assumptions were made to resolve apparent inconsistencies in licence returns or to overcome uncertainties associated with missing information. One important factor to consider was that the derived monthly grids represent total water abstracted but do not take into account water immediately returned to the river by the licence holder (e.g., from vegetable washing). Although in reality, abstractions and discharge datasets are related, it has not been possible to link individual abstraction licences and discharge permits. To minimise the uncertainty in net abstraction associated with these immediate water returns, Rameshwaran et al. (2022) made use of the loss factor term associated with surface water abstractions (see Table 1 in Rameshwaran et al. (2022)). The four loss factor categories (EA, 2020) are High (100%), Medium (60%), Low (3%) and Very Low (0.3%). For three of these categories (high, medium and low abstraction losses), an assumption is made here that any water returned to the river is accounted for in the discharge dataset. However, for the abstractions associated with ‘Very Low’ losses (termed ‘Through Flows’, e.g., Fish Pass/Canoe Pass, River Recirculation, Hydroelectric Power Generation), the returns are so high and localised that an assumption that returns are included in discharges cannot be made. For these ‘Very Low’ loss abstractions, we have assumed that only 0.3% of the water volume is removed mainly due to conveyance losses. This assumption is discussed in more detail in Rameshwaran et al. (2022).

## Hands-off-Flow (HoF) Conditions

Across England, surface water abstractions are often constrained by a HoF flow value (m3 day-1), requiring abstraction to cease (or reduce) if the river flow falls below this threshold. This requirement is designed to prevent the detrimental impact of excessive abstraction on the environment and protect river ecosystems during periods of low flows, particularly during drier years. This means that during drought periods when the river flow is below the local HoF threshold, the licence holder will be temporarily unable to abstract their full licensed amount. The HoF condition is often applied with reference to river flows at a different location, the “HoF impact location”. HoF impact locations are referenced in the EA’s WRGIS system and consist of (sometimes non-local) sub-catchments or waterbodies (rivers), rather than point locations. Previously in the G2G model, there was an assumption that the impact location was the local abstraction location (Rameshwaran et al., 2022), but for the current simulations, the G2G was enhanced to take account of non-local HoF impact locations for surface water abstractions. Specifically, the most downstream 1 km × 1 km grid-cell of each HoF impact waterbody was used as the “HoF impact grid-cell”, at which G2G-simulated flows are compared with the HoF condition to establish whether an abstraction can take place (Bell et al., 2023).

## Driving Data

The driving data used here consists of 1 km × 1 km daily precipitation from CEH-GEAR (Gridded Estimates of Areal Rainfall; Keller et al. (2015), Tanguy et al. (2016)), and 40 km x 40 km monthly short grass PE from the Met Office Rainfall and Evaporation Calculation System (MORECS; (Hough & Jones, 1997). For use in G2G, the 40 km x 40 km PE data were copied to each of the corresponding 1600 1 km × 1 km grid cells of the hydrological model grid and then divided equally over each time step. The 1 km × 1 km resolution daily precipitation was divided equally over each 15-minute model time step.

## Performance Assessment using Historical Observations

The G2G is a national-scale hydrological model that typically estimates a value of river flow for every 1 km × 1 km grid-cell across Great Britain and does not require at-site calibration using flow observations. It has one spatially consistent configuration for the whole model domain and is able to represent a wide range of hydrological regimes due to use of spatial datasets of terrain, soil/geology and land-cover in its construction. Until recently, it provided output consisting of spatially consistent gridded estimates of natural river flows for gauged and ungauged rivers across Great Britain. Past performance assessments for the natural simulations show that G2G estimates river flows reasonably well, performing best for catchments with a natural flow regime or little anthropogenic influences but less well where the regime is influenced by anthropogenic influences such as abstractions and discharges  (e.g. Bell et al., 2009; Rudd et al., 2017).

The performance of the G2G hydrological model was assessed using observed daily river flows at monitoring stations across England (Bell et al., 2023; Rameshwaran et al., 2022). For the assessment, driving data for the G2G consisted of gridded daily HadUK-Grid 1 km × 1 km precipitation and monthly MORECS data (Hough & Jones, 1997), which provides a national gridded potential evapotranspiration (PE) product on a 40 km grid. Time-series of daily minimum and maximum temperature datasets (Hollis et al., 2019) were also required to drive the snow module (Bell et al., 2016). G2G model simulations were undertaken for the historical period 1st January 1999 to 31st December 2014 for which observed anthropogenic influence data are available using time series of monthly observed datasets.

To assess the G2G model performance, the simulated daily river flows were compared with observed daily river discharge data for 626 gauging stations across England readily available on the UK National River Flow Archive (NRFA, nrfa.ceh.ac.uk and Dixon et al. (2013)).

Previously, Rameshwaran et al. (2022) showed that model performance is improved through the use of actual abstraction and discharge data across England between 1999 and 2014, particularly in catchments for which anthropogenic influences are discharge-dominated. The study also showed that the main improvement is in simulation of low flows. Similarly, the comparison of simulated and observed river flows undertaken here for 626 catchments indicates that the main improvement is in the simulation of low flows, for which the median performance is improved by 12.5%, while the improvement in the simulation of high flows is a more modest 1.5% (Bell et al., 2023). For only a very small number of catchments, observed and mean monthly abstractions were found to differ substantially over the 16-year assessment period, often because of changes in abstraction licences. For most other catchments, abstraction changes over this period were more modest, which was why observations of abstractions could be replaced by a monthly mean observed value with very little impact on river flow simulations.

## Summary of G2G Model Output Datasets

The G2G simulations provided here consist of files 1 km × 1 km resolution spatial grids of NATURAL and anthropogenically-impacted (Actual-Impacted and Mean-Impacted) daily mean river flows:

- NATURAL Flows: Simulated daily river flows (m3s-1) without anthropogenic impacts (i.e. no abstractions or discharges).

- Observed-Anthropogenic Flows: Simulated daily river flows (m3s-1) estimated using time-series of observed monthly abstractions for the period January 1999 to December 2014, and discharges.

- Mean-Anthropogenic Flows: Simulated daily river flows (m3s-1) estimated using mean monthly observed abstractions for the period January 2010 to December 2014 and discharges.

The time series of gridded flows are all provided in NetCDF format. River flows are only provided for G2G river grid-cells with an upstream catchment area of 15 km2 or above, for other grid-cells the value is set to missing.

Gridded flow estimates are provided for 2 spatial domains (Figures 1a&b):

- The gridded data for the NATURAL flows are provided for Great Britain: this is a 601 km × 980 km spatial domain on the British National Grid from lower left corner (55000, 0) to top right (656000, 980000) metres.

- The gridded data for the anthropogenic impacted flows are provided for England: this is a smaller 601 km × 660 km spatial domain on the British National Grid from lower left corner (55000, 0) to top right (656000, 660000) metres. The England region was delineated using the Local Authority Districts Boundaries UK BFC shapefile (December 2023: www.data.gov.uk). This region was extended into Wales to include upstream areas of the Severn catchment that drain into England, but abstractions and discharges in Wales were not included in the analysis as they were unavailable.

Table 1 presents the file naming convention for each of the available flow datasets.

Table 1. File naming convention for flow simulations (m3s-1).

| Anthropogenic Data used in G2G | Name of NetCDF file | Spatial domain |
| --- | --- | --- |
| None | G2G_DailyRiverFlow_NATURAL_19610101_20211231.nc | Great Britain |
| Observed abstractions and discharges | G2G_DailyRiverFlow_Observed_Anthropogenic_19990101_20141231.nc | England |
| 5-year mean abstractions and discharges | G2G_DailyRiverFlow_Mean_Anthropogenic_19610101_20211231.nc | England |

## Additional Datasets

Four further datasets are provided to aid the use of the flow data provided here:

- G2G_Drainage_Direction_Grid_1km.nc - Digitally derived outflow drainage directions in D8 format. These are derived on a 1 km x 1 km resolution across Great Britain in similar manner to the updated outflow drainage directions in Davies et al. (2024) but are not identical .

- G2G_Catchment_Area_Grid_1km.nc - Digitally derived catchment area (km2). The catchment area dataset provided here represents the number of upstream 1 km x 1 km cells draining to each cell and includes the cell itself. This dataset can be useful to identify the location of and area draining to ungauged catchments. This dataset is consistent with the drainage direction grid (above).

- G2G_Coastal_Cell_Grid_1km.nc - Digitally derived coastal grid-cells identifying land grid-cells for which outflows go to sea. This dataset is a (0,1) grid where 0 = inland 1 = coastal land outflow cells and will enable users to estimate river flows to sea. This dataset is consistent with the drainage direction grid (above).

- G2G_626_NRFA_Station_ID_Grid.csv - Estimated locations of 626 river flow gauging stations on the 1 km x 1 km grid (Appendix A: Table A1). The 1km x 1km grid provides the best locations corresponding to 626 gauging stations, referenced by their integer NRFA station number (nrfa.ceh.ac.uk). At these locations, the G2G flow estimates can be compared to observed river flows. The (integer) file format sets station to 0 for land, -9999 for sea, and the NRFA station number at gauging station locations.

Individual rivers can be identified from the geo-referenced gridded flows provided and the spatial resolution of the flow data provided here is on a 1 km × 1 km grid. All of these additional datasets are provided on the same spatial domain as the NATURAL flows.

All of these datasets are similar to those provided with the MaRIUS G2G dataset (Bell et al., 2018) and UK-SCAPE (Kay, 2021) but the versions here take account of minor changes to the G2G model setup since then.

## Example Plots

By way of example, Figure 3 provides an example plot of daily mean river flows (m3s-1) for natural and Actual-Impacted flow domains for 1st January 2010.

| (a) |  | (b) |
| --- | --- | --- |
|  | Flow (m3s-1) |  |

Figure 3. Example map of gridded daily mean river flows (m3s-1) for 1st January 2010.

## Associated Datasets

The gridded actual groundwater, surface water and tidal water abstraction, discharge and Hands-off Flow datasets for England (1999 to 2014) are available on CEDA:

- Data document: Data Documentation Identifier: https://doi.org/10.5281/zenodo.13746897 (Rameshwaran, Bell, Davies, Sadler, Beverton, & Thornton, 2025)

- Data: https://dx.doi.org/10.5285/18886f95ba84447f997efac96df456ad. (Rameshwaran, Bell, Davies, Sadler, Beverton, Thornton, et al., 2025)

The CS-NOW UKCP18-driven G2G flow estimates for historical and future periods are available on CEDA:

- Data document: Data Documentation Identifier: https://doi.org/10.5281/zenodo.10958783 (Rameshwaran, Bell, Davies, Baron, Keller, & Hannaford, 2025)

- Data: https://dx.doi.org/10.5285/cf66055440344d7eb9b6f834e81736c6 (Rameshwaran, Bell, Davies, Baron, Keller, Hannaford, et al., 2025).

## How to use the Data: Suggested Applications and Limitations

These river flow simulations could be used to investigate potential changes in high and low flow characteristics for both individual catchments or on a 1 km x 1 km grid across England, particularly as the flow data are provided at a daily timestep. The flows can also be used as input to flood inundation models requiring spatially consistent inputs of flows across a wide area.

It’s important to be aware of the assumptions, approximations and limitations of the simulated river flow data before using them:

- Conversion of observed monthly discharges and annual discharges to a consistent 1 km x 1 km grid across England required many assumptions and pragmatic estimates to be made. These are summarised in Rameshwaran et al. (2022)

- The G2G hydrological model has been shown to perform well for a wide range of catchments across the UK, including for the modelling of high flows/floods and low flows/droughts (Bell et al., 2016; Bell et al., 2009; Formetta et al., 2018; Kay, 2021; Rudd et al., 2017). The performance was typically better for catchments with more natural flow regimes, as the model did not until recently account for anthropogenic influences such as abstractions/discharges. Recent performance analyses that take account of abstractions/discharges indicate G2G performance is now enhanced in many anthropogenically-impacted catchments, particularly low-flow estimates (Rameshwaran et al., 2022).

- While the effect of urban/suburban land-cover on runoff is accounted for in the G2G, the effect of lake/reservoir storage and regulation is generally neglected; lake and many reservoir grid-cells are treated as though they were rivers, and water transfers are not included in G2G. This means that abstractions from lakes and rivers will be greatly underestimated in the G2G flow simulations. Similarly, although G2G flow estimates are often provided for coastal/tidal regions, the influences of tides and tidal abstractions are not included in the river flow simulations.

- The first two years of the G2G model flow estimates should be treated as a warm-up period.

- NetCDF format is commonly used to store and distribute scientific data, and the CF-compliant NetCDF files provided here each have a header containing metadata. The flow data themselves have 3 dimensions, Latitude, Longitude, and time, and a range of tools can be used to select data for particular coordinates or a specific region.

## Acknowledgements

G2G model development for anthropogenically influenced flow simulations discharge was supported by the Natural Environment Research Council award number NE/L010208/1 as part of the UK Drought and Water Scarcity programme - MaRIUS (Managing the Risks, Impacts and Uncertainties of droughts and water Scarcity) and NE/R016429/1 as part of the UK-SCAPE programme delivering National Capability. Future projections of natural and artificially influenced river flow (1980 to 2080) simulations work was supported by the Climate Services for a Net Zero Resilient World (CS-N0W) project Commissioned by the UK Department for Energy Security and Net Zero (DESNZ). This data publication is supported by the Natural Environment Research Council award number NE/X019063/1 as part of the Hydro-JULES programme delivering National Capability.

## References

Bell, V., Rameshwaran, P., Davies, H., Baron, H., Keller, V., & Hannaford, J. (2023). Hydrological modelling and artificial influences: performance assessment & future scenarios: CS-NOW-D2 Future water availability for water intensive energy infrastructure. Climate Services for a Net Zero Resilient World (CS-N0W) report, UKCEH, https://assets.publishing.service.gov.uk/media/672359d201ee25b06c4f9987/future-water-resources-hydrological-modelling.pdf, Accessed on 08/12/2025.

Bell, V. A., Kay, A. L., Davies, H. N., & Jones, R. G. (2016). An assessment of the possible impacts of climate change on snow and peak river flows across Britain [journal article]. Climatic Change, 136(3), 539-553. https://doi.org/10.1007/s10584-016-1637-x

Bell, V. A., Kay, A. L., Jones, R. G., Moore, R. J., & Reynard, N. S. (2009). Use of soil data in a grid-based hydrological model to estimate spatial variation in changing flood risk across the UK. Journal of Hydrology, 377(3), 335-350. https://doi.org/10.1016/j.jhydrol.2009.08.031

Bell, V. A., Rudd, A. C., Kay, A. L., & Davies, H. N. (2018). Grid-to-Grid model estimates of daily mean river flow for gauged catchments in Great Britain (1891 to 2015): observed driving data [MaRIUS-G2G-Oudin-daily] (NERC Environmental Information Data Centre. https://doi.org/10.5285/0ceb4f85-0bbf-49f0-ab70-cfc137ab7d4d

Davies, H. N., Rameshwaran, P., Bell, V. A., & Dadson, S. (2024). Spatially consistent physical characteristics of UK rivers: 1-km data. Geoscience Data Journal, 11(3), 284-291. https://doi.org/10.1002/gdj3.209

Dixon, H., Hannaford, J., & Fry, M. J. (2013). The effective management of national hydrometric data: experiences from the United Kingdom. Hydrological Sciences Journal, 58(7), 1383-1399. https://doi.org/10.1080/02626667.2013.787486

EA. (2020). Environment Agency Scheme of Abstraction Charges 2020/21. Environment Agency, https://www.gov.uk/government/publications/scheme-of-abstraction-charges-2020-to-2021, Accessed on 01/11/2024.

Formetta, G., Prosdocimi, I., Stewart, E., & Bell, V. (2018). Estimating the index flood with continuous hydrological models: an application in Great Britain. Hydrology Research, 49(1), 123-133. https://doi.org/10.2166/nh.2017.251

Hollis, D., McCarthy, M., Kendon, M., Legg, T., & Simpson, I. (2019). HadUK-Grid—A new UK dataset of gridded climate observations. Geoscience Data Journal, 6(2), 151-159. https://doi.org/10.1002/gdj3.78

Hough, M. N., & Jones, R. J. A. (1997). The United Kingdom Meteorological Office rainfall and evaporation calculation system: MORECS version 2.0-an overview. Hydrol. Earth Syst. Sci., 1(2), 227-239. https://doi.org/10.5194/hess-1-227-1997

Kay, A. L. (2021). Simulation of river flow in Britain under climate change: Baseline performance and future seasonal changes. Hydrological Processes, 35(4). https://doi.org/10.1002/hyp.14137

Keller, V. D. J., Tanguy, M., Prosdocimi, I., Terry, J. A., Hitt, O., Cole, S. J., Fry, M., Morris, D. G., & Dixon, H. (2015). CEH-GEAR: 1 km resolution daily and monthly areal rainfall estimates for the UK for hydrological and other applications. Earth Syst. Sci. Data, 7(1), 143-155. https://doi.org/10.5194/essd-7-143-2015

Rameshwaran, P., Bell, V. A., Brown, M. J., Davies, H. N., Kay, A. L., Rudd, A. C., & Sefton, C. (2022). Use of Abstraction and Discharge Data to Improve the Performance of a National-Scale Hydrological Model. Water Resources Research, 58(1), e2021WR029787. https://doi.org/10.1029/2021WR029787

Rameshwaran, P., Bell, V. A., Davies, H. N., Baron, H., Keller, V., & Hannaford, J. (2025). DATA DOCUMENTATION - CS-NOW: Gridded future projections of natural and artificially influenced river flows (1980 to 2080). Zenodo, https://doi.org/10.5281/zenodo.10958783.

Rameshwaran, P., Bell, V. A., Davies, H. N., Baron, H., Keller, V., Hannaford, J., & Rhodes-Smith, M. (2025). CS-NOW: Gridded future projections of natural and artificially influenced river flows (1980 to 2080). ERC EDS Centre for Environmental Data Analysis, https://dx.doi.org/10.5285/cf66055440344d7eb9b6f834e81736c6.

Rameshwaran, P., Bell, V. A., Davies, H. N., Sadler, P., Beverton, A., & Thornton, R. (2025). DATA DOCUMENTATION - Gridded actual groundwater, surface water and tidal water abstraction, discharge and Hands-off Flow datasets for England (1999 to 2014). Zenodo, https://doi.org/10.5281/zenodo.13746897.

Rameshwaran, P., Bell, V. A., Davies, H. N., Sadler, P., Beverton, A., Thornton, R., & Rhodes-Smith, M. (2025). Gridded actual groundwater, surface water and tidal water abstraction, discharge and Hands-off Flow datasets for England (1999 to 2014). NERC EDS Centre for Environmental Data Analysis, https://dx.doi.org/10.5285/18886f95ba84447f997efac96df456ad.

Rudd, A. C., Bell, V. A., & Kay, A. L. (2017). National-scale analysis of simulated hydrological droughts (1891–2015). Journal of Hydrology, 550, 368-385. https://doi.org/10.1016/j.jhydrol.2017.05.018

Tanguy, M., Dixon, H., Prosdocimi, I., Morris, D. G., & Keller, V. D. J. (2016). Gridded estimates of daily and monthly areal rainfall for the United Kingdom (1890-2015) [CEH-GEAR] (NERC Environmental Information Data Centre. https://doi.org/10.5285/33604ea0-c238-4488-813d-0ad9ab7c51ca

## Appendix A: Table 1. 626 Gauging stations.

| Site | Gauging station | Catchment area (km2) |
| --- | --- | --- |
| 22001 | Coquet at Morwick | 570 |
| 22006 | Blyth at Hartford Bridge | 269 |
| 22007 | Wansbeck at Mitford | 287 |
| 22009 | Coquet at Rothbury | 346 |
| 23001 | Tyne at Bywell | 2176 |
| 23002 | Derwent at Eddys Bridge | 118 |
| 23003 | North Tyne at Reaverhill | 1008 |
| 23004 | South Tyne at Haydon Bridge | 751 |
| 23006 | South Tyne at Featherstone | 322 |
| 23007 | Derwent at Rowlands Gill | 242 |
| 23008 | Rede at Rede Bridge | 344 |
| 23009 | South Tyne at Alston | 119 |
| 23011 | Kielder Burn at Kielder | 59 |
| 23016 | Ouse Burn at Crag Hall | 55 |
| 23017 | Team at Team Valley | 62 |
| 23022 | North Tyne at Uglydub | 242 |
| 24001 | Wear at Sunderland Bridge | 658 |
| 24002 | Gaunless at Bishop Auckland | 93 |
| 24003 | Wear at Stanhope | 172 |
| 24004 | Bedburn Beck at Bedburn | 75 |
| 24005 | Browney at Burnhall | 179 |
| 24008 | Wear at Witton Park | 455 |
| 24009 | Wear at Chester le Street | 1008 |
| 25001 | Tees at Darlington Broken Scar | 818 |
| 25004 | Skerne at Darlington South Park | 250 |
| 25005 | Leven at Leven Bridge | 196 |
| 25006 | Greta at Rutherford Bridge | 86 |
| 25008 | Tees at Barnard Castle | 509 |
| 25009 | Tees at Low Moor | 1264 |
| 25018 | Tees at Middleton in Teesdale | 242 |
| 25020 | Skerne at Preston le Skerne | 147 |
| 25021 | Skerne at Bradbury | 70 |
| 25023 | Tees at Cow Green Reservoir | 58 |
| 25029 | Leven at Foxton Bridge | 184 |
| 26002 | Hull at Hempholme Lock | 378 |
| 26005 | Gypsey Race at Boynton | 240 |
| 26012 | Foulness at Holme House Farm | 70 |
| 26013 | Driffield Trout Stream at Driffield | 53 |
| 27001 | Nidd at Hunsingore Weir | 484 |
| 27002 | Wharfe at Wetherby Flint Mill | 759 |
| 27003 | Aire at Beal | 1932 |
| 27005 | Nidd at Gouthwaite Reservoir | 114 |
| 27006 | Don at Sheffield Hadfields | 373 |
| 27007 | Ure at Westwick | 915 |
| 27009 | Ouse at Skelton | 3315 |
| 27021 | Don at Doncaster | 1256 |
| 27022 | Don at Rotherham Weir | 826 |
| 27023 | Dearne at Barnsley Weir | 119 |
| 27025 | Rother at Woodhouse Mill | 352 |
| 27026 | Rother at Whittington | 165 |
| 27028 | Aire at Armley | 692 |
| 27029 | Calder at Elland | 342 |
| 27030 | Dearne at Adwick | 311 |
| 27031 | Colne at Colne Bridge | 245 |
| 27034 | Ure at Kilgram Bridge | 510 |
| 27035 | Aire at Kildwick Bridge | 282 |
| 27040 | Doe Lea at Staveley | 68 |
| 27041 | Derwent at Buttercrambe | 1586 |
| 27042 | Dove at Kirkby Mills | 59 |
| 27043 | Wharfe at Addingham | 427 |
| 27048 | Derwent at West Ayton | 127 |
| 27049 | Rye at Ness | 239 |
| 27052 | Whitting at Sheepbridge | 50 |
| 27053 | Nidd at Birstwith | 218 |
| 27055 | Rye at Broadway Foot | 132 |
| 27056 | Pickering Beck at Ings Bridge | 69 |
| 27057 | Seven at Normanby | 122 |
| 27059 | Laver at Ripon Laver Weir | 88 |
| 27060 | Kyle at Newton On Ouse | 168 |
| 27061 | Colne at Longroyd Bridge | 72 |
| 27062 | Nidd at Skip Bridge | 516 |
| 27064 | Went at Walden Stubbs | 84 |
| 27065 | Holme at Huddersfield Queens Mill | 97 |
| 27069 | Wiske at Kirby Wiske | 216 |
| 27071 | Swale at Crakehill | 1363 |
| 27072 | Worth at Keighley | 72 |
| 27075 | Bedale Beck at Leeming | 160 |
| 27076 | Bielby Beck at Thornton Lock | 103 |
| 27077 | Bradford Beck at Shipley | 58 |
| 27079 | Calder at Methley | 930 |
| 27080 | Aire at Oulton Lemonroyd | 865 |
| 27083 | Foss at Huntington | 118 |
| 27085 | Cod Beck at Dalton Bridge | 209 |
| 27086 | Skell at Ripon Alma Weir | 120 |
| 27087 | Derwent at Low Marishes | 458 |
| 27088 | Calder at Mytholmroyd | 142 |
| 27089 | Wharfe at Tadcaster | 818 |
| 27090 | Swale at Catterick Bridge | 499 |
| 27091 | Crimple at Blackstones | 77 |
| 27092 | Esk at Briggswath | 325 |
| 27096 | Wharfe at Netherside Hall | 215 |
| 28001 | Derwent at Yorkshire Bridge | 126 |
| 28002 | Blithe at Hamstall Ridware | 163 |
| 28003 | Tame at Water Orton | 408 |
| 28007 | Trent at Shardlow | 4400 |
| 28008 | Dove at Rocester Weir | 399 |
| 28009 | Trent at Colwick | 7486 |
| 28011 | Derwent at Matlock Bath | 690 |
| 28012 | Trent at Yoxall | 1229 |
| 28014 | Sow at Milford | 591 |
| 28015 | Idle at Mattersey | 529 |
| 28018 | Dove at Marston on Dove | 883 |
| 28019 | Trent at Drakelow Park | 3072 |
| 28022 | Trent at North Muskham | 8231 |
| 28023 | Wye at Ashford | 154 |
| 28024 | Wreake at Syston Mill | 414 |
| 28026 | Anker at Polesworth | 368 |
| 28027 | Erewash at Sandiacre | 182 |
| 28031 | Manifold at Ilam | 149 |
| 28032 | Meden at Church Warsop | 63 |
| 28035 | Leen at Triumph Road Nottingham | 111 |
| 28036 | Poulter at Twyford Bridge | 128 |
| 28039 | Rea at Calthorpe Park | 74 |
| 28040 | Trent at Stoke-On-Trent | 53 |
| 28043 | Derwent at Chatsworth | 335 |
| 28046 | Dove at Izaak Walton | 83 |
| 28047 | Oldcotes Dyke at Blyth | 85 |
| 28048 | Amber at Wingfield Park | 139 |
| 28049 | Ryton at Worksop | 77 |
| 28050 | Torne at Auckley | 136 |
| 28052 | Sow at Great Bridgeford | 163 |
| 28053 | Penk at Penkridge | 272 |
| 28055 | Ecclesbourne at Duffield | 50 |
| 28056 | Rothley Brook at Rothley | 94 |
| 28060 | Dover Beck at Lowdham | 69 |
| 28061 | Churnet at Basford Bridge | 139 |
| 28066 | Cole at Coleshill | 130 |
| 28067 | Derwent at Church Wilne | 1178 |
| 28068 | Lathkill at Pickering Wood | 90 |
| 28072 | Greet at Southwell | 46 |
| 28074 | Soar at Kegworth | 1292 |
| 28079 | Meece Brook at Shallowford | 86 |
| 28080 | Tame at Lea Marston Lakes | 799 |
| 28082 | Soar at Littlethorpe | 184 |
| 28083 | Trent at Darlaston | 195 |
| 28085 | Derwent at St Mary's Bridge | 1054 |
| 28086 | Sence at South Wigston | 113 |
| 28091 | Ryton at Blyth | 231 |
| 28093 | Soar at Pillings Lock | 1108 |
| 28095 | Tame at Hopwas Bridge | 1422 |
| 28112 | Churnet at Quixhill | 213 |
| 28116 | Maun at Whitewater Bridge | 157 |
| 28117 | Derwent at Whatstandwell | 755 |
| 28118 | Meden at Perlethorpe | 97 |
| 29001 | Waithe Beck at Brigsley | 108 |
| 29002 | Great Eau at Claythorpe Mill | 77 |
| 29003 | Lud at Louth | 55 |
| 29005 | Rase at Bishopbridge | 67 |
| 30001 | Witham at Claypole Mill | 298 |
| 30002 | Barlings Eau at Langworth Bridge | 210 |
| 30003 | Bain at Fulsby Lock | 197 |
| 30004 | Lymn at Partney Mill | 62 |
| 30005 | Witham at Saltersford Total | 126 |
| 30011 | Bain at Goulceby Bridge | 63 |
| 30033 | Brant at Brant Broughton | 66 |
| 31001 | Eye Brook at Eye Brook Reservoir | 60 |
| 31004 | Welland at Tallington Total | 717 |
| 31006 | Gwash at Belmesthorpe | 150 |
| 31007 | Welland at Barrowden | 412 |
| 31008 | East Glen at Manthorpe | 136 |
| 31009 | West Glen at Shillingthorpe | 173 |
| 31010 | Chater at Fosters Bridge | 69 |
| 31013 | East Glen at Irnham | 72 |
| 31021 | Welland at Ashley | 251 |
| 31028 | Gwash at Church Bridge | 77 |
| 32003 | Harpers Brook at Old Mill Bridge | 74 |
| 32004 | Ise Brook at Harrowden | 194 |
| 32006 | Nene/Kislingbury at Upton Total | 223 |
| 32007 | Nene/Brampton at St Andrews Total | 233 |
| 32008 | Nene/Kislingbury at Dodford | 107 |
| 32010 | Nene at Wansford | 1530 |
| 32012 | Wootton Brook at Lady Bridge | 53 |
| 32019 | Slade Brook at Kettering | 58 |
| 32031 | Wootton Brook at Wootton Park | 74 |
| 33002 | Bedford Ouse at Bedford | 1460 |
| 33006 | Wissey at Northwold Total | 275 |
| 33007 | Nar at Marham | 153 |
| 33011 | Little Ouse at County Bridge Euston | 129 |
| 33012 | Kym at Meagre Farm | 138 |
| 33013 | Sapiston at Rectory Bridge | 206 |
| 33014 | Lark at Temple | 272 |
| 33015 | Ouzel at Willen | 277 |
| 33018 | Tove at Cappenham Bridge | 138 |
| 33019 | Thet at Melford Bridge | 316 |
| 33020 | Alconbury Brook at Brampton | 202 |
| 33021 | Rhee at Burnt Mill | 303 |
| 33022 | Ivel at Blunham | 541 |
| 33024 | Cam at Dernford | 198 |
| 33026 | Bedford Ouse at Offord | 2570 |
| 33027 | Rhee at Wimpole | 119 |
| 33028 | Flit at Shefford | 120 |
| 33029 | Stringside at Whitebridge | 99 |
| 33031 | Broughton Brook at Broughton | 67 |
| 33032 | Heacham at Heacham | 59 |
| 33033 | Hiz at Arlesey | 108 |
| 33034 | Little Ouse at Abbey Heath | 689 |
| 33035 | Ely Ouse at Denver Complex | 3430 |
| 33037 | Bedford Ouse at Newport Pagnell Total | 800 |
| 33039 | Bedford Ouse at Roxton | 1660 |
| 33044 | Thet at Bridgham | 278 |
| 33046 | Thet at Redbridge | 145 |
| 33050 | Snail at Fordham | 61 |
| 33051 | Cam at Chesterford | 141 |
| 33053 | Granta at Stapleford | 114 |
| 33055 | Granta at Babraham | 99 |
| 33056 | Quy Water at Lode | 76 |
| 33057 | Ouzel at Leighton Buzzard | 119 |
| 33058 | Ouzel at Bletchley | 215 |
| 33063 | Little Ouse at Knettishall | 101 |
| 33066 | Granta at Linton | 60 |
| 33070 | Lark at Fornham St Martin | 110 |
| 34001 | Yare at Colney | 232 |
| 34002 | Tas at Shotesham | 147 |
| 34003 | Bure at Ingworth | 165 |
| 34004 | Wensum at Costessey Mill | 571 |
| 34005 | Tud at Costessey Park | 73 |
| 34006 | Waveney at Needham Mill | 370 |
| 34007 | Dove at Oakley Park | 134 |
| 34010 | Waveney at Billingford Bridge | 149 |
| 34011 | Wensum at Fakenham | 162 |
| 34012 | Burn at Burnham Overy | 80 |
| 34014 | Wensum at Swanton Morley Total | 398 |
| 34018 | Stiffkey at Warham | 88 |
| 34019 | Bure at Horstead Mill | 313 |
| 35002 | Deben at Naunton Hall | 163 |
| 35003 | Alde at Farnham | 64 |
| 35004 | Ore at Beversham | 55 |
| 35010 | Gipping at Bramford | 298 |
| 35013 | Blyth at Holton | 93 |
| 36002 | Glem at Glemsford | 87 |
| 36003 | Box at Polstead | 54 |
| 36005 | Brett at Hadleigh | 156 |
| 36006 | Stour at Langham | 578 |
| 36007 | Belchamp Brook at Bardfield Bridge | 59 |
| 36008 | Stour at Westmill | 225 |
| 36010 | Bumpstead Brook at Broad Green | 28 |
| 36012 | Stour at Kedington | 76 |
| 36013 | Brett at Higham | 195 |
| 36015 | Stour at Lamarsh | 481 |
| 37001 | Roding at Redbridge | 303 |
| 37002 | Chelmer at Rushes Lock | 534 |
| 37003 | Ter at Crabbs Bridge | 78 |
| 37005 | Colne at Lexden | 238 |
| 37006 | Can at Beach's Mill | 228 |
| 37007 | Wid at Writtle | 136 |
| 37008 | Chelmer at Springfield | 190 |
| 37009 | Brain at Guithavon Valley | 61 |
| 37010 | Blackwater at Appleford Bridge | 247 |
| 37011 | Chelmer at Churchend | 73 |
| 37012 | Colne at Poolstreet | 65 |
| 37013 | Sandon Brook at Sandon Bridge | 75 |
| 37014 | Roding at High Ongar | 95 |
| 37015 | Cripsey Brook at Chipping Ongar | 62 |
| 37016 | Pant at Copford Hall | 63 |
| 37017 | Blackwater at Stisted | 139 |
| 37019 | Beam at Bretons Farm | 50 |
| 37020 | Chelmer at Felsted | 132 |
| 37022 | Holland Brook at Thorpe le Soken | 55 |
| 37023 | Roding at Loughton | 269 |
| 37024 | Colne at Earls Colne | 154 |
| 37031 | Crouch at Wickford | 72 |
| 37034 | Mar Dyke at Stifford | 91 |
| 38001 | Lee at Feildes Weir | 1036 |
| 38002 | Ash at Mardock | 79 |
| 38003 | Mimram at Panshanger Park | 134 |
| 38004 | Rib at Wadesmill | 137 |
| 38011 | Mimram at Fulling Mill | 99 |
| 38013 | Upper Lee at Luton Hoo | 71 |
| 38014 | Salmon Brook at Edmonton | 21 |
| 38017 | Mimram at Whitwell | 39 |
| 38018 | Upper Lee at Water Hall | 150 |
| 38023 | Lee flood relief at Low Hall | 1243 |
| 38026 | Pincey Brook at Sheering Hall | 55 |
| 38029 | Quin at Griggs Bridge | 50 |
| 38030 | Beane at Hartham | 175 |
| 38031 | Lee at Rye Bridge | 758 |
| 38032 | Lee at Lea Bridge | 1364 |
| 38033 | Upper Lee at Luton East Hyde | 71 |
| 39001 | Thames at Kingston | 9948 |
| 39002 | Thames at Days Weir | 3445 |
| 39003 | Wandle at South Wimbledon | 176 |
| 39004 | Wandle at Beddington Park | 122 |
| 39006 | Windrush at Newbridge | 363 |
| 39007 | Blackwater at Swallowfield | 355 |
| 39008 | Thames at Eynsham | 1616 |
| 39010 | Colne at Denham | 743 |
| 39011 | Wey at Tilford | 396 |
| 39012 | Hogsmill at Kingston upon Thames | 69 |
| 39013 | Colne at Berrygrove | 352 |
| 39014 | Ver at Hansteads | 132 |
| 39016 | Kennet at Theale | 1033 |
| 39019 | Lambourn at Shaw | 234 |
| 39020 | Coln at Bibury | 107 |
| 39021 | Cherwell at Enslow Mill | 552 |
| 39022 | Loddon at Sheepbridge | 165 |
| 39023 | Wye at Bourne End Hedsor | 137 |
| 39025 | Enborne at Brimpton | 148 |
| 39026 | Cherwell at Banbury | 199 |
| 39027 | Pang at Pangbourne | 171 |
| 39028 | Dun at Hungerford | 101 |
| 39029 | Tilling Bourne at Shalford | 59 |
| 39030 | Gade at Croxley Green | 184 |
| 39031 | Lambourn at Welford | 176 |
| 39032 | Lambourn at East Shefford | 154 |
| 39034 | Evenlode at Cassington Mill | 430 |
| 39035 | Churn at Cerney Wick | 124 |
| 39037 | Kennet at Marlborough | 142 |
| 39039 | Wye at High Wycombe | 68 |
| 39040 | Thames at West Mill Cricklade | 185 |
| 39042 | Leach at Priory Mill Lechlade | 77 |
| 39043 | Kennet at Knighton | 295 |
| 39044 | Hart at Bramshill House | 84 |
| 39046 | Thames at Sutton Courtenay | 3414 |
| 39049 | Silk Stream at Colindeep Lane | 29 |
| 39053 | Mole at Horley | 90 |
| 39056 | Ravensbourne at Catford Hill | 120 |
| 39057 | Crane at Cranford Park | 62 |
| 39068 | Mole at Castle Mill | 316 |
| 39069 | Mole at Kinnersley Manor | 142 |
| 39071 | Thames at Ewen | 64 |
| 39072 | Thames at Royal Windsor Park | 7046 |
| 39073 | Churn at Cirencester | 84 |
| 39074 | Ampney Brook at Sheepen Bridge | 74 |
| 39076 | Windrush at Worsham | 296 |
| 39077 | Og at Marlborough Poulton Fm | 59 |
| 39078 | Wey (North) at Farnham | 191 |
| 39079 | Wey at Weybridge | 1008 |
| 39081 | Ock at Abingdon | 234 |
| 39087 | Ray at Water Eaton | 84 |
| 39088 | Chess at Rickmansworth | 105 |
| 39089 | Gade at Bury Mill | 48 |
| 39090 | Cole at Inglesham | 140 |
| 39093 | Brent at Monks Park | 118 |
| 39094 | Crane at Marsh Farm | 81 |
| 39101 | Aldbourne at Ramsbury | 53 |
| 39102 | Misbourne at Denham Lodge | 95 |
| 39103 | Kennet at Newbury | 548 |
| 39104 | Mole at Esher | 470 |
| 39105 | Thame at Wheatley | 534 |
| 39106 | Mole at Leatherhead | 371 |
| 39109 | Coln at Fossebridge | 82 |
| 39110 | Coln at Fairford | 130 |
| 39111 | Thames at Staines | 8120 |
| 39114 | Pang at Frilsham | 90 |
| 39115 | Pang at Bucklebury | 109 |
| 39117 | Colne Brook at Hythe End | 930 |
| 39120 | Caker Stream at Alton | 88 |
| 39121 | Thames at Walton | 9292 |
| 39122 | Cranleigh Waters at Bramley | 110 |
| 39125 | Ver at Redbourn | 63 |
| 39127 | Misbourne at Little Missenden | 47 |
| 39128 | Bourne (South) at Addlestone | 90 |
| 39129 | Thames at Farmoor | 1609 |
| 39130 | Thames at Reading | 4634 |
| 39131 | Brent at Costons Lane Greenford | 146 |
| 39138 | Loddon at Twyford | 690 |
| 39140 | Ray at Islip | 290 |
| 39141 | Wey at Guildford | 690 |
| 39142 | Windrush at Bourton on the Water | 66 |
| 39143 | Dikler at Bourton on the Water | 91 |
| 39144 | Sor at Bodicote | 88 |
| 39148 | Thames at Maidenhead | 6910 |
| 40003 | Medway at Teston / East Farleigh | 1256 |
| 40004 | Rother at Udiam | 206 |
| 40005 | Beult at Stilebridge | 277 |
| 40007 | Medway at Chafford / Colliers Land Bridge | 255 |
| 40008 | Great Stour at Wye | 230 |
| 40009 | Teise at Stonebridge | 136 |
| 40010 | Eden at Penshurst / Vexour Bridge | 224 |
| 40011 | Great Stour at Horton | 345 |
| 40012 | Darent at Hawley | 191 |
| 40013 | Darent at Otford | 101 |
| 40016 | Cray at Crayford | 120 |
| 40018 | Darent at Lullingstone | 118 |
| 40020 | Eridge Stream at Hendal Bridge | 54 |
| 40022 | Great Stour at Chart Leacon | 73 |
| 40023 | East Stour at South Willesborough | 59 |
| 40029 | Len at Lenside | 70 |
| 41003 | Cuckmere at Sherman Bridge | 135 |
| 41004 | Ouse at Barcombe Mills | 396 |
| 41005 | Ouse at Gold Bridge | 181 |
| 41006 | Uck at Isfield | 88 |
| 41009 | Rother at Hardham | 346 |
| 41010 | Adur W Branch at Hatterell Bridge | 109 |
| 41011 | Rother at Iping Mill | 154 |
| 41012 | Adur E Branch at Sakeham | 93 |
| 41014 | Arun at Pallingham | 379 |
| 41015 | Ems at Westbourne | 58 |
| 41018 | Kird at Tanyards | 67 |
| 41019 | Arun at Alfoldean | 139 |
| 41022 | Lod at Halfway Bridge | 52 |
| 41023 | Lavant at Graylingwell | 87 |
| 41025 | Loxwood Stream at Drungewick | 92 |
| 41029 | Bull at Lealands | 41 |
| 41035 | North at Brookhurst | 54 |
| 42001 | Wallington at North Fareham | 111 |
| 42003 | Lymington at Brockenhurst | 99 |
| 42004 | Test at Broadlands | 1040 |
| 42005 | Wallop Brook at Broughton | 54 |
| 42006 | Meon at Mislingford | 73 |
| 42007 | Alre at Drove Lane Alresford | 57 |
| 42008 | Cheriton Stream at Sewards Bridge | 75 |
| 42009 | Candover Stream at Borough Bridge | 71 |
| 42011 | Hamble at Frogmill | 57 |
| 42012 | Anton at Fullerton | 185 |
| 42014 | Blackwater at Ower | 105 |
| 42016 | Itchen at Easton | 237 |
| 42023 | Itchen at Riverside Park | 415 |
| 42024 | Test at Chilbolton Total | 453 |
| 42025 | Lavant Stream at Leigh Park | 55 |
| 42026 | Wallop Brook at Bossington | 61 |
| 42027 | Dever at Bransbury | 122 |
| 43003 | Avon at East Mills Total | 1478 |
| 43004 | Bourne at Laverstock | 164 |
| 43005 | Avon at Amesbury | 324 |
| 43006 | Nadder at Wilton | 221 |
| 43007 | Stour at Throop | 1073 |
| 43008 | Wylye at South Newton | 445 |
| 43009 | Stour at Hammoon | 523 |
| 43010 | Allen at Loverley Farm | 94 |
| 43012 | Wylye at Norton Bavant | 112 |
| 43014 | East Avon at Upavon East | 86 |
| 43018 | Allen at Walford Mill | 177 |
| 43021 | Avon at Knapp Mill | 1706 |
| 43022 | Moors River at Hurn Court | 143 |
| 43023 | Ebble at Nunton Bridge | 107 |
| 43024 | Wylye at Stockton Park | 255 |
| 44001 | Frome at East Stoke Total | 414 |
| 44002 | Piddle at Baggs Mill | 183 |
| 44004 | Frome at Dorchester Total | 206 |
| 44014 | Piddle at Briantspuddle | 112 |
| 45001 | Exe at Thorverton | 601 |
| 45002 | Exe at Stoodleigh | 422 |
| 45003 | Culm at Woodmill | 226 |
| 45004 | Axe at Whitford | 289 |
| 45005 | Otter at Dotton | 203 |
| 45007 | Exe at Trews Weir | 1191 |
| 45008 | Otter at Fenny Bridges | 104 |
| 45009 | Exe at Pixton | 160 |
| 45011 | Barle at Brushford | 128 |
| 45012 | Creedy at Cowley | 262 |
| 46002 | Teign at Preston | 381 |
| 46003 | Dart at Austins Bridge | 248 |
| 46005 | East Dart at Bellever | 22 |
| 46008 | Avon at Loddiswell | 102 |
| 46013 | Bovey at Bovey Parke | 87 |
| 46014 | Teign at Chudleigh | 232 |
| 47001 | Tamar at Gunnislake | 917 |
| 47004 | Lynher at Pillaton Mill | 136 |
| 47005 | Ottery at Werrington Park | 121 |
| 47006 | Lyd at Lifton Park | 223 |
| 47007 | Yealm at Puslinch | 55 |
| 47008 | Thrushel at Tinhay | 113 |
| 47009 | Tiddy at Tideford | 37 |
| 47010 | Tamar at Crowford Bridge | 77 |
| 47011 | Plym at Carn Wood | 79 |
| 47015 | Tavy at Ludbrook | 197 |
| 47018 | Thrushel at Hayne Bridge | 58 |
| 47019 | Tamar at Polson Bridge | 470 |
| 47020 | Inny at Bealsmill | 105 |
| 47024 | Tavy at Tavistock Abbey Bridge | 96 |
| 48003 | Fal at Tregony | 87 |
| 48004 | Warleggan at Trengoffe | 25 |
| 48005 | Kenwyn at Truro | 19 |
| 48011 | Fowey at Restormel | 169 |
| 49001 | Camel at Denby | 209 |
| 50001 | Taw at Umberleigh | 826 |
| 50002 | Torridge at Torrington | 663 |
| 50006 | Mole at Woodleigh | 328 |
| 50007 | Taw at Taw Bridge | 71 |
| 50008 | Lew at Gribbleford Bridge | 71 |
| 50010 | Torridge at Rockhay Bridge | 258 |
| 50011 | Okement at Jacobstowe | 82 |
| 50012 | Yeo at Veraby | 54 |
| 50014 | Yeo at Collard Bridge | 80 |
| 51001 | Doniford Stream at Swill Bridge | 76 |
| 52003 | Halsewater at Halsewater | 88 |
| 52004 | Isle at Ashford Mill | 90 |
| 52005 | Tone at Bishops Hull | 202 |
| 52006 | Yeo at Pen Mill | 213 |
| 52007 | Parrett at Chiselborough | 75 |
| 52009 | Sheppey at Fenny Castle | 60 |
| 52010 | Brue at Lovington | 135 |
| 52011 | Cary at Somerton | 82 |
| 52014 | Tone at Greenham | 57 |
| 52017 | Congresbury Yeo at Iwood | 67 |
| 53002 | Semington Brook at Semington | 158 |
| 53004 | Chew at Compton Dando | 130 |
| 53005 | Midford Brook at Midford | 147 |
| 53006 | Frome (Bristol) at Frenchay | 149 |
| 53007 | Frome (Somerset) at Tellisford | 262 |
| 53008 | Avon at Great Somerford | 303 |
| 53009 | Wellow Brook at Wellow | 73 |
| 53013 | Marden at Stanley | 99 |
| 53017 | Boyd at Bitton | 48 |
| 53018 | Avon at Bathford | 1552 |
| 53022 | Avon at Bath ultrasonic | 1605 |
| 53023 | Sherston Avon at Fosseway | 90 |
| 53024 | Tetbury Avon at Brokenborough | 74 |
| 53025 | Mells at Vallis | 119 |
| 53026 | Frome (Bristol) at Frampton Cotterell | 79 |
| 53028 | By Brook at Middlehill | 102 |
| 53029 | Biss at Trowbridge | 78 |
| 54001 | Severn at Bewdley | 4325 |
| 54002 | Avon at Evesham | 2210 |
| 54004 | Sowe at Stoneleigh | 262 |
| 54006 | Stour (Worcs) at Kidderminster Callows Lane | 324 |
| 54007 | Arrow at Broom | 319 |
| 54008 | Teme at Tenbury | 1134 |
| 54010 | Stour (Warks) at Alscot Park | 319 |
| 54011 | Salwarpe at Harford Hill | 184 |
| 54015 | Bow Brook at Besford Bridge | 156 |
| 54017 | Leadon at Wedderburn Bridge | 293 |
| 54018 | Rea Brook at Hookagate | 178 |
| 54019 | Avon at Stareton | 347 |
| 54020 | Perry at Yeaton | 181 |
| 54023 | Badsey Brook at Offenham | 96 |
| 54024 | Worfe at Burcote | 258 |
| 54027 | Frome at Ebley Mill | 198 |
| 54029 | Teme at Knightsford Bridge | 1480 |
| 54032 | Severn at Saxons Lode | 6850 |
| 54036 | Isbourne at Hinton on the Green | 91 |
| 54040 | Meese at Tibberton | 168 |
| 54041 | Tern at Eaton upon Tern | 192 |
| 54044 | Tern at Ternhill | 93 |
| 54046 | Worfe at Cosford | 55 |
| 54048 | Dene at Wellesbourne | 102 |
| 54049 | Leam at Princes Drive Weir | 362 |
| 54050 | Leam at Eathorpe | 300 |
| 54057 | Severn at Haw Bridge | 9895 |
| 54063 | Stour (Worcs) at Prestwood Hospital | 90 |
| 54088 | Little Avon at Berkeley Kennels | 134 |
| 54089 | Avon at Bredon | 2674 |
| 54094 | Strine at Crudgington | 96 |
| 54096 | Hadley Brook at Wards Bridge | 53 |
| 54102 | Avon at Lilbourne | 109 |
| 54106 | Stour (Warks) at Shipston | 185 |
| 54107 | Arrow at Studley | 93 |
| 54112 | Leam at Kites Hardwick | 100 |
| 54113 | Itchen at Southam | 106 |
| 54114 | Avon at Warwick | 1012 |
| 54115 | Piddle Brook at Wyre Piddle | 105 |
| 68001 | Weaver at Ashbrook | 622 |
| 68003 | Dane at Rudheath | 407 |
| 68004 | Wistaston Brook at Marshfield Bridge | 93 |
| 68005 | Weaver at Audlem | 207 |
| 68007 | Wincham Brook at Lostock Gralam | 148 |
| 68018 | Dane at Congleton Park | 145 |
| 68020 | Gowy at Bridge Trafford | 156 |
| 68044 | Dane at Hug Bridge | 73 |
| 69002 | Irwell at Adelphi Weir | 559 |
| 69003 | Irk at Scotland Weir | 73 |
| 69005 | Glaze Brook at Little Woolden Hall | 152 |
| 69006 | Bollin at Dunham Massey | 258 |
| 69007 | Mersey at Ashton Weir | 660 |
| 69008 | Dean at Stanneylands | 59 |
| 69012 | Bollin at Wilmslow | 73 |
| 69015 | Etherow at Compstall | 156 |
| 69017 | Goyt at Marple Bridge | 183 |
| 69020 | Medlock at London Road | 58 |
| 69022 | Irwell at Irwell Vale | 101 |
| 69023 | Roch at Blackford Bridge | 186 |
| 69024 | Croal at Farnworth Weir | 145 |
| 69027 | Tame at Portwood | 150 |
| 69028 | Mersey at Brinksway | 517 |
| 69030 | Sankey Brook at Causey Bridges | 154 |
| 69032 | Alt at Kirkby | 90 |
| 69037 | Mersey at Westy | 2030 |
| 69041 | Tame at Broomstairs | 113 |
| 69043 | Irk at Collyhurst Weir | 72 |
| 69044 | Irwell at Bury Ground | 140 |
| 69045 | Bollin at Bollington Mill Total | 257 |
| 69803 | Roch at Rochdale | 111 |
| 70002 | Douglas at Wanes Blades Bridge | 198 |
| 70003 | Douglas at Wigan | 55 |
| 70004 | Yarrow at Croston Mill | 74 |
| 70005 | Lostock at Littlewood Bridge | 56 |
| 71001 | Ribble at Samlesbury | 1145 |
| 71004 | Calder at Whalley Weir | 316 |
| 71006 | Ribble at Henthorn | 456 |
| 71008 | Hodder at Hodder Place | 261 |
| 71009 | Ribble at New Jumbles Rock | 1053 |
| 71010 | Pendle Water at Barden Lane | 108 |
| 71011 | Ribble at Arnford | 204 |
| 71014 | Darwen at Blue Bridge | 128 |
| 72002 | Wyre at St Michaels | 275 |
| 72003 | Hindburn at Wray | 84 |
| 72004 | Lune at Caton | 983 |
| 72005 | Lune at Killington | 219 |
| 72008 | Wyre at Garstang | 114 |
| 72009 | Wenning at Wennington | 142 |
| 72011 | Rawthey at Brigflatts | 200 |
| 72014 | Conder at Galgate | 29 |
| 72015 | Lune at Lunes Bridge | 142 |
| 72016 | Wyre at Scorton Weir | 89 |
| 73002 | Crake at Low Nibthwaite | 73 |
| 73003 | Kent at Burneside | 74 |
| 73005 | Kent at Sedgwick | 209 |
| 73008 | Bela at Beetham | 131 |
| 73010 | Leven at Newby Bridge | 247 |
| 73011 | Mint at Mint Bridge | 66 |
| 73012 | Kent at Victoria Bridge | 183 |
| 73013 | Rothay at Miller Bridge House | 64 |
| 73014 | Brathay at Jeffy Knotts | 57 |
| 73017 | Kent at Bowston | 71 |
| 74001 | Duddon at Duddon Hall | 86 |
| 74005 | Ehen at Braystones | 126 |
| 74007 | Esk at Cropple How | 70 |
| 75002 | Derwent at Camerton | 663 |
| 75003 | Derwent at Ouse Bridge | 363 |
| 75004 | Cocker at Southwaite Bridge | 117 |
| 75005 | Derwent at Portinscale | 235 |
| 75007 | Glenderamackin at Threlkeld | 65 |
| 75016 | Cocker at Scalehill | 64 |
| 75017 | Ellen at Bullgill | 96 |
| 76003 | Eamont at Udford | 396 |
| 76004 | Lowther at Eamont Bridge | 159 |
| 76005 | Eden at Temple Sowerby | 616 |
| 76007 | Eden at Sheepmount | 2287 |
| 76008 | Irthing at Greenholme | 335 |
| 76009 | Caldew at Holm Hill | 147 |
| 76010 | Petteril at Harraby Green | 160 |
| 76014 | Eden at Kirkby Stephen | 69 |
| 76015 | Eamont at Pooley Bridge | 145 |
| 76017 | Eden at Great Corby | 1373 |
| 76019 | Roe Beck at Stockdalewath | 63 |