# Daily plot level (micro meteorological) data at Climoor field site in Clocaenog Forest, UK, 2025

The Climoor/Clocaenog field site is a climate change manipulation experiment initiated in 1998 that utilises automated roof technology to produce drought and warming experimental treatments that reflect climate change predictions for the next 20-30 years.

Author:	Sabine Reinsch

Version and version date:	Micromet – 07 Jan 2026

# Data access

The data can be accessed from the Environmental Information Data Centre under the Data collection: Daily plot level (micro meteorological) data at Climoor field site in Clocaenog Forest:

https://catalogue.ceh.ac.uk/documents/66aed381-44b9-4661-a6ca-fa196732f66b

# Data structure

The dataset comprises one spreadsheet (CLM_MICROMET_2025_summaryQA.csv) with 28 columns and 366 rows.

Data which were not recorded are marked with "NA", faulty data were replaced with "-9999".

Table 1.  Structure of data file CLM_MICROMET_2025_summaryQA.csv

| Header | Unit | Description |
| --- | --- | --- |
| Date | Year-Month-Day |  |
| TSoil5cm_Plot1_degC | oC | Soil temperature at 5cm depth for plot 1 – warming |
| TSoil5cm_Plot2_degC | oC | Soil temperature at 5cm depth for plot 2 – warming |
| TSoil5cm_Plot4_degC | oC | Soil temperature at 5cm depth for plot 3 – control |
| TSoil5cm_Plot4_degC | oC | Soil temperature at 5cm depth for plot 4 – drought |
| TSoil5cm_Plot5_degC | oC | Soil temperature at 5cm depth for plot 5 – drought |
| TSoil5cm_Plot6_degC | oC | Soil temperature at 5cm depth for plot 6 – control |
| TSoil5cm_Plot7_degC | oC | Soil temperature at 5cm depth for plot 7 – w arming |
| TSoil5cm_Plot8_degC | oC | Soil temperature at 5cm depth for plot 8 – drought |
| TSoil5cm_Plot9_degC | oC | Soil temperature at 5cm depth for plot 9 – control |
| TSoil20cm_Plot1_degC | oC | Soil temperature at 20cm depth for plot 1 – warming |
| TSoil20cm_Plot2_degC | oC | Soil temperature at 20cm depth for plot 2 – warming |
| TSoil20cm_Plot3_degC | oC | Soil temperature at 20cm depth for plot 3 – control |
| TSoil20cm_Plot4_degC | oC | Soil temperature at 20cm depth for plot 4 – drought |
| TSoil20cm_Plot5_degC | oC | Soil temperature at 20cm depth for plot 5 – drought |
| TSoil20cm_Plot6_degC | oC | Soil temperature at 20cm depth for plot 6 – control |
| TSoil20cm_Plot7_degC | oC | Soil temperature at 20cm depth for plot 7 – warming |
| TSoil20cm_Plot8_degC | oC | Soil temperature at 20cm depth for plot 8 – drought |
| TSoil20cm_Plot9_degC | oC | Soil temperature at 20cm depth for plot 9 – control |
| Soil_moisture_Plot1_m3_per_m-3 | m3m-3 | Soil volumetric water content for plot 1 – warming |
| Soil_moisture_Plot2_m3_per_m-3 | m3m-3 | Soil volumetric water content for plot 2 – warming |
| Soil_moisture_Plot3_m3_per_m-3 | m3m-3 | Soil volumetric water content for plot 3 – control |
| Soil_moisture_Plot4_m3_per_m-3 | m3m-3 | Soil volumetric water content for plot 4 – drought |
| Soil_moisture_Plot5_m3_per_m-3 | m3m-3 | Soil volumetric water content for plot 5 – drought |
| Soil_moisture_Plot6_m3_per_m-3 | m3m-3 | Soil volumetric water content for plot 6 – control |
| Soil_moisture_Plot7_m3_per_m-3 | m3m-3 | Soil volumetric water content for plot 7 – warming |
| Soil_moisture_Plot8_m3_per_m-3 | m3m-3 | Soil volumetric water content for plot 8 – drought |
| Soil_moisture_Plot9_m3_per_m-3 | m3m-3 | Soil volumetric water content for plot 9 – control |

# Site information

The Clocaenog field site is an automated manipulation experimental site that produces the potential change in climate for an upland moorland habitat over the next few decades. The experiment utilises automated roof technology, designed to work in the most sensitive and remote parts of the UK, designed to be powered by solar and wind power.  The site was established in 1998 and consists of replicated drought and warming treatments.

The site is located in Clocaenog Forest in North East Wales (53o 03' 19''N -03o 27' 55'' W).  It is a typical upland west-atlantic moorland, dominated by the evergreen shrub Calluna vulgaris (heather).  Heather makes up >60% of the plant biomass at the site, but Vaccinium myrtillus and Empetrum nigrum are also present.

A list of plant species present in the experiment plots includes,

Calluna vulgaris, Vaccinium myrtillus, Empetrum nigrum, Deschampsia flexuosa,*Pleurozium schreberii, *Hypnum jutlandicum, *Psuedosclerum purum, *Plagiothecium undulatum, *Rhytidiadelphus loreus, *Rhytidiadelphus squarrosus, *Hylocomium splendens, *Dicranum scoparium, *Polytrichum formosum.  Species with a * indicate bryophytes.

The growing season is ranges from June to August, shoulder seasons are April to May and September to October.  During winter (November to February), plants are dormant.

The soil is a humo-ferric podzol.  It is variable across the site, with E and Bh (eluvial and illuvial horizons) not always visible.  Where apparent, the E horizon is at 6-17cm depth, with Bh below onto parent material.

## Clocaenog climate change experimental site characteristics

Site location	 Clocaenog, Wales, UK

Grid ref.	 53° 03' 19''N, -03° 27' 55''W

### Climate (1997-2014)

Mean air temp.	8.0 °C

Mean soil temp.  in control plots 	7.5 °C

Mean annual rainfall	1411 mm

Total N-deposition	20-25 kg N ha-1 yr-1

### Soil

Soil type	Humo-ferric podzol.  Variable across the site, with Eg and Bh (eluvial and Illuvial horizons) not always visible.  Where apparent, e.g., horizon 6-17 cm depth, with Bh below onto parent material.

Soil parent material	Ordovician and Silurian shale or mudstone.

Table 2. Soil characteristics

|  | Soil horizon |  |  |  |
| --- | --- | --- | --- | --- |
|  | LF | Oh | Eg | Bh |
| Depth | 5 | 2 | 10 | 15 |
| %N | 1.43 | 1.40 | 0.23 | 0.10 |
| %C | 49 | 39 | 6 | 2 |
| C:N ratio | 34 | 28 | 27 | 21 |
| H (meq/100g) | 77.0 | 61.0 | 18.6 | 10.6 |
| Ca (meq/100g) | 8.7 | 1.9 | 0.3 | 0.1 |
| Mg (meq/100g) | 6.2 | 2.8 | 0.5 | 0.2 |
| Na (meq/100g) | 1.00 | 0.63 | 0.11 | 0.09 |
| K (meq/100g) | 1.82 | 1.33 | 0.24 | 0.14 |
| Al (meq/100g) | 0.11 | 0.23 | 0.11 | 0.61 |
| Cation Exchange Capacity * | 95 | 67 | 20 | 11 |
| Base saturation (%) | 18.7 | 9.9 | 5.7 | 4.8 |

* CEC calculated as sum of H+ Ca+ Mg+ Na+ and K+

### Vegetation

Ecosystem type	Wet upland heath

Species present	Calluna vulgaris, Vaccinium myrtillus, Empetrum nigrum, Deschampsia flexuosa,*Pleurozium schreberii, *Hypnum jutlandicum, *Psuedosclerum purum, *Plagiothecium undulatum, *Rhytidiadelphus loreus, *Rhytidiadelphus squarrosus, *Hylocomium splendens, *Dicranum scoparium, *Polytrichum formosum
Species with a * are bryophytes.

Annual litterfall (mean)	177 g m-2

Table 3. Vegetation cover and biomass

| Life form | Species | Cover in 1998 (beginning of experiment, %) | Biomass in 1998 (beginning of exp, g m-2) |
| --- | --- | --- | --- |
| Shrub | Calluna vulgaris | 98 | 2485 |
| Shrub | Vaccinium myrtillus | 66 | 435 |
| Shrub | Empetrum nigrum | 25 | 124 |
| Grass | Deschampsia flexuosa | 13 | 14 |
| Bryophytes | (with species above) | 73 | 520 |

### Chemistry

Table 4. Vegetation chemistry

|  | C. vulgaris | V. myrtillus | E. nigrum | D. flexuosa | P. schreberi | Litter |
| --- | --- | --- | --- | --- | --- | --- |
| C (%) | 54.3 | 52 | 54.7 | 46.5 | 49.5 | 54.4 |
| N (%) | 1.4 | 1.5 | 1.2 | 1.2 | 1.1 | 1.8 |
| C:N ratio | 40.3 | 35.7 | 46 | 41.3 | 43.8 | 30.8 |
| P (%) | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.15 |
| K (%) | 0.6 | 0.7 | 0.7 | 1.7 | 0.5 | 0.22 |
| Ca (%) | 0.4 | 1.1 | 0.5 | 0.2 | 0.3 | 0.66 |
| Mg (%) | 0.2 | 0.2 | 0.2 | 0.1 | 0.1 | 0.15 |
| Lignin (%) | 29.1 | 23.1 | 17.5 | 5.1 | 27.8 | 13 |
| Tannin (%) | 11.4 | 12.5 | 10.4 | 2.6 | 0.6 | 1.8 |
| Apha-cellulose (%) | 21.5 | 24.5 | 19.1 | 30.2 | 28 | 25.5 |
| Carbohydrates (%) | 10 | 8.8 | 8.9 | 18.7 | 5.5 | NA |

# Climate change treatment information

The experiment consists of two treatments, drought and warming.  Each has three replicate plots (4 m x 5 m).  There are also three control plots, where we have built scaffolding to mirror any shading that the experimental structure produces on the treatment plots (n=9 plots in total).

Table 5. Treatment plots

| Plot | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Treatment | warming | warming | control | drought | drought | control | warming | drought | control |
|  | W | W | C | D | D | C | W | D | C |

The drought treatment operated annually between May and September between 1999 and 2011.  The timing of the drought was chosen to replicate the drought seen in the UK in 1995.  The treatment works through a retractable polyethylene plastic roof that is triggered by a tipping bucket rainfall sensor.  Between the months of May and -September, whenever rainfall in detected the roof extends over the experimental plot area (4m x 5m) excluding the rainfall.  This reduced annual rainfall by ~20%, as the site is located in one of the wettest areas of the UK.  Some smaller rain events are missed if they are under the detection limit of the tipping bucket rainfall sensor, and this means that within the drought period we exclude ~80% of the rain.  Since 2012, the drought treatment was operated between April and October.  Since 2016, a change in rainfall patterns (likely sudden heavy rain spells) caused accumulation of rainwater on the roof surface, preventing the roofs from retracting properly.

The warming treatment works through retractable aluminium mesh curtains that cover the warming experimental plots during the night.  The aluminium mesh fabric reflects 96-97 % of infrared radiation, reducing the heat loss during night by 64%.  The roof is operated via a light sensor.  Following a period of 15 minutes of dark (judged to be when light levels fall below 200 lux).  A tipping bucket sensor retracts the warming roofs whenever rain is detected to try to prevent a reduction in rainfall within the warming treatment.  However, due to the time lag after a rain event is detected and the warming roofs are retracted, some rainfall is excluded by the warming treatment (on average, annual rainfall is reduced by 14%).

To prevent damage to the curtains, both drought and warming treatment curtains do not operate in high winds (>10 m s−1).  Table 6 gives details and timings of the treatments.

Over the years, the frames for of the climate change manipulations were exposed to the environment, causing the automated mechanism to fail more frequently, and thus needed considerably more time to maintain the treatments.  In 2016, 17 years after the frames were installed, plans were made to replace the original frames.  New mechanisms for anchoring the frames into the ground were tested, but none proofed efficient enough.  It was decided to install new frames on top of the old ones to cause as little disruption to the vegetation and surrounds as possible.  The new frames were installed in June 2017 (Figure 1 [b and c]).

Figure 1.  Picture above showing the frames for the automated manipulation of warming and drought at the Clocaenog field site. a) original roof frame (plot 6), b) and c) new frames installed on top of the original frame in June 2017 (Plot 1), d) and e) show trapped water on the drought roof, preventing the roofs to retract fully.  f) shows the effect of trapped water on roofs on the vegetation.  The vegetation was not permanently damaged and recovered when drought roofs are removed manually.Table 6: Clocaenog climate change experimental site – treatment details:

Table 6(a) Drought treatment Information

| Year | Annual precipitation at site (mm) | Dates of drought treatment | Reduction of annual rainfall (%) | Reduction of growing season rainfall (%) |
| --- | --- | --- | --- | --- |
| 1997 | 1244 | No drought treatment |  |  |
| 1998 | 1530 |  |  |  |
| 1999 | 1827 | 4th Jun - 31st Aug | 28 | 79 |
| 2000 | 2216 | 6th Jun - 29th Aug | 14 | 56 |
| 2001 | 1139 | 5th Jun - 4th Sep | 28 | 80 |
| 2002 | 1540 | 17th Jun - 24th Sep | 15 | 90 |
| 2003 | 1211 | 2nd Jun - 22nd Sep | 24 | 64 |
| 2004 | 1489 | 8th Jun – 1st Sep | 15 | 77 |
| 2005 | 896 | 28th Jun – 20th Sep | 21 | 51 |
| 2006 | 1380 | 13th Jun – 17th Oct | 19 | 81 |
| 2007 | 1524 | 16th Jul – 2nd Oct | 28 | 82 |
| 2008 | 1842 | 24th Apr – 25th Nov | 51 | 81 |
| 2009 | 1386 | 21st May – 22nd Oct | 29 | 69 |
| 2010 | 1041 | 7th May – 17th Sep | 30 | 67 |
| 2011 | 1291 | 4th May – 10th Sep | 23 | 83 |
| 2012 | 1399 | 1st May – 9th Oct | 44 | 70 |
| 2013 | 1210 | 24th Apr – 29th Oct | 29 | 65 |
| 2014 | 1231 | 17th Apr - 29th Oct | 40 | 56 |
| 2015 | 1399 | 7th Apr – 23th Sep | 36 | 47 |
| 2016 | 1289 | Off | 3 | NA |
| 2017 | 1405 | 16th Jun – 4th Aug | 0.6 | NA |
| 2018 | ** | 5th Apr – 12th Apr | -0.1 | NA |
| 2019 | ** | Off | 1.4 | NA |
| 2020 | ** | Off due to COVID | NA*** | NA*** |
| 2021 | ** | Off due to COVID | NA*** | NA*** |
| 2022 | ** | Off | NA*** | NA*** |
| 2023 | ** | Off | NA*** | NA*** |
| 2024 | ** | Off | NA*** | NA*** |
| 2025 | ** | Off | NA*** | NA*** |

*	the drought roofs were not in operation from 2016, first because of refurbishments of the frames, then because of water accumulation on the roofs (see section 2.0 for further information) and COVID-19 restrictions on field work.

**	rainfall sensor on site broken since 04 Jan 2018.

***	the site was not visited during this time, or reduced visits, causing the bottles to overflow and thus, there is no accurate measurement available on rain through-fall onto the plots.

Table 6(b) Warming treatment information from 1997 to 2021.
Calculated growing degree days (GDD) for control and warming treatments based on air temperature data measured in the plots.  We encourage the use of % change in GDD induced by warming as temperature data may not be complete throughout the year.  Warming roofs were turned on together with drought roofs, or independently (compare Table 6a).

| Year | Control growing degree days | Warming growing degree days (GDD) | Change due to warming (%) | Warming roofs operated separately from Drought roofs |
| --- | --- | --- | --- | --- |
| 1997 | NA | No warming treatment yet |  |  |
| 1998 | NA |  |  |  |
| 1999 | 1179 | 1159 | -2 |  |
| 2000 | 941 | 1152 | 22 |  |
| 2001 | 1308 | 1329 | 2 |  |
| 2002 | 1337 | 1391 | 4 |  |
| 2003 | 1389 | 1528 | 10 |  |
| 2004 | 1308 | 1452 | 11 |  |
| 2005 | 1344 | 1484 | 10 |  |
| 2006 | 1258 | 1465 | 16 |  |
| 2007 | NA | NA | NA |  |
| 2008 | NA | NA | NA |  |
| 2009 | 879 | 959 | 9 |  |
| 2010 | 1113 | 1192 | 7 |  |
| 2011 | 1107 | 1195 | 8 |  |
| 2012 | 1049 | 1185 | 13 |  |
| 2013 | 1048 | 1126 | 8 |  |
| 2014 | 1105 | 1193 | 8 |  |
| 2015 | 1161 | 1249 | 8 |  |
| 2016 | 1134 | 1227 | 8 | 6th Apr – 28 Oct |
| 2017 | 1135 | 1214 | 7 | 16th Jun – 16 Nov |
| 2018 | 223 | 234 | 5 | 5th Apr – 11th Dec |
| 2019 | 1172 | 1260 | 8 | 15th Apr – 16 Jul 13 Sep – 15 Nov |
| 2020 | 118 | 122 | 4 | Off due to COVID |
| 2021 | 860 | 927 | 8 | Off due to COVID |
| 2022 | 1281 | 1392 | 9 | Off |
| 2023 | 1192 | 1290 | 8 | Off |
| 2024 | 1144 | 1245 | 8 | Off |
| 2025 | 897 | 964 | 7 | Off |

*	GDD are calculated as averages for the three replicates = averages for min and max air temperatures in control and warming plots respectively.  Micromet data at the plot scale is used to calculate GDD. 
GDD_Control_year = sum (((Temp_air_min + Temp_air-max) / 2) – Temp_air_min) 
GDD_Warming_year =  sum ((Temp_air_min + Temp_air-max) / 2) – Temp_air_min) 
Change due to warming (%) = ((GDD_Warming_year * 100) / GDD_Control_year) - 100

# Equipment, protocols and data processing methodology

An automated weather station (AWS) that provides meteorological data is located on site (hereafter referred to as AWS met data).  There are also a range of sensors in the treatment plots (micro-met plot data).  As well as meteorological data, we have collected a range of abiotic and biotic data from within the plots.  Table 7 provides a summary of the data collected at the Clocaenog field site.

Table 7. Summary of data measured at the Clocaenog field site. Not all data are stored within the EIDC, particularly the infrequently collected smaller datasets or data collected after June 2015.  Contact Sabine Reinsch or Bridget Emmett (enquiries@ceh.ac.uk) in UKCEH Bangor for more details.

| Dataset | Sensors/Method | Frequency | Time scale |
| --- | --- | --- | --- |
| AWS meteorological data | Relative humidity, air temperature, Rainfall, Air pressure, Net radiation, Solar radiation, PAR, Wind speed and direction | Daily | 1999-current day |
| Micro-meteorological plot data | Plot based soil and air temperature and soil moisture | Daily | 1998-current day |
| Rainfall volume and rain chemistry | Ground level rain gauge and warren spring rainfall collector | Monthly | 1998-current day |
| Throughfall data (plot level rainfall) | Plot level rain collection | Fortnightly | 1999-current day |
| Water table depth data | Permeable tubes to the bedrock | Fortnightly | 2009-current day |
| Soil respiration data | IRGA measurements | Fortnightly | 1998-current day |
| Trace gas (CH4 and N2O) measurement | Closed static chambers | Two periods of fortnightly/monthly measurements | 1999-2000 and 2007-2009 |
| Net ecosystem CO2 exchange | Closed static chamber | Infrequent | 2002-2004 and 2009-2011 |
| Photosynthesis | IRGA | Infrequent | 2001-2003 |
| Vegetation survey data (plant biomass) | Pin pointing | Infrequent | 1998-2003 and 2007-2012 |
| Canopy reflectance | Remote sensing method for various indices of plant condition | Infrequent | 2000-2001 and 2003-2004 |
| Vegetation phenology, annual growth, and pathogen assessment | Measurements of shoot elongation and annual growth | Once a year | 1998-2013 |
| Vegetation chemistry | Elemental analysis of vegetation | Annual | 1998-2013 |
| Litterfall | Litterfall collectors | Monthly/yearly, infrequent | 1998-2001, 2008-2009 |
| Root biomass | Soil cores | Infrequent | 2003, 2008, 2010, 2011 |
| Soil microbial biomass | Chloroform incubation | Infrequent | 2000-2001, 2012 |
| Nitrogen mineralisation | Incubation cores | Seasonal for a number of years | 1999-2004 infrequent |
| SOM, SOC and bulk density | Soil cores | Infrequent | 1999-2004, 2013 |
| Soil solution and leachate chemistry | Lysimeters and suction samplers | Monthly | 1998-2008 |

## Micro-meteorological data - plot level data

Automated plot level (micro-met) measurements have been recorded since 2008.  Micro-met data between 1997 and 2008 have been collected as detailed below and in the data.  Soil temperature is measured with T107 sensor forms Campbell Scientific at 5, 10 and 20 cm soil depth.  Soil moisture has been measured using different methods over the years.  It is currently measured using TDR sensors (CS616 from Campbell Scientific, volumetric water content, m3/m3), located sloping from surface to 10 cm down the soil profile.  These were installed in 2008.  Prior to this, manually measurements using a ML-2 theta probe from Delta-t were carried out.  The frequency of sampling varied, but is detailed in the data.

Sensor data are sampled every minute.  Up to January 2016 the data were then logged as hourly, averages after January 2016 data have been logged as averaged half hourly averages.  Data quality control was performed using basic graphic tools in excel to visually examine the data and trends for all measurements.  Although this method seems less exact than setting prescribed limits that data must fit within, it does allow for human interpretation and memory of conditions at the site – for example, an automated limit in a script will fail to remember if it was a particularly cold week, etc.

Other meteorologically related data are: AWS meteorological data

Initially the AWS sensors were placed at 1 m above the soil surface, however after repeated thefts from the site in 2007, we had to mount it on a secured mast in a higher (4 m) position.  In the AWS, the following measures are taken with the indicated sensors:

Table 8.  AWS sensors and measurements

| Measure | Unit | Sensor | Company |
| --- | --- | --- | --- |
| Air temperature | °C | HMP 45C | Campbell Scientific |
| Relative humidity | 0-100% | HMP 45C | Campbell Scientific |
| Solar radiation | W/m2 | Skye SP1110 pyranometer | Skye Instruments |
| PAR (Photosynthetic active radiation) | µmol/m2/sec | SKP215 Quantum Sensor | Skye Instruments |
| Net radiation | W/m2 | NR-Lites | Campbell Scientific |
| Barometric pressure | mbar | CS100 Barometer | Campbell Scientific |
| Rainfall | mm | ARG100 Tipping Bucket | Campbell Scientific |
| Wind speed and direction | m/sec | Windsonic 2D Ultrasonic Anemometer | Windsonic |

For each of the sensors, measurements are sampled every minute and are provided in the form of hourly averages.  Data quality control was performed using basic graphic tools in excel to visually examine the data and trends for all measurements.  Although this method seems less exact than setting prescribed limits that data must fit within, it does allow for human interpretation and memory of conditions at the site – for example, an automated limit in a script will fail to remember if it was a particularly cold week, etc.

## Storage rain gauge data (site level rainfall) and rainfall chemistry

Rainfall at the site is measured using a ground level storage rain gauge (funnel diameter 12.7 cm).  Rain collected in the gauge accumulated over a period of two weeks.  Exception to this occurred over holiday periods, particularly Christmas, when samples accumulated over three or four weeks.  However, the quantity of rainfall accumulated never exceeded the storage capacity of the gauge.  Unless hourly data is required, this storage rain gauge data is preferred to the AWS data, being more robust, i.e., it is far less vulnerable to issues such as logger down time or when power is lost to the site, etc.  Rainfall data collected biweekly.

## Throughfall data (plot level rainfall)

Throughfall data is collected bi-weekly at the same time as the storage rain gauge data (above).  A funnel (diameter 16.8 cm) and bottle (3000 mL capacity) captures the rainfall for each experimental plot and the volume of this is noted.  A degree of calculation is needed with this data to make it comparable with other annual data from the site, such as annual rainfall.  This is because samples can over-flow in these samplers during periods of high rainfall, sometimes freeze, bottles get blown over, funnels fall off or get cracked, etc.

To gain seasonal and annual data for rainfall in each plot, the following was found to produce the most reliable dataset.  The throughfall data is used to calculate a percent change for each treatment, and this % difference is applied to the ground level rain gauge data (site level rainfall) to give the amount of rainfall in each treatment.  Within the calculation of percent difference in throughfall, when data is lost or the measurement is compromised (for example, the funnel falls off the bottle at some point in the week), then this plot is excluded from the treatment mean.  This does mean that on occasion, the treatment mean is calculated with less than the full three replicates.  Also, occasionally data has been infilled using an average reduction over all years.

# Appendix

53o 03’ 19’’N

-03o 27’ 55’’ W

Figure 2.  Site layout for the experimental site

Motor box

Figure 3.  Quadrat layout with the experimental plots; the motor box is located on the 5 m edge closest to “A” plots.

Each quadrat is 0.5 m2.

Figure 4.  Plot layout with quadrats and measurement areas indicated.