Multi-hazard datasets from 10 cities for natural hazard risk modelling

# Summary

This dataset includes hazard data from 10 different cities (Istanbul, Nablus, Chattogram, Cox’s Bazaar, Nairobi, Nakuru, Quito, Kokhana, Rapti and Darussalam), to be used in multi-hazard impact modelling. Each dataset represents the hazardscape of the case study area within the cities mentioned above. The datasets include data range of different data types based on the hazards being representedand the number of hazard scenarios developed. The produced hazard maps are simulations that reflect a “what-if” scenario. Characteristics of the hazard type related data is provided below:

Earthquake data: Data representing a potential earthquake hazard sourced from a seismic event, that may effect the study area. The intensity measure is either VS30 or SA for the simulated event.

Some of the files are in multiband raster format (rapti_eq_sc1.tif, rapti_eq_sc2.tif, rapti_eq_sc3.tif, nablus_eq_sc1). The multiband GeoTIFF files contain ground-motion intensity measures for each earthquake scenario. Band 1 represents PGA, while the remaining bands represent spectral acceleration values, Sa(T), at different structural vibration periods given in seconds, with values interpreted in units of g. These period-dependent Sa bands can be used to select or interpolate the appropriate ground-motion demand for buildings with different dynamic characteristics or approximate height classes.

Flood data: Data representing a potential flood event due to localised heavy rainfall or to overbank riverine inundation. The intensity measure is flood depth (height). Most models are outputs from hydrological models dominated by Caesar-Lisflood.

Landslide / debris flow data: Data representing a potential landslide or debris flow event that is triggered due to heavy rainfall. This uses the LaharFlow model.

Hazard parameter file: This file (in .pdf format) provides information on the background of the each hazard scenario such as the location of the source, attributes included and approach that is used (if available) to create it.

# File format

Multi-hazard datasets includes files with the extensions as below:

.csv, .tif, .tfw, .xml, .pdf

# File names and/or naming convention

List the file names of the data files (including the file extension).

ChattogramMultiHazardDataset/Chattogram_EarthquakeHazardParameters.pdf

ChattogramMultiHazardDataset/Chattogram_Eq_Sc1.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc1.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc10.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc10.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc11.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc11.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc12.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc12.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc13.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc13.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc14.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc14.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc15.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc15.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc16.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc16.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc2.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc2.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc3.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc3.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc4.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc4.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc5.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc5.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc6.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc6.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc7.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc7.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc8.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc8.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Eq_Sc9.tif

ChattogramMultiHazardDataset/Chattogram_Eq_Sc9.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Fl_Sc1.tfw

ChattogramMultiHazardDataset/Chattogram_Fl_Sc1.tif

ChattogramMultiHazardDataset/Chattogram_Fl_Sc1.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Fl_Sc1.tif.ovr

ChattogramMultiHazardDataset/Chattogram_Fl_Sc2.tfw

ChattogramMultiHazardDataset/Chattogram_Fl_Sc2.tif

ChattogramMultiHazardDataset/Chattogram_Fl_Sc2.tif.aux.xml

ChattogramMultiHazardDataset/Chattogram_Fl_Sc2.tif.ovr

ChattogramMultiHazardDataset/Chattogram_FloodHazardParameters.pdf

CoxsBazaarHazardDataset/CoxsBazaar_EarthquakeHazardParameters.pdf

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc1.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc1.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc10.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc10.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc11.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc11.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc12.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc12.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc13.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc13.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc14.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc14.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc15.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc15.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc16.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc16.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc2.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc2.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc3.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc3.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc4.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc4.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc5.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc5.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc6.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc6.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc7.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc7.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc8.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc8.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc9.tif

CoxsBazaarHazardDataset/CoxsBazaar_Eq_Sc9.tif.aux.xml

CoxsBazaarHazardDataset/CoxsBazaar_Fl_Sc1.tif

CoxsBazaarHazardDataset/CoxsBazaar_Fl_Sc2.tif

CoxsBazaarHazardDataset/CoxsBazaar_Fl_Sc3.tif

CoxsBazaarHazardDataset/CoxsBazaar_Fl_Sc4.tif

CoxsBazaarHazardDataset/CoxsBazaar_FloodHazardParameters.pdf

DarMultiHazardDataset/Dar_Fl_Sc1.tif

DarMultiHazardDataset/Dar_Fl_Sc2.tif

DarMultiHazardDataset/Dar_FloodHazardParameters.pdf

IstanbulMultiHazardDataset/Istanbul_EarthquakeHazardParameters.pdf

IstanbulMultiHazardDataset/Istanbul_Eq_Sc1.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc10.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc11.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc12.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc13.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc14.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc15.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc16.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc17.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc18.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc19.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc2.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc20.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc21.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc22.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc23.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc24.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc25.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc26.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc27.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc28.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc29.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc3.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc30.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc4.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc5.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc6.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc7.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc8.csv

IstanbulMultiHazardDataset/Istanbul_Eq_Sc9.csv

KokhanaMultiHazardDataset/Kokhana_EarthquakeHazardParameters.pdf

KokhanaMultiHazardDataset/Kokhana_Eq_Sc1.csv

KokhanaMultiHazardDataset/Kokhana_Fl_Sc1.tif

KokhanaMultiHazardDataset/Kokhana_Fl_Sc2.tif

KokhanaMultiHazardDataset/Kokhana_Fl_Sc3.tif

KokhanaMultiHazardDataset/Kokhana_FloodHazardParameters.pdf

NablusMultiHazardDataset/Nablus_EarthquakeHazardParameters.pdf

NablusMultiHazardDataset/Nablus_Eq_Sc1.tif

NablusMultiHazardDataset/Nablus_Fl_Sc1.tif

NablusMultiHazardDataset/Nablus_FloodHazardParameters.pdf

NairobiMultiHazardDataset/_nairobi_flood_depth_100yr.geojson

NairobiMultiHazardDataset/Nairobi_FloodHazardParameters.pdf

NakuruMultiHazardDataset/Nakuru_Fl_Sc1.tif

NakuruMultiHazardDataset/Nakuru_FloodHazardParameters.pdf

QuitoMultiHazardDataset/DF-SCEN01-DPTH.csv

QuitoMultiHazardDataset/Quito_DebrisFlowHazardParameters.pdf

RaptiMultiHazardDataset/Rapti_EarthquakeHazardParameters.pdf

RaptiMultiHazardDataset/Rapti_EQ_Sc1.tif

RaptiMultiHazardDataset/Rapti_EQ_Sc2.tif

RaptiMultiHazardDataset/Rapti_EQ_Sc3.tif

RaptiMultiHazardDataset/Rapti_Fl_Sc1.tif

RaptiMultiHazardDataset/Rapti_Fl_Sc2.tif

RaptiMultiHazardDataset/Rapti_FloodHazardParameters.pdf

RaptiMultiHazardDataset/Rapti_LandslideHazardParameters.pdf

RaptiMultiHazardDataset/Rapti_Ls_Sc1.tif

RaptiMultiHazardDataset/Rapti_Ls_Sc2.tif

# Nature and units of recorded values

In this section, the file contents are described to provide detailed information about what they include as attributes and about the definitions of the attributes.

Table 1: Description of variables in earthquake hazard data

| Data type | Float32 - Thirty-two-bit floating point |
| --- | --- |
| GDAL Driver Description | GTiff |
| GDAL Driver Metadata | GeoTIFF |
| Dataset Description | Earthquake scenario that considers climate change, which has an increasing effect on the flood intensity |
| Pixel Value | Corresponds to earthquake intensity (VS30 (m/s) or Spectral Accelaration (g)) |

Table 2: Description of variables in flood hazard data

| Data type | Float32 - Thirty two bit floating point |
| --- | --- |
| GDAL Driver Description | GTiff |
| GDAL Driver Metadata | GeoTIFF |
| Dataset Description | Flood scenario that does not consider climate change and is generated based on existing meteorological trends. |
| Pixel Value | Corresponds to flood intensity (flow depth/height) |

# Spatial coverage

# Different case study areas in Istanbul, Nablus, Chattogram, Cox’s Bazaar, Nairobi, Nakuru, Quito, Kokhana, Rapti and Darussalam in approximately 500 ha of areas.

# Collection/Generation/Transformation methods

- We employed QGIS software to manipulate and visualise the datasets.

- Remote sensing data is used to observe landslide areas.

- Site observations are conducted to determine the extent and severity of floods and landslides.

- Geophysical measurements were conducted to identify the subsurface characteristics that determine the surface response to a seismic event.

- Literature reviews and discussions with local science colleagues were conducted to understand the hazard contexts in the case study areas.

# Quality control

The spatial topological errors were removed using specific topological debugging software modules provided in QGIS software. In the case of the flood modelling, bridges and gorges that generate barriers on a DEM were removed using a channel carving algorithm.

# References

Cremen, G., Galasso, C. and McCloskey, J., 2022. Modelling and quantifying tomorrow's risks from natural hazards. Science of The Total Environment, 817, p.152552.

Gentile, R., Cremen, G., Galasso, C., Jenkins, L.T., Manandhar, V., Menteşe, E.Y., Guragain, R. and McCloskey, J., 2022. Scoring, selecting, and developing physical impact models for multi-hazard risk assessment. International Journal of Disaster Risk Reduction, 82, p.103365.

Jenkins, L. T., Creed, M. J., Tarbali, K., Muthusamy, M., Trogrlić, R. Š., Phillips, J. C., Watson, C. S., Sinclair, H. D., Galasso, C., & McCloskey, J. (2022). Physics-based simulations of multiple natural hazards for risk-sensitive planning and decision making in expanding urban regions. International Journal of Disaster Risk Reduction, 84, 103338. https://doi.org/10.1016/j.ijdrr.2022.103338

Thapa, S., Sinclair, H.D., Creed, M.J., Borthwick, A.G., Watson, C.S. and Muthusamy, M., 2024. Sediment transport and flood risk: Impact of newly constructed embankments on river morphology and flood dynamics in Kathmandu, Nepal. Water Resources Research, 60(10), p.e2024WR037742. https://doi.org/10.1029/2024WR037742

Süleyman, H., Çaktı, E. Assessing seismic hazard and uncertainty in Büyükçekmece using ground motion simulations. Bull Earthquake Eng 22, 4873–4895 (2024). https://doi.org/10.1007/s10518-024-01953-6

Watson, C.S., Gyawali, J., Creed, M. and Elliott, J.R., 2024. City-scale high-resolution flood models and the role of topographic data: a case study of Kathmandu, Nepal. Geocarto International, 39(1), p.2387073.