# Supporting documentation for:

# Daily and sub-daily hydrometeorological and soil moisture data (2013-2025) [COSMOS-UK]

Contents

1	Introduction	2

2	COSMOS-UK sites	3

3	Sub-hourly hydrometeorological and soil data	6

4	Daily hydrometeorological and soil data	13

5	Infilling	21

6	Completeness	22

7	Instrumentation	23

8	Changes to data	33

9	Author contributions	35

10	References	36

# Introduction

This dataset contains daily and sub-daily time series of hydrometeorological and soil moisture data from 51 sites across the COSMOS-UK network from 2013 to the end of 2025.

The dataset consists of a total of 258 files:

- 1 x COSMOS-UK sites metadata file (section 2)

  - COSMOS-UK_SiteMetadata_2013-2025.csv

  - Contains metadata about each of the 51 COSMOS-UK sites.

- 51 x Sub-hourly hydrometeorological and soil data files (section 3)

  - COSMOS-UK_<SITE ID>_HydroSoil_SH_<YEAR SITE OPENED>-2025.csv

  - Each file contains sub-hourly hydrometeorological and soil data for the length of record of a given site.

- 51 x Sub-hourly hydrometeorological and soil quality control flags files (section 3.6)

  - COSMOS-UK_<SITE ID>_HydroSoil_SH_<YEAR SITE OPENED>-2025_QC_Flags.csv

  - Each file contains sub-hourly flags describing the quality control tests failed by any removed data points from the hydrometeorological and soil data file for a given site.

- 51 x Sub-hourly hydrometeorological and soil data flags files (section 3.7)

  - COSMOS-UK_<SITE ID>_HydroSoil_SH_<YEAR SITE OPENED>-2025_Flags.csv

  - Each file contains sub-hourly data flags describing general information about each corresponding data point (i.e. whether data is missing, infilled, estimated, unchecked) in the hydrometeorological and soil data file for a given site.

- 1 x Sub-hourly hydrometeorological and soil metadata file (section 3)

  - COSMOS-UK_HydroSoil_SH_2013-2025_Metadata.csv

  - Contains metadata about the variables within the sub-hourly data files.

- 51 x Daily hydrometeorological and soil data files, including VWC, potential evapotranspiration (PE) and snow water equivalent (SWE) (section 4)

  - COSMOS-UK_<SITE ID>_HydroSoil_Daily _<YEAR SITE OPENED>-2025.csv

  - Each file contains daily hydrometeorological and soil data for the length of record of a given site.

- 51 x Daily hydrometeorological and soil data flags files (section 4.7)

  - COSMOS-UK_<SITE ID>_HydroSoil_Daily_<YEAR SITE OPENED>-2025_Flags.csv

  - Each file contains daily data flags describing general information about each corresponding data point (i.e. whether data is missing, infilled, estimated, unchecked) in the hydrometeorological and soil data file for a given site.

- 1 x Daily hydrometeorological and soil metadata (section 4)

  - COSMOS-UK_HydroSoil_Daily_2013-2025_Metadata.csv

  - Contains metadata about the variables within the daily data files.

Full details of each of these files is given in the linked sections.

For more information on the COSMOS-UK network as a whole, please visit the website https://cosmos.ceh.ac.uk/. This supporting document is based in part on the COSMOS-UK User Guide which is regularly updated and made available from the above referenced website.

COSMOS-UK is supported by the Natural Environment Research Council award number NE/Y006208/1 as part of the UKCEH NC-UK programme delivering National Capability.

# COSMOS-UK sites

## Filename

COSMOS-UK_SiteMetadata_2013-2025.csv

## Description

Figure 2.1 shows the locations of each site. Further details, including co-ordinates, soil type and land cover are provided in the site metadata file.

Figure 2.1 – COSMOS-UK site locations.

Note, the following sites have been decommissioned:

| Site | Decommission date |
| --- | --- |
| Wytham Woods | 01/10/2016 |
| Redmere | 20/09/2018 |
| Cochno | 16/11/2020 |
| Harwood Forest | 28/06/2022 |
| Crichton | 17/12/2024 |
| Writtle | 15/01/2025 |
| Cardington | 11/02/2025 |
| Tadham Moor | 21/10/2025 |
| Stoughton | 26/11/2025 |

## File content

The content of COSMOS-UK site metadata file is outlined in Table 2.1.

Table 2.1 - Details of columns within the file.

| Column name | Unit | Description |
| --- | --- | --- |
| SITE_NAME | - | Full site name. |
| SITE_ID | - | Unique five character identifier for site. |
| START_DATE | - | Site’s start date in format YYYY-MM-DD. |
| END_DATE | - | For sites no longer active, the end date in format YYYY-MM-DD. This is left blank if site is still active. |
| EASTING | metres | Site coordinate. Coord ref system: OSGB36, EPSG SRID code: 27700. |
| NORTHING | metres | Site coordinate. Coord ref system: OSGB36, EPSG SRID code: 27700. |
| EAST_NORTH_PROJECTION | - | EPSG SRID code: 27700 provided for reference. |
| LATITUDE | degrees | Site coordinate. Coord ref system: WGS84, EPSG SRID code: 4326. |
| LONGITUDE | degrees | Site coordinate. Coord ref system: WGS84, EPSG SRID code: 4326. |
| LAT_LONG_PROJECTION | - | EPSG SRID code: 4326 provided for reference. |
| ALTITUDE | metres | Altitude of the site above sea level. See Section 2.3.1. |
| SOIL_TYPE | - | Informal soil type description derived from site inspection and soil sampling. |
| LAND_COVER | - | Main land cover as observed at the site. |
| BULK_DENSITY | g cm-3 | Mass per unit volume derived from soil samples taken from the site1. |
| BULK_DENSITY_SD | g cm-3 | Standard deviation of bulk density across soil samples taken from site1. |
| SOIL_ORGANIC_CARBON | g / g | Organic carbon content within dry soil mass, derived from soil samples taken from the site1. |
| SOIL_ORGANIC_CARBON_SD | g / g | Standard deviation of soil organic carbon across soil samples taken from site1. |
| LATTICE_WATER | g / g | Lattice and bound water. Water integrated into mineral structures within the soil1. |
| LATTICE_WATER_SD | g / g | Standard deviation of lattice water across soil samples taken from site1. |

### Site altitude

Site altitude metadata is extracted from third-party elevation data mapping products.  For most sites, altitude is extract from the NEXTMap 10 m Digital Terrain Model (DTM) product.  Due to DTM coverage limitations, altitude data for Fivemiletown, Glenwherry and Hillsborough have been estimated from available Ordnance Survey of Northern Ireland (OSNI) contour data, and data for Glensaugh has been estimated from Ordnance Survey (OS) 25k mapping contours. Due to identified DTM accuracy issues in woodland, data for Alice Holt, Harwood Forest and Wytham Woods have also been estimated from OS 25k mapping contours.

# Sub-hourly hydrometeorological and soil data

## Data filename

COSMOS-UK_<SITE ID>_HydroSoil_SH_<YEAR SITE OPENED>-2025.csv

## Metadata filename

COSMOS-UK_HydroSoil_SH_2013-2025_Metadata.csv

## Description

A range of hydrometeorological and soil data at 30 minute resolution over the period October 2013 - December 2025. Missing values are given as -9999. The <SITE_ID> given in the filename identifies the site from the COSMOS-UK network where the data were measured.

## File content

The sub-hourly metadata file (Section 3.2) provided with the dataset contains details of the variables included in the sub-hourly data files. Table 3.1 is a descriptive supplement to this which can also be used to check the meaning of codes used in the metadata file.

See Section 6 for general completeness of the data.

Table 3.1 – Details of the sub-hourly variables recorded.

| Column name | Variable name | Unit | Aggregation | Infilled | Note |
| --- | --- | --- | --- | --- | --- |
| SITE_ID | Site | - | - | - |  |
| DATE_TIME | DateTime | - | - | - | GMT Dates given in ISO 8601 format: YYYY-MM-DDTHH:MM:SSZ. |
| LWIN | Incoming longwave radiation | Watts per square metre (W m-2) | Mean over preceding time period | Yes |  |
| LWOUT | Outgoing longwave radiation | Watts per square metre (W m-2) | Mean over preceding time period | Yes |  |
| SWIN | Incoming shortwave radiation | Watts per square metre (W m-2) | Mean over preceding time period | Yes |  |
| SWOUT | Outgoing shortwave radiation | Watts per square metre (W m-2) | Mean over preceding time period | Yes |  |
| RN | Net radiation | Watts per square metre (W m-2) | Mean over preceding time period |  | Derived from: (SWIN – SWOUT) + (LWIN – LWOUT) |
| PRECIP | Precipitation from Pluvio | Millimetres (mm) | Total over preceding time period |  |  |
| PRECIP_TIPPING | Precipitation from tipping bucket | Millimetres (mm) | Total over preceding time period |  |  |
| PRECIP_RAINE | Precipitation from rain[e] | Millimetres (mm) | Total over preceding time period |  |  |
| PA | Atmospheric Pressure | HectoPascals (hPa) | Mean over preceding time period | Yes |  |
| TA | Air Temperature | Degrees Celsius (℃) | Mean over preceding time period | Yes |  |
| WS | Wind speed | Metres per second (m s-1) | Mean over preceding time period | Yes | Absolute wind speed, not resultant wind speed of WD |
| WD | Wind direction | Degrees (°) | Mean over preceding time period | Yes |  |
| Q | Absolute Humidity | Grams per cubic metre (g m-3) | Mean over preceding time period |  | Derived from TA and RH |
| RH | Relative Humidity | Percent (%) | Mean over preceding time period | Yes |  |
| SNOW_DEPTH | Snow depth | Millimetres (mm) | Value at timestamp |  | Data only given on snow days. |
| UX | X component of wind speed | Metres per second (m s-1) | Mean over preceding time period |  |  |
| UY | Y component of wind speed | Metres per second (m s-1) | Mean over preceding time period |  |  |
| UZ | Z component of wind speed | Metres per second (m s-1) | Mean over preceding time period |  |  |
| G1 | Heat flux 1 | Watts per square metre (W m-2) | Mean over preceding time period | Yes |  |
| G2 | Heat flux 2 | Watts per square metre (W m-2) | Mean over preceding time period | Yes |  |
| TDT1_TSOIL TDT2_TSOIL | Soil temperature | Degrees Celsius (℃) | Value at timestamp |  | Soil temperature from TDT sensor 1 and 2 at 10 cm* |
| TDT1_VWC TDT2_VWC | Soil moisture | Percent (%) | Value at timestamp |  | Volumetric water content from TDT sensor 1 and 2 at 10 cm* |
| TDT3_TSOIL TDT4_TSOIL | Soil temperature | Degrees Celsius (℃) | Value at timestamp |  | Soil temperature from TDT sensor 3 and 4 at 5 cm* |
| TDT3_VWC TDT4_VWC | Soil moisture | Percent (%) | Value at timestamp |  | Volumetric water content from TDT sensor 3 and 4 at 5 cm* |
| TDT5_TSOIL TDT6_TSOIL | Soil temperature | Degrees Celsius (℃) | Value at timestamp |  | Soil temperature from TDT sensor 5 and 6 at 15 cm* |
| TDT5_VWC TDT6_VWC | Soil moisture | Percent (%) | Value at timestamp |  | Volumetric water content from TDT sensor 5 and 6 at 15 cm* |
| TDT7_TSOIL TDT8_TSOIL | Soil temperature | Degrees Celsius (℃) | Value at timestamp |  | Soil temperature from TDT sensor 7 and 8 at 25 cm* |
| TDT7_VWC TDT8_VWC | Soil moisture | Percent (%) | Value at timestamp |  | Volumetric water content from TDT sensor 7 and 8 at 25 cm* |
| TDT9_TSOIL TDT10_TSOIL | Soil temperature | Degrees Celsius (℃) | Value at timestamp |  | Soil temperature from TDT sensor 9 and 10 at 50 cm, except for Heytesbury site where sensors at 5 cm* |
| TDT9_VWC TDT10_VWC | Soil moisture | Percent (%) | Value at timestamp |  | Volumetric water content from TDT sensor 9 and 10 at 50 cm, except for Heytesbury site where sensors at 5 cm* |
| STP_TSOIL2 | Soil temperature at depth 2 cm | Degrees Celsius (℃) | Mean over preceding time period |  |  |
| STP_TSOIL5 | Soil temperature at depth 5 cm | Degrees Celsius (℃) | Mean over preceding time period |  |  |
| STP_TSOIL10 | Soil temperature at depth 10 cm | Degrees Celsius (℃) | Mean over preceding time period |  |  |
| STP_TSOIL20 | Soil temperature at depth 20 cm | Degrees Celsius (℃) | Mean over preceding time period |  |  |
| STP_TSOIL50 | Soil temperature at depth 50 cm | Degrees Celsius (℃) | Mean over preceding time period |  |  |

* See Section 7 Instrumentation for details.

## Methodology

Data are measured by a range of instruments and then recorded by data loggers on site (see Section 7 for details on instrumentation). Some values are instantaneous measurements, taken every 30 minutes, others are aggregated over 30 minutes (either sum or mean) on the logger. The “Aggregation” column in table 3.1 details the aggregated method for each variable. Timestamps for all aggregated data refer to the end of the 30 minute period, i.e. a time of 12:30:00, refers to an aggregation of data from 12:00.01 to 12:30.00. These data are then telemetered to a central data system based at UKCEH Wallingford every hour.

## Quality control

Data are subject to two stages of quality control (QC):

- QC tests automatically applied by a processing script. Data that fail these tests are flagged and removed. Each test is only applied to relevant variables. Details of the tests are given in Table 3.2 and details on which variables are subject to which test are given in Table 3.3.

- A daily visual inspection of all data on automatically generated plots.

Each automatic QC test comes with a unique flag value. This value is assigned to any data that fails a QC test, see Table 3.2 for the flag values used for each test. If the same data fails multiple tests, then the flag values are added together. The flag values are such that any combination added together will result in a unique final value, thus the exact tests a value fails can be traced back.

Data that fails QC are removed and the accumulated flag value is saved separately in the sub-hourly quality control flags dataset, with filename:

COSMOS-UK_<SITE ID>_HydroSoil_SH_<YEAR SITE OPENED>-2025_QC_Flags.csv

The QC flag files mirror the structure of their counterpart data file, except that all column names have “_QCFLAG” appended.

For example, if the value for PRECIP at site MOORH on 01-01-2019 00:30:00 was missing in the file (value is -9999), then the corresponding QC flag (in the PRECIP_QCFLAG column of the site’s QC file) would have a value larger than 0. Say that flag value was 72, then it can be deduced that data was removed because it failed the “low power” (8) and “out of range” (64) tests (see Table 3.2).  Note that a value of 0 indicates that the data point has passed all the QC tests.

Table 3.2 – Description of automatic quality control tests, and their flag values.

| Test | Description | Flag value |
| --- | --- | --- |
|  | The data point has passed all QC tests. | 0 |
| Missing | No data received from logger. | 1 |
| Zero data | Data equal to zero where this is not a possible value. For certain variables missing data are marked using a zero. For variables where this is true any zero values are removed as these are assumed to be missing. | 2 |
| Too few sample | Data with too few half hourly samples. For variables that are a sum or average of numerous continuous readings in the preceding half hour period; if less than a third of these readings are missing the measurement is considered unreliable and data are removed. | 4 |
| Low power | Data recorded where battery voltage is low. Low battery power can mean measurements are missing or unreliable. If the battery pack voltage goes below 11V the associated data will be removed. | 8 |
| Sensor fault | Data associated with a sensor that has a known fault. These are registered manually by the operations team. | 16 |
| Diagnostic flag | Data that has been assigned a diagnostic flag by the instrument. | 32 |
| Out of range | Data that are outside an acceptable range for that variable. Each variable measured at each site has a minimum and maximum value set. If the measurement of this particular variable goes out of this range it will be removed. | 64 |
| Secondary variable | Data dependant on another variable and the other variable is incorrect. Some measurements are dependent on the measurements of another variable being reasonable. For example, measurements of the components of radiation are not reliable when the body temperature (of the radiometer) measurement is out of the acceptable range. This test will remove values from the dependent variables if the main variable is not correct. | 128 |
| Midnight soil heat flux calibration | Due to residual heat from midnight calibrations, small spike in the soil heat flux data were regularly seen at time steps 00:30 and 01:00. Data at these time steps are routinely removed, and subsequently infilled using interpolation. | 256 |
| Spike | Data that are greater than a threshold value smaller/larger than the neighbouring values. If a value is greater than a certain threshold away from its neighbouring values this is removed. | 512 |
| Error codes | Data where the logger program has assigned an error code value due to a sensor/program fault. When there is a fault with the sensor for some variables the logger program can record a value of 7999. | 1024 |

Table 3.3 – This table details the quality control tests that are run for each variable. See table 3.1 for descriptions of each parameter.

| Parameter | Zero | Too few samples | Low power | Sensor fault | Diagnostic | Out of range | Secondary variable | Spike | Error codes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LWIN |  | x | x | x |  | x | x |  | x |
| LWOUT |  | x | x | x |  | x | x |  | x |
| SWIN |  | x | x | x |  | x | x |  | x |
| SWOUT |  | x | x | x |  | x | x |  | x |
| RN |  |  |  |  |  |  |  |  |  |
| PRECIP |  | x | x | x | x | x |  |  | x |
| PRECIP_TIPPING |  | x | x | x |  | x |  |  | x |
| PRECIP_RAINE |  | x | x | x |  | x |  |  | x |
| PA | x | x | x | x |  | x |  | x | x |
| TA |  | x | x | x |  | x |  | x | x |
| WS | x | x | x | x |  | x |  |  | x |
| WD |  | x | x | x |  | x |  |  | x |
| Q3 |  |  |  |  |  |  |  |  |  |
| RH | x | x | x | x |  | x |  |  | x |
| SNOW_DEPTH |  |  | x | x |  | x | x |  | x |
| UX |  |  | x | x |  | x |  |  | x |
| UY |  |  | x | x |  | x |  |  | x |
| UZ |  |  | x | x |  | x |  |  | x |
| G1 |  | x | x | x |  | x | x |  | x |
| G2 |  | x | x | x |  | x | x |  | x |
| TDT#_TSOIL |  |  | x | x |  | x |  |  | x |
| TDT#_VWC | x |  | x | x |  | x | x | x | x |
| STP_TSOIL2 |  | x | x | x |  | x |  |  | x |
| STP_TSOIL5 |  | x | x | x |  | x |  |  | x |
| STP_TSOIL10 |  | x | x | x |  | x |  |  | x |
| STP_TSOIL20 |  | x | x | x |  | x |  |  | x |
| STP_TSOIL50 |  | x | x | x |  | x |  |  | x |

## Data flags

Data flags are provided in a separate file:

COSMOS-UK_<SITE ID>_HydroSoil_SH_<YEAR SITE OPENED>-2025_Flags.csv

There are 4 possible flags, outlined in Table 3.4.  Note that blank cells in the CSV file indicate no issues with the corresponding data values.

Table 3.4 – Data flag values.

| Flag | Description |
| --- | --- |
| M | Missing. Where data are missing and not infilled. Note, for snow parameters, e.g. SNOW_DEPTH, the values are only given on snow days. This means there are no data on non-snow days, but these are not considered missing and will not have the M flag. |
| U | Unchecked. Where data have had no quality control. Note, variables that have been derived from QC’d data, e.g. net radiation, are not QC’d, however they are still considered checked. The only truly unchecked variable is the daily greenness proportion (GCC). |
| I | Infilled. Where missing data have been filled in directly using an infill method. See Section 5 for more details on infilling. |
| E | Estimated. Where the value contains some degree of uncertainty. This can be for one of two reasons: 1. The value was aggregated with less than a full set of data. For example, a daily mean temperature where some of the sub daily values were missing. 2. The value was aggregated or derived with some degree of infilled data. For example, if net radiation (RN) was calculated using an infilled SWIN value. |

The data flag files mirror the structure of their counterpart data file, except that all column names have “_FLAG” appended.

For example, if the value for TA_FLAG at site BUNNY on 01-01-2020 10:30:00 in the file…

COSMOS-UK_BUNNY_HydroSoil_SH_2013-2025_Flags.csv

was “I”, this means the corresponding data value in the file…

COSMOS-UK_BUNNY_HydroSoil_SH_2013-2025.csv

is an infilled value, not a raw measurement.

It is worth clarifying the difference between the QC flags, and the data flags.

- The QC flags explain why a data value was removed. They hold no information about the data present in the files, only the flagged results of automated quality control tests. If a non-zero QC flag has a corresponding data value, this implies that value was infilled or estimated.

- The data flags however, are information about the data value, mainly whether it is infilled or estimated. These are arguably more useful than the QC flags.

# Daily hydrometeorological and soil data

## Data Filename

COSMOS-UK_<SITE ID>_HydroSoil_Daily_<YEAR SITE OPENED>-2025.csv

## Metadata filename

COSMOS-UK_HydroSoil_Daily_2013-2025_Metadata.csv

## Description

Volumetric water content data, potential evapotranspiration and snow water equivalent are derived from hydrometeorological data and data from the Cosmic-Ray Neutron Sensor (CRNS) at a daily resolution over the period October 2013 - December 2025.  Missing values are given as -9999. The <SITE_ID> given in the filename identifies the site from the COSMOS-UK network where the data was measured.

## File content

The daily metadata file (Section 4.2) contains details of the variables included in the daily data files. Table 4.1 is a descriptive supplement to this which can also be used to check the meaning of codes used in the metadata file.

Volumetric water content data are derived at all sites. See Section 6 for completeness of the VWC data.

Table 4.1 - Details of the daily variables recorded.

| Column name | Variable name | Unit | Aggregation | Infilled | Note |
| --- | --- | --- | --- | --- | --- |
| SITE_ID | Site | - | - | - |  |
| DATE_TIME | DateTime | - | - | - | Dates given in format: YYYY-MM-DD |
| LWIN | Incoming longwave radiation | Megajoule per square metre per day (MJ m-2 day-1) | Daily total |  |  |
| LWOUT | Outgoing longwave radiation | Megajoule per square metre per day (MJ m-2 day-1) | Daily total |  |  |
| SWIN | Incoming shortwave radiation | Megajoule per square metre per day (MJ m-2 day-1) | Daily total |  |  |
| SWOUT | Outgoing shortwave radiation | Megajoule per square metre per day (MJ m-2 day-1) | Daily total |  |  |
| RN | Net radiation | Megajoule per square metre per day (MJ m-2 day-1) | Daily total |  |  |
| PRECIP | Precipitation from Pluvio | Millimetres (mm) | Daily total |  |  |
| PRECIP_TIPPING | Precipitation from tipping bucket | Millimetres (mm) | Daily total |  |  |
| PRECIP_RAINE | Precipitation from rain[e] | Millimetres (mm) | Daily total |  |  |
| PA | Atmospheric Pressure | Hectopascals (hPa) | Daily mean |  |  |
| TA | Air Temperature | Degrees Celsius (℃) | Daily mean |  |  |
| WS | Wind speed | Metres per second (m s-1) | Daily mean |  |  |
| WD | Wind direction | Degrees (°) | Daily mean |  | Absolute direction mean. Does not account for WS. |
| Q | Absolute Humidity | Grams per cubic metre (g m-3) | Daily mean |  |  |
| RH | Relative Humidity | Percent (%) | Daily mean |  |  |
| G1 | Heat flux 1 | Watts per square metre (W m-2) | Daily mean |  |  |
| G2 | Heat flux 2 | Watts per square metre (W m-2) | Daily mean |  |  |
| TDT1_TSOIL TDT2_TSOIL | Soil temperature | Degrees Celsius (℃) | Daily mean |  | Soil temperature from TDT sensor 1 and 2 at 10 cm* |
| TDT1_VWC TDT2_VWC | Soil moisture | Percent (%) | Daily mean |  | Volumetric water content from TDT sensor 1 and 2 at 10 cm* |
| TDT3_TSOIL TDT4_TSOIL | Soil temperature | Degrees Celsius (℃) | Daily mean |  | Soil temperature from TDT sensor 3 and 4 at 5 cm* |
| TDT3_VWC TDT4_VWC | Soil moisture | Percent (%) | Daily mean |  | Volumetric water content from TDT sensor 3 and 4 at 5 cm* |
| TDT5_TSOIL TDT6_TSOIL | Soil temperature | Degrees Celsius (℃) | Daily mean |  | Soil temperature from TDT sensor 5 and 6 at 15 cm* |
| TDT5_VWC TDT6_VWC | Soil moisture | Percent (%) | Daily mean |  | Volumetric water content from TDT sensor 5 and 6 at 15 cm* |
| TDT7_TSOIL TDT8_TSOIL | Soil temperature | Degrees Celsius (℃) | Daily mean |  | Soil temperature from TDT sensor 7 and 8 at 25 cm* |
| TDT7_VWC TDT8_VWC | Soil moisture | Percent (%) | Daily mean |  | Volumetric water content from TDT sensor 7 and 8 at 25 cm* |
| TDT9_TSOIL TDT10_TSOIL | Soil temperature | Degrees Celsius (℃) | Daily mean |  | Soil temperature from TDT sensor 9 and 10 at 50 cm, except for Heytesbury site where sensors at 5 cm* |
| TDT9_VWC TDT10_VWC | Soil moisture | Percent (%) | Daily mean |  | Volumetric water content from TDT sensor 9 and 10 at 50 cm, except for Heytesbury site where sensors at 5 cm* |
| STP_TSOIL2 | Soil temperature at depth 2 cm | Degrees Celsius (℃) | Daily mean |  |  |
| STP_TSOIL5 | Soil temperature at depth 5 cm | Degrees Celsius (℃) | Daily mean |  |  |
| STP_TSOIL10 | Soil temperature at depth 10 cm | Degrees Celsius (℃) | Daily mean |  |  |
| STP_TSOIL20 | Soil temperature at depth 20 cm | Degrees Celsius (℃) | Daily mean |  |  |
| STP_TSOIL50 | Soil temperature at depth 50 cm | Degrees Celsius (℃) | Daily mean |  |  |
| COSMOS_VWC | Soil moisture (volumetric water content) from CRNS | Percent (%) | Daily mean | Yes | See section 4.5.1 for derivation |
| CTS_MOD_CORR | Corrected moderated counts from CRNS | Count | Daily total |  | See section 4.5.1 for derivation |
| D86_75m | Effective depth of CRNS (D86 at 75m) | Centimetres (cm) | Daily mean |  | See section 4.5.1 for derivation |
| SNOW | Snow days | - | - |  | 1 if snow detected on day, 0 if not. Derived from Albedo |
| SNOW_DEPTH | Snow depth | Millimetres (mm) | Daily mean |  | Data only given on snow days |
| SWE | Snow water equivalent | Millimetres (mm) | Value at midday (12:00) |  | See section 4.5.2 for derivation |
| ALBEDO | Albedo | - | Mean between 10:00 and 14:00 |  | Derived from: SWOUTSWIN |
| PE | Potential evapo-transpiration | Millimetres (mm) | Daily total |  | See section 4.5.3 for derivation |
| GCC | Green colour content. Proportion of green from a PhenoCam image | - | Daily max |  | Derived from: GreenGreen+Red+Blue See section 4.5.4 for more |

* See Instrumentation for details.

## Methodologies

### COSMOS_VWC, CTS_MOD_CORR and D86_75m

Volumetric water content (VWC) measurements are achieved by monitoring the cosmic rays, the sub-atomic particles that originate from outer space, with a Cosmic-Ray Neutron Sensor (CRNS). See Section 7 for more details on the CRNS.

As the cosmic rays reach Earth’s atmosphere, they collide with atoms in the air and create a shower of secondary particles, which include high energy neutrons. As these neutrons head towards the Earth’s surface, they collide with other particles in the air and soil, losing energy with each collision. As it happens, hydrogen is the most effective element in terms of stopping power of these fast neutrons, therefore the amount of water within the soil (and air) correlates to the number of high energy (fast) neutrons picked up by the CRNS. This is the basis of the cosmic-ray soil moisture method. A sensor at the land surface will count more fast neutrons when there is little hydrogen (water) present and fewer fast neutrons when there is more hydrogen to remove the energy.

In order to derive an accurate VWC measurement from this, firstly correction factors are applied to the recorded neutron counts to account for variations in background cosmic ray intensity (as measured by a high altitude reference site at Jungfraujoch, Switzerland, http://www.nmdb.eu/station/jung/), and also site altitude, atmospheric pressure (PA) and absolute humidity (Q). PA and Q are taken from the quality controlled sub-hourly hydrometeorological and soil data described in Section 3. This adjusted number of counts is known as the ‘corrected counts’, given in the data as CTS_MOD_CORR.

There are currently three methods that can be used to derive water content from the corrected counts: (1) Site specific N0 method, (2) universal calibration method, also known as hydrogen molar fraction (hmf) method, and (3) neutron transport modelling (e.g. MCNP, COSMIC, URANOS). These methods are described in Baatz et al. (2014) and Bogena et al. (2015). The first of these methods is the most straightforward to apply and as a consequence the most widely used. Baatz et al. (2014) conclude that all three methods estimate soil water content with acceptable errors when compared to estimates determined using soil sampling and laboratory analysis.

COSMOS-UK uses the first of these methods in which a reference soil water content is obtained from field calibration, see Franz (2012) and Zreda et al. (2012). This reference value is then used in combination with an equation relating corrected counts to soil water content (with parameters applicable for a generic silica soil matrix; see Desilets et al. (2010)), to calculate a site specific N0 calibration coefficient. The COSMOS-UK procedure also follows the procedures in Zreda et al. (2012) and Franz et al. (2013) to account for the effects of lattice and bound water (structural water associated with clay minerals in the soil) and soil organic carbon (a minor constituent of mineral soils, but the major constituent of peat soils). The calculation of the depth and radial weights used to derive an effective soil water content from the corrected neutron counts now follows the method of Schrön et al. (2017), which accounts for the dependence of the cosmic-ray neutron sensor's sensing depth and radius on soil moisture, air humidity and air pressure, superseding the previous approaches.

In 2018, following an investigation into spurious trends in the derived VWC data, a modified correction factor is applied to the counts recorded by the CRNS, to adjust for fluctuations in incoming cosmic rays. The correction is based on Jungfraujoch neutron monitor count rates recorded at the time of interest (Iref) and during the CRNS calibration period (Iref’). It is of the form

Fi=γIrefIref'-1+1

in which ɣ is a scaling factor to adjust for geomagnetic effects, differences in elevation and differences between the CRNS and the reference counter. It is derived at each site using the in-situ CRNS and Jungfraujoch neutron monitor count rates. Until 2018 the scaling factor was always set to unity and the correction factor reduces to

Fi = Iref / Iref’

Operationally, the 30 minute neutron count totals are telemetered from the sites, aggregated to hourly (by summing the two 30 min values) and corrected according to the above. Daily VWC data are generated by taking a daily average of the hourly corrected count data and applying the volumetric water content equation as outlined in Evans et al. (2016).  D86 (measurement depth) data are calculated from this daily value using the method outlined in Schrön et al. (2017), building on the footprint model of Köhli et al. (2015).

### SWE and snow days

The neutron count rates detected by the CRNS are sensitive to presence of water held in a snowpack. Because of this, the VWC calculated during days with snow cover will typically be overestimated. However, the counts can be used to estimate the snow water equivalent (SWE).

For a given day, the presence or absence of snow cover is first established using the average value of the albedo between the times of 10:00 and 14:00 GMT. A period of snow cover is deemed to begin if average albedo exceeds a threshold of 50%, and end when it falls below 35%. These threshold values were chosen with the aim of only classifying a day as a snow day if a potentially detectable amount of snow is present (e.g. at least 2-3mm of SWE).

To estimate the SWE, the count rate from before the start of the snow event is used to estimate a count rate for snow-free conditions. Then the difference between the measured daily average count rate and the snow-free estimate are used to calculate the SWE. The method is described in full as Method 1 in Wallbank et al. (2020). The estimated count rate is then used to calculate snow free VWC.

### PE

Potential evapotranspiration (PE) is derived from the sub-hourly hydrometeorological data described above (see Section 3), using the Penman-Monteith method, (FAO 56, 1998). The variables used to calculate this are: Net radiation (RN), heat flux (mean of G1 and G2), air temperature (TA), relative humidity (RH), wind speed (WS) and atmospheric pressure (PA). Daily PE are the sum of sub-hourly PE values >= 0, i.e. negative sub-hourly PE are ignored.

### GCC

Each site is equipped with a PhenoCam, see Section 7. These take images of the site, and its surroundings, both north and south. The images are then processed by a script to determine the levels of red, green and blue (RGB) in them. The amount of each colour is given as a value between 0 and 255.

The main colour of interest is green, as this can be used as an indication of seasonal variation in the surrounding vegetation. Building on this, the proportion of green, compared to red and blue levels is calculated, as this normalises the “greenness” with varying light levels from time of day and weather conditions. This is the green colour content (GCC) data.

However, this is not done for the entire image. Instead, each site has its own mask that determines which section of the image is processed. The GCC data comes from masks that pick out the fields surrounding the site. For example:

This is a PhenoCam image from Cardington:

Before being processed for RGB data, it has a mask applied to it that just extracts the surrounding, north facing field:

Or rather, only this gets processed:

The PhenoCams take up to 5 images a day. The daily GCC is taken to be the image with the highest value of that day.

Please note, this RGB dataset has not undergone any quality control.

## Quality control

The daily values are not subjected to their own quality control. Instead they are all derived from sub-hourly values which have been quality controlled. See Section 3.6 for details on this.

## Data flags

Data flags are provided in a separate file:

COSMOS-UK_<SITE ID>_HydroSoil_Daily_<YEAR SITE OPENED>-2025_Flags.csv

The data flag files mirror the structure of their counterpart data file, except that all column names have “_FLAG” appended. See section 3.7 for details.

# Infilling

Infilling is the process of providing estimates of data values where there are gaps in the time series record.  Simple infilling is carried out using interpolation methods. This is only applied to selected variables, and for gaps that are considered reasonable.

Interpolation uses data around the gap to estimate the values in between. It is very effective for small gaps, and for data that has low variability, for example temperature and pressure, but less so for variables with high variability, like precipitation. The variables chosen for infilling are listed in Table 3.1 and Table 4.1, for sub-hourly and daily data respectively.

There two different types of interpolation used:

- Linear interpolation – this creates values that make a straight line across the gap.

- Order 2 Polynomial – this uses a larger window of data around the gap to create a smoother, curved line of points.

In order to determine which to use, the processing script answers three questions:

- Is the gap less than (or equal to) 10 values?

- How many values are there around the gap?

If there is just 1 value each side, then only linear interpolation can be used. If possible, larger gaps will use more (up to 4) values each side to estimate a polynomial interpolation.

- Which interpolation method is the best?

Each combination of gap size, interpolation type, and values around the gap has an associated RMSE (Root Mean Squared Error) score that provides an estimate of which method works best.

To calculate RMSE scores for each method, 5000 artificial, independent gaps are created with the chosen gap length and then infilled with the specific interpolation. With the results the RMSE is calculated between the estimates and the real values. This is completed for each variable chosen to be infilled.

# Completeness

Figure 6.1 outlines the percentage completeness of data over the period October 2013 - December 2025. The ‘Met’ and ‘Soil’ columns combine groups of measured variables and give the overall completeness. ‘VWC’ is the completeness of the derived volumetric water content data from the CRNS.

Met = Precipitation, humidity, air temperature, atmospheric pressure, short and long wave radiation, and wind speed and direction.

Soil = Soil heat flux, soil temperature, and volumetric water content measured by TDT sensors.

Figure 6.1 – Completeness of data for each site, for combined sets of variables.

# Instrumentation

Instruments used by the COSMOS-UK network are listed in section 7.1. Note that instrumentation has changed with time and that not all instruments are installed at all sites (see section 7.2).

This information is provided for reference only and implies no endorsement of the specific instrument or supplier by UKCEH.

## Instruments used by COSMOS-UK

| Cosmic-Ray Neutron Sensor (CRNS) The sensor counts fast neutrons which can be converted to soil moisture after field calibration. Data processing accounts for variations in atmospheric pressure, humidity, and the intensity of incoming cosmic rays. The measurement volume of the sensor is many tens of meters horizontally (possibly up to 200m) although measurement is inversely related to distance from the sensor. The effective depth varies with soil moisture but is typically in the range 15-40 cm. Köhli et al. (2015) provide a discussion of the sensor footprint. Model: Hydroinnova CRS-2000/B and CRS-1000/B http://hydroinnova.com/ps_soil.html#overview |  |  |
| --- | --- | --- |
| Rain gauge Provides the amount and intensity of solid and liquid precipitation. On-board processing algorithms account for spurious changes due to temperature or wind speed. Model: OTT Pluvio2(L) https://www.ott.com/products/meteorological-sensors-26/ott-pluvio2-weighing-rain-gauge-963/ |  |  |
| Tipping bucket rain gauge (TBR) Provides data on the amount of liquid precipitation at 0.2 mm resolution. Aerodynamically shaped to reduced wind-induced under-catch. Any solid precipitation collected in the funnel will be measured as it melts. Model: EML SBS500 http://www.emltd.net/sbs.html |  |  |
| Tipping weighing rain gauge (rain[e]) Provides precipitation amount and intensity. Combines a tipping bucket with a weighing system and self-emptying collecting vessel. Model: Lambrecht rain[e] https://www.lambrecht.net/en/products/precipitation/weighing-precipitation-sensor-rain-e |  |  |
| Point soil moisture sensor Soil moisture sensors at various depths use the TDT (time domain transmissometry) technique and provide absolute volumetric water content and soil temperature. Note that the soil moisture data are not calibrated to the site-specific soil type, but rely on generic calibration information. The sampling volume is a region around the waveguide which has a total length of 30 cm. Blonquist et al. (2005) suggest that the sampling volume is no greater than 15 cm (half-length of waveguide) x 6 cm (horizontal) x 3 cm (vertical). Model: Acclima Digital TDT ACC-SEN-SDI https://acclima.com/products.html |  |  |
| Soil heat flux plate Two heat flux plates buried at a depth of 0.03 m at each site provide the soil heat flux. These plates have a self-calibrating feature to maximise measurement accuracy; the in situ calibration is performed once a day. Model: Hukseflux HFP01SC self-calibrating heat flux plate https://www.hukseflux.com/products/heat-flux-sensors/heat-flux-meters/hfp01-heat-flux-sensor |  |  |
| Soil temperature sensor The near-surface soil temperature is measured at five depths (0.02, 0.05, 0.10, 0.20 and 0.50 m) using a profile of thermocouples. Model: Hukseflux STP01 https://www.hukseflux.com/products/heat-flux-sensors/soil-temperature-sensors/stp01-soil-temperature-sensor |  |  |
| Radiometer A four-component radiometer measures the individual radiation components using upward and downward facing pyranometers (for the shortwave components) and pyrgeometers (for the longwave components). The net radiation is calculated as the sum of the incoming minus the outgoing components and is usually the dominant term in the surface energy balance. In the photo the radiometer is at the end of the horizontal support. Model: Hukseflux NR01 https://www.campbellsci.com/nr01 |  |  |
| Air temperature, pressure, and relative humidity Air temperature and relative humidity are measured by a probe situated within a naturally aspirated radiation shield; barometric pressure is also measured here for some sites. Model: Rotronic HC2A-S3 within the Gill MetPak Pro Base Station http://www.gillinstruments.com/products/anemometer/metpak-pro-weather-station.html |  |  |
| Barometric pressure sensor A barometric pressure sensor which incorporates a Barocap® silicon capacitive pressure sensor encased in a plastic shell with an intake valve for pressure equalisation. Measures barometric pressure equivalent to an elevation range from below sea level to 4.5 km. Model: Vaisala PTB110 https://www.vaisala.com/en/products/instruments-sensors-and-other-measurement-devices/instruments-industrial-measurements/ptb110 |  |  |
| Temperature and humidity sensor Humidity and air temperature are measured by a capacitive thin film HUMICAP© polymer sensor and resistive platinum sensor (Pt100) respectively. Both the humidity and temperature sensors are located at the tip of the probe protected by a removable filter. Model: Vaisala HUMICAP HMP155A https://www.vaisala.com/en/products/instruments-sensors-and-other-measurement-devices/weather-stations-and-sensors/hmp155 |  |  |
| 3D sonic anemometer The 3-axis ultrasonic wind sensor monitors wind speeds of 0-50 m/s (0-100 mph) and wind direction in three dimensions. Model: GILL WindMaster http://www.gillinstruments.com/products/anemometer/windmaster.htm |  |  |
| Integrated 2D sonic anemometer High accuracy wind speed and direction integrated with automatic weather station. Model: Gill Integrated WindSonic http://gillinstruments.com/products/anemometer/weather-station-range.htm |  |  |
| PhenoCam A pair of cameras with almost 360° field of view provides visual information about the land cover (e.g. when crops are harvested, greenness of vegetation - hence the name which is a contraction of “phenology camera”). It can also provide information on cloud cover, snow cover, surface ponding and atmospheric visibility. Model: Mobotix S14 IP camera with hemispheric lenses https://www.mobotix.com/ |  |
| Snow depth sensor Sonic rangefinder designed specifically to measure snow depth. Model: Campbell Scientific SR50A https://www.campbellsci.ca/sr50a |  |  |
| SnowFox The sensor records the intensity of downward-directed secondary cosmic-rays that penetrate the snowpack. This intensity is proportional to the mass of snow traversed by cosmic-rays and is related to soil water equivalent (SWE) through a calibration function. Model: Hydroinnova SnowFox http://www.hydroinnova.com/_downloads/snowfox_v1.pdf |  |  |
| Micrologger Consists of measurement and control electronics, communication ports. Model: Campbell Scientific CR3000 https://www.campbellsci.com/cr3000 |  |  |

## Instruments installed at COSMOS-UK sites

All the instruments within the network are outlined in section 7.1. However, there is some variation across sites. This section outlines the differences in site setups and any changes over time.

Table 7.1 describes different setup features that can be present at each COSMOS-UK site.  Table 7.2 then outlines specifically which setup features each COSMOS-UK site has.

Table 7.1 – Types of site setups, installations, and changes.

| Site setup feature | Description |
| --- | --- |
| SNOW SITE | Site with snow sensors installed: snow depth sensor and SnowFox. |
| FULL TDT ARRAY | All sites have 2 TDT sensors installed at 10 cm depth. A selection also have a further 8 TDT sensors (10 in total) installed at various depths. These are the “full TDT array” sites. |
| ANEMOMETER | Most sites have the 3D sonic anemometer, meaning they measure the UX, UY and UZ wind components, while some have the 2D sonic anemometer. The ANEMOMETER column in table 7-2 gives the date of upgrade if a site has had a 2D sonic anemometer replaced with a 3D sonic anemometer. |
| TIPPING BUCKET | Some sites have had a tipping bucket rain gauge installed. The TIPPING BUCKET column in Table 7.2 gives the date of installation. If no date is given, then this sensor is not installed. |
| RAINE | Some sites have had a rain[e] tipping weighing rain gauge installed. The RAINE column in Table 7.2 gives the date of installation. If no date is given, then this sensor is not installed. |
| AWS and SWS | Air temperature, pressure and relative humidity are either measured by the combination of Rotronic + GILL MetPak sensors, or the Vaisala sensors. Many sites with the combined GILL MetPak setup have been converted to the Vaisala sensors. Table 7.2 states if the site currently has the GILL MetPak or Vaisala, and the date if it has been swapped to Vaisala. |

Table 7.2 – Details of site setup.

| Site name | Snow site | Full TDT array | Anemometer | Tipping bucket | rain[e] | GILL MetPak | Vaisala |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alice Holt4 |  |  | 2D |  |  | x |  |
| Balruddery |  |  | 3D |  |  |  | 29/06/2022 |
| Bickley Hall |  |  | 3D |  | 14/02/2022 |  | 14/02/2022 |
| Bunny Park |  |  | 3D |  | 25/02/2022 |  | 26/04/2022 |
| Cardington |  |  | 3D |  | 14/07/2022 |  | 15/03/2022 |
| Chimney Meadows |  | x | 12/02/2020 | 12/02/2020 | 07/09/2022 |  | 12/02/2020 |
| Chobham Common |  | x | 3D |  | 09/08/2022 |  | 09/08/2022 |
| Cochno | x |  | 3D |  |  |  | x |
| Cockle Park |  | x | 3D |  |  |  | 14/06/2022 |
| Crichton |  |  | 3D |  |  |  | 15/06/2022 |
| Cwm Garw | x |  | 3D |  |  |  | x |
| Easter Bush | x |  | 3D |  |  |  | 17/08/2022 |
| Elmsett |  | x | 3D |  |  |  | x |
| Euston |  | x | 3D |  |  |  | x |
| Fincham |  | x | 3D |  | 24/02/2022 |  | x |
| Fivemiletown |  | x | 3D |  |  |  | x |
| Gisburn Forest | x |  | 3D |  |  |  | 05/05/2022 |
| Glensaugh | x |  | 3D |  |  |  | 30/06/2022 |
| Glenwherry |  | x | 3D |  |  |  | x |
| Hadlow |  | x | 3D |  |  |  | x |
| Hartwood Home |  |  | 3D |  |  |  | 16/08/2022 |
| Harwood Forest4 |  |  | 2D |  |  | x |  |
| Henfaes Farm |  |  | 3D |  |  |  | 04/08/2022 |
| Heytesbury |  | x | 3D |  |  |  | x |
| Hillsborough |  | x | 3D |  |  |  | x |
| Hollin Hill |  |  | 3D |  | 27/06/2022 |  | 03/05/2022 |
| Holme Lacy |  | x | 3D |  | 24/05/2022 |  | x |
| The Lizard |  |  | 3D |  |  |  | 21/02/2022 |
| Loddington |  | x | 3D |  | 25/02/2022 |  | x |
| Lullington Heath |  |  | 3D |  | 23/08/2022 |  | 23/08/2022 |
| Moor House | x |  | 3D |  |  |  | 16/06/2022 |
| Morley |  | x | 3D |  |  |  | 18/05/2022 |
| Moreton Morrell |  |  | 3D |  | 08/09/2022 |  | x |
| North Wyke |  |  | 3D |  |  |  | 11/05/2022 |
| Plynlimon | x |  | 3D |  |  |  | 03/08/2022 |
| Porton Down |  |  | 3D |  |  |  | 08/11/2022 |
| Redhill |  |  | 3D |  | 23/08/2023 |  | 24/08/2022 |
| Redmere |  |  | 3D |  |  | x |  |
| Riseholme |  | x | 3D |  |  |  | x |
| Rothamsted |  |  | 3D |  | 10/08/2022 |  | 10/08/2022 |
| Sheepdrove |  | x | 11/02/2020 | 11/02/2020 | 13/09/2022 |  | 11/02/2020 |
| Sourhope | x |  | 3D |  |  |  | 18/08/2022 |
| Spen Farm |  | x | 3D |  |  |  | x |
| Stiperstones |  |  | 3D |  |  |  | 25/05/2022 |
| Stoughton |  |  | 3D |  | 09/03/2022 |  | 09/03/2022 |
| Sydling |  | x | 3D |  | 07/06/2022 |  | x |
| Tadham Moor |  |  | 3D |  |  |  | 12/05/2022 |
| Waddesdon |  | x | 13/02/2020 | 13/02/2020 | 10/07/2023 |  | 13/02/2020 |
| Wimpole |  | x | 3D |  |  |  | x |
| Writtle |  | x | 3D |  | 19/07/2023 |  | x |
| Wytham Woods |  |  | 2D |  |  | x |  |

### rain[E] height adjustment

The recommended rim height for a rain gauge in the UK is 0.305 meters above short grass; high enough to prevent splashing from ground surface, but not too high to be prone to wind undercatch effects (Muchan & Dixon 2019). However, this requires high levels of maintenance to ensure that the surrounding area of vegetation is kept sufficiently short to reduce the chance of blockages. We have found that, due to the extent or our network, we are unable to maintain this level of ground maintenance at all COSMOS-UK sites. As a result, we have seen loss of data from rain[e] gauges due to blockages from overgrown grass and other surrounding vegetation (particularly in summer months). Therefore, in 2023 we made the decision to raise rain[e] gauges to a rim height of 1.0 meters to reduce the chance of blockages. Although this means the rain[e] gauges may be more prone to wind undercatch issues, it provides a more reliable and consistent data stream.  This also means that the rain[e] gauges are at the same height as the Pluvio rain gauges. Table 7.3 lists the dates that the rain[e] gauges were raised at each site.

Table 7.3 - Dates of rain[e] height adjustments

| Site name | Date of rain[e] height adjustment | Comment |
| --- | --- | --- |
| Bickley Hall | 25/05/2023 |  |
| Bunny Park | 16/04/2024 |  |
| Cardington | 10/08/2023 |  |
| Chimney Meadows | 28/08/2024 |  |
| Chobham Common | 08/08/2023 |  |
| Fincham | 18/05/2023 |  |
| Hollin Hill | 25/04/2023 |  |
| Holme Lacy | 23/05/2023 |  |
| Loddington | 20/04/2023 |  |
| Lullington Heath | 22/08/2023 |  |
| Moreton Morrell | 07/09/2023 |  |
| Redhill | 23/08/2023 | First installed at 1.0 m height |
| Rothamsted | 09/08/2023 |  |
| Sheepdrove | 07/03/2023 |  |
| Stoughton | 19/04/2023 |  |
| Sydling | 31/05/2023 |  |
| Waddesdon | 10/07/2023 | First installed at 1.0 m height |
| Writtle | 19/07/2023 | First installed at 1.0 m height |

# Changes to data

Data processes are under constant review. Where improvements are made, the full time series is reprocessed. Table 8.1 lists these major changes.

Table 8.1 - History of major changes to data.

| Date | Change | Details |
| --- | --- | --- |
| Jun 2026 | VWC calibration values updated | New method for calculating CRNS depth / radial weights based on Schrön et al. (2017). These changes update REF_SOIL_MOISTURE, REF_CTS_MOD_CORR, N0_MOD, N_MIN and N_MAX, which in turn update VWC values. |
| Jun 2026 | D86 calculation updated | New method for calculating D86 effective depth from Schrön et al. (2017). This accounts for soil moisture, air pressure, and air humidity in a more complete way. |
| Jun 2026 | REF_Q0 updated | The absolute humidity on day of calibration (REF_Q0) was using Q calculated on the logger. We also calculate Q in the processing code and use this value as our ‘truth’ for any further applications. This change aligns this and switches to using the processor Q over the logger Q. |
| Jun 2026 | Snow depth QC | The snow depth from the SR50A sensor now has additional quality controlled using the signal quality strength. If too low, values are removed. |
| Jul 2025 | Added variables for infilling with interpolation. | Variables added: E_SAT_MEAN_30M HS _30M RHO_A_MEAN _30M TAU _30M TS _30M U_STAR _30M STP_TSOIL#_30M TDT#_TSOIL_30M TDT#_VWC_30M TNR01C_30M |
| May 2025 | Quality control test “zero” removed from LWIN, LWOUT, SWIN, SWOUT | The quality control “range” test for SWIN and SWOUT has a minimum value of -10, to account for calibration uncertainty – therefore zero values are valid. The quality control “range” test for LWIN and LWOUT has a minimum value of 150, therefore the “zero” test is redundant. |
| May 2025 | Temperature spike test added | Multiple spikes in temperature data spotted that can be automatically removed by the spike test. |
| Mar 2024 | Daily PE data fix | Fixed issue with Daily PE data values, which were not ignoring negative sub-hourly PE data within the daily sum. |
| Mar 2024 | Site altitude metadata updated | Site altitude values reviewed and updated where necessary based on latest available elevation mapping products – see Section 2.3.1. |
| Mar 2024 | VWC calibration values updated | New Gamma values calculated up to 01/12/2023. Bulk density recalculated based on a simple average of soil samples. Variables affected by bulk density changes are also recalculated: N0_MOD, N_MIN and N_MAX. |
| Feb 2023 | COSMOS VWC reprocessed at Hollin Hill | The pressure sensor installed between May 2021 and April 2022 was found to be incorrectly calibrated leading to low VWC values. A correction factor has been applied. |
| Feb 2023 | All 00:30 and 01:00 heat flux values (G1 and G2) now being removed. | Due to residual heat from midnight calibrations, small spike in the data were regularly seen. The gaps are subsequently infilled using interpolation. |
| Feb 2022 | Infilling applied | Air temperature, relative humidity, pressure, soil heat fluxes, radiation (short/long wave in/out), wind speed and direction, and daily VWC now infilled with interpolation for gaps smaller than 10 data points. Either linear or polynomial interpolation is used, depending on which is best suited to the condition of the gap (e.g. available data around the gap). |
| Jun 2021 | VWC calibration values updated | New Gamma values calculated up to 01/04/2021, along with calibration values affected; REF_CTS_MOD_CORR, N0_MOD, N_MIN and N_MAX. Bulk density recalculated now based on a weighted average of soil samples. |
| Jan 2021 | D86_75m updated | Correction to the calculation of effective depth of CRNS. |
| Jul 2020 | VWC calibration values updated | As the amount of available data grows, calibration values required for calculating VWC can be reviewed and in some cases updated. |
| Jul 2020 | CRNS counts infilled from temporarily installed secondary tube | Where a site has had two CRNSs, any missing data from primary tube now infilled from the secondary, after correction factor applied. |
| Jul 2020 | NMDB data infilled | Missing NMDB data from Jungfraujoch now infilled from other sites, after correction factor applied. |
| Jul 2020 | 30 min precipitation updated | The way 30 minute precipitation was calculated changed. |
| Apr 2020 | PE updated | Revision of implemented method plus a switch to calculate 30 min values and aggregate these to hourly and daily. Previously just daily. |
| Dec 2019 | SWE added | Snow days calculated from albedo, and method added to estimate counts and snow water equivalence (SWE) on these days. Wallbank et al. (2020). |
| Dec 2019 | Daily albedo added | Calculated from SWIN and SWOUT for daylight hours. |

# Author contributions

S. Stanley is the primary author of this document and first point of contact for these data.

H.M. Cooper, M. Fry, G. Jiménez-Arranz, R. Smith, S. Stanley O. Swain and H.C. Ward have managed the data; V. Antoniou, J.R. Blake, H.M. Cooper, M. Fry have compiled site metadata; A. Askquith-Ellis, J.R. Blake, M. Brooks, V. Cirstet, M. Clarke, N. Cowan, A. Cumming, J.G. Evans, D. Harvey, T. Howson, R. Morrison, F. O'Callaghan, D. Rylett, P.M. Scarlett, P. St Quintin, A. Retter, S. Teagle, J.L. Thornton, E.J. Trill, P. Vincent, H.C. Ward and J.B. Winterbourn have contributed to the system design, instrument setup, calibration and technical maintenance of the network; P. Farrand, W.D. Lord and A.C. Warwick have had responsibility for field engineering; L.A. Ball, J.R. Blake, J.G. Evans and R. Morrison were responsible for site selection and negotiation; L.A. Ball, E.S. Bennett, D.B. Boorman, J.G. Evans and H. Houghton-Carr, Y. Keen have project managed COSMOS-UK; J.R. Blake, D.B. Boorman, H.M. Cooper, J.G. Evans, D. Khamis, R. Morrison, G. V. Nash, M. Szczykulska and H.C. Ward contributed to derived data processing algorithms, data presentation, analysis and interpretation.

# References

Baatz, R., H. R. Bogena, H.-J. Hendricks Franssen, J. A. Huisman, W. Qu, C. Montzka, and H. Vereecken (2014), Calibration of a catchment scale cosmic-ray probe network: A comparison of three parameterization methods, Journal of Hydrology, 516, p. 231–244. https://doi.org/10.1016/j.jhydrol.2014.02.026

Blonquist, J.M., Jones, S.B., Robinson, D.A. (2005), A time domain transmission sensor with TDR performance characteristics. Journal of Hydrology, 314, p. 235-245. https://doi.org/10.1016/j.jhydrol.2005.04.005

Bogena, H. R., Huisman, J. A., Güntner, A., Hübner, C., Kusche, J., Jonard, F., Vey, S. and Vereecken, H.  (2015), Emerging methods for noninvasive sensing of soil moisture dynamics from field to catchment scale: a review, WIREs Water, 2(6), p. 635–647. https://doi.org/10.1002/wat2.1097

Desilets D, Zreda M, Ferré TPA. (2010), Nature’s neutron probe: land surface hydrology at an elusive scale with cosmic rays. Water Resources Research, 46(11): W11505. https://doi.org/10.1029/2009WR008726

Evans J. G., Ward H. C., Blake J. R., Hewitt E. J., Morrison R., Fry M., Ball L. A., Doughty L. C., Libre J. W., Hitt O. E., Rylett D., Ellis R. J., Warwick A. C., Brooks M., Parkes M. A., Wright G. M. H., Singer A. C., Boorman D. B. and Jenkins A. (2016),  Soil water content in southern England derived from a cosmic-ray soil moisture observing system – COSMOS-UK, Hydrological Processes, 30(26), p. 4987–4999. https://doi.org/10.1002/hyp.10929

FAO (1998). Crop evapotranspiration: Guidelines for computing crop water requirements. FAO Irrigation and drainage paper 56.

Franz TE. (2012), Installation and calibration of the cosmic-ray solar moisture probe. pp: 12.

Franz TE, Zreda M, Rosolem R, Ferre TPA. (2013), A universal calibration function for determination of soil moisture with cosmic-ray neutrons, Hydrology and Earth System Sciences, 17(2), p. 453–460. https://doi.org/10.5194/hess-17-453-2013

Köhli, M., M. Schron, M. Zreda, U. Schmidt, P. Dietrich and S. Zacharias (2015), Footprint characteristics revised for field-scale soil moisture monitoring with cosmic-ray neutrons, Water Resources Research, 51(7), p. 5772–5790. https://doi.org/10.1002/2015WR017169

Muchan, K. and Dixon, H (2019), Insights into rainfall undercatch for differing raingauge rim heights, Hydrology Research, 50(6), p. 1564-1576. https://doi.org/10.2166/nh.2019.024

Schrön, M., Köhli, M., Scheiffele, L., Iwema, J., Bogena, H. R., Lv, L., Martini, E., Baroni, G., Rosolem, R., Weimar, J., Mai, J., Cuntz, M., Rebmann, C., Oswald, S. E., Dietrich, P., Schmidt, U., and Zacharias, S. (2017), Improving calibration and validation of cosmic-ray neutron sensors in the light of spatial sensitivity, Hydrology and Earth System Sciences, 21(10), p. 5009–5030. https://doi.org/10.5194/hess-21-5009-2017

Wallbank J. R., Cole S. J., Moore R. J., Anderson S. R., Mellor E. J. (2020), Estimating snow water equivalent using cosmic-ray neutron sensors from the COSMOS-UK network, Hydrological Processes, 35(5), e14048. https://doi.org/10.1002/hyp.14048

Zreda M, Shuttleworth W, Zeng X, Zweck C, Desilets D, Franz T, Rosolem R. (2012), COSMOS: the COsmic-ray Soil Moisture Observing System, Hydrology and Earth System Sciences, 16, p. 4079–4099. https://doi.org/10.5194/hess-16-4079-2012

The soil sampling procedure is described in Evans et al. (2016).

NEXTMap Britain digital terrain model by Intermap. © 2009 Intermap Technologies Inc. All rights reserved.

Net radiation (RN) and absolute humidity (Q) are not run through any QC tests. This is because they are derived from values that have already been QC’d.

These sites were installed at pre-existing forest flux observation towers. Here meteorological sensors are located at the top of the tower above the forest canopy (whilst equipment at other sites is installed on a ~2 m station). The digital weighing rain gauge (Pluvio) receives rainfall collected at the top of the tower via a funnel and tube, and therefore does not accurately measure precipitation intensity. Precipitation accumulation data measured by the Pluvio at these sites are corrected for the smaller aperture area of the funnel.