Supporting information for data resource “Biologging data of captive red deer, Prague, Czechia, June-November 2023”

This dataset was generated as part of a project investigating sleep in red deer, to aid in the creation of a classification model to detect sleep in red deer from collar accelerometers, and to test arousal thresholds in sleeping deer.

•	Collection/generation methods

Data results from collar-mounted Daily Diary units (“DD”, Wildbytes Ltd, Swansea University) deployed on adult male and female red deer Cervus elaphus. Deployment 1 began on the 09/06/23, with data from 7 adult males. Deployment 2 began on the 03/09/23, with data from 6 adult males. Deployment 3 began on the 13/11/23 with data from 3 adult females. Data collection took place at the VUZV’s red deer facility in Uhrineves, Prague.  DD were recording acceleromter and magnetometer data at 20Hz. Collars were deployed and recovered by EM and BE, assisted by VUZV staff. Data was downloaded from DD and stored by EM.

•	Nature and Units of recorded values

Accelerometer records in m/s2, magnetometer records in gauss, thermometer records in degrees Celsius.

•	Quality control

Data has been screened for non-functional units and device failure; these units are excluded from the dataset as they did not record. This is raw data directly from the device, from which processed data suitable for analysis is to be derived.

•	Details of data structure

Data is provided as .txt files, as the raw acceleration, magnetometry, and temperature data exported from the raw data using the DDMT software. Raw and uncorrected magnetic heading is also provided. Data is of the following format:

| Data | Description |
| --- | --- |
| Time | UTC+1 time as hh:mm:ss.ss |
| Date | Date as dd/mm/yy |
| Acceleration X | Raw acceleration on X-axis (in m/s2) |
| Acceleration Y | Raw acceleration on Y-axis (in in m/s2) |
| Acceleration Z | Raw acceleration on Z-axis (in in m/s2) |
| Magnetometer X | Raw gauss on X axis |
| Magnetometer Y | Raw gauss on Y axis |
| Magnetometer Z | Raw gauss on Z axis |
| Temperature | Raw temperature (in C) |
| Mag heading | Raw magnetic heading (in degrees, 0-360) |

16 folders are included in total, with one folder per deployed collar, per deployment period, and 3,232 files in total. Naming conventions are given below, of the common format “deployment identifier”_”biologger”_”Collar colour/number”_”Animal”:

| Name | Description |
| --- | --- |
| Deployment Identifier | Given as month (June, August, November) |
| Biologger | Biologger unit identifier is the unique reference for each device, independent of animal |
| Collar colour/number | Given to aid individual ID from videos, where there is no collar number only colour is given |
| Animal | Animal name is given where possible to also aid identification (females cannot be identified without collar and are given NA for name) |

•	Sampling regime

Devices recorded at 20Hz.

•	Calibration steps

Calibration of units was conducted in the field, as per standard procedures. Accelerometers were stimulated along X, Z, Y axes in that order, followed by magnetic spheroid calibration suited for DDMT in-built magnetic correction algorithm where devices are rotated around all axes across 360-degrees. Calibration dates/times are provided in supporting documentation. These calibrations also serve to timestamp the data to UTC+1 time. In the provided data, raw magnetometer data has undergone correction.

Calibration dates and times are provided in the associated .csv file “calibration metadata”. Times are in UTC+1 time, column identifiers are given below:

cal_date: Date device was calibrated, all calibrations done on-site
dd_unit: DD unit identifying number
collar: Collar colour and/or number
cal_start: Time calibration started, used to set the time and date in DD data, from the first shake
cal_end: Time calibration ended, used to check you have the calibration data correctly
calibration: Calibrations follow standard, acceleration on x then z then y, followed by magnetometer offset.