# Working Title

Calculated air pollutant emissions from domestic solid fuel burning in Scotland 2023

# Summary

Gridded emissions of key air pollutants from domestic burning of solid fuels (wood, coal or manufactured solid fuel (MSF)) in Scotland. These emissions were generated from newly available fuel use and emission factor data, spatially distributed using population, land use and household Energy Performance Certificate (EPC) data. They were used in EMEP4UK modelling of air pollutant concentrations in Scotland caused by domestic solid fuel burning (Hood et al., 2026).

# File format

The data files are in GeoTIFF format. The data is on a regular grid at 1 km x 1 km resolution defined in British National Grid (OSGB, epsg code 27700).

# File names and/or naming convention

There are 18 files with the following name convention: SCOT_emis_t_POLL_FUEL.tif

Where POLL is the emitted pollutant, one of the following:

- BC – black carbon (part of fine particulates)

- CO – carbon monoxide

- NOx – nitrogen oxides (sum of NO and NO2)

- PM25 – particulates with aerodynamic diameter < 2.5 µm (PM2.5)

- SO2 – sulphur dioxide

- VOC – volatile organic compounds (excluding methane)

and FUEL is the solid fuel type, one of the following:

- coal - including house coal, petroleum coke, anthracite, lignite and peat

- msf – manufactured solid fuel (MSF)

- wood – including wood logs, briquettes and coffee logs

# Nature and units of recorded values

Each .tif file contains a single band representing gridded emission rates of the pollutant given in the file name in units of tonnes per year.

# Spatial coverage

The data file extents cover the whole of the UK and Ireland, but emissions are only non-zero in cells containing Scottish land mass.

The data is on a regular grid at 1 km x 1 km resolution defined in British National Grid (OSGB, epsg code 27700). The bounding coordinates are:

SW corner: x 0, y 7000

NE corner: x 656000, y 1217000

# Temporal coverage and resolution

The emissions represent annual total emissions in 2023.

The emissions were calculated based on fuel use survey data from 2022-2023 (Defra, 2024a). The emissions were spatially distributed based on gridded population data from the 2021/2022 Census (Carnell et al. 2025) and Energy Performance Certificate (EPC) data from Q1 2015 to Q4 2024 (published by the Scottish Government on a quarterly basis, obtained in early 2025, Scottish Government (2025)).

# Models

The data processing chain used to generate the emission maps is outlined in Figure 1, including the use of each input dataset. EPC and Scottish Urban-Rural classification data were Excel files. Postcode, Output Area and SCA maps were polygon data stored as shapefiles (.shp). Population data and final output emission maps were rasters, stored as .tiff files.

Figure 1: Data processing flow chart for generation of emission maps

## Input data

Fuel use data for indoor solid fuel burning was taken from the Defra ‘Domestic Burning Practices in the UK – Burning in UK homes and gardens’ survey (Defra 2024a). This was based on interviews carried out from July 2022 to June 2023 (Core Activity Survey 2022), with aggregated data published in 2024.

Total fuel volumes for the four nations of the UK were also calculated from the survey data and published in 2024 (Defra 2024b). This dataset also included proportions of wood fuels divided by: moisture categories; urban or rural; within or outside smoke control areas; appliance type (open fire or stove).

An earlier fuel use survey from 2018-2019 was used to distribute coal use between different residential groups (Kantar, 2020) due to limitations in the Defra 2024a data for coal fuels.

Emission factors data for pollutants CO, NOx, VOC, PM2.5, SO2, BC and condensable PM2.5 were taken from the Emission Factors for Domestic Solid Fuels (EFDSF) study (Allan et al., 2025). This study tested the effects of both differing fuels and burning appliances. The emission factors from the EFDSF are defined per energy contained in the fuel and were converted for the current study into emission per mass of fuel using the calorific value of the fuel (energy of fuel per unit mass).

Gridded population data for the UK based on the 2021/2022 Census and Land Cover Map 2021 (Carnell et al. 2025) were used in the spatial distribution of solid fuel emissions across Scotland.

Spatial definitions of urban or rural areas were based on the Scottish 2022 census 8-fold Urban Rural classification for each Census Output Area, with classification values of 5 and below considered urban.

A Spatial definition of Smoke control areas declared by local authorities throughout Scotland was obtained from the Scottish Government (Spatial Hub, 2025).

Energy Performance Certificate (EPC) data geolocated by postcode and UPRN were obtained from the Scottish Government. The data used in the study were downloaded in early 2025 and incorporates EPC generated between Q1 2015 and Q4 2024 (Scottish Government, 2025).

## Output data

Emission maps (.tif files generated using process and datasets outlined in Figure 1), were combined with NAEI emission data on domestic combustion from non-solid fuels (base year 2021, consistent with 2023 NAEI release; data provided by Ricardo via personal communication) and converted into EMEP input files (NetCDF format) for EMEP model runs.

# Collection/Generation/Transformation methods

All data were collected and generated digitally using Windows PCs. Data were stored in secure project folders within UKCEH-managed storage. Initial data processing was carried out using reproducible R scripts on PCs, which aggregated and processed a combination of spatial data files (shapefiles and raster’s) and non-spatial data files (excel spreadsheets and csv’s) into a number of rasters (.tif files). Full details of the methods and workflows are provided in the project report and workflow of the EMEP file generation is outlines in Figure 1. Quality assurance and quality control (QA/QC) checks were applied at multiple stages throughout data collection and processing to ensure the correctness and integrity of the data.

# Quality control

The fuel use distribution and emissions estimates were compared with previous estimates, for example from the Digest of United Kingdom Energy Statistics (DUKES) for 2022 (released 2023) and the UK National Atmospheric Emissions Inventory (NAEI).

The spatial distribution of total solid fuel emissions was visualised in maps of the whole of Scotland and details of specific areas of interest.

The emission data was used as input to the EMEP4UK model (Hood et al., 2026) and the model concentration outputs were compared with measurements (Di Marco et al., 2026) which helped to assess the accuracy of the emissions estimates.

# Miscellaneous

Further details about the processing methods for the emissions data are available in the final project report, Nemitz et al. (2025).

# References

Carnell, E., Tomlinson, S. J. and Reis, S.: UK gridded population at 1 km resolution for 2021 based on Census 2021/2022 and Land Cover Map 2021, NERC EDS Environmental Information Data Centre, https://doi.org/10.5285/7beefde9-c520-4ddf-897a-0167e8918595, 2025.

Defra: Domestic Burning Practices in the UK - Burning in UK homes and gardens, Defra., 2024a.

Defra: Quantification of the 2022-2023 Defra Domestic Burning Survey, Defra., 2024b.

Di Marco, C., Nemitz, E., Cash, J., Mullinger, N. and Hood, C.: Measured air pollutant concentrations from solid fuel burning in Scotland, 2023-2024, NERC EDS Environmental Information Data Centre, https://doi.org/10.5285/e9f39fd7-d181-46d7-a8ef-5cc0c011b932, 2026.

Digest of United Kingdom Energy Statistics (DUKES), 2023. https://www.gov.uk/government/statistics/digest-of-uk-energy-statistics-dukes-2023

Hood, C., Nemitz, E., Vieno, M., Liska, T., Scheffler, J., Beck, R., Carnell, E., Tomlinson, S. and Pearson, C.: European Monitoring and Evaluation Program Model for the UK (EMEP4UK) annual air pollutant concentrations from domestic solid fuel burning in Scotland, 2023, NERC EDS Environmental Information Data Centre, https://doi.org/10.5285/9123cb2b-739d-43cb-b00a-9678a41fc24a, 2026.

Kantar: Burning in UK Homes and Gardens - Research Report, Defra., 2020.

Nemitz, E.; Di Marco, C. F.; Hood, C.; Vieno, M.; Pearson, C.; Carnell, E.; Cash, J.; Mullinger, N.; Tomlinson, S. 2025 The contribution of indoor domestic solid fuel burning to Scotland’s air pollution. Final report to Scottish Government. Edinburgh, UK Centre for Ecology & Hydrology, 89pp. https://nora.nerc.ac.uk/id/eprint/540833/

Output area 2022 to Urban Rural Classification 2022, part of the 2022 Census Geography Products from National Records of Scotland. Available from https://www.nrscotland.gov.uk/publications/2022-census-geography-products/ (accessed 6th January 2026).

Scottish Government (2025) Domestic Energy Performance Certificates – Dataset to Q4 2024. Available with quarterly rolling updates at http://statistics.gov.scot/data/domestic-energy-performance-certificates (last accessed 15th January 2026)

Spatial Hub (2025) Smoke Control Areas – Scotland. Available at https://spatialdata.gov.scot/geonetwork/srv/api/records/7e138b52-e8c6-41ee-99d1-9882b5e3025d (last accessed 15th January 2026)