# Working Title

Measured air pollutant concentrations from solid fuel burning in Scotland 2023-2024

# Summary

This dataset includes measurements of air pollutants related to solid fuel burning carried out during two winter campaigns in Scotland. Continuous measurements at two routine monitoring sites in Scotland were also processed and analysed to identify solid fuel burning contributions. These measurements and processing were carried out as part of a project for the Scottish Government, aiming to understand the impacts of solid fuel burning on air pollution in Scotland.

# File format

The file format for each measurement site is CSV

# File names and/or naming convention

There are:

- 2 files with the following name convention: YYYY_StationName.csv containing campaign measurements; and

- 2 files with the name convention StationName.csv containing processed routine measurements

Where YYYY refers to the year the measurements were recorded and StationName is the name of the station or field site at which the measurements were recorded.

# Nature and units of recorded values

Missing values are indicated by 'N/A’

PM2.5 – particulate matter with aerodynamic diameter below 2.5 µm

PM1 – particulate matter with aerodynamic diameter below 1 µm

Table 1. Description of variables in 2023_Edinburgh.csv and in 2024_Charlestown.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| date | Time-stamp of the end of each measurement period | Format ‘dd/mm/yyyy HH:MM’ |
| NO3 | Nitrate in PM1 | µg m-3 |
| SO4 | Sulphate in PM1 | µg m-3 |
| NH4 | Ammonium in PM1 | µg m-3 |
| Cl | Chloride in PM1 | µg m-3 |
| Org | Organic matter in PM1 | µg m-3 |
| eBC | equivalent black carbon | µg m-3 |
| eBCwb | equivalent black carbon from wood burning with standard Absorption Ångström Exponent (AAE). Refer to Fig 4.15 in final report Nemitz et al. (2025) | µg m-3 |
| eBCtr | equivalent black carbon from traffic with standard AAE. Refer to Fig 4.15 in final report Nemitz et al. (2025) | µg m-3 |
| eBCwb_ss | equivalent black carbon from wood burning with site-specific AAE. Refer to Fig 4.15 in final report Nemitz et al. (2025) | µg m-3 |
| eBCtr_ss | equivalent black carbon from traffic with site-specific AAE. Refer to Fig 4.15 in final report Nemitz et al. (2025) | µg m-3 |
| eBCwb_mid | equivalent black carbon from wood burning with a mid-value AAE (between the standard and site-specific values). Refer to Fig 4.15 in final report Nemitz et al. (2025) | µg m-3 |
| eBCtr_mid | equivalent black carbon from traffic with a mid-value AAE (between the standard and site-specific values). Refer to Fig 4.15 in final Nemitz et al. (2025) | µg m-3 |
| cwood | PM2.5 concentration due to wood burning. Calculated with standard AAE. Refer to Fig 4.15 in final report Nemitz et al. (2025) | µg m-3 |
| cwood_ss | PM2.5 concentration due to wood burning calculated with site-specific AAE. Refer to Fig 4.15 in final report Nemitz et al. (2025) | µg m-3 |
| cwood_mid | PM2.5 concentration due to wood burning. Calculated with a mid-value AAE. Refer to Fig 4.15 in final report Nemitz et al. (2025) | µg m-3 |
| HOA | Hydrocarbon-like organic aerosol | µg m-3 |
| BBOA | Solid fuel organic aerosol | µg m-3 |
| COA | Cooking organic aerosol | µg m-3 |
| MO-OOA | More oxidised Oxygenated organic aerosol | µg m-3 |
| LO-OOA | Less oxidised Oxygenated organic aerosol | µg m-3 |
| PM1 | Mass concentration in PM1 size (aerodynamic diameter < 1 µm) | µg m-3 |
| PM2.5 | Mass concentration in PM2.5 size (aerodynamic diameter < 2.5 µm) | µg m-3 |
| NOx | Nitrogen oxides (sum of NO2 and NO) as NO2 | µg m-3 |
| CO | Carbon monoxide | mg m-3 |
| CO2 | Carbon dioxide | ppm |
| CH4 | Methane | ppb |
| O3 | Ozone | µg m-3 |
| T | Temperature | °C |
| wspeed | wind speed | m s-1 |
| wdirection | wind direction (degrees from north) | Deg ° |

Table 2. Description of variables in GlasgowTownhead.csv and in AuchencorthMoss.csv

| Variable/Column header | Description | Units |
| --- | --- | --- |
| date_2020_2024 | Time stamp for full timeseries for “cwood”, from 01/01/2020 to 31/12/2024, at the beginning of each measurement period | Format ‘dd/mm/yyyy HH:MM’ |
| cwood | PM2.5 concentration due to wood burning. | µg m-3 |
| Date_2023 | Time stamp for “eBCwb”, range from 01/01/2023 to 31/12/2023, at the beginning of each measurement period | Format ‘dd/mm/yyyy HH:MM’ |
| eBCwb | equivalent black carbon from wood burning with standard AAE | µg m-3 |

# Spatial coverage

Four measurement sites are included, with latitude-longitude coordinates (WGS84 datum) as follows:

- Infirmary Street, Edinburgh lat/lon 55.948558°, -3.184447°

- Charlestown lat/lon 56.037428°, -3.49945°

- Glasgow Townhead lat/lon 55.865782°, -4.243631°

- Auchencorth Moss lat/lon 55.792160°, -3.242900°

# Temporal coverage and resolution
2023_Edinburgh.csv includes data collected between 13/01/2023 09:00:00 and 06/03/2023 08:00:00

2024_Charlestown.csv includes data collected between 05/01/2024 09:00:00 and 25/03/2024 11:00:00

AuchencorthMoss.csv contains data covering the period between 01/01/2020 00:00 and 31/12/2024 00:00

GlasgowTownhead.csv contains data covering the period between 01/01/2020 00:00 and 31/12/2024 00:00

All data have an hourly resolution.

# Fieldwork and/or laboratory instrumentation

The aerosol components (NO3, SO4, Cl, NH4, Org) were measured using a High-Resolution Time of Flight Aerosol Mass Spectrometer (HR-TOF-AMS) (https://aerodyne.com/).

The equivalent Black carbon (eBC) was measured using an AE33 7-wavelength aethalometer (https://aerosolmageesci.com/).

PM1 and PM2.5 mass concentrations were measured using an optical particle counter (Dustdecoder 11-D, GRIMM Aerosol Technik).

CO2, CH4, O3 were measured using a Multi-Gas-Analyser (MGA, MIRO analytical AG).

Meteorological parameters were measured with a metstation (Vaisala Weather Transmitter WXT530).

The measurement height was 3m above ground level.

# Calibration steps and values

The HR-TOF-AMS was calibrated fortnightly following the guidelines of the manufacturer. Blanks were checked using an HEPA filter at the inlet.

# Collection/Generation/Transformation methods

The Aerosol Mass Spectrometer (AMS) data were analysed using a tool created by the University of Colorado in collaboration with Aerodyne Research Ltd, based on the software Igor Pro (Wavemetrics). Hourly data were then converted into an Excel spreadsheet and csv file.

The organic source components (HOA, BBOA, COA, MO-OOA, LO-OOA) were obtained from positive matrix factorisation (PMF) analysis (Paatero and Tapper, 1994). A tool based on Igor pro “PMF evaluation tool -PET v 3.08” was used here to determine the source apportionment (Ulbrich et al., 2009)

The black carbon source apportionment (eBCwb, eBCtr,  eBCwb,_ss eBCtr_ss,  eBCwb_mid,  eBCtr_mid) and the PM2.5 concentration due to solid fuel burning (Cwood,  Cwood_ss,  Cwood_mid ) were obtained by applying the aethalometer model (Font et al., 2022; Sandradewi et al., 2008)  to aethalometer measurements using different values of the Absorption Angstrom exponent (AAE) (Savadkoohi et al., 2024). The aethalometer data used as input to the Glasgow Townhead and Auchencorth Moss site analyses were downloaded from UK-Air (https://uk-air.defra.gov.uk/) for these monitoring stations, which are part of the UK Black Carbon Network.

# Quality control

- Missing data are due to calibration events, power cuts or issues with the operation of the instruments

- QA/QC analysis for AMS data followed the reanalysis steps detailed in the AMS-ACSM users procedures developed by Aerodyne Research Ltd, University of Colorado and Tofwerk

# Acknowledgement

UK Black Carbon network measurement data for Glasgow Townhead and Auchencorth Moss were obtained from uk-air.defra.gov.uk. © Crown copyright 2021 Defra via uk-air.defra.gov.uk, licensed under the Open Government Licence.

# References

Font, A., Ciupek, K., Butterfield, D. and Fuller, G. W.: Long-term trends in particulate matter from wood burning in the United Kingdom: Dependence on weather and social factors., Environ. Pollut., 314, 120105, https://doi.org/10.1016/j.envpol.2022.120105, 2022.

Nemitz, E.; Di Marco, C. F.; Hood, C.; Vieno, M.; Pearson, C.; Carnell, E.; Cash, J.; Mullinger, N.; Tomlinson, S. 2025 The contribution of indoor domestic solid fuel burning to Scotland’s air pollution. Final report to Scottish Government. Edinburgh, UK Centre for Ecology & Hydrology, 89pp. https://nora.nerc.ac.uk/id/eprint/540833/

Paatero, P. and Tapper, U.: Positive matrix factorization: A non-negative factor model with optimal utilization of error estimates of data values, Environmetrics, 5(2), 111–126, https://doi.org/10.1002/env.3170050203, 1994.

Sandradewi, J., Prévôt, A. S. H., Szidat, S., Perron, N., Alfarra, M. R., Lanz, V. A., Weingartner, E. and Baltensperger, U.: Using aerosol light absorption measurements for the quantitative determination of wood burning and traffic emission contributions to particulate matter., Environ. Sci. Technol., 42(9), 3316–3323, https://doi.org/10.1021/es702253m, 2008.

Savadkoohi, M., Pandolfi, M., Favez, O., Putaud, J.-P., Eleftheriadis, K., Fiebig, M., Hopke, P. K., Laj, P., Wiedensohler, A., Alados-Arboledas, L., Bastian, S., Chazeau, B., María, Á. C., Colombi, C., Costabile, F., Green, D. C., Hueglin, C., Liakakou, E., Luoma, K., Listrani, S., Mihalopoulos, N., Marchand, N., Močnik, G., Niemi, J. V., Ondráček, J., Petit, J.-E., Rattigan, O. V., Reche, C., Timonen, H., Titos, G., Tremper, A. H., Vratolis, S., Vodička, P., Funes, E. Y., Zíková, N., Harrison, R. M., Petäjä, T., Alastuey, A. and Querol, X.: Recommendations for reporting equivalent black carbon (eBC) mass concentrations based on long-term pan-European in-situ observations., Environ. Int., 185, 108553, https://doi.org/10.1016/j.envint.2024.108553, 2024.

Ulbrich, I. M., Canagaratna, M. R., Zhang, Q., Worsnop, D. R. and Jimenez, J. L.: Interpretation of organic components from Positive Matrix Factorization of aerosol mass spectrometric data, Atmospheric Chemistry and Physics, 9(9), 2891–2918, https://doi.org/10.5194/acp-9-2891-2009, 2009.