# Working Title: European Monitoring and Evaluation Programme Model for the UK (EMEP4UK) daily atmospheric composition and fluxes for 2022 - 2024

# Summary

This dataset includes daily averaged atmospheric composition for the UK for key air pollutants nitrogen, sulphur, particulates and ozone for the years 2022 - 2024. The dataset was generated with the European Monitoring and Evaluation Programme for the UK (EMEP4UK) model version 5.0, modified to improve wet deposition rates. The underlying meteorology was generated using the Weather Research and Forecasting (WRF) model version 4.4.2. The UK emissions were taken from the 2025 release of the National Atmospheric Emissions Inventory (NAEI), with base year 2023.

These model outputs were generated as part of the National Capability for UK Challenges (NC-UK) programme, funded by the UK Natural Environment Research Council (NERC), and the National Focal Centre (NFC) work on Mapping and Modelling Critical Loads and Critical Levels, funded by the UK Department for Environment, Food & Rural Affairs (Defra).

# File format

NetCDF

# File names and/or naming convention

There are 3 files with the following name convention:

EMEP4UK_met<metYYYY>_emis<emisYYYY>.nc

Where:

metYYYY is the year of meteorological data used for the modelling, (2022, 2023 or 2024); and

emisYYYY is the year of emissions data used for the modelling (2022 or 2023). 2023 emissions were also used for modelling 2024 as NAEI emissions for 2024 are not yet available.

# Nature and units of recorded values

Fine particulate matter has an aerodynamic diameter less than 2.5 µm.

Coarse particulate matter has an aerodynamic diameter between 2.5 µm and 10 µm.

Primary PM2.5 (SURF_ug_PPM25) is the primary component of fine particulate matter, which is emitted as particulates and dispersed within the model without chemical modification.

Secondary particulates are formed in the atmosphere from gaseous emissions.

Total PM2.5 (SURF_ug_PM25_rh50) is defined as: PM2.5 = SO42- + NO3- (fine) + NH4+ + Sea salt (fine) + Mineral Dust (fine) + Secondary organic aerosols (fine) + primary PM2.5 + 0.27 NO3- (coarse) + PM water at relative humidity of 50 %.

Table 1. Description of variables in EMEP4UK netCDF files.

| NetCDF Variable name | Description | Units |
| --- | --- | --- |
| HMIX | Mixing layer height | m |
| SURF_MAXO3 | Mean of daily maximum ozone | ppbv |
| SURF_PM25water | Water bound to PM2.5 particles | µg m-3 |
| SURF_ppb_C5H8 | Near-surface concentration of isoprene | ppbv |
| SURF_ppb_CO | Near-surface concentration of carbon monoxide | ppbv |
| SURF_ppb_HCHO | Near-surface concentration of formaldehyde | ppbv |
| SURF_ppb_HNO3 | Near-surface concentration of nitric acid | ppbv |
| SURF_ppb_NH3 | Near-surface concentration of ammonia gas | ppbv |
| SURF_ppb_NO | Near-surface concentration of nitrogen monoxide (nitric oxide) | ppbv |
| SURF_ppb_NO2 | Near-surface concentration of nitrogen dioxide | ppbv |
| SURF_ppb_O3 | Near-surface concentration of ozone | ppbv |
| SURF_ppb_SO2 | Near-surface concentration of sulphur dioxide | ppbv |
| SURF_ug_DUST | Near-surface concentration of dust particulates | µg m-3 |
| SURF_ug_DUST_NAT_C | Near-surface concentration of natural coarse dust | µg m-3 |
| SURF_ug_DUST_NAT_F | Near-surface concentration of natural fine dust | µg m-3 |
| SURF_ug_DUST_ROAD_C | Near-surface concentration of coarse resuspended road dust | µg m-3 |
| SURF_ug_DUST_ROAD_F | Near-surface concentration of fine resuspended road dust | µg m-3 |
| SURF_ug_DUST_WB_C | Near-surface concentration of coarse wind-blown dust | µg m-3 |
| SURF_ug_DUST_WB_F | Near-surface concentration of fine wind-blown dust | µg m-3 |
| SURF_ug_ECCOARSE | Near-surface concentration of coarse elemental carbon particulates | µg m-3 |
| SURF_ug_ECFINE | Near-surface concentration of fine elemental carbon particulates | µg m-3 |
| SURF_ug_HNO3 | Near-surface concentration of nitric acid | µg m-3 |
| SURF_ug_NH3 | Near-surface concentration of ammonia gas | µg m-3 |
| SURF_ug_NH4_F | Near-surface concentration of ammonium particulate | µg m-3 |
| SURF_ug_NO | Near-surface concentration of nitrogen monoxide (nitric oxide) | µg m-3 |
| SURF_ug_NO2 | Near-surface concentration of nitrogen dioxide | µg m-3 |
| SURF_ug_NO3_C | Near-surface concentration of coarse nitrate particulate | µg m-3 |
| SURF_ug_NO3_F | Near-surface concentration of fine nitrate particulate | µg m-3 |
| SURF_ug_PM_ASOA | Near-surface concentration of anthropogenic secondary organic aerosols (particulate) | µg m-3 |
| SURF_ug_PM_BSOA | Near-surface concentration of biogenic secondary organic aerosols (particulate) | µg m-3 |
| SURF_ug_PM_OM25 | Near-surface concentration of fine organic matter particulates | µg m-3 |
| SURF_ug_PM_PMFINE | Near-surface concentration of particulate matter with aerodynamic diameter < 2.5 µm | µg m-3 |
| SURF_ug_PM10 | Near-surface concentration of dry particulate matter with aerodynamic diameter < 10 µm | µg m-3 |
| SURF_ug_PM10_rh50 | Near-surface concentration of particulate matter with aerodynamic diameter < 10 µm at 50% relative humidity | µg m-3 |
| SURF_ug_PM25 | Near-surface concentration of dry particulate matter with aerodynamic diameter < 2.5 µm including 27% coarse nitrate | µg m-3 |
| SURF_ug_PM25_rh50 | Near-surface concentration of particulate matter with aerodynamic diameter < 2.5 µm including 27% coarse nitrate, at 50% relative humidity | µg m-3 |
| SURF_ug_PPM_C | Near-surface concentration of primary coarse particulates (aerodynamic diameter 2.5 µm - 10 µm) | µg m-3 |
| SURF_ug_PPM25 | Near-surface concentration of primary fine particulates (aerodynamic diameter < 2.5 µm) | µg m-3 |
| SURF_ug_SEASALT_C | Near-surface concentration of coarse sea salt particulates | µg m-3 |
| SURF_ug_SEASALT_F | Near-surface concentration of fine sea salt particulates | µg m-3 |
| SURF_ug_SIA | Near-surface concentration of secondary inorganic aerosols | µg m-3 |
| SURF_ug_SO2 | Near-surface concentration of sulphur dioxide | µg m-3 |
| SURF_ug_SO4 | Near-surface concentration of sulphate particulates | µg m-3 |
| SURF_ug_SS | Near-surface concentration of sea salt particulates | µg m-3 |
| SURF_ugC_PM_OM25 | Near-surface concentration of carbon in fine organic matter particulates | µgC m-3 |
| SURF_ugC_PM_OMCOARSE | Near-surface concentration of carbon in coarse organic matter particulates | µgC m-3 |
| SURF_ugN_NOX | Near-surface concentration of nitrogen in nitrogen oxides | µgN m-3 |
| SURF_ugN_OXN | Near-surface concentration of nitrogen in oxidised nitrogen species (gas and aerosol) | µgN m-3 |
| SURF_ugN_RDN | Near-surface concentration of nitrogen in reduced nitrogen species (gas and aerosol) | µgN m-3 |
| WDEP_HNO3 | Wet deposition of nitrogen in nitric acid | mgN m-2 |
| WDEP_OXN | Wet deposition of nitrogen in oxidised nitrogen species | mgN m-2 |
| WDEP_PREC | Total rainfall used for wet deposition | mm |
| WDEP_RDN | Wet deposition of nitrogen in reduced nitrogen species | mgN m-2 |
| WDEP_SOX | Wet deposition of sulphur in oxidised sulphur species | mgS m-2 |
| DDEP_OXN_m2Grid | Dry deposition of nitrogen in oxidised nitrogen species | mgN m-2 |
| DDEP_RDN_m2Grid | Dry deposition of nitrogen in reduced nitrogen species | mgN m-2 |
| DDEP_SOX_m2Grid | Dry deposition of sulphur in oxidised sulphur species | mgS m-2 |

# Spatial coverage

The model domain covers the UK and Republic and Ireland. The model grid is regular in a polar stereographic projected coordinate system at 3 km x 3 km horizontal resolution. The output files include both 1D variables for projected coordinate values at grid cell centres and 2D latitude and longitude variables which define the same locations. The polar stereographic projection is described in the following proj4 string: '+proj=stere +ellps=sphere +R=6370000 +lat_0=90.0 +lon_0=0.0 +x_0=0.0 +y_0=0.0 +units=km +k_0=0.9330127018922193 +no_defs +type=crs'

The bounding coordinates in longitude-latitude (spherical datum) are:

SW corner lon -10.9424° lat 48.8934°

SE corner lon 2.90434° lat 49.5153°

NE corner lon 4.8545° lat 61.2292°

NW corner lon -15.5621° lat 60.2845°

# Temporal coverage and resolution

The data are daily averages for the years 2022, 2023 and 2024.

# Models

The EMEP4UK rv5.0 model (Simpson et al., 2012; Vieno et al., 2016a, 2016b) is an atmospheric chemistry transport model (ACTM), derived from the EMEP MSC-W model rv5.0 (https://github.com/metno/emep-ctm), which simulates hourly to annual average atmospheric composition and deposition of various pollutants. For this dataset, a slightly modified version of EMEP4UK rv5.0 model was used, where the thresholds for allowing wet deposition were reverted to match the earlier rv4.36 model.

The 3D meteorology is calculated using the Weather Research and Forecasting Model version 4.4.2 (WRF: https://www.mmm.ucar.edu/models/wrf ).

The EMEP4UK and WRF model outer domain covers the EU with a resolution of 27 × 27 km2 with a nested inner domain covering the UK and Republic of Ireland with a resolution of 3 x 3 km2. The WRF model also uses an intermediate domain at 9 x 9 km2 resolution. Only the UK model domain results are included here.

The WRF model (Skamarock et al., 2019) included data assimilation (Newtonian nudging) of the fifth-generation European Centre for Medium-Range Weather Forecasts (ECMWF) atmospheric reanalysis of the global climate (ERA5) at 0.25°x0.25° degrees resolution, every 6 hours (http://cds.climate.copernicus.eu/ ).

## Model compiler

The WRF and EMEP4UK model FORTRAN codes were compiled with the gfortran compiler 11.5.0 20240719 (Red Hat 11.5.0-2). The calculations have been carried out using the UKCEH computer cluster POLAR (Rocky Linux version 9.6 (Blue Onyx)).

## Input data

Anthropogenic emissions of NOx, NH3, SO2, primary fine particulates (PM2.5), primary coarse particulates (PMcoarse), CO and non-methane VOC for all activity sectors in the UK are derived from the National Atmospheric Emission Inventory (NAEI, http://naei.defra.gov.uk) data for the corresponding year, with base year 2023 (released 2025).

The Republic of Ireland national emissions inventory (MapEIre - https://www.epa.ie/publications/research/air/research-317-mapeire-national-mapping-of-greenhouse-gas-and-non-greenhouse-gas-emissions-sources-project.php) was used within Ireland.

The EMEP emission estimates at a resolution of 0.1° × 0.1° provided by the Centre for Emission Inventories and Projections (CEIP, http://www.ceip.at/) were used for all other emissions.

UK land surface data were derived from the Land Cover Map 2015 (Rowland et al., 2017).

European land cover data and other default EMEP input data were obtained from the EMEP repository (https://github.com/metno/emep-ctm).

## Output data

Daily average concentration values output by EMEP4UK are included in this dataset. Each file contains data for a single year (2022, 2023 or 2024). The netCDF files were post-processed using NCO tools (version 5.3.3-foss-2023b) to reduce the number of variables included. The file names were also simplified.

## Details of the netCDF data structure

An example of the 2022 output file showing the SURF_ug_PM25_rh50 data variable is as follows. The i, j variables give the projected coordinate values for grid cell centres, while the lat, lon variables give the corresponding latitude-longitude values.

netcdf EMEP4UK_met2022_emis2022 {

dimensions:

time = UNLIMITED ; // (365 currently)

j = 447 ;

i = 369 ;

variables:

int Polar_Stereographic ;

Polar_Stereographic:grid_mapping_name = "polar_stereographic" ;

Polar_Stereographic:straight_vertical_longitude_from_pole = 0. ;

Polar_Stereographic:latitude_of_projection_origin = 90. ;

Polar_Stereographic:scale_factor_at_projection_origin = 0.933012701892219 ;

float SURF_ug_PM25_rh50(time, j, i) ;

SURF_ug_PM25_rh50:long_name = "SURF_ug_PM25_rh50" ;

SURF_ug_PM25_rh50:_FillValue = 9.96921e+36f ;

SURF_ug_PM25_rh50:numberofrecords = 365 ;

SURF_ug_PM25_rh50:class = "PM25_rh50" ;

SURF_ug_PM25_rh50:units = "ug/m3" ;

SURF_ug_PM25_rh50:current_date_first = 2022, 1, 2, 6 ;

SURF_ug_PM25_rh50:grid_mapping = "Polar_Stereographic" ;

SURF_ug_PM25_rh50:current_date_last = 2023, 1, 1, 0 ;

SURF_ug_PM25_rh50:coordinates = "lat lon" ;

float i(i) ;

i:standard_name = "projection_x_coordinate" ;

i:coord_axis = "x" ;

i:long_name = "EMEP grid x coordinate" ;

i:units = "km" ;

float j(j) ;

j:standard_name = "projection_y_coordinate" ;

j:coord_axis = "y" ;

j:long_name = "EMEP grid y coordinate" ;

j:units = "km" ;

float lat(j, i) ;

lat:long_name = "latitude" ;

lat:units = "degrees_north" ;

lat:standard_name = "latitude" ;

float lon(j, i) ;

lon:standard_name = "longitude" ;

lon:long_name = "longitude" ;

lon:units = "degrees_east" ;

double time(time) ;

time:long_name = "time at middle of period" ;

time:units = "days since 1900-1-1 0:0:0" ;

time:calendar = "standard" ;

}

# Quality control

For each year of model output an automated QA/QC process developed in R has been carried out, including map visualisations and comparisons with routine UK measurements. The QA/QC reports are available on request but are not included in this document.

The use of the content of this dataset is entirely at your own risk. We make no warranty or guarantee that the content is error free or fit for your intended use.

# Miscellaneous: Guidance for using the data

The model grid is regular in a polar stereographic projection (described in the ‘Spatial coverage’ section), you may need to define this projection in your application. This data set requires a NetCDF library which is, among others, readily available for R, Python, and FORTRAN.

# References

Rowland, C.S., Morton, R.D., Carrasco, L., McShane, G., O'Neil, A.W., Wood, C.M. (2017). Land Cover Map 2015 (vector, GB). NERC Environmental Information Data Centre. doi: 10.5285/6c6c9203-7333-4d96-88ab-78925e7a4e73

Simpson, D., Benedictow, A., Berge, H., Bergström, R., Emberson, L.D., Fagerli, H., Flechard, C.R., Hayman, G.D., Gauss, M., Jonson, J.E., et al. (2012). The EMEP MSC-W chemical transport model – technical description. Atmos. Chem. Phys. 12, 7825–7865.

Skamarock, W.C., Klemp, J.B., Dudhia, J., Gill, D.O., Liu, Z., Berner, J., Wang, W., Powers, J.G., Duda, M.G., Barker, D.M., et al. (2019). A Description of the Advanced Research WRF Model Version 4. UCAR/NCAR.

Vieno, M., Heal, M.R., Williams, M.L., Carnell, E.J., Nemitz, E., Stedman, J.R., and Reis, S. (2016a). The sensitivities of emissions reductions for the mitigation of UK PM2.5. Atmos. Chem. Phys. 16, 265–276.

Vieno, M., Heal, M.R., Twigg, M.M., MacKenzie, I.A., Braban, C.F., Lingard, J.J.N., Ritchie, S., Beck, R.C., Móring, A., Ots, R., et al. (2016b). The UK particulate matter air pollution episode of March–April 2014: more than Saharan dust. Environmental Research Letters 11, 044004.