Visible light images of a semi-deciduous tropical forest, Bobiri Forest Reserve, Ghana

– Supporting Information

As a condition of using these data you must cite it. See the catalogue page, under ‘Get data’ for the full citation.

Table of Contents

1	Overview	3

2	Parameters	3

3	File Information	3

3.1	File Format	3

3.2	File naming convention	3

3.3	File contents	3

3.4	Nature and units of recorded values	3

4	Spatial Coverage	3

5	Temporal Coverage	4

5.1	Temporal resolution	4

6	Data Acquisition and Processing	4

6.1	Experimental design/sampling regime	4

6.2	Collection/Generation/Transformation Methods	4

6.3	Fieldwork and/or laboratory instrumentation	4

6.4	Calibration steps and values	4

6.5	Analytical methods	5

6.6	Quality Control	5

7	Miscellaneous	5

8	References	5

# Overview

The dataset provides visible light images from a camera positioned on a tower at Bobiri Forest Reserve, Ghana. This site is a semi-deciduous tropical forest, with mean annual precipitation approx. 1500 mm, mean annual temperature approx. 25 °C, and loamy soil. The forest area around the tower was lightly selectively logged over 60 years ago. Focal species in the images are Triplochiton scleroxylon, Terminalia superba, Celtis zenkeri, Turreanthus africanus, Guarea cedrata and Pterygota macrocarpa. For more information on the site see Djagbletey et al. (2018) and Moore et al. (2018). The data are visible light images, predominately of tree canopies (leaves, branches, stems). Data were collected in order to compliment additional data on canopy temperature at the same site, but can also be used to look at vegetation phenology. Data were collected by the dataset authors.

# File Information

## File Folder structure and naming convention

Each folder contains data from one month, with the folder name in the format YYMM, for example 2307 contains data from July 2023.

Each file contains data from one image taken by the visible light camera. The file name reflects the site name, date of image in YYMMDD format, time of the image collection in HHMM format, and the file extension .jpg. For example, file Bobiri_230709_1830.txt was collected at the Bobiri site, at 18:30 on 9 July 2023.

## File contents

Each .jpg file contains a 2560 x 1920 pixel image of the forest canopy in standard RGB format. Each file represents on single image taken on the date and time specified in the file name.

# Spatial Coverage and Site Overview

The site is located at 6.69°N, 1.34°W in the Ashanti Region, Ghana, and is described in see Djagbletey et al. (2018) and Moore et al. (2018). Elevation of the site is 280 masl. The site is a natural semi-deciduous tropical forest. The area was logged lightly over 60 years ago. The field of view of the camera includes plot BOB-02 detailed in Moore et al. (2018).

# Temporal Coverage

## Temporal resolution

Data collection was initiated on 1 November 2022 and continues through to 30 November 2023. Some data is missing due to technical faults, and there are no image files for these instances. Data are collected at 10 minute temporal resolution, throughout both day and night, and all through all seasons of the year.

# Data Acquisition and Processing

## Experimental design/sampling regime

The data are purely observational, there are no experimental treatments. The camera is situated on a 60m tower. It is orientated towards plot BOB-02, at approximately 20° downwards from the horizontal. Pixel size varies with distance from the camera, viewing approximately a 100 m swath of forest, with the furthest trees at the horizon 100s m distance from the camera. The larger tree crowns discernable in the image are within 200 m of the camera. The position of the camera was chosen such that it observes a range of different species within the field of view, and includes the forest inventory plot BOB-02. Figure 1 shows an example image.

Figure 1. Visible light image, taken 30 November 2023 at 14:20.

## Collection/Generation/Transformation Methods

Data were recorded as .jpg files outputted from the camera. Labview software was used to control instrument using a bespoke program. Data were initially stored on a PC at the site, then copied to a University of Plymouth server.

## Fieldwork and/or laboratory instrumentation

Images were collected using a HiLook IPC-T651H-Z IP camera.

## Calibration steps and values

No post-processing has been applied to this data.

## Quality Control

Data provided are raw data from the camera.

# References

Djagbletey et al. (2018) Floristic composition and carbon stocks of tree species of different conservation status following selective logging in a moist semi-deciduous forest in Ghana. Ghana Journal of Forestry, 34, 15–34.

Moore et al. (2018) Forest biomass, productivity and carbon cycling along a rainfall gradient in West Africa. Global Change Biology, 24, e496–e510. DOI: 10.1111/gcb.13907