Title

Wildfire simulation outputs from Fire Dynamics Simulator (FDS) using real-world fuel data from forest plots, Oregon, USA, 2019

Summary

This dataset contains outputs of wildfire simulations in Fire Dynamics Simulator (FDS) in forest environments. It includes three simulations, each representing different simulation parameters. The forest plot used for the simulations consists of a 40 x 40 m Pinus ponderosa plot scanned with a Riegl-VZ400i in Sycan Marsh, Oregon, USA, during the summer of 2019. The inputs of the simulations are available at Cabo et al., 2025 https://doi.org/10.5285/467a735f-b03c-4c30-8781-82f0e11aec28.

File Format and structure

The dataset includes CSV files containing the numeric results of the simulations, OUT, TXT and INI files containing the specifications of the simulation (software version, date, number of threads, simulation domain, units, etc.), and FDS specific output files: PRT5 (Part 5 Files), S3D (Slice Files) SF (Surface Files), BND (Boundary Files), GBND (Gas Boundary Files), Q (Heat Release Files), SMV (Smokeview Files) and SZ (Smokeview Sizing Files).

CSV files:

- Simulation_x_cpu.csv: Table containing information about the CPU usage by FDS (Table 1).

- Simulation_x_devc.csv: Table containing the ground fuel dry mass, surface fuel dry mass, wood dry mass, ladder fuel dry mass left (kg) and the flame height (kW/m) at every time step (Table 2).

- Simulation_x_hrr.csv: Table containing, at every time step, information about the total energy budget of the simulation (HRR). The columns correspond to the integrals of the terms in the enthalpy transport equation described in FDS User Guide, section 22.10.1 (McGrattan et al., 2013) (Table 3). Further references about these terms and their units can be found in that section, available in https://pages.nist.gov/fds-smv/manuals.html.

- Simulation_x_steps.csv: Table containing information about the time steps (actual time, GPU time, step size, etc.) (Table 4).

Table 1: Column header description for Simulation_x_cpu.csv.

| Column header | Unit | Description |
| --- | --- | --- |
| Rank |  | Number of MPI process (starting at 0) |
| MAIN | s | Time allocated to MAIN routine |
| DIVG | s | Time allocated to DIVG routine |
| MASS | s | Time allocated to MASS routine |
| VELO | s | Time allocated to VELO routine |
| PRES | s | Time allocated to PRES routine |
| WALL | s | Time allocated to WALL routine |
| DUMP | s | Time allocated to DUMP routine |
| PART | s | Time allocated to PART routine |
| RADI | s | Time allocated to RADI routine |
| FIRE | s | Time allocated to FIRE routine |
| COMM | s | Time allocated to COMM routine |
| BLNK | s | Time allocated to BLNK routine. |
| HVAC | s | Time allocated to HVAC routine. |
| GEOM | s | Time allocated to GEOM routine. |
| VEGE | s | Time allocated to VEGE routine. |
| Total_T_used | s | Total computation time. |

Table 2: Column header description for Simulation_x_devc.csv

| Column header | Unit | Description |
| --- | --- | --- |
| Time | s | Simulation time step. |
| ground_dry_mass | kg | Remaining dry biomass of ground fuel. |
| surface_dry_mass | kg | Remaining dry biomass of surface fuel. |
| wood_dry_mass | kg | Remaining dry biomass of tree stems. |
| ladder_dry_mass | kg | Remaining dry biomass of ladder fuel. |
| flame_intensity | kW/m | Fire intensity (time-averaged over the time interval between time steps). |

Table 3: Column header description for Simulation_x_hrr.csv.

| Column header | Unit | Description |
| --- | --- | --- |
| Time | s | Simulation time step. |
| HRR | kW | Heat release rate of the fire. |
| Q_RADI | kW | Net rate of radiative heat transfer to the boundaries. |
| Q_CONV | kW | Net rate of convective heat transfer to the boundaries. |
| Q_COND | kW | Net rate of heat conduction into solid boundaries. |
| Q_DIFF | kW | Enthalpy associated with fluid species diffusion. |
| Q_PRES | kW | Energy rate associated with pressure work. |
| Q_PART | kW | Heat transfer associated with subgrid particles (e.g., vegetation, droplets). |
| Q_ENTH | kW | Rate of change of sensible enthalpy within the computational domain. |
| Q_TOTAL | kW | Sum of all energy terms (used to check energy conservation against Q_ENTH). |
| MLR_AIR | kg/s | Mass loss rate (or net mass flow rate) of air. |
| MLR_FUEL_VAPOR | kg/s | Mass loss rate (or net mass flow rate) of fuel vapor. |
| MLR_PRODUCTS | kg/s | Mass loss rate (or net mass flow rate) of combustion products. |
| MLR_WATER_VAPOR | kg/s | Mass loss rate (or net mass flow rate) of water vapor. |

Table 4: Column header description for Simulation_x_steps.csv

| Column header | Unit | Description |
| --- | --- | --- |
| Time_Step | s | The cumulative number of computational iterations taken by the FDS solver. |
| Wall_Time | Timestamp | The real-world clock time (ISO 8601 format) when the time step was completed. |
| Step_Size | s | The duration of the current computational time step. |
| Simulation_Time | s | The total physical time elapsed within the simulated fire scenario. |
| CPU_Time | s | The total actual processing time spent by the computer since the simulation started. |

Below is a list about the FDS-specific file formats. They are binary files meant to be used in SmokeView, a free, specialized visualization and analysis tool for FDS outputs. A brief description of each file format is included for context.

- PRT5 (Particle Files). Files that contain data related to particles within the simulation. They store information such as the position, velocity, and size of particles like water droplets, smoke, or other particulates over time. It is the format used by FDS 5 and later.

- S3D (3D Smoke Files) These files store data related to the 3D smoke and fire representation within Smokeview.

- SF (Slice Files) Surface files store 2D slice plane data for specific quantities (e.g., temperature, velocity) at predefined locations. They are used by Smokeview to create cross-sectional views of the simulation.

- BND (Boundary Files) Boundary files contain data on the boundaries of the computational domain.

- GBND (Global Boundary Files) Global bounds of all particle files.

- Q (Plot3D Files) Q files contain data related to the visualization of the results.

- SMV (Smokeview Files) These are the main control files for visualizing the simulation in Smokeview. They define how the other FDS outputs are loaded into Smokeview.

- SZ (Smokeview Sizing files) SZ files contain sizing information about the data files loaded by Smokeview.

Collection/Generation/Transformation methods

Methods use to collect the point cloud data, BDF files and FDS instruction files are described in Cabo et al. (2025).

The simulations were run using FDS 6.9.1 in April-May 2024.  Simulations 1 and 2 were run using 9 MPI processes and took approximately 25 min, whereas simulation 3 was run using 16 and took approximately 68.16 h.

Further information on how to use FDS to run fire simulations can be found in FDS-SMV documentation: https://pages.nist.gov/fds-smv/manuals.html.

How to use the data

To visualize the simulations in Smokeview using the provided output files, follow the steps below. This example uses the file simulation_1.fds as a reference.

Install FDS-SMV

Ensure that Fire Dynamics Simulator (FDS) and Smokeview (SMV) are installed on your system. You can download and install the latest version of FDS-SMV from the official NIST website: https://pages.nist.gov/fds-smv/.

Run SMV

You can view these outputs using SMV double clicking in simulation_1.smv or by running the following command in the CMDfds:

smokeview simulation_1

This command will open the visual outputs of the simulation, allowing you to visualize the fire behaviour within the forest environment. Further details on how to use Smokeview to explore simulation outputs can be found in FDS-SMV documentation: https://pages.nist.gov/fds-smv/manuals.html.

References

Cabo, C., Santin, C., Doerr, S., Laino, D., Parsons, R., & Hudak, A. (2025). Real-world fuel data from forest plots and input configuration for simulations of wildfires in Fire Dynamics Simulator (FDS), Oregon, USA, 2019. NERC EDS Environmental Information Data Centre. https://doi.org/10.5285/467a735f-b03c-4c30-8781-82f0e11aec28

McGrattan, K., McDermott, R., Weinschenk, C., & Forney, G. (2013). Fire dynamics simulator user guide (6th ed., NIST Special Publication 1019). National Institute of Standards and Technology. https://doi.org/10.6028/NIST.sp.1019