Title

3D point clouds of temperate forest stands in the UK and US, 2019-2020

Summary

This dataset consists of high-resolution 3D point clouds of temperate forest stands of diverse species. The point clouds were obtained across the US and the UK during 2019 and 2020. The US point clouds were acquired using a Riegl-VZ400i TLS scanner in Sycan Marsh, Oregon and the UK point clouds were acquired using a GeoSLAM ZEB Horizon MLS in Loch Lomond, Scotland. This dataset can be used to explore structural differences across forests and as test data for 3D point cloud pipelines.

File Format and structure

All point clouds are stored in the compressed LAZ format. The scalar fields stored within each file depend on the acquisition method and hardware used.

- UK point clouds were acquired using Mobile Laser Scanning (MLS) and store the standard LAS scalar fields: Intensity and GPS Time (recorded continuously as scanning time).

- US point clouds were acquired using Terrestrial Laser Scanning (TLS) and store the standard LAS fields: Intensity, Return Number, Number of Returns, and GPS Time (recorded as a constant value). Because they were acquired with a Riegl scanner, they additionally include the waveform-derived attributes Amplitude, Reflectance, and Deviation.

A brief description of each point attribute is provided in Table 1, and the primary tree species associated with each file is detailed in Table 2.

Table 1: Point cloud scalar fields.

| Scalar Field | Description | Source |
| --- | --- | --- |
| Intensity | Return strength of the laser pulse. | UK, US |
| GPS Time | Timestamp of pulse emission. Continuous for MLS; constant for TLS scans. | UK, US |
| Return Number | Sequential number of the return for a single emitted pulse (e.g., 1 for first return). | US |
| Number of Returns | Total count of returns recorded for a given emitted laser pulse. | US |
| Amplitude | Maximum optical power of the backscattered laser echo. | US |
| Reflectance | Range-independent target property indicating the ratio of reflected optical power. | US |
| Deviation | Measure of pulse shape distortion compared to the ideal emitted pulse shape. | US |

Table 1: Main Tree Species in each point cloud.

| Point cloud | Main Species |
| --- | --- |
| UK_2020-10-24_15-11-57 | Picea sitchensis |
| UK_2020-10-25_11-20-57 | Quercus robur |
| UK_2020-10-26_11-11-05 | Pinus sylvestris |
| UK_2020-10-26_14-53-29 | Quercus robur |
| UK_2020-10-26_16-10-36 | Picea sitchensis |
| UK_2020-10-28_11-31-08 | Picea sitchensis |
| UK_2020-10-28_13-10-15 | Larix decidua |
| UK_2020-10-28_13-36-51 | Larix decidua |
| UK_2020-10-28_16-10-06 | Picea abies |
| UK_2020-10-28_16-25-56 | Picea abies |
| UK_2020-10-28_17-00-15 | Quercus robur |
| USA_2019-09-27 | Pinus ponderosa |
| USA_2019-09-28 | Pinus ponderosa |

Collection/Generation/Transformation methods

Site description

US point clouds: Sycan Marsh preserve is a significant wetland area within the Klamath Basin, Oregon, USA, covering more than 8000 hectares. It includes not only the extensive marshland but also portions of the surrounding forested areas, creating a diverse landscape that supports a variety of ecosystems. The point clouds in this dataset come from a 40x40 m Pinus ponderosa forest plot within the preserve.

UK point clouds: Loch Lomond is characterized by a mild and wet maritime climate, with high annual precipitation and relatively stable temperatures year-round due to the moderating influence of the nearby Atlantic Ocean. Loch Lomond itself is part of the Loch Lomond and The Trossachs National Park, a protected area renowned for its landscapes of woodlands, hills, and freshwater lochs.

Point cloud acquisition

US point clouds: Point cloud acquisition was done between 8th and 12th July 2019 in a 200x200 m2 study plot of a ponderosa pine woodland in Sycan Marsh preserve, Oregon, USA. TLS point cloud sampling was performed along a 50 m grid to create a synoptic scan. The point clouds have been acquired with a RIEGL-VZ400i TLS system with a laser sampling rate 750 kHz and a vertical sampling density of 0.21 degrees, mounted on a tripod at a minimum height of 1.5 m. The scanner has a laser wavelength of 1050 nm and a range of 2000 m. Point clouds were georeferenced using the integrated L1 GPS receiver, compass and inclination sensors of the TLS system and ground control points whose coordinates were recorded using an RTK/Differential GPS receiver/base combination.

UK point clouds: Point cloud acquisition was done between 24th and 28th November 2020 in different plots of Loch Lomond. MLS point cloud sampling was performed along approximately 30x30 m plots. The point clouds have been acquired with a GeoSLAM ZEB Horizon handheld MLS system with a laser sampling rate 300 kHz and a vertical sampling density of 0.20 degrees, with a field of view of 360° x 270°. The scanner has a laser wavelength of 903 nm and a range of 100 m.

Point cloud processing

US: The raw scans were pre-processed using Riegl's proprietary software RiSCAN PRO to assign metadata (scanner position, scan ID, date/time) and to align all scans into a common coordinate system.

UK: The raw scans were pre-processed using Faro's proprietary software Faro Connect to assign metadata (scanner position, scan ID, date/time), apply an SLAM algorithm to each scan to register the point clouds and to denoise them. The point clouds were then exported to CloudCompare to trim them to the final shapes.